IEEE P802.11
Wireless LANs

| Normative Text Proposal for Diagnostics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95170 | 408-853-4527 | tolson@cisco.com |
| Bob O’Hara | Cisco Systems | 3625 Cisco Way, San Jose, CA 95134 | 408-853-5513 | bob.ohara@cisco.com |
| Emily H. Qi | Intel Corp. | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |
| Necati Canpolat | Intel Corp. | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-8014 | Necati.Canpolat@intel.com |
| Simon Black | Nokia |  |  | EXT-simon.3.black @nokia.com |
| Jari Jokela | Nokia |  |  | jari.jokela@nokia.com |
| Joe Epstein | Meru Networks | 1309 S Mary Ave, Sunnyvale CA | 408-215-5345 | jepstein@merunetworks.com |

Abstract

This document contains a normative text proposal in support of a Diagnostics protocol in accordance with REQ20370.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

TGv Editor: Replace section 7.3.2.38 with the following section in the next TGv draft.	2

7.3.2.38 Diagnostic Request element	2

7.3.2.38.1 Client Report request	3

7.3.2.38.2 802.11 Authentication request	4

7.3.2.38.3 Association request	4

7.3.2.38.4 802.1X Authentication request	4

7.3.2.38.5 Diagnostic Information Element descriptions	4

7.3.2.38.5.1 AP Descriptor	6

7.3.2.38.5.2 802.1X Credentials	7

7.3.2.38.5.3 Abort Reason	7

7.3.2.38.5.4 Profile ID	8

7.3.2.38.5.5 Client Report Group Type	8

7.3.2.38.5.6 Manufacturer OUI	9

7.3.2.38.5.7 Manufacturer ID String	9

7.3.2.38.5.8 Manufacturer Model String	9

7.3.2.38.5.9 Manufacturer Serial Number String	10

7.3.2.38.5.10 Radio Type	10

7.3.2.38.5.11 Firmware Version	10

7.3.2.38.5.12 MAC Address	10

7.3.2.38.5.13 Antenna Type	11

7.3.2.38.5.14 Antenna Gain	11

7.3.2.38.5.15 Radio Channels	12

7.3.2.38.5.16 TX Power	12

7.3.2.38.5.17 Data Rates	12

7.3.2.38.5.18 Cipher Suite	13

7.3.2.38.5.19 AKM Suite	13

7.3.2.38.5.20 EAP Method	13

7.3.2.38.5.21 Power Save Mode	13

7.3.2.38.5.22 Status Code	14

7.3.2.39 Diagnostic Report element	14

7.3.2.39.1 Client Report	15

7.3.2.39.2 802.11 Authentication Report	17

7.3.2.39.3 Association Report	18

7.3.2.39.4 802.1X Authentication Report	18

11.15.3.1 Client Report	18

11.15.3.1.1	Client Report Grouping	18

11.15.3.1.2	Configuration Profile Group	19

11.15.3.1.3	Operating Parameters Group	19

11.15.3.1.4	Manufacturer Information Group	19

11.15.3.1.5	Capabilities Group	19

11.15.3.2 802.11 Authentication	19

11.15.3.3 Association	19

11.15.3.4 802.1X Authentication	20

A.4.14 Wireless Network Management extensions	20

- TGv Editor: Replace section 7.3.2.38 with the following section in the next TGv draft.

- 7.3.2.38 Diagnostic Request element

The Diagnostic Request element contains a request that the receiving STA undertake the specified diagnostics action. The format of Diagnostic Request element is shown in Error: Reference source not found.

|  | Element ID | Length | Diagnostic Token | Diagnostic Request Type | Diagnostic Request |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure v1—Diagnostic Request element format

The Element ID field is equal to the Diagnostic Request value in Table 26.

The value of the Length field is variable and depends on the length of the Diagnostic Request field. The minimum value of the Length field is 2 (based on a minimum length for the Diagnostic Request field of 0 octets).

The Diagnostic Token is set to a number that is unique among the Diagnostic Request elements sent to each destination MAC address for which a corresponding Diagnostic Report element has not been received.

The Diagnostic Request Type is set to a number that identifies a type of diagnostic request. The defined Diagnostic Request Types are shown in Table v1.

Table v1—Diagnostic Request Type definitions

| Name | Request Type Value |
| --- | --- |
| Client Report | 0 |
| 802.11 Authentication | 1 |
| Association | 2 |
| 802.1X Authentication | 3 |
| Reserved | 4 - 255 |

The Diagnostic Request field contains zero or more Diagnostic Information Elements depending on the specific Diagnostic Request Type.

The Diagnostic Request element is included in a Diagnostic Request frame as described in Error: Reference source not found. The use of Diagnostic Request element and frames is described in Error: Reference source not found

TGv Editor: Insert the following sections after section 7.3.2.38

##### 7.3.2.38.1 Client Report request

The Diagnostic Request element corresponding to a Client Report Diagnostic Request Type is shown in Table v2

Table v2—Client Report request contents

| Order | Information |
| --- | --- |
| 1 | Client Report Group Type |

##### 7.3.2.38.2 802.11 Authentication request

The Diagnostic Request element corresponding to an 802.11 Authentication Request Type is shown in Table v3

Table v3—802.11 Authentication request contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | Profile ID |

##### 7.3.2.38.3 Association request

The Diagnostic Request element corresponding to an Association Request Type is shown in Table v4.

Table v4—Association request contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | Profile ID |

##### 7.3.2.38.4 802.1X Authentication request

The Diagnostic Request element corresponding to an 802.1X Authentication Request Type is shown in Table v5.

Table v5—Authentication request contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | 802.1X credentials |
| 4 | Profile ID |

##### 7.3.2.38.5 Diagnostic Information Element descriptions

The following sections describe the various Diagnostic Information Elements that may be included in Diagnostic Elements field of a Diagnostic Request element or a Diagnostic Report element. The format of a Diagnostic Information element is shown in Figure v2.

|  | Diagnostic Information Element ID | Length | Diagnostic Information Element Contents |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | Variable |

Figure v2—Diagnostic Information Element format

The Diagnostic Information Element ID field indicates the Diagnostic Information Element type and shall be set to any allocated value in Table v6—Diagnostic Information Element ID ValuesTable v6.

The Length field shall be set to total length of the element minus Element ID and Length fields.

The Diagnostic Information Element Contents field carries the content specifics of an individual element type and is described in the subsections that follow.

Table v6—Diagnostic Information Element ID Values

| Value | Description |
| --- | --- |
| 0 | AP descriptor |
| 1 | 802.1X credentials |
| 2 | Abort reason |
| 3 | Profile ID |
| 4 | Manufacturer OUI |
| 5 | Manufacturer ID string |
| 6 | Manufacturer model string |
| 7 | Manufacturer serial number string |
| 8 | Radio type |
| 9 | Firmware Version |
| 10 | MAC address |
| 11 | Antenna Type |
| 12 | Antenna gain |
| 13 | Radio channels |
| 14 | Data Rates |
| 15 | SSID |
| 16 | Tx power |
| 17 | Data rates |
| 18 | Cipher Suite |
| 19 | AKM Suite |
| 20 | EAP Method |
| 21 | Power Save Mode |
| 22 | Status Code |
| 23-65355 | Reserved |

###### 7.3.2.38.5.1 AP Descriptor

The format of the AP descriptor is described in Figure v3.

|  | BSSID | Channel Number | Regulatory Class |
| --- | --- | --- | --- |
| Octets: | 6 | 1 | 1 |

Figure v3—AP descriptor

The BSSID field is a 6-byte field as described in 7.1.3.3.3 that identifies the BSS of the AP Descriptor.

The Channel Number field indicates the current operating channel of the AP represented by the BSSID in the AP Descriptor.

The Regulatory Class field contains an enumerated value from Annex J specifying the frequency band in which the Channel Number is valid.

###### 7.3.2.38.5.2 802.1X Credentials

The format for the 802.1X credentials is shown in Figure v4.

|  | Credentials |
| --- | --- |
| Octets: | 1 |

Figure v4—802.11 credentials format

The Credentials field indicates the type of credential. The valid values for this field are shown in Table v7.

Table v7—802.1X Credential Type Values

| Value | Description |
| --- | --- |
| 0 | Pre-shared key |
| 1 | Username and password |
| 2 | X.509 certificate |
| 3 | Other certificate |
| 4 | One time password |
| 5 | SecureID token |
| 6–254 | Reserved |
| 255 | None |

###### 7.3.2.38.5.3 Abort Reason

The format for Abort Reason is shown in Figure v5.

|  | Abort Reason |
| --- | --- |
| Octets: | 1 |

Figure v5 - Abort Reason format

The values for the abort reason are shown in Table v8.

Table v8—Abort Reason

| Value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Timeout waiting for response from client |
| 2 | Cancelled by operator |
| 3-255 | Reserved |

###### 7.3.2.38.5.4 Profile ID

The format for the Profile ID is shown in Figure v6.

|  | Profile ID |
| --- | --- |
| Octets: | 1 |

Figure v6—Current Profile Indication format

The Profile ID field contains a unique identifier for referencing a configuration profile available on a device.

###### 7.3.2.38.5.5 Client Report Group Type

The format for the Client Report Group Type is shown in Figure v7.

|  | Group Type |
| --- | --- |
| Octets: | 1 |

Figure v7—Client Report Group Type format

The Client Report Group Type is a one byte field that indicates the type of information to be reported in the Client Report and shall be set to any of the allocated values in Table v9.

Table v9—Client Report Group Type definitions

| Client Report Group Type | Value |
| --- | --- |
| Manufacturer Information | 0 |
| Operating Parameters | 1 |
| Capabilities | 2 |
| Configuration Profile | 3 |
| Reserved | 4 – 255 |

###### 7.3.2.38.5.6 Manufacturer OUI

The format for the Manufacturer OUI is shown in Figure v8.

|  | OUI |
| --- | --- |
| Octets: | 3 |

Figure v8—Manufacturer OUI format

The OUI field contains an Organizational Unique Identification, the first 24-bits of the network connected device, which indicate the specific vendor for that device.

###### 7.3.2.38.5.7 Manufacturer ID String

The format for the Manufacturer ID string is shown in Figure v9.

|  | ID |
| --- | --- |
| Octets: | variable |

Figure v9—Manufacturer ID string format

The ID field contains an ASCII string indicating the manufacturer identifier of the wireless network adaptor.

###### 7.3.2.38.5.8 Manufacturer Model String

The format for the Manufacturer model string is shown in Figure v10.

|  | Model |
| --- | --- |
| Octets: | variable |

Figure v10—Manufacturer model string format

The Model field contains an ASCII string indicating the model of the wireless network adaptor.

###### 7.3.2.38.5.9 Manufacturer Serial Number String

The format for the Manufacturer serial number string is shown in Figure v11.

|  | Serial Number |
| --- | --- |
| Octets: | variable |

Figure v11—Manufacturer ID string format

The Serial Number field contains an ASCII string indicating the serial number of the wireless network adaptor.

###### 7.3.2.38.5.10 Radio Type

The format for the Radio Type is shown in Figure v12.

|  | Radio Type |
| --- | --- |
| Octets: | 1 |

Figure v12—Manufacturer ID string format

The Radio Type field contains the type of radio in use by the wireless network adaptor and shall be set to one of the values defined in dot11PHYType.

###### 7.3.2.38.5.11 Firmware Version

The format for the Firmware Version is shown in Figure v13.

|  | Firmware Version |
| --- | --- |
| Octets: | variable |

Figure v13—Firmware Version format

This Firmware Version field contains an ASCII string identifying the version of firmware currently installed on the wireless network adaptor.

###### 7.3.2.38.5.12 MAC Address

The format for the MAC address is shown in Figure v14.

|  | MAC Address |
| --- | --- |
| Octets: | 6 |

Figure v14—MAC address format

This MAC Address field contains the 48-bit IEEE 802 MAC address of the wireless client adaptor.

###### 7.3.2.38.5.13 Antenna Type

The format for the Antenna Type is shown in Figure v15.

|  | Antenna Type |
| --- | --- |
| Octets: | 1 |

Figure v15—Antenna Type format

The Antenna Type field contains the type of antenna connected to the wireless network adaptor and shall be set to one of the values in Table v10.

Table v10—Antenna Types

| Antenna Type | Value |
| --- | --- |
| Omni-directional collinear | 0 |
| Omni-directional diversity | 1 |
| Omni-directional other | 2 |
| Directional yagi | 3 |
| Directional panel | 4 |
| Directional sector | 5 |
| Directional other | 6 |
| Unknown | 7 |
| Reserved | 8-255 |

###### 7.3.2.38.5.14 Antenna Gain

The format for the Antenna gain is shown in Figure v16.

|  | Antenna Gain |
| --- | --- |
| Octets: | 1 |

Figure v16—Antenna gain format

The Antenna Gain field contains the peak gain in dBi of the antenna for directional antennas and the average gain in dBi for omni-directional antennas connected to the wireless network adaptor.

###### 7.3.2.38.5.15 Radio Channels

The format for the Radio channels is shown in Figure v17.

|  | Radio Channels |
| --- | --- |
| Octets: | variable |

Figure v17—Radio channels format

The Radio Channels field contains a list of one or more radio channel numbers. Each byte in the Radio Channels field represents an individual channel number.

###### 7.3.2.38.5.16 TX Power

The format for the Tx Power is shown in Figure v18.

|  | Tx Power Mode | Tx Power |
| --- | --- | --- |
| Octets: | 1 | variable |

Figure v18—Tx Power format

Tx Power Mode: This field identifies the transmit power mode of the client STA and shall be set to one of the values in Table v11.

Table v11—Tx Power Modes

| Tx Power Mode | Value |
| --- | --- |
| Fixed | 0 |
| Automatic | 1 |
| Reserved | 2-32 |

If the Tx Power Mode field is set to be Fixed then this field contains one or more Tx Power values where each byte in the Tx Power field contains a 2’s complement value indicating a Tx Power level in dBm. If the Tx Power Mode field is not set to Fixed then this field shall be omitted.

###### 7.3.2.38.5.17 Data Rates

The format for the Data Rates is shown in Figure v19.

|  | Data Rates |
| --- | --- |
| Octets: | variable |

Figure v19—Data Rates format

The Data Rates field contains one or more Data Rate value. Each byte in the Data Rate field is encoded with an MSB set to a 1 to indicate that the data rate is in the basic rate set and the remaining 6 bit value multiplied by 0.5 Mbit/s to indicate the data rate.

###### 7.3.2.38.5.18 Cipher Suite

The format for the Cipher Suite is shown in Figure v20.

|  | OUI | Suite Type |
| --- | --- | --- |
| Octets: | 3 | 1 |

Figure v20—Cipher Suite format

The OUI and Suite Type fields identify the cipher suite and shall be set to one of the values in Table 32.

###### 7.3.2.38.5.19 AKM Suite

The format for the AKM Suite is shown in Figure v21.

|  | OUI | AKM Suite |
| --- | --- | --- |
| Octets: | 3 | 1 |

Figure v21—AKM Suite format

The OUI and AKM Suite fields identify the authentication method and shall be set to one of the values in Table 34.

###### 7.3.2.38.5.20 EAP Method

The format for the EAP method is shown in Figure v22.

|  | EAP method |
| --- | --- |
| Octets: | 1 |

Figure v22—EAP method format

The EAP Method field contains a value that identifies a particular EAP method and shall be set to any valid IANA assigned EAP type defined at http://www.iana.org/assignments/eap-numbers.

###### 7.3.2.38.5.21 Power Save Mode

The format for the Power Save Mode is shown in Figure v23.

|  | Power Save Mode |
| --- | --- |
| Octets: | 1 |

Figure v23—Power Save Mode format

The Power Save Mode field indicates the type of power save configured on the wireless network adaptor and shall be set to one of the values in Table v12.

Table v12—Power Save Mode definition

| Power Save Mode | Value |
| --- | --- |
| Constantly Awake | 0 |
| Normal Power Save | 1 |
| U-APSD | 2 |
| S-APSD | 3 |
| Reserved | 4–254 |
| Unknown | 255 |

###### 7.3.2.38.5.22 Status Code

The format for the Status Code is shown in Figure v23.

|  | Status Code |
| --- | --- |
| Octets: | 1 |

Figure v24—Status Code format

The Status Code field shall contain the final 802.11 Status code, as defined in Table 23, received at the end of the applicable operation.

- 7.3.2.39 Diagnostic Report element

The Diagnostic Report element contains a Diagnostic report. The format of the Diagnostic Report element is shown in Error: Reference source not found.

|  | Element ID | Length | Diagnostic Token | Diagnostic Report Type | Diagnostic Status | Diagnostic Report |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure v25—Diagnostic Report element format

The Element ID field is equal to the Diagnostic Report value in Table 26.

The value of the Length field is variable and depends on the length of the Diagnostic Report field. The minimum value of the Length field is 3 (based on a minimum length for the Diagnostic Report field of 0 octets).

The Diagnostic Token field is set to the Diagnostic Token in the corresponding Diagnostic Request element.

The Diagnostic Report Type field is set to a number that identifies the Diagnostic report. Those Diagnostic Report Types that have been allocated are shown in Error: Reference source not found.

Table v13—Diagnostic Report Type Definitions for Diagnostic Report Request

| Name | Report Type Value |
| --- | --- |
| Client Report | 0 |
| 802.11 Authentication | 1 |
| Association | 2 |
| 802.1X Authentication | 3 |
| Reserved | 4 – 255 |

The Diagnostic Status field is set to a value in Error: Reference source not found, indicating the client’s response to the Diagnostic Request indicated by the Dialog Token.

Table v14—Diagnostic Status values

| Status Value | Description |
| --- | --- |
| 0 | Successful |
| 1 | Fail |
| 2 | Refused |
| 3 | Incapable |
| 4-255 | Reserved |

The Diagnostic Report field contains the results of the diagnostic request.

The Diagnostic Report element is included in a Diagnostic Report frame as described in Error: Reference source not found. The use of Diagnostic Report element and frames is described in Error: Reference source not found.

TGv Editor: Insert the following sections after section 7.3.2.39 in the current TGv draft.

##### 7.3.2.39.1 Client Report

The Diagnostic IEs field corresponding to a Client Report is populated differently depending on the Client Report Group Type element that was included in the Client Report Diagnostic Request element.

The Diagnostic IEs field corresponding to a Diagnostic Report element of type Client Report with a Client Report Group Type of Manufacturer Information is shown in Table v15.

Table v15—Client Report for Group Type Manufacturer Information contents

| Order | Information |
| --- | --- |
| 1 | Manufacturer OUI |
| 2 | Manufacturer ID string |
| 3 | Manufacturer model string |
| 4 | Manufacturer serial number string |
| 5 | Radio type |
| 6 | Firmware Version |
| 7 | MAC address |
| 8 | Antenna Type |
| 9 | Antenna gain |

The Diagnostic IEs field corresponding to a Diagnostic Report element of type Client Report with a Client Report Group Type of Operating Parameters is shown in Table v16.

Table v16—Client Report for Group Type Operating Parameters contents

| Order | Information |
| --- | --- |
| 1 | MAC Address |
| 2 | Radio channels |
| 3 | Tx Power |
| 4 | Data Rates |
| 5 | SSID |
| 6 | Cipher Suite |
| 7 | AKM Suite |
| 8 | EAP Method |
| 9 | Profile ID |

The Diagnostic IEs field corresponding to a Diagnostic Report element of type Client Report with a Client Report Group Type of Capabilities is shown in Table v17.

Table v17—Client Report for Group Type Capabilities contents

| Order | Information |
| --- | --- |
| 1 | Radio channels |
| 2 | Tx Power |
| 3 | Data Rates |
| 4 | Cipher Suite |
| 5 | AKM Suite |
| 6 | EAP Method |

The Diagnostic IEs field corresponding to a Diagnostic Report element of type Client Report with a Client Report Group Type of Configuration Profile is shown in Table v18.

Table v18—Client Report for Group Type Configuration Profile contents

| Order | Information |
| --- | --- |
| 1 | Profile ID |
| 2 | Radio channels |
| 3 | Tx Power |
| 4 | Data Rates |
| 5 | Cipher Suite |
| 6 | AKM Suite |
| 7 | EAP Method |
| 8 | SSID |

##### 7.3.2.39.2 802.11 Authentication Report

The Diagnostic IEs field corresponding to a Diagnostic Report element of type 802.11 Authentication Report is shown in Table v19.

Table v19—802.11 Authentication Report contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | Status Code |

##### 7.3.2.39.3 Association Report

The Diagnostic IEs field corresponding to a Diagnostic Report element of type Association Report is shown in Table v20.

Table v20—Association Report contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | Status Code |

##### 7.3.2.39.4 802.1X Authentication Report

The Diagnostic IEs field corresponding to a Diagnostic Report element of type 802.1X Authentication Report is shown in Table v21.

Table v21—802.1X Authentication Report contents

| Order | Information |
| --- | --- |
| 1 | AP Descriptor |
| 2 | EAP Method |
| 3 | Credential Type |
| 4 | Status Code |

TGv Editor: Insert the following sections after section 11.15.3 in the current TGv draft.

##### 11.15.3.1 Client Report

The Client Report Diagnostic Request is a mechanism used by STA and AP for the purpose of exchanging STA information. An AP may send a Diagnostic Request frame indicating a Client Report request to any STA that has associated and authenticated with the AP. The Client Report Request frame identifies what type of information is desired by selecting an information group. The following groups are available:

- Manufacturer’s Information – for retrieving data about the NIC card in use

- Operating Parameters – for retrieving the details of the STA’s current operational modes

- Capabilities – for retrieving information about the STA’s capabilities

- Configuration Profile – for retrieving information about the configuration of the STA

Upon receipt of the Diagnostic Request frame containing a Client Report Request the STA generates one or more Diagnostic Report elements each containing a Client Report that include the requested information.

#### Client Report Grouping

In order to simplify the information requests from the AP to client STA, various Client Report Group Types are provided.  When an AP sends a Client Report Request to the client STA it shall indicate a single Client Report Group Type.  The receiving client STA shall only send Diagnostic Information elements allowed for the requested group type.  The client STA shall send all available Diagnostic Information elements for the requested group type.

##### Configuration Profile Group

The purpose of the Configuration Profile group is for the AP to discover all the available configuration profiles as well as the current profile in use for an associated device.  Devices that support multiple profiles may include multiple Diagnostic Report elements in a single Diagnostic Report frame (or multiple frames if required).  Each Diagnostic Report element shall contain a Profile ID element that uniquely identifies the configuration profiles available on the device.

##### Operating Parameters Group

The purpose of the Operating Parameters Group is for an AP to discover various important operating settings currently in use for an associated device.  A client STA receiving a Client Report request type with a Client Report Group Type set to Operating Parameters shall respond with a Diagnostic Report frame that includes all available Client Diagnostic Information elements allowed for the group.  The client STA shall populate the report frame with the parameters that were in use at the time that the Diagnostic Request frame was received.

##### Manufacturer Information Group

The purpose of the Manufacturer Information Group is for an AP to discover all the static manufacturer specific data about an associated device.  A client STA receiving a Client Report request type with a Client Report Group Type equal to Manufacturer Information shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the group.

##### Capabilities Group

The purpose of the Capabilities Group is for an AP to discover the range of capabilities available to an associated device.  A client STA receiving a Client Report request type with a Client Report Group Type set to Client Capability shall respond with a Diagnostic Report frame that includes all available Diagnostic Information elements allowed for the group.

The Client Report Response elements included should describe the available capabilities and not current settings for the capabilities.  It is important that the AP discover what the client STA is capable of rather than what its current settings.

##### 11.15.3.2 802.11 Authentication

The purpose of the 802.11 Authentication diagnostic is to determine that the STA is able to perform an 802.11 authentication properly with a designated BSS. This test directs an 802.11 authentication to be completed with a specific AP.

To initiate the test, the AP shall send a Diagnostic Request frame containing an 802.11 authentication request to the STA. All parameters required to complete the 802.11 authentication are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an 802.11 authentication request, the STA shall cause the normally instantiated protocol stack to perform the 802.11 authentication using the requested parameters.

Upon completion of the test, the STA shall respond with a Diagnostic Report frame that includes a Diagnostic Report element that describes the result of the operation.

##### 11.15.3.3 Association

The purpose of the Association diagnostic is to determine that the STA is able to associate properly with a designated BSS. This test directs an association to be completed with a specific AP.

To initiate the test, the AP shall send a Diagnostic Request frame containing an Association request to the STA.  All parameters required to complete the association are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an Association request, the STA shall cause the normally instantiated protocol stack to perform the association using the requested parameters.

Upon completion of the test, the STA shall respond with a Diagnostic Report frame that includes a Diagnostic Report element that describes the result of the operation.

##### 11.15.3.4 802.1X Authentication

The purpose of the 802.1X Authentication diagnostic is to determine that the STA is able to properly complete an 802.1X authentication with a designated BSS. This test directs an association and 802.1X authentication to be completed with a specific AP.

To initiate the test, the AP shall send a Diagnostic Request frame containing an 802.1X Authentication request to the STA. All parameters required to complete the operation including the association and 802.1X authentication are included in the Diagnostic Request element.

Upon receipt of the Diagnostic Request frame containing a Diagnostic Request element specifying an 802.1X authentication request, the STA shall cause the normally instantiated protocol stack to perform the association and 802.1X authentication using the requested parameters.

Upon completion of the test, the STA shall respond with a Diagnostic Report frame that includes a Diagnostic Report element that describes the result of the operation.

TGK Editor: Insert the new rows below into clause A.4.13:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME4.3 | Client Report Diagnostic Type | 7.3.2.38, 7.3.2.38.1, 7.3.2.39, 7.3.2.39.1, 11.15.3.1 | CFv:M | Yes, No, N/A |
| RME4.4 | 802.11 Authentication Diagnostic Type | 7.3.2.38, 7.3.2.38.2, 7.3.2.39, 7.3.2.39.2, 11.15.3.2 | CFv:M | Yes, No, N/A |
| RME4.5 | Association Diagnostic Type | 7.3.2.38, 7.3.2.38.3, 7.3.2.39, 7.3.2.39.3, 11.15.3.3 | CFv:M | Yes, No, N/A |
| RME4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.38, 7.3.2.38.4, 7.3.2.39, 7.3.2.39.4, 11.15.3.4 | CFv:M | Yes, No, N/A |