IEEE P802.11
Wireless LANs

| Including FT Policy and Capability in MDIE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date March 06, 2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Frank Ciotti | Motorola | 7700 W. Parmer Ln Austin, TX 78726 | 512-996-5753 | frank.ciotti@motorola.com |
| Bill Marshall | AT&T | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |
| Mike Montemurro | Chantry Networks | 1900 Minnesotta Court Ste 125, Mississauga ON L5N3C9 | 905-363-6413 | Michael.montemurro@siemens.com |

Abstract

The document provides the text updates to TGr draft D1.1.  The text addresses the Beacon Bloat comments to provide mechanisms that reduce the size of beacon and probe response for Fast BSS Transitions.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Updates based on TGr Draft D1.1

Section 7.3.2.42, Page 22, line 20

Replace:

The Mobility Domain information element contains a Mobility Domain Identifier. The Access Point uses the

Mobility Domain Information Element to advertise that it is included in the group of APs that constitute the

Mobility Domain. The format for this information element is given in Figure 80AC.

The Length field shall be set to 6.

The Mobility Domain Identifier is a 48-bit value.

With:

The Mobility Domain information element contains the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. The Access Point uses the Mobility Domain Information Element to advertise that it is included in the group of APs that constitute the Mobility Domain, to advertise its support for Fast BSS Transition capability, and to advertrise its resource policy information.  The format for this information element is given in Figure 80AC.

The Length field shall be set to 7.

The Mobility Domain Identifier is a 48-bit value.

The Fast BSS Transition Capability and Resource Policy field is 1 octet.

When sent by an AP, the Fast BSS Transition Capability and Resource Policy value is defined in Figure 80AE. When sent by a STA to an AP, the Fast BSS Transition Capability and Resource Policy value is identical to the Policy value advertised by that AP.

Insert Figure 80AE here from Section 7.3.2.43.  Remove Figure 80AE from Section 7.3.2.43

Replace Title of Figure 80AE from “Figure 80AE—Fast BSS transition resource mechanism value”

With: “Figure 80AE—Fast BSS Transition Capability and Resource Policy value”

Fast BSS Transition Capability is set if the AP supports Fast BSS Transition

Reservation over air is set if the AP recommends Reservation be done over-the-air.

Reservation over DS is set if the AP recommends Reservation be done over-the-DS.

Reserve Option is set if AP requires a reservation if a RIC request will appear in a (re)association, and clear

if the STA’s use of reservation is optional

Reservation Limit is set to the maximum number of APs at which a STA can make concurrent reservations.

Value of zero indicates no limit.

The various combinations of mechanisms supported by the AP are shown in Table 31J:

Insert Table 31J  here from Section 7.3.2.43.  Remove Table 31J from Section 7.3.2.43

The STA can use information from the MDIE to determine the transition mechanisms supported and recommended

by the AP. The choice of executing any specific transition mechanism is outside the scope of this standard.

Replace Figure 80AE in Section 7.3.2.42

With: the following

Fast BSS Transition Capability

Reserve Option

Reservation Over DS

Reservation Limit

Reservation Over Air

Bit:       B0                       B1                                 B2                     B3                                             B4-7

Section 7.3.2.42, Page 22, line 20

Replace: Figure 80AC

With:

Element ID

Length

Mobility Domain

Identifier

Fast BSS Transition  Capability and Resource

Policy

Octets:    1                 1                          6                           1

Section 7.3.2.43, Page 22, line 47

Replace:

The Fast BSS Transition Information Element is defined to include network infrastructure resource reservation

and security policy information. An AP can advertise the FTIE by including the FTIE in a Beacon frame

and Probe Response frame. This information element is defined in Figure 80AD.

With:

The Fast BSS Transition Information Element (FTIE) is defined to include key holder identity information.  This information element is defined in Figure 80AD.

Section 7.3.2.43, Page 23, line 15

Replace: Figure 80AD

With:

Element ID

Length

R0KH-ID

R1KH-ID

# Octets:      1                      1                        16                                 16

The Length field shall be set to 16 for FTIE without the R1KH-ID field

The Length field shall be set to 32 for FTIE wih the R1KH-ID field