IEEE P802.11
Wireless LANs

| RCPI Comments Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-3-7 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 West Tasman Dr. San Jose, CA 95137 | 408-853-4527 | tolson@cisco.com |

Abstract

This document contains normative text changes for letter ballot 78 comments.  This document resolves comments: 1223 and 1225.

#### Instruct the TGk editor to replace the following sections (7.2.3.5 and 7.2.3.7) in the next TGk draft.

#### 7.2.3.5 Association Response frame format

Insert new rows into table 8 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 9 | RSNI | The RSNI information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new rows into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 9 | RSNI | The RSNI information element shall be present if dot11RadioMeasurementEnabled is true. |

[Instruct the TGk editor to insert the following text after section 10.3.2.2.2 in the current TGk draft.]

10.3.6.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.indication	(
PeerSTAAddress, 
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association was received. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by thepeer MAC as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MACentity as part of the association request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting association. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. Only one pairwise cipher suite and only one authenticated key suite are allowed in the RSN information element. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. |

10.3.6.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association request was received. |
| ResultCode | Enummeration | SUCCESS, REFUSED_REASON_UNSPECIFIED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result response to the association request from the peer MAC entity. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| AssociationID | Integer | 1-2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned to the peer MAC entity by the AP. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. |

10.3.7.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.indication	(
PeerSTAAddress, 
PeerSTAAddress,
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the reassociation was received. |
| CurrentAPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the AP with which thepeer station is currently associated. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the reassociation request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by thepeer MAC as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MACentity as part of the reassociation request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting reassociation. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Reassociation Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Reassociation Request frame was received. |

10.3.7.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the reassociation request was received. |
| ResultCode | Enummeration | SUCCESS, REFUSED_REASON_UNSPECIFIED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result response to the reassociation request from the peer MAC entity. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capabilities advertised by the AP. |
| AssociationID | Integer | 1-2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned to the peer MAC entity by the AP. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. |