IEEE P802.11
Wireless LANs

| TGp Comment Resolution Forms |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Doug Kavner | Raytheon | 1801 Hughes Drive, Fullerton, CA 92834 | 714-446-2276 | dkavner@raytheon.com |

Abstract

This document describes proposed resolutions to several comments from Letter Ballot 80 on IEEE 802.11p Draft 1.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  |  |  |  | LB80 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 39 | Aboba | 7.1.3.3.3 | Doug Kavner |  | 3/9/06 |

- COMMENT:  No mechanism is specified for preventing, detecting or correcting MAC address collisions.

- Commenter’s Suggested Remedy:  Add a reference to RFC 1750. Specify how stations claim and defend MAC addresses.

- Background, Explanation, Discussion, etc.:

It is not feasible to use RFC 1750 due to the highly transient nature of WAVE. A station could regenerate its own MAC address upon detecting a conflict, but this would enable an easy denial of service attack by a station that merely broadcasts frames with a TA of stations being attacked.  Studies show less than 50 short-term MAC address collisions per year within the United States under full deployment and the probability of one of these leading to a vehicle safety hazard is incredibly remote.

- Recommended Resolution of the Comment:

Reject the comment.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to xxxxxxxxxxxxxxxxxx     …  .

Motion by: ____________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

|  |  |  |  | LB80 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 41 | Jokela | 5.2.7 | Doug Kavner |  | 3/9/06 |

- COMMENT:  It is mentioned that Beacon frames are not used in WBSS. First comment is that what is the motivation to not to use one of the most fundamental frames in 802.11 system? Other comment is that is it not allowed to send Beacons at all while operating in WBSS mode. Same comments applies to Probe Request/Probe Response.

- Commenter’s Suggested Remedy:  Please clarify what is the reason not to use Beacon/Probe Request/Probe Response frames. If usage of these frames are not allowed in WBSS then I assume there are numerous places in the draft where this shall be highlighted.

- Background, Explanation, Discussion, etc.:

Beacons are not allowed in WAVE mode.  A beacon has many mandatory fields that are not needed by WAVE causing additional overhead.  WAVE also needs a data element greater than 255 bytes due to a security wrapper, and this is not supported in the current 802.11 beacon format.  The WAVE announcement frame serves the same general purpose as a beacon, adapted to WAVE.  It is possible to rename the frame to “WAVE beacon”, but this would likely cause too much confusion.

- Recommended Resolution of the Comment:

Agree that there are other sections in which the lack of beacons should be highlighted.  Need to identify those sections and correct as needed.  Explain that a WAVE announcement includes the TSF value as does a beacon.  Add appropriate text from explanation above into the description of WAVE in 802.11p.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to xxxxxxxxxxxxxxxxxx     …  .

Motion by: ____________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

|  |  |  |  | LB80 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 94 | Tokubo | All | Doug Kavner |  | 3/9/06 |

- COMMENT:  See many inconsistencies and issues that TGp will cause to the 802.11 spec in its current form.

- Commenter’s Suggested Remedy:  Become your own 802 WG and make your own specification (based off of 802.11 PHY/MAC). This will eliminate the need to "fit" WAVE/TGp into the 802.11 specification without "breaking" the standard.

- Background, Explanation, Discussion, etc.:

This is an enhancement to extend the 802.11 MAC & PHY to low latency, high reliability communications. The PHY is nearly identical to 802.11j justifying 802.11p being an 802.11 amendment.

- Recommended Resolution of the Comment:

Reject the comment.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to xxxxxxxxxxxxxxxxxx     …  .

Motion by: ____________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |