IEEE P802.11
Wireless LANs

| Proposal for 20-40 Coexistence |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sanjiv Nanda | Qualcomm, Inc. |  | +1 858 845 2375 | snanda@qualcomm.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document offers a proposal for the consideration of the 20-40 Adhoc. It includes straw polls for the consideration of the group.

# Observations

We make the following observations:

- A limited amount of complexity increase to enable efficient 40 MHz behavior is acceptable. Requiring a second receiver and preamble detector for the extension channel is excessive. Moreover, even with the second receiver, the NAV cannot be updated when the STA is transmitting on the control channel.

- Detection of an OBSS on the extension channel should cause the AP to switch the BSS to 20 MHz on the control channel, or to find another 40 MHz channel. This behavior is based on statistics gathered by the AP from one or more STAs in the BSS.

- If a STA is capable of detecting interference on the extension channel while doing CCA on the control channel, then it should not make a 40 MHz transmission. However, this does not solve the problem of collisions on the extension channel. Similar to the case of 20 MHz operation, collisions on the (extension) channel can occur at the receiver, even when the transmitter is unable to see energy on the extension channel.

Requiring CCA on the extension channel is insufficient for avoiding collisions.

Based on this we conclude that, a BSS management mechanism (b) is appropriate to deal with extension channel interference, rather than a MAC mechanism (c) that only provides a partial solution. The management mechanism is enabled through the measurement and reporting of PHY level metrics that can be implemented with very little complexity.

The above observations and the conclusions drawn are subjective. Nevertheless, these conclusions lead us to the following proposed text (that we had previously proposed at the JP January meeting).

Following the proposed text are several straw polls that describe modifications to the proposed text. These may be used to determine where consensus may emerge.

# Proposed Text

### Extension Channel Interference Detection and Reporting

Any 40 MHz HT STA shall be able to detect activity in the extension channel without being able to receive frames on the extension channel. At least one of the following types of extension channel interference events shall be detected and reported to the AP in an infrastructure BSS:

- Reduced C/I on the extension channel, when receiving a 40 MHz transmission.

- Interference on the extension channel, when receiving a 20 MHz transmission on the control channel.

- Carrier sense through energy detection on the extension channel, when idle and performing CCA on the control channel.

On preamble detection on the extension channel, the STA may enable itself to receive in that extension channel in order to determine if a legacy BSS has arrived on the extension channel. It is not necessary that this capability be available simultaneous with operation on the control channel.
 
The AP can obtain statistics on extension channel interference events using the [802.11h] CCA Request and Response Action frames.
 
A 20 MHz HT STA may provide this capability.

# Straw Polls

Straw Polls:

- At least one of the following types of extension channel interference events shall be detected and reported to the AP in an infrastructure BSS.

  - Replace “at least one” with “all”

Yes:

No:

Abstain:

- Carrier sense through energy detection on the extension channel, when idle and performing CCA on the control channel.

  - Replace “Carrier sense through energy detection” with “CCA (preamble detection)”

Yes:

No:

Abstain:

- Add the following text: “While doing CCA on both control and extension channels when the STA is ready to begin transmission, if the extension channel is determined to be busy, the STA shall not make a 40 MHz transmission.

Yes:

No:

Abstain:

- Add the following text: “A 40 MHz STA is required to maintain NAV only for the control channel.”

Yes

No

Abstain