IEEE P802.11
Wireless LANs

| Implicit Beamforming Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Ketchum | Qualcomm Incorporated | 9 Damonmill Square Suite 2A Concord, MA 01451 | 781-276-0915 | johnk@qualcomm.com |
| Bjorn A. Bjerke | Qualcomm Incorporated | 9 Damonmill Square Suite 2A Concord, MA 01451 | 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments related to Draft 1.0 subclause 20.3.5.1 Implicit beamforming (=subclause 21.3.11.1 in Draft 1.03):

- 8025, 260, 3503, 7416, 11895, 11896, 11897, 12192, 10078, 3991

The changes marked in this document are based on TGn Draft version 1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Replace the text in subclause 21.3.11.1 (Implicit feedback beamforming) with the text given below. Also, insert the new Figure nx (The baseband-to-baseband channel) provided below, update cross references to figures and subclauses, and assign equation numbers to all equations that appear on separate lines.

21.3.11.1 Implicit feedback beamforming

The inherent reciprocity of the time division duplex channel used by 802.11 technology enables the use of efficient exchanges of MIMO reference waveforms to allow a device to acquire knowledge of the channel over which it plans to transmit by observing MIMO reference received from the device to which it plans to transmit.  With this knowledge, the transmitting device can calculate a set of transmit steering matrices, , one for each subcarrier, which are intended to optimize the performance of the link.  This is referred to as implicit beamforming.

Referring to Figure n62 (The beamforming MIMO channel model), (TGn Editor: update cross reference to figure) beamforming transmissions from a STA A to a STA B using implicit techniques are enabled when STA B sends STA A a sounding PPDU, allowing STA A to form an estimate of the MIMO channel from STA B to STA A, for all subcarriers.  Due to the reciprocity relationship between the forward and reverse channels, the channel from STA A to STA B in subcarrier  is the matrix transpose of the channel from STA B to STA A in subcarrier , to within a complex scaling factor, i.e., .  Here  is the MIMO channel matrix from STA A to STA B at subcarrier  and  is the channel matrix from STA B to STA A at subcarrier .  STA A uses this relationship to compute transmit steering matrices that are suitable for transmitting to STA B over .

While the over-the-air channel between the antenna(s) at one STA and the antenna(s) at a second STA is reciprocal, the observed baseband-to-baseband channel used for communication may not be, as it includes the transmit and receive chains of the STAs.  Differences in the amplitude and phase characteristics of the transmit and receive chains associated with individual antennas degrade the inherent reciprocity of the over-the-air channel, and cause degradation of performance of implicit beamforming techniques.  The over-the-air calibration procedure described in subclause 9.17.2.2 (Calibration) (TGn Editor: update cross reference to 9.17.2.2) may be used to restore reciprocity.  The procedure provides the means for calculating a set of correction matrices that can be applied at the transmit side of a STA to correct the amplitude and phase differences between the transmit and receive chains in the STA.  If this is done in both the STAs that are communicating with each other, reciprocity is restored in the baseband-to-baseband response of the forward and reverse channels.

Figure nx (The baseband-to-baseband channel) (TGn Editor: update cross reference to Figure nx) illustrates the observed baseband-to-baseband channel, including reciprocity correction.  The amplitude and phase responses of the transmit and receive chains can be expressed as diagonal matrices with complex valued diagonal entries, of the form .  The relationship between the baseband-to-baseband channel, , and the over-the-air channel is

, (TGn Editor: assign equation number)

and, similarly, the relationship between and  is

. (TGn Editor: assign equation number)

Figure nx - The baseband-to-baseband channel

As an example, consider the case where calibration is performed at both STA A and STA B. The objective is to compute correction matrices  and  that restore reciprocity such that

. (TGn Editor: assign equation number)

The correction matrices are diagonal matrices with complex valued diagonal entries.  The reciprocity condition above is enforced when

, (TGn Editor: assign equation number)

and

,   (TGn Editor: assign equation number)

where  and are complex valued scaling factors.

Using these expressions for the correction matrices, the calibrated baseband-to-baseband channel between STA A and STA B is expressed as

,  (TGn Editor: assign equation number)

and if both sides apply the correction matrices, then we have that

.  (TGn Editor: assign equation number)

Focusing on STA A, the procedure for estimating  is as follows:

- STA A sends STA B a sounding PPDU, allowing STA B to estimate the channel matrices .

- STA B sends STA A a sounding PPDU, allowing STA A to estimate the channel matrices .

- STA B sends the quantized estimates of  to STA A.

- STA A uses its local estimates of  and the quantized estimates  received from STA B to compute the correction matrices .

Steps a) and b) occur over a short time interval to ensure that the channel changes as little as possible between measurements. A similar procedure is used to estimate at STA B.  The details of the computation of the correction matrices is implementation specific, and beyond the scope of this document.

This is an example. Applying correction matrices on the receive side is also possible.