IEEE P802.11
Wireless LANs

| Comparison of TGn channel coding techniques (BCC & LDPC) over the TGn MIMO channel D in 2x2 antenna configurations (MCS 8-15) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Devis Gatti | STMicroelectronics, NV | Via C. Olivetti 2, 20041 Agrate Brianza (Milano) Italy | +39.039.603.7236 | Devis.Gatti@st.com |
| Stefano Valle | STMicroelectronics, NV | Via Tolomeo 1, 20010 Cornaredo (Milano) Italy | +39.029.351.9284 | Stefano.Valle@st.com |

Abstract

This document presents an extensive comparison of the performance achievable with BCC and LDPCC coding over the with IEEE 802.11n  MIMO channel D with IEEE 802.11n MCS 8 to 15 that corresponds to the modes supporting Nss=2 spatial streams in 20MHz Bandwidth.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Simulation Details

According to the LDPCC PPDU encoding procedures, codewords of three different lengths are adopted depending on the packet length. For each mode, three different packet sizes have been simulated in order to compare the three codes against BCC. Packet sizes have been selected among those that minimize the amount of shortening, puncturing and repetition. Packet length selection is reported in Table 1.

## Simulation Conditions:

- perfect synchronization and CSI

- no impairments

- IEEE 802.11n channel model: Channel D, 2x2, element spacing 0.5 ;

- MMSE detection;

- 50 LDPCC iterations of Belief Propagation.

- 100 erroneous packets per SNR

Table 1

| MCS | FEC | Packet Data LENGTH [bytes] |
| --- | --- | --- |
| 8 | BCC | 37 |
| 8 | LDPCC 648 | 37 |
| 8 | BCC | 76 |
| 8 | LDPCC 1296 | 76 |
| 8 | BCC | 970 |
| 8 | LDPCC 1944 | 970 |
| 9 | BCC | 37 |
| 9 | LDPCC 648 | 37 |
| 9 | BCC | 76 |
| 9 | LDPCC 1296 | 76 |
| 9 | BCC | 970 |
| 9 | LDPCC 1944 | 970 |
| 10 | BCC | 56 |
| 10 | LDPCC 648 | 56 |
| 10 | BCC | 115 |
| 10 | LDPCC 1296 | 115 |
| 10 | BCC | 909 |
| 10 | LDPCC 1944 | 909 |
| 11 | BCC | 24 |
| 11 | LDPCC 648 | 24 |
| 11 | BCC | 76 |
| 11 | LDPCC 1296 | 76 |
| 11 | BCC | 970 |
| 11 | LDPCC 1944 | 970 |
| 12 | BCC | 37 |
| 12 | LDPCC 648 | 37 |
| 12 | BCC | 76 |
| 12 | LDPCC 1296 | 76 |
| 12 | BCC | 909 |
| 12 | LDPCC 1944 | 909 |
| 13 | BCC | 50 |
| 13 | LDPCC 648 | 50 |
| 13 | BCC | 102 |
| 13 | LDPCC 1296 | 102 |
| 13 | BCC | 970 |
| 13 | LDPCC 1944 | 970 |
| 14 | BCC | 56 |
| 14 | LDPCC 648 | 56 |
| 14 | BCC | 115 |
| 14 | LDPCC 1296 | 115 |
| 14 | BCC | 909 |
| 14 | LDPCC 1944 | 909 |
| 15 | BCC | 63 |
| 15 | LDPCC 648 | 63 |
| 15 | BCC | 128 |
| 15 | LDPCC 1296 | 128 |
| 15 | BCC | 1010 |
| 15 | LDPCC 1944 | 1010 |

## Simulation Results