IEEE P802.11
Wireless LANs

| TBR Centralized Routing Extension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Wei-Peng Chen | Fujitsu Laboratories of America | 1240 E. Arques Ave. Sunnyvale, CA 00180 USA | +1-408-530 4622 | wei-peng.chen@us.fujitsu.com |
| Youiti Kado | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6900 | kado@nict.go.jp |
| Kyeong-Soo Kim | STMicroelectronics, Inc. | 1060 East Brokaw Road, Mail Station 212, San Jose, CA 95131, USA | +1-408-621-4913 | kyeongsoo.kim@st.com |
| Azman-Osman Lim | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6868 | aolim@nict.go.jp |
| Yong Liu | Samsung Information System America | 75 West Plumeria Drive, San Jose, CA 95134, USA | +1-408-544-5649 | yo.liu@samsung.com |
| Masanori Nozaki | Oki Electric Industry Co., Ltd. | 2-5-7 Honmachi, Chuo-ku, Osaka 541-0053, Japan | +81-6-6260-0700 | nozaki765@oki.com |
| Shah Rahman | Cisco Systems, Inc. | Cisco Way, San Jose, CA 95134-1706, USA | +1-408-525-1351 | sharahma@cisco.com |
| Oyunchimeg Shagdar | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1532 | oyunaa@atr.jp |
| Mahdad N. Shirazi | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1506 | shirazi@atr.jp |
| Suhua Tang | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1544 | shtang@atr.jp |
| Bing Zhang | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6820 | zhang@nict.go.jp |

Abstract

This proposal contains an overview of the centralized routing extension to the TBR routing protocol of 802.11s P802.11s/ D0.01 proposal.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7.3.2.41.2	Route Reply Element

Add the following columns to Figure s24: Route Reply Element

| 1 | 6 | 4 |  | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| Number of Adjacent MPs | Adjacent MP Address #1 | Adjacent MP Link Metric #1 | … | Adjacent MP Address #N | Adjacent MP Link Metric #N |

Add and modify the following rows to Table s7: Route Reply Element Fields

| Flag | Bit 0: HWMP Administration (0 = Disabled; 1 = Enabled) If the flag is set, the corresponding route reply message appends the list of adjacency information (i.e., after the last “Source Sequence Number” field). Bit 1 – 7: Reversed |
| --- | --- |
| Number of Adjacent MPs | The number of adjacency information of a MP. This field exists only if Bit 0 of “Flag” is set. |
| Adjacent MP Address | The MAC address of the adjacent MPs. This field exists only if Bit 0 of “Flag” is set. |
| Adjacent MP Link Metric | The airtime cost in section 11A.4.2.1. This field exists only if Bit 0 of “Flag” is set. |

Add the following two sections after Section 7.3.2.41.4:

##### 7.3.2.41.5	Route Set (RSET) Element

The Route Set (RSET) frame format uses the action frame body format and is sent by Root to notify the destination MP in order to choose the best-known path from the source MP to the destination MP. This frame is typically sent in a unicast manner. The intermediate MP re-unicasts this frame. The frame format is shown in Figure 1.

| Octets: 1 | 1 | 1 | 1 | 6 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Mode Flags | RSET ID | Root Address | Root Sequence Number | Lifetime |

| 1 | 6 |  | 6 |
| --- | --- | --- | --- |
| Number of MPs | MP Address #1 (Notifying MP) | … | MP Address #N (Route Source) |

Figure 1: Route Set (RSET) Element

The fields contained in the element are as shown in Table 1.

Table 1: Route Set (RSET) Element Fields

| Field | Value/Description |
| --- | --- |
| ID | T.B.D. |
| Length | Variable |
| Mode Flags | Bit 0 – 7: Reserved |
| RSET ID | A sequence number uniquely identifying the particular RSET when taken in conjunction with the Root’s MAC address |
| Root Address | The MAC address of Root that resolve the better route |
| Root Sequence Number | The current sequence number to be used in the route entry pointing towards Root |
| Lifetime | The remaining number of times the request may be forwarded |
| Number of MPs | The number of MPs of the route that being set |
| MP Address | The MAC address of the MPs for which a route is desired (MP #1 is the notifying MP address and MP #N is the source address) |

##### 7.3.2.41.6	Route Notification (RNTF) Element

The Route Notification (RNTF) frame format uses the action frame body format and is sent by a MP to notify the next hop MP in order to reserve a path between the source MP and destination MP. This frame is typically sent in a unicast manner. The intermediate MP re-unicasts this frame. The frame format is shown in Figure 2.

| Octets: 1 | 1 | 1 | 1 | 6 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Mode Flags | RNTF ID | Notifying MP Address | Notifying MP Sequence Number | Lifetime |

| 6 | 4 | 4 | 1 | 6 |  | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Root Address | Root Sequence Number | Cumulative Link Metric | Number of MPs | MP Address #1 (Notifying MP) | … | MP Address #N (Route Source) |

Figure 2: Route Notification (RNTF) Element

The fields contained in the element are as shown in Table 2

Table 2: Route Notification (RNTF) Element Fields

| Field |  | Value/Description |
| --- | --- | --- |
| ID |  | T.B.D. |
| Length |  | Variable |
| Flags |  | Bit 0 – 7: Reserved |
| RNTF ID | A sequence number uniquely identifying the particular RNTF when taken in conjunction with the notifying MP’s MAC address |  |
| Notifying MP Address |  | The MAC address of the notifying MP which originated the RNTF for which the route is supplied |
| Notifying MP Sequence Number |  | The current sequence number to be used in the route entry pointing towards the notifying MP |
| Lifetime |  | The time in milliseconds for which MPs receiving the RNTF consider the route to be valid |
| Root Address | The MAC address of Root that resolve the better route |  |
| Root Sequence Number | The current sequence number to be used in the route entry pointing towards Root |  |
| Cumulative Link Metric | This field is the cumulative link metric from the destination MP to the source MP |  |
| Number of MPs | The number of MPs of the route that being set |  |
| MP Address | The MAC address of the MPs for which a route is desired (MP #1 is the notifying MP address and MP #N is the source address) |  |

##### 7.3.2.49	Mesh Portal/Root Announcement Element

Add and modify the following row to Table s14: Mesh Portal/Root Advertisement Element Fields

| Mode Flags | Bit 0: Announcement type (0 = Portal; 1 = Root) Bit 1: HWMP Registration (0 = Disabled; 1 = Enabled) If the flag is set, registration of MPs with the root portal occurs, otherwise it does not. Bit 2: HWMP Administration (0 = Disabled; 1 = Enabled) If the flag is set, MPs upload the route reply message with the list of adjacency information. Bit 3 – 7: Reserved |
| --- | --- |

Add the following text after the line 45, page 68 as follows:

In HWMP, a tree topology is formed when the root sets the HWMP-Registration flag in the root announcement message. By additionally setting the HWMP-Administration flag in the same root announcement message, the root also can enable a centralized routing scheme providing higher efficiency and manageability based on the whole network topology. The centralized routing scheme can provide a wide range of benefits, such as emergency services, load balancing, multi-channel operations, and so on. If the HWMP-Administration flag is set in the announcement message, MPs upload the route reply message with the list of adjacency information. Upon receiving the route reply messages from MPs, the root can build a whole network topology in addition to the tree topology and also able to provide the best-metric route for each source-destination pair. When a source MP wants to send a message, the root can notify a destination MP and the corresponding intermediate MPs about the route information by using a route set message to the destination MP. Then, the destination MP notifies the intermediate MPs along the route with a route notification message. With these two messages, the root can notify any MPs by using the centralized scheme in corresponding to link-metrics including traffic, QoS, channel and so on.

Add the following text after the line 27, page 82 as follows:

Whenever an MP chooses a new parent in the tree, it checks the HWMP-Administration flag in the root announcement message. If this flag is set, it sends a gratuitous RREP with a list of adjacency interface information to the root. Each intermediate MPs hearing the gratuitous RREP adds the MP address contained in the message to its forwarding table. These gratuitous RREP messages are needed by the Root in order to compute the whole network topology. The MPs move to the "Forwarding" state only after completing topology administration with Root.