IEEE P802.11
Wireless LANs

| LB83 Normative Text Changes for including Information about AP’s Measurement Pilot transmission in Probe Response and Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Päivi Ruuska | Nokia |  | +358504835433 | paivi.m.ruuska@nokia.com |
| Jari Jokela | Nokia |  | +358504860445 | jari.jokela@nokia.com |

Abstract

This document contains normative text changes to TGk draft to include optional element about the AP’s Measurement Pilot transmissions in Probe Response. This document also contains normative text changes to include optional element about Neighbor AP’s Measurement Pilot transmission in Neigbor Report.

Changes are shown with respect to TGk draft D4.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Text Changes to include Measurement Pilot Transmission Information in the Probe Response

#### 7.2.3.9 Probe Response frame format

Insert new row 27 and change the last row as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 27 | Measurement Pilot Transmission Information | May be included if dot11RadioMeasurementEnabled is true. |
| 242728-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

### 7.3.2 Information Elements

Insert Element IDs 57 into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Measurement Pilot Transmission Information | 57 |
| Reserved | 515758-220 |

Insert the following new clause (7.3.2.41) after 7.3.2.40:

#### 7.3.2.41 Measurement Pilot Transmission Information element

The Measurement Pilot Transmission Information element contains a Measurement Pilot Transmission field as shown in Figure 112K.

|  | Element ID | Length | Measurement Pilot Transmission |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |  |

Figure 112K—Measurement Pilot Transmission Information element format

The length field shall be set to 2.

The Measurement Pilot Transmission Field contains the Measurement Pilot Interval as specified in 7.3.1.19, if the Measurement Pilot frames are transmitted. The field is set to all zeroes if the Measurement Pilot frames are not transmitted.

##### 10.3.2.2.2 Semantics of the Service Primitive

Insert the following row at the end of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Measurement Pilot Transmission | As defined in frame format | As defined in frame format | The value of Measurement Pilot Transmission from the Measurement Pilot Transmission Information element if such an element was present in the Probe Response. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |

# Annex A

### A.4.13 Radio Resource Measurement extensions

Insert RRM22 in the end of the table:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM22 | Measurement Pilot Transmission Information element included in Probe Response frame | 7.2.3.9, 7.3.2.41 | CFk:O | Yes, No, N/A |

# Text Changes to include Measurement Pilot Transmission Information in the Neigbor Report

#### 7.3.2.36 Neighbor Report element

Insert sub-element ID 2 into Table 43B and change the Reserved row accordingly:

Table 43A—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 2 | Measurement Pilot Transmission Information |
| 23-255 | Reserved |

Insert following Measurement Pilot Transmission Information sub-element description after the end of TSF Information description:

The Measurement Pilot Transmission Information sub-element contains the Measurement Pilot Transmission field.

|  | Measurement Pilot Transmission |
| --- | --- |
| Octets: | 2 |

Figure 112G—Measurement Pilot Information sub-element format

The value of Measurement Pilot Transmission Information sub-element length is 2.

The Measurement Pilot Transmission field contains the Measurement Pilot Interval as specified in 7.3.1.19, if the reported AP is transmitting Measurement Pilots frames. The field is set to zeroes if the reported AP is not transmitting the Measurement Pilots frames.

### 11.12.2 Receiving a Neighbor Report

Insert the following paragraph at the end of clause 11.12.2

A serving AP may include a Measurement Pilot Transmission field in the Neighbor Report element only if it is able to guarantee that the reported neighbor AP is not transmitting the Measurement Pilot frames, or is transmitting the Measurement Pilot frames with the Measurement Pilot Interval known by the serving AP.

# Annex A

### A.4.13 Radio Resource Measurement extensions

Insert RRM12.3:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM12 | Neighbor ReportProcedure | 11.12.2, 11.12.3 | CFk:M | Yes, No, N/A |
| RRM12.1 | Neighbor Report Procedure | 11.12.2, 11.12.3 | CFk:M | Yes, No, N/A |
| RRM12.2 | TBTT Offset in Neighbor Report | 7.4.5.5, 11.12.3 | CFk:O | Yes, No, N/A |
| RRM12.3 | Measurement Pilot Transmission Information in Neighbor Report | 7.3.2.36, 11.12.2 | CFk:O | Yes, No, N/A |