IEEE P802.11
Wireless LANs

| Interference Diagnostics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jari Jokela | Nokia | Visiokatu 1, 33720 Tampere, Finland | +358504860445 | jari.jokela @nokia.com |
| Paivi Ruuska | Nokia |  | +358504835433 | paivi.m.ruuska @nokia.com |
| Mika Kasslin | Nokia |  | +358504836294 | Mika.kasslin@nokia.com |
| Menzo Wentink | Conexant |  |  | menzo.wentink@conexant.com |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |
| Marian Rudolph | InterDigital Communications | Montreal, QC | 514-9046258 | Marian.Rudolf@InterDigital.com |
| Allan Thomson | Cisco |  |  |  |
| Jason Trachewsky | Broadcom Corporation |  |  |  |

Abstract

This proposal includes mechanism to enable interference reporting from the peer STAs.

This proposal is compatible with the D0.05 draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert the following new definition:

3.v.3 Co-located interference:  Co-located interference is caused by other radio system or other system emitting undesired radio energy located in the same physical device as the 802.11 system, and therefore may be known a priori by the 802.11 system.  Co-located interference is expected to cause degradation of 802.11 system performance either periodically or continuously.

Change 7.3.2.35 as follows

      - Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v15.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

- —Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field. The minimum value of the length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure v16.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B27 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | FBMS | Proxy ARP Service | Co-located Interference Reporting | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 9 |  |

- —Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in 11.5.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in 11.5.3. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.1.1. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in clause 11.5.4. The Presence bit set to 0 indicates that the STA does not support this service.

- The FBMS bit set to 1 indicates the STA supports FBMS as described in section 11.2.1.5. The FBMS bit set to 0 indicates the STA does not support FBMS.

- The Proxy ARP Service bit set to 1 indicates the AP is providing proxy ARP service.  If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. The Proxy ARP Service bit set to 0 indicates the AP is not providing proxy ARP service for any associated client.

- The Co-located Interference Reporting bit set to 1 indicates the STA supports Co-located Interference Reporting as described in 11.15.8. The Co-located Interference Reporting bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of a Wireless Network Management Capability element shall be interpreted as the STA having no advertised Wireless Network Management Capabilities.

Change 7.4.6 as follows

Modify Table v44 to be as follows:

- Table v44—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 0 | Event Log Request |
| 1 | Event Log Report |
| 2 | Diagnostic Request |
| 3 | Diagnostic Report |
| 4 | Presence Request |
| 5 | Presence Response |
| 6 | Presence Configuration Request |
| 7 | Presence Configuration Response |
| 8 | Roaming Management Query |
| 9 | Roaming Management Request |
| 10 | Roaming Management Response |
| 11 | FBMS Request |
| 12 | FBMS Response |
| 13 | Co-located Interference Request |
| 14 | Co-located Interference Response |
| 15-255 | Reserved |

Insert the following after 7.4.6.13

### 7.4.6.14 Co-located Interference Request frame format

The Co-located Interference Request frame uses the Action frame body format and is transmitted by a STA to request Co-located Interference Responses. The format of the Co-located Interference Request frame body is shown in Figure vn.

|  | Category | Action | Dialog Token | Request Info |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

- Figure vn — Co-located Interference Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Co-located Interference Request, as specified in Table v44 in 7.4.6.

The Dialog Token field is set to any nonzero value to identify the request/response transaction..

The Request Info field is shown in Figure vn+1

|  | Automatic Response Enabled | Report Timeout |
| --- | --- | --- |
| Bits: | 1 | 7 |

- Figure vn+1 — Request Info field format

The Automatic Response Enabled field is set to 1 if the requesting STA wants to enable the receiving STA to send the Co-located Interference Response frames automatically with reporting period interval (see 7.4.6.15) or when the STA realizes change in the co-located interference. Change event could occur when the co-located interference is realized, when the level of co-located interference significantly changes, periodicity of co-located interference changes, or when co-located interference is disappeared. The requesting STA can disable automatic reporting by sending Co-located Interference Request with Automatic Response Enabled field set to 0.

The Report Timeout field contains a value in units of 10TU during which a reporting STA shall not generate further Co-located Interference Responses after the previous Co-located Interference Response.

### 7.4.6.15 Co-located Interference Response

The Co-located Interference Response frame uses the Action frame body format and is transmitted by a response to Co-located Interference Request. The format of the Co-located Interference Response frame body is shown in Figure vn+2.

|  | Category | Action | Dialog Token | Response Info |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

- Figure vn+2 — Co-located Interference Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Co-located Interference Response, as specified in Table v44 in 7.4.6.

The Dialog Token field is set to the nonzero value received in the Co-located Interference Request frame to indetify the request/response transaction..

The Response Info field is shown in Figure vn+3. Each Co-located Interference Response may include up to 16 separate Response Info fields.

|  | Report Period | Interference Level | Interference Level Accuracy/Interference Index | Interference Interval | Interference Burst Length | Interference Start Time | Interference Center Frequency | Interference Bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 2 | 4 | 2 | 2 |

- Figure vn+3 — Response Info field format

The Report Period field indicates how often the STA will automatically report the co-located interference. The field is in units of 100 TUs. See 11.15.8 for further details.

The Interference Level field is a signed integer indicating the level of the expected self-interference in units of dBm averaged over a 4 s period during a interference period.. If no co-located interference is present the field is set to 0. The interference level is referenced to the connector of the currently in-use receiving antenna.

Bits 0-3 of Interference Level Accuracy/Interference Index field is an unsigned integer indicating the expected accuracy of the estimate of interference in dB (95% confidence interval). If accuracy is unknown then bits 0-3 shall be set to ‘1’. Bits 4-7 of Interference Level Accuracy/Interference Index indicates the Interference Index that shall be unique for each type of interference source. See 11.15.8 for further details.

The Interference Interval field indicates the interval between two successive periods of interference in microseconds. In case the interval between two successive periods of interference is variable the field shall be set to 65535. In case the interference is non-periodic the field shall be set to zeroes.

The Interference Burst Length field indicates the duration of each period of interference in microseconds. In case the duration of each period of interference is variable the field shall be set to 65535. In case the interference is non-periodic the field shall be set to zeroes.

The Interference Start Time field contains the least significant 4 bytes of the TSF timer at the start of the next interference burst. In case either the Interference Interval or the Interference Burst Length fields are set to 65535, this field shall indicate the average duty cycle. The average duty cycle value is defined as Integer ((232-1)[average interference burst length (microsecond)]/[average interference interval (microsecond)]).  In case the interference is non-periodic the Interference Start Time field shall be set to zeroes.

The Interference Center Frequency field indicates the approximated center frequency of interference in MHz.  If center frequency is unknown or interference covers the whole channel, the center frequency of the channel is reported.

The Interference Bandwidth field indicates the approximate 3-dB bandwidth of the interference signal in kHz.  If bandwidth of the interference signal is unknown, the field shall be set to 65535.

Insert the following after 10.3.40

    - Co-located Interference request

This set of primitives supports the exchange of co-located interference information between peer SMEs.

      - MLME-CLINTERFERENCEREQUEST.request

        - Function

This primitive requests the transmission of co-located interference request action frame to a peer entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-CLINTERFERENCEREQUEST.request	(
Peer MAC Address,
Dialog Token,
Request Info
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the group MAC address | The address of the peer MAC entity to which the co-loated interference request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the co-located interference transaction. |
| Request Info | Integer | As defined in the frame format | Specifies the requested information |

        - When generated

This primitive is generated by the SME to request that a Co-located Interference Request frame be sent to a peer entity.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Co-located Interference Request frame. This frame is then scheduled for transmission.

      - MLME-CLINTERFERENCEREQUEST.confirm

        - Function

This primitive reports the result of a co-located interference request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-CLINTERFERENCEREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the co-located interference transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Co-located Interference Request frame. |

        - When Generated

This primitive is generated by the MLME when the request to transmit a Co-located Interference Request frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-CLINTERFERENCEREQUEST.indication

        - Function

This primitive indicates that a Co-located Interference Request frame has been received requesting a Co-located Interference Response.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-CLINTERFERENCEREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Request Info
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the group MAC address | The address of the peer MAC entity from which the Co-located Interference request is sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the co-located interference transaction. |
| Request Info | Integer | As defined in the frame format | Specifies the requested information |

        - When generated

This primitive is generated by the MLME when a valid Co-located Interference Request frame is received.

        - Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the Co-located Interference reporting procedure as described in  11.15.8.

    - Co-located Interference Response

This set of primitives supports the exchange of co-located interference information between peer SMEs.

      - MLME-CLINTERFERENCERESPONSE.request

        - Function

This primitive requests the transmission of Co-located Interference Report to a peer entity, in response to a received Co-Located Interference Request frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- CLINTERFERENCERESPONSE.request	(
Peer MAC Address,
Dialog Token,
Response Info
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Co-located Interference Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the co-located interference transaction. |
| Response Info | Integer | As defined in the frame format | Specifies the co-located interference characterictics. |

        - When generated

This primitive is generated by the SME to request that a Co-located Interference Response frame be sent to a peer entity to convey co-located interference information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Co-located Interference Response frame. This frame is then scheduled for transmission.

      - MLME-CLINTERFERENCERESPONSE.confirm

        - Function

This primitive reports the result of a request to send a Co-located Interference Response action frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- CLINTERFERENCERESPONSE.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the co-located interference transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Co-located Interference Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit a Co-located Interference Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME- CLINTERFERENCERESPONSE.indication

        - Function

This primitive indicates that a Co-located Interference Response frame has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- CLINTERFERENCERESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Response Info
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the presence response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the presence transaction. |
| Response Info | Integer | As defined in the frame format | Specifies the co-located interference characterictics. |

        - When generated

This primitive is generated by the MLME when a valid Co-located Interference Response frame is received.

Insert 11.15.8 as follows:

### 11.15.8 Co-located Interference Reporting

Co-located Interference reporting allows a requesting STA to receive information concerning the co-located interference being experienced by other STA on the serving channel. Such interference may be due to an interaction between radios where a reporting STA is co-located with another radio device. Co-located Interference information can be used by the requesting STA to manage communication to reporting STA such that the effect of the interference is limited.

A STA that supports Co-located Interference reporting shall set the Co-located Interference Reporting capability bit in the Wireless Network Management Capabilities element to 1.

A requesting STA may request a Co-located Interference Report from other TA by sending a Co-located Interference Request. The STA, if it accepts the request, shall return a Co-located Interference Report. Alternatively, a requesting STA may request that automatic co-located interference reporting is enabled at other  STAs that have indicated interference diagnostics capability. To enable Co-located Interference reporting, the STA shall send a Co-located Interference Request frame with Automatic Response bit set to 1.

A STA accepting a request for automatic co-located interference reporting shall send a Co-located Interference Report to the requesting STA if it realizes that co-located radio or other co-located interferer is causing performace degradation to its WLAN receiver. The reporting STA shall send Co-located Interference Reports with interval indicated by the Report Period field or if the the level of the interference caused by co-located interferer is significantly changing or if the time chraracteristics of the interference is significantly changing . The reporting STA shall not generate Interference Responses more frequently than indicated by the Report Timeout field in Interference Request. Only the latest report is valid.

The reportig STA shall use Interference Index in Co-located Interference Report to indicate different types of interference. For example if the reporting STA is realizing co-located interference originated from two interference sources the reporting STA shall report both type of interferences using separate Response Info fields having separate Interference Index. Both Response Info fields can be sent in the same Co-located Interference Response frame and both can have the same Report Period.

Co-located Interference reporting shall be terminated on receipt of a Co-located Interference Request frame with Automatic Response bit set to 0.

A reporting STA may report any co-located interference that it is able to determine and that is causing degradation to its performance. The methods used by a reporting STA to determine the periodicity, level of interference, accuracy of the reported interference level, interference center frequency and interference bandwidth are outside the scope of this standard.

Insert A.4.14 into Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

Insert new item RME5:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME10 | Co-located Interference Reporting | - 7.4.6.14 7.4.6.15 - 11.5.8 | CFv:M | Yes, No, N/A |

References: