IEEE P802.11
Wireless LANs

| RDIE Status Codes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-04-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| TBD |  |  |  |  |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

Submission 11-06-0650-00-000r-qos-resource-control-flows.ppt proposed several changes to the TGr reservation mechanism.  This submission proposes the text changes to implement (1) the 32-bit status code, instead of a 1-bit status, (2) clarify that all of the RDIEs are processed by the target AP, and (3) clarification of the handling of the status values.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7.3.2.45.2, Figure 113R, change “Control Options” to “Resource Descriptor Count”, and insert “Status Code” of two octets following the count.

7.3.2.45.2, Delete Figure 113S, and the paragraph that starts “The Control Options field…”

7.3.2.45.2, change the paragraph that starts “The Status bit is used…” to “The Status value is used…”, and move it to follow the paragraph that starts “The Resource Descriptor Count…”

8A.4.1, third paragraph, change as follows:

If the STA does not receive a response to the Authentication-Confirm[#1256], or if the status returned by the target AP in the Authentication-Ack is nonzero[#1403], then the STA shall abandon this transition attempt[#1232].

8A.4.1, fifth paragraph, change as follows:

On successful completion of the Authentication-Confirm/Authentication-Ack exchange, the [#1263]resources identified in the RIC response with status code zero shall be reserved until the Reassociation Deadline.

8A.4.2, third paragraph, change as follows:

If the STA does not receive a response to the FT Action Confirm (after a reasonable number of retries to recover from lost over-the-DS messages), or if the status returned by the target AP in the FT Action Acknowledgement is nonzero[#1403], then the STA shall abandon this transition attempt[#1232].

8A.4.2, fifth paragraph, change as follows:

On successful completion of the Action Confirm/Action Acknowledge exchange, the [#1263]resources identified in the RIC response with status code zero shall be reserved until the Reassociation Deadline.

8A.7.2, eleventh paragraph, change as follows:

When sent by an AP in response to a RIC Request, the RIC Response consists of an RRIE[#716, #717] followed by a list of one or more Resource Responses, incuding one response for each of the Resource Requests that were contained in the RIC Request. The basic format of a RIC Response is shown in Figure Error: Reference source not found.

8A.7.3.1, third paragraph, change as follows:

The STA shall consider the resources that it desires at the next AP. For QoS resources, each traffic stream shall be requested by a separate RDIE and associated TSPEC(s). The RDIE Identifier in the RDIE shall be an arbitrary value chosen by the STA that uniquely identifies the RDIE within the RIC, the [0791]Status[#1401, #1412] code shall be set to zero, and the Resource Count shall be set to the number of alternative Resource [#492]Descriptors[0560] that follow.

8A.7.3.1, fifth through eighth paragraphs, change as follows:

A reservation request is considered successful by a STA if the status code 0 is returned in each RDIE[0791].

If the frame containing the response to the reservation request contains a status code other than 0, the STA considers that the request has failed, and that no resources are being held at the target AP.  According to the procedures of 8A.4, the STA abandons the transition attempt to this target AP.

The response from the target AP contains a RIC Response, with the [#716]RDIEs in the response indicating[Ed] which resources were considered by the target AP and the setting of the Status[#1401, #1412] code indicating[Ed] which Resource [#492]Descriptors[0560] were reserved by the AP.

The RDIE Identifier in the RDIE enables the STA to match the response with the RDIE in the request. The "status[#1401, #1412]" code is interpreted as follows:

-- Status = 0[#1401, #1412] indicates that the resources have been [#1402]reserved. The RDIE may be followed by the Resource [#492]Descriptor[0560] that was accepted.

-- Status non-zero (one of the values from 7.3.1.9)[#1401, #1412] indicates that the resources could not be allocated. The RDIE may be followed by a suggested Resource [#492]Descriptor[0560] that could have been allocated.

8A.7.3.2, fifth paragraph, change as follows:

In response to a RIC Request, the AP shall construct a RIC Response and include it in the response message. [#716]The RIC Response shall comprise one RDIE for each RDIE in the RIC Request. The RDIEs shall be in the same order as in the request and the RDIE Identifier in each RDIE shall be the value of the RDIE Identifier in the corresponding RDIE in the request. The Status[#1401, #1412] code shall be set according to the result of the allocation request as follows:

-- Status = 0[#1401, #1412] indicates that the resources have been [#1413]reserved. The RDIE shall also be followed by the Resource [#492]Descriptor[0560] that was accepted.

-- Status non-zero[#1401, #1412] indicates that the resources could not be allocated. The Status code contains a value from 7.3.1.9 indicating the reason for the failure. In this case the AP may include a single Resource [#492]Descriptor[0560] following the RDIE indicating a suggested resource that could have been allocated. The Resource Count field shall be set to '0' or '1' depending whether the suggested Resource [#492]Descriptor[0560] is attached.

8A.7.3.2, sixth paragraph, delete. (“Absense of an RDIE in the response indicates that the RDIE and related Resource Descriptors were not considered by the AP.”)