IEEE P802.11
Wireless LANs

| TGr Pre-Authentication |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | RIM Corporation | 5090 Commerce Drive Mississauga, ON | +1-905-629-4746 ext 4999 | mmontemurro@rim.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document describes the text changes to support preauthentication for TGr associations. This resolves TGr D2.0 Issue 56.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background: TGr pre-authentication enables a subsequent TGr First Association to occur without incurring the time delay of a full 802.1X entity authentication. In the absence of a standardized Key Transfer protocol, this provides a faster BSS transition when many mobility domains are used, e.g. one mobility domain per multi-vendor-AP device.

Successful TGr Pre-authentication results in delivery of the MSK to the Authenticator of the target AP.

FT Initial association then proceeds as indicated in Figure 154J, with the exception that the 802.1X authentication exchange has already occurred.

The target AP advertises its BSSID value. When the target AP receives the pre-authentication frame containing an encapsulated 802.1X frame, the target AP must forward that frame to its authenticator.

The DA of pre-authentication frames sent by the STA is the BSSID of the target AP. The target AP may be able to support 802.11i and 802.11r associations simultaneously, and could use a single BSSID for both types of associations, or the AP could use multiple (virtual) BSSIDs, one for each.

If the STA is doing 11i pre-auth, then the BSSID is the MAC address of the Authenticator, even if .11r is also supported.

Change Clause 8.4.6.1 as indicated below.

8.4.6.1 Preauthentication and RSNA key management

A STA shall not use preauthentication except when pairwise keys are employed. Preauthentication shall not

be used unless the target AP advertises the preauthentication capability in the RSN information element.

When preauthentication is used, then

a) Authentication is independent of BSS Transition.

b) The STA’s Supplicant may authenticate with multiple APs or MDs at a time.

NOTE—Preauthentication can be useful as a performance enhancement, as reassociation will not include the protocol

overhead of a full reauthentication when it is used.

Preauthentication uses the IEEE 802.1X protocol and state machines with EtherType 88-C7, rather than the

EtherType 88-8E. Only IEEE 802.1X frame types EAP-Packet and EAPOL-Start are valid for

preauthentication.

NOTE—Some IEEE 802.1X Authenticators may not bridge IEEE 802.1X frames, as suggested in C.1.1 of IEEE

P802.1X™-2004. Preauthentication uses a distinct EtherType to enable such devices to bridge preauthentication

frames.

A STA’s Supplicant can initiate preauthentication when it has completed the 4-Way Handshake or  Initial Association with a Mobility Domain in the ESS and configured

the required temporal keys. To effect preauthentication, the STA’s Supplicant sends an IEEE 802.1X

EAPOL-Start message with the DA being the BSSID of a targeted AP and the RA being the BSSID of the

AP with which it is associated. When dot11FastBSSTransitionEnabled is set to false, the target AP shall use a BSSID equal to the MAC address of its Authenticator. When dot11FastBSSTransition Enabled is set to true, the target AP shall use the BSSID of the AP, and forward a received preauthentication frame to its R0KH.

As preauthentication frames do not use the IEEE 802.1X EAPOL EtherType field, the AP with which

the STA is currently associated need not apply any special handling. The AP and the MAC in the STA shall

handle these frames in the same way as other frames with arbitrary EtherType field values that require distribution

via the DS.

An AP’s Authenticator that receives an EAPOL-Start message via the DS may initiate IEEE 802.1X authentication

to the STA via the DS. The DS will forward this message to the AP with which the STA is

associated.

The result of preauthentication may be an MSK, if the IEEE 802.1X authentication completes successfully.

If preauthentication produces an MSK, then, when the Supplicant's STA associates with the preauthenticated

AP, the Supplicant can use the PMK with the 4-Way Handshake or the PMK-R0 with the  FT 4-Way Handshake.

When the AS completes the authentication, it, passes the

master session key (MSK), (a portion of

which is used to derive the PMK or PMK-R0)  to the Authenticator.

If the Supplicant and Authenticator lose synchronization with respect to the MSK, the 4-Way Handshake or FT 4-Way Handshake will fail. In such circumstances, the MIB variable

dot11RSNAStats-4WayHandshakeFailures shall be incremented.

A STA’s Supplicant may initiate preauthentication with an AP with preauthentication

enabled regardless of whether or not the targeted AP is within radio range.

Even if a STA has preauthenticated, it is still possible that it may have to undergo a full IEEE 802.1X

authentication, as the AP’s Authenticator may have purged its MSK due to, for example, unavailability

of resources, delay in the STA associating, etc.

In an RSN in which  dot11FastBSSTransitionEnabled is set to true, a STA may use pre-authentication to perform an FT Initial Association with an AP in another Mobility Domain. The STA stores the PMK-R0 and uses it to transition to an AP in the new Mobility Domain.  PMKID negotiation is not used as the STA holds only one PMK-R0 for the mobility domain. The STA shall use the FT Initial Association to transition to the target AP.

References: