IEEE P802.11
Wireless LANs

| LB84 Replacement text for 802.11n Draft 1.0 Clause 20.3.2 PLCP Frame Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A, Concord, MA 01742, USA | 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments related to Draft 1.0 subclause 20.3.2 PLCP Frame Format (=subclause 21.3.2 in Draft 1.03):

- 4000, 182, and 7382

The changes marked in this document are based on TGn Draft version 1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

TGn Editor: Modify subclause 21.3.2 (PLCP frame format) as shown below, changing the text and replacing Figure n50 (PLCP packet format) with new Figure n50 (PLCP packet format). Keep footnote 1 unchanged (not shown here).

21.3.2 PLCP frame format

Two formats are defined for the PLCP (PHY Layer Convergence Protocol): HT Mixed Mode and Greenfield.

These two formats are called HT formats. Figure n50 shows the non-HT format1 and the HT formats. In addition

to the HT formats, there is a non-HT duplicate format (specified in 21.3.10.11) that duplicates the 20

MHz non-HT packet in two 20 MHz halves of a 40 MHz channel.

Figure n50- PLCP packet format

The elements of the PCLP packet are:

- L-STF: Non-HT Short Training Field

- L-LTF: Non-HT Long Training Field

- L-SIG: Non-HT SIGNAL Field

- HT-SIG: HT SIGNAL Field

- HT-STF: HT Short Training Field

- HT-GF-STF: HT Greenfield Short Training Field

- HT-LTF1:  First HT Long Training Field (Data HT-LTF)

- HT-LTFs: Additional HTLong Training Fields (Data HT-LTFs and Extension HT-LTFs)

- Data – The data field includes the PSDU (PHY Service Data Unit)

The HT-SIG, HT-STF and HT-LTFs exist only in HT packets. In non-HT and non-HT duplicate formats only

the L-STF, L-LTF, L-SIG and Data fields exist.

In both HT Mixed Mode and HT Greenfield frame formats, there are two types of HT-LTFs: Data HT-LTFs (DLTFs) and Extension HT-LTFs (ELTFs).  DLTFs are always included in HT frames to provide the necessary reference for the receiver to form a channel estimate that allows it to demodulate the data portion of the frame.  The number of DLTFs, NDLTF, may be either 1, 2, or 4, and is determined by the number of space time streams being transmitted in the frame (see Table n74 (TGn Editor: update reference to Table n74 (Number of DLTFs required for data space time streams))).  ELTFs provide additional reference in sounding PPDUs so that the receiver can form an estimate of additional dimensions of the channel beyond those that are used by the data portion of the frame.  The number of ELTFs, NELTF, may be either 0, 1, 2, or 4 (see Table n75 (TGn Editor: update reference to Table n75 (Number of ELTFs required for extension spatial streams))).  PLCP preambles in which DLTFs are followed by ELTFs are referred to as staggered preambles. The HT Mixed Mode and HT Greenfield frame formats shown in Figure n50 both contain staggered preambles for illustrative purposes.