IEEE P802.11
Wireless LANs

| IGTK and DHV update examples |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Fabrice Stevens | France Telecom | 38-40 rue du general leclerc, 92794 Issy les Moulineaux, France | +33145298164 | Fabrice.stevens@orange-ft.com |

Abstract

Adds the IGTK and the DHV to the Group Key Update example in clause 8.5.4.

The changes are based on draft .11ma D7.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Delete the following sentence in clause 8.5.4:

"In this Clause, the description for GTK also applies to the IGTK and DHV."

Insert the following in clause 8.5.4:

"Change the text in clause 8.5.4 as follows:

Message 1: Authenticator → Supplicant: EAPOL-Key(1,1,1,0,G,0,Key RSC,0, MIC, GTK[NGTK], IGTK[NIGTK], DHV)])

Message 2: Supplicant → Authenticator: EAPOL-Key(1,1,0,0,G,0,0,MIC,0,0,0)

Here, the following assumptions apply:

— Key RSC denotes the last frame sequence number sent using the GTK.

— GTK[NGTK] denotes the GTK encapsulated with its key identifier as defined in 8.5.2 using the KEK defined in 8.5.1.2 and associated IV.

— IGTK[NIGTK] denotes the IGTK encapsulated with its key identifier as defined in 8.5.2 using the KEK defined in 8.5.1.2 and associated IV.

— DHV denotes the DHV encapsulated as defined in 8.5.2 using the KEK defined in 8.5.1.2 and associated IV."

Add clause 8.5.4.1 as follows:

"Change the following text in clause 8.5.4.1 from:

Key Data = encrypted, encapsulated GTK and the GTK’s key identifier (see 8.5.2)

To:

Key Data = encrypted, encapsulated

- GTK and the GTK’s key identifier (see 8.5.2)

- IGTK, IGTK's key identifier, and sequence number (see 8.5.2)

- DHV (see 8.5.2)

Change bullet c) in clause 8.5.4.1 as follows:

c) Uses the MLME-SETKEYS.request primitive to configure the temporal GTK, IGTK, and DHV into its IEEE 802.11 MAC."

Add clause 8.5.4.4 as follows:

"Change the second sentence in clause 8.5.4.4 as follows:

The state machines in 8.5.6 and 8.5.7 change the GTK, IGTK, and DHV in use by the network.

Change the content of the first message in Figure 154 as follows:

EAPOL-Key(1,1,1,0,G,0,Key RSC,0, MIC,GTK[KeyIDGTK], IGTK[KeyIDIGTK], DHV)

Change the action performed by the station upon reception of the first message in Figure 154 as follows:

Decrypt GTK and set in Key IDGTK

Decrypt IGTK and set in Key IDIGTK

Decrypt and set DHV

Change the content of the second message in Figure 154 as follows:

EAPOL-Key(1,1,0,0,G,0,0,0,MIC,0,0,0)

Change the action performed by the access point upon reception of the second message in Figure 154 as follows:

Set GTK in Key IDGTK

Set GTK in Key IDIGTK

Change the last paragraph of clause 8.5.4.4 as follows:

The following steps occur:

a) The Authenticator generates a new GTK, IGTK, and DGTK. It derives a DHV from that DGTK. It encapsulates the GTK, IGTK, and DHV and sends an EAPOL-Key frame containing the GTK, IGTK, and DHV (Message 1), along with the last sequence number used with the GTK (RSC) and the last sequence number used with the IGTK (PN).

b) On receiving the EAPOL-Key frame, the Supplicant validates the MIC, decapsulates the GTK, IGTK, and DHV , and uses the MLME-SETKEYS.request primitive to configure the GTK, IGTK, DHV, RSC, and PN in its STA.

c) The Supplicant then constructs and sends an EAPOL-Key frame in acknowledgment to the Authenticator.

d) On receiving the EAPOL-Key frame, the Authenticator validates the MIC. If the GTK, IGTK, and DHV are not already configured into IEEE 802.11 MAC, after the Authenticator has delivered the GTK, IGTK, and DHV to all associated STAs, it uses the MLME-SETKEYS.request primitive to configure the GTK, IGTK, and DHV into the IEEE 802.11 STA.

"

Update the state machine diagrams in sections 8.5.6 and 8.5.7 to add the IGTK and DHV alongside GTK