IEEE P802.11
Wireless LANs

| Annex I and J 3650 MHz Band |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | MS SJ-10-5 170 W. Tasman Dr., San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

Re REV-ma D7.0, proposed Annex I and J 3650 MHz band designations for TGy. The real draft will have Unique Number Before Publication (UNBP) abbreviations for inserted table elements. Based on CFR47, Section 90.1301 et seq.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex I

(informative)

# Regulatory classes

## I.1 External regulatory references

Change the documents column in Table I.1 as shown:

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| United States | Federal Communications Commission (FCC) | FCC CFR47 [B8], Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401– 15.407, Section 90.210, Section 90.1201-90.1217, Section 90.1301 et seq. | FCC |

Insert new row and change last row in Table I.2 as shown:

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 6 shared 3650 MHz band | FCC CFR47, Clause 90.1323 | Reserved | Reserved |
| 67-255 | Reserved | Reserved | Reserved |

Change fifth through tenth rows and footnotes of Table I.3 as shown:

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 3 transmit power controla | Reserved | ETSI EN 301 389-1 | Reserved |
| 4 dynamic frequency selectiona | Reserved | ETSI EN 301 389-1 | Reserved |
| 5 Independent basic service set (IBSS) prohibited | ReservedFCC CFR47, Clause 90.1333 | Reserved | MIC EO Article 49.21 |
| 6 4 ms CSab | Reserved | Reserved | MIC EO Articles 49.20, 49.21 |
| 7 licensed base stationc | Reserved FCC CFR47, Clause 90.1331 | Reserved | MIC EO Article 49.21 |
| 8 mobile stationc | ReservedFCC CFR47, Clause 90.1333 | Reserved | MIC EO Articles 49.20, 49.21 |

aTPC and DFS procedures may be used to improve sharing spectrum where there are no explicit regulatory requirements.

abThe Japanese 4 ms CS rule says no STA can transmit for more than 4 ms without carrier sensing, whether transmitting frames or fragments, unless it is controlled by another STA

cThe deployment in US 3650 MHz band excludes operation near grandfathered satellite earth stations, Federal government radiolocation facilities and Canadian and Mexican borders. If mutual agreement can be reached, the FCC will permit operation within the declared exclusion zones. Operation near Canadian and Mexican borders is subject to current and future agreements with Canada and Mexico.

Insert new second and third rows in Table I.4 as shown:

| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | Europe (EIRP) |
| --- | --- | --- |
| 3.650-3.700 fixed/base | 1W/MHz EIRP | Reserved |
| 3.650-3.700 mobile/portable | 40 mW/MHz EIRP | Reserved |

# Annex J

# (normative)

# Country information element and regulatory classes

Change the title, insert six new rows and change the last row of Table J.1 as shown:

Table J.1—Regulatory classes for the 4.9 GHz and 5 GHz bands in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | 3.000 | 20 | TBD | 1W/MHz EIRP | 6 | 3, 4, 5, 7 |
| 12 | 3.000 | 20 | TBD | 40mW/MHz EIRP | 6 | 3, 4, 5, 8 |
| 13 | 3.000 | 10 | TBD | 1W/MHz EIRP | 6 | 3, 4, 5, 7 |
| 13 | 3.000 | 10 | TBD | 40mW/MHz EIRP | 6 | 3, 4, 5, 8 |
| 14 | 3.0025 | 5 | TBD | 1W/MHz EIRP | 6 | 3, 4, 5, 7 |
| 14 | 3.0025 | 5 | TBD | 40mW/MHz EIRP | 6 | 3, 4, 5, 8 |
| 1115-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |