IEEE P802.11
Wireless LANs

| 3650 MHz Mobile Service Enablement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-10-5 San Jose, CA 95134-1706 | 408-527-0815 | petere@cisco.com |

Abstract

Re REV-ma D7.0, proposed draft language for description of the enablement procedure for mobile/portable operation in US 3650-3700 MHz band. The MSE LCI is based on P802.11k-D4.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 3. Definitions

Insert the following new definitions in alphabetical order and renumber accordingly:

3.1 base station: The term ''base station'' means a station, other than a mobile

station, used for radio communication with mobile stations.

3.2 mobile service enablement (MSE): Facilities mandated to satisfy requirements in some regulatory domains for mobile operation conditioned on the reception and decoding of an enabling signal transmitted by a licensed base station.

# 4. Abbreviations and acronyms

Insert the following new acronym in alphabetical order:

MSE		mobile service enablement

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the first paragraph, and insert new order in table 8 as shown:

The frame body of a management frame of subtype Beacon contains the information shown in Table 8. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall include a Country information element in the transmission of Beacon frames. Optionally, the Beacon frame format may also include the information described in either or both of FH Parameters and FH Pattern Table elements. If both FH Parameters and FH Pattern Table elements are sent, they shall describe the same hopping pattern. Note that the information described in FH Parameters and FH Pattern Table elements also may be contained in the Probe Response frame.

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | MSE callsign | The MSE callsign element shall be present if dot11SpectrumManagementRequired is true and dot11MSERequired is true. The MSE callsign element may be present if dot11SpectrumManagementRequired is true and dot11MSERequired is false. |

#### 7.2.3.8 Probe Request frame format

Insert new row 6 as follows:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | MSE Parameter Set | The MSE Parameter Set information element shall be present within Probe Request frames generated by STAs using Clause 17 PHYs PHYs with dot11MSERequired set to true. The MSE Parameter Set information element may be present within Probe Request frames generated by STAs with dot11MSERequired set to false. |

#### 7.2.3.9 Probe Response frame format

Change the first paragraph as follows:

The frame body of a management frame of subtype Probe Response contains the information shown in Table 12. If the dot11MultiDomainCapabilityEnabled attribute is true, the Probe Response frame contains a Country information element and all information elements identified by the Requested Element IDs of a Request information element. Note that the information returned as a result of a Probe Request frame with a Request information element may include the FH parameters and/or the FH Pattern Table possibly replicating optional elements identified by orders 12 and 13. When a Probe Response frame is returned in response to a Probe Request frame which contained a Request information element, any of the requested elements that are listed in table 12 with a specific ordering shall be included in the Probe Response both in their listed ordered location as specified in table 12 and in the ordered location reserved for the list of requested elements, where the requested elements appear in increasing numerical element ID order.

Insert the order 23 information field and change the starting order of the Requested information elements field in Table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | MSE callsign | Shall be included if dot11SpectrumManagementRequired is true and dot11MSERequired is true. |

## 7.3 Management frame body components

### 7.3.1 Fields that are not information elements

Insert the following new clause:

#### 7.3.1.18 MSE callsign field

An MSE callsign field includes an MSE Location Configuration Information report, which contains Latitude, Longitude, Altitude and Azimuth information. The MSE LCI report format is shown in Figure 53A.

Figure 53A—MSE Location Configuration Information

This structure and information fields are little-endian, per conventions defined in 7.1.1, and are based on the LCI format described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”.

The definition of elements within the MSE LCI report shall be as defined in IETF RFC3825 section 2.1 or as defined herein.

The Radio Azimuth Report field contains five subfields as shown in Figure .

|  | B0 | B1 |  |  | B2 | B3 | B6 | B7 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Agreement |  |  | MSE | Azimuth Type | Azimuth Resolution |  | Azimuth |  |
| Bits: | 1 |  | 1 |  | 1 | 4 |  | 9 |  |

Figure 53B—Radio Azimuth Report subfield

Azimuth Agreement is set to TRUE to report that the STA is operating within a national policy area or an international agreement area near a national border, otherwise is set to FALSE.

MSE is set to TRUE to report that the base station is enabling the operation of mobiles with MSE, otherwise is set to FALSE.

Azimuth Type is set to TRUE to report the Azimuth of radio transmission.

Azimuth Resolution is 4 bits, indicating the number of valid bits in the Azimuth.

Azimuth is a 9-bit unsigned integer value in degrees from True North. Omni-directional transmission is reported with a value of 511 decimal.

11. MAC sublayer management entity

In sert the following new clause after clause 11.10:

## 11.11 MSE procedure

Regulations that apply to the US 3650 MHz band require base stations to implement a mechanism to enable mobile station operation. Similar regulations exist in other regulatory domains. This standard describes such a mechanism, referred to as mobile service enablement (MSE).

This subclause describes MSE procedures that can be used to satisfy these and similar future regulatory requirements. The procedures may also satisfy needs in other frequency bands and be useful for other purposes.

STAs shall use the MSE procedures defined in this subclause if dot11MSERequired is true.  dot11MSERequired shall be set to TRUE when regulatory authorities require MSE. It may also be set to TRUE in other circumstances. The MSE procedures provide for the following:

--Creation of a Basic Service Area for mobile operation

--Mobile operation with MSE

For the purpose of MSE, the following statements apply:

- A STA with dot11MSERequired set to TRUE shall not operate in a BSS or IBSS unless it has received a beacon frame or probe response frame from a base station with Spectrum Management bit set to 1 in the Capability Information field, and with Radio Azimuth MSE set to 1 in the MSE callsign field within the most recent [60] seconds.

11.11.1 Creation of a BSA for mobile operation

A base station may allow the creation of a Basic Service Area for mobile operation where regulatory requirements permit, and signifies that by setting the Radio Azimuth MSE to 1 in the MSE callsign field. It is not required that mobile operation proceeds from all STAs using the same enabling signal, rather that each STA shall receive and decode an enabling signal from a base station. A base station may remove a basis for the operation of mobile stations and their Basic Service Area by setting the Radio Azimuth MSE to 0 in the MSE callsign field.

11.11.2 Mobile operation with MSE

An AP in an MSE BSS may schedule quiet intervals using DFS procedures, so that all STAs can receive and decode an enabling signal from a base station on a timely basis.

Similarly, only the STA starting an IBSS may specify a schedule of quiet intervals using DFS procedures.

# Annex I

Insert new subclause at end of Annex I:

## I.3 3650 MHz and MSE operation

## References:

802.11-06/855r1 Annex I and J 3650 MHz.doc

P802.11k-D4.0 Amendment 9: Radio Resource Measurement

FCC     http://hraunfoss.fcc.gov/edocs_public/attachmatch/FCC-05-56A1.pdf