IEEE P802.11
Wireless LANs

| 3650 MHz Mobile Service Enablement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-10-5 San Jose, CA 95134-1706 | 408-527-0815 | petere@cisco.com |

Abstract

Re REV-ma D7.0, proposed draft language for description of the enablement procedure for dependent STA operation in US 3650-3700 MHz band. The LCI is based on P802.11k-D4.0

Where there is dispute, the material contained in single square brackets, [ ], represents a majority view, while material contained in double square brackets, [[ ]], represents a minority view.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 3. Definitions

Insert the following new definitions in alphabetical order and renumber accordingly:

3.1 base station: The term ''base station'' means a publically registered station that is physically indentifiable/locatable by the information it transmits in a beacon, and that may enable dependent stations.

3.2 dependent station: Any station whose operation is conditioned on the reception and decoding of an enabling signal transmitted by a registered base station.

3.1 fixed station: The term ''fixed station'' means a publically registered station that is physically indentifiable/locatable by the information it transmits in a beacon.

3.4 unregistered dependent enablement (UDE): Procedures to satisfy requirements in some regulatory domains that makes operation conditional on the reception and decoding of an enabling signal transmitted by a registered base station.

# 4. Abbreviations and acronyms

Insert the following new acronym in alphabetical order:

UDE		unregistered dependent enablement

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert new order 25 before ‘Last’ in table 8 as shown:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | UDE registered location | The UDE request information element shall be present if dot11UDERequired is set to true or dot11RegLocRequired is set to true. |

#### 7.2.3.8 Probe Request frame format

Insert new row 5 before ‘Last’ as shown:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | UDE request | The UDE request information element may be present within Probe Request frames generated by STAs with dot11UDERequired set to true or with dot11RegLocRequired set to true. |

#### 7.2.3.9 Probe Response frame format

Insert new order 23 information field after order 22 in Table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | UDE registered location | Is present only if dot11RegLocRequired is true or dot11UDERequired is true. |

## 7.3 Management frame body components

### 7.3.2 Information elements

Insert a new element after element 50 in Table 26, and change the following element as shown:

Table 26—Element IDs

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| UDE registered location (see 7.3.2.36) | 51 | 16 |

Insert new clause before 7.4 and renumber accordingly:

#### 7.3.2.36 UDE registered location field

An UDE registered location field includes an UDE Location Configuration Information report, which contains Latitude, Longitude and Altitude information. The UDE LCI report format is shown in Figure 53A.

Figure 53A—UDE Location Configuration Information

This structure and information fields are little-endian, per conventions defined in 7.1.1, and are based on the LCI format described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”.

The definition of elements within the UDE LCI report shall be as defined in IETF RFC 3825 section 2.1 or as defined herein.

Altitude Type (AT) codes are:

1: Meters - in 2s-complement fixed-point 22-bit integer part with 8-

bit fraction

If AT = 1, an AltRes value 0.0 would indicate unknown altitude.  The

most precise Altitude would have an AltRes value of 30.  Many values

of AltRes would obscure any variation due to vertical datum

differences.

2: Floors - in 2s-complement fixed-point 22-bit integer part with

8-bit fraction

AT = 2 for Floors enables representing altitude in a form more

relevant in buildings which have different floor-to-floor dimensions.

An altitude coded as AT = 2, AltRes = 30, and Altitude = 0.0 is

meaningful even outside a building, and represents ground level at

the given latitude and longitude.  Inside a building, 0.0 represents

the floor level associated with ground level at the main entrance.

This document defines a number; one must arrive at the number by

counting floors from the floor defined to be 0.0.

The values represented by this AT will be of local significance.

Since buildings and floors can vary due to lack of common control,

the values chosen to represent the characteristics of an individual

building will be derived and agreed upon by the operator of the

building and the intended users of the data.  Attempting to

standardize this type of function is beyond the scope this document.

Sub-floors can be represented by non-integer values.  Example: a

mezzanine between floor 1 and floor 2 could be represented as a value

= 1.1.  Example: (2) mezzanines between floor 4 and floor 5 could be

represented as values = 4.1 and 4.2 respectively.

Floors located below ground level could be represented by negative

values.

Larger values represent floors that are above (higher in altitude)

floors with lower values.

3: Height Above Ground, Meters - in 2s-complement fixed-point 22-bit integer part with 8-bit fraction

If AT = 3, an AltRes value 0.0 would indicate unknown altitude.  The

most precise Altitude would have an AltRes value of 30.

Datum is a three-bit field, rather than the eight-bit field defined in RFC 3825, and the codes used are as defined in RFC 3825.

RegLoc Agreement is set to TRUE to report that the STA is operating within a national policy area or an international agreement area near a national border, otherwise is set to FALSE.

RegLoc UDE is set to TRUE to report that the base station is enabling the operation of STAs with UDE, otherwise is set to FALSE.

Dependent STA is set to TRUE to report that the STA is operating with the enablement of the base station whose LCI is being reported, otherwise is set to FALSE.

The most-significant two bits of the UDE LCI are reserved.

# 10. Layer management

## 10.3 MLME SAP interface

### 10.3.2 Scan

#### 10.3.2.2 MLME-SCAN.confirm

#### 10.3.2.2.2 Semantics of the service primitive

Insert the following row at the end of the BSSDescription table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| UDE registered location | As defined in information element | As defined in frame format | The information from the UDE registered location information element, if such a field is present in probe response or beacon, else null |

### 10.3.10 Start

#### 10.3.10.1 MLME-START.request

#### 10.3.10.1.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-START.request(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet

VendorSpecificInfo,

UDE registered location

)

Insert the following entry to the end of the table in the subclause:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| UDE registered location | As defined in information element | As defined in frame format | The information from the UDE registered location information element. Present only if dot11RegLocRequired is true or dot11UDERequired is true. |

11. MAC sublayer management entity

Insert the following new clause after clause 11.10:

## 11.11 UDE procedures

Regulations that apply to the US 3650 MHz band require base stations to implement a mechanism to enable mobile station operation. Similar regulations exist in other regulatory domains. This standard describes such a mechanism, referred to as unregistered dependent enablement (UDE).

This subclause describes UDE procedures that can be used to satisfy these and similar future regulatory requirements. Regulations that apply to the US 3650 MHz band require fixed stations and base stations to have their operating locations registered. Licensees of stations suffering or causing harmful interference are expected to cooperate and resolve problems by mutually satisfactory arrangements. The procedures may also satisfy needs in other frequency bands and be useful for other purposes.

STAs shall use the UDE procedures defined in this subclause if dot11UDERequired is true.  dot11UDERequired shall be set to TRUE when regulatory authorities require UDE. It may also be set to TRUE in other circumstances. If licensees in more than one frequency band of a country use UDE procedures, then a dot11RegLocRequired and a dot11UDERequired shall be defined for each frequency band of that country.

The UDE procedures provide for the following:

--Fixed station and base station operation

--Creation of a Basic Service Area for dependent operation

--Dependent STA operation with UDE

For the purpose of UDE, the following statements apply:

- A STA with dot11UDERequired set to TRUE shall not operate in a BSS or IBSS unless it has received a beacon frame or probe response frame from a base station with Spectrum Management bit set to 1 in the Capability Information field, and with RegLoc UDE set to TRUE in the UDE registered location field within the most recent [60] seconds.

- A STA with dot11UDERequired set to FALSE shall not support the UDE registered location order unless dot11RegLocRequired is set to TRUE.

11.11.1 Fixed station and base station operation

Fixed stations and base stations shall have dot11RegLocRequired set to TRUE, and shall transmit UDE registered location in every beacon, and shall set the Dependent STA bit to FALSE. If the registered location of the fixed or base station is within a national policy area, such as within a Fixed Satellite Service exclusion zone, or the registered location is within an international agreement area near a national border, RegLoc Agreement shall be set to TRUE, otherwise it shall be set to FALSE.

The registered location Latitude, Longitude and Altitude fields shall be reported to their full resolutions, which may exceed the resolutions required by regulatory authorities. The Altitude Type shall be 3, Height Above Ground Meters (altitude in meters above adjacent terrain), unless another altitude type is required for operation in the regulatory domain. The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation in the regulatory domain.

11.11.2 Creation of a BSA for dependent operation

A base station may allow the creation of a Basic Service Area for dependent STA operation where regulatory requirements permit, and signifies that by setting the RegLoc UDE to TRUE in the UDE registered location field. It is not required that dependent STA operation proceeds from all STAs using the same enabling signal, rather that each dependent STA shall receive and decode an enabling signal from a base station. A base station may remove a basis for the operation of dependent STAs and their Basic Service Area by setting the RegLoc UDE to FALSE in the UDE registered location field.

11.11.3 Dependent STA operation with UDE

Dependent STAs shall have dot11RegLocRequired set to TRUE, and shall transmit the enabling base station UDE registered location in every beacon, and shall set the Dependent STA bit to TRUE. All dependent STAs shall have dot11UDERequired set to TRUE. An AP in an UDE BSS may schedule quiet intervals using DFS procedures, so that all STAs can receive and decode an enabling signal from a base station on a timely basis. Similarly, only the STA starting an IBSS may specify a schedule of quiet intervals using DFS procedures.

# Annex A

## A.4 PICS-IEEE Std 802.11, 2006 Edition

### A.4.3 IUT configuration

Append this entry to the end of this table

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFy | 3.65-3.70 GHz band in US | 7.3.2.36, 11.11, 17.3.6, Annex I, Annex J | CF6&CF8&CF10&CF11:O | Yes □ No □ N/A □ |

Insert this new clause after A.4.16

### A.4.17 UDE functions

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| MS1 | Fixed station operation with RegLoc | 11.11.1 | CFy:M | Yes □ No □ N/A □ |
| MS2 | Base station operation with RegLoc | 11.11.1 | CFy:M | Yes □ No □ N/A □ |
| MS3 | Base station creation of BSA | 11.11.2 | CFy:M | Yes □ No □ N/A □ |
| MS4 | Base station operation with UDE | 11.11.1 | CFy:M | Yes □ No □ N/A □ |
| MS5 | Dependent STA operation with UDE | 11.11.3 | CFy:M | Yes □ No □ N/A □ |

# Annex D

In the Major section of Annex D, insert the following comments at the end of dot11smt section, behind comment dot11RegulatoryClassesTable:

-- dot11LCIUDETable       ::= { dot11smt 14 }

-- dot11LCIUDEReportTable ::= { dot11smt 15 }

In the dot11StationConfig table of Annex D, insert two entries at the end of the dot11StationConfigEntry sequence list as follows:

dot11DLSAllowed         TruthValue,

dot11LCIUDEImplemented  TruthValue,

dot11LCIUDERequired     TruthValue }

Insert the following elements to the end of dot11StationConfigTable element definitions after

dot11DLSAllowed:

dot11LCIUDEImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation

is capable of supporting LCI UDE. The capability

is disabled otherwise. The default value of this attribute

is FALSE."

::= { dot11StationConfigEntry 43 }

dot11LCIUDERequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA will use the defined Unregistered Dependent

Enablement procedures if and only if this attribute

is TRUE. The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 44 }

In SMT MIB of Annex D, insert the following LCI UDE Table MIBs after the

Regulatory Classes MIB

-- ********************************************************************

-- * dot11LCIUDE TABLE

-- ********************************************************************

dot11LCIUDETable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIUDEEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for Unregistered

Dependent Enablement."

::= { dot11smt 14 }

dot11LCIUDEEntry OBJECT-TYPE

SYNTAX Dot11LCIUDEReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIUDETable

Indexed by dot11LCIUDEIndex."

INDEX { dot11LCIUDEIndex }

::= { dot11LCIUDETable 1 }

Dot11LCIUDEEntry ::=

SEQUENCE {

dot11LCIUDEIndex Unsigned32,

dot11LCIUDEIfIndex InterfaceIndex,

dot11LCIUDEFrequencyBandSupported INTEGER,

dot11LCIUDELatitudeResolution INTEGER,

dot11LCIUDELatitudeInteger INTEGER,

dot11LCIUDELatitudeFraction Unsigned32,

dot11LCIUDELongitudeResolution INTEGER,

dot11LCIUDELongitudeInteger INTEGER,

dot11LCIUDELongitudeFraction Unsigned32,

dot11LCIUDEAltitudeType INTEGER,

dot11LCIUDEAltitudeResolution INTEGER,

dot11LCIUDEAltitudeInteger Integer32,

dot11LCIUDEAltitudeFraction Unsigned32,

dot11LCIUDEDatum INTEGER,

dot11RegLocAgreement INTEGER,

dot11RegLocUDE INTEGER,

dot11DependentSTA INTEGER }

dot11LCIUDEIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI UDE Report elements in dot11LCIUDETable,

greater than 0."

::= { dot11LCIUDEEntry 1 }

dot11LCIUDEIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Each 802.11 interface is represented by an ifEntry. Interface

Tables in this MIB are indexed by ifIndex.”

::= { dot11LCIUDEEntry 2 }

dot11LCIUDEFrequencyBandSupported OBJECT-TYPE

SYNTAX INTEGER (0..7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Frequency Band Supported is 3 bits indicating the particular

OFDM frequency Band being supported."

::= { dot11LCIUDEEntry 3 }

dot11LCIUDELatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude."

::= { dot11LCIUDEEntry 4 }

dot11LCIUDELatitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

9 bits of integer portion of Latitude."

::= { dot11LCIUDEEntry 5 }

dot11LCIUDELatitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

25 bits of fraction portion of Latitude."

::= { dot11LCIUDEEntry 6 }

dot11LCIUDELongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude."

::= { dot11LCIUDEEntry 7 }

dot11LCIUDELongitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

9 bits of integer portion of Longitude."

::= { dot11LCIUDEEntry 8 }

dot11LCIUDELongitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

25 bits of fraction portion of Longitude."

::= { dot11LCIUDEEntry 9 }

dot11LCIUDEAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2),

hagm(3) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Altitude Type is four bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

floors : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

hagm : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction"

::= { dot11LCIUDEEntry 10 }

dot11LCIUDEAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid

bits in the altitude."

::= { dot11LCIUDEEntry 11 }

dot11LCIUDEAltitudeInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction. This field contains the fixed-point

Part of Altitude."

::= { dot11LCIUDEEntry 12 }

dot11LCIUDEAltitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction. This field contains the fraction part

of Altitude."

::= { dot11LCIUDEEntry 13 }

dot11LCIUDEDatum OBJECT-TYPE

SYNTAX INTEGER (0..7)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Datum is a three-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI."

::= { dot11LCIUDEEntry 14 }

dot11RegLocAgreement OBJECT-TYPE

SYNTAX INTEGER {

noagreement(0),

agreement(1),

}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"RegLocAgreement reports the RegLoc STA’s Agreement status.

Zero indicates it is operating away from national borders and outside

national policy zones. One indicates it is operating by agreement near

national boarders or inside national policy zones."

DEFVAL{ 0 }

::= { dot11LCIUDEEntry 15 }

dot11RegLocUDE OBJECT-TYPE

SYNTAX INTEGER {

dependentstasnotenabled(0),

dependentstasenabled(1),

}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"RegLocUDE reports the RegLoc STA’s UDE status.

Zero indicates Dependent STAs are not enabled.

One indicates Dependent STA operation is enabled."

DEFVAL{ 0 }

::= { dot11LCIUDEEntry 16 }

dot11DependentSTA OBJECT-TYPE

SYNTAX INTEGER {

notdependentsta(0),

dependentsta(1),

}

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reports the Dependent STA status of the STA that

sent the beacon or Probe Response with this information. Zero

indicates that STA is not operating as a Dependent STA. One

indicates that STA is operating as a Dependent STA."

DEFVAL{ 0 }

::= { dot11LCIUDEEntry 17 }

-- ********************************************************************

-- * End of dot11LCIUDE TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11LCIUDEReport TABLE

-- ********************************************************************

dot11LCIUDEReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIUDEReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of LCIUDE reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11smt 15 }

dot11LCIUDEReportEntry OBJECT-TYPE

SYNTAX Dot11LCIUDEReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIUDEReportTable

Indexed by dot11LCIUDEReportIndex."

INDEX { dot11LCIUDEReportIndex }

::= { dot11LCIUDEReportTable 1 }

Dot11LCIUDEReportEntry ::=

SEQUENCE {

dot11LCIUDEReportIndex Unsigned32,

dot11LCIUDERptIfIndex InterfaceIndex,

dot11LCIUDERptSTAAddress MacAddress,

dot11LCIUDERptFrequencyBandReported INTEGER,

dot11LCIUDERptLatitudeResolution INTEGER,

dot11LCIUDERptLatitudeInteger INTEGER,

dot11LCIUDERptLatitudeFraction Unsigned32,

dot11LCIUDERptLongitudeResolution INTEGER,

dot11LCIUDERptLongitudeInteger INTEGER,

dot11LCIUDERptLongitudeFraction Unsigned32,

dot11LCIUDERptAltitudeType INTEGER,

dot11LCIUDERptAltitudeResolution INTEGER,

dot11LCIUDERptAltitudeInteger Integer32,

dot11LCIUDERptAltitudeFraction Unsigned32,

dot11LCIUDERptDatum INTEGER,

dot11RptRegLocAgreement INTEGER,

dot11RptRegLocUDE INTEGER,

dot11RptDependentSTA INTEGER }

dot11LCIUDEReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI UDE Report elements in dot11LCIUDEReportTable,

greater than 0."

::= { dot11LCIUDEReportEntry 1 }

dot11LCIUDERptIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of LCI Report has been

received on"

::= { dot11LCIUDEReportEntry 2 }

dot11LCIUDERptSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this LCI report"

::= { dot11LCIUDEReportEntry 3 }

dot11LCIUDERptFrequencyBandReported OBJECT-TYPE

SYNTAX INTEGER (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Frequency Band Reported is 3 bits indicating the particular

OFDM frequency Band being reported."

::= { dot11LCIUDEReportEntry 4 }

dot11LCIUDERptLatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude."

::= { dot11LCIUDEReportEntry 5 }

dot11LCIUDERptLatitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

9 bits of integer portion of Latitude."

::= { dot11LCIUDEReportEntry 6 }

dot11LCIUDERptLatitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

25 bits of fraction portion of Latitude."

::= { dot11LCIUDEReportEntry 7 }

dot11LCIUDERptLongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude."

::= { dot11LCIUDEReportEntry 8 }

dot11LCIUDERptLongitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

9 bits of integer portion of Longitude."

::= { dot11LCIUDEReportEntry 9 }

dot11LCIUDERptLongitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

25 bits of fraction portion of Longitude."

::= { dot11LCIUDEReportEntry 10 }

dot11LCIUDERptAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2),

hagm(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude Type is four bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

floors : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

hagm : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction"

::= { dot11LCIUDEReportEntry 11 }

dot11LCIUDERptAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid

bits in the altitude."

::= { dot11LCIUDEReportEntry 12 }

dot11LCIUDERptAltitudeInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction. This field contains the fixed-point

Part of Altitude."

::= { dot11LCIUDEReportEntry 13 }

dot11LCIUDERptAltitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction. This field contains the fraction part

of Altitude."

::= { dot11LCIUDEReportEntry 14 }

dot11LCIUDERptDatum OBJECT-TYPE

SYNTAX INTEGER (0..7)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Datum is a three-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI."

::= { dot11LCIUDEReportEntry 15 }

dot11RptRegLocAgreement OBJECT-TYPE

SYNTAX INTEGER {

noagreement(0),

agreement(1),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"RptRegLocAgreement reports the RegLoc STA’s Agreement status.

Zero indicates it is operating away from national borders and outside

national policy zones. One indicates it is operating by agreement near

national boarders or inside national policy zones."

DEFVAL{ 0 }

::= { dot11LCIUDEReportEntry 16 }

dot11RptRegLocUDE OBJECT-TYPE

SYNTAX INTEGER {

dependentstasnotenabled(0),

dependentstasenabled(1),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"RptRegLocUDE reports the RegLoc STA’s UDE status.

Zero indicates Dependent STAs are not enabled.

One indicates Dependent STA operation is enabled."

DEFVAL{ 0 }

::= { dot11LCIUDEReportEntry 17 }

dot11RptDependentSTA OBJECT-TYPE

SYNTAX INTEGER {

notdependentsta(0),

dependentsta(1),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute reports the Dependent STA status of the STA that

sent the beacon or Probe Response with this information. Zero

indicates that STA is not operating as a Dependent STA. One

indicates that STA is operating as a Dependent STA."

DEFVAL{ 0 }

::= { dot11LCIUDEReportEntry 18 }

-- ********************************************************************

-- * End of dot11LCIUDEReport TABLE

-- ********************************************************************

In the dot11Compliance statements of Annex D, change MANDATORY-GROUPS as shown:

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities

that implement the IEEE 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase67,

dot11MACbase2, dot11CountersGroup2,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup }

In the dot11Compliance section, change dot11SMTbase6 as shown:

dot11SMTbase6 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed }

STATUS currentdeprecated

DESCRIPTION

"The SMTbase6 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 34 }

In the dot11Compliance statements of Annex D, insert new dot11SMTbase7 after the last dot11PhyOFDMComplanceGroup and renumber accordingly:

dot11SMTbase7 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedInQBSS,

dot11DLSAllowed,

dot11LCIUDEImplemented,

dot11LCIUDERequired }

STATUS current

DESCRIPTION

"The SMTbase7 object class provides the necessary support at the

STA to manage the processes in the STA such that the STA may work

cooperatively as a part of an IEEE 802.11 network."

::= { dot11Groups 37 }

## References:

802.11-06/855r3 Annex I and J 3650 MHz.doc

802.11-06/867r2 References for Mobile Service Enablement

802.11-06/955r1 OFDM PHY 3650 MHz Band.doc

P802.11k-D4.0 Amendment 9: Radio Resource Measurement

FCC     http://hraunfoss.fcc.gov/edocs_public/attachmatch/FCC-05-56A1.pdf

IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”