IEEE P802.11
Wireless LANs

| TGp LB81 Comment Resolution Form – 20.3.9.4, 20.3.9.5 and 20.3.9.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Robert Soranno | JHU/APL | 11100 Johns Hopkins Rd. Laurel, MD 20723-6099 | 443-778-8947 | Robert.Soranno@jhuapl.edu |
| Carl Kain | Mitretek |  |  | ckain@mitretek.org |
| Wayne Fisher | ARINC | 2551 Riva Rd. Annapolis, MD 2140 | 410-266-4958 | wfisher@arinc.com |

Abstract

This document addresses comments received against sub-clauses 20.3.9.4, 20.3.9.5 and 20.3.9.6  of 802.11p.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  |  |  |  | LB81 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 882 1178 1197 1212 | Aldana Kobayashi Moorti Trachewsky | 20.3.9.4 | Robert Soranno, Carl Kain Wayne Fisher |  | June 28, 2006 |
| 371 883 1179 1198 1213 | Ariyavisitakul Aldana Kobayashi Moorti Trachewsky | 20.3.9.5 | Robert Soranno, Carl Kain Wayne Fisher |  | June 28, 2006 |

- Comment:

a.	(882, 1178, 1197; 1212)

Why +/-10 ppm accuracy was chosen over +/-20 ppm accuracy as specified by 802.11a standard? An explanation could be that for the vehicular speeds tighter clock frequency tolerance is required. Was this simulated or analyzed?

Why +/-10 ppm accuracy was chosen over +/-20 ppm accuracy as specified by 802.11a standard?  This requirement increases device complexity, and the rationale for the tighter requirement isn't clear.

b.	(371, 883, 1179, 1198; 1213)

Why +/-10 ppm accuracy was chosen over +/-20 ppm accuracy as specified by 802.11a standard? An explanation could be that for the vehicular speeds tighter clock frequency tolerance is required. Was this simulated or analyzed?

Why +/-10 ppm accuracy was chosen over +/-20 ppm accuracy as specified by 802.11a standard?  This requirement increases device complexity, and the rationale for the tighter requirement isn't clear.

- Suggested Remedy (If appropriate):

a.		Change to +/-20 ppm accuracy (unless simulation results show

otherwise).

Change to +/-20 ppm accuracy.

b.		Change to +/-20 ppm accuracy (unless simulation results show

otherwise).

Change to +/-20 ppm accuracy.

- Background, Explanation, Discussion, Etc.

Results from initial testing using IEEE 802.11a devices as substitutes for 802.11p devices in a high speed environment indicated some problems could result in synching and maintaining signal lock which could be mitigated by requiring a more stringent frequency tolerance of ±10 ppm vs. the current  ±20 ppm requirement that is sufficient for non-mobile and mobile platforms not required to communicate over high speeds.

- Recommended Resolution of the Comment:

Sub-clauses 17.3.9.4 and 17.3.9.5 of 802.11-REVma/D7.0 address 20 MHz, 10 MHz and 5 MHz transmit channel center frequency tolerance and symbol clock frequency tolerance, respectively, for operations below the 5.85 – 5.925 GHz band.  For 5 MHz channels, ±10 ppm accuracy is required.  It is, therefore, recommended that the requirements of 20.3.9.4 and 20.3.9.5 be moved to these sub-clauses, and those sections be removed from 802.11p.

5.	Motions

Move to instruct the editor to provide the following modifications to 802.11p, Draft 1:

- Insert a Clause 17 into 802.11p entitled “Orthogonal frequency division multiplexing (OFDM) PHY specification for the 5 GHz band

- Within this clause insert sub-clause 17.3.9.4 entitled “Transmit center frequency tolerance” and 17.3.9.5 entitled “Symbol clock frequency tolerance”

- Change line 3 of 17.3.9.4, page 628 from, “…for 10 MHz channels, and shall be ± 10 ppm maximum for 5 MHz channels” to “for 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz channels and 10 MHz channels operating in the WAVE mode.  The new subclause should then appear as:

The transmitted center frequency tolerance shall be ±20 ppm maximum for 20 MHz channels and 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz channels and 10 MHz channels operating in the WAVE mode.  The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

- Change line 9 of 17.3.9.5, page 628 from, “…for 10 MHz channels, and shall be ± 10 ppm maximum for 5 MHz channels” to “for 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz 		channels and 10 MHz channels operating in the WAVE mode.  The new 	subclause should then read:

The symbol clock frequency tolerance shall be ±20 ppm maximum for 20 MHz channels and 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz channels and 10 MHz channels operating in the WAVE mode.  The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

- delete sub-clauses 20.3.9.4 and 20.3.9.5 from 802.11p

Instruct the editor to provide the following resolutions to the editorial comments sited above to 20.3.9.4 and 20.3.9.5:

a.	Comments:	Reject

Reason:	Improved stability determined necessary and is already

accommodated in latest version of standard

b.	Comments:	Reject

Reason:	Improved stability determined necessary and is already

Accommodated in latest version of standard

Motion by: ___Robert Soranno___

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

|  |  |  |  | LB81 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 1092 1571 1671 2076 | Ware Noens Buttar Shvodian | 20.3.9.6 | Robert Soranno |  | June 30, 2006 |

- Comment:

- (1092, 1571, 1671, 2076)

Reword first sentence to state "Transmit modulation accuracy specifications are described in this subclause."

- Suggested Remedy (If appropriate):

a.	see comment

- Background, Explanation, Discussion, Etc.

All of sub-clause 20.3.9.6 references associated sub-clauses in sub-clause 17.3.9.6 of 802.11-REVma/D7.  No modifications to the existing text is recommended.

- Recommended Resolution of the Comment:

Remove sub-clause 20.3.9.6

5.	Motions

Move to provide the following modifications to 802.11p, Draft 1:

Instruct the editor to remove sub-clause 20.3.9.6 from 802.11p, Draft 1

Instruct the editor to provide the following resolutions to the editorial comments sited above to 20.3.9.6:

a.	Comments:	Reject

Reason:	Sub-clause removed from document

Motion by: ___Robert Soranno___

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |