IEEE P802.11
Wireless LANs

| Explicit Feedback Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +97248655738 | solomon.trainin@intel.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvalce, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Tomoya Yamaura | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | yamauratomoya@yahoo.co.jp |
| Yuichi Morioka | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | Yuichi.Morioka@jp.sony.com |
| Assaf Kasher | Intel Corporation | POB 1659, Haifa 31015, Israel | +972-4-8651547 | assaf.kasher@intel.com |
| Gal Basson | Intel Corporation | POB 1659, Haifa 31015, Israel | +97248655405 | gal.basson@intel.com |
| Joonsuk Kim | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3455 | joonsuk@broadcom.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | +44 1223763457 | adrian.p.stephens@intel.com |

Abstract

This document contains 802.11n D1.0 submission of text changes related to the Explicit Transmit Beamforming. This document covers following comments:

640(C), 641(C), 642(C), 643(C), 1131(C), 1187(C), 1188(C), 1189(C), 1191(C), 1192(C), 1193(C), 1432(C), 1433(C), 1435(C), 1436(C), 1437(C), 1439(C), 1440(C), 1441(C), 1476(A), 1653(C), 2164(C), 2165(C), 2282(C), 2284(C), 2298(C), 2299(C), 2305(C), 2307(C), 2324(C), 2325(C), 2697(C), 2719(C), 2727(C), 3880(C), 3971(C, partially), 4201(C), 4238(C), 4526(C), 4786(C), 7281(C), 7370(C), 7671(A), 7822(C), 7838(A), 8070(C, partially), 8089(C), 10015(A) 10071(C, partially), 10072(C, partially), 10073(C), 11894(A), 11990(C), 12000(C, partially), 12002(C), 12003(C), 12004(C), 12008(R), 12009(R), 12010(R), 12011(C) partial, 12012(C), 12013(C), 12014(C), 12015(C), 12016(C), 12017(C), 12019(C), 12020(C), 12021(C), 12022(C), 12023(C)

Here, A, C, and R denote “accepted”, “counter”, and “rejected”, respectively. These comments are not reflected in the spreadsheet

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The following Comments have been resolved: 12016(C), 12017(C)

Modify the subclause 7.1.3.8 as follows:

Modify the figure n1 as follows:

| # | B0 B15 | B16 17 | B18 19 | B20 B21 | B22B23 | B24 | B25 B29 | B30 | B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Link Adaptation Control | Calibration Position | Calibration Sequence | Feedback request Reserved | CSI/ Steering | ZLF Announcement | Reserved | AC Constraint | RDG/More PPDU |
| # Bits | 16 | 2 | 2 | 2 | 2 | 1 | 5 | 1 | 1 |

Remove the following text and table n7:

The Feedback Request field contains position of the feedback: Immediate Feedback means SIFS after PPDU that contains Request; Aggregated means feedback aggregated with any other response in the same TxOP; Unsolicited Feedback means that the feedback may be sent in the TxOP obtained by the Responder.

- Table n7 – Feedback request

| Field | Meaning | Definition |
| --- | --- | --- |
| Feedback request | Position of the CSI feedback | 00 – No request 01 – Unsolicited feedback only 10 – Immediate feedback 11 – Aggregated feedback |

Modify the Table n8 as follows:

- Table n8 – CSI/Steering

| Field | Meaning | Definition |
| --- | --- | --- |
| CSI/Steering | Type of feedback | 00 – No feedback required 00 01 – CSI 01 10 – uncompressed Steering Matrix 10 11 – compressed Steering Matrix 11 – Reserved |

7.1.3.1.2 Type and Subtype fields

Insert the Control Frame Wrapper subtype of type Control

Modify the section of control frames in Table 1 as follows:

- Table 1—Valid type and subtype combinations

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 01 | Control | 0000- 0110 | Reserved |
| 01 | Control | 0111 | Control Frame Wrapper |
| 01 | Control | 1000 | Block Ack Request (BlockAckReq) |
| 01 | Control | 1001 | Block Ack (BlockAck) |
| 01 | Control | 1010 | PS-Poll |
| 01 | Control | 1011 | RTS |
| 01 | Control | 1100 | CTS |
| 01 | Control | 1101 | ACK |
| 01 | Control | 1110 | CF-End |
| 01 | Control | 1111 | CF-End + CF-Ack |

7.2.1.8.4 Control Frame Wrapper

| Octets: 2 | 2 | 6 | 2 | 4 |  | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration /ID | Address 1 | Carried frame control | HT Control | Carried frame | FCS |

- Figure n8a – Control frame Wrapper

The Control frame wrapper frame is used to carry any other control frame together with a High Throughput Control field.

The Frame control field is defined in 7.2.1. The value for the subtype subfield is the value from Table 1 of 7.1.3.1.2 that corresponds to Control Frame Wrapper.

The value for the Duration/ID field of the Control Frame Wrapper frame is generated by following the rules for the Duration/ID field of the control frame that is being carried

The value for the Address1 field of the Control Frame Wrapper frame is generated by following the rules for the Address1 field of the control frame that is being carried.

The Carried frame control field contains the value of the frame control field of the carried control frame.

The Carried frame field contains the fields that follow the Address1 field of the control frame that is being carried, excluding FCS.

The HT Control field is defined in 7.1.3.8.

The FCS field is defined in 7.1.3.7.

The following Comments have been resolved: 12019(C), 12020(C), 12021(C), 12022(C), 640(C), 641(C), 642(C), 643(C), 1187(C), 1188(C), 1189(C), 1191(C), 1192(C), 1193(C), 4201(C), 2164(C), 2165(C), 10073(C),

Change all occurrences of “unsolicited” where they occur within the draft with reference to explicit feedback operations, to “delayed”.

Change the Transmit beamforming capabilities in the 7.3.2.47.6 as in the follows:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TxBF capable Implicit TxBF receiving capable | Receive staggered sounding capable | Transmit staggered sounding capable | Receive ZLF capable | Transmit ZLF capable | Implicit TxBF capable | Calibration | Explicit CSI TxBF capable | Explicit uncompressed steering matrix capable | Explicit compressed steering matrix capable |

| B11 B12 | B13 B14 | B15 B16 | B17 B18 | B19 B20 | B21 B22 | B23 B24 | B25 B26 | B27 B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Explicit TxBF CSI feedback | Explicit uncompressed Steering Matrix feedback | Explicit compressed Steering Matrix feedback | Minimal grouping | CSI number of beamformer antennae support | Uncompressed steering matrix of beamformer antennae support | Compressed steering matrix of beamformer antennae support | CSI max number of rows beamformer support | Reserved |

Change the figure n19 as follows

| TxBF Capability field | Definition | Encoding |
| --- | --- | --- |
| Tx Beamforming capable Implicit TxBF receiving capable | Indicates whether or not this STA can receive TxBF steered frames Indicates whether or not this STA can receive steering frame using implicit feedback | Set to 0 if not supported Set to 1 if supported |
| Receive staggered sounding capable | Indicates whether or not this STA can receive staggered sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Transmit staggered sounding capable | Indicates whether or not this STA can transmit staggered sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Receive ZLF capable | Indicates whether or not this receiver can interpret zero-length frames as sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Transmit ZLF capable | Indicates whether or not this STA can transmit zero-length frames as sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Implicit TxBF capable | Indicates whether or not this STA can apply Implicit transmit beamforming. | Set to 0 if not supported Set to 1 if supported |
| Calibration | Indicates that the STA can participate in a calibration procedure initiated by another STA that is capable of generating an immediate response Sounding PPDU, and can provide a MIMO Channel Measurement Report in response to the receipt of a Sounding PPDU. Also indicates the capability of applying reciprocity correction vector received from initiator of calibration sequence When the ‘Implicit TxBF capable’ field is set to 1, this field shall be set to 11. | Set to 00 if not supported Set to 01 if the STA has limited capability of being involved in calibration, but cannot apply reciprocity correction vector received from initiator, and cannot initiate calibration Set to if 10 if the STA has limited capability of being involved in calibration, cannot apply reciprocity correction vector received from initiator, but can initiate calibration Set to 11 if fully supported |
| Explicit CSI TxBF capable | Indicates whether or not this STA can apply transmit beamforming using CSI explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit non-uncompressed Steering Matrix capable | Indicates whether or not this STA can apply transmit beamforming using non-compressed steering matrix explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit compressed steering matrix capable | Indicates whether or not this STA can apply transmit beamforming using compressed steering matrix explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit BF CSI feedback | Indicates whether or not this receiver can return CSI explicit feedback. Any combination of those 3 bits is allowed | Set to 000 if not supported The 3 bits separately indicate support for 3 feedback mechanisms. Any combination of these bits is valid. 001 indicates unsolicited delayed feedback 010 indicates immediate feedback 100 indicates aggregated feedback 11 indicates delayed and immediate feedback |
| Explicit non-compressed Steering Matrix feedback | Indicates whether or not this receiver can return non-compressed Steering Matrix explicit feedback. | Set to 000 if not supported The 3 bits separately indicate support for 3 feedback mechanisms. Any combination of these bits is valid. 001 indicates unsolicited delayed feedback 010 indicates immediate feedback 100 indicates aggregated feedback 11 indicates delayed and immediate feedback |
| Explicit compressed Steering Matrix feedback | Indicates whether or not this STA can apply transmit beamforming using explicit compressed Steering Matrix feedback. The STA shall be able both to compress and to use a compressed steering matrix on its transmission for TXBF. | Set to 000 if not supported The 3 bits separately indicate support for 3 feedback mechanisms. Any combination of these bits is valid. 001 indicates unsolicited delayed feedback 010 indicates immediate feedback 100 indicates aggregated feedback 11 indicates delayed and immediate feedback |
| Minimal grouping | Minimal grouping used for explicit feeback report | Set to 00 if support groups of 1 (no grouping) 01 indicates groups of 1, 2 10 indicates groups of 1, 4 11 indicates groups of 1, 2, 4 |
| CSI number of beamformer antennas support | Indicates the maximum number of beamformer antennas the beamformee can support when CSI feedback is required | 00 support single TX antenna sounding 01 support 2 TX antenna sounding 10 support 3 TX antenna sounding 11 support 4 TX antenna sounding |
| Non-compressed steering matrix number of beamformer antennas support | Indicates the maximum number of beamformer antennas the beamformee can support when non-compressed steering matrix feedback is required | Set to 00 for single TX antenna sounding Set to 01 for 2 TX antenna sounding Set to 10 for 3 TX antenna sounding Set to 11 for 4 TX antenna sounding |
| Compressed steering matrix number of beamformer antennas support | Indicates the maximum number of beamformer antennas the beamformee can support when compressed steering matrix feedback is required | Set to 00 for single TX antenna sounding Set to 01 for 2 TX antenna sounding Set to 10 for 3 TX antenna sounding Set to 11 for 4 TX antenna sounding |
| CSI max number of rows beamformer support | The maximum number of rows of CSI explicit feedback from beamformee the beamformer can support when CSI feedback is required | Set to 00 for single row of CSI Set to 01 for 2 rows of CSI Set to 10 for 3 rows of CSI Set to 11 for 4 rows of CSI |

The following Comments have been resolved: 12003(C), 1476(A)

Modify the second paragraph of Section 9.9.1.2 as follows:

The TXOP limit duration values are advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP. A TXOP limit value of 0 indicates that a single MSDU or MMPDU or A-MSDU or A-MPDU along with any ACK or Block Ack response frame, and/or ZLF and/or Explicit Feedback Response, in addition to a possible RTS/CTS exchange and/or CTS to itself, may be transmitted at any rate for each TXOP. A CF-End frame may be transmitted to cancel remaining NAV time if the prediction of the expected frame exchange duration was not accurate.

The following Comments have been resolved: 12011(C)partial, 12012(C), 12013(C), 4238(C), 7822(C), 2282(C), 2284(C), 2298(C), 2299(C) 2305(C), 2307(C), 2324(C), 2325(A)

Modify the table n29 as follows:

- Table n29 – MIMO CSI Matrices

| Order | Information | Name | Size (byte) | Value |
| --- | --- | --- | --- | --- |
| 1 | Category | HT | 1 | TBD |
| 2 | Action | MIMO CSI Matrices Message | 1 | 6 |
| 3 | CSI Matrices Control | CSI Matrices Control | 2 | See text. |
| 4 | MCMR Segment Sequence | MCMR Segment Sequence | 1 | Unused Reserved |
| 5 | Explicit Feedback Sequence Sounding Time Stamp | Explicit Feedback Sequence Sounding Time Stamp | 1 4 | See text |
| 6 | CSI Matrices Report | CSI Matrices Report | see text | See text |

Modify the table n30 as follows:

- Table n30 – Use of CSI Matrices control field (MIMO CSI Matrices frame)

| Field | Description |
| --- | --- |
| Nc | Number of columns in each CSI matrix (Number of transmit antennas in the STA transmitting into the channel) |
| Nr | Number of rows in each CSI matrix |
| Explicit Channel Feedback | Set to 1 |
| Explicit Feedback Format | See Table n31 |

Modify the table n36 as follows:

- Table n36 – MIMO Uncompressed Steering Matrices

| Order | Information | Name | Size (byte) | Value |
| --- | --- | --- | --- | --- |
| 1 | Category | HT | 1 | TBD |
| 2 | Action | MIMO Uncompressed Steering Matrices Message | 1 | 7 |
| 3 | CSI Matrices Control | CSI Matrices Control | 2 | See text. |
| 4 | MCMR Segment Sequence | MCMR Segment Sequence | 1 | Unused Reserved |
| 5 | Explicit Feedback Sequence Sounding Time Stamp | Explicit Feedback Sequence Sounding Time Stamp | 1 4 | See text |
| 6 | Steering Matrices Report | Steering Matrices Report | see text | See text |

Modify the table n43 as follows:

- Table n43 – Compressed Steering Matrices Feedback

| Order | Information | Name | Size (byte) | Value |
| --- | --- | --- | --- | --- |
| 1 | Category | HT | 1 | TBD |
| 2 | Action | Compressed Steering Matrices Feedback | 1 | 8 |
| 3 | CSI Matrices Control | CSI Matrices Control | 2 | See text. |
| 4 | MCMR Segment Sequence | MCMR Segment Sequence | 1 | Unused Reserved |
| 5 | Receiver SNR | RX SNR | 1 | Average Signal to Noise Ratio in the STA sending the report. |
| 6 | Explicit Feedback Sequence Sounding Time Stamp | Explicit Feedback Sequence Sounding Time Stamp | 1 4 | See text |
| 7 | Quantized Steering Matrices Feedback Information | Quantized Steering Matrices Feedback Information | Variable | See text |

Modify the text in the subclause 7.4.7.6 as follows:

Each MIMO Matrices feedback frame has a unique sequence number encoded in the explicit feedback sequence field.  This is incremented between measurements   contains a Sounding Time Stamp field. The Sounding Time Stamp field contains the lower four bytes of the TSF timer value sampled at the instant that the MAC receives the signal PHY-CCA.indicate(IDLE) which corresponds to the end of the reception of the sounding packet which was used to generate the feedback information contained in the MIMO matrices feedback frame.

Remove the text in the subclause 7.4.7.7 as follows:

Each Steering Matrices feedback has a unique sequence number encoded in the explicit feedback sequence field.  This is incremented between measurements.

Remove the text in the subclause 7.4.7.8 as follows:

Each Steering Matrices feedback has a unique sequence number encoded in the explicit feedback sequence field.  This is incremented between measurements.

Add the following text and table to the subclause 7.4a.4 A-MPDU as follows:

Table n53a—A-MPDU contents MPDUs using explicit feedback

| MPDU | Description |
| --- | --- |
| CTS | Clear to Send |
| Ack | Acknowledgement |
| BlockAck | Block ACK MPDUs |
| MTBA | Contains acknowledgement for data received under MTBA ack policy in the previous PSMP-UTT/PSMP-DTT |
| #### QoS Null embedding of Management Action | A QoS Null +HTC frame with the MA field of the HT Control Field set to 1 carries a Management Action Body with explicit feedback response |

An A-MPDU with explicit feedback response shall contain only one control frame of any subtype mentioned in Table n53a A-MPDU contents MPDUs using explicit feedback.

Modify the text in the subclause 9.21.2 as follows:

  - When feeding back the full size providing channel feedback information, the QoS Null +HTC with MA field of the HT Control Field set to 1 is used for MIMO CSI Matrices frame feedback as defined in subclause 7.4.8.8 Multiple CSI Matrices frames may be required to provide the complete feedback information, in which case to feedback the full size channel, and all these frames of one feedback should set identical explicit feedback sequence number.  the value of the Sounding Time Stamp field within each feedback frame shall correspond to the arrival time of the sounding frame that was used to generate the feedback information that is contained in the frame.

The following Comments have been resolved: 12023(C), 8089(C), 1432(C), 1433(C), 1435(C), 1436(C), 1437(C), 2697(C), 10071(C, partially), 10072(partially), 1131(C), 1653(C), 4786(C), 4526(C), 12015(C), 12000(C, partially), 3971(C, partially), 8070(C, partially), 11990 (C),

Modify the 9.20.3 subclause as follows:

    - Explicit feedback beamforming

Remove the following header and text:

      - Introduction (informative)

(Ed: use of “informative” in a section heading is deprecated by the style guide.  Consider moving to Annex.)

When using explicit beamforming the initiator is using feedback it receives from the responder to calculate the steering matrices for beamforming. This feedback may have 1 of those 3 formats:

Channel State Information – The responder sends to the requestor the MIMO channel coefficients.

Uncompressed steering matrices – the responder sends to the requestor the calculated steering matrices, the transmitter may use those for transmission as the steering matrices.

Compressed steering matrices – the responder sends to the requestor the compressed steering matrices, the transmitter may use those for transmission as the steering matrices after the de-compression of the data.

Remove the following header and text:

      - Feedback request and response rules

There are three feedback types and associated rules:

- Immediate – the receiver shall return the feedback a SIFS after end of the PPDU that contains the related request. The feedback may be a single frame or aggregated with another response. The transmitter shall start transmission of the next PPDU no earlier than PIFS after the PPDU containing the request if it did not receive the expected response PPDU.

- Aggregated – the receiver may return the feedback aggregated (with A-MPDU aggregation) with a response frame. The transmitter shall not request aggregated feedback if there is not enough time left in TxOP for the response. The transmitter shall not expect (Ed: reword “shall not expect") getting the feedback of each request.

- Unsolicited – the receiver provides feedback at times of its own choosing (e.g. within its own TXOP). An unsolicited request may be transmitted once per association to inform the responder of the expected type of feedback.

The Responder shall use QoS Null +HTC frame with the MA field of the HT Control Field set to 1 for immediate and aggregated feedback of CSI Matrices, Uncompressed Steering Matrices, and Compressed Steering Matrices frames. The Ack policy shall be set to No Ack in the QoS control field.

      - Transmit Beamforming with explicit Feedback

In this example the AP requests and uses the explicit feedbacks of CSI for computing the steering matrices for transmit beamforming (Figure n45) A similar frame exchange can also accommodate computation of the steering matrices at the responder. In this case, instead of sending back CSI, the responding STA will return quantized steering matrices to the initiating AP, and the AP may use it for transmission. The feedback of steering matrices can be compressed or uncompressed. The type of information returned to Initiator is subject of advertised capabilities. The same rules apply to non-AP STAs as well.

The sequence is as follows:

- The sequence is initiated by an AP which sends an un-steered sounding PPDU containing a CSI Feedback Request.

- The responding client STA uses the sounding packet to make a channel estimate, and collects the CSI of the channel

- The CSI are quantized and returned to the AP in a PPDU containing a CFB (CSI Feedback) message.

- The AP uses the result to transmit a steered PPDU to the client STA.  If the AP has subsequent PPDUs to send in steered mode, this PPDU may also be a sounding PPDU.

Steps b), c), and d) may be repeated for an ongoing beamforming exchange.  If latencies cause the channel estimates or steering matrices to get stale, then step a) needs to be repeated.

Figure n45 – Frame exchange for transmit beamforming with explicit feedback

Insert the following text:

The Requestor (beamformer) uses feedback response information which it receives from the responder (beamformee) to calculate a steering matrix for Transmit beamforming. This feedback response information may have one of three formats:

- Channel State Information – The responder sends the MIMO channel coefficients to the requestor;

- Uncompressed steering matrices – the responder sends calculated steering matrices to the requestor;

- Compressed steering matrices – the responder sends compressed steering matrices to the requestor.

- The supported formats shall be advertised in the responder’s HT Capability information element.

The requestor shall discard the feedback response information if the TSF time at PHY_CCA.indicate(IDLE) which was generated by the feedback response information frame’s arrival minus the value from the Sounding Time Stamp field in the feedback response information frame is greater than dot11MaxCSIFeedbackDelay.

The responder’s responding capabilities shall be advertised in HT Capability information elements contained in beacon, probe request and response, and association request and response frames which are transmitted by the responder. Devices which are capable of acting as a responder shall advertise one or both of the following response capabilities:

- Immediate - the responder is capable of sending feedback response information SIFS after receiving a sounding PPDU and/or is capable of sending feedback response information aggregated in a PPDU that contains a MAC response within the requestor’s TXOP

- Delayed- the responder is not capable of sending the feedback response information in SIFS time after receiving a sounding PPDU and shall not send the feedback response information in SIFS time after receiving a sounding PPDU, but it is capable of sending the feedback response information in a TxOP that it obtains.

The sounding frame types supported by the responder - staggered and/or ZLF - shall be advertised in HT capability information elements which are transmitted by the responder.

Any Station that advertises itself as any of Explicit TXBF CSI feedback capable or Explicit uncompressed Steering Matrix feedback capable or Explicit compressed Steering Matrix feedback capable shall submit explicit feedback based on the receipt of a +HTC sounding PPDU in which the CSI/Steering field has a non-zero value and which does not contain extension HT-LTFs. The generation of feedback in response to the receipt of such a frame shall always be supported by the responder which advertises any of Explicit TXBF CSI feedback capable or Explicit uncompressed Steering Matrix feedback capable or Explicit compressed Steering Matrix feedback capable, independently of the values of the Receive staggered sounding and the Receive ZLF capabilities.

The requestor shall set the NOT_SOUNDING parameter of the TXVECTOR to zero in the PHY-TXSTART.request(TXVECTOR) primitive corresponding to each packet that is used for sounding.

The requestor shall set the response type format indicated in the CSI/steering field of the HTC field of any sounding frame but the ZLF sounding frame and of any PPDU with the ZLF sounding announcement bit set to 1 to one of the non-zero values (CSI or compressed steering matrix or uncompressed steering matrix) that corresponds to a type which is supported by the responder.

The responder shall use the response type format (CSI or compressed steering matrix or uncompressed steering matrix) indicated in the CSI/steering field of the HTC field of any sounding frame but the ZLF sounding frame and of any PPDU with the ZLF sounding announcement bit set to 1 to return the response feedback information.

The MAC shall interpret the receipt of a PHY-RXSTART.indicate(RXVECTOR) with the NOT_SOUNDING parameter value set to 0 as a sounding packet. The non-ZLF sounding packet that is directed to a receiver’s MAC address and that contains a non-zero CSI/steering field of the HTC field shall be interpreted as a response feedback request.

A ZLF sounding packet announced by a PPDU (9.22) with the ZLF sounding announcement bit set to 1 and with a non-zero CSI/steering field shall be interpreted as a response feedback request.

The type of response feedback information requested is indicated in the CSI/Steering field of the HTC field of a non-ZLF sounding frame and in the CSI/Steering field of the HTC field of a ZLF-announcement PPDU.

The responder shall not return CSI feedback information that is generated from a received frame which is not interpreted as a response feedback request.

The responder that advertises itself as immediate feedback capable shall transmit immediate or aggregated feedback response information in a frame that is appropriate for the current frame exchange sequence as follows:

If a MAC response is required (CTS, ACK, BA), then the feedback response information may be aggregated with the MAC response, otherwise the feedback response information shall be sent SIFS after the reception of the sounding PPDU. If ZLF sounding is used, then the transmission of a frame containing feedback response information may follow the ZLF, but the MAC response frame is transmitted a SIFS after reception of the PPDU that elicited the MAC response.

If the immediate feedback capable responder cannot transmit the aggregated or immediate CSI/Steering response information in a SIFS time after the end of the received sounding packet, it may transmit the feedback response information in an aggregate with an ACK or BA in the same TxOP.

The responder shall not respond with feedback information and shall not aggregate the feedback response information with an ACK or BA if there is not enough time remaining in the TXOP to deliver the aggregated response.

The requester shall resume transmissions of subsequently queued frames (if any) if a PHY_RXSTART.indication does not occur during the ACKTimeout interval after the end of the transmission of a sounding packet and if the time remaining in the TXOP is sufficient for the transmission of the next frame.

A responder transmitting immediate and aggregated feedback response shall use a QoS Null embedding as per 7.2.2.2 with the Ack policy set to No Ack in the QoS control field of this frame.

A responder transmitting delayed feedback response information shall use a Management Action frame to send this information within a separate TXOP which is obtained using the highest priority AC as per 9.1.3.1.

The responder shall insert the Sounding Time Stamp field in each frame which contains the feedback response information.

A STA which has been granted a Reverse Direction Grant may act as a requester during the reverse direction grant time period, provided that the reverse direction grant rules are obeyed.

A responder that advertises itself as delayed feedback capable shall not transmit immediate or aggregated feedback response information, unless it also advertises itself as immediate feedback capable.

The requestor may use the following worst case parameters to estimate the duration of the expected frame that contains the feedback response information: Basic MCS, Mixed Mode, Minimal Grouping.

An Explicit Feedback Request can be combined with an MRQ. If the response contains a Steering Matrix then the returned MCS shall be derived from the same information that was used to generate this particular steering matrix. If the response contains channel coefficients then the returned MCS shall be derived from an analysis of the sounding frame that was used to generate the channel coefficients. The MFB field of “all ones” (meaning no feedback) may be used when the responder is unable to generate the MCS in time for inclusion in the response transmission frame. A CSI capable STA may be incapable of generating MCS feedback

The following Comments have been resolved:10015, 11894, 12002(C), 12004(C), 1439(C), 1440(C), 1441(C), 2727(C), 3880(C), 7370(C), 7671(A), 3880, 7281(C), 7838(A), 2719(C)

The following Comments have been rejected: 12008, 12009, 12010

- Modify the Section 9.22 as follows:

If an HT-STA has indicated receive capability for zero-length sounding frames in the HT capabilities IE during association, that STA shall process a zero-length MAC frame (ZLF) as a sounding frame if the destination of the sounding frame is determined to match itself as described in this subclause, and if the source of the sounding frame can be ascertained as described in this subclause.

The indicated MAC length of zero within a ZLF shall be an implicit indication that the frame is a sounding frame.

The requestor shall set the NOT_SOUNDING parameter of the TXVECTOR to zero in the PHY-TXSTART.request(TXVECTOR) primitive corresponding to ZLF to indicate that this frame is a sounding frame.

Only the current TXOP owner or RDG grantee shall be permitted to send a ZLF. The TxOP owner shall not assert to 1 the ZLF announcement and RDG fields both together. The RDG grantee shall not assert to 1 the ZLF announcement and assert to 0 the More PPDU fields both together.

A transmitter that sends a ZLF shall send the ZLF within SIFS of after sending a non-ZLF frame not requiring an immediate response, with the ZLF Announcement bit set to 1, which does not require an immediate response or it shall send the ZLF within a SIFS after of successfully receiving a correctly formed and addressed immediate response to a non-ZLF frame requiring an immediate response, with the ZLF Announcement bit set to 1 which required an immediate response. This rule insures that the ZLF receiver knows that it will receive a ZLF, and that it can ascertain the source and destination of the ZLF and that the receiver and transmitter both know when the immediate response and ZLF shall be transmtted relative to the frame containing the ZLF announcement indication.

A transmitter shall send only one ZLF frame per ZLF announcement, unless the non-ZLF frame with the ZLF Announcement bit set to 1 includes a value in the DATA subfield of the ASC subfield of the HTC control field which is greater than one, in which case, the transmitter shall send the number of ZLF frames indicated in the DATA field.

The transmitter shall use the control frame wrapper (7.2.1.8.4) to announce ZLF in non-HT PPDU control frames.

Feedback information generated from the reception of a ZLF shall be transmitted using delayed any of the feedback rules and signaling as appropriate, e.g. immediate or delayed.

If the reception immediately preceeding (the frame ending at SIFS or less from the start of the ZLF) the ZLF has an FCS error, then the ZLF shall be dropped by the receiver.

If the HT Control field of a frame with the ZLF Announcement bit set to 1 announces a number of ZLF sounding frames that is greater than one as indicated by the value provided in the data portion of the ASC subfield of the Link Adaptation control field of the HTC field, then the transmitter shall transmit an additional ZLF within SIFS from after the end of each transmitted ZLF until the total number of ZLF transmissions is equal to the announced number of ZLFs.

The number of sounded spatial streams in ZLF is detrmined by MCS. The MCS of ZLF are used for determination of number of spatial streams only. The MCS of ZLF shall not be limited by the receiving station’s supported MCSs. The requestor shall set the NUM_EXTEN_SS parameter of the TXVECTOR to zero in the PHY TXSTART.request(TXVECTOR) primitive corresponding to ZLF.

A STA shall not transmit a ZLF with a destination address corresponding to a STA which does not support TXBF.

The following Comments have been resolved: 2727(C)

Insert the following lines at the end of 9.22.1:

If the frame immediately previous (within in SIFS distance) to the reception of a ZLF frame is also a ZLF frame, then the destination of the ZLF frame is equal to the destination of the immediately previous ZLF frame.

Insert the following lines at the end of 9.22.2:

If the frame immediately previous (within in SIFS distance) to the reception of a ZLF frame is also a ZLF frame, then the source of the ZLF frame is equal to the source of the immediately previous ZLF frame.

The following Comments has been resolved: 12014(C)

Add to the MIB sequence as follows:

- Dot11PhyHTEntry ::= SEQUENCE {

dot11ReceiveStaggerSoundingOptionImplemented TruthValue,

dot11TransmitStaggerSoundingOptionImplemented TruthValue,

dot11ReceiveZLFOptionImplemented TruthValue,

dot11TransmitZLFOptionImplemented TruthValue,

dot11ImplicitTxBFOptionImplemented TruthValue,

dot11CalibrationOptionImplemented INTEGER,

dot11ExplicitCSITxBFOptionImplemented TruthValue,

dot11ExplicitUncompressedSteeringMatrixOptionImplemented TruthValue,

dot11ExplicitBFCSIFeedbackOptionImplemented INTEGER,

dot11ExplicitUncompressedSteeringMatrixFeedbackOptionImplemented INTEGER,

dot11ExplicitCompressedSteeringMatrixFeedbackOptionImplemented INTEGER,

dot11NumberBeamFormingCSISupportAntenna INTEGER,

dot11NumberUncompressedSteeringMatrixSupportAntenna INTEGER,

dot11NumberCompressedSteeringMatrixSupportAntenna INTEGER

dot11MaxCSIFeedbackDelay Unsigned32

}

Add new line to the table n87 in the section dot11 Tx BF Config Table after dot11NumberCompressedSteeringMatrixSupportAntenna

| dot11MaxCSIFeedbackDelay | Implementation dependent | Static |
| --- | --- | --- |

Add new definition after definition of dot11NumberCompressedSteeringMatrixSupportAntenna

dot11MaxCSIFeedbackDelay OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the maximum allowed delay in microseconds between the time of sounding packet issuing and the time of feedback response arriving(9.20.3)"

::= { dot11TxBFConfigEntry 15 }