IEEE P802.11
Wireless LANs

| Q-Removal from QSTA, QAP, QBSS, etc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | AT&T | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

P802.11e added the acronym and notation Qxxx for an “xxx” that supports the QoS extensions. This has caused numerous other groups to similarly add decoration to the standard acronyms for 802.11 elements.

Such an acronym scheme is exponential complexity in the document. For example, if a “Balloon” is initially defined, and subsequent amendments defined a “Red Balloon” and, later, a “Big Balloon”, no additional definition would be needed for a “Big Red Balloon” (or a “Red Big Balloon”), since the English description allows multiple adjectives prior to a noun. But if the acronym RB was defined, then there would be a need to add acronym definitions BB, BRB, and RBB because of the combined terms.

It is proposed to remove this complexity before it spreads further. This submission identifies all of the places in D7.0 that need to be updated with this change.

A separate submission (11-06-0555-xx) proposes the additional step of making the Fast BSS Transition Initial Association procedure to be always Association Request/Response frames, instead of (Re)association request/response.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

There are several general principles applied in these changes.

Normal IEEE Style requires the first use of an acronym to include its meaning.  This is done throughout clause 3 in the case of the QoS acronyms, and the acronym can be simply be deleted after its equivalent text. For example, uses of “QoS STA (QSTA)” can be simply replaced by “QoS STA”. The acronym definitions themselves in clause 4 can also be simply deleted.

It is always permissible to replace an acronym with its meaning, such as a replacement of QSTA with QoS STA, QAP with QoS AP, etc.  In general, this is done except where one of the following exceptions apply.

Items that are simply names of information elements (such as QBSS Load IE) or error code names (such as QSTA_LEAVING), or MLME parameter names (NonAPQSTAAddress) can be globally changed without any effect.

Certain information elements (such as the QoS Capabilities IE) only appear in frames when certain MIB variables (dot11QoSCapability) are set true. When the information element is being described, there is no need to specify QAP and QSTA, as the conditions for including the information element in the frames already states that the AP/STA supports QoS.

Certain frame types only appear, and can only be generated, by STAs that support the QoS facility.  The standard already requires that an AP and STA check the support of the QoS frame types at the receiver before sending them.  It is therefore unnecessary to specify that the originator of a QoS+Data frame is a QSTA rather than simply a STA.

Extensions such as BlockAck, APSD, and DLS are indicated by separate bits in the Capabilities IE.  APs and STAs that support these capabilities could more-correctly be called BlockAck AP, or DLS STA.  The standard already requires that an AP and STA check the support of these capabilities before using the extension.  Therefore, the fact that the AP/STA also supports QoS is of no additional consequence, and QAP/QSTA can be changed to AP/STA without any loss of meaning.

The following places in 11ma D7.0 to be changed as follows:

| Clause | Page (paper) | Page (pdf) | Line | Usage | Replace highlighted text with: |
| --- | --- | --- | --- | --- | --- |
| 3.1 | 6 | 48 | 4 | A label for the common set of enhanced distributed channel access (EDCA) parameters that are used by a quality of service (QoS) station (QSTA) to contend for the channel in order to transmit medium access control (MAC) service data units (MSDUs) with certain priorities. | Delete |
| 3.7 | 6 | 48 | 23 | The aggregation of delivery and/or poll schedules by the quality of service (QoS) access point (QAP) for a particular non-access point (non-AP) QoS station (QSTA) into a single service period (SP). | Delete |
| 3.7 | 6 | 48 | 23 | The aggregation of delivery and/or poll schedules by the quality of service (QoS) access point (QAP) for a particular non-access point (non-AP) QoS station (QSTA) into a single service period (SP). | Delete |
| 3.26 | 7 | 49 | 41 | A quality of service (QoS) basic service set (QBSS) will have one DCF and one HCF. | Delete |
| 3.34 | 8 | 50 | 14 | A quality of service (QoS) access point (QAP) AC used for the delivery of traffic to a non-access point (non-AP) QoS station (QSTA) in an unscheduled service period (SP) triggered by the station (STA). | Delete |
| 3.34 | 8 | 50 | 16 | A quality of service (QoS) access point (QAP) AC used for the delivery of traffic to a non-access point (non-AP) QoS station (QSTA) in an unscheduled service period (SP) triggered by the station (STA). | Delete |
| 3.36 | 8 | 50 | 21 | A unidirectional link from one non-access point (non-AP) quality of service (QoS) station (QSTA) to another non-AP QSTA operating in the same infrastructure QoS basic service set (QBSS) that does not pass through a QoS access point (QAP). Once a direct link has been set up, all frames between the two non-AP QSTAs are exchanged directly. | Delete |
| 3.36 | 8 | 50 | 21 | A unidirectional link from one non-access point (non-AP) quality of service (QoS) station (QSTA) to another non-AP QSTA operating in the same infrastructure QoS basic service set (QBSS) that does not pass through a QoS access point (QAP). Once a direct link has been set up, all frames between the two non-AP QSTAs are exchanged directly. | STA |
| 3.36 | 8 | 50 | 22 | A unidirectional link from one non-access point (non-AP) quality of service (QoS) station (QSTA) to another non-AP QSTA operating in the same infrastructure QoS basic service set (QBSS) that does not pass through a QoS access point (QAP). Once a direct link has been set up, all frames between the two non-AP QSTAs are exchanged directly. | Delete |
| 3.36 | 8 | 50 | 23 | A unidirectional link from one non-access point (non-AP) quality of service (QoS) station (QSTA) to another non-AP QSTA operating in the same infrastructure QoS basic service set (QBSS) that does not pass through a QoS access point (QAP). Once a direct link has been set up, all frames between the two non-AP QSTAs are exchanged directly. | Delete |
| 3.36 | 8 | 50 | 24 | A unidirectional link from one non-access point (non-AP) quality of service (QoS) station (QSTA) to another non-AP QSTA operating in the same infrastructure QoS basic service set (QBSS) that does not pass through a QoS access point (QAP). Once a direct link has been set up, all frames between the two non-AP QSTAs are exchanged directly. | STAs |
| 3.51 | 9 | 51 | 20 | The prioritized carrier sense multiple access with collision avoidance (CSMA/CA) access mechanism used by quality of service (QoS) stations (QSTAs) in a QoS basic service set (QBSS). This access mechanism is also used by the QoS access point (QAP) and operates concurrently with hybrid coordination function (HCF) controlled channel access (HCCA). | Delete |
| 3.51 | 9 | 51 | 21 | The prioritized carrier sense multiple access with collision avoidance (CSMA/CA) access mechanism used by quality of service (QoS) stations (QSTAs) in a QoS basic service set (QBSS). This access mechanism is also used by the QoS access point (QAP) and operates concurrently with hybrid coordination function (HCF) controlled channel access (HCCA). | Delete |
| 3.51 | 9 | 51 | 21 | The prioritized carrier sense multiple access with collision avoidance (CSMA/CA) access mechanism used by quality of service (QoS) stations (QSTAs) in a QoS basic service set (QBSS). This access mechanism is also used by the QoS access point (QAP) and operates concurrently with hybrid coordination function (HCF) controlled channel access (HCCA). | Delete |
| 3.52 | 9 | 51 | 25 | A logical function in a quality of service (QoS) station (QSTA) that determines, using enhanced distributed channel access (EDCA), when a frame in the transmit queue with the associated access category (AC) is permitted to be transmitted via the wireless medium (WM). | Delete |
| 3.66 | 10 | 52 | 26 | A coordination function that combines and enhances aspects of the contention-based and contention-free access methods to provide quality of service (QoS) stations (QSTAs) with prioritized and parameterized QoS access to the wireless medium (WM), while continuing to support non-QSTAs (nQSTAs) for best-effort transfer. | Delete |
| 3.66 | 10 | 52 | 27 | A coordination function that combines and enhances aspects of the contention-based and contention-free access methods to provide quality of service (QoS) stations (QSTAs) with prioritized and parameterized QoS access to the wireless medium (WM), while continuing to support non-QSTAs (nQSTAs) for best-effort transfer. | Delete |
| 3.66 | 10 | 52 | 30 | It supports a uniform set of frame formats and exchange sequences that QSTAs may use during both the contention period (CP) and the contention-free period (CFP). | STAs |
| 3.67 | 10 | 52 | 38 | The HC performs bandwidth management including the allocation of transmission opportunities (TXOPs) to QoS stations (QSTAs). | Delete |
| 3.67 | 10 | 52 | 39 | The HC is collocated with a QoS access point (QAP). | Delete |
| 3.68 | 10 | 52 | 43 | The channel access mechanism utilized by the hybrid coordinator (HC) to coordinate contention-free media use by quality of service (QoS) stations (QSTAs) for downlink unicast, uplink, and direct-link transmissions. | Delete |
| 3.91 | 12 | 54 | 8 | Non-access point (non-AP) quality of service (QoS) station (QSTA): | Delete |
| 3.91 | 12 | 54 | 10 | A non-AP QSTA does not have a hybrid coordinator (HC) and uses the QoS AP (QAP) for the distribution system services (DSSs). | STA |
| 3.91 | 12 | 54 | 11 | A non-AP QSTA does not have a hybrid coordinator (HC) and uses the QoS AP (QAP) for the distribution system services (DSSs). | Delete |
| 3.93 | 12 | 54 | 16 | Non-quality of service (non-QoS) access point (nQAP): | Delete |
| 3.94 | 12 | 54 | 19 | Non-quality of service (non-QoS) basic service set (nQBSS): | Delete |
| 3.95 | 12 | 54 | 22 | Non-quality of service (non-QoS) station (nQSTA): | Delete |
| 3.118 | 14 | 56 | 1 | Quality of service (QoS) access point (QAP): | Delete |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | QoS access point |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | non-QoS access point |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | Delete |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | QoS access point |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | a non-QoS access point |
| 3.118 | 14 | 56 | 2 | The functions of a QAP are a superset of the functions of a non-QAP (nQAP), and thus a QAP is able to function as an nQAP to non-QoS stations (nQSTAs). | Delete |
| 3.119 | 14 | 56 | 5 | Quality of service (QoS) basic service set (QBSS): | Delete |
| 3.119 | 14 | 56 | 7 | An infrastructure QBSS contains a QoS access point (QAP). | QoS BSS |
| 3.119 | 14 | 56 | 7 | An infrastructure QBSS contains a QoS access point (QAP). | Delete |
| 3.121 | 14 | 56 | 12 | Quality of service (QoS) independent basic service set (QIBSS): | Delete |
| 3.122 | 14 | 56 | 15 | Quality of service (QoS) station (QSTA): | Delete |
| 3.122 | 14 | 56 | 16 | A QSTA acts as a non-QSTA (nQSTA) when associated in a non-QoS basic service set (nQBSS). | Delete |
| 3.122 | 14 | 56 | 16 | A QSTA acts as a non-QSTA (nQSTA) when associated in a non-QoS basic service set (nQBSS). | Delete |
| 3.131 | 14 | 56 | 49 | The SP that is scheduled by the quality of service (QoS) access point (QAP). | Delete |
| 3.135 | 15 | 57 | 8 | A contiguous time during which one or more downlink unicast frames are transmitted to a quality of service (QoS) station (QSTA) and/or one or more transmission opportunities (TXOPs) are granted to the same QSTA. | Delete |
| 3.135 | 15 | 57 | 9 | A contiguous time during which one or more downlink unicast frames are transmitted to a quality of service (QoS) station (QSTA) and/or one or more transmission opportunities (TXOPs) are granted to the same QSTA. | STA. |
| 3.155 | 16 | 58 | 38 | The classification process, performed above the MAC service access point (MAC_SAP) at a quality of service (QoS) access point (QAP) uses the parameter values for a given TS to examine each incoming MSDU and determine whether this MSDU belongs to that TS. | Delete |
| 3.155 | 16 | 58 | 39 | TCLAS may also occur at non-access point (non-AP) QoS station (QSTA) with multiple streams. | Delete |
| 3.157 | 16 | 58 | 49 | The quality of service (QoS) characteristics of a data flow to and from a non-access point (non-AP) QoS station (QSTA). | Delete |
| 3.161 | 17 | 59 | 15 | An interval of time when a particular quality of service (QoS) station (QSTA) has the right to initiate frame exchange sequences onto the wireless medium (WM). | Delete |
| 3.161 | 17 | 59 | 17 | The TXOP is either obtained by the QSTA by successfully contending for the channel or assigned by the hybrid coordinator (HC). | STA |
| 3.162 | 17 | 59 | 20 | A quality of service (QoS) station (QSTA) that has either been granted a TXOP by the hybrid coordinator (HC) or successfully contended for a TXOP. | Delete |
| 3.164 | 17 | 59 | 27 | A non-access point (non-AP) quality of service (QoS) station (QSTA) AC where frames of subtype QoS Data and QoS Null from the non-AP QSTA that map to the AC trigger an unscheduled service period (SP) if one is not in progress. | Delete |
| 3.164 | 17 | 59 | 28 | A non-access point (non-AP) quality of service (QoS) station (QSTA) AC where frames of subtype QoS Data and QoS Null from the non-AP QSTA that map to the AC trigger an unscheduled service period (SP) if one is not in progress. | STA |
| 3.170 | 17 | 59 | 48 | The period that is started when a non-access point (non-AP) quality of service (QoS) station (QSTA) transmits a trigger frame to the QoS access point (QAP). | Delete |
| 3.170 | 17 | 59 | 48 | The period that is started when a non-access point (non-AP) quality of service (QoS) station (QSTA) transmits a trigger frame to the QoS access point (QAP). | Delete |
| 4 | 22 | 64 | 20 | nQAP | Delete entry |
| 4 | 22 | 64 | 21 | nQBSS | Delete entry |
| 4 | 22 | 64 | 22 | nQSTA | Delete entry |
| 4 | 23 | 65 | 15 | QAP | Delete entry |
| 4 | 23 | 65 | 16 | QBSS | Delete entry |
| 4 | 23 | 65 | 17 | QIBSS | Delete entry |
| 4 | 23 | 65 | 24 | QSTA | Delete entry |
| 5.2.6 | 32 | 74 | 32 | QoS basic service set (QBSS): The QoS network | Delete |
| 5.2.6 | 32 | 74 | 36 | The QoS enhancements are available to QoS stations (QSTAs) associated with a QoS access point (QAP) in a QBSS. | Delete |
| 5.2.6 | 32 | 74 | 37 | The QoS enhancements are available to QoS stations (QSTAs) associated with a QoS access point (QAP) in a QBSS. | Delete |
| 5.2.6 | 32 | 74 | 37 | The QoS enhancements are available to QoS stations (QSTAs) associated with a QoS access point (QAP) in a QBSS. | QoS BSS |
| 5.2.6 | 32 | 74 | 37 | A subset of the QoS enhancements is available for use between QSTAs that are members of the same QoS independent basic service set (IBSS) (QIBSS). | STAs |
| 5.2.6 | 32 | 74 | 38 | A subset of the QoS enhancements is available for use between QSTAs that are members of the same QoS independent basic service set (IBSS) (QIBSS). | Delete |
| 5.2.6 | 32 | 74 | 38 | Because a QSTA implements a superset of station (STA) functionality, as defined in this standard, the QSTA may associate with a non-QoS access point (nQAP) in a non-QoS basic service set (BSS) (nQBSS), to provide non-QoS MAC data service when there is no QBSS with which to associate. | QoS station (STA) |
| 5.2.6 | 32 | 74 | 38 | Because a QSTA implements a superset of station (STA) functionality, as defined in this standard, the QSTA may associate with a non-QoS access point (nQAP) in a non-QoS basic service set (BSS) (nQBSS), to provide non-QoS MAC data service when there is no QBSS with which to associate. | Delete |
| 5.2.6 | 32 | 74 | 39 | Because a QSTA implements a superset of station (STA) functionality, as defined in this standard, the QSTA may associate with a non-QoS access point (nQAP) in a non-QoS basic service set (BSS) (nQBSS), to provide non-QoS MAC data service when there is no QBSS with which to associate. | STA |
| 5.2.6 | 32 | 74 | 40 | Because a QSTA implements a superset of station (STA) functionality, as defined in this standard, the QSTA may associate with a non-QoS access point (nQAP) in a non-QoS basic service set (BSS) (nQBSS), to provide non-QoS MAC data service when there is no QBSS with which to associate. | Delete (twice) |
| 5.2.6 | 32 | 74 | 41 | Because a QSTA implements a superset of station (STA) functionality, as defined in this standard, the QSTA may associate with a non-QoS access point (nQAP) in a non-QoS basic service set (BSS) (nQBSS), to provide non-QoS MAC data service when there is no QBSS with which to associate. | QoS BSS |
| 5.2.6 | 32 | 74 | 43 | The enhancements that distinguish QSTAs from non-QoS STAs (nQSTAs) and QAPs from nQAPs are collectively term the QoS facility. | …distinguish QoS STAs fron non-QoS STAs and QoS APs from non-QoS APs are… |
| 5.2.6 | 32 | 74 | 45 | The quantity of certain, QoS-specific, mechanisms may vary among QoS implementations, as well as between QSTAs and QAPs, over ranges specified in subsequent clauses. | QoS STAs |
| 5.2.6 | 32 | 74 | 45 | The quantity of certain, QoS-specific, mechanisms may vary among QoS implementations, as well as between QSTAs and QAPs, over ranges specified in subsequent clauses. | QoS APs |
| 5.2.6 | 32 | 74 | 48 | A QSTA or QAP must implement these core QoS facilities necessary for its QoS functions to interoperate with other QSTAs in the QBSS. | QoS STA |
| 5.2.6 | 32 | 74 | 48 | A QSTA or QAP must implement these core QoS facilities necessary for its QoS functions to interoperate with other QSTAs in the QBSS. | QoS AP |
| 5.2.6 | 32 | 74 | 50 | A QSTA or QAP must implement these core QoS facilities necessary for its QoS functions to interoperate with other QSTAs in the QBSS. | STAs |
| 5.2.6 | 32 | 74 | 50 | A QSTA or QAP must implement these core QoS facilities necessary for its QoS functions to interoperate with other QSTAs in the QBSS. | BSS |
| 5.2.6 | 32 | 74 | 51 | Functions such as the Block Ack, DLS, and APSD are separate from the core QoS facilities, and the presence of these functions is indicated by QSTAs separately from the core QoS facilities. | STAs |
| 5.2.6 | 33 | 75 | 14 | A non-access point (non-AP) QSTA based on its requirements requests the HC for TXOPs, both for its own transmissions as well as for transmissions from the QAP to itself. | STA |
| 5.2.6 | 33 | 75 | 14 | A non-access point (non-AP) QSTA based on its requirements requests the HC for TXOPs, both for its own transmissions as well as for transmissions from the QAP to itself. | AP |
| 5.2.6 | 33 | 75 | 16 | The request is initiated by the station management entity (SME) of the non-AP QSTA. | STA |
| 5.2.6 | 33 | 75 | 16 | The HC, which is collocated at the QAP, either accepts or rejects the request based on an admission control policy. | AP |
| 5.2.6 | 33 | 75 | 18 | If the request is accepted, the HC schedules TXOPs for both the QAP and the non-AP QSTA. | AP |
| 5.2.6 | 33 | 75 | 18 | If the request is accepted, the HC schedules TXOPs for both the QAP and the non-AP QSTA. | STA |
| 5.2.6 | 33 | 75 | 19 | For transmissions from the non-AP QSTA, the HC polls the non-AP QSTA based on the parameters supplied by the non-AP QSTA at the time of its request. | STA (three times) |
| 5.2.6 | 33 | 75 | 20 | For transmissions to the non-AP QSTA, the QAP directly obtains TXOPs from the collocated HC and delivers the queued frames to the non-AP QSTA, again based on the parameters supplied by the non-AP QSTA. | STA |
| 5.2.6 | 33 | 75 | 20 | For transmissions to the non-AP QSTA, the QAP directly obtains TXOPs from the collocated HC and delivers the queued frames to the non-AP QSTA, again based on the parameters supplied by the non-AP QSTA. | AP |
| 5.2.6 | 33 | 75 | 20 | For transmissions to the non-AP QSTA, the QAP directly obtains TXOPs from the collocated HC and delivers the queued frames to the non-AP QSTA, again based on the parameters supplied by the non-AP QSTA. | STA (twice) |
| 5.2.6 | 33 | 75 | 26 | nQSTAs may associate in a QBSS, if allowed to associate by the QAP. | Non-QoS STAs |
| 5.2.6 | 33 | 75 | 26 | nQSTAs may associate in a QBSS, if allowed to associate by the QAP. | QoS BSS |
| 5.2.6 | 33 | 75 | 26 | nQSTAs may associate in a QBSS, if allowed to associate by the QAP. | AP |
| 5.2.6 | 33 | 75 | 27 | All directed frames that are sent to nQSTAs by a QAP shall not use the frame formats associated with the QoS facility. | …sent to non-QoS STAs by an AP shall… |
| 5.2.6 | 33 | 75 | 29 | A QSTA associated in an nQBSS shall act as a nQSTA. | A QoS STA associated in a non-QoS BSS shall act as a non-QoS STA. |
| 5.4.1.3 | 36 | 78 | 52 | QoS traffic scheduling provides intra-QBSS QoS frame transfers under the HCF, using either contention-based on controlled channel access. | intra-BSS |
| 5.4.1.3 | 36 | 78 | 53 | At each TXOP, a traffic scheduling entity at the QSTA selects a frame for transmission, from the set of frames at the heads of a plurality of traffic queues, based on requested UP and/or parameter values in the traffic specification (TSPEC) for the requested MSDU. | STA |
| 5.4.6 | 42 | 84 | 40 | In support of such applications, this standard defines a MAC service that enables layers above the MAC to accurately synchronize application-dependent timers shared among QSTAs. | STAs |
| 5.4.6 | 42 | 84 | 44 | Although the timer synchronization methods and accuracy requirements are application-dependent and are beyond the scope of this standard, they rely on an indication from each QSTA’s MAC that is provided essentially simultaneously, via multicast, to the QSTAs. | STA’s |
| 5.4.6 | 42 | 84 | 44 | Although the timer synchronization methods and accuracy requirements are application-dependent and are beyond the scope of this standard, they rely on an indication from each QSTA’s MAC that is provided essentially simultaneously, via multicast, to the QSTAs. | STAs. |
| 5.4.6 | 42 | 84 | 48 | The last symbol is observed on the air by QSTAs within a BSS while the delay between the observation and the delivery of the indication is known within the MAC by design (and communicated to the application by implementation-dependent means). | STAs |
| 5.7 | 44 | 86 | 48 | A QIBSS supports operation under the HCF using TXOPs gained through the EDCA mechanism. | QoS IBSS |
| 5.7 | 44 | 86 | 50 | A QIBSS has no HC and does not support polled TXOP operation and setting up of TSPEC. | QoS IBSS |
| 6.1.1 | 53 | 95 | 16 | All STAs will support the data service, but only QSTAs in a QBSS differentiate their MSDU delivery according to the designated traffic category of traffic stream (TS) of individual MSDUs. | QoS STAs |
| 6.1.1 | 53 | 95 | 16 | All STAs will support the data service, but only QSTAs in a QBSS differentiate their MSDU delivery according to the designated traffic category of traffic stream (TS) of individual MSDUs. | QoS SS |
| 6.1.1 | 53 | 95 | 21 | Because operation of certain functions of the MAC may cause reordering of some MSDUs, as discussed in more detail below, in nQSTAs there are two service classes within the data service. | non-QoS STAs |
| 6.1.1 | 53 | 95 | 26 | There are two service classes available in a QSTA: QoSAck and QoSNoAck. | QoS STA |
| 6.1.1 | 53 | 95 | 29 | In QSTAs either associated in a QBSS or having membership in a QIBSS, the MAC uses a set of rules that tends to cause higher UP MSDUs in a BSS to be sent before lower UP MSDUs in the BSS. | QoS STAs |
| 6.1.1 | 53 | 95 | 29 | In QSTAs either associated in a QBSS or having membership in a QIBSS, the MAC uses a set of rules that tends to cause higher UP MSDUs in a BSS to be sent before lower UP MSDUs in the BSS. | BSS |
| 6.1.1 | 53 | 95 | 29 | In QSTAs either associated in a QBSS or having membership in a QIBSS, the MAC uses a set of rules that tends to cause higher UP MSDUs in a BSS to be sent before lower UP MSDUs in the BSS. | an IBSS |
| 6.1.1 | 53 | 95 | 34 | In a QBSS with some QSTAs supporting the QoS facility and others not supporting the QoS facility, in delivering an MSDU to an nQSTA, the QSTA uses the access category (AC) corresponding to the UP of the MSDU. | In a BSS with some STAs supporting… |
| 6.1.1 | 53 | 95 | 34 | In a QBSS with some QSTAs supporting the QoS facility and others not supporting the QoS facility, in delivering an MSDU to an nQSTA, the QSTA uses the access category (AC) corresponding to the UP of the MSDU. | a non-QoS STA, the QoS STA |
| 6.1.1.1.1 | 53 | 95 | 46 | Determination of UP of received frames at the QAP sent by other STAs in the BSS. | AP |
| 6.1.1.1.1 | 53 | 95 | 48 | The received unicase frames at the QAP may be as follows: | AP |
| 6.1.1.1.1 | 53 | 95 | 50 | Non-QoS subtypes, in which case the QAP shall assign to them a priority of Contention, if they are received during the contention period (CP), or ContentionFree, if they are received during the contention-free period (CFP). | AP |
| 6.1.1.1.1 | 53 | 95 | 53 | QoS subtypes, in which case the QAP shall infer the UP value from the TID in the QoS control field directly for TID values between 0 and 7. | AP |
| 6.1.1.1.1 | 53 | 95 | 54 | For TID values between 8 and 15, the QAP shall extract the UP value in the UP subfield of the TS Info field in the associated TSPEC or from the UP field in the associated TCLAS (traffic classification) element, as applicable. | AP |
| 6.1.1.1.1 | 54 | 96 | 4 | QAPs deliver the UP with the received MSDUs to the DS. | QoS APs |
| 6.1.1.2 | 54 | 96 | 16 | Outgoing MSDUs with UP values 0 through 7 are handled by MAC entities at QSTAs in accordance with the UP. | STAs |
| 6.1.1.2 | 54 | 96 | 20 | Outgoing MSDUs with priority parameter values 8 through 15 are handled by MAC entities at QSTAs in accordance with the UP value determined from the UP subfield as well as other parameter values in the selected TSPEC. | STAs |
| 6.1.1.2 | 54 | 96 | 25 | The non-integer values of the priority parameter are allowed at all nQSTAs. | non-QoS STAs |
| 6.1.1.2 | 54 | 96 | 26 | The use of priority value of ContentionFree is deprecated at QSTAs. | QoS STAs |
| 6.1.1.2 | 54 | 96 | 27 | The integer values of the priority parameter (i.e., TID) are supported only at QSTAs that are either associated in a QBSS or members of a QIBSS. | QoS STAs |
| 6.1.1.2 | 54 | 96 | 27 | The integer values of the priority parameter (i.e., TID) are supported only at QSTAs that are either associated in a QBSS or members of a QIBSS. | QoS BSS … QoS IBSS. |
| 6.1.1.2 | 54 | 96 | 28 | A range of 0 through 15 is supported by QSTAs associated in a QBSS; whereas a range of 0 through 7 is supported by QSTAs that are members of a QIBSS. | QoS STAs … QoS BSS |
| 6.1.1.2 | 54 | 96 | 28 | A range of 0 through 15 is supported by QSTAs associated in a QBSS; whereas a range of 0 through 7 is supported by QSTAs that are members of a QIBSS. | QoS STAs … QoS IBSS. |
| 6.1.1.2 | 54 | 96 | 29 | If a QSTA is associated in a nQBSS, the QSTA is functioning as an nQSTA, so the priority value is always Contention or ContentionFree. | If a QoS STA is associated in a non-QoS BSS, the STA is functioning as a non-QoS STA, |
| 6.1.1.2 | 54 | 96 | 32 | At QSTAs associated in a QBSS, MDSUs with a priority of Contention are considered equivalent to MSDUs with TID 0, and those with a priority of ContentionFree are delivered using the contention-free delivery if a point coordinator (PC) is present in the QAP. | At QoS STAs associated in a QoS BSS, |
| 6.1.1.2 | 54 | 96 | 32 | At QSTAs associated in a QBSS, MDSUs with a priority of Contention are considered equivalent to MSDUs with TID 0, and those with a priority of ContentionFree are delivered using the contention-free delivery if a point coordinator (PC) is present in the QAP. | AP |
| 6.1.1.2 | 54 | 96 | 35 | At QSTAs associated in an nQBSS, all MSDUs with an integer priority are considered equivalent to MSDUs with a priority of Contention. | At STAs associated in a non-QoS BSS, |
| 6.1.1.2 | 54 | 96 | 38 | If a QSTA is associated in a QBSS, the MSDUs it receives in a QoS data frame are reported with the TID value contained in the MAC header of that frame. | If a STA is associated in a QoS BSS, |
| 6.1.1.2 | 54 | 96 | 39 | The MSDUs such a QSTA receives in non-QoS data frames are reported to LLC with a priority of Contention, if they are received during the CP, or Contention-Free if they are received during the CFP. | The MSDUs such a STA receives… |
| 6.1.1.3 | 54 | 96 | 43 | Interpretation of service class parameter in MAC service primitives in a QSTA | STA |
| 6.1.1.3 | 54 | 96 | 46 | In QSTAs, the value of the service class parameter in the MAC service primitive (see 6.2) may be a non-integer value of QoSAck or QoSNoAck. | QoS STAs |
| 6.1.1.3 | 54 | 96 | 49 | When an MSDU is received from the MAC-SAP and the recipient STA is a QSTA with the service class set to: | QoS STA |
| 6.1.1.3 | 54 | 96 | 55 | If the sender QSTA is a QAP and the frame has a multicast/broadcast DA, then the MSDU is buffered for transmission and is also sent to the DS. | STA |
| 6.1.1.3 | 54 | 96 | 55 | If the sender QSTA is a QAP and the frame has a multicast/broadcast DA, then the MSDU is buffered for transmission and is also sent to the DS. | an AP |
| 6.1.1.3 | 55 | 97 | 4 | When an MSDU is received from the MAC_SAP and the recipient STA is not a QSTA, the MSDU is transmitted using a non-QoS data frame. | QoS STA |
| 6.1.1.3 | 55 | 97 | 8 | When a QoS data frame is received from another QSTA, the service class parameter in MA-UNITDATA.indication primitive is set to: | STA |
| 6.1.1.3 | 55 | 97 | 17 | When a non-QoS data frame is received from an nQSTA, the service class parameter is MA-UNITDATA.indication primitive is set to: | a STA |
| 6.1.1.3 | 55 | 97 | 19 | QoSAck, if the frame is a unicast frame and is acknowledged by the QSTA. | STA |
| 6.1.1.3 | 55 | 97 | 21 | QoSNoAck, if the frame is a broadcast/multicase frame and is not acknowledged by the QSTA. | STA |
| 6.1.1.3 | 55 | 97 | 23 | Note that the broadcast/multicast frames sent by nQSTA are not acknowledged regardless of the service class parameter in MA-UNITDATA.indication primitive. | a non-QoS STA |
| 6.1.3 | 56 | 98 | 6 | In an nQSTA, the MAC does not intentionally record MSDUs except as may be necessary to improve the likelihood of successful delivery based on the current operational (“power management”) mode of the designated recipient STA(s). | a non-QoS STA |
| 6.1.3 | 56 | 98 | 17 | In QSTAs operating in a QBSS, there are two service classes, designated as QoSAck and QoSNoAck (see 6.1.1.3 for more information) | QoS STAs |
| 6.1.3 | 56 | 98 | 17 | In QSTAs operating in a QBSS, there are two service classes, designated as QoSAck and QoSNoAck (see 6.1.1.3 for more information) | BSS |
| 6.1.3 | 56 | 98 | 31 | QSTAs operating in an nQBSS shall follow the reordering rules as defined for an nQSTA. | STAs |
| 6.1.3 | 56 | 98 | 31 | QSTAs operating in an nQBSS shall follow the reordering rules as defined for an nQSTA. | a non-QoS BSS |
| 6.1.3 | 56 | 98 | 31 | QSTAs operating in an nQBSS shall follow the reordering rules as defined for an nQSTA. | a non-QoS STA |
| 6.2.1.3.2 | 60 | 102 | 34 | e) Undeliverable: unsupported prioirity (for priorities other than Contention or ContentionFree at an nQSTA; or for priorities other than Contention, ContentionFree, or an integer between and including 0 and 15 at a QSTA); | a non-QoS STA |
| 6.2.1.3.2 | 60 | 102 | 34 | e) Undeliverable: unsupported prioirity (for priorities other than Contention or ContentionFree at an nQSTA; or for priorities other than Contention, ContentionFree, or an integer between and including 0 and 15 at a QSTA); | QoS STA |
| 6.2.1.3.2 | 60 | 102 | 37 | f) Underliverable: unsupported service class (for service classes other than ReorderableMulticast or StrictlyOrdered for nQSTAs and service classes other than QoSAck or QoSNoAck for QSTAs); | non-QoS STAs |
| 6.2.1.3.2 | 60 | 102 | 37 | f) Underliverable: unsupported service class (for service classes other than ReorderableMulticast or StrictlyOrdered for nQSTAs and service classes other than QoSAck or QoSNoAck for QSTAs); | QoS STAs |
| 6.2.1.3.2 | 60 | 102 | 41 | g) Unavailable priority (for ContentionFree when no PC or HC is available, or an integer between and including 1 and 15 at a QSTA that is associated in an nQBSS, or an integer between and including 8 and 15 at a QSTA that is a member of an IBSS, in which case the MSDU is transmitted with a profvded priority of Contention); | STA |
| 6.2.1.3.2 | 60 | 102 | 41 | g) Unavailable priority (for ContentionFree when no PC or HC is available, or an integer between and including 1 and 15 at a QSTA that is associated in an nQBSS, or an integer between and including 8 and 15 at a QSTA that is a member of an IBSS, in which case the MSDU is transmitted with a provided priority of Contention); | a non-QoS BSS |
| 6.2.1.3.2 | 60 | 102 | 41 | g) Unavailable priority (for ContentionFree when no PC or HC is available, or an integer between and including 1 and 15 at a QSTA that is associated in an nQBSS, or an integer between and including 8 and 15 at a QSTA that is a member of an IBSS, in which case the MSDU is transmitted with a provided priority of Contention); | STA |
| 6.2.1.3.2 | 60 | 102 | 44 | h) Undeliverable: unavailable service class (for StrictlyOrdered service when the STA’s power management mode is other than “active” for nQSTAs; QSTAs do not return this value as they do not provide the StrictlyOrdered service); | …for non-QoS STAs; QoS STAs do not… |
| 6.2.1.3.2 | 61 | 103 | 9 | In nQSTAs, the value of this parameter is ReorderableMulticast or StrictlyOrdered. In QSTAs, it is QoSAck or QoSNoAck. | non-QoS STAs |
| 6.2.1.3.2 | 61 | 103 | 9 | In nQSTAs, the value of this parameter is ReorderableMulticast or StrictlyOrdered. In QSTAs, it is QoSAck or QoSNoAck. | QoS STAs |
| 7.1.3.1.3 | 66 | 108 | 23 | A data frame direct from one STA to another STA within the same IBSS, or a data frame direct from one non-AP QSTA to another non-AP QSTA within the same QBSS, as well as all management and control frames. | STA |
| 7.1.3.1.3 | 66 | 108 | 23 | A data frame direct from one STA to another STA within the same IBSS, or a data frame direct from one non-AP QSTA to another non-AP QSTA within the same QBSS, as well as all management and control frames. | STA |
| 7.1.3.1.3 | 66 | 108 | 23 | A data frame direct from one STA to another STA within the same IBSS, or a data frame direct from one non-AP QSTA to another non-AP QSTA within the same QBSS, as well as all management and control frames. | BSS |
| 7.1.3.1.8 | 67 | 109 | 23 | For a non-AP QSTA that has the More Data Ack subfield set in its QoS Capability information element and also has APSD enabled, a QAP may set the More Data field to 1 in ACK frames to this non-AP QSTA to indicate that the QAP has a pending transmission for the non-AP QSTA. | STA (3 times) |
| 7.1.3.1.8 | 67 | 109 | 23 | For a non-AP QSTA that has the More Data Ack subfield set in its QoS Capability information element and also has APSD enabled, a QAP may set the More Data field to 1 in ACK frames to this non-AP QSTA to indicate that the QAP has a pending transmission for the non-AP QSTA. | an AP |
| 7.1.3.1.8 | 67 | 109 | 23 | For a non-AP QSTA that has the More Data Ack subfield set in its QoS Capability information element and also has APSD enabled, a QAP may set the More Data field to 1 in ACK frames to this non-AP QSTA to indicate that the QAP has a pending transmission for the non-AP QSTA. | AP |
| 7.1.3.1.10 | 67 | 109 | 43 | All QSTAs set this subfield to 0. | QoS STAs |
| 7.1.3.2 | 67 | 109 | 53 | In frames transmitted by the PC and nQSTAs during the CFP, the Duration/ID field is set to a fixed value of 32768. | non-QoS STAs |
| 7.1.3.2 | 68 | 110 | 1 | c) In all other frames sent by nQSTAs and control frames sent by QSTAs, the Duration/ID field contains a duration value as defined for each frame type in 7.2. | non-QoS STAs |
| 7.1.3.2 | 68 | 110 | 1 | c) In all other frames sent by nQSTAs and control frames sent by QSTAs, the Duration/ID field contains a duration value as defined for each frame type in 7.2. | QoS STAs |
| 7.1.3.2 | 68 | 110 | 3 | d) In data and management frames sent by QSTAs, the Duration/ID field contains a duration value as defined for each frame type in 7.1.4. | QoS STAs |
| 7.1.3.4.1 | 70 | 112 | 23 | nQSTAs as well as QSTAs operating as nQSTAs because they are in an nQBSS or nQIBSS, assign sequence numbers to management frames and data frames (QoS subfield of the Subtype field is set to 0), from a single modulo-4096 counter, starting at 0 and incrementing by 1 for each MSDU or MMPDU. | Non-QoS STAs, as well as QoS STAs operating as non-QoS STAs… |
| 7.1.3.4.1 | 70 | 112 | 23 | nQSTAs as well as QSTAs operating as nQSTAs because they are in an nQBSS or nQIBSS, assign sequence numbers to management frames and data frames (QoS subfield of the Subtype field is set to 0), from a single modulo-4096 counter, starting at 0 and incrementing by 1 for each MSDU or MMPDU. | a non-QoS BSS or non-QoS IBSS, |
| 7.1.3.4.1 | 70 | 112 | 27 | QSTAs associated in a QBSS maintain one modulo-4096 counter, per TID, per unique receiver (specified by the Address 1 field of the MAC header). | QoS STAs associated in a QoS BSS … |
| 7.1.3.4.1 | 70 | 112 | 31 | Sequence numbers for management frames, QoS data frames with a broadcast/multicast address in the Address 1 field, and all non-QoS data frames sent by QSTAs are assigned using an additional single modulo-4096 counter, starting at 0 and incrementing by 1 for each MSDU or MMPDU. | QoS STAs |
|  |  |  |  | Note: 7.1.3.5 is the QoS Control field, which is only present in frames that have the QoS bit in the Subtype set to 1, which are only sent by/to QoS STAs and QoS APs. Therefore use of the “Q” designation in this subclause is redundant. |  |
| 7.1.3.5 | 70 | 112 | 52 | Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP QSTA) and frame type and subtype. | STA |
|  | 71 | 113 | 44 | The requirement to respond to that TID is nonbinding, and a QSTA may respond with any frame. | STA |
| 7.1.3.5.4 | 72 | 114 | 44 | The TXOP Limit subfield is an 8-bit field that is present in QoS data frames of subtypes that indlude CF-Poll and specifies the time limit on a TXOP granted by a QoS (+)CF-Poll frame from an HC in a QBSS. | BSS |
| 7.1.3.5.4 | 72 | 114 | 45 | In QoS data frames with subtypes that include CF-Poll, the addressed QSTA is granted a TXOP that begins a SIFS period after this frame and lasts no longer than the number of 32 us periods specified by the TXOP limit value. | STA |
| 7.1.3.5.4 | 72 | 114 | 49 | The TXOP limit is inclusive of the PHY and IFS overhead, and a QAP should account for the overhead when granting TXOPs. | an AP |
| 7.1.3.5.5 | 72 | 114 | 54 | The Queue Size subfield is an 8-bit field that indicates the amount of buffered traffic for a given TC or TS at the non-AP QSTA sending this frame. | STA |
| 7.1.3.5.7 | 73 | 115 | 44 | QAP PS Buffer State subfield | AP |
| 7.1.3.5.5 | 73 | 115 | 1 | The Queue Size subfield is present in QoS data frames sent by STAs associated in a QBSS with bit 4 of the QoS Control field set to 1. | BSS |
| 7.1.3.5.5 | 73 | 115 | 2 | The QAP may use information contained in the Queue Size subfield to determine the TXOP duration assigned to non-AP QSTA. | AP |
| 7.1.3.5.5 | 73 | 115 | 2 | The QAP may use information contained in the Queue Size subfield to determine the TXOP duration assigned to non-AP QSTA. | STA |
| 7.1.3.5.5 | 73 | 115 | 4 | The queue size value is the total size, rounded up to the nearest multiple of 256 octets and expressed in units of 256 octets, of all MSDUs buffered at the QSTA (excluding the MSDU of the present QoS data frame) in the delivery queue used for MSDUs with TID value equal to the value in the TID subfield of this QoS Control field. | STA |
| 7.1.3.5.6 | 73 | 115 | 20 | The TXOP Duration Request subfield is present in QoS data frames sent by non-AP QSTAs associated in a QBSS with bit 4 of the QoS Control field set to 0. | STAs |
| 7.1.3.5.6 | 73 | 115 | 20 | The TXOP Duration Request subfield is present in QoS data frames sent by non-AP QSTAs associated in a QBSS with bit 4 of the QoS Control field set to 0. | BSS |
| 7.1.3.5.6 | 73 | 115 | 21 | The QAP may choose to assign a TXOP duration shorter than that requested in the TXOP Duration Requested subfield. | AP |
| 7.1.3.5.6 | 73 | 115 | 29 | The TXOP duration requested is inclusive of the PHY and IFS overhead, and a QSTA should account for this when attempting to determine whether a given transmission fits within a specified TXOP duration. | STA |
| 7.1.3.5.7 | 73 | 115 | 32 | QAP PS Buffer State subfield | AP |
| 7.1.3.5.7 | 73 | 115 | 34 | The QAP PS Buffer State subfield, defined in Figure 21, is an 8-bit field that indicates the PS buffer state at the QAP for a non-AP QSTA. | AP |
| 7.1.3.5.7 | 73 | 115 | 34 | The QAP PS Buffer State subfield, defined in Figure 21, is an 8-bit field that indicates the PS buffer state at the QAP for a non-AP QSTA. | AP |
| 7.1.3.5.7 | 73 | 115 | 34 | The QAP PS Buffer State subfield, defined in Figure 21, is an 8-bit field that indicates the PS buffer state at the QAP for a non-AP QSTA. | STA |
| 7.1.3.5.7 | 73 | 115 | 35 | The QAP PS Buffer State subfield is further subdivided into three subfields: Buffer State Indicated, Highest-Prioirty Buffered AC, and QAP Buffered Load. | AP |
| 7.1.3.5.7 | 73 | 115 | 35 | The QAP PS Buffer State subfield is further subdivided into three subfields: Buffer State Indicated, Highest-Prioirty Buffered AC, and QAP Buffered Load. | AP |
| 7.1.3.5.7 | 73 | 115 | 50 | The Buffered State Indicated subfield is 1 bit in length and is used to indicate whether the QAP PS buffer state is specified. | AP |
| 7.1.3.5.7 | 73 | 115 | 51 | A value of 1 indicates that the QAP PS buffer state is specified. | AP |
| 7.1.3.5.7 | 73 | 115 | 55 | The Highest-Priority Buffered AC subfield is 8-bits in length and is used to indicate the AC of the highest priority traffic remaining that is buffered at the QAP, excluding the MSDU of the present frame. | AP |
| 7.1.3.5.7 | 74 | 116 | 1 | The QAP Buffered Load subfield is 4 bits in length and is used to indicate the total buffer size, rounded up to the nearest multiple of 4096 octets and expressed in units of 4096 octets, of all MSDUs buffered at the QAP (excluding the MSDU of the present QoS data frame). | AP |
| 7.1.3.5.7 | 74 | 116 | 1 | The QAP Buffered Load subfield is 4 bits in length and is used to indicate the total buffer size, rounded up to the nearest multiple of 4096 octets and expressed in units of 4096 octets, of all MSDUs buffered at the QAP (excluding the MSDU of the present QoS data frame). | AP |
| 7.1.3.5.7 | 74 | 116 | 3 | A QAP Buffered Load field value of 15 indicates that the buffer size is greater than 57344 octets. | An AP |
| 7.1.3.5.7 | 74 | 116 | 4 | A QAP Buffered Load subfield value of 0 is used solely to indicate the absence of any buffered traffic for the indicated highest priority buffered AC when the Buffer State Indicated bit is 1. | An AP |
| 7.1.3.5.7 | 74 | 116 | 9 | When the Buffered State Indicated subfield is set to 0, the Highest-Priority Buffered AC subfield and the QAP Buffered Load subfield are reserved; and the values of these subfields are either unspecified or unknown. | AP |
| 7.1.4 | 75 | 117 | 1 | Within all data or management frames sent in a CP by the QSTAs outside of a controlled access phase (CAP), following a contention access of the channel, the Duration/ID field is set to one of the following values: | QoS STAs |
| 7.1.4 | 75 | 117 | 22 | 2) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit (dot11EDCAQAPTableTXOPLimit for the QAP) for that AC minus the already used time within the TXOP. | AP |
| 7.2.1.1 | 76 | 118 | 19 | For all RTS frames sent by nQSTAs, the duration value is the time, in microseconds, required to transmit the pending data or management frame, plus one CTS frame, plus one ACK frame, plus three SIFS intervals. | non-QoS STAs |
| 7.2.1.1 | 76 | 118 | 23 | For all RTS frames sent by QSTAs under EDCA, following a contention access of the channel, the duration value is set in the following manner: | STAs |
| 7.2.1.2 | 77 | 119 | 6 | At an nQSTA, if the CTS is the first frame in the exchange and the pending data or management frame requires acknowledgment, the duration value is the time, in microseconds, required to transmit the pending data or management frame, plus two SIFS intervals plus one ACK frame. | a non-QoS STA |
| 7.2.1.2 | 77 | 119 | 8 | At an nQSTA, if the CTS is the first frame in the exchange and the pending data or management frame does not require acknowledgment, the duration value is the time, in microseconds, required to transmit the pending data or management frame, plus one SIFS interval. | a non-QoS STA |
| 7.2.1.2 | 77 | 119 | 14 | For all CTS frames sent by QSTAs as the first frame in the exchange under EDCA, the duration value is set in the following manner: | STAs |
| 7.2.1.3 | 77 | 119 | 53 | For ACK frames sent by nQSTAs, if the More Fragments bit was set to 0 in the Frame Control field of the immediately previous directed data or management frame, the duration value is set to 0. | non-QoS STAs |
|  |  |  |  | Note: Block Ack (7.2.1.7 and 7.2.1.8) is advertised by a separate capability bit; use of QSTA/QAP is redundant. |  |
| 7.2.1.7 | 79 | 121 | 23 | The RA field of the BlockAckReq frame is the address of the recipient QSTA. | STA |
| 7.2.1.7 | 79 | 121 | 26 | The TA field is the address of the QSTA transmitting the BlockAckReq frame. | STA |
| 7.2.1.8 | 80 | 122 | 20 | The RA field of the BlockAck frame is the address of the recipient QSTA that requested the Block Ack. | STA |
| 7.2.1.8 | 80 | 122 | 23 | The TA field is the address of the QSTA transmitting the BlockAck frame. | STA |
| 7.2.2 | 81 | 123 | 20 | A QSTA always uses QoS data frames for data transmissions to other QSTAs. | QoS STA |
| 7.2.2 | 81 | 123 | 20 | A QSTA always uses QoS data frames for data transmissions to other QSTAs. | QoS STAs |
| 7.2.2 | 81 | 123 | 20 | A QSTA uses frames with the QoS subfield of the Subtype field set to 0 for data transmissions to nQSTAs. | QoS STA |
| 7.2.2 | 81 | 123 | 20 | A QSTA uses frames with the QoS subfield of the Subtype field set to 0 for data transmissions to nQSTAs. | non-QoS STAs |
| 7.2.2 | 81 | 123 | 21 | An nQSTA always uses frames with the QoS subfield of the Subtype field set to 0 for data transmissions to other STAs. | A non-QoS STA |
| 7.2.2 | 81 | 123 | 23 | All STAs use frames with the QoS subfield of the Subtype field set to 0 for broadcast data frames unless a transmitting QSTA knows that all STAs in a QBSS have QoS capability, in which case the transmitting QSTAs uses QoS data frames. | STA |
| 7.2.2 | 81 | 123 | 23 | All STAs use frames with the QoS subfield of the Subtype field set to 0 for broadcast data frames unless a transmitting QSTA knows that all STAs in a QBSS have QoS capability, in which case the transmitting QSTAs uses QoS data frames. | BSS |
| 7.2.2 | 81 | 123 | 23 | All STAs use frames with the QoS subfield of the Subtype field set to 0 for broadcast data frames unless a transmitting QSTA knows that all STAs in a QBSS have QoS capability, in which case the transmitting QSTAs uses QoS data frames. | STA |
| 7.2.2 | 81 | 123 | 26 | All STAs use frames with the QoS subfield of the Subtype field set to 0 for multicast data frames unless it is known to the transmitter that all STAs in the QBSS that are members of the multicast group have QoS capability, in which case QSTAs use QoS data frames. | BSS |
| 7.2.2 | 81 | 123 | 26 | All STAs use frames with the QoS subfield of the Subtype field set to 0 for multicast data frames unless it is known to the transmitter that all STAs in the QBSS that are members of the multicast group have QoS capability, in which case QSTAs use QoS data frames. | STAs |
| 7.2.2 | 82 | 124 | 41 | Within all data frames sent by the QSTA, the Duration/ID field contains a duration value as defined in 7.1.4. | QoS STA |
| 7.2.2 | 82 | 124 | 42 | Within all data frames sent during the CP by nQSTAs, the Duration/ID field is set according to the following rules: | non-QoS STAs |
| 7.2.3 | 83 | 125 | 41 | Within all management frames sent by the QSTA, the Duration field contains a duration value as defined in 7.1.4. | QoS STA |
| 7.2.3 | 83 | 125 | 42 | Within all management frames sent during the CP by nQSTAs, the Duration field is set according to the following rules: | non-QoS STAs |
| 7.2.3.1 | 85 | 127 | 29 | QBSS Load | BSS |
| 7.2.3.1 | 85 | 127 | 29 | The QBSS Load information element is only present within Beacon frames generated by QAPs. The QBSS Load element is present when dot11QosOption-Implemented and dot11QBSSLoadImplemented are both true. | BSS |
| 7.2.3.1 | 85 | 127 | 29 | The QBSS Load information element is only present within Beacon frames generated by QAPs. The QBSS Load element is present when dot11QosOption-Implemented and dot11QBSSLoadImplemented are both true. | Delete, second sentence sufficient. |
| 7.2.3.1 | 85 | 127 | 29 | The QBSS Load information element is only present within Beacon frames generated by QAPs. The QBSS Load element is present when dot11QosOption-Implemented and dot11QBSSLoadImplemented are both true. | BSS |
| 7.2.3.1 | 85 | 127 | 32 | The EDCA Parameter Set information element is only present within Beacon frames generated by QAPs. The EDCA Parameter Set element is present when dot11QosOptionImplemented is true and the QoS Capability element is not present. | Delete, second sentence sufficient. |
| 7.2.3.9 | 90 | 132 | 5 | QBSS Load | BSS |
| 7.2.3.9 | 90 | 132 | 4 | The QBSS Load element is present when dot11QosOptionImplemented and dot11QBSSLoadImplemented are both true. The QBSS Load information element is always present within Probe Response frames generated by QAPs when dot11QBSSLoadImplemented is true. | Delete, first sentence sufficient |
| 7.2.3.9 | 90 | 132 | 11 | The EDCA Parameter Set element is present when dot11QosOptionImplemented is true. The EDCA Parameter Set information element is always present within Probe Response frames generated by QAPs. | Delete, first sentence sufficient |
| 7.3.1.4 | 93 | 135 | 30 | QSTA requesting association in a QBSS | QoS STA |
| 7.3.1.4 | 93 | 135 | 30 | QSTA requesting association in a QBSS | QoS BSS |
| 7.3.1.4 | 93 | 135 | 39 | An nQAP sets the CF-Pollable and CF-Poll Request subfield values in Association Response and Reassociation Response management frames equal to the values in the last Beacon or Probe Response frame that it transmitted. | A non-QoS AP |
| 7.3.1.4 | 93 | 135 | 48 | No PC at nQAP | non-QoS AP |
| 7.3.1.4 | 93 | 135 | 49 | PC at nQAP for delivery only (no polling) | non-QoS AP |
| 7.3.1.4 | 93 | 135 | 50 | PC at nQAP for delivery and polling | non-QoS AP |
| 7.3.1.4 | 93 | 135 | 53 | QAP (HC) does not use CFP for delivery of unicast data frames | QoS AP |
| 7.3.1.4 | 93 | 135 | 54 | QAP (HC) uses CFP for delivery, but does not send CF-Polls to nQSTAs | QoS AP |
| 7.3.1.4 | 93 | 135 | 54 | QAP (HC) uses CFP for delivery, but does not send CF-Polls to nQSTAs | non-QoS STAs |
| 7.3.1.4 | 94 | 136 | 3 | QAP (HC) uses CFP for delivery, and sends CF-Polls to nQSTAs | QoS AP |
| 7.3.1.4 | 94 | 136 | 3 | QAP (HC) uses CFP for delivery, and sends CF-Polls to nQSTAs | non-QoS STAs |
| 7.3.1.4 | 94 | 136 | 16 | QSTAs within an ESS set the Privacy subfield to 1 in DLS Request and DLS Response frames if encryption is required for all data frames exchanged. | STAs |
| 7.3.1.4 | 95 | 137 | 28 | QAPs set the APSD subfield to 1 within the Capability Information field when the MIB attribute dot11APSDOptionImplemented is true and set it to 0 otherwise. | APs |
| 7.3.1.4 | 95 | 137 | 48 | QSTAs set the Delayed Block Ack subfield to 1 within the Capability Information field when the MIB attribute dot11DelayedBlockAckOptionImplemented is true and set it to 0 otherwise. | STAs |
| 7.3.1.4 | 95 | 137 | 51 | QSTAs set the Immediate Block Ack subfield to 1 within the Capability Information field when the MIB attribute dot11ImmediateBlockAckOptionImplemented is true and set it to 0 otherwise. | STAs |
| 7.3.1.7 | 97 | 139 | 40 | Disassociated because QAP lacks sufficient bandwidth for this QSTA | QoS AP |
| 7.3.1.7 | 97 | 139 | 40 | Disassociated because QAP lacks sufficient bandwidth for this QSTA | QoS STA |
| 7.3.1.7 | 97 | 139 | 44 | Disassociated because QSTA is transmitting outside the limits of its TXOPs | STA |
| 7.3.1.7 | 97 | 139 | 45 | Requested from peer QSTA as the QSTA is leaving the QBSS (or resetting) | STA |
| 7.3.1.7 | 97 | 139 | 45 | Requested from peer QSTA as the QSTA is leaving the QBSS (or resetting) | STA |
| 7.3.1.7 | 97 | 139 | 45 | Requested from peer QSTA as the QSTA is leaving the QBSS (or resetting) | BSS |
| 7.3.1.7 | 97 | 139 | 46 | Requested from peer QSTA as it does not want to use the mechanism | STA |
| 7.3.1.7 | 97 | 139 | 47 | Requested from peer QSTA as the QSTA received frames using the mechanism for which a setup is required | STA |
| 7.3.1.7 | 97 | 139 | 47 | Requested from peer QSTA as the QSTA received frames using the mechanism for which a setup is required | STA |
| 7.3.1.7 | 97 | 139 | 49 | Requested from peer QSTA due to timeout | STA |
| 7.3.1.7 | 97 | 139 | 51 | Peer QSTA does not support the requested cipher suite | STA |
| 7.3.1.9 | 99 | 141 | 21 | Association denied because QAP has insufficient bandwidth to handle another QSTA | QoS AP |
| 7.3.1.9 | 99 | 141 | 21 | Association denied because QAP has insufficient bandwidth to handle another QSTA | QoS STA |
| 7.3.1.9 | 99 | 141 | 25 | Association (with QBSS) denied because the requesting STA does not support the QoS facility | QoS BSS |
| 7.3.1.9 | 99 | 141 | 31 | The TS has not been created because the request cannot be honored; however, a suggested TSPEC is provided so that the initiating QSTA may attempt to set another TS with the suggested changes to the TSPEC | STA |
| 7.3.1.9 | 99 | 141 | 46 | The Destination STA is not present within this QBSS | BSS |
| 7.3.1.9 | 99 | 141 | 47 | The Destination STA is not a QSTA | QoS STA |
|  |  |  |  | Note: QoS Info field (7.3.1.17) only appears in frames sent by QSTAs/QAPs. Use of “Q” is redundant in this subclause |  |
| 7.3.1.17 | 103 | 145 | 14 | QoS Info field when sent by a QAP | an AP |
| 7.3.1.17 | 103 | 145 | 37 | QoS Info field when set by a non-AP QSTA | STA |
| 7.3.1.17 | 103 | 145 | 3 | The contents of the field are dependent on whether the QSTA is a QAP or a non-AP QSTA. | STA |
| 7.3.1.17 | 103 | 145 | 3 | The contents of the field are dependent on whether the QSTA is a QAP or a non-AP QSTA. | an AP |
| 7.3.1.17 | 103 | 145 | 3 | The contents of the field are dependent on whether the QSTA is a QAP or a non-AP QSTA. | STA |
| 7.3.1.17 | 103 | 145 | 7 | The format of the QoS Info field, when sent by the QAP, is defined in Figure 52. | AP |
| 7.3.1.17 | 103 | 145 | 16 | In the Beacon frame, the EDCA Parameter Set Update Count subfield is initially set by the QAP to 0 and is incremented every time any of the AC parameters changes. | AP |
| 7.3.1.17 | 103 | 145 | 19 | QAPs set the Q-Ack subfield to 1 when the MIB attribute dot11QAckOptionImplemented is true and set it to 0 otherwise. | APs |
| 7.3.1.17 | 103 | 145 | 22 | QAPs set the Queue Request subfield to 1 if they can process a nonzero Queue Size subfield in the QoS Control field in QoS data frames and set it to 0 otherwise. | APs |
| 7.3.1.17 | 103 | 145 | 26 | QAPs set the TXOP Request subfield to 1 if they can process a nonzero TXOP Duration Requested subfield in the QoS Control field in QoS data frames and set it to 0 otherwise. | APs |
| 7.3.1.17 | 103 | 145 | 29 | The format of the QoS Info field, when sent by the non-AP QSTA, is defined in Figure 53. | STA |
| 7.3.1.17 | 103 | 145 | 46 | Non-AP QSTAs set the Q-Ack subfield to 1 when the MIB attribute dot11QAckOptionImplemented is true and set it to 0 otherwise. | STAs |
| 7.3.1.17 | 103 | 145 | 49 | The Max SP Length subfield is 2 bits in length and indicates the maximum number of total buffered MSDUs and MMPDUs the QAP may deliver to a non-AP QSTA during any SP triggered by the non-AP QSTA. | AP |
| 7.3.1.17 | 103 | 145 | 49 | The Max SP Length subfield is 2 bits in length and indicates the maximum number of total buffered MSDUs and MMPDUs the QAP may deliver to a non-AP QSTA during any SP triggered by the non-AP QSTA. | STA (twice) |
| 7.3.1.17 | 104 | 146 | 5 | QAP may deliver all buffered MSDUs and MMPDUs. | AP |
| 7.3.1.17 | 104 | 146 | 6 | QAP may deliver a maximum of two MSDUs and MMPDUs per SP. | AP |
| 7.3.1.17 | 104 | 146 | 7 | QAP may deliver a maximum of four MSDUs and MMPDUs per SP. | AP |
| 7.3.1.17 | 104 | 146 | 8 | QAP may deliver a maximum of six MSDUs and MMPDUs per SP. | AP |
| 7.3.1.17 | 104 | 146 | 13 | Non-AP QSTAs set the More Data Ack subfield to 1 to indicate that they can process Ack frames with the More Data bit in the Frame Control field set to 1 and will remain in the awake state. | STAs |
| 7.3.1.17 | 104 | 146 | 14 | Non-AP QSTAs set the More Data Ack subfield to 0 otherwise. | STAs |
| 7.3.1.17 | 104 | 146 | 15 | For QAPs, the More Data Ack subfield is reserved. | APs |
| 7.3.2 | 104 | 146 | 53 | QBSS Load | BSS |
|  |  |  |  | Note: QBSS Load element (7.3.2.28), EDCA parameter set element (7.3.2.29), TSPEC element (7.3.2.30), TCLAS element (7.3.2.31), TS Delay element (7.3.2.32), TCLAS Processing (7.3.2.33), and Schedule element (7.3.2.34): All are sent by/to QSTAs/QAPs, and further use of “Q” is redundant. |  |
| 7.3.2.28 | 134 | 176 | 40 | QBSS Load element format | BSS |
| 7.3.2.28 | 134 | 176 | 28 | QBSS Load element | BSS |
| 7.3.2.28 | 134 | 176 | 30 | The QBSS Load element contains information on the current STA population and traffic levels in the QBSS. | BSS |
| 7.3.2.28 | 134 | 176 | 30 | The QBSS Load element contains information on the current STA population and traffic levels in the QBSS. | BSS |
| 7.3.2.28 | 134 | 176 | 31 | This element may be used by the non-AP QSTA for vendor-specific AP selection algorithm when roaming. | STA |
| 7.3.2.28 | 134 | 176 | 39 | The STA Count field is interpreted as an unsigned integer that indicates the total number of STAs currently associated with this QBSS. | BSS |
| 7.3.2.28 | 134 | 176 | 45 | The Channel Utilization field is defined as the percentage of time, normalized to 255, the QAP sensed the medium was busy, as indicated by either the physical or virtual carrier sense (CS) mechanism. | AP |
| 7.3.2.28 | 135 | 177 | 2 | The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests. | STAs |
| 7.3.2.28 | 135 | 177 | 2 | The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests. | an AP |
| 7.3.2.29 | 135 | 177 | 9 | The EDCA Parameter Set element provides information needed by non-AP QSTAs for proper operation of the QoS facility during the CP. | STAs |
| 7.3.2.29 | 135 | 177 | 18 | The EDCA Parameter Set element is used by the QAP to establish policy (by changing default MIB attribute values), to change policies when accepting new STAs or new traffic, or to adapt to changes in offered load. | AP |
| 7.3.2.29 | 135 | 177 | 21 | The most recent EDCA parameter set element received by a non-AP QSTA is used to update the appropriate MIB values. | STA |
| 7.3.2.29 | 135 | 177 | 26 | This subfield is used by non-AP QSTAs to determine whether the EDCA parameter set has changed and requires updating the appropriate MIB attributes. | STAs |
| 7.3.2.29 | 135 | 177 | 52 | The AIFSN subfield indicates the number of slots after a SIFS duration a non-AP QSTA should defer before either invoking a backoff or starting a transmission. | STA |
| 7.3.2.29 | 136 | 178 | 37 | The default values used by non-AP QSTAs for the parameters in the EDCA Parameter Set element are defined in Table 37. | STAs |
| 7.3.2.30 | 136 | 178 | 42 | The TSPEC element contains the set of parameters that define the characteristics and QoS expectations of a traffic flow, in the context of a particular non-AP QSTA, for use by the HC and non-AP QSTA(s) in support of QoS traffic transfer using the procedures defined in 11.4. | STA |
| 7.3.2.30 | 136 | 178 | 42 | The TSPEC element contains the set of parameters that define the characteristics and QoS expectations of a traffic flow, in the context of a particular non-AP QSTA, for use by the HC and non-AP QSTA(s) in support of QoS traffic transfer using the procedures defined in 11.4. | STA |
| 7.3.2.30 | 136 | 178 | 49 | Non-AP QSTAs set the value of any parameters to unspecified if they have no information for setting that parameter. | STAs |
| 7.3.2.30 | 136 | 178 | 50 | The HC may change the value of parameters that have been set unspecified by the QSTA to any value that it deems appropriate, including leaving them unspecified | STA |
| 7.3.2.30 | 137 | 179 | 49 | The combination of the TSID and Direction subfields identify the TS, in the context of the non-AP QSTA, to which the TSPEC applies. | STA |
| 7.3.2.30 | 137 | 179 | 51 | The same TSID may be used for multiple TSs at different non-AP QSTAs | STAs |
| 7.3.2.30 | 137 | 179 | 52 | A non-AP QSTA may use the TSID subfield value for a downlink TSPEC and either an uplink or a direct-link TSPEC at the same time. | STA |
| 7.3.2.30 | 137 | 179 | 53 | A non-AP QSTA shall not use the same TSID for both uplink and direct-link TS. | STA |
| 7.3.2.30 | 138 | 180 | 7 | Uplink (MSDUs are sent from the non-AP QSTA to HC) | STA |
| 7.3.2.30 | 138 | 180 | 9 | Downlink (MSDUs are sent from the HC to the non-AP QSTA) | STA |
| 7.3.2.30 | 138 | 180 | 10 | Direct link (MSDUs are sent from the non-AP QSTA to another non-AP QSTA) | STA |
| 7.3.2.30 | 138 | 180 | 10 | Direct link (MSDUs are sent from the non-AP QSTA to another non-AP QSTA) | STA |
| 7.3.2.30 | 138 | 180 | 32 | It is set to 1 by a non-AP QSTA to indicate that an aggregate schedule is required. | STA |
| 7.3.2.30 | 138 | 180 | 34 | It is set to 1 by the QAP if an aggregate schedule is being provided to the non-AP QSTA. | AP |
| 7.3.2.30 | 138 | 180 | 34 | It is set to 1 by the QAP if an aggregate schedule is being provided to the non-AP QSTA. | STA |
| 7.3.2.30 | 138 | 180 | 51 | When the Schedule and APSD subfields are set to 1, the QAP shall set the aggregation bit to 1, indicating that an aggregate schedule is being provided to the non-AP QSTA. | AP |
| 7.3.2.30 | 138 | 180 | 51 | When the Schedule and APSD subfields are set to 1, the QAP shall set the aggregation bit to 1, indicating that an aggregate schedule is being provided to the non-AP QSTA. | STA |
| 7.3.2.30 | 140 | 182 | 9 | The service start time indicates to QAP the time when a non-AP QSTA first expects to be ready to send frames and a power-saving non-AP QSTA will be awake to receive frames. | AP |
| 7.3.2.30 | 140 | 182 | 9 | The service start time indicates to QAP the time when a non-AP QSTA first expects to be ready to send frames and a power-saving non-AP QSTA will be awake to receive frames. | STA |
| 7.3.2.30 | 140 | 182 | 9 | The service start time indicates to QAP the time when a non-AP QSTA first expects to be ready to send frames and a power-saving non-AP QSTA will be awake to receive frames. | STA |
| 7.3.2.30 | 140 | 182 | 11 | This may help the QAP to schedule service so that the MSDUs encounter small delays in the MAC and help the power-saving non-AP QSTAs to reduce power consumption. | AP |
| 7.3.2.30 | 140 | 182 | 11 | This may help the QAP to schedule service so that the MSDUs encounter small delays in the MAC and help the power-saving non-AP QSTAs to reduce power consumption. | STA |
| 7.3.2.30 | 141 | 183 | 15 | The UP, Minimum Data Rate, Mean Data Rate, Peak Data Rate, Burst Size, Minimum PHY Rate, and Delay Bound fields in a TSPEC element express the QoS expectations requested by a non-AP QSTA, if this TSPEC was issued by that non-AP QSTA, or provided by the HC, if this TSPEC was issued by the HC, when these fields are specified with nonzero values. | STA |
| 7.3.2.30 | 141 | 183 | 15 | The UP, Minimum Data Rate, Mean Data Rate, Peak Data Rate, Burst Size, Minimum PHY Rate, and Delay Bound fields in a TSPEC element express the QoS expectations requested by a non-AP QSTA, if this TSPEC was issued by that non-AP QSTA, or provided by the HC, if this TSPEC was issued by the HC, when these fields are specified with nonzero values. | STA |
| 7.3.2.30 | 141 | 183 | 18 | Unspecified parameters in these fields as indicated by a zero value indicate that the non-AP QSTA does not have specific requirements for these parameters if the TSPEC was issued by that non-AP QSTA or that the HC does not provide any specific values for these parameters if the TSPEC was issued by the HC. | STA |
| 7.3.2.30 | 141 | 183 | 18 | Unspecified parameters in these fields as indicated by a zero value indicate that the non-AP QSTA does not have specific requirements for these parameters if the TSPEC was issued by that non-AP QSTA or that the HC does not provide any specific values for these parameters if the TSPEC was issued by the HC. | STA |
| 7.3.2.31 | 141 | 183 | 25 | The TCLAS element specifies an information element that contains a set of parameters necessary to identify incoming MSDUs (from a higher layer in all QSTAs or from the DS in an AP) with a particular TS to which they belong. | STAs |
| 7.3.2.32 | 143 | 185 | 31 | The TS Delay element is used in the ADDTS Response frame transmitted by the HC to a non-AP QSTA and indicates the time after which the ADDTS may be retried. | STA |
| 7.3.2.32 | 143 | 185 | 40 | The Delay field is 4 octets long and specifies the amount of time, in TUs, a non-AP QSTA should wait before it reinitiates setup of a TS. | STA |
| 7.3.2.32 | 143 | 185 | 44 | The TS Delay element is set to 0 when a QAP does not want to serve any TSPECs for an indeterminate time and it does not know this time a priori. | an AP |
| 7.3.2.34 | 144 | 186 | 33 | The Schedule element is transmitted by the HC to a non-AP QSTA to announce the schedule that the HC/QAP follows for admitted streams originating from or destined to that non-AP QSTA in the future. | STA |
| 7.3.2.34 | 144 | 186 | 33 | The Schedule element is transmitted by the HC to a non-AP QSTA to announce the schedule that the HC/QAP follows for admitted streams originating from or destined to that non-AP QSTA in the future. | AP |
| 7.3.2.34 | 144 | 186 | 33 | The Schedule element is transmitted by the HC to a non-AP QSTA to announce the schedule that the HC/QAP follows for admitted streams originating from or destined to that non-AP QSTA in the future. | STA |
| 7.3.2.34 | 144 | 186 | 35 | The information in this element may be used by the non-AP QSTA for power management, internal scheduling, or any other purpose. | STA |
| 7.3.2.34 | 145 | 187 | 11 | The Aggregation subfield is set to 1 if the schedule is an aggregate schedule for all TSIDs associated with the non-AP QSTA to which the frame is directed. | STA |
| 7.3.2.34 | 145 | 187 | 20 | The QAP uses this field to confirm or modify the service start time indicated in the TSPEC request. | AP |
|  |  |  |  | Note: QoS Action frames (7.4.2) are only sent by QSTAs/QAPs; all uses of “Q” are redundant. |  |
| 7.4.2.1 | 149 | 191 | 32 | The TCLAS element is optional at the discretion of the non-AP QSTA that sends the ADDTS Request frame, regardless of the setting of the access policy (EDCA or HCCA). | STA |
| 7.4.2.4 | 150 | 192 | 53 | The Schedule frame is transmitted by the HC to a non-AP QSTA to announce the schedule of delivery of data and polls. | STA |
| 7.4.4.1 | 154 | 196 | 4 | An ADDBA Request frame is sent by an originator of Block Ack to another QSTA. | STA |
| 7.5 | 156 | 198 | 25 | QSTA | STA (3 times) |
| 7.5 | 156 | 198 | 25 | QAP | AP (2 times) |
| 7.5 | 157 | 199 | 11 | QSTA | STA (3 times) |
| 7.5 | 157 | 199 | 11 | QAP | AP (2 times) |
| 7.5 | 157 | 199 | 41 | Tbe frame subtype is transmitted by a QSTA in an IBSS pursuant to receiving directed request. | QoS STA |
| 7.5 | 157 | 199 | 43 | Rda frame subtype is received if QSTA is addressee. | QoS STA |
| 7.5 | 157 | 199 | 46 | Rq frame subtype is received if QSTA is not the addressee, but has set the Q-Ack subfield in QoS Capability element to 1. | QoS STA |
| 9.1 | 258 | 300 | 16 | Note that in an nQSTA, HCF is not present. | a non-QoS STA |
| 9.1 | 258 | 300 | 16 | In a QSTA implementation, both DCF and HCF are present. | QoS STA |
|  |  |  |  | Note: HCF (9.1.3) is only present in QoS networks. Generally all use of “Q” in the QSTA/QAP is redundant. However, various statements in this clause make statements about required behavior, and these are left as QSTA/QAP-specific. |  |
| 9.1.3 | 259 | 301 | 22 | The QoS facility includes an additional coordination function called HCF that is only usable in QoS network (QBSS) configurations. | Delete |
| 9.1.3 | 259 | 301 | 23 | The HCF shall be implemented in all QSTAs. | QoS STAs |
| 9.1.3 | 259 | 301 | 30 | QSTAs may obtain TXOPs using one or both of the channel access mechanisms specified in 9.9. | STAs |
| 9.1.3.1 | 259 | 301 | 37 | The EDCA mechanism provides differentiated, distributed access to the WM for QSTAs using eight different UPs. | STAs |
| 9.1.3.1 | 259 | 301 | 38 | The EDCA mechanism defines four access categories (ACs) that provide support for the delivery of traffic with UPs at the QSTAs. | STAs |
| 9.1.3.1 | 259 | 301 | 41 | For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters from the EDCA Parameter Set element or from the default values for the parameters when no EDCA Parameter Set element is received from the QAP of the QBSS with which the QSTA is associated, where | AP |
| 9.1.3.1 | 259 | 301 | 41 | For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters from the EDCA Parameter Set element or from the default values for the parameters when no EDCA Parameter Set element is received from the QAP of the QBSS with which the QSTA is associated, where | BSS |
| 9.1.3.1 | 259 | 301 | 41 | For each AC, an enhanced variant of the DCF, called an enhanced distributed channel access function (EDCAF), contends for TXOPs using a set of EDCA parameters from the EDCA Parameter Set element or from the default values for the parameters when no EDCA Parameter Set element is received from the QAP of the QBSS with which the QSTA is associated, where | STA |
| 9.1.3.1 | 259 | 301 | 46 | a) The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the QAP and by MIB attribute table dot11EDCATable at the non-AP QSTA. | AO |
| 9.1.3.1 | 259 | 301 | 46 | a) The parameters used by the EDCAF to control its operation are defined by MIB attribute table dot11QAPEDCATable at the QAP and by MIB attribute table dot11EDCATable at the non-AP QSTA. | STA |
| 9.1.3.1 | 259 | 301 | 49 | b) The minimum specified idle duration time is not the constant value (DIFS) as defined for DCF, but is a distinct value (contained in the MIB attribute table dot11QAPEDCATableAIFSN for a QAP and in the MIB table dot11EDCATableAIFSN for a non-AP QSTA; see 9.9.1) assigned either by a management entity or by a QAP. | an AP |
| 9.1.3.1 | 259 | 301 | 49 | b) The minimum specified idle duration time is not the constant value (DIFS) as defined for DCF, but is a distinct value (contained in the MIB attribute table dot11QAPEDCATableAIFSN for a QAP and in the MIB table dot11EDCATableAIFSN for a non-AP QSTA; see 9.9.1) assigned either by a management entity or by a QAP. | STA |
| 9.1.3.1 | 259 | 301 | 49 | b) The minimum specified idle duration time is not the constant value (DIFS) as defined for DCF, but is a distinct value (contained in the MIB attribute table dot11QAPEDCATableAIFSN for a QAP and in the MIB table dot11EDCATableAIFSN for a non-AP QSTA; see 9.9.1) assigned either by a management entity or by a QAP. | an AP |
| 9.1.3.1 | 259 | 301 | 53 | c) The contention window limits aCWmin and aCWmax, from which the random backoff is computed, are not fixed per PHY, as with DCF, but are variable (contained in the MIB attribute tables dot11QAPEDCACWmin and dot11QAPEDCACWmax for a QAP and in the MIB attribute tables dot11EDCATableCWmin and dot11EDCATableCWmax for a non-AP QSTA) and assigned by a management entity or by a QAP. | an AP |
| 9.1.3.1 | 259 | 301 | 53 | c) The contention window limits aCWmin and aCWmax, from which the random backoff is computed, are not fixed per PHY, as with DCF, but are variable (contained in the MIB attribute tables dot11QAPEDCACWmin and dot11QAPEDCACWmax for a QAP and in the MIB attribute tables dot11EDCATableCWmin and dot11EDCATableCWmax for a non-AP QSTA) and assigned by a management entity or by a QAP. | STA |
| 9.1.3.1 | 259 | 301 | 53 | c) The contention window limits aCWmin and aCWmax, from which the random backoff is computed, are not fixed per PHY, as with DCF, but are variable (contained in the MIB attribute tables dot11QAPEDCACWmin and dot11QAPEDCACWmax for a QAP and in the MIB attribute tables dot11EDCATableCWmin and dot11EDCATableCWmax for a non-AP QSTA) and assigned by a management entity or by a QAP. | an AP |
| 9.1.3.1 | 260 | 302 | 25 | d) Collisions between contending EDCAFs within a QSTA are resolved within the QSTA so that the data frames from the higher priority AC receives the TXOP and the data frames from the lower priority colliding AC(s) behave as if there were an external collision on the WM. | STA |
| 9.1.3.1 | 260 | 302 | 25 | d) Collisions between contending EDCAFs within a QSTA are resolved within the QSTA so that the data frames from the higher priority AC receives the TXOP and the data frames from the lower priority colliding AC(s) behave as if there were an external collision on the WM. | STA |
| 9.1.3.1 | 260 | 302 | 31 | e) During an EDCA TXOP won by an EDCAF, a QSTA may initiate multiple frame exchange sequences to transmit MMPDUs and/or MSDUs within the same AC. | STA |
| 9.1.3.1 | 260 | 302 | 32 | The duration of this EDCA TXOP is bounded, for an AC, by the value in dot11QAPEDCATXOPLimit MIB variable for a QAP and in dot11EDCATableTXOPLimit MIB table for a non-AP QSTA | an AP |
| 9.1.3.1 | 260 | 302 | 32 | The duration of this EDCA TXOP is bounded, for an AC, by the value in dot11QAPEDCATXOPLimit MIB variable for a QAP and in dot11EDCATableTXOPLimit MIB table for a non-AP QSTA | STA |
| 9.1.3.1 | 260 | 302 | 34 | A value of 0 for this duration means that the EDCA TXOP is limited to a single MSDU or MMPDU at any rate in the operational set of the QBSS. | BSS |
| 9.1.3.1 | 260 | 302 | 38 | The QAP announces the EDCA parameters in selected Beacon frames and in all Probe Response and (Re)Association Response frames by the inclusion of the EDCA Parameter Set information element. | QoS AP |
| 9.1.3.1 | 260 | 302 | 39 | If no such element is received, the QSTAs shall use the default values for the parameters. | STAs |
| 9.1.3.1 | 260 | 302 | 40 | The fields following the QoS Info field in the EDCA Parameter Set information element shall be included in all Beacon frames occurring within two or more delivery traffic indication message (DTIM) periods following a change in AC parameters to assure that all QSTAs are able to receive the updated EDCA parameters. | STAs |
| 9.1.3.1 | 260 | 302 | 43 | A QSTA shall update its MIB values of the EDCA parameters within an interval of time equal to one beacon interval after receiving an updated EDCA parameter set. | QoS STA |
| 9.1.3.1 | 260 | 302 | 45 | QSTAs update the MIB attributes and store the EDCA Parameter Set update count value in the QoS Info field. | QoS STAs |
| 9.1.3.1 | 260 | 302 | 46 | A QAP may change the EDCA access parameters by changing the EDCA Parameter Set element in the Beacon and Probe Response frames. | An AP |
| 9.1.3.1 | 260 | 302 | 47 | However, the QAP should change them only rarely. | AP |
| 9.1.3.1 | 260 | 302 | 48 | A QSTA shall use the EDCA Parameter Set Update Count Value subfield in the QoS Capability element of all Beacon frames to determine if the QSTA is using the current EDCA Parameter Values. | QoS STA |
| 9.1.3.1 | 260 | 302 | 48 | A QSTA shall use the EDCA Parameter Set Update Count Value subfield in the QoS Capability element of all Beacon frames to determine if the QSTA is using the current EDCA Parameter Values. | STA |
| 9.1.3.1 | 260 | 302 | 50 | If the EDCA Parameter Set update count value in the QoS Capability element is different from the value that has been stored, the QSTA shall query the updated EDCA parameter values by sending a Probe Request frame to the QAP. | QoS STA |
| 9.1.3.1 | 260 | 302 | 50 | If the EDCA Parameter Set update count value in the QoS Capability element is different from the value that has been stored, the QSTA shall query the updated EDCA parameter values by sending a Probe Request frame to the QAP. | AP |
| 9.1.3.1 | 260 | 302 | 54 | The QAP may use a different set of EDCA parameters than it advertises to the QSTAs in its QBSS. | AP |
| 9.1.3.1 | 260 | 302 | 54 | The QAP may use a different set of EDCA parameters than it advertises to the QSTAs in its QBSS. | STAs |
| 9.1.3.1 | 260 | 302 | 54 | The QAP may use a different set of EDCA parameters than it advertises to the QSTAs in its QBSS. | BSS |
| 9.1.3.1 | 261 | 303 | 2 | A QSTA shall also send management frames using the access category AC_VO before associating with any BSS, even if there is no QoS facility available in that BSS. | QoS STA |
| 9.1.3.2 | 261 | 303 | 15 | The HC is collocated with the QAP of the QBSS and uses the HC’s higher priority of access to the WM to initiate frame exchange sequences and to allocate TXOPs to itself and other QSTAs in order to provide limited-duration controlled access phase (CAP) for contention-free transfer of QoS data. | AP |
| 9.1.3.2 | 261 | 303 | 15 | The HC is collocated with the QAP of the QBSS and uses the HC’s higher priority of access to the WM to initiate frame exchange sequences and to allocate TXOPs to itself and other QSTAs in order to provide limited-duration controlled access phase (CAP) for contention-free transfer of QoS data. | BSS |
| 9.1.3.2 | 261 | 303 | 15 | The HC is collocated with the QAP of the QBSS and uses the HC’s higher priority of access to the WM to initiate frame exchange sequences and to allocate TXOPs to itself and other QSTAs in order to provide limited-duration controlled access phase (CAP) for contention-free transfer of QoS data. | STAs |
| 9.1.3.2 | 261 | 303 | 21 | TXOP allocations and contention-free transfers of QoS traffic can be based on the HC’s QBSS-wide knowledge of the amounts of pending traffic belonging to different TS and/or TCs and are subject to QBSS-specific QoS policies. | BSS |
| 9.1.3.2 | 261 | 303 | 21 | TXOP allocations and contention-free transfers of QoS traffic can be based on the HC’s QBSS-wide knowledge of the amounts of pending traffic belonging to different TS and/or TCs and are subject to QBSS-specific QoS policies. | BSS |
| 9.1.3.2 | 261 | 303 | 26 | A QAP may indicate availability of CF-Polls to nQSTAs, thereby providing non-QoS contention-free transfers during the CFP. | An AP |
| 9.1.3.2 | 261 | 303 | 26 | A QAP may indicate availability of CF-Polls to nQSTAs, thereby providing non-QoS contention-free transfers during the CFP. | non-QoS STAs |
| 9.1.3.2 | 261 | 303 | 27 | This provisioning of contention-free transfers during the CFP to nQSTAs, however, is not recommended. | non-QoS STAs |
| 9.1.3.2 | 261 | 303 | 28 | Implementers are cautioned that QSTAs are not required to interpret data subtypes that include QoS +CF-Ack in frames not addressed to themselves unless they set the Q-Ack subfield in the QoS Capability information element to 1. | QoS STAs |
| 9.1.3.2 | 261 | 303 | 30 | QSTAs are also not required to interpret data subtypes that are non-QoS (+)CF-Poll frames (i.e., data frames with bits 7, 5, and 4 in the Frame Control field set to 0, 1, and 0, respectively); therefore, QSTAs cannot be treated as CF-Pollable STAs. | QoS STAs |
| 9.1.3.2 | 261 | 303 | 30 | QSTAs are also not required to interpret data subtypes that are non-QoS (+)CF-Poll frames (i.e., data frames with bits 7, 5, and 4 in the Frame Control field set to 0, 1, and 0, respectively); therefore, QSTAs cannot be treated as CF-Pollable STAs. | QoS STAs |
| 9.1.3.2 | 261 | 303 | 32 | This requires a QAP that provides non-QoS CF-polling to adhere to frame sequence restrictions considerably more complex than, and less efficient than, those specified for either PCF or HCF. | an AP |
| 9.1.3.2 | 261 | 303 | 34 | In addition, the achievable service quality is likely to be degraded when nQSTAs are associated and being polled. | non-QoS STAs |
| 9.1.3.2 | 261 | 303 | 39 | A QSTA may initiate multiple frame exchange sequences during a polled TXOP of sufficient duration to perform more than one such sequence | STA |
| 9.1.4 | 261 | 303 | 51 | When an HC is operating in a QBSS, it may generate an alternation of CFP and CP in the same way as a PC, using the DCF access method only during the CP. | BSS |
| 9.1.5 | 262 | 304 | 7 | QSTAs may use fragmentation to use the medium efficiently in consideration of the duration available in granted TXOPs, as long as the rules in 9.4 are followed. | STAs |
| 9.1.5 | 262 | 304 | 38 | Unless interrupted due to medium occupancy limitations for a given PHY or TXOP limitations for QSTA, the fragments of a single MSDU or MMPDU are sent as a burst during the CP, using a single invocation of the DCF or EDCA medium access procedure. | STA |
| 9.1.6 | 263 | 305 | 5 | In a QSTA, the TID parameter of the MA-UNITDATA.request results in a TID being specified for the transmitted MSDU. | QoS STA |
| 9.2.3.4 | 266 | 308 | 21 | The AIFS shall be used by QSTAs to transmit all data frames (MPDUs), all management frames (MMPDUs), and the following control frames: PS-Poll, RTS, CTS (when not transmitted as a response to the RTS), BlockAckReq, and BlockAck (when not transmitted as a response to the BlockAckReq). | QoS STAs |
| 9.2.3.4 | 266 | 308 | 23 | A QSTA using the EDCA shall obtain a TXOP for an AC if the QSTA’s CS mechanism (see 9.2.1) determines that the medium is idle at the AIFS[AC] slot boundary (see 9.9.1.3), after a correctly received frame, and the backoff time for that AC has expired. | STA |
| 9.2.3.4 | 266 | 308 | 23 | A QSTA using the EDCA shall obtain a TXOP for an AC if the QSTA’s CS mechanism (see 9.2.1) determines that the medium is idle at the AIFS[AC] slot boundary (see 9.9.1.3), after a correctly received frame, and the backoff time for that AC has expired. | STA’s |
| 9.2.3.4 | 266 | 308 | 28 | A QSTA using the EDCA shall not transmit within an EIFS-DIFS+AIFS[AC] plus any backoff time after that QSTA determines that the medium is idle following reception of a frame for which the PHYRXEND.indication primitive reported an error or a frame for which the MAC FCS value was not correct, unless a subsequent reception of an error-free frame resynchronizes the STA. | STA |
| 9.2.3.4 | 266 | 308 | 31 | This resynchronization allows the QSTA to transmit using the AIFS[AC] following that subsequent frame, provided that the backoff time for that AC has expired. |  |
| 9.2.3.4 | 266 | 308 | 35 | A non-AP QSTA computes the time periods for each AIFS[AC] from the dot11EDCATableAIFSN attributes in the MIB. | QoS STA |
| 9.2.3.4 | 266 | 308 | 36 | QSTAs update their dot11EDCATableAIFSN values using information in the most recent EDCA Parameter Set element of Beacon frames received from the QAP of the QBSS (see 7.3.2.28). | QoS STAs |
| 9.2.3.4 | 266 | 308 | 36 | QSTAs update their dot11EDCATableAIFSN values using information in the most recent EDCA Parameter Set element of Beacon frames received from the QAP of the QBSS (see 7.3.2.28). | AP |
| 9.2.3.4 | 266 | 308 | 36 | QSTAs update their dot11EDCATableAIFSN values using information in the most recent EDCA Parameter Set element of Beacon frames received from the QAP of the QBSS (see 7.3.2.28). | BSS |
| 9.2.3.4 | 266 | 308 | 38 | A QAP computes the time periods for each AIFS[AC] from the dot11QAPEDCATableAIFSN attributes in its MIB. | QoS AP |
| 9.2.9 | 276 | 318 | 20 | In a QSTA, the sequence numbers for QoS (+)Data frames are generated by different counters for each TID and receiver pair and shall be incremented by one for each new MSDU corresponding to the TID/receiver pair. | QoS STA |
| 9.2.9 | 276 | 318 | 25 | The receiving QSTA shall also keep a cache of recently received <Address 2, TID, sequence-number, fragment-number> tuples for all QSTAs from whom it has received QoS data frames | QoS STA |
| 9.2.9 | 276 | 318 | 25 | The receiving QSTA shall also keep a cache of recently received <Address 2, TID, sequence-number, fragment-number> tuples for all QSTAs from whom it has received QoS data frames | STAs |
| 9.2.9 | 276 | 318 | 28 | A receiving QSTA is also required to keep only the most recent cache entry per <Address 2, TID, sequence-number> triple, storing only the most recently received fragment number for that triple. | QoS STA |
| 9.2.9 | 276 | 318 | 35 | A receiver QSTA shall also reject as a duplicate frame any frame that has the Retry bit set in the Frame Control field and that matches an <Address 2, TID, sequence-number, fragment-number> tuple of an entry in the cache. | QoS STA |
| 9.6 | 288 | 330 | 37 | In QSTAs, the Duration/ID field may cover multiple frames and may involve using the PLME-TXTIME.request primitive several times. | QoS STAs |
| 9.6 | 289 | 331 | 18 | An nQSTA shall ensure that no more than one MSDU or MMPDU from a particular SA to a particular individual RA is outstanding at a time. | A non-QoS STA |
| 9.6 | 289 | 331 | 22 | For all transmissions not using the acknowledgment policy of Block Ack, a QSTA shall ensure that no more than one MSDU or MMPDU with a particular TID from a particular SA to a particular individual RA is outstanding at any time. | QoS STA |
| 9.6 | 289 | 331 | 37 | For MSDUs belonging to the service class of QoSAck when the receiver is a QSTA, the QoS data frames that are used to send these MSDUs shall have the Ack Policy subfield in the QoS Control field set to Normal Ack or Block Ack. | QoS STA |
| 9.6 | 289 | 331 | 39 | For MSDUs belonging to the service class of QoSNoAck when the receiver is a QSTA, the QoS data frames that are used to send these MSDUs shall have the Ack Policy subfield in the QoS Control field set to No Ack. | QoS STA |
|  |  |  |  | Note: HCF (9.9) is only present in QoS networks. Same considerations as 9.1.3. |  |
| 9.9 | 293 | 335 | 18 | The TXOP may be obtained by a QSTA winning an instance of EDCA contention (see 9.9.1) during the CP or by a non-AP QSTA receiving a QoS (+)CFPoll frame (see 9.9.2) during the CP or CFP. | STA |
| 9.9 | 293 | 335 | 18 | The TXOP may be obtained by a QSTA winning an instance of EDCA contention (see 9.9.1) during the CP or by a non-AP QSTA receiving a QoS (+)CFPoll frame (see 9.9.2) during the CP or CFP. | STA |
| 9.9.1.2 | 294 | 336 | 9 | The TXOP limit duration values are advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP. | AP |
| 9.9.1.2 | 294 | 336 | 9 | The TXOP limit duration values are advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP. | AP |
| 9.9.1.2 | 294 | 336 | 14 | Non-AP QSTAs shall ensure that the duration of TXOPs obtained using the EDCA rules do not exceed the TXOP limit. | STAs |
| 9.9.1.2 | 294 | 336 | 19 | A QSTA shall fragment a unicast MSDU so that the transmission of the first MPDU of the TXOP does not cause the TXOP limit to be exceeded at the PHY rate selected for the initial transmission attempt of that MPDU. | STA |
| 9.9.1.2 | 294 | 336 | 29 | A QAP may send broadcast/multicast frames without using any protection mechanism. | QoS AP |
| 9.9.1.2 | 294 | 336 | 30 | In a QIBSS, broadcast/multicast frames shall be sent one at a time, and backoff shall be performed after the transmission of each of the broadcast/multicast frames. | QoS IBSS |
| 9.9.1.3 | 294 | 336 | 42 | The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP QSTAs and is advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP | STAs |
| 9.9.1.3 | 294 | 336 | 42 | The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP QSTAs and is advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP | AP |
| 9.9.1.3 | 294 | 336 | 42 | The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP QSTAs and is advertised by the QAP in the EDCA Parameter Set information element in Beacon and Probe Response frames transmitted by the QAP | AP |
| 9.9.1.3 | 294 | 336 | 44 | The value of AIFSN[AC] shall be greater than or equal to 1 for QAPs. | APs |
| 9.9.1.3 | 295 | 337 | 11 | c) When any other EDCAF at this QSTA transmitted a frame requiring acknowledgment, the earlier of | STA |
| 9.9.1.4 | 296 | 338 | 38 | If a QSTA has in its transmit queue an additional frame of the same AC as the one just transmitted and the duration of transmission of that frame plus any expected acknowledgment for that frame is less than the remaining medium occupancy timer value, then the QSTA may commence transmission of that frame at SIFS after the completion of the immediately preceding frame exchange sequence. | STA |
| 9.9.1.4 | 296 | 338 | 38 | If a QSTA has in its transmit queue an additional frame of the same AC as the one just transmitted and the duration of transmission of that frame plus any expected acknowledgment for that frame is less than the remaining medium occupancy timer value, then the QSTA may commence transmission of that frame at SIFS after the completion of the immediately preceding frame exchange sequence. | STA |
| 9.9.1.4 | 296 | 338 | 42 | The intention of using the multiple frame transmission shall be indicated by the QSTA through the setting of the duration/ID values in one of the following two ways (see 7.1.4): | STA |
| 9.9.1.4 | 297 | 339 | 1 | No other AC at the QSTA shall transmit before the expiry of the NAV set by the frame that resulted in a transmission failure. | STA |
| 9.9.1.4 | 297 | 339 | 2 | All other ACs at the QSTA shall treat the medium as busy until the expiry of the NAV set by the frame that resulted in a transmission failure, just as they would if they had received that transmission from another QSTA. | STA |
| 9.9.1.4 | 297 | 339 | 2 | All other ACs at the QSTA shall treat the medium as busy until the expiry of the NAV set by the frame that resulted in a transmission failure, just as they would if they had received that transmission from another QSTA. | STA |
| 9.9.1.4 | 297 | 339 | 6 | A frame exchange may be a multicast frame, a frame transmitted with No Ack policy (for which there is no expected acknowledgment), or a unicast frame followed by a correctly received ACK frame transmitted by either a non-AP QSTA or a QAP. | STA |
| 9.9.1.4 | 297 | 339 | 6 | A frame exchange may be a multicast frame, a frame transmitted with No Ack policy (for which there is no expected acknowledgment), or a unicast frame followed by a correctly received ACK frame transmitted by either a non-AP QSTA or a QAP. | an AP |
| 9.9.1.4 | 297 | 339 | 10 | Note that, as for an EDCA TXOP, a multiple frame transmission is granted to an EDCAF, not to a non-AP QSTA or QAP, so that the multiple frame transmission is permitted only for the transmission of a frame of the same AC as the frame that was granted the EDCA TXOP. | STA |
| 9.9.1.4 | 297 | 339 | 10 | Note that, as for an EDCA TXOP, a multiple frame transmission is granted to an EDCAF, not to a non-AP QSTA or QAP, so that the multiple frame transmission is permitted only for the transmission of a frame of the same AC as the frame that was granted the EDCA TXOP. | AP |
| 9.9.1.5 | 297 | 339 | 33 | The transmission attempt collides internally with another EDCAF of an AC that has higher priority, that is, two or more EDCAFs in the same QSTA are granted a TXOP at the same time. | STA |
| 9.9.1.5 | 297 | 339 | 42 | a) If the QSRC[AC] or the QLRC[AC] for the QSTA has reached dot11ShortRetryLimit or dot11LongRetryLimit respectively, CW[AC] shall be reset to CWmin[AC]. | QoS STA |
| 9.9.1.6 | 298 | 340 | 6 | QSTAs shall maintain a short retry counter and a long retry counter for each MSDU or MMPDU that belongs to a TC requiring acknowledgment. | QoS STAs |
| 9.9.1.6 | 298 | 340 | 8 | QSTAs also maintain a short retry counter and a long retry counter for each AC. | QoS STAs |
| 9.9.1.6 | 298 | 340 | 11 | After transmitting a frame that requires acknowledgment, the QSTA shall perform the acknowledgment procedure, as defined in 9.2.8. | STA |
| 9.9.1.6 | 298 | 340 | 14 | This short retry count and the QSTA QSRC[AC] shall be reset when a MAC frame of length less than or equal to dot11RTSThreshold succeeds for that MSDU or MMPDU. | QoS STA |
| 9.9.1.6 | 298 | 340 | 30 | QSTAs shall retry failed transmissions until the transmission is successful or until the relevant retry limit is reached. | STAs |
| 9.9.1.6 | 298 | 340 | 33 | With the exception of a frame belonging to a TID for which Block Ack is set up, a QSTA shall not initiate the transmission of any management or data frame to a specific RA while the transmission of another management or data frame with the same RA and having been assigned its sequence number from the same sequence counter has not yet completed to the point of success, retry fail, or other MAC discard (e.g., lifetime expiry). | QoS STA |
| 9.9.1.6 | 298 | 340 | 40 | QSTAs shall maintain a transmit MSDU timer for each MSDU passed to the MAC. | QoS STAs |
| 9.9.1.6 | 298 | 340 | 42 | If the value of this timer exceeds the appropriate entry in dot11EDCATableMSDULifetime, then the MSDU, or any remaining, undelivered fragments of that MSDU, shall be discarded by the source QSTA without any further attempt to complete delivery of that MSDU. | STA |
| 9.9.2 | 298 | 340 | 49 | This allows it to transfer MSDUs to non-AP QSTAs and to allocate TXOPs to non-AP QSTAs. | STAs |
| 9.9.2 | 298 | 340 | 49 | This allows it to transfer MSDUs to non-AP QSTAs and to allocate TXOPs to non-AP QSTAs. | STAs |
| 9.9.2 | 298 | 340 | 54 | Most important is that HCF frame exchange sequences may be used among QSTAs associated in a QBSS during both the CP and the CFP. | STAs |
| 9.9.2 | 298 | 340 | 54 | Most important is that HCF frame exchange sequences may be used among QSTAs associated in a QBSS during both the CP and the CFP. | BSS |
| 9.9.2 | 298 | 340 | 55 | Another significant difference is that the HC grants a non-AP QSTA a polled TXOP with duration specified in a QoS(+)CF-Poll frame. | STA |
| 9.9.2 | 299 | 341 | 2 | Non-AP QSTAs may transmit multiple frame exchange sequences within given polled TXOPs, subject to the limit on TXOP duration. | STAs |
| 9.9.2 | 299 | 341 | 5 | All STAs inherently obey the NAV rules of the HCF because each frame transmitted under HCF by the HC or by a non-AP QSTA contains a duration value chosen to cause STAs in the BSS to set their NAVs to protect the expected subsequent frames. | STA |
| 9.9.2 | 299 | 341 | 10 | All non-AP QSTAs shall be able to respond to QoS (+)CF-Poll frames received from an HC with the Address 1 field matching their own addresses. | QoS STAs |
| 9.9.2.1 | 299 | 341 | 28 | The HC gains control of the WM as needed to send QoS traffic to non-AP QSTAs and to issue QoS (+)CFPoll frames to non-AP QSTAs by waiting a shorter time between transmissions than the STAs using the EDCA procedures. | STAs |
| 9.9.2.1.1 | 299 | 341 | 43 | The HC may also issue QoS (+)CF-Poll frames to associated non-AP QSTAs during the CFP. | STAs |
| 9.9.2.1.1 | 299 | 341 | 48 | Only a QAP that also issues non-QoS CF-Poll frames to associated CF-Pollable STAs may end a CFP with a CF-End+CF-Ack frame and only when the CF-End+CF-Ack is acknowledging a reception from a CF-Pollable nQSTA. | an AP |
| 9.9.2.1.1 | 299 | 341 | 48 | Only a QAP that also issues non-QoS CF-Poll frames to associated CF-Pollable STAs may end a CFP with a CF-End+CF-Ack frame and only when the CF-End+CF-Ack is acknowledging a reception from a CF-Pollable nQSTA. | non-QoS STA |
| 9.9.2.1.1 | 299 | 341 | 51 | The use of a non-QoS CF-Poll frame by a QAP to a non-AP QSTA is deprecated (for further discussion, see 7.3.1.4). | an AP |
| 9.9.2.1.1 | 299 | 341 | 51 | The use of a non-QoS CF-Poll frame by a QAP to a non-AP QSTA is deprecated (for further discussion, see 7.3.1.4). | QoS STA |
| 9.9.2.1.3 | 300 | 342 | 32 | It should be noted that the recovery rules from the absence of an expected reception are different from EDCA because in this case the NAVs of all the STAs in the QBSS have already been set up by the transmissions by the HC. | BSS |
| 99.2.1.3 | 300 | 342 | 36 | Finally, since an HC is collocated with the QAP, the QAP may recover using the rules described in this subclause even if the recovery is from the absence of an expected reception. | AP |
| 99.2.1.3 | 300 | 342 | 36 | Finally, since an HC is collocated with the QAP, the QAP may recover using the rules described in this subclause even if the recovery is from the absence of an expected reception. | AP |
| 9.9.2.1.3 | 300 | 342 | 40 | If the beginning of reception of an expected response, as detected by the occurrence of PHY-CCA.indication(busy) primitive at the QSTA that is expecting the response, does not occur during the first slot time following SIFS, then | STA |
| 9.9.2.1.3 | 300 | 342 | 46 | b) If the transmitting STA is a non-AP QSTA, it shall initiate recovery by transmitting at a PIFS after the end of the last transmission, if the polled TXOP limit is greater than 0 and at least one frame (re)transmissions can be completed within the remaining duration of a nonzero polled TXOP limit. | QoS STA |
| 9.9.2.1.3 | 300 | 342 | 51 | If the transmitted frame is not of type QoS (+)CF-Poll and the expected response frame is not received correctly, regardless of the occurrence of the PHY-RXSTART.indication primitive, the QSTA may initiate recovery following the occurrence of PHY-CCA.indication(idle) primitive so that a SIFS time interval occurs between the last energy on the air and the transmission of the recovery frame. | QoS STA |
| 9.9.2.1.3 | 301 | 343 | 12 | Non-AP QSTAs that receive a QoS (+)CF-Poll frame shall respond within a SIFS period, regardless of the NAV setting. | STAs |
| 9.9.2.1.3 | 301 | 343 | 13 | If a response is not received but a PHY-CCA.indication(busy) primitive occurs during the slot following SIFS and is followed by a PHY-RXSTART.indication or PHY-RXEND.indication primitive prior to a PHY-CCA.indication(idle) primitive, then the HC shall assume that the transmitted QoS (+)CF-Poll frame was successfully received by the polled non-AP QSTA. | STA |
| 9.9.2.1.3 | 301 | 343 | 23 | If an HC receives a frame from a QSTA with a duration/ID covering only the response frame, the HC shall assume that the QSTA is terminating its TXOP, and the HC may initiate other transmissions or allow the channel to go into the CP. | STA |
| 9.9.2.1.3 | 301 | 343 | 27 | If a polled non-AP QSTA has no queued traffic to send or if the MPDUs available to send are too long to be transmitted within the specified TXOP limit, the non-AP QSTA shall send a QoS (+)Null frame. | QoS STA |
| 9.9.2.1.3 | 301 | 343 | 39 | If there are no frames to be sent to the HC, then the non-AP QSTA shall send to the HC a QoS Null with the Queue Size subfield in the QoS Control field set to 0. | QoS STA |
| 9.9.2.1.3 | 301 | 343 | 41 | If there is not enough time within the unused portion of the TXOP to transmit either the QoS Null frame or the frame with the Duration/ID field covering only the response frame, then the non-AP QSTA shall cease control of the channel. | STA |
| 9.9.2.1.3 | 301 | 343 | 43 | If the beginning of the reception of an expected ACK response frame to the final frame does not occur, detected as the nonoccurrence of PHYCCA.indication(busy) primitive at the non-AP QSTA that is expecting the response during the first slot time following SIFS, the non-AP QSTA shall retransmit the frame or transmit a QoS Null frame, with the Ack Policy subfield set to Normal Ack and the Queue Size subfield set to 0, after PIFS from the end of last transmission, until such time that it receives an acknowledgment or when there is not enough time remaining in the TXOP for sending such a frame. | QoS STA |
| 9.9.2.1.3 | 301 | 343 | 50 | If a PHY-CCA.indication(busy) primitive occurs at the non-AP QSTA that is expecting the ACK response frame during the first slot following SIFS after the end of the transmission of the final frame, it shall be interpreted as indicating that the channel control has been successfully transferred and no further frames shall be transmitted by the non-AP QSTA in the TXOP, even though the ACK frame from HC may be incorrectly received. | STA |
| 9.9.2.2 | 302 | 344 | 11 | Within a polled TXOP, a QSTA may initiate the transmission of one or more frame exchange sequences, with all such sequences nominally separated by a SIFS interval. | STA |
| 9.9.2.2 | 302 | 344 | 13 | The QSTA shall not initiate transmission of a frame unless the transmission and any acknowledgment or other immediate response expected from the peer MAC entity are able to complete prior to the end of the remaining TXOP duration. | STA |
| 9.9.2.2 | 302 | 344 | 15 | All transmissions, including the response frames, within the polled TXOP are considered to be the part of the TXOP, and the HC shall account for these when setting the TXOP limit. If the TXOP Limit subfield in the QoS Control field of the QoS data frame that includes CF-Poll is set to 0, then the QSTA to which the frame is directed to shall respond with either one MPDU or one QoS Null frame. | STA |
| 9.9.2.2 | 302 | 344 | 23 | The HC shall ensure that the full duration of any granted TXOP meets these requirements so that non-AP QSTAs may use the time prior to the TXOP limit of a polled TXOP without checking for these constraints. | STA |
| 9.9.2.2 | 302 | 344 | 25 | Subject to these limitations, all decisions regarding what MSDUs and/or MMPDUs are transmitted during any given TXOP are made by the QSTA that holds the TXOP. | STA |
| 9.9.2.2.1 | 302 | 344 | 43 | A HC shall set its own NAV to prevent its transmitting during a TXOP that it has granted to a QSTA through an HCCA poll. | STA |
| 9.9.2.2.1 | 302 | 344 | 45 | However, the HC may reclaim the TXOP if a QSTA is not using it or ends the TXOP early (see 9.9.2.1.3). | STA |
| 9.9.2.2.1 | 302 | 344 | 51 | In other words, the total TXOP size assigned should include an extra time allocated (i.e., n × aSlotTime, where n is the number of times the QSTA needs to sense the channel and is given by floor(TXOP limit/limit imposed in the regulatory domain). | STA |
| 9.9.2.2.1 | 302 | 344 | 54 | This TID value does not pertain to the TXOP limit value and does not place any constraints on what frame(s) the addressed QSTA may send in the granted TXOP. | STA |
| 9.9.2.2.1 | 303 | 345 | 1 | In a CFP or CP, if the HC has no more QSTAs to poll and it has no more data, management, BlockAckReq, or BlockAck frames to send, it may reset the NAVs of all QSTAs in the QBSS by sending a QoS CF-Poll frame with the RA matching its own MAC address and with the Duration/ID field set to 0. | STA |
| 9.9.2.2.1 | 303 | 345 | 1 | In a CFP or CP, if the HC has no more QSTAs to poll and it has no more data, management, BlockAckReq, or BlockAck frames to send, it may reset the NAVs of all QSTAs in the QBSS by sending a QoS CF-Poll frame with the RA matching its own MAC address and with the Duration/ID field set to 0. | STAs |
| 9.9.2.2.1 | 303 | 345 | 1 | In a CFP or CP, if the HC has no more QSTAs to poll and it has no more data, management, BlockAckReq, or BlockAck frames to send, it may reset the NAVs of all QSTAs in the QBSS by sending a QoS CF-Poll frame with the RA matching its own MAC address and with the Duration/ID field set to 0. | BSS |
| 9.9.2.2.1 | 303 | 345 | 3 | When the QAP contains a PC, during the CFP, it may reset the NAVs of all STAs within the QBSS (i.e., corresponding to the same BSSID) by sending a CF-End frame, regardless of how the NAVs have been originally set. | AP |
| 9.9.2.2.1 | 303 | 345 | 3 | When the QAP contains a PC, during the CFP, it may reset the NAVs of all STAs within the QBSS (i.e., corresponding to the same BSSID) by sending a CF-End frame, regardless of how the NAVs have been originally set. | BSS |
| 9.9.2.2.1 | 303 | 345 | 7 | A non-AP QSTA that receives a CF-End frame containing the BSSID of the QBSS, in which the non-AP QSTA is associated, shall reset the NAV value to 0. | STA |
| 9.9.2.2.1 | 303 | 345 | 7 | A non-AP QSTA that receives a CF-End frame containing the BSSID of the QBSS, in which the non-AP QSTA is associated, shall reset the NAV value to 0. | STA |
| 9.9.2.2.1 | 303 | 345 | 8 | A non-AP QSTA that receives a QoS (+)CF-Poll frame with a MAC address in the Address 1 field that matches the HC’s MAC address and the Duration/ID field value equal to zero shall reset the NAV value to 0. | STA |
| 9.9.2.2.1 | 303 | 345 | 13 | When a QSTA receives a QoS (+)CF-Poll frame containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address 1 field of the frame containing the QoS (+)CF-Poll. | STA |
| 9.9.2.2.1 | 303 | 345 | 13 | When a QSTA receives a QoS (+)CF-Poll frame containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address 1 field of the frame containing the QoS (+)CF-Poll. | BSS |
| 9.9.2.2.1 | 303 | 345 | 13 | When a QSTA receives a QoS (+)CF-Poll frame containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address 1 field of the frame containing the QoS (+)CF-Poll. | STA |
| 9.9.2.2.1 | 303 | 345 | 13 | When a QSTA receives a QoS (+)CF-Poll frame containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address 1 field of the frame containing the QoS (+)CF-Poll. | STA |
| 9.9.2.2.1 | 303 | 345 | 13 | When a QSTA receives a QoS (+)CF-Poll frame containing the BSSID of the QBSS in which the QSTA is associated, that QSTA shall update the NAV if necessary and shall save the TXOP holder address for that QBSS, which is the MAC address from the Address 1 field of the frame containing the QoS (+)CF-Poll. | BSS |
| 9.9.2.2.1 | 303 | 345 | 15 | If an RTS frame is received with the RA address matching the MAC address of the QSTA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the QSTA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV. | STA |
| 9.9.2.2.1 | 303 | 345 | 15 | If an RTS frame is received with the RA address matching the MAC address of the QSTA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the QSTA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV. | STA |
| 9.9.2.2.1 | 303 | 345 | 21 | When a QSTA receives a frame addressed to it and requires an acknowledgment, it shall respond with an ACK or QoS +CF-Ack frame independent of its NAV. | STA |
| 9.9.2.2.1 | 303 | 345 | 22 | A non-AP QSTA shall accept a polled TXOP by initiating a frame exchange sequence independent of its NAV. | STA |
| 9.9.2.3 | 303 | 345 | 30 | The requirement to respond to that TID is nonbinding, and a QSTA may respond with any frame. | STA |
| 9.9.2.3 | 303 | 345 | 31 | Upon receiving a QoS (+)CF-Poll frame, a non-AP QSTA may send any frames, i.e., QoS data frames belonging to any TID as well as management frames in the obtained TXOP. | STA |
| 9.9.2.3 | 303 | 345 | 36 | Non-AP QSTAs are not allowed to send QoS (+)CFPoll frames. | STAs |
| 9.9.2.3 | 303 | 345 | 37 | Non-AP QSTAs shall not send QoS (+)Data frames in response to any data frame other than the QoS (+)CF-Poll frames. | STAs |
| 9.9.2.3 | 303 | 345 | 40 | If a QSTA has set up at least one TS for which the Aggregation subfield in the associated TSPEC is set to 0, the QAP shall use only QoS CF-Poll or QoS CF-Ack+CF-Poll frames to poll the QSTA and shall never use QoS (+)Data+CF-Poll to poll the QSTA. | STA |
| 9.9.2.3 | 303 | 345 | 40 | If a QSTA has set up at least one TS for which the Aggregation subfield in the associated TSPEC is set to 0, the QAP shall use only QoS CF-Poll or QoS CF-Ack+CF-Poll frames to poll the QSTA and shall never use QoS (+)Data+CF-Poll to poll the QSTA. | AP |
| 9.9.2.3 | 303 | 345 | 40 | If a QSTA has set up at least one TS for which the Aggregation subfield in the associated TSPEC is set to 0, the QAP shall use only QoS CF-Poll or QoS CF-Ack+CF-Poll frames to poll the QSTA and shall never use QoS (+)Data+CF-Poll to poll the QSTA. | STA |
| 9.9.2.3 | 303 | 345 | 40 | If a QSTA has set up at least one TS for which the Aggregation subfield in the associated TSPEC is set to 0, the QAP shall use only QoS CF-Poll or QoS CF-Ack+CF-Poll frames to poll the QSTA and shall never use QoS (+)Data+CF-Poll to poll the QSTA. | STA |
| 9.9.2.3 | 303 | 345 | 42 | It should be noted that although QoS (+)CF-Poll is a data frame, but it should be transmitted at one of the rates in the BSSBasicRateSet parameter in order to set the NAV of all QSTA that are not being polled (see 9.6). | STAs |
| 9.9.2.3 | 303 | 345 | 47 | QSTAs shall use QoS data frames for all MSDU transfers to another QSTA. | QoS STAs |
| 9.9.2.3 | 303 | 345 | 47 | QSTAs shall use QoS data frames for all MSDU transfers to another QSTA. | QoS STA |
| 9.9.2.3 | 303 | 345 | 48 | Furthermore, either the Queue Size subfield shall indicate the amount of queued traffic present in the output queue that the QSTA uses for traffic belonging to this TC or TS, or the TXOP Duration Requested subfield shall indicate the duration that the QSTA desires for the next TXOP for traffic belonging to this TC or TS. | STA |
| 9.9.2.3 | 303 | 345 | 48 | Furthermore, either the Queue Size subfield shall indicate the amount of queued traffic present in the output queue that the QSTA uses for traffic belonging to this TC or TS, or the TXOP Duration Requested subfield shall indicate the duration that the QSTA desires for the next TXOP for traffic belonging to this TC or TS. | STA |
| 9.9.2.3 | 303 | 345 | 52 | A non-AP QSTA that wishes to inform the HC of queue status may use the QoS Null frame indicating the TID and the queue size or TXOP duration request (also see 9.9.2.3.1). | STA |
| 9.9.2.3 | 304 | 346 | 1 | QSTAs shall be able to receive QoS +CF-Ack frames. | QoS STAs |
| 9.9.2.3 | 304 | 346 | 1 | The HC may use QoS Data+CF-Ack frames to send frames to the same non-AP QSTA a SIFS after receiving the final transmission of the previous TXOP. | STA |
| 9.9.2.3 | 304 | 346 | 2 | The HC may also use QoS Data+CF-Ack frames to send frames to any other non-AP QSTA a SIFS after receiving the final transmission of the previous TXOP, if the non-AP QSTA that sent the final transmission of the previous TXOP has set the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame to 1. | STA |
| 9.9.2.3 | 304 | 346 | 6 | In both CFP and CP, QSTAs shall always respond to QoS data frames having the Ack Policy subfield in the QoS Control field set to Normal Ack with an ACK frame, unless the acknowledgment is piggybacked in which case it shall use a QoS +CF-Ack frame. | STAs |
| 9.9.2.3 | 304 | 346 | 8 | Piggybacked frames are allowed only in CFP or within TXOPs initiated by the HC. The HC shall not send a QoS data frame containing a +CF-Ack with an Address 1 that does not correspond to the address of the QSTA for which the +CF-Ack is intended, unless the QSTA to which the +CF-Ack is intended, sets the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame. | STA |
| 9.9.2.3 | 304 | 346 | 8 | Piggybacked frames are allowed only in CFP or within TXOPs initiated by the HC. The HC shall not send a QoS data frame containing a +CF-Ack with an Address 1 that does not correspond to the address of the QSTA for which the +CF-Ack is intended, unless the QSTA to which the +CF-Ack is intended, sets the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame. | STA |
| 9.9.2.3 | 304 | 346 | 12 | QSTAs are not required to be able to transmit QoS data frames with subtypes that include +CF-Ack. | STAs |
| 9.9.2.3 | 304 | 346 | 15 | A QAP that has dot11QAckOptionImplemented true may allow associations with STAs advertising support for the Q-Ack option. | An AP |
| 9.9.2.3 | 304 | 346 | 16 | Such associations do not require the QAP to employ piggyback acknowledgments directed toward that associated STA in frames that are not directed to that associated STA. | AP |
| 9.9.2.3 | 304 | 346 | 18 | QSTAs shall be able to process received QoS data frames with subtypes that include +CF-Ack when the QSTA to which the acknowledgment is directed is the same as the QSTA addressed by the Address 1 field of that QoS data frame. | QoS STAs |
| 9.9.2.3 | 304 | 346 | 18 | QSTAs shall be able to process received QoS data frames with subtypes that include +CF-Ack when the QSTA to which the acknowledgment is directed is the same as the QSTA addressed by the Address 1 field of that QoS data frame. | STA |
| 9.9.2.3 | 304 | 346 | 18 | QSTAs shall be able to process received QoS data frames with subtypes that include +CF-Ack when the QSTA to which the acknowledgment is directed is the same as the QSTA addressed by the Address 1 field of that QoS data frame. | STA |
| 9.9.2.3 | 304 | 346 | 21 | A QSTA that does not set the Q-Ack subfield to 1 in the QoS Capability information element in the (Re)Association Request frame is not required to handle the received QoS(+)Data+CF-Ack frames that are addressed to other QSTAs. | STA |
| 9.9.2.3 | 304 | 346 | 21 | A QSTA that does not set the Q-Ack subfield to 1 in the QoS Capability information element in the (Re)Association Request frame is not required to handle the received QoS(+)Data+CF-Ack frames that are addressed to other QSTAs. | STAs |
| 9.9.2.3 | 304 | 346 | 22 | The net effect of these restrictions on the use of QoS +CF-Ack frames is that the principal QoS +CF-Ack subtype that is useful is the QoS Data+CF-Ack frame, which can be sent by a non-AP QSTA as the first frame in a polled TXOP when that TXOP was conveyed in a QoS Data+CF-Poll(+CF-Ack) frame and the outgoing frame is directed to the HC’s QSTA address. | STA |
| 9.9.2.3 | 304 | 346 | 22 | The net effect of these restrictions on the use of QoS +CF-Ack frames is that the principal QoS +CF-Ack subtype that is useful is the QoS Data+CF-Ack frame, which can be sent by a non-AP QSTA as the first frame in a polled TXOP when that TXOP was conveyed in a QoS Data+CF-Poll(+CF-Ack) frame and the outgoing frame is directed to the HC’s QSTA address. | STA |
| 9.9.2.3 | 304 | 346 | 27 | QoS (Data+)CF-Poll+CF-Ack frames are also useful if the HC wants to grant another TXOP to a different non-AP QSTA a SIFS after receiving the final transmission of a non-AP QSTA’s previous TXOP, if the non-AP QSTA that sent the final transmission of the previous TXOP has set the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame to 1. | STA |
| 9.9.2.3 | 304 | 346 | 27 | QoS (Data+)CF-Poll+CF-Ack frames are also useful if the HC wants to grant another TXOP to a different non-AP QSTA a SIFS after receiving the final transmission of a non-AP QSTA’s previous TXOP, if the non-AP QSTA that sent the final transmission of the previous TXOP has set the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame to 1. | STA’s |
| 9.9.2.3 | 304 | 346 | 27 | QoS (Data+)CF-Poll+CF-Ack frames are also useful if the HC wants to grant another TXOP to a different non-AP QSTA a SIFS after receiving the final transmission of a non-AP QSTA’s previous TXOP, if the non-AP QSTA that sent the final transmission of the previous TXOP has set the Q-Ack subfield in the QoS Capability information element in the (Re)Association Request frame to 1. | STA |
| 9.9.2.3 | 304 | 346 | 34 | HCF contention-based channel access shall not be used to transmit MSDUs belonging to an established TS (with the HC’s acceptance of the associated TSPEC), unless the granted TSPEC indicates it is permitted to do so when the Access Policy subfield of the TS Info field is set to “HCCA, EDCA mixed mode” (HEMM), the polled QSTA utilized the full TXOP provided by the HC, and it has to send more MPDUs. | STA |
| 9.9.2.3 | 304 | 346 | 36 | When this QSTA sends frames belonging to a TS using contention-based channel access, it shall encode the TID subfield in the QoS data frame with the TID associated with the TS. | STA |
| 9.9.2.3 | 304 | 346 | 39 | When the QAP grants a TSPEC with the Access Policy subfield set to HEMM and if the corresponding AC needs admission control, the QAP shall include the medium time that specifies the granted time for EDCA access in the ADDTS Response frame. | AP |
| 9.9.2.3 | 304 | 346 | 39 | When the QAP grants a TSPEC with the Access Policy subfield set to HEMM and if the corresponding AC needs admission control, the QAP shall include the medium time that specifies the granted time for EDCA access in the ADDTS Response frame. | AP |
| 9.9.2.3.1 | 304 | 346 | 45 | Non-AP QSTAs may send TXOP requests during polled TXOPs or EDCA TXOPs using the QoS Control field in the QoS data or QoS Null frame directed to the HC, with the TXOP Duration Requested or Queue Size subfield value and TID subfield value indicated to the HC. | STAs |
| 9.9.2.3.1 | 304 | 346 | 47 | QAPs indicate whether they process TXOP request or queue size in the QoS Info field in the Beacon, Probe Response, and (Re)Association Response frames. | APs |
| 9.9.2.3.1 | 304 | 346 | 49 | QAPs shall process requests in at least one format. | APs |
| 9.9.2.3.1 | 304 | 346 | 49 | The QAP may reallocate TXOPs if the request belongs to TS or update the EDCA parameter set if the above request belongs to TC. | AP |
| 9.9.2.3.1 | 304 | 346 | 50 | Non-AP QSTAs shall use only the request format that the QAP indicates it can process. | STAs |
| 9.9.2.3.1 | 304 | 346 | 50 | Non-AP QSTAs shall use only the request format that the QAP indicates it can process. | AP |
| 9.9.2.3.2 | 305 | 347 | 3 | QSTAs may send an RTS frame as the first frame of any frame exchange sequence for which improved NAV protection is desired, during either the CP or CFP, and without regard for dot11RTSThreshold. | STAs |
| 9.9.2.3.2 | 305 | 347 | 6 | If a QSTA sends an RTS frame and does not receive an expected CTS frame, then the recovery rules are as specified in 9.9.2.1.3. | QoS STA |
| 9.9.2.3.2 | 305 | 347 | 9 | If NAV protection is desired for a transmission to the QAP in response to a QoS data frame with a subtype that includes CF-Poll, the polled non-AP QSTA is allowed to send a CTS frame (as a CTS frame is shorter than a QoS data frame and can be sent with a higher probability that it will be received by other STAs) with the RA containing its own MAC address in order to set the NAV in its own vicinity without the extra time to send an RTS frame. | AP |
| 9.9.2.3.2 | 305 | 347 | 9 | If NAV protection is desired for a transmission to the QAP in response to a QoS data frame with a subtype that includes CF-Poll, the polled non-AP QSTA is allowed to send a CTS frame (as a CTS frame is shorter than a QoS data frame and can be sent with a higher probability that it will be received by other STAs) with the RA containing its own MAC address in order to set the NAV in its own vicinity without the extra time to send an RTS frame. | STA |
| 9.9.3 | 305 | 347 | 19 | Admission control is also required when a QSTA desires guarantee on the amount of time that it can access the channel. | STA |
| 9.9.3 | 305 | 347 | 20 | The HC, which is in the QAP, is used to administer admission control in the network. | AP |
| 9.9.3.1 | 305 | 347 | 31 | A non-AP QSTA may support admission control procedures in 9.9.3.1.2 to send frames in the AC where admission control is mandated; but, if it does not support that procedure, it shall use EDCA parameters of a lower priority AC, as indicated in Table 65, that does not require admission control. | STA |
| 9.9.3.1 | 305 | 347 | 33 | QAPs shall support admission control procedures, at least to the minimal extent of advertising that admission is not mandatory on its ACs. | APs |
| 9.9.3.1 | 305 | 347 | 37 | The QAP uses the ACM (admission control mandatory) subfields advertised in the EDCA Parameter Set element to indicate whether admission control is required for each of the ACs. | AP |
| 9.9.3.1 | 305 | 347 | 39 | While the CWmin, CWmax, AIFS, TXOP limit parameters may be adjusted over time by the QAP, the ACM bit shall be static for the duration of the lifetime of the BSS. | AP |
| 9.9.3.1 | 305 | 347 | 40 | An ADDTS Request frame shall be transmitted by a non-AP QSTA to the HC in order to request admission of traffic in any direction (i.e., uplink, downlink, direct, or bidirectional) employing an AC that requires admission control. | STA |
| 9.9.3.1 | 305 | 347 | 44 | The QAP shall associate the received UP of the ADDTS Request frame with the appropriate AC as per the UP-to-AC mappings described in 9.1.3.1. | AP |
| 9.9.3.1 | 305 | 347 | 45 | The non-AP QSTA may transmit unadmitted traffic for the ACs for which the QAP does not require admission control. | STA |
| 9.9.3.1 | 305 | 347 | 45 | The non-AP QSTA may transmit unadmitted traffic for the ACs for which the QAP does not require admission control. | AP |
| 9.9.3.1 | 305 | 347 | 46 | If a QSTA desires to send data without admission control using an AC that mandates admission control, the QSTA shall use EDCA parameters that correspond to a lower priority and do not require admission control. | STA |
| 9.9.3.1 | 305 | 347 | 46 | If a QSTA desires to send data without admission control using an AC that mandates admission control, the QSTA shall use EDCA parameters that correspond to a lower priority and do not require admission control. | STA |
| 9.9.3.1.1 | 306 | 348 | 4 | Procedures at the QAP | AP |
| 9.9.3.1.1 | 306 | 348 | 6 | Regardless of the AC’s ACM setting, the QAP shall respond to an ADDTS Request frame with an ADDTS Response frame that may be to accept or deny the request. | AP |
| 9.9.3.1.1 | 306 | 348 | 7 | On receipt of an ADDTS Request frame from a non-AP QSTA, the QAP shall make a determination about whether to | STA |
| 9.9.3.1.1 | 306 | 348 | 7 | On receipt of an ADDTS Request frame from a non-AP QSTA, the QAP shall make a determination about whether to | AP |
| 9.9.3.1.1 | 306 | 348 | 13 | The algorithm used by the QAP to make this determination is a local matter. | AP |
| 9.9.3.1.1 | 306 | 348 | 13 | If the QAP decides to accept the request, the QAP shall also derive the medium time from the information conveyed in the TSPEC element in the ADDTS Request frame. | AP |
| 9.9.3.1.1 | 306 | 348 | 13 | If the QAP decides to accept the request, the QAP shall also derive the medium time from the information conveyed in the TSPEC element in the ADDTS Request frame. | AP |
| 9.9.3.1.1 | 306 | 348 | 15 | The QAP may use any algorithm in deriving the medium time, but K.2.2 provides a procedure that may be used. | AP |
| 9.9.3.1.1 | 306 | 348 | 16 | Having made such a determination, the QAP shall transmit a TSPEC element to the requesting non-AP QSTA contained in an ADDTS Response frame. | AP |
| 9.9.3.1.1 | 306 | 348 | 16 | Having made such a determination, the QAP shall transmit a TSPEC element to the requesting non-AP QSTA contained in an ADDTS Response frame. | STA |
| 9.9.3.1.1 | 306 | 348 | 18 | If the QAP is accepting the request, the Medium Time field shall be specified. | AP |
| 9.9.3.1.2 | 306 | 348 | 20 | Procedure at non-AP QSTAs | STAs |
| 9.9.3.1.2 | 306 | 348 | 24 | The non-AP QSTA may subsequently decide to explicitly request medium time for the AC that is associated with the specified priority. | STA |
| 9.9.3.1.2 | 306 | 348 | 28 | In order to make such a request, the non-AP QSTA shall transmit a TSPEC element contained in an ADDTS Request frame with the following fields specified (i.e., nonzero): Nominal MSDU Size, Mean Data Rate, Minimum PHY Rate, Inactivity Interval, and Surplus Bandwidth Allowance. | STA |
| 9.9.3.1.2 | 306 | 348 | 33 | On receipt of a TSPEC element contained in a ADDTS Response frame indicating that the request has been accepted, the non-AP QSTA shall recompute the admitted_time for the specified EDCAF as follows: | STA |
| 9.9.3.1.2 | 306 | 348 | 39 | The non-AP QSTA may choose to tear down the explicit request at any time. | STA |
| 9.9.3.1.2 | 306 | 348 | 39 | For the teardown of an explicit admission, the non-AP QSTA shall transmit a DELTS frame containing the TSID and direction that specify the TSPEC to the QAP. | STA |
| 9.9.3.1.2 | 306 | 348 | 39 | For the teardown of an explicit admission, the non-AP QSTA shall transmit a DELTS frame containing the TSID and direction that specify the TSPEC to the QAP. | AP |
| 9.9.3.1.2 | 306 | 348 | 43 | If the non-AP QSTA sends or receives a DELTS frame, it shall recompute the admitted_time for the specified EDCAF as follows: | STA |
| 9.9.3.1.2 | 306 | 348 | 48 | To describe the behavior at the non-AP QSTA, two parameters are defined. | STA |
| 9.9.3.1.2 | 306 | 348 | 48 | The parameter used_time signifies the amount of time used, in units of 32 us, by the non-AP QSTA in dot11EDCAAveragingPeriod. | STA |
| 9.9.3.1.2 | 306 | 348 | 49 | The parameter admitted_time is the medium time allowed by the QAP, in units of 32 us, in dot11EDCAAveragingPeriod. | AP |
| 9.9.3.1.2 | 306 | 348 | 51 | The non-AP QSTA shall update the value of used_time: | STA |
| 9.9.3.1.2 | 307 | 349 | 9 | However, a non-AP QSTA may choose to temporarily replace the EDCA parameters for that EDCAF with those specified for an AC of lower priority, if no admission control is required for those ACs. | STA |
| 9.9.3.1.2 | 307 | 349 | 13 | If, for example, a non-AP QSTA has made and had accepted an explicit admission for a TS and the channel conditions subsequently worsen, possibly including a change in PHY data rate so that it requires more time to send the same data, the non-AP QSTA may make a request for more admitted_time to the QAP and at the same time downgrade the EDCA parameters for that AC for short intervals in order to send some of the traffic at the admitted priority and some at the unadmitted priority, while waiting for a response to the admission request. | STA |
| 9.9.3.1.2 | 307 | 349 | 13 | If, for example, a non-AP QSTA has made and had accepted an explicit admission for a TS and the channel conditions subsequently worsen, possibly including a change in PHY data rate so that it requires more time to send the same data, the non-AP QSTA may make a request for more admitted_time to the QAP and at the same time downgrade the EDCA parameters for that AC for short intervals in order to send some of the traffic at the admitted priority and some at the unadmitted priority, while waiting for a response to the admission request. | STA |
| 9.9.3.1.2 | 307 | 349 | 13 | If, for example, a non-AP QSTA has made and had accepted an explicit admission for a TS and the channel conditions subsequently worsen, possibly including a change in PHY data rate so that it requires more time to send the same data, the non-AP QSTA may make a request for more admitted_time to the QAP and at the same time downgrade the EDCA parameters for that AC for short intervals in order to send some of the traffic at the admitted priority and some at the unadmitted priority, while waiting for a response to the admission request. | AP |
| 9.9.3.2 | 307 | 349 | 22 | When the HC provides controlled channel access to non-AP QSTAs, it is responsible for granting or denying polling service to a TS based on the parameters in the associated TSPEC. | STAs |
| 9.9.3.2 | 307 | 349 | 39 | Specifically, if a TS is admitted by the HC, then the scheduler shall service the non-AP QSTA during an SP. | STA |
| 9.9.3.2 | 307 | 349 | 40 | An SP is a contiguous time during which a set of one or more downlink unicast frames and/or one or more polled TXOPs are granted to the QSTA. | STA |
| 9.9.3.2 | 307 | 349 | 50 | A QAP shall schedule the transmissions in HCCA TXOPs and communicate the service schedule to the non-AP QSTA. | An AP |
| 9.9.3.2 | 307 | 349 | 50 | A QAP shall schedule the transmissions in HCCA TXOPs and communicate the service schedule to the non-AP QSTA. | STA |
| 9.9.3.2 | 308 | 350 | 1 | The HC shall provide an aggregate service schedule if the non-AP QSTA sets the Aggregation field in its TSPEC request. | STA |
| 9.9.3.2 | 308 | 350 | 2 | If the QAP establishes an aggregate service schedule for a non-AP QSTA, it shall aggregate all HCCA streams for the QSTA. | AP |
| 9.9.3.2 | 308 | 350 | 2 | If the QAP establishes an aggregate service schedule for a non-AP QSTA, it shall aggregate all HCCA streams for the QSTA. | STA |
| 9.9.3.2 | 308 | 350 | 3 | The service schedule is communicated to the non-AP QSTA in a Schedule element contained in an ADDTS Response frame. | STA |
| 9.9.3.2 | 308 | 350 | 9 | The service schedule could be subsequently updated by a QAP as long as it meets TSPEC requirements. | an AP |
| 9.9.3.2 | 308 | 350 | 19 | A non-AP QSTA may affect the service schedule by modifying or deleting its existing TS as specified in 11.4. | STA |
| 9.9.3.2 | 308 | 350 | 22 | K.3.1 provides guidelines for deriving an aggregate service schedule for a single non-AP QSTA from the non-AP QSTA’s admitted TS. The schedule shall meet the QoS requirements specified in the TSPEC. | STA |
| 9.9.3.2 | 308 | 350 | 22 | K.3.1 provides guidelines for deriving an aggregate service schedule for a single non-AP QSTA from the non-AP QSTA’s admitted TS. The schedule shall meet the QoS requirements specified in the TSPEC. | STA’s |
| 9.9.3.2 | 308 | 350 | 34 | Non-AP QSTAs may have an operational rate lower than the minimum PHY rate due to varying conditions on the channel for a short time and still may be able to sustain the TS without changing the minimum PHY rate in the TSPEC. | STAs |
| 9.9.3.2 | 308 | 350 | 48 | If the HC admits the stream with the alternative set of TSPEC parameters, these parameters are indicated to the non-AP QSTA through the ADDTS Response frame. | STA |
| 9.9.3.2 | 309 | 351 | 1 | If a QSTA specifies a nonzero minimum SI and if the TS is admitted, the HC shall generate a schedule that conforms to the specified minimum SI. | STA |
|  |  |  |  | Note: BlockAck (9.10) is a capability advertised separately by the STA and AP. Use of QSTA/QAP is redundant. |  |
| 9.10.1 | 309 | 351 | 15 | In this subclause, the QSTA with data to send using the Block Ack mechanism is referred to as the originator, and the receiver of that data as the recipient. | STA |
| 9.10.1 | 309 | 351 | 25 | The Block Ack mechanism does not require the setting up of a TS; however, QSTAs using the TS facility may choose to signal their intention to use Block Ack mechanism for the scheduler’s consideration in assigning TXOPs. | QoS STAs |
| 9.10.2 | 309 | 351 | 37 | A QSTA that intends to use the Block Ack mechanism for the transmission of QoS data frames to a peer should first check whether the intended peer QSTA is capable of participating in Block Ack mechanism by discovering and examining its Delayed Block Ack and Immediate Block Ack capability bits. | STA |
| 9.10.2 | 309 | 351 | 37 | A QSTA that intends to use the Block Ack mechanism for the transmission of QoS data frames to a peer should first check whether the intended peer QSTA is capable of participating in Block Ack mechanism by discovering and examining its Delayed Block Ack and Immediate Block Ack capability bits. | STA |
| 9.10.2 | 309 | 351 | 40 | If the intended peer QSTA is capable of participating, the originator sends an ADDBA Request frame indicating the TID for which the Block Ack is being set up. | STA |
| 9.10.2 | 309 | 351 | 43 | The receiving QSTA shall respond by an ADDBA Response frame. | STA |
| 9.10.2 | 309 | 351 | 44 | The receiving QSTA, which is the intended peer, has the option of accepting or rejecting the request. | STA |
| 9.10.2 | 309 | 351 | 45 | When the QSTA accepts, then a Block Ack agreement exists between the originator and recipient. | STA |
| 9.10.2 | 309 | 351 | 46 | When the QSTA accepts, it indicates the type of Block Ack and the number of buffers that it shall allocate for the support of this block. | STA |
| 9.10.2 | 309 | 351 | 47 | If the receiving QSTA rejects the request, then the originator shall not use the Block Ack mechanism. | STA |
| 9.10.3 | 310 | 352 | 52 | If no protective mechanism is used, then the first frame that is sent as a block shall have a response frame and shall have the Duration field set so that the NAVs are set to appropriate values at all STAs in the QBSS. | BSS |
| 9.10.3 | 311 | 353 | 27 | In the Block-Ack frame, the QSTA acknowledges only the MPDUs starting from the starting sequence control until the MPDU with the highest sequence number (modulo 212) that has been received, and the QSTA shall set bits in the Block Ack bitmap corresponding to all other MPDUs to 0. | STA |
| 9.10.3 | 311 | 353 | 27 | In the Block-Ack frame, the QSTA acknowledges only the MPDUs starting from the starting sequence control until the MPDU with the highest sequence number (modulo 212) that has been received, and the QSTA shall set bits in the Block Ack bitmap corresponding to all other MPDUs to 0. | STA |
| 9.11 | 313 | 355 | 49 | The usage of No Ack is determined by the policy at the QSTA. | QoS STA |
| 9.12 | 320 | 362 | 39 | QAP – Frame is transmitted by a QAP | QoS AP |
| 9.12 | 321 | 363 | 25 | (* An hcf-sequence represents all the sequences that may be generated under HCCA. The sequence may be initiated by an HC within a CFP, or it may be initiated by a QSTA using EDCA channel access. *) | STA |
| 9.12 | 321 | 363 | 40 | (* A poll-sequence is the start of a polled TXOP, in which the HC delivers a polled TXOP to a QSTA. The poll may or may not piggyback a CF-Ack according to whether the previous frame received by the HC was a Data frame. *) | STA |
| 9.12 | 322 | 364 | 34 | (* Polled-txop-content is what may occur after the delivery of a polled TXOP. A QSTA transmits the first frame in this sequence *) | QoS STA |
| 9.12 | 322 | 364 | 40 | (* A polled-txop-termination may be used by a QSTA to terminate the polled TXOP. The data frame is addressed to the HC, which regains control of the medium and may re-use any unused polled TXOP duration. *) | QoS STA |
| 10.3.2.2.2 | 335 | 377 | 33 | QBSS Load | BSS |
| 10.3.2.2.2 | 335 | 377 | 33 | The values from the QBSS Load information element if such an element was present in the probe response or beacon, else null. | BSS |
| 10.3.6.2.2 | 347 | 389 | 38 | Specifies the EDCA parameter set that the QSTA should use. | STA |
| 10.3.6.4.2 | 349 | 391 | 38 | Specifies the EDCA parameter set that the QSTA should use. | STA |
| 10.3.7.2.2 | 353 | 395 | 43 | Specifies the EDCA parameter set that the QSTA should use. | STA |
| 10.3.7.4.2 | 357 | 399 | 38 | Specifies the EDCA parameter set that the QSTA should use. | STA |
| 10.3.10.1.2 | 365 | 407 | 20 | The initial EDCA parameter set values to be used in the QBSS. | BSS |
| 10.3.25 | 400 | 442 | 19 | Non-AP QSTA | QoS STA (2 times) |
| 10.3.25 | 400 | 442 | 20 | Non-AP QSTA and HC | QoS STA (3 times) |
| 10.3.25 | 400 | 442 | 3 | This mechanism supports the process of adding, modifying, or deleting a TS in a QBSS using the procedures defined in 11.4. | BSS |
| 10.3.25.1.1 | 400 | 442 | 28 | It is valid at the non-AP QSTA and requests the HC to admit the new or changed TS. | STA |
| 10.3.25.1.3 | 401 | 443 | 21 | This primitive is generated by the SME at a non-AP QSTA to request the addition of a new (or modification of an existing) TS in order to support parameterized QoS transport of the MSDUs belonging to this TS when a higher layer protocol or mechanism signals the QSTA to initiate such an addition (or modification). | STA |
| 10.3.25.1.3 | 401 | 443 | 21 | This primitive is generated by the SME at a non-AP QSTA to request the addition of a new (or modification of an existing) TS in order to support parameterized QoS transport of the MSDUs belonging to this TS when a higher layer protocol or mechanism signals the QSTA to initiate such an addition (or modification). | STA |
| 10.3.25.1.4 | 401 | 443 | 27 | The non-AP QSTA operates according to the procedures defined in 11.4. | STA |
| 10.3.25.2.2 | 402 | 444 | 40 | When the result code is REJECTED_FOR_DELAY_PERIOD, provides the amount of time a QSTA should wait before attempting another ADDTS request frame. | STA |
| 10.3.25.2.2 | 403 | 445 | 9 | If the suggested changes are acceptable to the non-AP QSTA, it is the responsibility of the non-AP QSTA to set up the TS with the suggested changes | STA |
| 10.3.25.2.2 | 403 | 445 | 9 | If the suggested changes are acceptable to the non-AP QSTA, it is the responsibility of the non-AP QSTA to set up the TS with the suggested changes | STA |
| 10.3.25.2.3 | 403 | 445 | 19 | This primitive is generated when that MLME-ADDTS.request primitive is found to contain invalid parameters, when a timeout occurs, or when the non-AP QSTA receives a response in the form of an ADDTS Response frame in the corresponding QoS Action frame from the HC. | STA |
| 10.3.25.3.2 | 404 | 446 | 14 | Non-AP QSTA address | STA |
| 10.3.25.3.2 | 404 | 446 | 28 | Non-AP QSTA address | STA |
| 10.3.25.3.2 | 404 | 446 | 28 | Contains the MAC address of the non-AP QSTA that initiated the MLME-ADDTS.request primitive. | STA |
| 10.3.25.3.2 | 404 | 446 | 52 | The TCLAS is optional at the discretion of the non-AP QSTA that originated the request. | STA |
| 10.3.25.3.3 | 405 | 447 | 3 | This primitive is generated by the MLME as a result of receipt of a request to add (or modify) a TS by a specified non-AP QSTA in the form of an Add TS request in the corresponding QoS Action frame. | STA |
| 10.3.25.3.4 | 405 | 447 | 8 | The SME is notified of the request for a TS addition (or modification) by a specified non-AP QSTA. | STA |
| 10.3.25.4.1 | 406 | 448 | 5 | This primitive responds to the request for a TS addition (or modification) by a specified non-AP QSTA MAC entity. | STA |
| 10.3.25.4.2 | 406 | 448 | 15 | Non-APQSTAAddress | STA |
| 10.3.25.4.2 | 406 | 448 | 41 | Non-APQSTAAddress | STA |
| 10..3.25.4.2 | 406 | 448 | 41 | Contains the non-AP QSTA address of the matching MLME-ADDTS.indication primitive. | STA |
| 10.3.25.4.2 | 406 | 448 | 44 | When the result code is REJECTED_FOR_DELAY_PERIOD, provides the amount of time a QSTA should wait before attempting another ADDTS request. | STA |
| 10.3.25.4.2 | 407 | 449 | 7 | The DialogToken and non-APQSTAAddress parameters contain the values from the matching MLMEADDTS.indication primitive. | STA |
| 10.3.25.4.2 | 407 | 449 | 15 | The difference may lie in the QoS (e.g., minimum data rate, mean data rate, and delay bound) values, as a result of admission control performed at the SME of the HC on the TS requested to be added (or modified) by the non-AP QSTA. | STA |
| 10.3.25.4.4 | 407 | 449 | 28 | This primitive approves addition (or modification) of a TS requested by a specified non-AP QSTA MAC entity, with or without altering the TSPEC. | STA |
| 10.3.25.4.4 | 407 | 449 | 31 | This primitive causes the MAC entity at the HC to send an ADDTS Response frame in the corresponding QoS Action management frame to the requesting non-AP QSTA containing the specified parameters. | STA |
| 10.3.25.5.1 | 408 | 450 | 8 | This primitive may be generated at either a non-AP QSTA or the HC. | STA |
| 10.3.25.5.2 | 408 | 450 | 16 | Non-APQSTAAddress, | STA |
| 10.3.25.5.2 | 408 | 450 | 26 | Non-APQSTAAddress (HC only) | STA |
| 10.3.25.5.2 | 408 | 450 | 26 | (At the HC only) Specifies the MAC address of the non-AP QSTA that initiated this TS. | STA |
| 10.3.25.5.2 | 408 | 450 | 31 | QSTA_LEAVING | STA_LEAVING, |
| 10.3.25.5.3 | 408 | 450 | 41 | This primitive is generated by the SME at a QSTA to initiate deletion of a TS when a higher layer protocol or mechanism signals the QSTA to initiate such a deletion. | STA |
| 10.3.25.5.3 | 408 | 450 | 41 | This primitive is generated by the SME at a QSTA to initiate deletion of a TS when a higher layer protocol or mechanism signals the QSTA to initiate such a deletion. | STA |
| 10.3.25.5.4 | 408 | 450 | 51 | If this primitive was generated at the HC, the frame is sent to the specified non-AP QSTA MAC address. | STA |
| 10.3.25.5.4 | 408 | 450 | 51 | If this primitive was generated at the non-AP QSTA, the frame is sent to its HC. | STA |
| 10.3.25.6.2 | 409 | 451 | 12 | Non-APQSTAAddress, | STA |
| 10.3.25.6.2 | 409 | 451 | 23 | Non-APQSTAAddress (HC only) | STA |
| 10.3.25.6.2 | 409 | 451 | 23 | (HC only) Contains the non-AP QSTA MAC address of the matching request. | STA |
| 10.3.25.7.2 | 410 | 452 | 13 | Non-APQSTAAddress, | STA |
| 10.3.25.7.2 | 410 | 452 | 22 | Non-APQSTAAddress (HC only) | STA |
| 10.3.25.7.2 | 410 | 452 | 22 | (HC only) Specifies the MAC address of the non-AP QSTA for which the TS is being deleted. | STA |
| 10.3.25.7.2 | 410 | 452 | 28 | QSTA_LEAVING | STA_LEAVING |
| 10.3.25.7.3 | 410 | 452 | 46 | Without reception of an MSDU belonging to that TS by the MAC entity of the HC when a non-AP QSTA is the source STA of that TS. | STA |
| 10.3.26.1.3 | 411 | 453 | 34 | This primitive is generated by the SME at a non-AP QSTA to set up a direct link with another non-AP QSTA. | STA |
| 10.3.26.1.3 | 411 | 453 | 34 | This primitive is generated by the SME at a non-AP QSTA to set up a direct link with another non-AP QSTA. | STA |
| 10.3.26.1.2 | 411 | 453 | 22 | Specifies the MAC address of the non-AP QSTA that is the intended immediate recipient of the data flow. | STA |
| 10.3.26.2.2 | 412 | 454 | 25 | Specifies the MAC address of the non-AP QSTA that is the intended immediate recipient of the data flow. | STA |
| 10.3.26.2.2 | 412 | 454 | 31 | NOT_QSTA, | NOT_QOS_STA, |
| 10.3.26.3.2 | 413 | 455 | 19 | Specifies the MAC address of the non-AP QSTA that is the sender of the data flow. | STA |
| 10.3.26.4 | 414 | 456 | 5 | When initiated by a non-AP QSTA, this primitive requests the teardown of the direct link with a specified peer MAC entity. | STA |
| 10.3.26.4.1 | 414 | 456 | 6 | When initiated by a QAP, this primitive requests the teardown of direct link between two specified MAC entities. | an AP |
| 10.3.26.4.3 | 414 | 456 | 34 | This primitive is generated by the SME at a QSTA for tearing down a direct link with another non-AP QSTA. | STA |
| 10.3.26.4.3 | 414 | 456 | 34 | This primitive is generated by the SME at a QSTA for tearing down a direct link with another non-AP QSTA. | STA |
| 10.3.26.4.2 | 414 | 456 | 20 | Specifies the MAC address of the non-AP QSTA that is the intended immediate recipient of the teardown. | STA |
| 10.3.26.4.2 | 414 | 456 | 23 | Specifies the MAC address of the non-AP QSTA with which the DLS is being torn down. This parameter is applicable only at the QAP. | STA |
| 10.3.26.4.2 | 414 | 456 | 23 | Specifies the MAC address of the non-AP QSTA with which the DLS is being torn down. This parameter is applicable only at the QAP. | AP |
| 10.3.26.4.2 | 414 | 456 | 26 | QSTA_LEAVING, | STA_LEAVING, |
| 10.3.26.5.2 | 415 | 457 | 18 | Specifies the MAC address of the non-AP QSTA that is the intended immediate recipient of the teardown. | STA |
| 10.3.26.5.2 | 415 | 457 | 21 | Specifies the MAC address of the non-AP QSTA with which the direct link is being torn down. This parameter is applicable only at the QAP. | STA |
| 10.3.26.5.2 | 415 | 457 | 21 | Specifies the MAC address of the non-AP QSTA with which the direct link is being torn down. This parameter is applicable only at the QAP. | AP |
| 10.3.26.6.2 | 416 | 458 | 17 | Specifies the MAC address of the non-AP QSTA that is the sender of the data flow. | STA |
| 10.3.26.6.2 | 416 | 458 | 20 | QSTA_LEAVING | STA_LEAVING, |
| 10.3.28.1.2 | 420 | 462 | 22 | PeerQSTAAddress, | STA |
| 10.3.28.1.2 | 420 | 462 | 36 | PeerQSTAAddress | STA |
| 10.3.28.1.3 | 421 | 463 | 3 | This primitive is generated by the SME at a QSTA to request initiation (or modification) of Block Ack with the specified peer MAC entity. | STA |
| 10.3.28.1.4 | 421 | 463 | 8 | The QSTA sends the ADDBA Request frame to the specified peer MAC entity. | STA |
| 10.3.28.2.2 | 422 | 464 | 25 | PeerQSTAAddress | STA |
| 10.3.28.2.2 | 422 | 464 | 25 | Specifies the address of the peer MAC entity with which the Block Ack initiation (or modification) was attempted. This value must match the PeerQSTAAddress parameter specified in MLMEADDBA.request primitive. | STA |
| 10.3.28.2.2 | 422 | 464 | 13 | PeerQSTAAddress, | STA |
| 10.3.28.2.2 | 422 | 464 | 30 | Specifies the number of MPDUs that can be held in peerQSTAAddress buffer. | STA |
| 10.3.28.4.2 | 425 | 467 | 22 | PeerQSTAAddress | STA |
| 10.3.28.4.2 | 425 | 467 | 22 | Specifies the address of the peer MAC entity that attempted the Block Ack initiation (or modification). This value must match the PeerQSTAAddress parameter specified in MLME-ADDBA.indication primitive. | STA |
| 10.3.28.4.2 | 425 | 467 | 11 | PeerQSTAAddress, | STA |
| 10.3.28.5.2 | 426 | 468 | 19 | PeerQSTAAddress | STA |
| 10.3.28.5.2 | 426 | 468 | 27 | QSTA_LEAVING | STA_LEAVING |
| 10.3.28.5.2 | 426 | 468 | 11 | PeerQSTAAddress, | STA |
| 10.3.28.5.3 | 426 | 468 | 33 | This primitive is generated by the SME at a QSTA to request deletion of Block Ack with the specified peer MAC entity. | STA |
| 10.3.28.5.4 | 426 | 468 | 38 | The QSTA sends the DELBA frame to the specified peer MAC entity. | STA |
| 10.3.28.6.2 | 427 | 469 | 20 | PeerQSTAAddress | STA |
| 10.3.28.6.2 | 427 | 469 | 20 | Specifies the address of the peer MAC entity with which the Block Ack deletion was attempted. This value must match the PeerQSTAAddress parameter specified in MLME-DELBA.request primitive. | STA |
| 10.3.28.6.2 | 427 | 469 | 13 | PeerQSTAAddress, | STA |
| 10.3.28.7.2 | 428 | 470 | 19 | PeerQSTAAddress | STA |
| 10.3.28.7.2 | 428 | 470 | 27 | QSTA_LEAVING | STA_LEAVING |
| 10.3.28.7.2 | 428 | 470 | 11 | PeerQSTAAddress, | STA |
| 10.3.29.1.2 | 429 | 471 | 25 | Non-APQSTAAddress | STA |
| 10.3.29.1.2 | 429 | 471 | 25 | MAC address of the non-AP QSTA to which the Schedule frame shall be sent. | STA |
| 10.3.29.1.2 | 429 | 471 | 27 | Specifies the schedule for the non-AP QSTA, including the SI (minimum and maximum), TXOP duration (minimum and maximum), and specification interval. | STA |
| 10.3.29.1.1 | 429 | 471 | 18 | Non-APQSTAAddress, | STA |
| 10.3.29.1.3 | 429 | 471 | 32 | This primitive is generated by the SME at the HC to send the schedule information, in the form of a Schedule frame, to a specified non-AP QSTA when the schedule information for the non-AP QSTA is changed. | STA |
| 10.3.29.1.3 | 429 | 471 | 32 | This primitive is generated by the SME at the HC to send the schedule information, in the form of a Schedule frame, to a specified non-AP QSTA when the schedule information for the non-AP QSTA is changed. | STA |
| 10.3.29.1.4 | 429 | 471 | 37 | This primitive causes the MAC entity at the HC to send a Schedule frame to the non-AP QSTA specified in the primitive containing the specified Schedule parameters. | STA |
| 10.3.29.2.4 | 430 | 472 | 29 | The SME is notified of the result of the MLME-SCHEDULE.request primitive. If the result code is SUCCESS, the Schedule element has been correctly sent by the HC to the non-AP QSTA in the Schedule frame. | STA |
| 10.3.29.3.2 | 431 | 473 | 17 | Specifies the schedule for the non-AP QSTA, including the SI (minimum and maximum), TXOP duration (minimum and maximum), and specification interval. | STA |
| 10.3.29.3.1 | 431 | 473 | 5 | This primitive reports the reception of a new schedule by the non-AP QSTA in the form of a Schedule frame. It is valid at the non-AP QSTA. | STA |
| 10.3.29.3.1 | 431 | 473 | 5 | This primitive reports the reception of a new schedule by the non-AP QSTA in the form of a Schedule frame. It is valid at the non-AP QSTA. | STA |
| 11.2.1 | 449 | 491 | 49 | A non-AP QSTA may be in PS mode before the setup of DLS or Block Ack. | QoS STA |
| 11.2.1 | 449 | 491 | 49 | Once DLS is set up with another non-AP QSTA, the non-AP QSTA suspends the PS mode and shall always be awake. | STA |
| 11.2.1 | 449 | 491 | 49 | Once DLS is set up with another non-AP QSTA, the non-AP QSTA suspends the PS mode and shall always be awake. | STA |
| 11.2.1 | 449 | 491 | 53 | Uplink Block Ack, Block Acks for any TID with a schedule, and any Block Acks to APSD QSTA continue to operate normally. | STA |
|  |  |  |  | Note: Power management with APSD (11.2.1.4) is advertised separately through a Capability bit in the Capability Information Element. Use of “Q” for QSTA/QAP is redundant. Likewise references to (Q)STAs and (Q)APs that use APSD in later subclauses of 11.2.1. |  |
| 11.2.1.4 | 451 | 493 | 44 | QAPs capable of supporting automatic power save delivery (APSD) shall signal this capability through the use of the APSD subfield in the Capability Information field in Beacon, Probe Response, and (Re)Association Response management frames. | QoS APs |
| 11.2.1.4 | 451 | 493 | 48 | Non-AP QSTAs use the Power Management field in the Frame Control field of a frame to indicate whether it is in active or PS mode. | QoS STAs |
| 11.2.1.4 | 451 | 493 | 49 | As APSD is a mechanism for the delivery of downlink frames to power-saving STAs, the frames of a non-AP QSTA using APSD shall have the Power Management bit in the Frame Control field set to 1 for buffering to take place at the QAP. | STA |
| 11.2.1.4 | 451 | 493 | 49 | As APSD is a mechanism for the delivery of downlink frames to power-saving STAs, the frames of a non-AP QSTA using APSD shall have the Power Management bit in the Frame Control field set to 1 for buffering to take place at the QAP. | AP |
| 11.2.1.4 | 451 | 493 | 54 | Non-AP QSTAs may use U-APSD to have some or all of their frames delivered during unscheduled SPs. | STA |
| 11.2.1.4 | 452 | 494 | 1 | Non-AP QSTAs may use S-APSD to schedule delivery of some or all of their frames during scheduled SPs. | STA |
| 11.2.1.4 | 452 | 494 | 4 | If there is no unscheduled SP in progress, the unscheduled SP begins when the QAP receives a trigger frame from a non-AP QSTA, which is a QoS data or QoS Null frame associated with an AC the STA has configured to be trigger-enabled. | AP |
| 11.2.1.4 | 452 | 494 | 4 | If there is no unscheduled SP in progress, the unscheduled SP begins when the QAP receives a trigger frame from a non-AP QSTA, which is a QoS data or QoS Null frame associated with an AC the STA has configured to be trigger-enabled. | STA |
| 11.2.1.4 | 452 | 494 | 6 | An unscheduled SP ends after the QAP has attempted to transmit at least one MSDU or MMPDU associated with a delivery-enabled AC and destined for the non-AP QSTA, but no more than the number indicated in the Max SP Length field if the field has a nonzero value. | AP |
| 11.2.1.4 | 452 | 494 | 6 | An unscheduled SP ends after the QAP has attempted to transmit at least one MSDU or MMPDU associated with a delivery-enabled AC and destined for the non-AP QSTA, but no more than the number indicated in the Max SP Length field if the field has a nonzero value. | STA |
| 11.2.1.4 | 452 | 494 | 10 | In order to configure a QAP to deliver frames during an unscheduled SP, the non-AP QSTA designates one or more of its ACs to be delivery-enabled and one or more of its AC to be trigger-enabled. | an AP |
| 11.2.1.4 | 452 | 494 | 10 | In order to configure a QAP to deliver frames during an unscheduled SP, the non-AP QSTA designates one or more of its ACs to be delivery-enabled and one or more of its AC to be trigger-enabled. | STA |
| 11.2.1.4 | 452 | 494 | 11 | A non-AP QSTA may configure a QAP to use U-APSD using two methods. | STA |
| 11.2.1.4 | 452 | 494 | 11 | A non-AP QSTA may configure a QAP to use U-APSD using two methods. | an AP |
| 11.2.1.4 | 452 | 494 | 13 | First, a non-AP QSTA may set individual UAPSD Flag bits in the QoS Info subfield of the QoS Capability element carried in (Re)Association Request Frames | STA |
| 11.2.1.4 | 452 | 494 | 15 | When all four U-APSD Flag subfields are set to 1 in (Re)Association Request frames, all the ACs associated with the non-AP QSTA are trigger- and delivery-enabled during (re)association. | STA |
| 11.2.1.4 | 452 | 494 | 17 | When all four UAPSD Flag subfields are set to 0 in (Re)Association Request frames, none of the ACs associated with the non-AP QSTA is trigger- or delivery-enabled during (re)association. | STA |
| 11.2.1.4 | 452 | 494 | 21 | Alternatively, a non-AP QSTA may designate one or more AC as trigger-enabled and one or more AC as delivery-enabled by sending an ADDTS Request frame per AC to the QAP with the APSD subfield set to 1 and the Schedule subfield set to 0 in the TS Info field in the TSPEC element. | STA |
| 11.2.1.4 | 452 | 494 | 21 | Alternatively, a non-AP QSTA may designate one or more AC as trigger-enabled and one or more AC as delivery-enabled by sending an ADDTS Request frame per AC to the QAP with the APSD subfield set to 1 and the Schedule subfield set to 0 in the TS Info field in the TSPEC element. | AP |
| 11.2.1.4 | 452 | 494 | 28 | A non-AP QSTA may set an AC to be trigger- or delivery-enabled for its own use by setting up TSPECs with the APSD subfield set to 1 and the Schedule subfield set to 0 in the uplink or downlink direction, respectively. | STA |
| 11.2.1.4 | 452 | 494 | 35 | In order to use a scheduled SP for a TS when the access policy is controlled channel access, a non-AP QSTA shall send an ADDTS Request frame to the QAP with the APSD and Schedule subfields of the TS Info field in the TSPEC element both set to 1. | STA |
| 11.2.1.4 | 452 | 494 | 35 | In order to use a scheduled SP for a TS when the access policy is controlled channel access, a non-AP QSTA shall send an ADDTS Request frame to the QAP with the APSD and Schedule subfields of the TS Info field in the TSPEC element both set to 1. | AP |
| 11.2.1.4 | 452 | 494 | 38 | To use a scheduled SP for a TS for a AC when the access policy is contention-based channel access, a non-AP QSTA shall send an ADDTS Request frame to the QAP with the APSD and Schedule subfields of the TS Info field in the TSPEC element both set to 1. | STA |
| 11.2.1.4 | 452 | 494 | 38 | To use a scheduled SP for a TS for a AC when the access policy is contention-based channel access, a non-AP QSTA shall send an ADDTS Request frame to the QAP with the APSD and Schedule subfields of the TS Info field in the TSPEC element both set to 1. | AP |
| 11.2.1.4 | 452 | 494 | 41 | If the APSD mechanism is supported by the QAP and the QAP accepts the corresponding ADDTS Request frame from the non-AP QSTA, the QAP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the QAP. | AP |
| 11.2.1.4 | 452 | 494 | 41 | If the APSD mechanism is supported by the QAP and the QAP accepts the corresponding ADDTS Request frame from the non-AP QSTA, the QAP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the QAP. | AP |
| 11.2.1.4 | 452 | 494 | 41 | If the APSD mechanism is supported by the QAP and the QAP accepts the corresponding ADDTS Request frame from the non-AP QSTA, the QAP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the QAP. | STA |
| 11.2.1.4 | 452 | 494 | 41 | If the APSD mechanism is supported by the QAP and the QAP accepts the corresponding ADDTS Request frame from the non-AP QSTA, the QAP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the QAP. | AP |
| 11.2.1.4 | 452 | 494 | 41 | If the APSD mechanism is supported by the QAP and the QAP accepts the corresponding ADDTS Request frame from the non-AP QSTA, the QAP shall respond to the ADDTS Request frame with a response containing the Schedule element indicating that the requested service can be accommodated by the QAP. | AP |
| 11.2.1.4 | 452 | 494 | 45 | A non-AP QSTA using scheduled SP shall first wake up to receive downlink unicast frames buffered and/or polls from the AP/HC. | STA |
| 11.2.1.4 | 452 | 494 | 47 | The QAP may modify the service start time by indicating so in the Schedule element in ADDTS Response frame and in Schedule frames. | AP |
| 11.2.1.4 | 453 | 495 | 1 | If scheduled services periods are supported in a QBSS, a QSTA may use both unscheduled and scheduled APSD on different ACs at the same time. | BSS |
| 11.2.1.4 | 453 | 495 | 1 | If scheduled services periods are supported in a QBSS, a QSTA may use both unscheduled and scheduled APSD on different ACs at the same time. | STA |
| 11.2.1.4 | 453 | 495 | 2 | When a non-AP QSTA establishes scheduled delivery for an AC, that AC shall be considered delivery-enabled. | STA |
| 11.2.1.4 | 453 | 495 | 3 | However, the QAP shall not transmit frames associated with that AC during an SP that is initiated by a trigger frame, and it shall not treat frames associated with the AC that are received from the STA as trigger frames. | AP |
| 11.2.1.4 | 453 | 495 | 5 | The QAP shall decline any ADDTS Request frame that indicates the use of both scheduled and unscheduled APSD to be used on the same AC at the same time. | AP |
| 11.2.1.4 | 453 | 495 | 8 | APSD shall be used only to deliver unicast frames to a QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 14 | QAPs that implement and signal their support of APSD shall maintain an APSD and an access policy status for each currently associated non-AP QSTA that indicates whether the non-AP QSTA is presently using APSD and shall maintain the schedule (if any) for the non-AP QSTA. | APs |
| 11.2.1.5 | 453 | 495 | 14 | QAPs that implement and signal their support of APSD shall maintain an APSD and an access policy status for each currently associated non-AP QSTA that indicates whether the non-AP QSTA is presently using APSD and shall maintain the schedule (if any) for the non-AP QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 14 | QAPs that implement and signal their support of APSD shall maintain an APSD and an access policy status for each currently associated non-AP QSTA that indicates whether the non-AP QSTA is presently using APSD and shall maintain the schedule (if any) for the non-AP QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 14 | QAPs that implement and signal their support of APSD shall maintain an APSD and an access policy status for each currently associated non-AP QSTA that indicates whether the non-AP QSTA is presently using APSD and shall maintain the schedule (if any) for the non-AP QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 18 | A QAP implementing APSD shall, if a non-AP QSTA is using APSD and is in PS mode, temporarily buffer the MSDU or management frames destined to that non-AP QSTA. | An AP |
| 11.2.1.5 | 453 | 495 | 18 | A QAP implementing APSD shall, if a non-AP QSTA is using APSD and is in PS mode, temporarily buffer the MSDU or management frames destined to that non-AP QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 18 | A QAP implementing APSD shall, if a non-AP QSTA is using APSD and is in PS mode, temporarily buffer the MSDU or management frames destined to that non-AP QSTA. | STA |
| 11.2.1.5 | 453 | 495 | 24 | MSDUs, or management frames, destined for PS QSTAs using APSD shall be temporarily buffered in the APSD-capable QAP. | STAs |
| 11.2.1.5 | 453 | 495 | 24 | MSDUs, or management frames, destined for PS QSTAs using APSD shall be temporarily buffered in the APSD-capable QAP. | AP |
| 11.2.1.5 | 453 | 495 | 32 | At every beacon interval, the APSD-capable QAP shall assemble the partial virtual bitmap containing the buffer status of nondelivery-enabled ACs (if there exists at least one nondelivery-enabled AC) per destination for non-AP QSTAs in PS mode and shall send this out in the TIM field of the Beacon frame. | AP |
| 11.2.1.5 | 453 | 495 | 32 | At every beacon interval, the APSD-capable QAP shall assemble the partial virtual bitmap containing the buffer status of nondelivery-enabled ACs (if there exists at least one nondelivery-enabled AC) per destination for non-AP QSTAs in PS mode and shall send this out in the TIM field of the Beacon frame. | STAs |
| 11.2.1.5 | 453 | 495 | 35 | When all ACs are delivery-enabled, the APSD-capable QAP shall assemble the partial virtual bitmap containing the buffer status for all ACs per destination for non-AP QSTAs. | AP |
| 11.2.1.5 | 453 | 495 | 35 | When all ACs are delivery-enabled, the APSD-capable QAP shall assemble the partial virtual bitmap containing the buffer status for all ACs per destination for non-AP QSTAs. | STAs |
| 11.2.1.5 | 453 | 495 | 37 | d) If a non-AP QSTA has set up a scheduled SP, it shall automatically wake up at each SP. | STA |
| 11.2.1.5 | 453 | 495 | 37 | Therefore, the APSD-capable QAP shall transmit frames associated with admitted traffic with the APSD subfield set to 1 in the TSPECs buffered for the non-AP QSTA during a scheduled SP. | AP |
| 11.2.1.5 | 453 | 495 | 37 | Therefore, the APSD-capable QAP shall transmit frames associated with admitted traffic with the APSD subfield set to 1 in the TSPECs buffered for the non-AP QSTA during a scheduled SP. | STA |
| 11.2.1.5 | 453 | 495 | 40 | If the non-AP QSTA has set up to use unscheduled SPs, the QAP shall buffer frames belonging to deliveryenabled ACs until it has received a trigger frame associated with a trigger-enabled AC from the non-AP QSTA, which indicates the start of an unscheduled SP. | STA |
| 11.2.1.5 | 453 | 495 | 40 | If the non-AP QSTA has set up to use unscheduled SPs, the QAP shall buffer frames belonging to deliveryenabled ACs until it has received a trigger frame associated with a trigger-enabled AC from the non-AP QSTA, which indicates the start of an unscheduled SP. | AP |
| 11.2.1.5 | 453 | 495 | 40 | If the non-AP QSTA has set up to use unscheduled SPs, the QAP shall buffer frames belonging to deliveryenabled ACs until it has received a trigger frame associated with a trigger-enabled AC from the non-AP QSTA, which indicates the start of an unscheduled SP. | STA |
| 11.2.1.5 | 453 | 495 | 43 | A trigger frame received by the QAP from a non-AP QSTA that already has an unscheduled SP underway shall not trigger the start of a new unscheduled SP. | AP |
| 11.2.1.5 | 453 | 495 | 43 | A trigger frame received by the QAP from a non-AP QSTA that already has an unscheduled SP underway shall not trigger the start of a new unscheduled SP. | STA |
| 11.2.1.5 | 453 | 495 | 45 | The QAP transmits frames destined for the non-AP QSTA and associated with delivery-enabled ACs during an unscheduled SP. | AP |
| 11.2.1.5 | 453 | 495 | 45 | The QAP transmits frames destined for the non-AP QSTA and associated with delivery-enabled ACs during an unscheduled SP. | STA |
| 11.2.1.5 | 454 | 496 | 6 | For a non-AP QSTAs using U-APSD, the QAP transmits one frame destined for the non-AP QSTA from any AC that is not delivery-enabled in response to PS-Poll from the non-AP QSTA. | STAs |
| 11.2.1.5 | 454 | 496 | 6 | For a non-AP QSTAs using U-APSD, the QAP transmits one frame destined for the non-AP QSTA from any AC that is not delivery-enabled in response to PS-Poll from the non-AP QSTA. | AP |
| 11.2.1.5 | 454 | 496 | 6 | For a non-AP QSTAs using U-APSD, the QAP transmits one frame destined for the non-AP QSTA from any AC that is not delivery-enabled in response to PS-Poll from the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 6 | For a non-AP QSTAs using U-APSD, the QAP transmits one frame destined for the non-AP QSTA from any AC that is not delivery-enabled in response to PS-Poll from the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 8 | When all ACs associated with the non-AP QSTA are delivery-enabled, QAP transmits one frame from the highest priority AC. | STA |
| 11.2.1.5 | 454 | 496 | 8 | When all ACs associated with the non-AP QSTA are delivery-enabled, QAP transmits one frame from the highest priority AC. | AP |
| 11.2.1.5 | 454 | 496 | 14 | For a non-AP QSTA using U-APSD, the More Data field shall be set to indicate the presence of further buffered MSDUs or management frames that do not belong to delivery-enabled ACs. | STA |
| 11.2.1.5 | 454 | 496 | 16 | When all ACs associated with the non-AP QSTA are delivery-enabled, the More Data field shall be set to indicate the presence of further buffered MSDUs or management frames belonging to delivery-enabled ACs. | STA |
| 11.2.1.5 | 454 | 496 | 18 | If there are buffered frames to transmit to the STA, the QAP may set the More Data bit in a QoS +CF-Ack frame to 1, in response to a QoS data frame to indicate that it has one or more pending frames buffered for the PS STA identified by the RA address in the QoS +CF-Ack frame. | AP |
| 11.2.1.5 | 454 | 496 | 21 | A QAP may also set the More Data bit in an ACK frame in response to a QoS data frame to indicate that it has one or more pending frames buffered for the PS STA identified by the RA address in the ACK frame, if that PS STA has set the More Data Ack subfield in the QoS Capability information element to 1. | An AP |
| 11.2.1.5 | 454 | 496 | 31 | h) At each scheduled APSD SP for a non-AP QSTA, the APSD-capable QAP shall attempt to transmit at least one MSDU or MMPDU, associated with admitted TSPECs with the APSD and Schedule subfields both set to 1, that are destined for the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 31 | h) At each scheduled APSD SP for a non-AP QSTA, the APSD-capable QAP shall attempt to transmit at least one MSDU or MMPDU, associated with admitted TSPECs with the APSD and Schedule subfields both set to 1, that are destined for the non-AP QSTA. | AP |
| 11.2.1.5 | 454 | 496 | 31 | h) At each scheduled APSD SP for a non-AP QSTA, the APSD-capable QAP shall attempt to transmit at least one MSDU or MMPDU, associated with admitted TSPECs with the APSD and Schedule subfields both set to 1, that are destined for the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 33 | At each unscheduled SP for a non-AP QSTA, the QAP shall attempt to transmit at least one MSDU or MMPDU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 33 | At each unscheduled SP for a non-AP QSTA, the QAP shall attempt to transmit at least one MSDU or MMPDU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the non-AP QSTA. | AP |
| 11.2.1.5 | 454 | 496 | 33 | At each unscheduled SP for a non-AP QSTA, the QAP shall attempt to transmit at least one MSDU or MMPDU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the non-AP QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 37 | The More Data bit of the directed data or management frame associated with delivery-enabled ACs and destined for that non-AP QSTA indicates that more frames are buffered for the delivery-enabled ACs. | STA |
| 11.2.1.5 | 454 | 496 | 40 | The More Data bit set in MSDUs or management frames associated with nondelivery-enabled ACs and destined for that non-AP QSTA indicates that more frames are buffered for the nondelivery-enabled ACs. | STA |
| 11.2.1.5 | 454 | 496 | 43 | A QAP may also set the More Data bit to 1 in a QoS +CF-Ack frame in response to a QoS data frame to indicate that it has one or more pending frames buffered for the target STA identified by the RA address in the QoS +CF-Ack frame. | An AP |
| 11.2.1.5 | 454 | 496 | 51 | The QAP considers APSD QSTA to be in awake state after it has sent a QoS +CF-Ack frame, with the EOSP subfield in the QoS Control field set to 0, to the APSD QSTA. | AP |
| 11.2.1.5 | 454 | 496 | 51 | The QAP considers APSD QSTA to be in awake state after it has sent a QoS +CF-Ack frame, with the EOSP subfield in the QoS Control field set to 0, to the APSD QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 51 | The QAP considers APSD QSTA to be in awake state after it has sent a QoS +CF-Ack frame, with the EOSP subfield in the QoS Control field set to 0, to the APSD QSTA. | STA |
| 11.2.1.5 | 454 | 496 | 52 | If necessary, the QAP may generate an extra QoS Null frame, with the EOSP set to 1. | AP |
| 11.2.1.5 | 454 | 496 | 53 | When the QAP has transmitted a directed frame to the non-AP QSTA with the EOSP subfield set to 1 during the SP except for retransmissions of that frame, the QAP shall not transmit any more frames using this mechanism until the next SP. | AP |
| 11.2.1.5 | 454 | 496 | 53 | When the QAP has transmitted a directed frame to the non-AP QSTA with the EOSP subfield set to 1 during the SP except for retransmissions of that frame, the QAP shall not transmit any more frames using this mechanism until the next SP. | STA |
| 11.2.1.5 | 454 | 496 | 53 | When the QAP has transmitted a directed frame to the non-AP QSTA with the EOSP subfield set to 1 during the SP except for retransmissions of that frame, the QAP shall not transmit any more frames using this mechanism until the next SP. | AP |
| 11.2.1.5 | 455 | 497 | 1 | The QAP shall set the EOSP subfield to 1 to indicate the end of the SP in APSD. | AP |
| 11.2.1.5 | 455 | 497 | 3 | i) If the QAP does not receive an acknowledgment to a directed MSDU or management frame sent to a non-AP QSTA in PS mode following receipt of a PS-Poll from that non-AP QSTA, it may retransmit the frame for at most the lesser of the maximum retry limit and the MIB attribute dot11QAPMissingAckRetryLimit times before the next beacon, but it shall retransmit that frame at least once before the next beacon, time permitting and subject to its appropriate lifetime limit. | AP |
| 11.2.1.5 | 455 | 497 | 3 | i) If the QAP does not receive an acknowledgment to a directed MSDU or management frame sent to a non-AP QSTA in PS mode following receipt of a PS-Poll from that non-AP QSTA, it may retransmit the frame for at most the lesser of the maximum retry limit and the MIB attribute dot11QAPMissingAckRetryLimit times before the next beacon, but it shall retransmit that frame at least once before the next beacon, time permitting and subject to its appropriate lifetime limit. | STA |
| 11.2.1.5 | 455 | 497 | 3 | i) If the QAP does not receive an acknowledgment to a directed MSDU or management frame sent to a non-AP QSTA in PS mode following receipt of a PS-Poll from that non-AP QSTA, it may retransmit the frame for at most the lesser of the maximum retry limit and the MIB attribute dot11QAPMissingAckRetryLimit times before the next beacon, but it shall retransmit that frame at least once before the next beacon, time permitting and subject to its appropriate lifetime limit. | STA |
| 11.2.1.5 | 455 | 497 | 11 | j) If the QAP does not receive an acknowledgment to a directed MSDU sent with the EOSP subfield set to 1, it shall retransmit that frame at least once within the same SP, subject to applicable retry or lifetime limit. | AP |
| 11.2.1.5 | 455 | 497 | 18 | The QAP may base the aging function on the listen interval specified by the non-AP QSTA in the (Re)Association Request frame. | AP |
| 11.2.1.5 | 455 | 497 | 18 | The QAP may base the aging function on the listen interval specified by the non-AP QSTA in the (Re)Association Request frame. | STA |
| 11.2.1.5 | 455 | 497 | 21 | When a QAP is informed that an APSD-capable non-AP QSTA is not using APSD, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current PS mode of the non-AP QSTA. | an AP |
| 11.2.1.5 | 455 | 497 | 21 | When a QAP is informed that an APSD-capable non-AP QSTA is not using APSD, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current PS mode of the non-AP QSTA. | STA |
| 11.2.1.5 | 455 | 497 | 21 | When a QAP is informed that an APSD-capable non-AP QSTA is not using APSD, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current PS mode of the non-AP QSTA. | AP |
| 11.2.1.5 | 455 | 497 | 21 | When a QAP is informed that an APSD-capable non-AP QSTA is not using APSD, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current PS mode of the non-AP QSTA. | STA |
| 11.2.1.5 | 455 | 497 | 21 | When a QAP is informed that an APSD-capable non-AP QSTA is not using APSD, then the QAP shall send buffered MSDUs and management frames (if any exist) to that non-AP QSTA according to the rules corresponding to the current PS mode of the non-AP QSTA. | STA |
| 11.2.1.7 | 456 | 498 | 29 | When a non-AP QSTA that is using U-APSD and has all ACs deliveryenabled detects that the bit corresponding to its AID is set in the TIM, the non-AP QSTA shall issue a trigger frame or a PS-Poll frame to retrieve the buffered MSDU or management frames. | STA |
| 11.2.1.7 | 456 | 498 | 29 | When a non-AP QSTA that is using U-APSD and has all ACs deliveryenabled detects that the bit corresponding to its AID is set in the TIM, the non-AP QSTA shall issue a trigger frame or a PS-Poll frame to retrieve the buffered MSDU or management frames. | STA |
| 11.2.1.7 | 456 | 498 | 40 | If a non-AP QSTA receives a QoS +CF-Ack frame from its QAP with the More Data bit set to 1, then the QSTA shall operate exactly as if it received a TIM with its AID bit set. | STA |
| 11.2.1.7 | 456 | 498 | 40 | If a non-AP QSTA receives a QoS +CF-Ack frame from its QAP with the More Data bit set to 1, then the QSTA shall operate exactly as if it received a TIM with its AID bit set. | AP |
| 11.2.1.7 | 456 | 498 | 40 | If a non-AP QSTA receives a QoS +CF-Ack frame from its QAP with the More Data bit set to 1, then the QSTA shall operate exactly as if it received a TIM with its AID bit set. | STA |
| 11.2.1.7 | 456 | 498 | 41 | If a non-AP QSTA has set the More Data Ack subfield in QoS Capability information element to 1, then if it receives an ACK frame from its QAP with the More Data bit set to 1, the QSTA shall operate exactly as if it received a TIM with its AID bit set. | STA |
| 11.2.1.7 | 456 | 498 | 41 | If a non-AP QSTA has set the More Data Ack subfield in QoS Capability information element to 1, then if it receives an ACK frame from its QAP with the More Data bit set to 1, the QSTA shall operate exactly as if it received a TIM with its AID bit set. | AP |
| 11.2.1.7 | 456 | 498 | 41 | If a non-AP QSTA has set the More Data Ack subfield in QoS Capability information element to 1, then if it receives an ACK frame from its QAP with the More Data bit set to 1, the QSTA shall operate exactly as if it received a TIM with its AID bit set. | STA |
| 11.2.1.7 | 456 | 498 | 43 | For example, a QSTA that is using the PS-Poll delivery method shall issue a PS-Poll frame to retrieve a buffered frame. | STA |
| 11.2.1.9 | 457 | 499 | 16 | Receive operation for non-AP QSTAs using APSD | STAs |
| 11.2.1.9 | 457 | 499 | 19 | A non-AP QSTA using APSD shall operate as follows to receive an MSDU or management frame from the QAP: | STA |
| 11.2.1.9 | 457 | 499 | 19 | A non-AP QSTA using APSD shall operate as follows to receive an MSDU or management frame from the QAP: | AP |
| 11.2.1.9 | 457 | 499 | 22 | a) If a scheduled SP has been set up, the non-AP QSTA wakes up at its scheduled start time. (The non-AP QSTA shall wake up early enough to receive transmissions at the scheduled SP.) | STA |
| 11.2.1.9 | 457 | 499 | 22 | a) If a scheduled SP has been set up, the non-AP QSTA wakes up at its scheduled start time. (The non-AP QSTA shall wake up early enough to receive transmissions at the scheduled SP.) | STA |
| 11.2.1.9 | 457 | 499 | 24 | b) If the non-AP QSTA is initiating an unscheduled SP, the non-AP QSTA wakes up and transmits a trigger frame to the QAP. When one or more ACs are not delivery-enabled, the non-AP QSTA may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the QAP. | STA |
| 11.2.1.9 | 457 | 499 | 24 | b) If the non-AP QSTA is initiating an unscheduled SP, the non-AP QSTA wakes up and transmits a trigger frame to the QAP. When one or more ACs are not delivery-enabled, the non-AP QSTA may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the QAP. | STA |
| 11.2.1.9 | 457 | 499 | 24 | b) If the non-AP QSTA is initiating an unscheduled SP, the non-AP QSTA wakes up and transmits a trigger frame to the QAP. When one or more ACs are not delivery-enabled, the non-AP QSTA may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the QAP. | AP |
| 11.2.1.9 | 457 | 499 | 24 | b) If the non-AP QSTA is initiating an unscheduled SP, the non-AP QSTA wakes up and transmits a trigger frame to the QAP. When one or more ACs are not delivery-enabled, the non-AP QSTA may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the QAP. | STA |
| 11.2.1.9 | 457 | 499 | 24 | b) If the non-AP QSTA is initiating an unscheduled SP, the non-AP QSTA wakes up and transmits a trigger frame to the QAP. When one or more ACs are not delivery-enabled, the non-AP QSTA may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the QAP. | AP |
| 11.2.1.9 | 457 | 499 | 28 | c) The non-AP QSTA shall remain awake until it receives a QoS data frame addressed to it, with the EOSP subfield in the QoS Control field set to 1. | STA |
| 11.2.1.9 | 457 | 499 | 30 | d) The non-AP QSTA may send additional PS-Poll frames if the More Data subfield is set to 1 in downlink unicast data or management frames that do not belong to any deliver-enabled ACs. The non-AP QSTA may send additional trigger frames if the More Data subfield is set to 1 in downlink unicast data or management frames that belong to delivery-enabled ACs. | STA |
| 11.2.1.9 | 457 | 499 | 30 | d) The non-AP QSTA may send additional PS-Poll frames if the More Data subfield is set to 1 in downlink unicast data or management frames that do not belong to any deliver-enabled ACs. The non-AP QSTA may send additional trigger frames if the More Data subfield is set to 1 in downlink unicast data or management frames that belong to delivery-enabled ACs. | STA |
| 11.3 | 463 | 505 | 4 | ii) QoS data subtypes allowed to/from non-AP QSTA(s) that are associated with QAP(s). | STA |
| 11.3 | 463 | 505 | 4 | ii) QoS data subtypes allowed to/from non-AP QSTA(s) that are associated with QAP(s). | AP |
| 11.3 | 463 | 505 | 5 | iii) Data frames between QSTAs in a QBSS with FC bits “To DS” and “From DS” both false. | STAs |
| 11.3 | 463 | 505 | 5 | iii) Data frames between QSTAs in a QBSS with FC bits “To DS” and “From DS” both false. | BSS |
|  |  |  |  | Note: Traffic Stream operation (11.4) is specific to QoS STAs and QoS APs. Use of “Q” is redundant. |  |
| 11.4.1 | 468 | 510 | 35 | A TS may have one or more TCLAS (within the discretion of the QSTA that sets up the stream) associated with it. | STA |
| 11.4.1 | 468 | 510 | 36 | The QAP uses the parameters in the TCLAS elements to filter the MSDUs belonging to this TS so that they can be delivered with the QoS parameters that have been set up for the TS. | AP |
| 11.4.1 | 468 | 510 | 42 | Following a successful negotiation, a TS is created, identified within the non-AP QSTA by its TSID and direction, and identified within the HC by a combination of TSID, direction, and non-AP QSTA address. | STA |
| 11.4.1 | 468 | 510 | 46 | It is always the responsibility of the non-AP QSTA to initiate the creation of a TS regardless of its direction. | STA |
| 11.4.1 | 468 | 510 | 47 | In the direct-link case, it is the responsibility of the non-AP QSTA that is going to send the data to create the TS. | STA |
| 11.4.1 | 468 | 510 | 48 | In this case, the non-AP QSTA negotiates with the HC to gain TXOPs that it uses to send the data. | STA |
| 11.4.1 | 468 | 510 | 48 | There is no negotiation between the originator and recipient non-AP QSTAs concerning the TS: the originator can discover the capabilities of the recipient (rates, BlockAck) using the DLS. | STAs |
| 11.4.1 | 468 | 510 | 52 | In the case of traffic relayed by a QAP, the sending and receiving non-AP QSTAs may both create individual TS for the traffic. | an AP |
| 11.4.1 | 468 | 510 | 52 | In the case of traffic relayed by a QAP, the sending and receiving non-AP QSTAs may both create individual TS for the traffic. | STAs |
| 11.4.1 | 469 | 511 | 1 | A non-AP QSTA may simultaneously support up to eight TSs from the HC to itself and up to eight TSs from itself to other QSTAs, including the HC. | STA |
| 11.4.1 | 469 | 511 | 1 | A non-AP QSTA may simultaneously support up to eight TSs from the HC to itself and up to eight TSs from itself to other QSTAs, including the HC. | STAs |
| 11.4.1 | 469 | 511 | 5 | A HC may simultaneously support up to eight downlink TSs and up to eight uplink TSs per associated non-AP QSTA. | STA |
| 11.4.3 | 470 | 512 | 1 | Initially a TS is inactive. A QSTA shall not transmit any QoS data frames using an inactive TS. | STA |
| 11.4.3 | 470 | 512 | 3 | Following a successful TS setup initiated by the non-AP QSTA, the TS becomes active, and either the non-AP QSTA or the HC may transmit QoS data frames using this TSID (according to the Direction field). | STA |
| 11.4.3 | 470 | 512 | 3 | Following a successful TS setup initiated by the non-AP QSTA, the TS becomes active, and either the non-AP QSTA or the HC may transmit QoS data frames using this TSID (according to the Direction field). | STA |
| 11.4.3 | 470 | 512 | 7 | While the TS is active, the parameters of the TSPEC characterizing the TS can be renegotiated, when the renegotiation is initiated by the non-AP QSTA. | STA |
| 11.4.3 | 470 | 512 | 11 | An active TS becomes inactive following a TS deletion process initiated at either non-AP QSTA or HC. | STA |
| 11.4.4 | 470 | 512 | 46 | The non-AP QSTA SME decides that a TS needs to be created. | STA |
| 11.4.3 | 470 | 512 | 53 | The non-AP QSTA MAC transmits the TSPEC in an ADDTS Request frame to the HC and starts a response timer called ADDTS timer of duration dot11ADDTSResponseTimout. | STA |
| 11.4.3 | 471 | 513 | 25 | The non-AP QSTA MAC receives this management frame and cancels its ADDTS timer. | STA |
| 11.4.3 | 471 | 513 | 28 | The non-AP QSTA SME decides whether the response meets its needs. | STA |
| 11.4.3 | 471 | 513 | 29 | Otherwise, the whole process can be repeated using the same TSID and direction and a modified TSPEC until the non-AP QSTA SME decides that the granted TXOP is adequate or inadequate and cannot be improved. | STA |
| 11.4.3 | 471 | 513 | 37 | If the AP/HC grants a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block Ack and the Direction field set to either downlink or bidirectional, then it shall initiate a Block Ack negotiation by sending an ADDBA Request frame to the non-AP QSTA that originated the ADDTS Request frame. | STA |
| 11.4.3 | 471 | 513 | 39 | If a non-AP QSTA is granted a TXOP for an ADDTS Request frame with the Ack Policy subfield set to Block Ack and the Direction field set to other than downlink, then it shall initiate a Block Ack negotiation by sending an ADDBA Request frame to the recipient of the TS. | STA |
| 11.4.5 | 471 | 513 | 55 | In both cases, if the request is not for an existing TS, the non-AP QSTA MAC shall send a DELTS to the HC specifying the TSID and direction of the failed request just in case the HC had received the request and it was the response that was lost. | STA |
| 11.4.6 | 472 | 514 | 45 | In the case of a non-AP QSTA, the MSDUs are transmitted using QoS data frames, when the non-AP QSTA is polled by the HC. | STA |
| 11.4.6 | 472 | 514 | 45 | In the case of a non-AP QSTA, the MSDUs are transmitted using QoS data frames, when the non-AP QSTA is polled by the HC. | STA |
| 11.4.7 | 473 | 515 | 25 | QSTA_LEAVING | STA_LEAVING |
| 11.4.7 | 473 | 515 | 16 | There are two types of TS deletion: non-AP QSTA-initiated and HC-initiated. | STA |
| 11.4.7 | 473 | 515 | 37 | Figure 205 shows the case of TS deletion initiated by the non-AP QSTA and the case of TS deletion initiated by the HC. | STA |
| 11.4.7 | 473 | 515 | 42 | Reassociation causes the non-AP QSTA and QAP to clear their state, and the non-AP QSTA will have to reinitiate the setup. | STA |
| 11.4.7 | 473 | 515 | 42 | Reassociation causes the non-AP QSTA and QAP to clear their state, and the non-AP QSTA will have to reinitiate the setup. | AP |
| 11.4.7 | 473 | 515 | 42 | Reassociation causes the non-AP QSTA and QAP to clear their state, and the non-AP QSTA will have to reinitiate the setup. | STA |
| 11.4.8 | 474 | 516 | 44 | This is to ensure that the non-AP QSTA is actually using the assigned TXOP for the given TSID. | STA |
| 11.4.8 | 474 | 516 | 48 | Any other use of a polled TXOP delivered to the non-AP QSTA is considered to be activity on all direct-link TS associated with that non-AP QSTA. | STA |
| 11.4.8 | 474 | 516 | 48 | Any other use of a polled TXOP delivered to the non-AP QSTA is considered to be activity on all direct-link TS associated with that non-AP QSTA. | STA |
| 11.4.8 | 474 | 516 | 51 | In response to an inactivity timeout, the HC shall send a DELTS frame to the non-AP QSTA with the result code set to TIMEOUT and inform its SME using the MLME-DELTS.indication primitive. | STA |
| 11.4.10 | 475 | 517 | 11 | For uplink and direct link, a suspended TS may be reinstated by a non-AP QSTA by sending a QoS data or QoS Null frame. | STA |
| 11.4.10 | 475 | 517 | 15 | For downlink, a suspended TS is reinstated by the HC when the QAP receives an MSDU from a higher layer. | AP |
|  |  |  |  | Note: Block Ack operation (11.5) is advertised separately in the Capability Information Element, and is already dependent on QoS capabilities at the STA and AP. Use of “Q” in QSTA/QAP in this clause is redundant. |  |
| 11.5.1 | 476 | 518 | 40 | Upon receipt of an MLME-ADDBA.request primitive, an initiating QSTA that intends to send QoS data frames under the Block Ack mechanism shall set up the Block Ack using the following procedure: | STA |
| 11.5.1 | 476 | 518 | 43 | a) Check whether the intended peer QSTA is capable of participating in the Block Ack mechanism by discovering and examining its “Delayed Block Ack” and “Immediate Block Ack” capability bits. | STA |
| 11.5.1 | 476 | 518 | 47 | b) If an ADDBA Response frame is received with the matching dialog token and the TID and with a status code set to a value of 0, the QSTA has established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive indicating the successful completion of the Block Ack setup. | STA |
| 11.5.1 | 476 | 518 | 47 | b) If an ADDBA Response frame is received with the matching dialog token and the TID and with a status code set to a value of 0, the QSTA has established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive indicating the successful completion of the Block Ack setup. | STA |
| 11.5.1.1 | 476 | 518 | 52 | c) If an ADDBA Response frame is received with the matching dialog token and the TID and with a status code set to a value other than 0, the QSTA has not established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive indicating the failure of the Block Ack setup. | STA |
| 11.5.1.1 | 476 | 518 | 52 | c) If an ADDBA Response frame is received with the matching dialog token and the TID and with a status code set to a value other than 0, the QSTA has not established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive indicating the failure of the Block Ack setup. | STA |
| 11.5.1.1 | 477 | 519 | 1 | d) If there is no response from the recipient within dot11ADDBAFailureTimeout, the QSTA has not established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive with a result code of TIMEOUT. | STA |
| 11.5.1.1 | 477 | 519 | 1 | d) If there is no response from the recipient within dot11ADDBAFailureTimeout, the QSTA has not established a Block Ack mechanism with the recipient QSTA; and the MLME shall issue an MLME-ADDBA.confirm primitive with a result code of TIMEOUT. | STA |
| 11.5.1.2 | 477 | 519 | 9 | a) When an ADDBA Request frame is received from another QSTA, the MLME shall issue an MLME-ADDBA.indication primitive. | STA |
| 11.5.1.2 | 477 | 519 | 11 | b) Upon receipt of the MLME-ADDBA.response primitive, the QSTA shall respond by an ADDBA Response frame with a result code as defined in 7.4.4.2. | STA |
| 11.5.2 | 477 | 519 | 34 | The procedure at the two QSTAs is described in 11.5.2.1 and 11.5.2.2 and illustrated in Figure 208. | STAs |
| 11.5.2.1 | 477 | 519 | 53 | Upon receipt of an MLME-DELBA.request primitive, the QSTA MAC/MLME shall tear down the Block Ack using the following procedure: | STA |
| 11.5.2.1 | 478 | 520 | 13 | QSTA_LEAVING | STA_LEAVING |
| 11.5.2.1 | 478 | 520 | 1 | a) The QSTA shall transmit a DELBA frame. | STA |
| 11.5.2.2 | 478 | 520 | 22 | A QSTA shall issue a MLME-DELBA.indication primitive with the parameter ReasonCode having a value of REQUESTED when a DELBA frame is received. | STA |
| 11.5.3 | 478 | 520 | 34 | When a timeout of BlockAckTimeout is detected, the QSTA shall send a DELBA frame to the peer QSTA with the Reason Code field set to TIMEOUT and shall issue a MLME-DELBA.indication primitive with the ReasonCode parameter having a value of TIMEOUT. | STA |
| 11.5.3 | 478 | 520 | 34 | When a timeout of BlockAckTimeout is detected, the QSTA shall send a DELBA frame to the peer QSTA with the Reason Code field set to TIMEOUT and shall issue a MLME-DELBA.indication primitive with the ReasonCode parameter having a value of TIMEOUT. | STA |
| 11.5.3 | 478 | 520 | 41 | If such a QSTA receives a BlockAckReq frame, it may respond with an ACK frame and shall respond with a DELBA frame using the normal access mechanisms. | STA |
| 11.5.3 | 478 | 520 | 53 | The originator QSTA may send an ADDBA Request frame in order to update Block ACK Timeout Value. | STA |
| 11.5.3 | 478 | 520 | 53 | If the updated ADDBA Request frame is accepted, both QSTAs initialize the timer to detect Block ACK timeout. | STA |
| 11.6.1 | 480 | 522 | 3 | However, a MLME-HL-SYNC.indication primitive allows the transmitting QSTA to know exactly when each Sync packet is transmitted. | STA |
| 11.6.1 | 480 | 522 | 4 | A receiving QSTA can similarly note the time when each Sync packet is received as well as the sequence number for that frame. | STA |
| 11.6.1 | 480 | 522 | 6 | The receiving QSTA would save this receive time indication for each packet along with its sequence number and compare the indication of the previously received Sync packet to the time stamp in the most currently received packet. | STA |
| 11.6.1 | 480 | 522 | 22 | In this case, the non-AP QSTA that generates the time stamps notes the reception of the multicast Sync frame from the AP as the indication to save the higher clock value for the next Sync frame. | STA |
| 11.6.1 | 480 | 522 | 24 | Receiving QSTAs would also similarly note the time when each Sync packet is received from the AP. | STAs |
| 11.6.2 | 480 | 522 | 31 | Procedure at the QSTA | STA |
|  |  |  |  | Note: Support for DLS (11.7) is already dependent on QoS support, so use of “Q” in QSTA/QAP is redundant. |  |
| 11.7 | 480 | 522 | 52 | However, STAs with QoS facility (i.e., QSTAs) may transmit frames directly to another QSTA by setting up such data transfer using DLS. | Delete |
| 11.7 | 480 | 522 | 52 | However, STAs with QoS facility (i.e., QSTAs) may transmit frames directly to another QSTA by setting up such data transfer using DLS. | STA |
| 11.7 | 481 | 523 | 13 | QAP | AP |
| 11.7 | 481 | 523 | 19 | QSTA-1 and QSTA-2 | STA (twice) |
| 11.7 | 480 | 522 | 53 | The need for this protocol is motivated by the fact that the intended recipient may be in PS mode, in which case it can be awakened only by the QAP. | AP |
| 11.7 | 481 | 523 | 8 | DLS does not apply in a QIBSS, where frames are always sent directly from one STA to another. | an IBSS |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | AP |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 24 | a) A STA, QSTA-1, that intends to exchange frames directly with another non-AP STA, QSTA-2, invokes DLS and sends a DLS Request frame to the QAP (step 1a in Figure 210). This request contains the rate set, capabilities of QSTA-1, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 28 | b) If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS, and QSTA-2 is indeed a QSTA, then the QAP forwards the DLS Request frame to the recipient, QSTA-2 (step 1b in Figure 210). | STA |
| 11.7 | 481 | 523 | 28 | b) If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS, and QSTA-2 is indeed a QSTA, then the QAP forwards the DLS Request frame to the recipient, QSTA-2 (step 1b in Figure 210). | STA |
| 11.7 | 481 | 523 | 28 | b) If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS, and QSTA-2 is indeed a QSTA, then the QAP forwards the DLS Request frame to the recipient, QSTA-2 (step 1b in Figure 210). | QoS STA |
| 11.7 | 481 | 523 | 28 | b) If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS, and QSTA-2 is indeed a QSTA, then the QAP forwards the DLS Request frame to the recipient, QSTA-2 (step 1b in Figure 210). | AP |
| 11.7 | 481 | 523 | 28 | b) If QSTA-2 is associated in the BSS, direct streams are allowed in the policy of the BSS, and QSTA-2 is indeed a QSTA, then the QAP forwards the DLS Request frame to the recipient, QSTA-2 (step 1b in Figure 210). | STA |
| 11.7 | 481 | 523 | 31 | c) If QSTA-2 accepts the direct stream, it sends a DLS Response frame to the QAP (step 2a in Figure 210), which contains the rate set, (extended) capabilities of QSTA-2, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 31 | c) If QSTA-2 accepts the direct stream, it sends a DLS Response frame to the QAP (step 2a in Figure 210), which contains the rate set, (extended) capabilities of QSTA-2, and the MAC addresses of QSTA-1 and QSTA-2. | AP |
| 11.7 | 481 | 523 | 31 | c) If QSTA-2 accepts the direct stream, it sends a DLS Response frame to the QAP (step 2a in Figure 210), which contains the rate set, (extended) capabilities of QSTA-2, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 31 | c) If QSTA-2 accepts the direct stream, it sends a DLS Response frame to the QAP (step 2a in Figure 210), which contains the rate set, (extended) capabilities of QSTA-2, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 31 | c) If QSTA-2 accepts the direct stream, it sends a DLS Response frame to the QAP (step 2a in Figure 210), which contains the rate set, (extended) capabilities of QSTA-2, and the MAC addresses of QSTA-1 and QSTA-2. | STA |
| 11.7 | 481 | 523 | 35 | d) The QAP forwards the DLS Response frame to QSTA-1 (step 2b in Figure 210), after which the direct link becomes active and frames can be sent from QSTA-1 to QSTA-2 and from QSTA-2 to QSTA-1. | AP |
| 11.7 | 481 | 523 | 35 | d) The QAP forwards the DLS Response frame to QSTA-1 (step 2b in Figure 210), after which the direct link becomes active and frames can be sent from QSTA-1 to QSTA-2 and from QSTA-2 to QSTA-1. | STA |
| 11.7 | 481 | 523 | 35 | d) The QAP forwards the DLS Response frame to QSTA-1 (step 2b in Figure 210), after which the direct link becomes active and frames can be sent from QSTA-1 to QSTA-2 and from QSTA-2 to QSTA-1. | STA (4 times) |
| 11.7 | 481 | 523 | 45 | When a QSTA transitions to a differnent QAP after a DLS is set up, the DLS shall be torn down as described in 11.7.4. | STA |
| 11.7 | 481 | 523 | 45 | When a QSTA transitions to a differnent QAP after a DLS is set up, the DLS shall be torn down as described in 11.7.4. | AP |
| 11.7 | 481 | 523 | 49 | Note in this case the DLS cannot be torn down because a teardown message cannot be sent because the QSTAs are not on the same QAP. | STAs |
| 11.7 | 481 | 523 | 49 | Note in this case the DLS cannot be torn down because a teardown message cannot be sent because the QSTAs are not on the same QAP. | AP |
| 11.7.1.1 | 482 | 524 | 22 | Setup procedure at the QSTA | STA |
| 11.7.1.1 | 482 | 524 | 24 | All QSTAs shall maintain a list of QSTAs with which a direct link has been established. | STAs |
| 11.7.1.1 | 482 | 524 | 24 | All QSTAs shall maintain a list of QSTAs with which a direct link has been established. | STAs |
| 11.7.1.1 | 482 | 524 | 26 | Upon receipt of an MLME-DLS.request primitive from the SME, the QSTA shall do one of the following actions: | STA |
| 11.7.1.1 | 482 | 524 | 28 | Issue an MLME-DLS.confirm primitive with a result code of SUCCESS if the peer MAC address of MLME-DLS.request primitive is in the list of QSTAs with which direct link has been established; or | STAs |
| 11.7.1.1 | 482 | 524 | 32 | If the QSTA does not receive a DLS Response frame within DLSResponseTimeout after sending the DLS Request frame, the QSTA shall issue an MLME-DLS.confirm primitive with the result code of TIMEOUT. | STA |
| 11.7.1.1 | 482 | 524 | 32 | If the QSTA does not receive a DLS Response frame within DLSResponseTimeout after sending the DLS Request frame, the QSTA shall issue an MLME-DLS.confirm primitive with the result code of TIMEOUT. | STA |
| 11.7.1.1 | 482 | 524 | 36 | Upon receipt of the DLS Request frame from the QAP (step 1b in Figure 210), the QSTA MAC shall send to the AP a DLS Response frame (step 2a in Figure 210) with a result code of | AP |
| 11.7.1.1 | 482 | 524 | 36 | Upon receipt of the DLS Request frame from the QAP (step 1b in Figure 210), the QSTA MAC shall send to the AP a DLS Response frame (step 2a in Figure 210) with a result code of | STA |
| 11.7.1.1 | 482 | 524 | 38 | SUCCESS if the QSTA is willing to participate in the direct link with the source QSTA. The QSTA shall also issue an MLME-DLS.indication primitive to the SME and shall add the source QSTA to the list of the QSTAs, if not already present, with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 38 | SUCCESS if the QSTA is willing to participate in the direct link with the source QSTA. The QSTA shall also issue an MLME-DLS.indication primitive to the SME and shall add the source QSTA to the list of the QSTAs, if not already present, with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 38 | SUCCESS if the QSTA is willing to participate in the direct link with the source QSTA. The QSTA shall also issue an MLME-DLS.indication primitive to the SME and shall add the source QSTA to the list of the QSTAs, if not already present, with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 38 | SUCCESS if the QSTA is willing to participate in the direct link with the source QSTA. The QSTA shall also issue an MLME-DLS.indication primitive to the SME and shall add the source QSTA to the list of the QSTAs, if not already present, with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 38 | SUCCESS if the QSTA is willing to participate in the direct link with the source QSTA. The QSTA shall also issue an MLME-DLS.indication primitive to the SME and shall add the source QSTA to the list of the QSTAs, if not already present, with which direct link has been established. | STAs |
| 11.7.1.1 | 482 | 524 | 41 | REFUSED if the QSTA is not willing to participate in the direct link. | STA |
| 11.7.1.1 | 482 | 524 | 44 | Upon receipt of the DLS Response frame from the QAP (step 2b in Figure 210), the QSTA shall issue an MLME-DLS.confirm primitive. | AP |
| 11.7.1.1 | 482 | 524 | 44 | Upon receipt of the DLS Response frame from the QAP (step 2b in Figure 210), the QSTA shall issue an MLME-DLS.confirm primitive. | STA |
| 11.7.1.1 | 482 | 524 | 46 | If the result code is SUCCESS, the direct link is considered to be established with the destination QSTA in the DLS Response frame, and the QSTA MAC shall add the destination QSTA to the list of QSTAs with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 46 | If the result code is SUCCESS, the direct link is considered to be established with the destination QSTA in the DLS Response frame, and the QSTA MAC shall add the destination QSTA to the list of QSTAs with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 46 | If the result code is SUCCESS, the direct link is considered to be established with the destination QSTA in the DLS Response frame, and the QSTA MAC shall add the destination QSTA to the list of QSTAs with which direct link has been established. | STA |
| 11.7.1.1 | 482 | 524 | 46 | If the result code is SUCCESS, the direct link is considered to be established with the destination QSTA in the DLS Response frame, and the QSTA MAC shall add the destination QSTA to the list of QSTAs with which direct link has been established. | STAs |
| 11.7.1.2 | 482 | 524 | 52 | Setup procedure at the QAP | AP |
| 11.7.1.2 | 482 | 524 | 54 | Upon receipt of the DLS Request frame (step 1a in Figure 210), the QAP shall | AP |
| 11.7.1.2 | 483 | 525 | 30 | NOT_QSTA | NOT_QoS_STA |
| 11.7.1.2 | 483 | 525 | 1 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not Allowed in the BSS, if direct links are not allowed in the QBSS (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 1 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not Allowed in the BSS, if direct links are not allowed in the QBSS (step 2b in Figure 210). | BSS |
| 11.7.1.2 | 483 | 525 | 3 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not Present, if the destination QSTA is not present in the QBSS (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 3 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not Present, if the destination QSTA is not present in the QBSS (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 3 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not Present, if the destination QSTA is not present in the QBSS (step 2b in Figure 210). | BSS |
| 11.7.1.2 | 483 | 525 | 6 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not a QSTA, if the destination STA does not have QoS facility (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 6 | Send DLS Response frame to the QSTA that sent the DLS Request frame with a result code of Not a QSTA, if the destination STA does not have QoS facility (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 8 | Send the DLS Request frame, with all fields having the same value as the DLS Request frame received by the AP, to the destination QSTA (step 1b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 11 | Upon receipt of the DLS Response frame from a QSTA (step 2a in Figure 210), the QAP shall send DLS Response frame to the source QSTA (step 2b in Figure 210). | STA |
| 11.7.1.2 | 483 | 525 | 11 | Upon receipt of the DLS Response frame from a QSTA (step 2a in Figure 210), the QAP shall send DLS Response frame to the source QSTA (step 2b in Figure 210). | AP |
| 11.7.1.2 | 483 | 525 | 11 | Upon receipt of the DLS Response frame from a QSTA (step 2a in Figure 210), the QAP shall send DLS Response frame to the source QSTA (step 2b in Figure 210). | STA |
| 11.7.2 | 483 | 525 | 35 | Both the QSTAs may use direct link for data transfers using any of the access mechanisms defined in this standard. | STAs |
| 11.7.2 | 483 | 525 | 36 | QSTAs may also set up Block Ack if needed. | STAs |
| 11.7.2 | 483 | 525 | 36 | If needed, the QSTAs may set up TSs with the HC to ensure they have enough bandwidth or use polled TXOPs for data transfer. | STAs |
| 11.7.3 | 483 | 525 | 43 | The DLS teardown procedure is divided into QSTA-initiated and QAP-initiated DLS teardown. | STA |
| 11.7.3 | 483 | 525 | 43 | The DLS teardown procedure is divided into QSTA-initiated and QAP-initiated DLS teardown. | AP |
| 11.7.3 | 483 | 525 | 43 | The QSTA-initiated DLS teardown message flow is illustrated in Figure 212. | STA |
| 11.7.3.1 | 483 | 525 | 46 | QSTA-initiated DLS teardown procedure | STA |
| 11.7.3.1 | 483 | 525 | 48 | Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QSTA shall initiate the teardown of the direct link by sending the DLS Teardown frame to the QAP. | STA |
| 11.7.3.1 | 483 | 525 | 48 | Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QSTA shall initiate the teardown of the direct link by sending the DLS Teardown frame to the QAP. | AP |
| 11.7.3.1 | 484 | 526 | 3 | QSTA | STA (twice) |
| 11.7.3.1 | 484 | 526 | 3 | QAP | AP |
| 11.7.3.1 | 484 | 526 | 17 | QSTA-initiated DLS teardown message flow | STA |
| 11.7.3.1 | 484 | 526 | 25 | QSTA_LEAVING | STA_LEAVING |
| 11.7.3.1 | 484 | 526 | 25 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 27 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 28 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 29 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 34 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 35 | Non-AP QSTA | STA |
| 11.7.3.1 | 484 | 526 | 38 | Upon receipt of the DLS Teardown frame (from the QAP), the QSTA shall issue an MLME-DLSTeardown. indication primitive to the SME and shall delete the QSTA from the list of the QSTAs with which direct link has been established. | AP |
| 11.7.3.1 | 484 | 526 | 38 | Upon receipt of the DLS Teardown frame (from the QAP), the QSTA shall issue an MLME-DLSTeardown. indication primitive to the SME and shall delete the QSTA from the list of the QSTAs with which direct link has been established. | STA |
| 11.7.3.1 | 484 | 526 | 38 | Upon receipt of the DLS Teardown frame (from the QAP), the QSTA shall issue an MLME-DLSTeardown. indication primitive to the SME and shall delete the QSTA from the list of the QSTAs with which direct link has been established. | STA |
| 11.7.3.1 | 484 | 526 | 38 | Upon receipt of the DLS Teardown frame (from the QAP), the QSTA shall issue an MLME-DLSTeardown. indication primitive to the SME and shall delete the QSTA from the list of the QSTAs with which direct link has been established. | STAs |
| 11.7.3.1 | 484 | 526 | 42 | The DLS teardown procedure shall apply to a specific DLS session, as each QSTA may have multiple simultaneous DLS sessions with other QSTAs. | STA |
| 11.7.3.1 | 484 | 526 | 42 | The DLS teardown procedure shall apply to a specific DLS session, as each QSTA may have multiple simultaneous DLS sessions with other QSTAs. | STAs |
| 11.7.3.1 | 484 | 526 | 46 | Prior to disassociation/deauthentication from the QAP, the QSTA (QSTA1) shall initiate the teardown of any direct links it has by sending a DLS Teardown frame to the QAP. | AP |
| 11.7.3.1 | 484 | 526 | 46 | Prior to disassociation/deauthentication from the QAP, the QSTA (QSTA1) shall initiate the teardown of any direct links it has by sending a DLS Teardown frame to the QAP. | STA |
| 11.7.3.1 | 484 | 526 | 46 | Prior to disassociation/deauthentication from the QAP, the QSTA (QSTA1) shall initiate the teardown of any direct links it has by sending a DLS Teardown frame to the QAP. | STA1 |
| 11.7.3.1 | 484 | 526 | 46 | Prior to disassociation/deauthentication from the QAP, the QSTA (QSTA1) shall initiate the teardown of any direct links it has by sending a DLS Teardown frame to the QAP. | AP |
| 11.7.3.1 | 484 | 526 | 47 | If the DLS Teardown frame Max Retry Limit was reached with no response from the QAP the QSTA shall send a DLS Teardown frame to its peer DLS QSTA (QSTA2). | AP |
| 11.7.3.1 | 484 | 526 | 47 | If the DLS Teardown frame Max Retry Limit was reached with no response from the QAP the QSTA shall send a DLS Teardown frame to its peer DLS QSTA (QSTA2). | STA |
| 11.7.3.1 | 484 | 526 | 47 | If the DLS Teardown frame Max Retry Limit was reached with no response from the QAP the QSTA shall send a DLS Teardown frame to its peer DLS QSTA (QSTA2). | STA |
| 11.7.3.1 | 484 | 526 | 47 | If the DLS Teardown frame Max Retry Limit was reached with no response from the QAP the QSTA shall send a DLS Teardown frame to its peer DLS QSTA (QSTA2). | STA2 |
| 11.7.3.1 | 484 | 526 | 52 | A recipient QSTA (QSTA2), either on expiry of its DLSResponseTimeout or on receipt of a DLS Teardown frame with ReasonCode set to PEER_INITIATED (from QSTA1), shall send a DLS Teardown frame to the QAP with ReasonCode set to PEER_INITIATED. | STA |
| 11.7.3.1 | 484 | 526 | 52 | A recipient QSTA (QSTA2), either on expiry of its DLSResponseTimeout or on receipt of a DLS Teardown frame with ReasonCode set to PEER_INITIATED (from QSTA1), shall send a DLS Teardown frame to the QAP with ReasonCode set to PEER_INITIATED. | STA2 |
| 11.7.3.1 | 484 | 526 | 52 | A recipient QSTA (QSTA2), either on expiry of its DLSResponseTimeout or on receipt of a DLS Teardown frame with ReasonCode set to PEER_INITIATED (from QSTA1), shall send a DLS Teardown frame to the QAP with ReasonCode set to PEER_INITIATED. | STA1 |
| 11.7.3.1 | 484 | 526 | 52 | A recipient QSTA (QSTA2), either on expiry of its DLSResponseTimeout or on receipt of a DLS Teardown frame with ReasonCode set to PEER_INITIATED (from QSTA1), shall send a DLS Teardown frame to the QAP with ReasonCode set to PEER_INITIATED. | AP |
| 11.7.3.1 | 485 | 527 | 1 | A QAP that receives a DLS Teardown frame with ReasonCode set to PEER_INITIATED should send a DLS Teardown frame to any QSTAs that have a DLS link established with QSTA1. NOTE -The failed QSTA may re-establish its DLS link according to Section 11.7. | An AP |
| 11.7.3.1 | 485 | 527 | 1 | A QAP that receives a DLS Teardown frame with ReasonCode set to PEER_INITIATED should send a DLS Teardown frame to any QSTAs that have a DLS link established with QSTA1. NOTE -The failed QSTA may re-establish its DLS link according to Section 11.7. | STAs |
| 11.7.3.1 | 485 | 527 | 1 | A QAP that receives a DLS Teardown frame with ReasonCode set to PEER_INITIATED should send a DLS Teardown frame to any QSTAs that have a DLS link established with QSTA1. NOTE -The failed QSTA may re-establish its DLS link according to Section 11.7. | STA1 |
| 11.7.3.1 | 485 | 527 | 1 | A QAP that receives a DLS Teardown frame with ReasonCode set to PEER_INITIATED should send a DLS Teardown frame to any QSTAs that have a DLS link established with QSTA1. NOTE -The failed QSTA may re-establish its DLS link according to Section 11.7. | STA |
| 11.7.3.2 | 485 | 527 | 5 | Teardown procedure at the QAP | AP |
| 11.7.3.2 | 485 | 527 | 7 | Upon receipt of the DLS Teardown frame from a QSTA, the QAP shall send a DLS Teardown frame to the destination QSTA. | STA |
| 11.7.3.2 | 485 | 527 | 7 | Upon receipt of the DLS Teardown frame from a QSTA, the QAP shall send a DLS Teardown frame to the destination QSTA. | AP |
| 11.7.3.2 | 485 | 527 | 7 | Upon receipt of the DLS Teardown frame from a QSTA, the QAP shall send a DLS Teardown frame to the destination QSTA. | STA |
| 11.7.3.2 | 485 | 527 | 10 | Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QAP shall announce the tearing down of the direct link by sending the DLS Teardown frame to the two QSTAs using the direct link. | AP |
| 11.7.3.2 | 485 | 527 | 10 | Upon receipt of MLME-DLSTeardown.request primitive from the SME, the QAP shall announce the tearing down of the direct link by sending the DLS Teardown frame to the two QSTAs using the direct link. | STAs |
| 11.7.3.2 | 485 | 527 | 13 | For PeerKey, the only applicable values of the ReasonCode are STAKEY_MISMATCH and QSTA_ LEAVING. | STA_LEAVING |
| 11.7.3.3 | 485 | 527 | 20 | QAP-initiated DLS Teardown Procedure | AP |
| 11.7.3.3 | 485 | 527 | 22 | The QAP-initiated DLS teardown procedure may be used in cases where the QSTA is unable to initiate the DLS teardown. | AP |
| 11.7.3.3 | 485 | 527 | 22 | The QAP-initiated DLS teardown procedure may be used in cases where the QSTA is unable to initiate the DLS teardown. | STA |
| 11.7.3.3 | 485 | 527 | 23 | The QAP should initiate a DLS teardown procedure when the QAP detects that either end of a DLS link has left the BSS without teardown of the DLS link. | AP |
| 11.7.3.3 | 485 | 527 | 23 | The QAP should initiate a DLS teardown procedure when the QAP detects that either end of a DLS link has left the BSS without teardown of the DLS link. | AP |
| 11.7.3.3 | 485 | 527 | 28 | If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED. NOTE - The QAP can also initiate DLS teardown for implementation-dependent reasons. | STAs |
| 11.7.3.3 | 485 | 527 | 28 | If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED. NOTE - The QAP can also initiate DLS teardown for implementation-dependent reasons. | STA |
| 11.7.3.3 | 485 | 527 | 28 | If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED. NOTE - The QAP can also initiate DLS teardown for implementation-dependent reasons. | AP |
| 11.7.3.3 | 485 | 527 | 28 | If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED. NOTE - The QAP can also initiate DLS teardown for implementation-dependent reasons. | STA |
| 11.7.3.3 | 485 | 527 | 28 | If there are one or more QSTAs with open DLS connections with the QSTA being removed, the QAP shall send a DLS Teardown frame to each such QSTA with ReasonCode AP_INITIATED. NOTE - The QAP can also initiate DLS teardown for implementation-dependent reasons. | AP |
| 11.7.4 | 485 | 527 | 38 | When the direct link becomes inactive due to the timeout, the MAC issues an MLME-DLSTeardown.indication primitive to the SME and sends a DLS Teardown frame to the QAP, with the peer MAC address as the destination MAC address and the reason code set to TIMEOUT. | AP |
| 11.7.4 | 485 | 527 | 40 | All frames shall henceforth be sent via the QAP. The procedure is illustrated in Figure 213. | AP |
| 11.7.4 | 485 | 527 | 43 | If there has been a TS setup for the data transfer, it is the responsibility of the QSTAs to renegotiate the parameters of the TSPEC with the HC. | STAs |
| 11.7.4 | 485 | 527 | 46 | When two QSTAs have set up a direct link, either QSTA may send DLS Request frames in order to update the DLS timeout value. | STAs |
| 11.7.4 | 485 | 527 | 46 | When two QSTAs have set up a direct link, either QSTA may send DLS Request frames in order to update the DLS timeout value. | STA |
| 11.7.4 | 485 | 527 | 47 | If the updated DLS Request frame is accepted, both QSTAs initialize the timer to detect DLS timeout. | STAs |
| 11.7.5 | 486 | 528 | 4 | The STKSA remains valid even if the QSTA disassociates from the originating QAP, but the STKSA shall be deleted before a QSTA attempts another Association or Reassociation. | STA |
| 11.7.5 | 486 | 528 | 4 | The STKSA remains valid even if the QSTA disassociates from the originating QAP, but the STKSA shall be deleted before a QSTA attempts another Association or Reassociation. | AP |
| 11.7.5 | 486 | 528 | 4 | The STKSA remains valid even if the QSTA disassociates from the originating QAP, but the STKSA shall be deleted before a QSTA attempts another Association or Reassociation. | STA |
| 11.7.5 | 486 | 528 | 5 | If a QAP transmits a Deauthenticate or Disassociate message to a QSTA, the QAP shall also initiate teardowns for any existing DLS. | an AP |
| 11.7.5 | 486 | 528 | 5 | If a QAP transmits a Deauthenticate or Disassociate message to a QSTA, the QAP shall also initiate teardowns for any existing DLS. | STA |
| 11.7.5 | 486 | 528 | 5 | If a QAP transmits a Deauthenticate or Disassociate message to a QSTA, the QAP shall also initiate teardowns for any existing DLS. | AP |
| D | 1001 | 1043 | 36 | "This attribute, when TRUE, indicates that the station implementation is capable of sending and receiving frames from a non-AP QSTA in the QBSS. | STA |
| D | 1001 | 1043 | 36 | "This attribute, when TRUE, indicates that the station implementation is capable of sending and receiving frames from a non-AP QSTA in the QBSS. | BSS |
| D | 1001 | 1043 | 46 | "This attribute is available only at a QAP. When TRUE, this attribute indicates that the QAP implementation is capable of delivering data and polls to stations using APSD." | an AP |
| D | 1001 | 1043 | 46 | "This attribute is available only at a QAP. When TRUE, this attribute indicates that the QAP implementation is capable of delivering data and polls to stations using APSD." | AP |
| D | 1002 | 1044 | 2 | "This attribute, when TRUE, indicates that the station implementation is capable of interpreting the CF-Ack bit in a received frame of type data even if the frame is not directed to the QoS station. The capability is disabled, otherwise. A QSTA is capable of interpreting the CF-Ack bit in a received data frame if that station is the recipient of the data frame, regardless of the value of this MIB variable. The default value of this attribute is FALSE." | STA |
| D | 1002 | 1044 | 16 | "This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP implementation is capable of generating and transmitting the QBSS load element in the Beacon and Probe Response frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | an AP |
| D | 1002 | 1044 | 16 | "This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP implementation is capable of generating and transmitting the QBSS load element in the Beacon and Probe Response frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | AP |
| D | 1002 | 1044 | 16 | "This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP implementation is capable of generating and transmitting the QBSS load element in the Beacon and Probe Response frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | BSS |
| D | 1002 | 1044 | 28 | "This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP is capable of processing the Queue Size field in QoS Control field of QoS Data type frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | an AP |
| D | 1002 | 1044 | 28 | "This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP is capable of processing the Queue Size field in QoS Control field of QoS Data type frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | AP |
| D | 1002 | 1044 | 39 | This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP is capable of processing the TXOP Duration requested field in QoS Control field of QoS Data type frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | an AP |
| D | 1002 | 1044 | 39 | This attribute is available only at a QAP. This attribute, when TRUE, indicates that the QAP is capable of processing the TXOP Duration requested field in QoS Control field of QoS Data type frames. The capability is disabled, otherwise. The default value of this attribute is FALSE." | AP |
| D | 1003 | 1045 | 6 | "This attribute, when TRUE, indicates that the station may associate in an nQBSS. When FALSE, this attribute indicates that the station may onlyshall not associate in an nQBSS. The default value of this attribute is TRUE." | a non-QoS BSS |
| D | 1003 | 1045 | 6 | "This attribute, when TRUE, indicates that the station may associate in an nQBSS. When FALSE, this attribute indicates that the station may onlyshall not associate in an nQBSS. The default value of this attribute is TRUE." | non-QoS BSS |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | AP |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | BSS |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 17 | "This attribute available at the QAP, when TRUE, indicates that non-AP QSTAs may set up DLS and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the stations shall not set up DLS nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | BSS |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STA |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | BSS |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STA |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | STAs |
| D | 1003 | 1045 | 29 | "This attribute available at the non-AP QSTAs, when TRUE, indicates that QSTA may set up DLS or accept DLS Requests, and communicate with other non-AP QSTAs in the QBSS. When FALSE, this attribute indicates that the QSTA shall not set up DLS nor accept DLS, nor communicate with other non-AP QSTAs in the QBSS. The default value of this attribute is TRUE." | BSS |
| D | 1019 | 1061 | 13 | "This attribute indicates the number of times the QAP may retry a frame for which it does not receive an ACK for a STA in power-save mode after receiving a PS-Poll and sending a directed response or after the QAP does not receive an ACK to a directed MPDU sent with the EOSP set to 1." | AP |
| D | 1019 | 1061 | 13 | "This attribute indicates the number of times the QAP may retry a frame for which it does not receive an ACK for a STA in power-save mode after receiving a PS-Poll and sending a directed response or after the QAP does not receive an ACK to a directed MPDU sent with the EOSP set to 1." | AP |
| D | 1024 | 1066 | 49 | "Conceptual table for EDCA default parameter values at a non-AP QSTA. This table shall contain the four entries of the EDCA parameters corresponding to four possible ACs. Index 1 corresponds to AC_BK, index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO." | STA |
| D | 1025 | 1067 | 39 | "This attribute shall specify the value of the minimum size of the window that shall be used by a QSTA for a particular AC for generating a random number for the backoff. The value of this attribute shall be such that it could always be expressed in the form of 2X - 1, where X is an integer. The default value for this attribute is 1) aCWmin, if dot11EDCATableIndex is 1 or 2. 2) (aCWmin+1)/2 - 1, if dot11EDCATableIndex is 3. 3) (aCWmin+1)/4 - 1, if dot11EDCATableIndex is 4." | STA |
| D | 1025 | 1067 | 53 | "This attribute shall specify the value of the maximum size of the window that shall be used by a QSTA for a particular AC for attribute shall be such that it could always be expressed in the form of 2X - 1, where X is an integer. The default value for this attribute is 1) aCWmax, if dot11EDCATableIndex is 1 or 2. 2) aCWmin, if dot11EDCATableIndex is 3. 3) (aCWmin+1)/2 - 1, if dot11EDCATableIndex is 4." | STA |
| D | 1026 | 1068 | 14 | "This attribute shall specify the number of slots, after a SIFS duration, that the QSTA, for a particular AC, shall sense the medium idle either before transmitting or executing a backoff. The default value for this attribute is 1) 7, if dot11EDCATableIndex is 1, 2) 3, if dot11EDCATableIndex is 2 3) 2, otherwise." | STA |
| D | 1026 | 1068 | 26 | "This attribute shall specify the maximum number of microseconds of an EDCA TXOP for a given AC at a non-AP QSTA. The default value for this attribute is 1) 0 for all PHYs, if dot11EDCATableIndex is 1 or 2; this implies that the sender can send one MSDU in an EDCA TXOP, 2) 3008 microseconds for IEEE 802.11a/g PHY and 6016 microseconds for IEEE 802.11b PHY, if dot11EDCATableIndex is 3, 3) 1504 microseconds for IEEE 802.11a/g PHY and 3264 microseconds for IEEE 802.11b PHY, if dot11EDCATableIndex is 4." | STA |
| D | 1027 | 1069 | 7 | SMT QAP EDCA Config TABLE | AP |
| D | 1027 | 1069 | 14 | "Conceptual table for EDCA default parameter values at the QAP. This table shall contain the four entries of the EDCA parameters corresponding to four possible ACs. Index 1 corresponds to AC_BK, index 2 to AC_BE, index 3 to AC_VI, and index 4 to AC_VO." | AP |
| D | 1028 | 1070 | 5 | "This attribute shall specify the value of the minimum size of the window that shall be used by a QAP for a particular AC for generating a random number for the backoff. The value of this attribute shall be such that it could always be expressed in the form of 2X - 1, where X is an integer. The default value for this attribute is 1) aCWmin, if dot11QAPEDCATableIndex is 1 or 2. 2) (aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 3. 3) (aCWmin+1)/4 - 1, if dot11QAPEDCATableIndex is 4." | an AP |
| D | 1028 | 1070 | 19 | "This attribute shall specify the value of the maximum size of the window that shall be used by a QAP for a particular AC for generating a random number for the backoff. The value of this attribute shall be such that it could always be expressed in the form of 2X - 1, where X is an integer. The default value for this attribute is 1) aCWmax, if dot11QAPEDCATableIndex is 1. 2) 4*(aCWmin+1) - 1, if dot11QAPEDCATableIndex is 2. 3) aCWmin, if dot11QAPEDCATableIndex is 3. 4) (aCWmin+1)/2 - 1, if dot11QAPEDCATableIndex is 4." | an AP |
| D | 1028 | 1070 | 36 | "This attribute shall specify the number of slots, after a SIFS duration, that the QAP, for a particular AC, shall sense the medium idle either before transmitting or executing a backoff. The default value for this attribute is 1) 7, if dot11QAPEDCATableIndex is 1, 2) 3, if dot11QAPEDCATableIndex is 2 3) 1, otherwise." | AP |
| D | 1028 | 1070 | 49 | "This attribute shall specify the maximum number of microseconds of an EDCA TXOP for a given AC at the QAP. The default value for this attribute is 1) 0 for all PHYs, if dot11QAPEDCATableIndex is 1 or 2; this implies that the sender can send one MSDU in an EDCA TXOP, 2) 3008 microseconds for IEEE 802.11a/g PHY and 6016 microseconds for IEEE 802.11b PHY, if dot11QAPEDCATableIndex is 3, 3) 1504 microseconds for IEEE 802.11a/g PHY and 3264 microseconds for IEEE 802.11b PHY, if dot11QAPEDCATableIndex is 4." | AP |
| D | 1029 | 1071 | 10 | "This attribute shall specify (in TUs) the maximum duration an MSDU, for a given AC, would be retained by the MAC at the QAP before it is discarded. The default value for this parameter shall be 500." | AP |
| D | 1029 | 1071 | 26 | End of SMT QAP EDCA Config TABLE | AP |
| K | 1162 | 1204 | 16 | (not specified by non-AP QSTA; only an output from the HC) | STA |
| K.2.1 | 1162 | 1204 | 25 | The AC parameters chosen by the QAP should account for unadmitted traffic in these ACs. | AP |
| K.2.2 | 1162 | 1204 | 31 | It is recommended that the QAP use the following procedure to derive medium time in its ADDTS Response frame: | AP |
| K.3.1 | 1163 | 1205 | 7 | The HC establishes the SI for each admitted TS for a non-AP QSTA to derive the aggregate minimum SI contained in the non-AP QSTA’s service schedule. | STA |
| K.3.1 | 1163 | 1205 | 7 | The HC establishes the SI for each admitted TS for a non-AP QSTA to derive the aggregate minimum SI contained in the non-AP QSTA’s service schedule. | STA’s |
| K.3.1 | 1163 | 1205 | 12 | The HC may use an aggregate “token bucket specification” to police a non-AP QSTA’s admitted flows. | STA’s |
| K.3.1 | 1163 | 1205 | 14 | The aggregate mean data rate is equal to the sum of the mean data rates of all of the non-AP QSTA’s admitted TSs. | STA’s |
| K.3.1 | 1163 | 1205 | 15 | The aggregate burst size is equal to the sum of the burst size of all of the non-AP QSTA’s admitted TSs. | STA’s |
| K.3.2 | 1163 | 1205 | 52 | The number of retries may be chosen (as below) to meet a particular probability of dropping a packet because it exceeds its delay bound. Note that for multiple streams, this SI should be the aggregate of all SIs requested, because the QSTA is assigned the TXOPs, not any particular stream. | STA |
| K.3.2 | 1164 | 1206 | 27 | The Surplus Bandwidth Allowance parameter ensures the requesting QSTA is allocated a minimum amount of excess time by the scheduler to so that application dropped packet rates are bounded. | STA |
| K.3.3.1 | 1165 | 1207 | 48 | If both Maximum Service Interval and Delay Bound parameters are specified by the non-AP QSTA in the TSPEC, the scheduler uses the Maximum Service Interval parameter for the calculation of the schedule. | STA |
| K.3.3.1 | 1166 | 1208 | 12 | Schedule for stream from QSTA | STA |
| K.3.3.1 | 1166 | 1208 | 49 | Schedule for streams from QSTAs i to k | STAs |
| K.3.3.1 | 1166 | 1208 | 3 | This value is the scheduled SI for all non-AP QSTAs with admitted streams. | STAs |
| K.3.3.1 | 1166 | 1208 | 36 | Stream from QSTA i is admitted. | STA |
| K.3.3.1 | 1167 | 1209 | 1 | The scheduler might also choose to move the TXOPs for the QSTAs following the QSTA dropped to use the unused time. | STAs |
| K.3.3.1 | 1167 | 1209 | 1 | The scheduler might also choose to move the TXOPs for the QSTAs following the QSTA dropped to use the unused time. | STA |
| K.3.3.1 | 1167 | 1209 | 2 | An example is shown in Figure K.3, when the stream for QSTA j is removed. | STA |
| K.3.3.1 | 1167 | 1209 | 3 | However, this option might require the announcement of a new schedule to all QSTAs. | STAs |
| K.3.3.1 | 1167 | 1209 | 13 | For example, a scheduler might generate different scheduled SIs ( ) for different QSTAs, and/or a scheduler might consider accommodating retransmissions while allocating TXOP durations. | STAa |