IEEE P802.11
Wireless LANs

| Power save and discovery services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kazuyuki Sakoda | Sony Corporation | 2-17-1 Higashi-gotanda, Shinagawa-ku, Tokyo, 141-0022 Japan | +81-3-6409-3201 | sako@wsc.sony.co.jp |

Abstract

Proposed resolution text for CID222

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This text provides a resolution to CID222 [Error: Reference source not found].

CID: 222

Comment: The relationship among power management, neighbor discovery, and route discovery is not described at all.

Proposed Change: Recommend to sort out several cases for power save mode.

Regarding neighbor discovery :

A1. MP in power save mode which does not intend to establish a new peer link.

A2. MP in power save mode which intends to establish a new peer link.

Regarding routing :

B1. MP in power save mode which  does not intend to act as a forwarding node.

B2. MP in power save mode which intends to act as a forwarding node.

Neighboring MP's or power saving MP's behavior should vary according to these intentions. A1 power saving MP does not have to transmit beacon nor probe response frames, whereas A2 power saving MP should notify the existance of itself to a potential new MP's neighbor discovery. B1 power saving MP do not have to activate the route discovery mechanism, whereas B2 power saving MP should be a part of route discovery mechanism. In order to include B2 MP in a route, neighboring MPs have to wake it up, during the route discovery.

Resolution: Defer-NS

Proposed resolution

Add some explanation how the neighbour discovery or route discovery will work with power save mode.

Corresponding chnages in the text should be made in the following subclauses:

11A.12 Mesh Beaconing and Synchronization

11A.13 Power Management in a Mesh (Optional)

The following text was taken from the Draft and has been modified with “Track Changes” on:

11A.12 Mesh Beaconing and Synchronization

11A.12.1 Synchronization

Synchronization and beacon generation services in a WLAN mesh are based upon the procedures defined in clause 11.1 of the IEEE 802.11-1999 specification for Infrastructure and IBSS modes of operation.

It is optional for an MP to support synchronization. An MP supporting synchronization may choose to be either synchronizing or unsynchronizing based on either its own requirements or the requirements of its peers.  MP’s synchronization behavior is communicated through the “synchronization capability field” within the WLAN Mesh Capability element. The synchronizing behaviour for the two classes is defined as follows.

11A.12.1.1 Unsynchronizing MPs

An unsynchronizing MP is a MP that maintains an independent TSF timer and does not update the value of its TSF timer based on time stamps and offsets received in beacons or probe responses from other MPs.  An unsynchronizing MP may start its TSF timer independently of other MPs.  The “Synchronizing with peer MP” bit in the “Synchronization Capability” field of the WLAN Mesh Capability element, when set to 0, indicates that an MP is currently an unsynchronizing MP. A MP that supports synchronization may elect to be an unsynchronizing MP if it is communicating with peers that are not requesting synchronization.

11A.12.1.2 Synchronizing MPs (Optional)

A synchronizing MP is an MP that updates its timer based on the time stamps and offsets (if any) received in beacons and probe responses from other synchronizing MPs.  The “Synchronized with peer MP” bit in the “Synchronization Capability” field of the WLAN Mesh Capability element, when set to 1, indicates that the MP is currently a synchronizing MP.

Synchronizing MPs should attempt to maintain a common TSF time called the Mesh TSF time.  An MAP maintains the mesh TSF in terms of its TSF timer and its self TBTT offset such that the sum of the self TSF timer and the self TBTT offset equals the mesh TSF time. All beacons and probe responses by such MAPs carry the Beacon Timing IE to advertise its self offset value relative to the Mesh TSF time.

Synchronizing MPs translate the received time stamps and offsets (if any) from beacons and probe responses from other synchronizing MPs to their own timer base, and update their timer as described as follows:

Translated time stamp = Received time stamp + Received offset (if any) – Receiver’s offset (if any);

Any synchronizing MP will adopt the translated time stamp as its own if it is later than the timer value of self as described for IBSS mode of synchronization.

Synchronizing MPs may optionally choose to update their offsets instead of their timers. The offset update process in this case is as below.

If (received time stamp + received offset) > (self time + self offset)

New self offset value = received time stamp + received offset – self time.

Note that the “Received offset” above is the “self offset” in the received Beacon Timing IE from the neighbor MP, and the “Receiver’s offset” is the receiving MP’s own self offset.

11A.12.1.3 Interaction between synchronizaing and unsynchronizing MPs

A synchronizing MP may or may not request synchronization from its peers. However, if an MP requests synchronization from its peers, it has to be a synchronizing MP at that time. Initially, an MP may be in unsynchronized state, but it may switch to synchronized state and vice-versa based on either its own requirements or the requirements of peers.

An unsynchronizing MP may change into a synchronizing MP if it is capable of synchronizing, by setting its “Synchronizing with peer MP” bit to 1.

A synchronizing MP that associates with an unsynchronizing MAP and intends to enter power save mode may need to maintain additional information to wake up at the MAP’s DTIM interval during its PS operations as described below.

11A.12.2 Beaconing

Any MP may choose to beacon either as defined in the IBSS mode or as defined in the infrastructure mode of operation in IEEE 802.11-1999 and 802.11E.

11A.12.2.1 Beaconing by unsynchronizing MPs

Unsynchronizing MPs generate beacons according to the beacon generation procedures defined in clause 11.1.2.1 in IEEE 802.11-1999. Unsynchronizing MPs choose their own beacon interval and TSF independent of other MPs.

Unsynchronizing MPs may implement beacon collision avoidance defined in section 6.11.3 to reduce the chances that it will transmit beacons at the same time as one of its neighbors.

Unsynchronizing MAPs shall treat any associated non-AP MPs and neighboring LW-MPs operating in PS mode identical to legacy STA, meaning that the MAP shall assume that the MPs will wake up for the BSS DTIM beacon of the MAP in PS operations.

11A.12.2.2 Beaconing by synchronizing MPs

Synchronizing MAPs generate beacons according to the beacon generation procedures described in clause 11.1.2.1 in IEEE 802.11-1999.  The value of the aBeaconPeriod attribute used by synchronizing MAPs shall equal a sub-multiple of the Mesh DTIM interval. Synchronizing MAPs shall use and advertise a non-zero self TBTT offset value using the Beacon Timing element.

Synchronizing non-AP MPs generate beacons according to the beacon generation procedures described for IBSS operation in the 802.11-1999 standard (section 11.1.2.2), unless acting as a designated beacon broadcaster (see section 6.11.2.3). A non-AP MP that receives a beacon from an MP with the Mesh ID the same as its own after TBTT and before being able to send its own beacon may cancel that beacon transmission.  Specifically, the following rules apply for beacon transmission.

- Suspend the decrementing of backoff timers for any non Beacon traffic.

- Calculate a random delay uniformly distributed over the range of zero and twice aCWmin X aSlot time. The CWmin is as used for AC_VO.

- Wait for the period of random delay, decrementing the random delay timer using the same algorithm as for back off.

- If a beacon with the same Mesh ID arrives before the random delay timer expires, cancel the remaining random timer delay and the pending beacon transmission.

- Send a beacon if the random delay has expired and no beacon has arrived during the delay period.

Each synchronizing MP can select its own Beacon interval, but all synchronizing MPs need to share a common Mesh DTIM interval. The Beacon intervals selected by MP must always be a submultiple of the Mesh DTIM interval. A synchronizing MP that establishes a Mesh selects its Beacon interval and the MP DTIM period and establishes the common Mesh DTIM interval of the mesh. Mesh DTIM interval equals the product of the beacon interval and the MP DTIM period. A synchronizing MP that joins an existing mesh needs to adopt the Mesh DTIM interval of the mesh.

Note that the Mesh DTIM interval and the BSS DTIM interval in MAPs do not have to be identical. MPs use the DTIM IE to advertise the Mesh DTIM interval, whereas the TIM IE is used for advertising the DTIM interval in a BSS. The DTIM Period of these IEs do not have to be identical since one will be used for the AP service while the other for the Mesh service.

An unsynchronizing MP intending to enter the power save state shall become a synchronizing MP and request synchronization from peers prior to leaving the active state. MPs supporting PS operation may use the optional designated beacon broadcaster approach described in section 6.11.4 and 6.12.8.

An MP operating in Power Save will set its MP DTIM period to 1 prior to leaving the active state (meaning the beacon interval is the same as the mesh DTIM interval) and would therefore attempt to beacon only once on every Mesh DTIM interval.

A LW-MP (as described in section 4.2.2.1) may opt not to beacon if it is able to detect beacons from a beacon broadcaster. A LW-MP that opts not to beacon will have to resume beaconing if it does not receive any beacons for 3 Mesh DTIM intervals.

11A.12.2.3 Designated Beacon Broadcaster

The designated beacon broadcaster approach may only be used by LW-MPs that support power save operation. It enables to have a designated MP perform beaconing for a defined period of time while all other MPs defer from sending beacons.

An MP that serves as the designated beacon broadcaster will transmit its beacon using the procedure as described for infrastructure AP operation section 11.1.2.1 in the 802.11-1999 standard (i.e., will not use random backoff). The general operation of the power management in a mesh network is discussed in Section 6.12, specifically the beacon broadcaster is discussed in Section 6.12.8.

An MP supporting this option that received at least one beacon from an MP that is marked as Designated Beacon Broadcaster in the last 2 Mesh DTIM intervals will not schedule a Beacon for transmission.

11A.12.2.4 Change of Beacon broadcaster

The beacon broadcaster role must be changed periodically. An MP needs to relinquish its role as the designated beacon broadcaster after no more than MAX_CONT_BB Mesh DTIM intervals. A suggested value of MAX_CONT_BB is 32.

In every Mesh DTIM interval, the current beacon broadcaster sets the BB switch bit to 1 if it wants to change the beacon broadcaster (see Section 6.12.8.1). In this case the neighbor that is first in the neighbor list shall start acting as the beacon broadcaster and shall send the next Mesh DTIM beacon.

The BB has to make sure that the neighbor appearing at the head of the list is supporting power save transmission and Designated Beacon broadcasting.

If a neighbor assigned to be the beacon broadcaster fails to transmit its beacon (shuts down or goes out of range), other MPs will attempt to take over its role.  MPs supporting the deterministic beacon broadcasting will start attempting to send beacons using the standard backoff procedure with a Neighbor list IE designating itself as the BB if it does not receive 3 consecutive beacons from the last designated BB.

If another MP already transmitted a Beacon with a neighbor list IE designating itself as the BB, then the MP will cancel its pending Beacon.

11A.12.3 Mesh Beacon Collision Avoidance (MBCA) mechanism

MPs may optionally adjust their TSF timers to reduce the chances that they will transmit beacons at the same time as one of their neighbors.

Individual MPs may take steps either prior to, or during association, with a WLAN mesh to select a TBTT that does not conflict with its mesh neighbors. An MP may adjust its TSF timer if it discovers that its TBTT may repeatedly collide with the TBTT of a neighbor.  Options a MP has for adjusting its TSF include advancing or suspending the TSF for a period of time.

An MP may collect and report information about the TBTT of neighboring synchronizing MAPs and unsynchronizing MPs using a variety of techniques. The following describes options to receive such information from neighboring MPs.

- Information from synchronizing neighbors

MPs that are synchronizing collect the beacon timing information of their neighbors and report it through the beacon timing IE. This IE may be transmitted in selected beacons, and in action frames responding to requests for such information. Synchronizing MPs may choose any frequency of including the beacon timing information in the Beacon Timing IE in their beacons. The beacon timing information may also be requested via action frames (described in Sec. 6.11.3.1), with the response through the beacon timing IE in action frames.  The action frame approach is especially useful for use by synchronizing MAPs to proactively detect and avoid beacon collisions. Synchronizing MPs are required to be able to respond to requests for such information using the beacon timing IE. Synchronizing non-AP MPs, when using the Beacon Timing IE, set the Self TBTT Offset field in the Self Beacon Timing field to 0.

- Information from unsynchronizing neighbors

Unsynchronizing MPs may optionally collect and report beacon timing information of their neighbors.  Since unsynchronizing MPs do not track the mesh TSF, they report beacon time offsets relative to their self TSF.  This information either may be periodically transmitted in beacons at whatever periodicity the MP chooses, or it may be transmitted based on a request response approach through action frames. The Beacon Timing IE is used to report this information in beacons as well as in action frames as a response to request action frames. The Self Beacon Timing field in the Beacon Timing IE is set to all zeros in this case.

In addition, 802.11k beacon reports may be used by MPs to exchange beacon timing information of their neighbors, with the usage as defined by 802.11k.

As an option, Synchronizing MAPs may occasionally delay their beacons after their TBTTs for a random time. The random delay is chosen so that the transmission time is interpreted by the MAP as not colliding with other beacons. This behavior further helps in discovery of neighbors through beacons in case they choose colliding offsets. The MBCA mechanism may then be used for choosing non-colliding offsets, in case any colliding offsets are observed

11A.12.3.1 Action frames for beacon timing request and resonse

Neighbors’ Beacon Timing information may be requested and provided through action frames as described above. Section 7.4.5.11 and Section 7.4.5.12 show the frame formats — including category and action field values — for the “Beacon Timing Request” and “Beacon Timing Response” frames, respectively.

11A.13 Power Management in a Mesh (Optional)

This sub-clause describes the power management mechanisms to use within a mesh network

The power save mode operation in a mesh environment has to be coordinated with the neighbor discovery mechanisms and route discovery mechanisms, both of which are salient but essential mechanisms for mesh operation. MPs in a power save mode shall clearly behave whether it intends to be a part of these mechanisms or not. MP’s power save mode behavior will be diverse depending upon these intentions. It should be noted that the MP’s inappropriate behavior in terms of power save mode may degrade not only the performance of local MP but also the performance of entire mesh.

11A.13.1 Power management and neighbor discovery mechanism

There are possibly 2 cases for MP in a power save mode in terms of the power save mode coordination with neighbor discovery.

| neighbor discovery active PS MP | MP in power save mode which intends to establish a new peer link. |
| --- | --- |
| neighbor discovery passive PS MP | MP in power save mode which does not intends to establish a new peer link. |

Neighbor discovery active PS MP shall keep on beaconing periodically, so that the neighboring MPs can determine the existence of the MP. Neighbor discovery active PS MP shall transmit beacon frame at least once in a mesh DTIM period, though it may enter the doze state if a series of conditions are fulfilled as described later in this subclause. Neighbor discovery active PS MP shall process the probe request frame if it is received, and respond with probe response frame.

MP which tries to establish a new peer link with MPs in power save mode shall perform passive scan. Since MPs in power save mode may transmit beacon at low rate, MP which tries to establish a new peer link with MPs in power save mode should perform passive scan relatively longer time compared to passive scan in BSS infrastructrure mode operation. MPs in power save mode which sets longer DTIM interval may not be found by the neighbor MPs due to the shorter scan duration. MPs in power save mode should set shorter DTIM interval, if it intends to establish a new peer link with MPs with higher probability.

Neighbor discovery passive PS MP may not transmit beacon periodically, since it does not have to let the newly arrived neighboring MPs to determine the existence of itself. However, it is recommended that MP transmit some frames to neighbors periodically in order to let the neighboring MPs to perform the maintenance of the peer link, if the MP is a mobile device and it has a risk of physical topology changes.

In summary, MP shall select how to beacon depending on the MP’s intention for the new establishment of the peer link. While MP’s beaconing behavior may transit from one to another, the trigger to change the beaconing profile is beyond the scope of this standard.

11A.13.2 Power management and route discovery mechanism

There are 2 cases for MP in a power save mode in terms of the power save mode coordination with route discovery.

| route discovery active PS MP | MP in power save mode which intends to act as a forwarding MP. |
| --- | --- |
| route discovery passive PS MP | MP in power save mode which does not intends to act as a forwarding MP. |

Route discovery passive PS MP may not receive nor process the route discovery related mesh management frames. Neighboring MPs of route discovery passive PS MP may not deliver the route discovery related mesh discovery frames and shall try to establish a new route without a participation of route discovery passive PS MP.

A MP whose neighbors are all in the power save mode or MP which find the destination MP of the path is in power save mode shall deliver the route discovery related mesh management frames to the MP in power save mode.

Since the 802.11s standard assumes multiple route discovery mechanisms and some of the route discovery mechanim assumes immediate packet delivery, all the forwarding MPs should assume that forwarding neighbor MPs are in active state all the time.

11A.13.3 Basic approach

A mesh point supporting power save operation may either operate in an active or power save state. The mesh point will advertise its power save state to all neighboring mesh points by using it’s beacons and by sending a Null-Data frame with the PS bit active.

Mesh points in power save mode shall periodically listen for DTIM beacons.  Mesh point waking to receive a beacon will stay awake for a minimum period of ATIM window as indicated in their beacons, before returning to sleep.

Mesh points in power save mode shall also wakeup according to any negotiated schedule as part of TSPEC setup with other mesh points.  The mesh point will remain awake until the end of service period

Mesh point wishing to communicate with mesh points that are in power save would buffer the traffic targeted for these mesh points.  They could deliver the traffic to the mesh point in one of 3 ways:

- Send traffic to these mesh points only on agreed schedules as negotiated as part of APSD (Automatic Power Save Delivery) TSPEC setup

- Send directed or broadcast ATIM packets to mesh point in power save during ATIM window in order to signal them to remain awake and wait for further traffic

- Send a single null data packet to mesh point in power save during their ATIM window in order to reactivate a flow that has been suspended or to signal power save state change.

All non-AP MPs that support the mesh power save mechanism shall support synchronization as described in 6.11. Such non-AP MPs shall become synchronizing MPs if they are not already, if they receive beacons or probe responses with the “Request Synchronization from Peers” bit set in the ‘Synchronization Capability’ field of WLAN Mesh Capability IE. All non-AP MPs that intend to enter power save state shall be synchronizing MPs, as defined in clause 6.11, and shall request synchronization from peers through the ‘synchronization capability’ field in their WLAN Mesh Capability IE. MAPs may support power save with or without being synchronizing MPs (that is, even when acting as unsynchronizing MPs).

Any MP that wishes to communicate with an unsynchronizing MAP, and enters PS mode shall wake up for the BSS DTIM beacon of the MAP. If such an MP wishes to communicate with more than one unsynchronizing MAP, it shall wake up for the BSS DTIM beacons for each such MAP in addition to any Mesh DTIM TBTT which may be scheduled for its synchronizing MP neighbors.

LW-MPs that aim to communicate with their unsynchronizing MAP neighbors and enter PS state must wake up for the BSS DTIM beacons for each such MAP in addition to any Mesh DTIM TBTT which may be scheduled for its synchronizing MP neighbors.  Alternatively, a lightweight MP may associate with a MAP as a simple STA if it intends to enter PS mode.

The PS operation of an unsynchronizing MAP is based upon IEEE 802.11 infrastructure power management operation.  In particular, STAs (including MPs) changing Power Management mode shall inform the MAP of this fact using the Power Management bits within the Frame Control field of transmitted frames.  Unsynchronizing MAP shall not arbitrarily transmit MSDUs to MP operating in a PS mode, but shall buffer MSDUs and only transmit them at designated times (during BSS DTIM intervals).

11A.13.4 Initialization of power management within a mesh

The following procedure shall be used to initialize power management within a new mesh or on joining an existing mesh.

- A mesh point that joins a mesh will update its ATIM window and DTIM interval according to the received values from beacons in the mesh

- The mesh point sets its own power save state and advertises it in its beacons

- A mesh point that creates a mesh would set the values of ATIM window, DTIM interval and power save state and advertise them in its beacons

- The start of the ATIM window will be measured from TBTT

11A.13.5 Mesh point power save transitions

A mesh point may change its state to power save only if the following conditions are fulfilled:

- The mesh point supports power save operation

- All of the mesh points that the mesh point is connected to (has peer relationships) are capable of transmitting traffic to mesh points operating in power save mode

A mesh point changing power mode to power save will inform all it’s mesh neighbors of the change by sending a Null-Data frame to a broadcast address with the power bit in its frame control header set. The packet will be sent during the ATIM window of a DTIM beacon and will be repeated at least twice on two different DTIM intervals. If a beacon is received with the PS state bit of the specific MP in Neighbor list not updated, the MP will continue to send the Null-Data packet on the next DTIM. The mesh point will include a Mesh PS IE with a value of power save in its following beacons.

A mesh point changing power mode to active will inform all its mesh neighbors of the change by sending a Null-Data frame to a Broadcast address with the power bit in its frame control header cleared. The packet will be sent during the ATIM window of a DTIM beacon and will be repeated twice on two consecutive DTIM intervals. The mesh point will include a Mesh PS IE with a value of Active in its following beacons. The mesh point will transfer to the active state immediately with no relation to when the Beacons will be sent.

A mesh point operating in power save mode will set the power bit in the frame control header of every outgoing frame.  A mesh point operating in active mode will clear the power bit in the frame control header of every out going frame.

A mesh point in power save mode will transition between awake and doze states according to the following rules:

- A mesh point will enter awake state prior to every TBTT that matches the mesh DTIM interval

- If a mesh point that entered the awake state due to the DTIM TBTT event and had not sent an ATIM, did not receive a directed or multicast ATIM may return to the Doze state following the end of the ATIM window

- If a mesh point received an ATIM frame it may return to doze state after receiving a packet with the more bit in the control field cleared from all the sources that sent an ATIM packet to it.

- A mesh point that transitions to the awake state may transmit a beacon, but this would not prevent it from returning to doze state following the ATIM window

- In addition a mesh point will enter awake state prior to every agreed schedule as negotiated as part of a periodic APSD TSPEC exchange with other mesh points

- A mesh point entering awake state may return to doze state after receiving and/or sending a directed frame to/from the specific flow for which this schedule was set with EOSP bit set or with the expiration of the maximal service interval for that flow.

- A mesh point may transition to awake state if it has traffic to transmit at any given point of time

11A.13.6 Frame transmission

The following description applies to mesh points that are supporting power save operation.  Mesh points that do not support this capability do not have to track the power save state of other mesh points and will only use the standard procedure for frame transmission.

A mesh point will store information on the power save state of all its neighbors by monitoring their beacon’s Mesh PS IE and by extracting information from the neighbor list IE in beacons of other MPs.

A mesh point considers the mesh to operate in a power save mode if any one of its neighbors is operating in the power save mode.  In a mesh that operates in the Active state, frames may be sent at any time to other mesh points.  For a mesh that operates in a power save scheme the following rules apply for transmission:

- All broadcast and multicast traffic will be buffered by mesh points that perceive the mesh to operate in power save mode. These packets will be transmitted only on DTIM intervals

- All unicast traffic targeted to mesh points in power save will be buffered. These packets will be transmitted only on the DTIM interval.

- Mesh points will not transmit other types of packets but ACK, CTS, RTS, ATIM, Beacon & Null Data during the ATIM window.

- Mesh points that transmit to mesh points in power save state (including broadcast and multicast) will set the More bit in frame control headers to indicate if more frames are to be transmitted for the specific destination.

- All other aspects of ATIM based transmission are as defined in 802.11 specification section 11.2.2.4

- For traffic that belongs to a flow for which an APSD TSPEC and schedule was setup with another mesh point, the transmission will be performed according to the agreed schedule.

11A.13.7 Power management operation with APSD

Power management operation with APSD in a mesh is very similar to the operation in a BSS.

The following are the modifications compared to the description provided in section 802.11-1999 Section 11.2.1.4

The Mesh supports periodic and aperiodic APSD operation modes. The periodic APSD mode is similar to Scheduled APSD. The aperiodic APSD is similar to Unscheduled APSD. The periodic and aperiodic APSDs use the same signaling as scheduled and unscheduled APSDs, but they can be used only with neighbors in a mesh that support it..

Use of aperiodic APSD in a mesh may be limited.  It may only be used if one of the mesh points in the link is Active and the other in Power Save and EDCA is used for transport of the data. In this case the mesh point that operates in power save mode will be the one to initiate the data exchange by sending packets to the other mesh point triggering it to send the traffic back.

Periodic APSD can be used for all cases (i.e., both mesh points in power save or only one, EDCA or HCCA access).

The ADDTS request is modified to include a Schedule element that describes the requested schedule from the mesh point.  The ADDTS response will include the Schedule that can be supported by the other mesh point and that should be used for this flow.  If this schedule is not acceptable to the originating mesh point it may reattempt the ADDTS request with modified schedule or tear down the flow.

For periodic APSD both sides can initiate transactions as long as they are sent within the service interval defined.  The service interval will last up to the maximal service duration as defined in the schedule IE or if EOSP is declared in frames sent/received for that flow.  For a unidirectional flow the originating mesh point sets the EOSP when it wants to end the service interval. The interval will be considered terminated once the ACK is received for that packet (if ACK is required).  For a bi-directional flow the service period will end only after both ends of the flow send a packet with EOSP bit set and the matching ACK packets are received.

11A.13.8 Power Save parameters selection

The power save operation of a mesh point is controlled by a set of global parameters.  The following are the global mesh parameters with their default recommended values:

Beacon Period: 100TU

DTIM period: 10

ATIM Window: 10TU

Mesh points may wish to use other parameters but doing so may effect the power save efficiency and also delay the service initiation in the mesh.

11A.13.9 TS Reinstatement

A mesh point wishing to reinstate a TS with another mesh point that is operating in Power save mode will send a QoS-Null frame to the mesh point in power save during its ATIM window.

11A.13.10 Beacon broadcaster

By default the beacon broadcaster in a mesh is rotating between the MPs. MAPs may need to send a beacon even if another MP had already sent a beacon on that same TBTT.

MPs that support power save operation must also support the deterministic beacon broadcasting selection as described in this section.

An MP not supporting this service will use a random backoff procedure for sending of Beacons as described for IBSS operation in the 802.11-1999 standard. An MP that receives a Beacon after TBTT and before being able to send its own beacon may cancel that beacon transmission. MPs that do implement this service and are set to be the Beacon Broadcaster (BB) will transmit Beacons immediately after TBTT or after a PIFS interval after the clearing of CCA in case CCA was active during TBTT.

An MP that supports this option and starts a mesh will set itself to be the Beacon broadcaster.

11A.13.11 Change of Beacon broadcaster

It is possible to change the beacon broadcaster. In every DTIM period the current beacon broadcaster sets the BB switch bit to 1 if it wants to change the beacon broadcaster. In this case the neighbor that is first in the neighbor list shall start acting as the beacon broadcaster and shall send the next DTIM beacon.

The BB has to make sure that the neighbor appearing at the head of the list is supporting power save transmission and Beacon broadcasting.

If a neighbor assigned to be the beacon broadcaster fails to transmit its beacon (shuts down or goes out of range), other MPs will attempt to take over its role.  MPs supporting the deterministic beacon broadcasting will start attempting to send beacons using the standard backoff procedure with a Neighbor list IE designating itself as the BB if it does not receive 3 consecutive beacons from the last designated BB.

If another MP already transmitted a Beacon with a neighbor list IE designating itself as the BB, then the MP will cancel its pending Beacon.

11A.13.12 Beacon broadcaster power save state

Beacon broadcaster can enter the power save state where it sends only DTIM beacons and stays awake for the next beacon period. In this case, MPs shall not send any frames to the beacon broadcaster during normal beacon periods.

If beacon broadcaster is in power save state it sets the BB PS State bit to 1.

11A.13.13 Naïve Mesh operation (Informative)

This section describes the operation of a naive mesh that does not include any Mesh APs and is also not supporting any routing capabilities.

This type of mesh would be mainly useful in cases were all Mesh point are maintaining a neighbor relationship with each other, as such it does not need to follow the association procedures and may use the three address format to directly exchange frames between the mesh members.

For this specific case the Mesh point does not have to support any of the route messages and link state announcements defined in this specification.

The mesh point will include in its WLAN Mesh Capability IE a Peer Capability field with only bit 15 set. This would signal that the MP is not supporting association with any other MPs and does not support any 802.1X capabilities.

Since the mesh point is not going to receive any association request, and it has no need to initiate one itself, it does not have to support the association messages as well as any of congestion control messages that can be exchanged only between associated peers.

The power save operation is an optional feature and may be implemented by the MPs of the Naive mesh.

A naive mesh supporting power save may use the basic power save scheme for simple data exchange and may optionally extend to include the APSD support for real time power save stream delivery.

References:

[1] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D0.01, March 2006.

[2] Resolution of informal comments received May 1, 2006. doc.: IEEE 802.11-06/0602-09.