IEEE P802.11
Wireless LANs

| Normative Text Proposal for Network Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Patrick Mo | Huawei | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-137-510-553-82 | moly@huawei.com |
| Zhonghui Yao | Huawei | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-755-896-52137 | yaoth@huawei.com |
| Jianfeng Yi | Huawei | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-755-896-50954 | yijianfeng@huawei.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document is the text version of the proposal ‘11-06-0273-00-000u-proposal-network-selection-cluster’.

# Contents

Contents	2

3．Definitions	3

5	General description	3

5.2	Components of the IEEE 802.11 architecture	3

5.2.7	Interworking service	3

7	Frame formats	4

7.2	Format of individual frame types	4

7.2.3	Management Frames	4

7.2.3.1	Beacon frame format	4

7.2.3.4	Association request frame format	4

7.2.3.6	Reassociation request frame format	4

7.2.3.8	Probe request frame format	4

7.2.3.9	Probe response frame format	5

7.3	Management frame body components	5

7.3.2	Information Element	5

7.3.2.36	Path element	5

7.3.2.37	Interworking capability element	5

7.3.2.38	Container element	6

7.4 Action frame format details	7

7.4.5	Network selection action frame format	7

7.4.5.1	External network information Request frame format	7

7.4.5.2	External network information Response frame format	7

10	Layer management	9

10.3	MLME SAP interface	9

10.3.2	Scan	9

10.3.2.1	MLME-SCAN.request	9

10.3.2.2	MLME-SCAN.confirm	9

10.3.6	Associate	10

10.3.6.1	MLME-ASSOCIATE.request	10

10.3.6.2	MLME-ASSOCIATE.confirm	10

10.3.6.3	MLME-ASSOCIATE.indication	11

10.3.7	Reassociate	11

10.3.8	External network information discovery	11

10.3.29.1	MLME-EXTNETINF.request	11

10.3.29.2	MLME- EXTNETINF.confirm	12

10.3.29.3	MLME- EXTNETINF.indication	13

10.3.29.4	MLME- EXTNETINF.response	13

10.3.30	MLME SAP interface for Remote Request	14

10.3.30.1	MLME-REMOTEREQ.indication	14

10.3.30.2	MLME-REMOTEREQ.response	15

11	MLME	17

11.11	Network selection procedure	17

11.11.1	Passive discovery	17

11.11.2	Active discovery	17

11.11.3	External network information discovery flows	17

Annex D	19

Reference	19

# 3．Definitions

Insert the following new definitions after 3.176, renumbering as necessary:

3.177 Entrance: the entrance is in charge of exchanging external network information between a WLAN and external networks, and can be regarded as a logical entity by which AP can retrieve information from external network.

# General description

Insert the following text to the end of the indented list of section 5.2:

n) Interworking Service

[NOTE – the number of the item shall be renumbered after the other standards prior to 11u have been approved.]

## Components of the IEEE 802.11 architecture

Insert the following new clauses after section 5.2.6

### Interworking service

Before a STA associates with a WLAN, it needs know whether its subscription to a SSP would allow it to access this WLAN, so some information discovery mechanisms will be provided for the STA to obtain the related external network information from the external network through a WLAN. An entrance can assist a STA to achieve the external network information discovery.

The entrance (see figure 7) is in charge of exchanging external network information between a WLAN and external networks, and can be regarded as a logical entity by which AP can retrieve information from external network. The entrance may be located on the portal connected to the external network.

STA

AP

1)  External Network

Discovery Request

4)  External Network

Discovery Response

Entrance

External Networks

2) Information

Query Request

Information Query

3) Information

Query Response

Figure 7 ― Architecture of network selection

# Frame formats

## Format of individual frame types

### Management Frames

#### Beacon frame format

Insert the following rows at the appropriate location in Table 8:

Table 8 ― Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 12 | Interworking Capability IE | The Interworking Capability IE may be present if the AP supports interworking functionality. |
| 13 | Path IE | The Path IE may be present if the AP supports interworking functionality. |

#### Association request frame format

Insert the following rows at the appropriate location in Table 10:

Table 10 ― Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Path IE | The Path IE may be present if the non-AP STA supports interworking functionality. |

#### Reassociation request frame format

Insert the following rows at the appropriate location in Table 12:

Table 12 ― Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Path IE | The Path IE may be present if the non-AP STA supports interworking functionality. |

#### Probe request frame format

Insert the following rows at the appropriate location in Table 14 :

Table 14 ― Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Path IE | The Path IE may be present if the non-AP STA supports interworking functionality. |
| x | Container IE | The Container IE may be present if the non-AP STA supports interworking functionality. |
| x | Interworking Capability IE | The Interworking Capability IE may be present if the non-AP STA supports interworking functionality. |

#### Probe response frame format

Insert the following rows at the appropriate location in Table15:

Table 15 ― Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Path IE | The Path IE may be present if theAP supports interworking functionality. |
| x | Container IE | The Container IE may be present if the AP supports interworking functionality. |
| x | Interworking Capability IE | The Interworking Capability IE may be present if the non-AP STA supports interworking functionality. |

## Management frame body components

### Information Element

Inert the following new subsections after subclause 7.3.2.35

[Note - the changes of the 11k/11r/11w will affect the numbering of the subclause.]

#### Path element

Path element indicates the extended service set identifier that current BSS belong to and the SSPNs with that the ESS has a subscription;

The ESSID is a MAC address. The SSPN Number indicates the length of SSPN ID List. The SSPN ID List is a list of SSPN identification which is to be defined.

| Element ID | Length | ESSID | SSPN Number | SSPN ID List |
| --- | --- | --- | --- | --- |

Octets:            1                  1               6                     1                         variable

Figure 114 ― Path element format

#### Interworking capability element

Interworking capability element indicates the interworking capability of local WLAN which includes the whether can access the external network or not、network access control policy indicated by Interworking Capability Indication, supported online enrolment methods indicated by Enrolment methods counter and Enrolment methods List.

| Element ID | Length | Interworking Capability Indication | Enrolment methods counter | Enrolment methods List |
| --- | --- | --- | --- | --- |

Octets:              1                     1                           1                                 1                      variable

Figure 115 ― Interworking Capability element format

The first bit of the Interworking capability indication indicates the capability of accessing external network. The second and third bit of the Interworking capability indication indicates the network access control policy which has the following combinations. The other bits reserved.

| Interworking capability | Credential | Enrolment | Reserved |
| --- | --- | --- | --- |

Bit:                       1                               1                    1                     5

Figure 116 ― Interworking Capability Indication field

Table 44 ― Combination of Network Access Control Policy

| Credential | Enrolment | Network Access Control Policy |
| --- | --- | --- |
| No | No | Open |
| No | Yes | Online Enrolment |
| YES | No | Credential |
| YES | YES | Both enrolment and credential |

[Note - the information element proposed here needs to be merged into the information element proposed in network section proposal.]

#### Container element

Container element is used to indicate the requesting and responding for external network information. Its format is defined as follows.

| Element ID | Length | Request/Response for external network information |
| --- | --- | --- |

Octets:            1                  1                                          variable

Figure 117 ― Container element format

The information of interworking service type or charges is contained in the Request/Response for external network information field.

## 7.4 Action frame format details

Insert the following subclauses after subclause 7.4.4, renumbering as necessary.

### Network selection action frame format

Two Action frame formats are defined for network selection. An Action Value field, in the octet field immediately after the Category field, differentiates the two formats.

#### External network information Request frame format

The External network information Request frame uses the Action frame body format and is transmitted by a STA requesting AP to return external network information requested. The format of the External network information Request frame body is shown in Figure 119.

The Category field is to be defined (representing external network information discovery)

Table 58 ― Network selection Action Value filed values field

| Action value field | Description |
| --- | --- |
| 0 | External network information request |
| 1 | External network information response |
| 2~255 | Reserved |

| Category | Action Value | Container IE |
| --- | --- | --- |

Octets:           1                        1                    variable

Figure 119 ― External network information Request frame body format

The Action Value field shall be set to 0 (representing an External network information Request frame).

The External network information Request Elements field shall contain one or more of Container element described in 7.3.2.38. The number and length of the Container elements in an External network information Request frame is limited by the maximum allowed MMPDU size.

#### External network information Response frame format

The External network information Response frame uses the Action frame body format and is transmitted by an AP responding to STA by external network information which is corresponding to external network information requested. The format of the External network information Response frame body is shown in Figure xxx.

The Category field is to be defined (representing external network information discovery)

| Category | Action Value | Container IE |
| --- | --- | --- |

Octets:              1                      1                    variable

Figure 120 ― External network information Response frame body format

The Action Value field shall be set to 1 (representing an External network information Response frame).

The External network information Response Elements field shall contain one or more of Container element described in7.3.2.38. The number and length of the Container elements in a External network information Response frame is limited by the maximum allowed MMPDU size.

# Layer management

## MLME SAP interface

### Scan

#### MLME-SCAN.request

##### Semantics of the service primitive

Change the primitive parameters of subclause 10.3.2.1.2:

The primitive parameters are as follows:

MLME-SCAN.request (

BSSType,

BSSID,

SSID,

ScanType,

ProbeDelay,

ChannelList,

MinChannelTime,

MaxChannelTime,

Path IE,

Interworking Capability IE,

Container IE

)

Insert the following rows to the table of subclause 10.3.2.1.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Path IE | As defined in frame format | As defined in frame format | The value from the Path IE. |
| Container IE | As defined in frame format | As defined in frame format | The value from the Container IE. |
| Interworking Capability IE | As defined in frame format | As defined in frame format | The value from the Interworking Capability IE. |

#### MLME-SCAN.confirm

Change the primitive parameters of subclause 10.3.2.2.2:

##### Semantics of the service primitive

Add the following rows to the BSSDescription table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Path IE | As defined in frame format | As defined in frame format | The value from the Path IE. |
| Container IE | As defined in frame format | As defined in frame format | The value from the Container IE. |
| Interworking Capability IE | As defined in frame format | As defined in frame format | The value from the Interworking Capability IE. |

### Associate

#### MLME-ASSOCIATE.request

Change the primitive parameters of subclause 10.3.6.1.2:

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.request (

PeerSTAAddress,

AssociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

PathIE

)

Insert the following rows to the table of subclause 10.3.6.1.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PathIE | As defined in frame format | As defined in frame format | The value from the Path IE. This element may be present if dot11InterworkingOptionImplement is true. |

#### MLME-ASSOCIATE.confirm

Change the primitive parameters of subclause 10.3.6.2.2:

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.confirm (

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

Path IE

)

Insert the following rows to the table of subclause 10.3.6.2.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Path IE | As defined in frame format | As defined in frame format | The value from the Path IE. This element may be present if dot11InterworkingOptionImplement is true. |

#### MLME-ASSOCIATE.indication

Change the primitive parameters of subclause 10.3.6.3.2:

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.indication (

PeerSTAAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

Path IE

)

Insert the following rows to the table of subclause 10.3.6.3.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Path IE | As defined in frame format | As defined in frame format | The value from the Path IE. This element may be present if dot11InterworkingOptionImplement is true. |

### Reassociate

It is also needed to amend the relative primitives of Reassociate as the Associate, and here omits to show the amendment of the Reassociate.

### External network information discovery

#### MLME-EXTNETINF.request

##### Function

This primitive indicates a requesting for external network information which can be access though Local WLAN.

##### Semantices of the service primitive

The primitive parameters are as follow:

MLME- EXTNETINF.request (

ExternalNetworkInformationRequestMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ExternalNetwork-InformationReque-stMessage | It is defined as Container element | It is defined as Container element | It is used to request external network information which can be acess though Local WLAN. |

##### When generated

This primitive is generated by the SME to request that a external network information discovery request action frame be sent to external network information server to retrieve external network information.

##### Effect of receipt

On receipt of this primitive, the MLME constructs an external network information discovery request action request frame containing the Container element specified.This frame is then scheduled for transmission.

#### MLME- EXTNETINF.confirm

##### Function

This primitive reports the result of a request to send external network information request action frame.

##### Semantices of the service primitive

The primitive parameters are as follow:

MLME- EXTNETINF.confirm (

ExternalNetworkInformationResponseMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ExternalNetwork-InformationResp-onseMessage | It is defined as Container element | It is defined as Container element | Reports the outcome of a request to send an external network information request action frame. |

##### When generated

This primitive is generated by the MLME when the request to transmit an external network information request action frame completes.

##### Effect of receipt

On receipt of this primitive, the SME return the External network information response message to upper layer.

#### MLME- EXTNETINF.indication

##### Function

This primitive indicates an external network information request action frame has been received requesting the external network information.

##### Semantices of the service primitive

The primitive parameters are as follow:

MLME- EXTNETINF.indication (

ExternalNetworkInformationRequestMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ExternalNetworkIn-formationRequestM-essage message | It is defined as Container element | It is defined as Container element | It is used to request external network information which can be acess though Local WLAN. |

##### When generated

This primitive is generated by the MLME when a valid external network information request action frame is received.

##### Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the requested external network information.

#### MLME- EXTNETINF.response

##### Function

This primitive is used to send a response to a specific peer MAC entity that requested external network information with the STA that issued this primitive.

##### Semantices of the service primitive

The primitive parameters are as follow:

MLME- EXTNETINF.response (

ExternalNetworkInformationResponseMessage

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ExternalNetworkInforma-tionResponseMessage | It is defined as Container element | It is defined as Container element | Reports the outcome of a request to send an external network information request action frame. |

##### When generated

This primitive is generated by the SME of a STA as a response to an MLME- EXTNETINF.indication primitive.

##### Effect of receipt

This primitive initiates transmission of a response to the specific peer MAC entity that requested external network information.

### MLME SAP interface for Remote Request

[Note: It is feasible to use the remote request broker mentioned in TGr to support interworking information query through the DS.]

Before the AP broadcasts the beacon to the STAs or responds the probe response to the STA, it will exchange message with the Entrance over the DS. So some primitives will be defined by TGu to support the interworking information query.

#### MLME-REMOTEREQ.indication

##### Function

This primitive should be issued by the MAC to the RRB when it receives a probe request frame from the STA or when it is preparing the relevant information about the external networks for beacon broadcast.

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTEREQ.indication (

PeerMACAddress,

InformationQueryRequest,

RemoteRequestTimeout,

RequestIdentifier

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| PeerMAC- Address | MACAddress | Any valid individual MAC address | Specifies the MAC address of the entity which responses to the “Information Query Request”. |
| Information-QueryRequest | the query quest about the external networks | As defined in the element about query request | Specifies the query quest about the external networks |
| RemoteRequ-estTimeout | Integer | >= 0 | Specifies a time limit (in TU) after which the remote request procedure will be terminated. |
| RequestIdent-ifier | Integer | ranging from 0 to 216-1 | An identifier that will be used by the AP and the MAC entity within the DS to match a remote request with a remote response. |

##### When generated

This primitive is generated by the MAC as a result of receipt a probe request frame from one STA or as a result of preparing some external network information for beacon broadcast.

##### Effect of receipt

Upon receipt of this primitive, the RRB sends a message of “Information Query Request” to the peer MAC address.

#### MLME-REMOTEREQ.response

##### Function

This primitive reports the query results of external network inforamtion.

##### Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTEREQ.response (

MACAddress,

RequestIdentifier,

Query result,

InformationQueryResponse

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MACAddress | MACAddress | Any valid individual MAC address | Specifies the MAC address of the entity which sends the “Information Query Response”. |
| Request-Identifier | Integer | ranging from 0 to 216-1 | An identifier that will be used by the AP and the MAC entity within the DS to match a remote request with a remote response. |
| Query-Result |  | SUCCESSFUL, TIMEOUT | Indicates the result of the query. SUCCESSFUL means the operation of query external networks information is successful, and the TIMEOUT menas it is timeout when waiting for the query result. |
| Information-Query-Response | the query quest about the external networks | As defined in the element about the query response | Specifies the query response about the external networks |

##### When generated

This primitive is generated by the MAC to the RRB to indicate that it has received the message of “Information Qurey Response” from the peer MAC entity within the DS.

##### Effect of receipt

On receipt of this primitive, the RRB of the AP is notified of the results of the query about the external networks information. If the result is SUCCESSFUL, the RRB will construct one frame cotaining the query response information based on either the beacon broadcast case or the probe response case. Else if the result is TIMEOUT, no external network information is contained in the beacon or probe response.

# MLME

Insert the following subclauses after subclause 11.10, renumbering as necessary.

## Network selection procedure

### Passive discovery

An AP can announce interworking capability information and path information which is about the path from local network to external network by beacon transmission.

The beacon frame contains the interworking capability information element which includes the information as follows: whether supporting interworking with external network or not, whether supporting online registration or not, and which online registration methods be supported.

The beacon frame can contain the path element which contains the SSPNs having a direct subscription with the AP.

The passive discovery provides the following procedures:

― The AP may need to retrieve the external network information from the external network information server by entrance entity.

― The AP announces the interworking capability information and path information by beacon transmission.

― The STAs receive the beacon.

### Active discovery

The STAs can use active discovery to retrieve the interworking capability information overloaded by interworking capability IE, path information overloaded by path IE and external network information overloaded by Container IE from the current WLAN.

Interworking capability IE and Path IE can be contained by probe request/response frame when the STA uses the active discovery.

Container IE can be contained by probe request/response frame and network selection action frame. The requested external information concludes Interworking service type, charge information and so on.

The active discovery provides the following procedures:

― The STA which would like to retrieve interworking capability information or path information can send a probe request frame to AP. The STA which would like to retrieve external network information can send a network selection action request frame to AP.

― The AP receives the coming frame, and then collects the corresponding information. The AP may need to retrieve information from the external network information server by entrance entity.

― The AP returns the corresponding probe response frame or network selection action response frame to the STA which sends the probe request frame or network selection action request frame.

― The STA which sends the probe response or action response frame receives the probe response frame or network selection action response frame.

### External network information discovery flows

When a STA wants to inquire the external network information by action frame, it will initiate the external network information discovery as figure 214.

MLME-EXTNETINF.request

SME

MLME

MLME

MLME-ROMOTEREQ.indication

MLME-EXTNETINF.confirm

MLME-ROMOTEREQ.response

IEEE802.11 STA

IEEE802.11 AP

RRB

Network selection action request

Network selection action response

Information Query request

Entrance

Information Query response

Figure 214--External network information discovery initiated by network selection action frame

When the AP receives a probe request which contains the Path IE or when the AP wants to broadcast a beacon, the AP will initiate the external network information discovery as figure 215.

Information Query response

MLME

MLME-ROMOTEREQ.indication

MLME-ROMOTEREQ.response

IEEE802.11 AP

RRB

Probe Request or preparing for beacon broadcast

Information Query request

Entrance

Probe response or beacon of external

Network information

Figure 215-- External network information discovery initiated by probe request or beacon broadcast.

# Annex D

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11InterworkingCapabilityEnabled TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11InterworkingCapabilityEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting interworking with external network.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }

# Reference

[1]	IEEE P802.11ma-D7.0

[2]	11-05-0822-09-000u-tgu-requirements

[3] 	11-06-0273-00-000u-proposal-network-selection-cluster.ppt