IEEE P802.11
Wireless LANs

| Disassociate Imminent Reinsertion |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Martin Lefkowitz | Consultant | 6966 Shepherd Canyon Rd, Oakland CA 94611 | 510-338-1122 | lefko@sbcglobal.net |

Abstract

This document contains instructions to the editor on how to (re)insert the Disassociate imminent request and report back into the 802.11k draft amendment.  The instructions are based on version 3.7 of the 802.11 document.  The instructions to the editor are in

Times New Roman 14 pt italicised Bold

The changes to the paragraph may be noted in blue for clarity.  It is expected that the color will be removed when the edits are applied.  It is assumed that the editior can, and will, use editorial discression when appropriate.  When there are questions please email Martin Lefkowitz with cc to Richard Pain.

.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

Change

The request/response measurements are:

  - Beacon

  - Frame

  - Channel Load

  - Noise Histogram

  - STA Statistics

  - Location Configuration Information

  - Neighbor Report

  - Link Measurement

  - QoS Metrics

To

The request/response measurements are:

  - Beacon

  - Frame

  - Channel Load

  - Noise Histogram

  - STA Statistics

  - Location Configuration Information

  - Neighbor Report

  - Disassociate Imminent

  - Link Measurement

  - QoS Metrics

Insert after Neighbor Report Paragraph

Dissassociate Imminent

The Disassociation Imminent Action Frame is sent by an AP to indicate to a STA that it may be dissociated after an included time out value.  The message contains information from the table dot11RRMNeighborReportTable in the MIB concerning neighbour APs.  It’s intent is to inform the STA that it’s association may be terminated via a disassociation message while giving updated information concerning its Neighbour AP’s.

Change

Table 57A — Radio Measurement Action field values

| Action field value | Description |
| --- | --- |
| 0 | Radio Measurement Request |
| 1 | Radio Measurement Report |
| 2 | Link Measurement Request |
| 3 | Link Measurement Report |
| 4 | Neighbor Report Request |
| 5 | Neighbor Report Response |
| 6-255 | Reserved |

To

Table 57A — Radio Measurement Action field values

| Action field value | Description |
| --- | --- |
| 0 | Radio Measurement Request |
| 1 | Radio Measurement Report |
| 2 | Link Measurement Request |
| 3 | Link Measurement Report |
| 4 | Neighbor Report Request |
| 5 | Neighbor Report Response |
| 6 | Disassociation Imminent |
| 6-255 | Reserved |

#### Insert after 7.4.5.6 Neighbor Report Response frame format

#### 7.4.5.7 Disassociation Imminent Frame Format

The format of the Disassociation Imminent frame format is shown in Figure 0-6.

|  | Category | Action | Dialog Token | Activation Delay | Neighbor Report Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure 0-6 – Disassociation Imminent frame body format

The Category field shall be set equal to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set equal to the value indicating Neighbor Report Response, as specified in Table 57A in 7.4.5.

The Dialog Token field shall be set equal to the value in the corresponding Neighbor Report Request frame. If the Neighbor Report Response frame is not being transmitted in response to a Neighbor Report Request frame then the Dialog token shall be set equal to zero.

The activation delay shall be set equal to the number of Beacon intervals until the AP may send the disassocation frame to the STA. The miminum value is 1, indicating the next TBTT.

The Neighbor Report Elements field shall contain the Neighbor Report elements described in 7.3.2.36. If the STA has no information in response to the Neighbor Report Request, the Neighbor Report elements shall be omitted. The number and length of the Neighbor Report Elements in a Neighbor Report frame is limited by the maximum allowed MMPDU size.

Insert after Clause 10.3.30 Renumber as needed

### 10.3.31 Disassociation Imminent

The following MLME primitives support the signaling of Disassociation Imminent.

#### 10.3.31.1 MLME-DISASSOCIATEIMMINENT.request

##### 10.3.30.1.1 Function

This primitive requests that a disassociation imminent be sent to a non-AP STA. It is valid only at a Radio Measurement capable AP.

##### 10.3.30.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPRESP.request	(
PeerSTAAddress,
ResultCode,
DialogToken,

ActivationDelay
NeighborListSet
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity to which a Neighbor Report Response frame is to be sent. |
| DialogToken | Integer | 0-255 | The Dialog Token to identify the disassociation imminent transaction. |
| ActivationDelay | Integere | 0-255 | The number of Beacon Intervals that will pass before a Disassociation Frame may be sent. |
| NeighborListSet | Set of Neighbor List elements each as defined in the Neighbor Report element format | Set of Neighbor List elements each as defined in the Neighbor Report element format | A set of Neighbor List elements derived from the MIB table dot11RRMNeighborReportTable each representing a neighboring AP being reported as defined in the Neighbor Report element format. |

##### 10.3.29.1.3 When Generated

This primitive is generated by the SME to request a disassociation imminent be sent to a non-AP STA.

##### 10.3.29.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Disassociation Imminent frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

#### 10.3.30.2 MLME-DISASSOCIATEIMMINENT.confirm

##### 10.3.30.2.1 Function

This primitive reports the result of an attempt to send a Disassociation Imminent frame to a peer STA. It is valid only at a Radio Measurement capable AP.

##### 10.3.30.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DISASSOCIATEIMMINENT.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-DISASSOCIATEIMMINENT.request. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Disassocaiton Imminent transaction. Set to the value in the corresponding MLME-DISASSOCIATIONIMMINENT.request. |

##### 10.3.30.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-DISASSOCIATEIMMINENT.request and indicates the result of the request to send a Disassociation Imminent frame.

This primitive is generated when the Disassociation Imminent frame is successfully transmitted, the MLME-DISASSOCIAITEIMMINENT.request primitive contains invalid parameters, or when a timeout or other failure occurs.

##### 10.3.30.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.30.3 MLME-DISASSOCIATEIMMINENT.indication

##### 10.3.30.3.1 Function

This primitive indicates that a Disassociation Imminent has been received. It is valid only at a Radio Measurement capable non-AP STA.

##### 10.3.30.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DISASSOCIATEIMMINENT.indication	(
PeerSTAAddress, 
DialogToken,

ActivationDelay,
NeighborListSet
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the AP from which the Neighbor Report Response frame was received. |
| DialogToken | Integer | 0-255 | The Dialog Token received in the Neighbor Report Response frame to identify the neighbor report transaction. |
| ActivationDelay | Integere | 0-255 | The number of Beacon Intervals that will pass before a Dissassociation Frame may be sent. |
| NeighborListSet | Set of Neighbor List elements each as defined in the Neighbor Report information element format | Set of Neighbor List elements each as defined in the Neighbor Report information element format | A set of Neighbor List elements derived from the MIB table dot11RRMNeighborReportTable, each representing a neighboring AP being reported as defined in the Neighbor Report information element format. |

##### 10.3.30.3.3 When Generated

This primitive is generated by the MLME when a valid Neighbor Report Response Frame is received.

##### 10.3.30.3.4 Effect of Receipt

On receipt of this primitive, the amount of time before a dissassocation frame may be send and neighbor report data may be available to the SME.

Insert After clause 11.12 Renumber as needed

### 11.13 Disassociation Imminent

When a STA receives a Disassociate Imminent message, meaning it has n number of Beacon Intervals before a disassociate message may come, it will;

- Determine which AP it shall (re)associate to next based on its internal algorithms that are outside the scope of the specification.

- If the STA determines that it should switch to another ESS, it may request a Neighbour Report for that SSID from the current AP.