IEEE P802.11
Wireless LANs

| Normative Text for Vendor Specific Action MLME SAP Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-7-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Emily H. Qi | Intel Corporation | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |

- Abstract

This document is a normative text proposal to address 802.11 REV-ma Recirculation 2 Comment #30 to add MLME SAP interface for Vendor Specific Action frame.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert the following clause after clause 10.3.29:

    - Vendor Specific Action

This set of primitives supports the signaling of Vendor Specific Action frames between peer SMEs.

      - MLME-VSPECIFIC.request

        - Function

This primitive requests transmission of a Vendor Specific Action frame to a peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-VSPECIFIC.request	(
Peer MAC Address,
Vendor Specific Content
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Vendor Specific Action frame is sent. |
| Vendor Specific Content | Set of information elements and vendor specific fields | Set of information elements and vendor specific fields | A set of information elements as defined in 7.3.2. A set of vendor specific fields specifying the required fields for the Vendor Specific Action frame. |

        - When Generated

This primitive is generated by the SME to request that a Vendor Specific Action frame be sent to a peer entity.

        - Effect of Receipt

On receipt of this primitive, the MLME constructs a Vendor Specific Action frame containing the set of information elements and vendor specific fields. The STA then attempts to transmit the frame to the peer entity.

      - MLME-VSPECIFIC.confirm

        - Function

This primitive reports the result of a request to send a Vendor Specific Action frame to the peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-VSPECIFIC.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME- VSPECIFIC.request. |

        - When Generated

This primitive is generated by the MLME when the request to transmit a Vendor Specific Action frame completes and indicates the results of the request.

        - Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

      - MLME-VSPECIFIC.indication

        - Function

This primitive indicates that a Vendor Specific Action frame has been received from a peer entity.

        - Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-VSPECIFIC.indication	(
Peer MAC Address,
Vendor Specific Content
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which a Vendor Specific Action frame was received. |
| Vendor Specific Content | Set of information elements and vendor specific fields | Set of information elements and vendor specific fields | A set of information elements as defined in 7.3.2. A set of vendor specific fields specifying the required fields for the Vendor Specific Action frame. |

        - When Generated

This primitive is generated by the MLME when a valid Vendor Specific Action frame is received.

        - Effect of Receipt

On receipt of this primitive, the Vendor Specific Content can be made available for SME processes.