IEEE P802.11
Wireless LANs

| Theoretical Throughput Limits TGT Draft Appendix A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Larry Green | Ixia | 402 E. Gutierrez Street Santa Barbara, CA 93101 | +1 818 444 2901 | lgreen@ixiacom.com |
| Ken Balmy | Ixia | 402 E. Gutierrez Street Santa Barbara, CA 03101 | +1 818 444 2904 | kbalmy@ixiacom.com |
| Marc Emmelmann | Technical University Berlin | Einsteinufer 25 10587 Berlin, Germany | +49 30 314 24580 | emmelmann@ieee.org |

Abstract

Appendix A provides a methodology for calculating Theoretical Throughput Limits (TTL) for IEEE 802.11a/b/g networks.  A Four-Step Methodology is based on sequential steps to calculate TxTime, Frame Start-to-Frame Start interval, Frame Rate and Theoretical Throughput Limit measured in megabits per second.  Equations and timing parameters are provided for all IEEE 802.11 modulation types.

The TTL calculation methodology is useful in predicting throughput performance of IEEE 802.11 networks

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).   The material in this document is subject to change in form and content after further study.  The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group.  If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This contribution is intended to be inserted as Annex A into IEEE P802.11.2, TGT Draft 0.90, Recommended Practice for the Evaluation of 802.11 Wireless Performance.

Equations and timing parameters throughout this document are considered to be accurate, but may vary from one implementation to another.  Actual test results may also be influenced by proprietary implementations of the IEEE 802.11 Standard.

1.0 Introduction

Annex A provides a methodology for calculating Theoretical Throughput Limits (TTL) for IEEE 802.11a/b/g networks.  The IEEE 802.11 standard defines data rates in terms of signaling rate at the Physical Layer Convergence Procedure (PLCP) Layer, in the form of PLCP Protocol Data Units (PPDUs).  The PPDU data is modulated and transmitted over the RF link at this rate, generally known as “PHY data rate”.  In practice, MSDU data throughput will be significantly lower than the PHY data rate.

The basic Theoretical Throughput Limit (TTL) calculation takes into account the overhead associated with frame preambles, PLCP header, RF modulation type, interframe spacing, packet acknowledgement delay, and number of backoff slots, resulting in a theoretical limit on MSDU throughput.  IFS and related frame timing is defined by DCF mechanisms.

This upper boundary is useful in predicting performance of 802.11a/b/g networks.

For simplification, basic TTL is calculated over a single unidirectional data path by assuming constant-size data frames transmitted with minimum frame spacing, without collisions, without retries, without RTS/CTS controls, without fragmentation, without EDCA services, and without management frame overhead.  These forms of overhead may be factored into the basic TTL calculation as needed. Upper layer protocol processing and application overhead is not considered in TTL calculations.  Note that TTL calculations may be influenced by special algorithms used in proprietary 802.11a/b/g devices.

Clause references are made to IEEE P802.11-REVma/D7.0, dated June, 2006.

2.0 TTL Calculation Methodology

2.1 Four-Step Methodology

The TTL calculation methodology is based on four sequential steps:

1)  Calculate TxTime, the time in microseconds to transmit one data frame, including frame preamble, frame header, and RF modulation parameter fields.

2)  Calculate Frame Start-to-Frame Start Interval, the time in microseconds to transmit one data frame, with acknowledgement, including SIF, DIF and backoff time.  For backoff time, the average backoff in the best case is used where the medium is available at first attempt and the number of backoff slots is selected from [1 to CWmin].

3)  Calculate Frame Rate, the number of frames per second that can be transmitted across the air interface.

4)  Derive Theoretical Throughput Limit in megabits per second.  This value represents an upper boundary on 802.11 network performance at the MSDU level.

An example of the Four-Step Methodology calculation is provided for 802.11a OFDM in Section 4.4, deriving the Theoretical Throughput Limit in megabits per second.

2.2  PHY Modes and Modulation Types

TTL calculations are derived from equations and parameters based on 802.11 modulation types, protocol timing and frame preambles, organized by clauses in the 802.11 Standard:

Clause 14    FHSS

Clause 15    DSSS

Clause 16    IR

Clause 17    OFDM, including an example of TTL calculation for 802.11a

Clause 18    HR/DSSS

Clause 19.1 ERP-OFDM

Clause 19.5 ERP-OFDM

Clause 19.6 ERP-PBCC

Clause 19.7 DSSS-OFDM

3.0 Terminology for TTL Calculations

Ceiling…….round up to nearest multiple of significance

CkSwTime...Clock Switch Time for PBCC

CWmin……best-case (theoretical) Contention Window time measured in Slots

DIFS………Distributed Inter-frame Space

Floor………round down to nearest multiple of significance

FS…………Frame Start

Length…….number of octets in data payload

Ndbps……..number of data bits per symbol (function of PHY rate)

PLCPtime…time to transmit PLCP header

PHYrate…...data transfer rate in bits per second at PLCP Layer

Slot time…..backoff timing interval

SIFS…….....Short Inter-Frame Space

SigEx……...Signal Extension

Tack…….....time to transmit Acknowledgement (ACK) Frame of 14-bytes

Tpream……time to transmit preamble

TxTime…....time to transmit one data frame

Tsignal….....time to transmit Signal Field

Tsym……....time to transmit Service Field (defines symbol clock and code)

4.0  TTL Equations and Timing Parameters

4.1  FHSS  Clause 14

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + PLCPtime + Ceiling ((Length * 8) / PHYrate)

Tpream = 96 usec		PLCPtime = 32 usec

Backoff Slot = 50 usec		CWmin = 15

SIFS = 28 usec			PHYrate = 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5

Tack @ 1 Mbps = 208 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Slots = 28 + (2 * 28) = 84 usec

Best-case Backoff = CWmin / 2 = 15 / 2 = 7.5

Step 3)  Frame Rate Calculation (frames per second)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.2  DSSS  Clause 15

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + PLCPtime + Ceiling ((Length * 8) / PHYrate)

Tpream = 144 usec		PLCPtime = 48 usec

Backoff Slot = 20 usec		CWmin = 31

SIFS = 10 usec			PHYrate = 1, 2 Mbps

Tack @ 2 Mbps = 152 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Slots = 10 + (2 * 20) = 50 usec

Best-case Backoff = CWmin / 2 = 31 / 2 = 15.5

Step 3)  Frame Rate Calculation (frames per second)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.3 IR  Clause 16

Step 1)  TxTime Calculation (usec)

1 Mbps TxTime = Tpream + PLCPtime + Ceiling ((4 + (Length * 8) / PHYrate)

2 Mbps TxTime = Tpream + PLCPtime + Ceiling ((2 + (Length * 8) / PHYrate)

For 1 Mbps:  Tpream = 16 usec	 PLCPtime = 41 usec	Tack = 208 usec

For 2 Mbps:  Tpream = 20 usec	 PLCPtime = 25 usec	Tack = 152 usec

CWmin = 63		SIFS = 10 usec		PHYrate = 1, 2 Mbps

Backoff Slot = 8 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Slots = 10 + (2 * 8) = 26 usec

Best-case Backoff = CWmin / 2 = 63 / 2 = 31.5

Step 3)  Frame Rate Calculation (usec)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.4  OFDM  802.11a   Clause 17

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + Tsignal + Tsym * Ceiling ((Pad + (Length * 8) + Tail) / Ndbps)

Tpream = 16 usec	     	Tsig = 4 usec			Tsym = 4 usec

SIFS = 16 usec		    	Backoff Slot = 9 usec		CWmin = 15

Tack @ 24 Mbps = 28 usec        Pad = 16			Tail = 6

PHY Rate (Mbps) =	6	9	12	18	24	36	48	54

Ndbps =     24         36	48	72	96       144       192       216

Step 2)  Frame Start-to-Frame Start Interval Calculation (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Slots = 16 + (2 * 9) = 34

Best-case Backoff = CWmin / 2 = 15 /2 = 7.5

Step 3)  Frame Rate Calculation (frames per second)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

With default parameters of the BSS basic rate set, the duration and PHY rate at which ACKs are transmitted is listed in Table 1.

Table 1  ACK PHY Rate and Duration

| MPDU PHY Rate | ACK PHY Rate | ACK Duration |
| --- | --- | --- |
| 54 | 24 | 28 |
| 48 | 24 | 28 |
| 36 | 24 | 28 |
| 24 | 24 | 28 |
| 18 | 12 | 32 |
| 12 | 12 | 32 |
| 9 | 6 | 44 |
| 6 | 6 | 44 |

Example TTL Calculation for Clause 17, 802.11a OFDM

PHYrate = 54 Mbps

Length = 1024 bytes  (This is an arbitrary value chosen for this example and may range from 0 to 2304 octets without encryption.)

Step 1)  TxTime as a function of PHY Rate and Length

TxTime = 16 + 4 + 4 * Ceiling ((16 + (8 * 1024 + 6) / 216) = 176 usec

Step 2)  Frame Start to Frame Start Interval

FS-to-FS Interval = 176 + 16 + 28 + 34 + (7 * 9) = 321.5 usec

Step 3)  Frame Rate

FR = Floor (1000000 / 321.5 = 3110.4 frames per second

Step 4)  Theoretical Throughput Limit

TTL = 3110.4 * (1024 * 8) / 1000000 = 25.48 Mbps

4.5 HR/DSSS   Clause 18

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + PLCPtime + Ceiling(((Length + PBCC) * 8) / PHYrate)

PBCC Indicator = 1 for PBCC, 0 for CCK

PBCC Mode @ 33 Mbps, add 1 usec for Clock Switch Time

Long  Preamble = 144 usec	Long PLCP Header = 48 usec

Short Preamble  =   72 usec	Short PLCP Header = 24 usec

Backoff Slot = 20 usec		CWmin = 31	SIFS = 10 usec

DSSS PHYrate =  2, 5.5, 11	PBCC PHYrate = 5.5, 11, 22, 33

Tack @ 2 Mbps = 152 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Slots = 10 + (2 * 20) = 50 usec

Best-case Backoff = CWmin / 2 = 31 / 2 = 15.5

Step 3)  Frame Rate Calculation (usec)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.6  ERP-DSSS/CCK   Clause 19.1

Requires Short Preamble and Short PLCP Header

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + PLCPtime + Ceiling ((Length * 8) / PHYrate)

Tpream = 72 usec		PLCPtime = 24 usec		Backoff Slot = 20 usec

PHYrate = 2, 5.5, 11 Mbps 	Tack @ 5.5 Mbps = 116 usec	CWmin = 31

SIFS = 10 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Backoff Slots

Best-case Backoff = CWmin / 2 = 31 / 2 = 15.5

Step 3)  Frame Rate Calculation (usec)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.7  ERP-OFDM    Clause  19.5

Step 1)  TxTime Calculation (usec)

TxTime = Tpream + Tsignal + Tsym * Ceiling ((16 + (8 * Length) + 6) / Ndbps) + SigEx

Tpre = 16 usec			Tsignal = 4 usec		Tsym = 4 usec

SIFS = 10 usec			Backoff Slot = 9 usec		CWmin = 15

Tack @ 24 Mbps = 28 usec 	SigEx = 6 usec

PHY Rate (Mbps) =	6	9	12	18	24	36	48	54

Ndbps =     24         36	48	72	96       144       192       216

Step 2)  Frame Start-to-Frame Start Interval (usec)

TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Backoff Slots = 10 + (2 * 9) = 28 usec

Backoff = CWmin / 2 = 7.5

Step 3)  Frame Rate Calculation (frames per second)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.8  ERP-PBCC   Clause 19.6

Step 1)  TxTime Calculation (usec)

For PBCC 5.5 and 11 Mbps:

TxTime = Tpream + PLCPtime + Ceiling(((Length + PBCC) * 8) / PHYrate)

For ERP-PBCC 33 Mbps:

TxTime = Tpream + PLCPtime + Ceiling(((Length + PBCC) * 8) / PHYrate)

+ CkSwTime

Long  Preamble = 144 usec	Long PLCP Header = 48 usec

Short Preamble  =   72 usec	Short PLCP Header = 24 usec

PBCC: CCK = 0, PBCC =1	CkSwTime = 1 usec	Backoff Slot = 20 usec

SIFS = 10 usec			CWmin = 31

Tack @ 2 Mbps = 152 usec

Step 2)  Frame Start-to-Frame Start Interval (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Backoff Slots

Best-case Backoff = CWmin / 2 = 31 / 2 = 15.5

Step 3)  Frame Rate Calculation (usec)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

4.9  DSSS-OFDM   Clause  19.7

Step 1)  TxTime Calculation (usec)

TxTime = TpreamDSSS + PLCPtimeDSSS + TpreamOFDM + Tsignal + 4 * Ceiling

((+ 8 * (Length) + Pad) / Ndbps ) + SigEx

Long  Preamble = 144 usec	Long PLCP Header = 48 usec

Short Preamble  =   72 usec	Short PLCP Header = 24 usec

Tpream = 144 usec		PLCPtime = 48 usec	TpreamOFDM = 8 usec

Tsignal = 4 usec		SIFS = 10 usec		Backoff Slot = 9 usec

Tack @ 24 Mbps = 28 usec	SigEx = 6 usec		CWmin = 31

PHY Rate (Mbps) =	6	9	12	18	24	36	48	54

Ndbps =     24         36	48	72	96       144       192       216

Step 2)  Frame Start-to-Frame Start Interval Calculation (usec)

FS-to-FS Interval = TxTime + SIFS + Tack + DIFS + Backoff

DIFS = SIFS + 2 Backoff Slots = 10 + (2 * 9) = 28 usec

Best-case Backoff = CWmin / 2 = 31 / 2 = 15.5

Step 3)  Frame Rate Calculation (frames per second)

FR = 1000000 / FS-to-FS Interval

Step 4)  Theoretical Throughput Limit Derivation (Mbps)

TTL = Frame Rate * Length * 8 / 1000000

# 5.0 Frame Sequences

The frame sequences used for data transfer have a large impact on the maximum frame rate.  To compute an accurate estimate of the maximum frame rate, identification of the proper frame sequence is required.  In general, DCF is the access method most commonly used.

## 5.1 Distributed Coordination Function (DCF)

In the notation of Clause 9.7, variants of DCF are described as follows:

{{RTS – CTS –} | CTS –} [Frag – ACK –] Last – ACK

A DCF transaction begins with an optional RTS – CTS exchange or a single CTS-to-self, followed by one or more “fragment – ACK” pairs conveying fragments of an MSDU, followed by a final fragment and ACK (“last – ACK”).  The em-dash “–” denotes a SIFS between the frames.  For the purposes of this section, fragmentation will not be considered, as it does not yield the maximum MSDU rate.  Given this, the notation will be changed to refer to an “MSDU” rather than a “last” fragment.  This yields the following frame sequence combinations:

MSDU – ACK

CTS – MSDU – ACK

RTS – CTS – MSDU – ACK

Following the completion of each of these frame sequences, there is a DIFS period, followed by a random number of backoff slots, determined by the current contention window.  This constitutes a complete DCF transmission.