IEEE P802.11
Wireless LANs

| Text Proposal for LB84 Comment Resolution CIDs Related to the Smoothing Bit Definition and CSD Restriction |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1 858 521 5885 | verceg@broadcom.com |

Abstract

This document includes proposed text for modification of the TGn 1.06 draft. The text is related to the LB84 Comment IDs (CIDs) 2946, 2953, 7167, 12134, 11976, and 8134.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Introduction

This document addresses CIDs 2946, 2953, 7167, 12134, 11976, and 8134.

# CIDs 2946, 2953, 7167, 12134, 11976

These CIDs refer to the fact that it is not clearly defined when the smoothing is set to one and when to zero. We suggest to clarify the definition already present in section on Spatial Mapping and to add text referring to this definition when the smoothing bit is mentioned. We also change the requirement from 600ns to 800ns to accommodate smoothing with 4 space-time streams.

TGn editor: Replace the following paragraph on pg 246-247, ln 63-1, Draft Version 1.06:

“If the smoothing field in the HT-SIG is not set to 0, the time domain channel between any space time stream and any transmit chain input, induced by the frequency dependency in the matrix Qk and the CSD added according to Cyclic shift values of HT portion of the packet, shall not exceed 600ns.”

With the following paragraph:

“If 95 percent of the sum of the energy from all impulse responses of the time domain channels between all space time streams and all transmit chain inputs, induced by the CSD added according to Table n61 and the frequency-dependence in the matrix Qk, is contained within 800 ns, the smoothing bit should be set to 1, otherwise it shall be set to zero.”

# CID 8134

Pg         Ln

| 20.3.4.7.1 | 212 | 9-10 | It is stated that when spatial expansion is used, the additional CSD value per TX shall not exceed 200 ns. It is not clear what this exactly means. For instance, for 2 transmitters and 2 spatial streams, the second transmitter uses -400 ns cyclic delay. If spatial expansion is used with 4 transmitters, is it then allowed to use cyclic delay values of {0, -400, +200, -600} or {0, -400, -200, -600}? | Clarify the text. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGn editor: Remove the following sentence on pg 247, ln 6-7, Draft Version 1.06:

“When spatial expansion is applied to the transmission, allowing more transmit chains to be used than spatial streams, the additional CSD value for each TX chain shall not exceed 200 ns.”

Reason for Removal:

The 200ns restriction as written is meaningless because it is impossible to delineate between "additional CSD" and "frequency-dependence of Qk".  Furthermore, spatial expansion is generally allowed to introduce arbitrarily long impulse responses (e.g. with "different spatial expansion over subcarriers" or with SVD beamforming), so it makes no sense to restrict the "additional CSD" to 200 ns when longer impulse responses are already allowed.  In summary, this restriction is both impractical and unnecessary, so we propose eliminating it.