IEEE P802.11
Wireless LANs

| Normative Text Proposal for WLAN Paging and Idle Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sunghyun Choi | Seoul National University | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-2-880-1753 | schoi@snu.ac.kr |
| Kyunghun Jang | Samsung Advanced Institute of Technology |  |  | khjang@samsung.com |
| Moo Ryong Jeong | DoCoMo USA Labs | 3240 Hillview Avenue, Palo Alto CA | 650-496-4761 | jeong@docomolabs-usa.com |
| Sunggeun Jin | ETRI | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-28801755 | sgjin@mwnl.snu.ac.kr |
| Youngsoo Kim | Samsung Advanced Institute of Technology |  |  | kimyoungsoo@samsung.com |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA | 9254742290 | matta@trapezenetworks.com |
| R. R. Miller | AT&T | 180 Park Ave, Florham Park NJ | 973-236-6920 | rrm@att.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | 408-754-1213 | Subbu@arubanetworks.com |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-264-7799 | Emily.H.Qi@intel.com |
| Dorothy Stanley | Aruba Networks | Warenville, IL | 630-363-1389 | dstanley@arubanetworks.com |
| James Tsai | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-5509 | James.tsai@intel.com |
| Jesse Walker | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-1849 | Jesse.Walker@intel.com |
| Fujio Watanabe | DoCoMo USA Labs | 240 Hillview Avenue, Palo Alto, CA | 650-496-4726 | watanabe@docomolabs-usa.com |

Abstract

This document contains normative text for a Paging and Idle Mode proposal to meet the TGv Power Saving objective, REQ2010, improving mobile client power efficiency and battery life. The text is aligned with P802.11-REVma/D7.0 and P802.11v D0.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

3. Definition	4

4. Abbreviations and Acronyms	5

7. Frame formats	6

7.1 MAC frame formats	6

7.2.3.1 Beacon frame format	6

7.2.3.9 Probe Response frame format	7

7.3.2 Information Elements	7

7.3.2.43 Paging Service element	8

7.3.2.44 Paging Indication element	9

7.3.2.45 Idle Mode Request element	9

7.3.2.46 Idle Mode Response	10

7.4 Action frame format details	12

7.4.6 Wireless Network Management action details	12

7.4.6.8 Idle Mode Request frame format	12

7.4.6.9 Idle Mode Response frame format	12

10. Layer management	14

10.3 MLME SAP Interface	14

10.3.38 Paging Inidication	14

10.3.38.1 MLME-PIREQUEST.request	14

10.3.38.1.1 Function	14

10.3.38.1.2 Semantics of the service primitive	14

10.3.38.1.3 When generated	14

10.3.38.1.4 Effect of receipt	14

10.3.38.2 MLME-PIREQUEST.confirm	14

10.3.38.2.1 Function	14

10.3.38.2.2 Semantics of the service primitive	14

10.3.38.2.3 When generated	15

10.3.38.2.4 Effect of receipt	15

10.3.38.3 MLME-PIREQUEST.indication	15

10.3.38.3.1 Function	15

10.3.38.3.2 Semantics of the service primitive	15

10.3.38.3.3 When generated	15

10.3.38.3.4 Effect of receipt	15

10.3.39 Idle Mode request	16

10.3.39.1 MLME-IMREQUEST.request	16

10.3.39.1.1 Function	16

10.3.39.1.2 Semantics of the service primitive	16

10.3.39.1.3 When generated	16

10.3.39.1.4 Effect of receipt	16

10.3.39.2 MLME-IMREQUEST.confirm	16

10.3.39.2.1 Function	16

10.3.39.2.2 Semantics of the service primitive	16

10.3.39.2.3 When generated	17

10.3.39.2.4 Effect of receipt	17

10.3.39.3 MLME-IMREQUEST.indication	17

10.3.39.3.1 Function	17

10.3.39.3.2 Semantics of the service primitive	17

10.3.39.3.3 When generated	18

10.3.39.3.4 Effect of receipt	18

10.3.40 Idle Mode response	18

10.3.40.1 MLME-IMRESPONSE.request	18

10.3.40.1.1 Function	18

10.3.40.1.2 Semantics of the service primitive	18

10.3.40.1.3 When generated	19

10.3.40.1.4 Effect of receipt	19

10.3.40.2 MLME-IMRESPONSE.confirm	19

10.3.40.2.1 Function	19

10.3.40.2.2 Semantics of the service primitive	19

10.3.40.2.3 When generated	19

10.3.40.2.4 Effect of receipt	20

10.3.40.3 MLME-IMRESPONSE.indication	20

10.3.40.3.1 Function	20

10.3.40.3.2 Semantics of the service primitive	20

10.3.40.3.3 When generated	20

11. MAC sublayer management entity	21

11.3 Relationships between services	21

11.15 Wireless Network Management Procedures	21

11.15.7 Idle Mode and Paging	21

11.15.7.1 Paging Capability Discovery	22

11.15.7.2 Paging Service Information Advertisement	22

11.15.7.3 Enter Idle Mode	22

11.15.7.4 Idle Mode Update	23

11.15.7.4.1 Paging Group Update	23

11.15.7.4.2 Keep-Alive Update	23

11.15.7.5 Paging	23

11.15.7.5.1 AP operation for sending Paging Indication	24

11.15.7.5.2  STA operation for receiving Paging Indication	24

11.15.7.6 Exit Idle Mode	24

Annex A	25

A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition	25

A.4.3 IUT configuration	25

A.4.14 Wireless Network Management extensions	25

# 3. Definition

3.v.1 Idle Mode: An extended power save mode for non-AP STAs. While in Idle Mode, a STA needs not listen for every Beacon frame, may traverse multiple BSSs in an ESS, and still be reachable from the ESS.

3.v.2 Home AP: The AP to which a STA is associated, and to which the STA signals to enter Idle Mode. The Home AP maintains state for the Idle Mode STA.

3.v.3 Paging Domain: A set of BSSs within the same ESS, identified by a Paging Domain Identifier. APs in the same paging domain advertise the same Paging Domain Identifier. A Paging Domain is composed of a number of Paging Groups.

3.v.4 Paging Domain Identifier: A 48-bit value that identifies a Paging Domain.

3.v.5 Paging Group: A group of APs in which a STA can be paged. A number of Paging Groups constitute a Paging Domain.

3.v.6 Paging Group Identifier: A one octet value that identifies a Paging Group

3.v.7 Paging Server: An entity that provides paging service for a STA. Paging Server can be in the AP (distributed) or in a separate network entity (centralized). The Paging Server maintains paging service information for STAs  in Idle Mode within a Paging Domain. Paging service information includes the STA MAC Address, Paging Group ID, Paging Domain ID, Paging Server ID, and Home AP.

3.v.8 Paging Server Identifier: A 48-bit value that identifies the Paging Server.

# 4. Abbreviations and Acronyms

DPIM: Delivery Paging Indication Message

# 7. Frame formats

## 7.1 MAC frame formats

#### 7.2.3.1 Beacon frame format

Insert new element in Table 8 as indicated below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 29 | Paging Service | The Paging Service element is present only within Beacon frames generated by APs which support Wireless Network Management Paging Service. |
| 30 | Paging Indication | The Paging Indication element is present only within Beacon frames generated by APs which support Wireless Network Management Paging Service and the DPIM Count field in the Paging Service element is 0. |

#### 7.2.3.8 Probe Response frame format

Insert a new row into table 14 as follows:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Paging Service | The Paging Service element is present only within Probe Request frames generated by non-AP STAs which support Wireless Network Management Paging Service. |

#### 7.2.3.9 Probe Response frame format

Insert a new row into table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 27 | Paging Service | The Paging Service element is present only within Probe Response frames generated by APs which support Wireless Network Management Paging Service. |
| 28 | Paging Indication | The Paging Indication element is present only within Probe Response frames generated by APs which support Wireless Network Management Paging Service. |

### 7.3.2 Information Elements

Insert new rows into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Paging Service | x+9 |
| Paging Indication | x+10 |
| Idle Mode Request | x+11 |
| Idle Mode Response | x+12 |
| Reserved | x+13, 220 |

Change Clause 7.3.2.35 as shown below:

7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v2.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

- Figure v2 —Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure 3.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | Paging | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 11 |  |

- Figure v3 —Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in Error: Reference source not found. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in Error: Reference source not found. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in xxx. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in clause Error: Reference source not found. The Presence bit set to 0 indicates that the STA does not support this service.

- The Paging bit set to 1 indicates that the STA supports Paging Service and Idle mode operation as described in 11.15.7. The Paging bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

Add new clauses after Clause 7.3.2.42 as shown below:

### 7.3.2.43 Paging Service element

The Paging Service information element contains paging service and paging indication delivery information. The format of the Paging Service Information element is shown in Figure v-1.

|  | Element ID | Length | Paging Domain ID | Paging Server ID | Paging Group ID | Paging Interval | DPIM Count |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 1 | 1 | 1 |

- Figure v-1—Paging Service element format

The Element ID field is equal to the Paging Service Information value in Table 26.

The value of the Length field is set to 15.

The Paging Domain ID field contains a 6-byte value that identifies Paging Domain.

The Paging Server ID field contains a 6-byte value which identifies the entity that provides the Paging Service.

The Paging Group ID field contains one byte value that identifies the Paging Group.

The Paging Interval field indicates the number of Beacon intervals between successive DPIMs. If the Paging Interval equals the Beacon interval, the Paging Interval field has the value 1. The Paging Interval value 0 is reserved. The Paging Interval field is a single octet. The Paging Interval is set to 0 in the Paging Service element contained in the Probe Request frame.

The DPIM Count field indicates how many beacons (including the current frame) appear before the next DPIM. A DPIM Count of 0 indicates that the current Beacon is a DPIM. The DPIM count field is a single octet. If the Paging Service information element is present in Probe Response frame, the DPIM Count field is set to the value that appears in the next beacon. The DPIM Count is set to 0 in the Paging Service element contained in the Probe Request.

### 7.3.2.44 Paging Indication element

The Paging Indication Information element contains a paging bitmap and one or more Paged STA Indication sub-elements. The format of the Paging Indication element is shown in Figure v-2.

|  | Element ID | Length | Page Bitmap Control | Partial Virtual Bitmap |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1– 251 |

- Figure v-2— Paging Indication element format

The Element ID field is equal to the Paging Indication value defined in Table 26.

The Length field for this element indicates the length of the information field, which is constrained as described below.

The Page Bitmap Control field is a single octet. Bit 0 of the field contains the Page Indicator bit associated with Paging ID 0. This bit is set to 1 in Paging Indication elements when one or more STAs are currently paged. The remaining 7 bits of the field form the Bitmap Offset.

The Partial Virtual Bitmap, maintained by the Paging Server, consists of 2008 bits, and is organized into 251 octets such that bit number N (0 ≤ N ≤ 2007) in the bitmap corresponds to bit number (N mod 8) in octet number N / 8where the low-order bit of each octet is bit number 0, and the high order bit is bit number 7.  Each bit in the partial virtual bitmap corresponds to paging for a specific station within the Paging Domain that the Paging Server had received a paging frame at the time the paging indication is transmitted. Bit number N is 0 if there are no paging for the station whose Paging ID is N. If any paging notification for that station arrive at the Paging Server, bit number N in the page-indication virtual bitmap is 1.

The Partial Virtual Bitmap field consists of octets numbered N1 through N2 of the paging indication virtual bitmap, where N1 is the largest even number such that bits numbered 1 through (N1 8) – 1 in the bitmap are all 0 and N2 is the smallest number such that bits numbered (N2 + 1) 8 through 2007 in the bitmap are all 0. In this case, the Bitmap Offset subfield value contains the number N1/2, and the Length field will be set to (N2 – N1)+4.

In the event that all bits other than bit 0 in the virtual bitmap are 0, the Partial Virtual Bitmap field is encoded as a single octet equal to 0, and the Bitmap Offset subfield is 4.

### 7.3.2.45 Idle Mode Request element

The Idle Mode Request element is used to request Idle Mode operation. The format of the Idle Mode Request element is shown in Figure v-5.

|  | Element ID | Length | Request Type | STA Address | Paging Domain ID | Paging Server ID | Paging Group ID |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 6 | 6 | 1 |

- Figure v-5— Idle Mode Request sub-element format

The Element ID field is equal to the Idle Mode Request value defined in Table 26.

The value of the Length field is set to 13.

The Request Type is set to a number that identifies a Idle Mode request. The Request Types for Idle Mode requests are shown in Table v-2

- Table v-2—Request Type Definitions

| Name | Request Type |
| --- | --- |
| Exit Idle Mode | 0 |
| Enter Idle Mode | 1 |
| Update Idle Mode | 2 |
| Reserved | 3 – 255 |

The STA Address field contains the MAC address of the STA sending Idle Mode Request.

The Paging Domain ID field contains a 6-byte value that identifies Paging Domain.

The Paging Server ID field contains a 6-byte value that identifies the entity that provides the paging service.

The Paging Group ID field contains one byte value that identifies the Paging Group.

### 7.3.2.46 Idle Mode Response

The Idle Mode Response element is used to respond to an Idle Mode operation request. The format of the Idle Mode Response sub-element is shown in Figure v-5.

|  | Element ID | Length | Response Type | Response Status | STA Address | Paging Server ID | Paging Group ID | Paging ID | Keep-alive Timer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 | 6 | 2 | 2 | 1 |

- Figure v-5— Idle Mode Request sub-element format

The Element ID field is equal to the Idle Mode Response value defined in Table 26.

The value of the Length field is set to 17.

The STA Address field contains the MAC address of the STA sending Idle Mode Request.

The Response Type is set to a number that identifies an Idle Mode Response. The Response Type for Idle Mode response is shown in Table v-3.

- Table v-3—Response Type Definitions

| Name | Response Type |
| --- | --- |
| Reserved | 0 |
| Enter Idle Mode | 1 |
| Update Idle Mode | 2 |
| Reserved | 3 – 255 |

The Response Status is set to a value in Table v-4, indicating the Paging Server’s response to the Idle Mode Request indicated by the Dialog Token.

- Table v-4—Response Status values

| Status Value | Description |
| --- | --- |
| 0 | Successful |
| 1 | Fail |
| 2 | Refused |
| 3 | Incapable |
| 4–255 | Reserved |

The STA Address field contains the MAC address of the STA sending Idle Mode Request.

The Paging Server ID field contains a 6-byte value that identifies the entity that provides the Paging Service.

The Paging Group ID field contains one byte value that identifies the Paging Group.

The Paging ID field is a value assigned by the Pagng Server to specify the indetifier of a STA for paging assigned. The Paging ID value is a unique value within the Paging Group. The value assigned as the Paging ID is in the range 1–2007.

The Keep-alive Timer field indicates how often a STA in Idle Mode sends an Idle Mode update frame. It is a one byte value of the number of  Paging Intervals.

## 7.4 Action frame format details

### 7.4.6 Wireless Network Management action details

Insert the following rows into Table v7 and change the reserved row accordingly:

Table v44—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 11 | Idle Mode Request |
| 12 | Idle Mode Response |
| 13-255 | Reserved |

Insert the following new clause after Clause 7.4.6.7 of 802.11v Draft 0.03 as indicated below:

#### 7.4.6.8 Idle Mode Request frame format

The Idle Mode Request frame uses the Action frame body format and is transmitted by a non-AP STA requesting to either enter or update Idle Mode. The format of the Idle Mode Request frame body is shown in Figure v-8.

|  | Category | Action | Dialog Token | Idle Mode Request Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure v-8— Idle Mode Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Request, as specified in Table v7 in 7.4.6.

The Dialog Token field is a nonzero value which identifies the Idle Mode Request/Response transaction. The dialog token is unique for each Idle Mode request frame sent to a given destination MAC address.

The Idle Mode Request Element field contains the Idle Mode elements, described in Clause 7.3.2.45.

#### 7.4.6.9 Idle Mode Response frame format

The Idle Mode Response frame uses the Action frame body format and is transmitted by a AP STA in response to the Idle Mode Request frame.  The format of the Idle Mode Request frame body is shown in Figure v-9.

|  | Category | Action | Dialog Token | Idle Mode Response Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure v-9— Idle Mode Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Response, as specified in Table v7 in 7.4.6.

The Dialog Token field is set to the nonzero value received in the Idle Mode Request frame to identify the request/response transaction.

The Idle Mode Response Element field contains the Idle Mode Response element, described in Clause 7.3.2.46.

# 10. Layer management

#### 10.3 MLME SAP Interface

Add new clauses after clause 10.3.38 of TGv D0.03 as shown below:

    - Paging Inidication

This set of primitives supports the advertising of Paging information from a SME to peer SMEs.

      - MLME-PIREQUEST.request

        - Function

This primitive requests the transmission of Paging Indication to peer entities.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.request	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | paging indication element | paging indication element | A paging indication element describing the paged STAs information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the SME to request that a Beacon frame containing Paging Indication be sent to a peer entities to convey paging indication information at the next time the beacon is transmitted.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Beacon frame containing a paging indication element specified. This frame is then scheduled for transmission.

      - MLME-PIREQUEST.confirm

        - Function

This primitive reports the result of a paging indication.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Beacon frame containing Paging Indication. |

        - When generated

This primitive is generated by the MLME when the request to transmit a beacon frame containing Paging Indication completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-PIREQUEST.indication

        - Function

This primitive indicates that a Beacon frame containing Paging Indication has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.indication	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | paging indication element | paging indication element | A paging indication element describing the paged STAs information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the MLME when a valid Beacon frame containing Paging Indication is received.

        - Effect of receipt

On receipt of this primitive, the SME searchs paging indication element and checks whether the STA is being paged. If the STA is being paged, the STA wakes up and reassoicates with an AP. Otherwise, the STA stays in Idle mode.

    - Idle Mode request

This set of primitives supports the initiation, update and temination of Idle Mode operation between peer SMEs.

      - MLME-IMREQUEST.request

        - Function

This primitive requests the transmission of Idle Mode Request action frame to a peer entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Request
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| Idle Mode Request | idle mode request element | idle mode request element | A idle mode request element describing the STA Idle Mode Request information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Request frame be sent to a peer entity to convey idle mode request information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Request frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMREQUEST.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Request frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Request frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMREQUEST.indication

        - Function

This primitive indicates that an Idle Mode Request frame has been received requesting an Idle Mode Request transaction.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Request
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode request transaction. |
| Idle Mode Request | idle mode request element | idle mode request element | A idle mode request element describing the STA Idle Mode Request information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Request frame is received.

        - Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the Idle Mode Request transaction.

    - Idle Mode response

This set of primitives supports the exchange of Idle Mode parameters between peer SMEs.

      - MLME-IMRESPONSE.request

        - Function

This primitive requests the transmission of Idle Mode Response action frame to a peer entity, in response to a received Idle Mode Request frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Response
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | idle mode response element | idle mode response element | A idle mode response element describing the STA Idle Mode Response information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Response frame be sent to a peer entity to convey idle mode response information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Response frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMRESPONSE.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- IMRESPONSE.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMRESPONSE.indication

        - Function

This primitive indicates that an Idle Mode Response frame has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Response
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | idle mode response element | idle mode response element | A idle mode response element describing the STA Idle Mode Response information. See clause 7.3.2.43. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Response frame is received.

# 11. MAC sublayer management entity

#### 11.3 Relationships between services

Change list item a)2)vi as follows:

vi) Spectrum Management action, Wireless Network Management Presence Request, Presence Response, Idle Mode Request and Idle Mode Response Action Frames

Within an IBSS, action frames are class 1.

## 11.15 Wireless Network Management Procedures

Add new clauses after clause 11.15.6 of TGv D0.03 as shown below:

- 11.15.7 Idle Mode and Paging

Idle Mode is an extended power save mode for non-AP STAs. While in Idle Mode, a STA needs not listen for every Beacon frame, may traverse multiple BSSs in a Paging Domain and still be reachable within Paging Domain.

An ESS may comprise one or more Paging Domains.  Paging Services within a Paging Domain are managed by a Paging Server. The Paging Server maintains paging service information for STAs in Idle Mode within a Paging Domain. Paging service information includes the STA MAC Address, Paging Group ID, Paging Domain ID, Paging Server ID, and Home AP. Paging Server can be in the AP (distributed) or in a separate network entity (centralized).

The Paging Group is an identifier for a set of APs. A STA in a Paging Group moves among BSSs in a Paging Group and stays in Idle Mode..  A Paging Indication frame is sent by the APs in a Paging Group, not an all APs in an entire Paging Domain or ESS.

Idle Mode and Paging operation comprises the following procedures:

— Paging Capability Discovery and  Paging Service Information Advertisement

— Enter Idle Mode

— Update Idle Mode (Keep-alive and Paging Group ID update) if needed

— Paging

— Exit Idle Mode

Figure v-xxx below show the typical idle mode and paging use.

Figure v-xxx A Typical Idle Mode and Paging Procedure

- 11.15.7.1 Paging Capability Discovery

A STA discovers the paging capability via the Wireless Network Management Capability and the Paging Service element included in the Beacon and Probe Response Frames. When the “Paging” bit of the Wireless Network Management Capabilities Information Element is set to ‘1’, the Paging capability is supported.

- 11.15.7.2 Paging Service Information Advertisement

APs in a Paging Domain supporting the Paging Service shall include the Paging Service Information Element defined in  7.5.3 in beacon frames and in Probe Response frames.  The Paging Domain, Paging Server ID, and Paging Group ID in the Paging Service Information are used by non-AP STAs in the Idle Mode Request frame.  The STA uses the Paging Interval and DPIM Count to determine how often a Paging Indication is broadcast and when the next Paging Indication will be sent.

- 11.15.7.3 Enter Idle Mode

A non-AP STA signals its intent to enter Idle Mode by sending an Idle Mode Request frame to the Home AP with a Request-Type = “Enter Idle Mode” (0x01), STA address, Paging Domain ID received in Paging Service Information Advertisement, Paging Service ID received in Paging Service Information Advertisement, and Paging Group ID received in Paging Service Information Advertisement.  Upon receiving an Enter Idle Mode Request from the non-AP STA, the AP forwards the Enter Idle Mode Request to the Paging Server specified in the Idle Mode Request.

Upon receipt of the Enter Idle Mode Request, the Paging Server responds the Home AP an Enter Idle Mode Response with an unique Paging ID within the Paging Group, a Keep-Alive Timer if the result of Idle Mode Request is successful.  If the value of Response Status is not “Successful”, the Paging ID shall be set to 0, and the Keep-Alive Timer shall be set to 0 in the Idle Mode Response. The Paging Server creates, and maintains paging service information for the Enter Idle Mode requesting STA. The Home AP forwards the Idle Mode Response with a with a Response-Type = “Enter Idle Mode” (0x01) to the STA.

If the STA does not receive the Enter Idle Mode Response within 180 seconds (“T180”) after it sends the Enter Idle Mode Request, the STA shall retransmit the Enter Idle Mode Request as long as the retry count is non-zero.

If a successful Idle Mode Response is received, the Home AP shall notify the Paging Server to page the STA if the Home AP receives any directed frame that belongs to the non-AP STA in Idle Mode.

- 11.15.7.4 Idle Mode Update

The Idle Mode Update provides the Paging Server to with status update for STAs in Idle Mode. There are two types of Idle Mode Update: Paging Group Update and Keep-Alive Update.

- 11.15.7.4.1 Paging Group Update

A non-AP STA in Idle mode shall perform Idle Mode Update when the STA detects that the STA has moved out of its Paging Group (e.g., none of the APs in its Paging Group is reachable).   The STA shall send an Idle Mode Request message with Message-Type = “Update Idle Mode” (0x02) to an AP in the new Paging Group, within the same Paging Domain, including the Paging Domain ID and new Paging Group ID as received via Paging Service Information Advertisement.

Upon receiving an Update Idle Mode Request from the non-AP STA, the AP forwards the Update Idle Mode Request to the Paging Server specified in the Idle Mode Request. Upon receipt of  Idle Mode Update information from the STA, the Paging Server shall update the STA information, assign an unique Paging ID within the Paging Group, and Keep-Alive Timer.

Upon receipt of response from the Paging Server, the AP responds to the non-AP STA with an Idle Mode Response with an unique Paging ID within the Paging Group, a Keep-Alive Timer if the result of Idle Mode Request is successful.  If the value of Response Status is not “Successful”, the Paging ID shall be set to 0, and the Keep-Alive Timer shall be set to 0.

- 11.15.7.4.2 Keep-Alive Update

Keep-Alive Update is used by the STA in Idle mode to notify the Paging Server that the STA is still in the Paging Domain and is still using the paging service. The Paging Server may decide not to maintain the paging service for a STA if one or more Keep-alive updates have been missed.

The STA shall send an Idle Mode Request frame with a Request-Type = “Update Idle Mode” (0x02) to its Paging Server upon  Keep-Alive Timer expiration. When the Paging Server receives an Idle Mode Request frame with a Request Type of “Update Idle Mode” from the STA, the Paging Server shall re-set its Keep-Alive Timer or set a new Keep-Alive timer.

- 11.15.7.5 Paging

Paging is the procedure used by an AP to notify the STA in Idle Mode that directed frames are buffered for delivery.

- 11.15.7.5.1 AP operation for sending Paging Indication

When directed frames for a STA in Idle Mode arrive STA’s Home-AP, the Home-AP shall notify the Paging Server to trigger a Paging Operation to the STA. The Paging Server shall notify all APs in the Paging Group to page the STA. The AP that supports Paging Service shall send a Beacon containing a Paging Indication element when the DPIM count field in the Paging Server element is set to 0.  The AP shall only include Paging Indication element in the Beacon while DPIM count field with a value of 0.

The Paging Indication element shall identify the STAs in the Idle Mode for which STA is paged. This information is

coded in a partial virtual bitmap, as described in 7.3.2.44. The STA in Idle mode is assigned a Paging ID by the Paging Server as part of the entering Idle Mode process. The AP shall identify those STAs for which it needs to wake up to receive the traffic by setting bits in the Paging Indication partial virtual bitmap that correspond to the appropriate Paging ID’s.

Upon receipt of a Probe Request frame, an AP that supports the Paging Service shall include the Paging Indication element in its Probe Response frame.

- 11.15.7.5.2  STA operation for receiving Paging Indication

A STA in Idle mode shall wake up prior to the Paging Interval and perform time sychonization with the received beacon frame.  When STA detects that DPIM count field is 0, the STA shall check whether the bit corresponding to its Paging ID is set in the Paging Indication element.

A STA in Idle Mode may send a Probe Request containing a Paging Service element on supported channels in the Regulatory Class where the channel is valid for the current regulatory domain. The Paging Interval and DPIM Count shall be set to 0 in the Paging Service element contained in the Probe Request. An AP which supports the Paging Service shall respond with a Probe Response,  including the Paging Indication element.

When the STA detects that the bit corresponding to its Paging ID in the Paging Indication element is set, the STA shall exit Idle Mode by sending an Idle Mode Request message with a Request-Type = “Exit Idle Mode” (0x01) to the Home AP if the Home AP is still reachable. If the STA has moved out the coverage of its Home AP, the STA shall perform a reassociation or BSS transition operation to associate with an AP, and then send Exit Idle Mode Rquest to the current assoicated AP.

The mechanism used to transfer buffered frame from the Home AP to the AP with which the STA is currently associated is outside of the scope of this standard.

- 11.15.7.6 Exit Idle Mode

A STA may exit Idle Mode by sending an Idle Mode Request frame with a Request-Type = “Exit Idle Mode” (0x01) at anytime. A STA shall exit Idle Mode if:

— The STA has a frame to transmit

— The STA receives a valid Paging Indication that indicates the STA is being paged.

Upon receipt of an  Idle Mode Request frame with a Request Type of “Exit Idle Mode”, the AP shall forward the Idle Mode Request frame to the Paging Server  specified in Idle Mode Request frame. The Paging Server  terminates the paging service for the requesting STA and notifies the STA’s Home AP..

Insert A.4.14 into Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

Append this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFv | Is Wireless Network Management supported? |  | O | Yes, No, N/A |

Insert this new clause after clause A.4.13 of P802.11v D0.04:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME5 | Paging Service | 11.15.4 | CFv:O | Yes, No. N/A |
| RME5.1 | Idle Mode Request | 7.3.2.45, 7.4.8 | (CFv AND RME5):M | Yes, No, N/A |
| RME5.2 | Idle Mode Response | 7.3.2.46, 7.4.9 | (CFv AND RME5):M | Yes, No, N/A |
| RME5.3 | Paging Indication | 7.3.2.44, 7.4.8 | (CFv AND RME5):M | Yes, No, N/A |