IEEE P802.11
Wireless LANs

| Normative Text Proposal for WLAN Paging and Idle Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sunghyun Choi | Seoul National University | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-2-880-1753 | schoi@snu.ac.kr |
| Kyunghun Jang | Samsung Advanced Institute of Technology |  |  | khjang@samsung.com |
| Moo Ryong Jeong | DoCoMo USA Labs | 3240 Hillview Avenue, Palo Alto CA | 650-496-4761 | jeong@docomolabs-usa.com |
| Sunggeun Jin | ETRI | Kwanak P.O.Box 34, Seoul 151-600, Korea | +8228801755 | sgjin@mwnl.snu.ac.kr |
| Youngsoo Kim | Samsung Advanced Institute of Technology |  |  | kimyoungsoo@samsung.com |
| Mee-Suk LEE | Korea Telecom | 17, Woomyenon-dong Seocho-gu Seoul Korea | +8225266174 | kitlee@kt.co.kr |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA | 9254742290 | matta@trapezenetworks.com |
| R. R. Miller | AT&T | 180 Park Ave, Florham Park NJ | 973-236-6920 | rrm@att.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA | 408-754-1213 | Subbu@arubanetworks.com |
| Henry Ptasinski | Broadcom Corp. | 190 Mathilda Place Sunnyvale, CA, 94086 | 408-543-3316 | henryp@broadcom.com |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-264-7799 | Emily.H.Qi@intel.com |
| Dorothy Stanley | Aruba Networks | Warenville, IL | 630-363-1389 | dstanley@arubanetworks.com |
| James Tsai | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-5509 | James.tsai@intel.com |
| Jesse Walker | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-712-1849 | Jesse.Walker@intel.com |
| Fujio Watanabe | DoCoMo USA Labs | 240 Hillview Avenue, Palo Alto, CA | 650-496-4726 | watanabe@docomolabs-usa.com |

Abstract

This document contains normative text for a Paging and Idle Mode proposal to meet the TGv Power Saving objective, REQ2010, improving mobile client power efficiency and battery life. The text is aligned with P802.11-REVma/D9.0 and P802.11v D0.05.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

3. Definitions	4

4. Abbreviations and Acronyms	5

7. Frame formats	6

7.1 MAC frame formats	6

7.2.3.1 Beacon frame format	6

7.2.3.6 Reassociation Request frame format	6

7.2.3.7 Reassociation Response frame format	6

7.2.3.8 Probe Response frame format	7

7.2.3.9 Probe Response frame format	8

7.3.2 Information Elements	8

7.3.2.48 Paging Service element	9

7.3.2.49 Paging Indication element	10

7.3.2.50 Idle Mode Request element	11

7.3.2.51 Idle Mode Response element	11

7.4 Action frame format details	13

7.4.6 Wireless Network Management action details	13

7.4.6.13 Idle Mode Request frame format	13

7.4.6.14 Idle Mode Response frame format	13

8. Security	15

8.8 Paging Service Protection	15

8.8.1 Paging Service Protection Key Hierarchy	15

8.8.2 PSMK Key Procedures	15

8.8.2 PSTK Key Procedures	16

10. Layer management	17

10.3 MLME SAP Interface	17

10.3.7	Reassociate	17

10.3.7.1 MLME-REASSOCIATE.request	17

10.3.7.1.2 Semantics of the service primitive	17

10.3.7.2 MLME-REASSOCIATE.confirm	17

10.3.7.2.2 Semantics of the service primitive	17

10.3.7.3 MLME-REASSOCIATE.indication	18

10.3.7.3.2 Semantics of the service primitive	18

10.3.7.4 MLME-REASSOCIATE.response	18

10.3.7.4.2 Semantics of the service primitive	18

10.3.41 Paging Indication	19

10.3.41.1 MLME-PIREQUEST.request	19

10.3.41.1.1 Function	19

10.3.41.1.2 Semantics of the service primitive	19

10.3.41.1.3 When generated	19

10.3.41.1.4 Effect of receipt	19

10.3.41.2 MLME-PIREQUEST.confirm	20

10.3.41.2.1 Function	20

10.3.41.2.2 Semantics of the service primitive	20

10.3.41.2.3 When generated	20

10.3.41.2.4 Effect of receipt	20

10.3.41.3 MLME-PIREQUEST.indication	20

10.3.41.3.1 Function	20

10.3.41.3.2 Semantics of the service primitive	20

10.3.41.3.3 When generated	21

10.3.41.3.4 Effect of receipt	21

10.3.42 Idle Mode request	21

10.3.42.1 MLME-IMREQUEST.request	21

10.3.42.1.1 Function	21

10.3.42.1.2 Semantics of the service primitive	21

10.3.42.1.3 When generated	22

10.3.42.1.4 Effect of receipt	22

10.3.42.2 MLME-IMREQUEST.confirm	22

10.3.42.2.1 Function	22

10.3.42.2.2 Semantics of the service primitive	22

10.3.42.2.3 When generated	23

10.3.42.2.4 Effect of receipt	23

10.3.42.3 MLME-IMREQUEST.indication	23

10.3.42.3.1 Function	23

10.3.42.3.2 Semantics of the service primitive	23

10.3.42.3.3 When generated	24

10.3.42.3.4 Effect of receipt	24

10.3.43 Idle Mode response	24

10.3.43.1 MLME-IMRESPONSE.request	24

10.3.43.1.1 Function	24

10.3.43.1.2 Semantics of the service primitive	24

10.3.43.1.3 When generated	25

10.3.43.1.4 Effect of receipt	25

10.3.43.2 MLME-IMRESPONSE.confirm	25

10.3.43.2.1 Function	25

10.3.43.2.2 Semantics of the service primitive	25

10.3.43.2.3 When generated	26

10.3.43.2.4 Effect of receipt	26

10.3.43.3 MLME-IMRESPONSE.indication	26

10.3.43.3.1 Function	26

10.3.43.3.2 Semantics of the service primitive	26

10.3.43.3.3 When generated	27

11. MAC sublayer management entity	28

11.3 Relationships between services	28

11.15 Wireless Network Management Procedures	28

11.15.8 Idle Mode and Paging	28

11.15.8.1 Paging Service Capability Discovery	29

11.15.8.2 Paging Service Advertisement	29

11.15.8.3 Enter Idle Mode	30

11.15.8.4 Idle Mode Update	30

11.15.8.5 Paging	30

11.15.8.5.1 AP Paging operation	30

11.15.8.5.2  STA operation for receiving Paging Indication	31

11.15.8.6 Exit Idle Mode	31

Annex A	32

A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition	32

A.4.3 IUT configuration	32

A.4.14 Wireless Network Management extensions	32

Annex D	32

# 3. Definitions

Insert the following new definitions:

3.v.3 Idle Mode: An extended power save mode for non-AP STAs. While in Idle Mode, a STA does not listen for every Beacon frame, may traverse multiple BSSs in an ESS without maintaining the associated state with APs, and still be reachable from the ESS.

3.v.4 Home AP: The AP to which a STA is associated when it enters Idle Mode. The Home AP maintains state for the Idle Mode STA.

3.v.5 Paging Group: A group of APs in which a STA can be paged. A set of Paging Groups constitute a Mobility Domain.

3.v.6 Paging Group Identifier: A one octet value that identifies a Paging Group.

3.v.7 Paging Server: An entity that provides the paging service for STAs. The Paging Server maintains paging service information for STAs in Idle Mode. A Paging Server serves one or more Paging Groups.

3.v.8 Paging Server Identifier: A 48-bit value that identifies a Paging Server.

3.v.9 Paging Service Pairwise Master Key: A key used to derive the Paging Service Temporal Key

3.v.10 Paging Service Temporal Key: A key used to provide integrity and source authenticity protection for Idle Mode Request and Response frames.

# 4. Abbreviations and Acronyms

Insert the following new acronym in alphabetical order:

DPIM		Delivery Paging Indication Message

PSMK		Paging Service Pairwise Master Key

PSTK 		Paging Service Temporal key

# 7. Frame formats

## 7.1 MAC frame formats

#### 7.2.3.1 Beacon frame format

Insert new element in Table 8 as indicated below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 31 | Paging Service | The Paging Service element is present only within Beacon frames generated by APs when dot11PagingServiceEnabled is true. |
| 32 | Paging Indication | The Paging Indication element is present only within Beacon frames generated by APs when dot11PagingServiceEnabled is true and the DPIM Count field in the Paging Service element is 0. |

#### 7.2.3.6 Reassociation Request frame format

Insert a new row into table 12 as shown below:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 15 | Idle Mode Request | Idle Mode Request may be present if dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

| Table 13—Reassociation Response frame body Order | Information | Notes |
| --- | --- | --- |
| 11 | Idle Mode Response | Idle Mode Response may be present if dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1 and Idle Mode Request is present in corresponding Reassociation Request frame. |

#### 7.2.3.8 Probe Response frame format

Insert a new row into table 14 as follows:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Paging Service | The Paging Service element is present only within Probe Request frames generated by non-AP STAs when dot11PagingServiceEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Insert a new row into table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 28 | Paging Service | The Paging Service element is present only within Probe Response frames generated by APs when dot11PagingServiceEnabled is true. |
| 29 | Paging Indication | The Paging Indication element is present only within Probe Response frames generated by APs when dot11PagingServiceEnabled is true. |

### 7.3.2 Information Elements

Insert new rows into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Paging Service | x+14 |
| Paging Indication | x+15 |
| Idle Mode Request | x+16 |
| Idle Mode Response | x+17 |
| Paging Protection | x+18 |
| Reserved | x+19, 220 |

Change Clause 7.3.2.35 as shown below:

7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v2.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

- Figure v2 —Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure 3.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B6B7 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | FBMS | Proxy ARP Service | Paging Service | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 119 |  |

- Figure v3 —Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in 11.15.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in 11.15.3. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.1.1. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in 11.15.4. The Presence bit set to 0 indicates that the STA does not support this service.

- The FBMS bit set to 1 indicates the STA supports FBMS as described in section 11.2.1. The FBMS bit set to 0 indicates the STA does not support FBMS.

- The Proxy ARP Service bit set to 1 indicates the AP is providing proxy ARP service.  If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. The Proxy ARP Service bit set to 0 indicates the AP is not providing proxy ARP service for any associated client.

- The Paging bit set to 1 indicates that the STA supports Paging Service and Idle mode operation as described in 11.15.8. The Paging bit set to 0 indicates that the STA does not support this service.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

Add new clauses after Clause 7.3.2.47 as shown below:

### 7.3.2.48 Paging Service element

The Paging Service information element contains paging service and paging indication delivery information. The format of the Paging Service Information element is shown in Figure v-1.

|  | Element ID | Length | Paging Server ID | Paging Group ID | Paging Interval | DPIM Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 | 1 | 1 |

- Figure v-1—Paging Service element format

The Element ID field is equal to the Paging Service information element value in Table 26.

The value of the Length field is set to 9.

The Paging Server ID field contains a 6 octet Paging Server ID.

The Paging Group ID field contains one octet value that identifies the Paging Group.

The Paging Interval field indicates the number of Beacon intervals between successive DPIMs. If the Paging Interval equals the Beacon interval, the Paging Interval field has the value 1. The Paging Interval field is a single octet. The Paging Interval is set to 0 when the Paging Service element is contained in the Probe Request frame. The Paging Interval value 0 is reserved when the Paging Service element is contained in the Beacon and Probe Response frame.

The DPIM Count field indicates how many beacons (including the current frame) appear before the next DPIM. A DPIM Count of 0 indicates that the current Beacon is a DPIM. The DPIM count field is a single octet. If the Paging Service information element is present in a Probe Response frame, the DPIM Count field is set to the value that appears in the next beacon. The DPIM Count is set to 0 in the Paging Service element contained in the Probe Request.

### 7.3.2.49 Paging Indication element

The Paging Indication information element contains a Page Bitmap Control field and Partial Virtual Bitmap field. The format of the Paging Indication element is shown in Figure v-2.

|  | Element ID | Length | Page Bitmap Control | Partial Virtual Bitmap |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1– 251 |

- Figure v-2— Paging Indication element format

The Element ID field is equal to the Paging Indication value defined in Table 26.

The Length field for this element indicates the length of the information field, which is constrained as described below.

The Page Bitmap Control field is a single octet. Bit 0 of the field contains the Page Indicator bit associated with Paging ID 0. This bit is set to 1 in Paging Indication elements when one or more STAs are being paged. The remaining 7 bits of the field form the Page Bitmap Offset.

The Partial Virtual Bitmap, maintained by the Paging Server, consists of 2008 bits, and is organized into 251 octets such that bit number N (0 ≤ N ≤ 2007) in the bitmap corresponds to bit number (N mod 8) in octet number N / 8where the low-order bit of each octet is bit number 0, and the high order bit is bit number 7.  Each bit in the partial virtual bitmap corresponds to a specific station in the Paging Group for which the Paging Server had received a paging frame at the time the paging indication element was created. Bit number N is set to 0 if there is no page for the station whose Paging ID is N. If the Paging Server has a paging notification for a station, bit N in the page-indication virtual bitmap is set to 1.

The Partial Virtual Bitmap field consists of octets numbered N1 through N2 of the paging indication virtual bitmap, where N1 is the largest even number such that bits numbered 1 through (N1 8) – 1 in the bitmap are all 0 and N2 is the smallest number such that bits numbered (N2 + 1) 8 through 2007 in the bitmap are all 0. In this case, the Page Bitmap Offset subfield value contains the number N1/2, and the Length field will be set to (N2 – N1)+4.

In the event that all bits other than bit 0 in the virtual bitmap are 0, the Partial Virtual Bitmap field is encoded as a single octet equal to 0, and the Page Bitmap Offset subfield is 0, and the Length field is 4.

### 7.3.2.50 Idle Mode Request element

The Idle Mode Request element is used by the non-AP STA to enter, exit or update Idle Mode operation. The format of the Idle Mode Request element is shown in Figure v-5.

|  | Element ID | Length | Request Type | STA Address | Paging Server ID | Paging Group ID |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 6 | 1 |

- Figure v-5— Idle Mode Request element format

The Element ID field is equal to the Idle Mode Request value defined in Table 26.

The value of the Length field is set to 14.

The Request Type is set to a number that identifies a Idle Mode request. The Request Types for Idle Mode requests are shown in Table v-2.

- Table v-2—Request Type Definitions

| Name | Request Type |
| --- | --- |
| Exit Idle Mode | 0 |
| Enter Idle Mode | 1 |
| Update Idle Mode | 2 |
| ACK Enter Idle Mode | 3 |
| Reserved | 4 – 255 |

The STA Address field contains the MAC address of the STA sending the Idle Mode Request element.

The Paging Server ID field contains a 6-octet value that identifies the entity that provides the paging service.

The Paging Group ID field contains one octet value that identifies the Paging Group.

The Idle Mode Request element is included in Idle Mode Request frame, as described in 7.4.6.13, and Reassociation Request frame, as described in 7.2.3.6.

### 7.3.2.51 Idle Mode Response element

The Idle Mode Response element contains the response to a STA’s Idle Mode operation request. The format of the Idle Mode Response sub-element is shown in Figure v-5.

|  | Element ID | Length | Response Status | STA Address | Paging Server ID | Paging Group ID | Paging ID | Keep-Alive Timer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 6 | 2 | 2 | 2 |

- Figure v-5— Idle Mode Request element format

The Element ID field is equal to the Idle Mode Response value defined in Table 26.

The value of the Length field is set to 19.

The STA Address field contains the STA address from  the received Idle Mode Request element.

The Response Status is set to a value in Table v-4, indicating the Paging Server’s response to the Idle Mode Request.

- Table v-4—Response Status values

| Status Value | Description |
| --- | --- |
| 0 | Successful |
| 1 | Failed |
| 2 | Refused - Reason Unspecified |
| 3 | Incapable - Reason Unspecified |
| 4 | Refused - The STA is already subscribed to FBMS |
| 5 | Incapable – Insufficient service capacity in the Home AP |
| 6 | Incapable – Insufficient service capacity in the Paging Group |
| 7–255 | Reserved |

The Paging Server ID field contains a 6-octet value that identifies the entity that provides the Paging Service.

The Paging Group ID field contains one octet value that identifies the Paging Group.

The Pageing ID field is a value assigned by the Paging Server to specify the indentifier of a STA for the paging service. The Paging ID value is a unique value within a Paging Group, and is in the range of 1–2007.

The Keep-Alive Timer field indicates how often a STA in Idle Mode sends an Idle Mode update frame. It is a two octet value of  Paging Intervals between successive Idle Mode Update frames sent by the STA.

The Idle Mode Response element is included in Idle Mode Response frame, as described in 7.4.6.14, and Reassociation Response frames, as described in 7.2.3.7.

7.3.2.52 Paging Protection element

The Paging Protection element is used to provide forgery protection for the Idle Mode Request and Idle Mode Response elements. The format of the Paging Protection element is shown in Figure v-6.

|  | Element ID | Length | Nonce | Sequence Number | MIC |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 16 | 6 | 16 |

- Figure v-6 Paging MIC sub-element format

The value of the Length of the Paging Protection element is 38 octets.

The Nonce field contatins a 16-octet random number. When the Paging Protection element is included in an Idle Mode Request element with a Request type of “1” (Enter Idle Mode)  or “3” (ACK Enter Idle Mode) the field contains the SPagingNonce generated by the non-AP STA. When the Paging Protection element is included in an Idle Mode Response element the field contains the PNonce generated by the Paging Server. When the Paging Protection element is included in an Idle Mode Request or Response frame with a Request type of “2” (Update Idle Mode), the Nonce value is set to 0.

The Sequence Number field contains a 6-byte value that enables replay protection.

The MIC field contains a 16-byte message integrity code calculated over all of the preceding information element fields in the Idle Mode Request or Idle Mode Response element. When the Paging Protection element is included in an Idle Mode Request element with a Request type of “1” (Enter Idle Mode) the field is set to 0.

## 7.4 Action frame format details

### 7.4.6 Wireless Network Management action details

Insert the following rows into Table v7 and change the reserved row accordingly:

Table v44—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 13 | Idle Mode Request |
| 14 | Idle Mode Response |
| 15-255 | Reserved |

Insert the following new clause after Clause 7.4.6.12 as indicated below:

#### 7.4.6.13 Idle Mode Request frame format

The Idle Mode Request frame uses the Action frame body format and is transmitted by a non-AP STA to enter, exit or update Idle Mode operation. The format of the Idle Mode Request frame body is shown in Figure v-8.

|  | Category | Action | Dialog Token | Idle Mode Request Element | Paging Protection Element (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 16 | 40 |

Figure v-8— Idle Mode Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Request, as specified in Table v7 in 7.4.6.

The Dialog Token field is a nonzero value which identifies the Idle Mode Request/Response transaction. The dialog token is unique for each Idle Mode request frame sent to a given destination MAC address.

The Idle Mode Request Element field contains the Idle Mode elements, described in 7.3.2.50.

The Paging Protection Element field contains the Paging Protection element, described in 7.3.2.52. This optional field is included when dot11PagingProtectionEnabled is set to TRUE.

#### 7.4.6.14 Idle Mode Response frame format

The Idle Mode Response frame uses the Action frame body format and is transmitted by a AP STA in response to the Idle Mode Request frame with a Request type of “1” (Enter Idle Mode).  The format of the Idle Mode Request frame body is shown in Figure v-9.

|  | Category | Action | Dialog Token | Idle Mode Response Element | Paging Protection Element (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 21 | 30 |

Figure v-9— Idle Mode Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Idle Mode Response, as specified in Table v7 in 7.4.6.

The Dialog Token field is set to the nonzero value received in the Idle Mode Request frame to identify the request/response transaction.

The Idle Mode Response Element field contains the Idle Mode Response element, described in 7.3.2.51.

The Paging Protection Element field contains the Paging Protection element, described in 7.3.2.52. This optional field is included when dot11PagingProtectionEnabled is set to TRUE.

# 8. Security

Insert a new Clause after 8.7  as follows:

### 8.8 Paging Service Protection

When RSNA is enabled, the Idle Mode Request and Idle Mode Response frames may be protected using RSNA management frame protection.  In addition, Paging Service Protection may also be enabled. When Paging Service Protection is enabled, the Idle Mode Request and Idle Mode Response frames shall have integrity and source authenticity protection using  the Paging Service Protection pairwise keys known to the non-AP STA and the Paging Server.

The Paging Indication element included in the beacon frame is not protected.

### 8.8.1 Paging Service Protection Key Hierarchy

The Paging Service Protection key hierarchy consists of the:

- Paging Service Pairwise Master Key (PSMK), a key used to derive the Paging Service Temporal Key (PSTK), and the

- Paging Service Temporal Key (PSTK), a key to used provide integrity and source authenticity protection for Paging Service Idle Mode Request and Response frames.

The PSMK and PSTK are derived from the PMK-R0 key as follows:

PSMK = KDF-256(PMK-R0, “Paging Service” || PMKR0Name ||STA MAC address || Paging Server ID )

Where

- KDF-256 is the KDF function as defined in 8.5A.3 used to generate a key of length 256 bits.

- PMK-R0 is the first level of the FT Key Hierarchy.

- “Paging Service” is 0x506167696e672053657276696365

- PMKR0Name is defined in 8.5A.4.

- STA MAC address is the MAC address of the STA.

- Paging Server ID is the 6 octet Paging Server ID.

The PSTK is constructed with the random values SPagingNonce and PNonce exchanged in the initial Idle Mode Request/Response exchange, and is derived from PSMK:

PSTK = KDF-128(PSMK, “Paging Service Idle Mode Key” || SPagingNonce  || PNonce)

Where

- KDF-128 is the KDF function as defined in 8.5A.3 used to generate a key of length 128 bits.

- PSMK is the 256 bit Paging Service Pairwise Master Key as defined above.

- “Paging Service Idle Mode Key” is 0x506167696e6720536572766963652049646c65204d6f6465204b6579.

- SPagingNonce is the random value contributed by the STA.

- PNonce is the random value contributed by the Paging Server.

### 8.8.2 PSMK Key Procedures

The PSMK shall be securely delivered from the R0KH to the Paging Server.

### 8.8.2 PSTK Key Procedures

The PSTK is used to protect the contents of the Idle Mode Request element with Request Types “0” (Exit Idle Mode), “2”, (Update Idle Mode), and “3”, (ACK Enter Idle Mode)  and contents of the Idle Mode Response element. The following procedures are used to obtain and use the PSTK:

- The non-AP STA generates a random number SPagingNonce, creates the Paging Protection element and Idle Mode Request element  for the Idle Mode Request with a Request Type of “1” (Enter Idle Mode), and sends the Idle Mode Request action frame to the Home AP. The Home AP forwards the Idle Mode Request and Paging Protection element fields to the Paging Server.

- In response to receiving the Paging Protection element in an Idle Mode Request element, the Paging Server generates a random number PNonce, creates a Paging Protection element and sends the Idle Mode Response element and the Paging Protection element to the Home AP. The Home AP sends an Idle Mode Response action frame,  including the received Idle Mode Response and Paging Protection element to the requesting STA. The protocol between the Home AP and the Paging Server is outside the scope of this document.

- Upon receipt of the Paging Protection element in the Idle Mode Response frame, the non-AP STA sends an Idle Mode Request action frame with a Request Type of “3” (ACK Enter Idle Mode) to the Home AP, The Idle Mode Request frame contains the Idle Mode Request Element and includes the SPagingNonce value in the Paging Protection element. The Home AP forwards the Idle Mode Request and Paging Protection element fields to the Paging Server.

- Once the non-AP STA and the Paging Server exchange SPagingNonce and PNonce, the non-AP STA and the Paging Server derive and install the PSTK to use with subsequent Idle Mode Request element with Request Types “0” (Exit Idle Mode), and “2”, (Update Idle Mode).

- If the first Idle Mode Response from the Paging Server does not include the PNonce, there is no MIC or Paging Service Protection for subsequent Idle Mode Request and Idle Mode Response elements.

- Use of  Paging Service Protection is determined by local policy.

- The Paging Protection MIC shall be calculated using the PSTK and the AES-128-CMAC algorithm when either the pairwise or group cipher is AES-CCMP. The output of the AES-128-CMAC algorithm shall be 128 bits.

- The Paging Protection MIC shall be calculated using the PSTK and the HMAC-MD5 algorithm when neither the pairwise nor group cipher is AES-CCMP. The output of the AES-128-CMAC algorithm shall be 128 bits.

# 10. Layer management

#### 10.3 MLME SAP Interface

    - Reassociate

- 10.3.7.1 MLME-REASSOCIATE.request

- 10.3.7.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
WirelessManagementCapabilities,
FBMSRequest,
SupportedRegulatoryClasses,
IdleModeRequest)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| IdleModeRequest | As defined in Idle Mode Request element | As defined in Idle Mode Request element | Specifies the proposed service parameters for the Idle Mode Request. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1. |

- 10.3.7.2 MLME-REASSOCIATE.confirm

- 10.3.7.2.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-REASSOCIATE.confirm	(
Resultcode,
CapabilityInformation,
AssoicationID,
SupportedRates,
EDCAParameterSet,
WirelessManagementCapabilities
FBMSResponse, 
SupportedRegulatoryClasses,
IdleModeResponse)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| IdleModeResponse | As defined in Idle Mode Response element | As defined in Idle Mode Response element | Specifies the proposed service parameters for the Idle Mode Response. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1 and Idle Mode Request is present in corresponding Reassociation Request frame. |

- 10.3.7.3 MLME-REASSOCIATE.indication

- 10.3.7.3.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAddress,
CapabilityInformation,
ListenInterval,
SSID,
SupportedRates,
RSN,
QoSCapability,
WirelessManagementCapabilities
FBMSRequest,
SupportedRegulatoryClasses,
IdleModeRequest)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| IdleModeRequest | As defined in Idle Mode Request element | As defined in Idle Mode Request element | Specifies the proposed service parameters for the Idle Mode Request. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1. |

- 10.3.7.4 MLME-REASSOCIATE.response

- 10.3.7.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
WirelessManagementCapabilities
FBMSResponse, 
SupportedRegulatoryClasses,
IdleModeResponse)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| IdleModeResponse | As defined in Idle Mode Response element | As defined in Idle Mode Response element | Specifies the proposed service parameters for the Idle Mode Response. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and Paging bit in the Wireless Network Management Capability is set to 1 and Idle Mode Request is present in corresponding Reassociation Request frame. |

Add new clauses after clause 10.3.40 as shown below:

    - Paging Indication

This set of primitives supports the advertising of Paging information from an SME to peer SMEs.

      - MLME-PIREQUEST.request

        - Function

This primitive requests the transmission of Paging Indication to peer entities.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.request	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | Paging Indication element | Paging Indication element | A Paging Indication element describing the paged STAs.. See clause 7.3.2.48. |

        - When generated

This primitive is generated by the SME to request that a Beacon frame containing Paging Indication be sent to a peer entities to convey paging indication information at the next time the beacon is transmitted.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Beacon frame containing a paging indication element specified. This frame is then scheduled for transmission.

      - MLME-PIREQUEST.confirm

        - Function

This primitive reports the result of a paging indication.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Beacon frame containing Paging Indication. |

        - When generated

This primitive is generated by the MLME when the request to transmit a beacon frame containing Paging Indication completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-PIREQUEST.indication

        - Function

This primitive indicates that a Beacon frame containing Paging Indication has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-PIREQUEST.indication	(
Paging Indication Element
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Paging Indication Element | Paging Indication Element | Paging Indication element | A paging indication element describing the paged STAs information. See clause 7.3.2.48. |

        - When generated

This primitive is generated by the MLME when a valid Beacon frame containing Paging Indication is received.

        - Effect of receipt

On receipt of this primitive, the SME searchs paging indication element and checks whether the STA is being paged. If the STA is being paged, the STA wakes up and reassoicates with an AP. Otherwise, the STA stays in Idle mode.

    - Idle Mode request

This set of primitives supports the initiation, update and temination of Idle Mode operation between peer SMEs.

      - MLME-IMREQUEST.request

        - Function

This primitive requests the transmission of Idle Mode Request action frame to a peer entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Request,
Paging Protection
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| Idle Mode Request | Idle Mode Request element | Idle Mode Request element | A Idle Mode Request element describing the STA Idle Mode Request information. See 7.3.2.50. |
| Paging Protection | Paging Protection Element | Paging Protection element | An optional Paging Protection element providing information for protection of the contents of the Idle Mode Request element, see 7.3.2.52. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Request frame be sent to a peer entity to convey idle mode request information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Request frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMREQUEST.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode Request transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Request frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Request frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMREQUEST.indication

        - Function

This primitive indicates that an Idle Mode Request frame has been received requesting an Idle Mode Request transaction.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Request,
Paging Protection
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode request transaction. |
| Idle Mode Request | Idle Mode Request element | Idle Mode Request element | A Idle Mode Request element describing the STA Idle Mode Request information. See 7.3.2.50. |
| Paging Protection | Paging Protection Element | Paging Protection element | An optional Paging Protection element providing information for protection of the contents of the Idle Mode Request element, see 7.3.2.52. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Request frame is received.

        - Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the Idle Mode Request transaction.

    - Idle Mode response

This set of primitives supports the exchange of Idle Mode parameters between peer SMEs.

      - MLME-IMRESPONSE.request

        - Function

This primitive requests the transmission of Idle Mode Response action frame to a peer entity, in response to a received Idle Mode Request frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.request	(
Peer MAC Address,
Dialog Token,
Idle Mode Response,
Paging Protection
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | Idle Mode Response element | Idle Mode Response element | A Idle Mode Response element describing the STA Idle Mode Response information. See 7.3.2.51. |
| Paging Protection | Paging Protection Element | Paging Protection element | An optional Paging Protection element providing information for protection of the contents of the Idle Mode Response element, see 7.3.2.52. |

        - When generated

This primitive is generated by the SME to request that an Idle Mode Response frame be sent to a peer entity to convey idle mode response information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Idle Mode Response frame containing a paging parameter element specified. This frame is then scheduled for transmission.

      - MLME-IMRESPONSE.confirm

        - Function

This primitive reports the result of an idle mode request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME- IMRESPONSE.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Idle Mode Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit an Idle Mode Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-IMRESPONSE.indication

        - Function

This primitive indicates that an Idle Mode Response frame has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-IMRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Idle Mode Response,
Paging Protection
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Idle Mode Response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Idle Mode transaction. |
| Idle Mode Response | Idle Mode Response element | Idle Mode Response element | A Idle Mode Response element describing the STA Idle Mode Response information. See clause 7.3.2.51. |
| Paging Protection | Paging Protection Element | Paging Protection element | An optional Paging Protection element providing information for protection of the contents of the Idle Mode Request element, see 7.3.2.52. |

        - When generated

This primitive is generated by the MLME when a valid Idle Mode Response frame is received.

# 11. MAC sublayer management entity

#### 11.3 Relationships between services

Change list item a)2)vi as follows:

vi) Spectrum Management action, Wireless Network Management Presence Request, Presence Response, Idle Mode Request and Idle Mode Response Action Frames

Within an IBSS, action frames are class 1.

## 11.15 Wireless Network Management Procedures

Add new clauses after clause 11.15.7 of TGv D0.05 as shown below:

- 11.15.8 Idle Mode and Paging

Idle Mode is an extended power save mode for non-AP STAs. While in Idle Mode, a STA needs not to listen for every Beacon frame, may traverse multiple BSSs in a Paging Group and still be reachable within the Paging Group.

A Mobility Domain may comprise one or more Paging Groups.  Paging Services within a Paging Group are managed by a Paging Server. The Paging Server maintains paging service information for STAs in Idle Mode within a Paging Group. Paging service information includes the STA MAC Address, Paging Group ID, Paging Server ID, and Home AP.

The Paging Group is an identifier for a set of APs within a Mobility Domain. A STA in a Paging Group moves among BSSs in a Paging Group and stays in Idle Mode.  When a STA moves from one Paging Group to a new Paging Group, the STA shall perform a reassociation or BSS transition operation to associate with an AP in the new Paging Group.

Idle Mode and Paging operation comprises the following procedures:

— Paging Capability Discovery and  Paging Service Information Advertisement

— Enter Idle Mode

— Update Idle Mode operation if needed

— Paging

— Exit Idle Mode

Figure v-10 below shows an example of Idle Mode and Paging operation.

Figure v-10 Example Idle Mode and Paging Operation

- 11.15.8.1 Paging Service Capability Discovery

A STA discovers the paging capability via the Wireless Network Management Capability and the Paging Service element included in the Beacon and Probe Response frames. When the “Paging” bit of the Wireless Network Management Capabilities Information Element is set to ‘1’, Paging Service and Idle Mode operation are supported.

- 11.15.8.2 Paging Service Advertisement

APs in a Mobility Domain which supports the Paging Service shall include the Paging Service Information Element defined in  7.3.2.48 in Beacon frames and in Probe Response frames.  The Paging Server ID and Paging Group ID in the Paging Service information element are included in the Idle Mode Request element by non-AP STAs.  The STA uses the Paging Interval and DPIM Count to determine how often the Beacon frame contains the Paging Indication information element and when the next Paging Indication information element will be sent.

The Paging Service information element includes a partial virtual bitmap, used to indicate which STAs in the Paging Group are being paged. The Paging Server provides the partial virtual bitmap to the APs, by means which are outside the scope of this standard.

- 11.15.8.3 Enter Idle Mode

A non-AP STA signals its intent to enter Idle Mode by sending an Idle Mode Request frame to the Home AP or including the Idle Mode Request element in the Reassociation Request frame to the target AP with a Request Type of “Enter Idle Mode” (0x01), STA address, Paging Service ID, and Paging Group ID.  Upon receiving an Enter Idle Mode Request from the non-AP STA, the AP forwards the Idle Mode Request element to the Paging Server indicated in the received Idle Mode Request frame or Reassociation Request frame.

Upon receipt of the Idle Mode Request element with a Request Type of “Enter Idle Mode (0x01)”, the Paging Server responds to the Home AP with an Idle Mode Response element including a unique Paging ID within the Paging Group, and a Keep-Alive Timer if the result of Idle Mode Request is successful.  The Paging Server creates, and maintains paging service information for the Idle Mode STA. The Home AP forwards the Idle Mode Response element in an Idle Mode Response frame or  Reassociation Response frame to the STA. If the value of Response Status is not “Successful”, the Paging ID shall be set to 0, and the Keep-alive Timer shall be set to 0 in the Idle Mode Response element.

If the STA does not receive an Idle Mode Response within 180 seconds after it sends the Idle Mode Request frame, the STA shall retransmit the Idle Mode Request frame until the transmission is successful or until the relevant retry limit (dot11PagingServiceRequestRetryLimit) is reached.

If the Home AP receives an Idle Mode Response element with a Response Value of “0” (Successful) from the Paging Server, the Home AP shall request the Paging Server to page the STA when the Home AP receives directed frames for the non-AP STA in Idle Mode.

- 11.15.8.4 Idle Mode Update

The Idle Mode Update is used by the non-AP STA in Idle Mode to notify the Paging Server that the STA is still in the Paging Group and is still in Idle Mode. The Paging Server may decide not to maintain the Paging Service for a Idle Mode STA if the Paging Server does not receive one or more Idle Mode Update indications from the STA

An Idle Mode STA shall send an Idle Mode Request frame with a Request Type of “Update Idle Mode” (0x02) to an AP in the same Paging Group upon  Keep-Alive Timer expiration. The AP receiving an Update Idle Mode Request frame from the non-AP STA forwards the Idle Mode Request element to the Paging Server specified in the Idle Mode Request frame. Upon receipt of  the Idle Mode request element with a Request Type of “2” (Update Idle Mode) from the STA, the Paging Server shall re-set its Keep-Alive Timer.

- 11.15.8.5 Paging

Paging is the procedure used by the Paging Server and the APs to notify the STA in Idle Mode that directed frames are buffered for delivery. Broadcast and Multicast frames are not buffered for delivery to a STA in Idle Mode.

- 11.15.8.5.1 AP Paging operation

When directed frames for a STA in Idle Mode arrive at a STA’s Home-AP, the Home-AP shall request that the Paging Server page the STA.

The Paging Indication element shall identify the STAs in Idle Mode which are being paged. This information is

coded in a partial virtual bitmap, as described in 7.3.2.49. The STA in Idle Mode is assigned a Paging ID by the Paging Server upon entering Idle Mode. The Paging Server shall identify the STAs being paged by setting bits in the Paging Indication partial virtual bitmap that correspond to the appropriate Paging ID’s, and deliver the partial virtual bitmap to the APs.

The APs that support the Paging Service shall send a Beacon frame containing a Paging Indication information element when the DPIM count field in the Paging Service element is 0.  The AP shall only include the Paging Indication element in a Beacon in which the DPIM count field has a value of 0.

An AP that supports the Paging Service shall include the Paging Indication element in its Probe Response frames.

- 11.15.8.5.2  STA operation for receiving Paging Indication

A STA in Idle Mode shall wake up prior to the end of the Paging Interval and perform time synchronization with the received beacon frame.  When the STA detects that the DPIM count field is 0, the STA shall determine if the bit corresponding to its Paging ID is set in the Paging Indication element.

When the STA detects that the bit corresponding to its Paging ID in the Paging Indication element is set, the STA shall exit Idle Mode by sending an Idle Mode Request frame with a Request Type of “1” (Exit Idle Mode) to the Home AP, if the Home AP is still reachable. If the STA has moved out the coverage of its Home AP, the STA shall perform a reassociation or BSS transition operation to associate with an AP in its Paging Group, and then send Idle Mode Request to the AP with a Request Type of “1” (Exit Idle Mode).

A STA in Idle Mode may send a Probe Request frame containing a Paging Service element on supported channels in the Regulatory Class where the channel is valid for the current regulatory domain. The Paging Interval and DPIM Count shall be set to 0 in the Paging Service element contained in the Probe Request. An AP which supports the Paging Service shall respond with a Probe Response frame that includes the Paging Indication element.

The mechanism used to transfer buffered frames from the Home AP to the AP with which the STA is currently associated is outside of the scope of this standard.

- 11.15.8.6 Exit Idle Mode

A STA exits Idle Mode by sending an Idle Mode Request frame with a Request Type of “Exit Idle Mode” (0x01). A STA shall exit Idle Mode if the STA has a frame to transmit or if the STA receives a Paging Indication element indicating that the STA is being paged.

Upon receipt of an  Idle Mode Request frame with a Request Type of “Exit Idle Mode”, the AP shall forward the Idle Mode Request element to the Paging Server specified in Idle Mode Request frame. The Paging Server  terminates the paging service for the requesting STA and notifies the STA’s Home AP that the STA has exited Idle Mode, by means which are outside the scope of this standard.

Insert the following entries into A.4.14 in Annex A as follows:

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME10 | Paging Service | 11.15.7, 7.3.2.48 | CFv:O | Yes, No. N/A |
| RME10.1 | Idle Mode Request | 7.3.2.50, 7.4.6.13 | (CFv AND RME5):M | Yes, No, N/A |
| RME10.2 | Idle Mode Response | 7.3.2.51, 7.4.6.14 | (CFv AND RME5):M | Yes, No, N/A |
| RME10.3 | Paging Indication | 7.3.2.49, | (CFv AND RME5):M | Yes, No, N/A |
| RME10.4 | Paging Service Protection | 7.3.2.52, 8.8 | CFv:O | Yes, No, N/A |

Add the following MIB attributes in Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11ProtectedPagingEnabled TruthValue }

dot11PagingServiceEnabled TruthValue

dot11PagingServiceRequestRetryLimit INTERGER }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11ProtectedPagingEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is supports the Protected Paging Service.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }

dot11PagingServiceEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is supports the Wireless Network Management Paging Service.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }

dot11PagingServiceRequestRetryLimit OBJECT-TYPE

SYNTAX INTEGER (1..100)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the number of times the non-AP STA may retry an Idle Mode Request frame for which it does not receive an Idle Mode Response frame. The default value of this attribute is 0.”

::= { dot11StationConfigEntry TBD }