IEEE P802.11
Wireless LANs

| Updates to TGr Security Clauses |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | Research in Motion | 5090 Commerce Blvd, Mississauga, ON L4W 5W4, Canada | +1-905-629-4746 | mmontemurro@rim.com |

Abstract

This submission captures some of the results for the discussion on the TGr security architecture discussed during the IEEE 802.11r adhoc meeting in June 2006.

This document covers capturing the requirements for key holders as well as the key revocation process.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 3.98C as follows:

3.98C Pairwise Master Key R0 Key Holder Identifier (R0KH-ID): A 48 octet identifier that names the holder of the PMK-R0 Key.

Change 3.98F as follows:

3.98F Pairwise Master Key R1 Key Holder Identifier (R1KH-ID): A 6 octet identifier that names the holder of the PMK-R1 key.

Change the length of the “FTIE” in table 26 to “90 to 253”

Change the length of the R0KH-ID to 48 octets in figure 113M.

Change the length of the R1KH-ID to 6 octets in table 43C.

Add the following sentence at the end of the fourth paragraph in Clause 8.5A.1

When the key lifetime expires, each key holder must delete their respective PMK-R0, PMK-R1 or PTK SA.

Insert the following clause after Clause 8.5A.5 Key Hierarchy:

8.5A.6 Fast BSS-Transition Key Holders

8.5A.6.1 Key Holder requirements

The R0 and R1 key holders are responsible for the derivation of keys in the TGr key hierarchy.  For Fast-BSS Transition, the functions of the IEEE 802.1X Authenticator are distributed among the R0, and R1 key holders. Each key holder shall have a name that is communicated to the STA and other key holders which is bound into the key derivation. Each key holder name shall be mapped to a physical entity in the DS where it resides.

The R0KeyHolder shall meet the following requirements:

- The R0KeyHolder shall be co-resident with the NAS Client functionality of the IEEE 802.1X Authenticator.

- The R0KeyHolder identifier (R0KH-ID) shall be set to the NAS ID as defined in RFC 2865.

- The R0KeyHolder identity shall be advertised in the association response during FT Initial association, and in the FT-Response during FT base and FT reservation mechanisms.

- The R0KeyHolder and the Supplicant shall mutually derive the PMK-R0 based upon a successful derivation of the MSK during Initial Association.

- The R0KeyHolder will derive PMK-R1 keys based on the PMK-R0 SA.

- When the PMK-R0 lifetime expires, the R0KeyHolder shall delete the PMK-R0 SA and should revoke all PMK-R1s derived from the PMK-R0.

The R1KeyHolder shall meet the following requirements:

- The R1KeyHolder identity (R1KH-ID) shall be set to the MAC address of the physical entity that stores the PMK-R1 and uses it to generate the PTK. That same MAC address shall be used to advertise the PMK-R1 identity to the STA and the R0KeyHolder.

- The R1KeyHolder identity shall be advertised in the (re)association response during FT Initial Association, and in the FT-Response during FT base and FT reservation mechanisms.

- The R1KeyHolder and the Supplicant shall mutually derive the PTK as part of the FT base and FT reservation mechanisms, based on the PMK-R1 SA.

- The R1KeyHolder shall install the PTK in the MAC and signal the MAC to open the IEEE 802.1X controlled port upon successful derivation of the PTKSA.

- The R1KeyHolder shall provide the IEEE 802.1X Auththenticator function to derive and distribute the GTK to all connected STA’s.

- When the PMK-R1 lifetime expires, the R1KeyHolder shall delete the PMK-R1 SA and shall revoke all PTKs derived from the PMK-R1 using the MLME-DELETEKEYS primitive.

Rename Clause 8.5A.6 to a subclause labelled 8.5A.6.2 .

Updated the length of  dot11FTR0KeyHolderID on page 86, line 11, to 48 octets.

Updated the length of  dot11FTR1KeyHolderID on page 86, line 37, to 6 octets.