IEEE P802.11
Wireless LANs

| Identifying Cached PMK-R1s During Fast Transition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Frank Ciotti | Motorola | 7700 W. Parmer Ln, PL67, Austin, TX | 512-996-5753 | Frank.Ciotti@ motorola.com |
| Tim Wilson | Motorola | 1301 E. Algonquin Rd., Schaumburg IL | 847-576-5296 | Tim.wilson@ motorola.com |
| Steve Emeott | Motorola | 1301 E. Algonquin Rd., Schaumburg IL | 847-576-8268 | Steve.Emeott@ motorola.com |
| Tony Braskich | Motorola | 1301 E. Algonquin Rd., Schaumburg IL | 847-538-0760 | Tony.Braskich@ motorola.com |

Abstract

The removal of PMKR1Name from the FT Authentication Request message (Msg1) prevents the FT Target AP from being able to determine, after the receipt of Msg1, if a cached PMK-R1 that it may hold for a STA corresponds to the STA’s current PMK-R1, and if not, query the PMK-R0 Key Holder for a new PMK-R1 context.  This submission provides normative text to resolve this issue by supplying the PMKR0Name in Msg1, thus providing the Target AP to with enough information (when combined with other parameters) to derive PMKR1Name and compare it with its cached PMK-R1 key context.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

In Clause 8A.5.1 –

change the first paragraph:

Fast BSS Transition Request is used by the STA to initiate a Fast BSS Transition. When RSNA is enabled, the STA shall include the R0KH-ID in the FTIE. The target AP can use the R0KH-ID and the STA MAC address to ensure it has the corresponding PMK-R1 or initiate a request for the appropriate PMK-R1. Additionally, the STA shall also include the SNonce as its random contribution to ensure the derivation of a fresh PTK.

to:

Fast BSS Transition Request is used by the STA to initiate a Fast BSS Transition. When RSNA is enabled, the STA shall include the R0KH-ID and the SNonce in the FTIE, and the PMKR0Name in the RSNIE. The target AP can use the PMKR0Name to derive the PMKR1Name, and if the target AP does not have the key identified by PMKR1Name, it may attempt to retrieve that key from the R0KH identified by R0KH-ID. See 8.5A.6.  Tthe STA includes the SNonce as its random contribution to ensure the derivation of a fresh PTK.

change:

— PMKID count shall be set to 0 (i.e., there are no PMKID fields).

to:

— PMKID Count shall be set to 1.

— PMKID field shall contain the PMKR0Name.

In Clause 8A.5.2 –

change:

— PMKID count shall be set to 0 (i.e., there are no PMKID fields).

to:

— PMKID Count shall be set to 1.

— PMKID field shall be set to the value contained in the first message of this sequence.

In Figures 154B, 154C, 154F and 154G –

add PMKR0Name to the list of RSNIE  parameters in the FT Request and Response messages.