IEEE P802.11
Wireless LANs

| Response to Interpretation Request #1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bob O'Hara | Cisco Systems | 3625 Cisco Way San Jose, CA 95134 | +1 408 853 5513 | bob.ohara@cisco.com |

Abstract

Response to the interpretation request on the use of RTS/CTS with fragmentation.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Interpretation Number: 1-07/06 (RTS/CTS with fragmentation)

Topic: ambiguity of RTS/CTS usage description with fragmentation

Relevant Clauses: 9.2.5.2, 9.2.5.3, 9.2.5.6, 9.2.5.7, 9.2.8, 9.7

Classification: Unambiguous

Interpretation Request

Clause 9.2.5.6 states that: RTS-CTS mechanism precedes the first fragment only in case of transmission of multiple fragments by a source STA, where (RTS threshold < Fragmentation Threshold). It also says "no further RTS/CTS frames need to be generated after the RTS/CTS that began the frame exchange sequence even though subsequent fragments may be larger than dot11RTSThreshols". The section also discusses the worst-case scenario, where an acknowledgment of a fragment is lost and is never heard by the source station.

How does the frame transmission begin when there is an acknowledgment loss? Will there be an RTS-CTS exchange by the source STA again before retrying the lost fragment?

Clause 9.2.5.6 says that the RTS-CTS mechanism should only occur before the transmission of the first fragment; the Frame Exchange Sequence in clause 9.7 also depicts this behavior, i.e. {RTS-CTS-}[Frag - ACK -]Last - ACK

However, clause 9.2.5.5 states: "When a source STA transmits a fragment, it shall release the channel, then immediately monitor the channel for an acknowledgment.

If no acknowledgment is received, then should the source STA perform a random backoff and contend for the channel? Would the source STA be required to send a RTS at this point to reserve the NAV again after gaining access on the channel?

The standard is not clear from these clauses what the desired behavior should be.

Interpretation Response

The standard is unambiguous on this issue.  Clauses 9.2.5.2, 9.2.5.3, 9.2.5.7, and 9.2.8 describe the use of the backoff procedure in various situations, including that when an ACK is not received in response to an individually addressed data frame (fragment).  The description of fragmentation in the standard does not modify this behavior.  Therefore, the standard requires a backoff procedure to be performed after loss of an ACK frame.

Clause 9.2.6 describes the requirement to use RTS before the transmission of any MPDU longer than dot11RTSThrshold.  Clause 9.2.5.6 modifies this general requirement when transmitting a fragment burst.  RTS/CTS is required to precede the initial fragment in a fragment burst and is required not to be transmitted before any other fragments in that same fragment burst.  Clause 9.7 also shows RTS/CTS preceding the initial fragment of a fragment burst.

The standard is unambiguous on the topic of RTS/CTS usage with fragmentation.