IEEE P802.11
Wireless LANs

| LB83 Annex IJ normative text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-1-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr., MS SJ-10-5, San Jose, CA 95134-1706 | 408-527-0815 | petere@cisco.com |

Abstract

LB83 Annex I and J normative text with respect to 802.11 REV-ma D7.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex I

(informative)

# Regulatory classes

## I.1 External regulatory references

Insert a new row before the last Emissions limits set row of Table I.2 and change as shown:

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 6 ISM bands | FCC CFR47 [B8], Section 15.247 | Reserved | Reserved |
| 67-255 | Reserved | Reserved | Reserved |

Insert a new row before the last Behavior limits set row of Table I.3 and change as shown:

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 10 ISM bands | FCC CFR47 [B8], Section 15.247 | Reserved | Reserved |
| 1011-255 | Reserved | Reserved | Reserved |

Insert two new rows in Table I.6 as shown:

| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | Europe (EIRP) |
| --- | --- | --- |
| 2.400-2.4835 | 1 000 | 100 mW |
| 5.725-5.850 | 1 000 with antenna gain per FCC 47 CFR 15.247 (b)(4)(ii)(iii) | — |

# Annex J

# (normative)

# Country information element and regulatory classes

Change the title, change channel set in row 5, insert a new row after 5 and change the last row of Table J.1 as shown:

### Table J.1—Regulatory classes for 4.9 GHz and 5 GHz bands in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 20 | 149, 153, 157, 161, 165 | 1 000 | 4 | 1 |
| 6 | 2.407 | 25 | 1-11 | 1 000 | 4 | 9 |
| 57-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change the title, insert a new row and change the last row of Table J.2 as shown:

### Table J.2—Regulatory classes for 5 GHz bands in Europe

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 2.407 | 25 | 1-13 | 100 | 4 | 9 |
| 45-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change the title, insert two new rows and change the last row of Table J.3 as shown:

### Table J.3—Regulatory classesfor 4.9 GHz and 5 GHz bands in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (dBm) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 2.407 | 25 | 1-13 | 10 mW/MHz | 4 | 6, 9 |
| 31 | 2.414 | 25 | 14 | 10 mW/MHz | 4 | 6, 9 |
| 3032-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |