IEEE P802.11
Wireless LANs

| Network Selection – Normative text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Necati Canpolat (Editor) | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Vivek Gupta | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 712 1754 | vivek.g.gupta@intel.com |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Srinivas Sreemanthula | Nokia | 6000 Connection Dr, Irving, TX 75039 | +1972-894-4356 | srinivas.sreemanthula@nokia.com |
| Ronny Yongho Kim | LG Electronics, Inc. | 533 Hogye-ldong Dongan-gu Anyang-shi Kyongki-do Korea | +82 31 450 1879 | ronnykim@lge.com |
| Eleanor Hepworth | Siemens Roke Manor Research | Roke Manor, Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833146 | eleanor.hepworth@roke.co.uk |
| Sabine Demel | T-Mobile | Vienna, Austria |  | sabine.demel@t-mobile.at |
| Marian Rudolf | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6258 | marian.rudolf@interdigital.com |
| Farooq Bari | Cingular Wireless | 7277 164th Avenue N.E. Redmond, WA 98052 USA | +1 425 580 5526 | farooq.bari@cingular.com |
| Hong Cheng | Panasonic | BLK1022 TaiSeng Ave #06-3530 Singapore 534415 | +65 6550 5477 | hong.cheng@sg.panasonic.com |
| Zhonghui Yao, Liangyao Mo | Huawei | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-755-89650528 +86-137-51055382 | moly@huawei.com yaoth@huawei.com |

Abstract

This document provides a normative text proposal in support of requirement R11Nx (Network Selection Cluster Requirements) in 05/822r11.

- R11N1:”Define functionality by which a STA can determine whether its subscription to an SSPN would allow it to access a particular 802.11 AN before actually joining a BSS within that 802.11 AN. Proposals must describe their consideration of scalability.”

- R11N1: "Define functionality by which a STA can determine whether its subscription to an SSPN would allow it to access a particular 802.11 AN before actually joining a BSS within that 802.11 AN. Proposals must describe their consideration of scalability.”

- R11N2: “The mechanism described in requirement R10N1 must allow a STA that has multiple credentials with an SSPN to select the correct credentials when authenticating with a Local Network.”

- R11N3: “Define functionality to support authentication with multiple SSPNs through a single AP.”

- R11N4: “Define functionality by which a STA can determine which interworking services are available before joining a BSS.”

- R11N5: “Functionality shall be provided by which APs can advertise (before connection) the charges that will be made for use of the network if connection is authorized based on an SSPN subscription.”

- R11N6: “Functionality shall be provided by which during the connection process a STA can be informed of the actual charges to be applied to this session.”

- R11N7: “It should be possible to inform a STA about unbroadcasted SSIDs without causing the STA to probe for each preferred SSID.”

- R11N8: “Define functionality by which the STA is able to determine what online enrolment (also called online subscription) methods are supported by the local network.”

This text proposal is based on P802.11-REVma-D6.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

5.9 Generic Adversisement Services	5

5.9.1 Pre-association information services	5

7. Frame formats	6

7.2 Format of individual frame types	6

7.2.3 Management frames	6

7.2.3.1 Beacon frame format	6

7.2.3.4 Association Request frame format	6

7.2.3.5 Association Response frame format	6

7.2.3.6 Reassociation Request frame format	7

7.2.3.7 Reassociation Response frame format	7

7.2.3.9 Probe Response frame format	8

7.3 Management frame body components	8

7.3.1 Fixed fields	8

7.3.1.9 Status Code Field	8

7.3.1.18 GAS Query ID	8

7.3.11 Action field	9

7.3.2 Information Elements	10

7.3.2.35 Interworking Capability Information Element	10

7.3.2.36 GAS Capability Information element	11

7.3.2.37 GAS Request Information element	12

7.3.2.38 GAS  Response Information element	13

7.3.2.39 GAS Traffic Indication Map Information Element	13

7.3.2.41 GAS Comeback Delay	14

7.3.2.42 ESSID Information Element	15

7.4	Action frame format details	15

7.4.6	Interworking action details	15

7.4.6.1	GAS Initial Request Action frame format	15

7.4.6.2	GAS Initial Response Action frame format	16

7.4.6.3	GAS Comeback Request Action frame format	17

7.4.6.4	GAS Comeback Response Action frame format	17

10. Layer Management	19

10.3	MLME SAP interface	19

11. MLME	26

11.11 Interworking Procedures : Generic Advertisement Services	26

11.11.1.2 Multicast Mechanism	26

11.11.2 Unicast Mechanism	27

Annex A	28

Amendments to the 802.11 draft	29

A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition	29

A.4.3 IUT configuration	29

A.4.16 Interworking extensions	30

Annex D	30

5 General Description

## 5.9 Generic Adversisement Services

The purpose of Generic Advertisement Services (GAS) functionality is to enable the STA to identify the availability and information related to the desired network services located in the backend. The requirement for this functionality was originally identified by deployment issues associated with public access hotspots where it is currently impossible for users to determine whether they will be able to access network services based solely on information currently advertised by the network. There is a need non-AP STA to query for advanced advertisement services in state-1 that the access network supports beyond AP.

## 5.9.1 Pre-association information services

There are a number of reasons why providing this information pre-association is beneficial:

- The information is available earlier to support more informed decision about which AP to associate with – this prevents a less efficient style of operation where the STA has to associate with an AP before discovering the information and then decide whether to stay associated or not.

- It is possible to query multiple networks in parallel.

- The STA can discover information about APs that are not part of the same management group as the AP to which it is currently associated.  For example, supporting the selection of an AP belonging to a different hotspot with an appropriate roaming agreement in place.

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert a new row into table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | Generic Advertising Service Traffic Indication Map | Present in Beacon if any of the supported Advertisement Protocols are configured for multicast delivery. |
| 26 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingEnabled is true. |
| 27 | Generic Advertisement Service Capability | Advertisement Service Capability shall be present if dot11AdvertisementServiceEnabled is true. |
| 28 | ESSID | ESSID shall be present if dot11InterworkingEnabled is present |

#### 7.2.3.4 Association Request frame format

Insert a new row into table 10 as shown below:

Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingCapabilityEnabled is true. |

#### 7.2.3.5 Association Response frame format

Insert a new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingCapabilityEnabled is true. |

#### 7.2.3.6 Reassociation Request frame format

Insert a new row into table 12 as shown below:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 12 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingCapabilityEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | Interworking Capability | Interworking Capability shall be present if dot11InterworkingCapabilityEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Insert a new row before the last line into table 15 and modify the last row as shown below:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 24 | Interworking Capability | Interworking Capability shall be present if dot11InterwokingEnabled is true. |
| 25 | Generic Advertisement Service Capability | Advertisement Service Capability shall be present if dot11AdvertisementServiceEnabled is true. |
| 28 | ESSID | ESSID shall be present if dot11InterworkingEnabled is present |

## 7.3 Management frame body components

- 7.3.1 Fixed fields

#### 7.3.1.9 Status Code Field

Insert the following:

Table 23 ― Status Codes

| Reason Code | Meaning |
| --- | --- |
| 52 | No outstanding GAS request available |
| 53 | GAS Protocol(s) not supported |
| 54 | GAS Response not received from backend network |
| 55 | Reequested delivery method not supported for this GAS protocol. |
| 56 | Requested information not available in the backend adverstisement service |
| 56-65535 | Reserved |

Insert the following:

#### 7.3.1.18 GAS Query ID

GAS Query ID is returned by the AP in the GAS Initial Response frame to indicate that a backend query is being carried out for the non-AP STA. The non-AP STA shall use the GAS Query ID in the GAS Comeback Request to retrieve the response information after expiry of GAS Comeback delay in the unicast mechanism. The GAS Query ID is included in the GAS Comeback Reponse frames in both unicast and multicast mechanisms.

|  | GAS Query ID |
| --- | --- |
| Octets: | 1 |

Figure u1—GAS Query ID

- 7.3.11 Action field

Insert the following new row into table 24 and update the reserved value as shown:

Table 24—Category values

| Value | Name | See subclause |
| --- | --- | --- |
| x | Interworking | 7.4.6 |
| (x+1)–127 | Reserved | — |

NOTE— x will be defined.

### 7.3.2 Information Elements

Insert Element ID x into Table 26 and change the Reserved row accordingly:

Table 26 ― Element IDs

| Information Element | Element ID |
| --- | --- |
| Reserved | 51 |
| Interworking Capability | 52 |
| GAS Capability | 53 |
| GAS Request | 54 |
| GAS Response | 55 |
| GAS Traffic Indication Map | 56 |
| GAS Query ID | 57 |
| GAS Comeback Delay | 58 |
| ESSID | 59 |
| Reserved | 60-126 |

Insert the following new clauses after 7.3.2.34

#### 7.3.2.35 Interworking Capability Information Element

The Interworking Capability Information Element contains information about the interworking service capabilities of an STA as shown in 1.

|  | Element ID | Length | Interworking Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure u2—Interworking Capability Information element format

The Element ID field is equal to the Interworking Capability value in Table 26.

The Length field is the length of the Interworking Capabilities field. The Length field is 2.

The Interworking Capabilities field is a bit field indicating the advertised interworking capabilities of the STA. The Interworking Capabilities field is shown in Error: Reference source not found.

|  | B0 | B1 | B2 | B3 | B15 |
| --- | --- | --- | --- | --- | --- |
|  | QoS Map | Expedited Bandwidth Request | Emergency Services Only | Reserved |  |
| Bits: | 1 | 1 | 1 | 13 |  |

Figure u3—Interworking Capabilities field

- The QoS Map Service capability bit set to 1 indicates the AP supports QoS Map Service.  The QoS Map Service capability bit set to o indicates the AP does not support QoS Map Service.

- The Expedited Bandwidth Request Service capability bit set to 1 indicates the AP supports Expedited Bandwidth Request Service.  The Expedited Bandwidth Request Service capability bit set to 0 indicates the AP does not support Expedited Bandwidth Request Service.

- The ESO (Emergency Services Only on this SSID) capability set to 1 indicates higher layer emergency call services are reachable via this SSID.  The ESO capability set to 0 indicates other means are necessary to determine if higher layer emergency call services are reachable via this SSID.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of an Interworking Capability element shall be interpreted as the STA having no advertised Interworking Capabilities.

#### 7.3.2.36 GAS Capability Information element

The GAS Capability Information element contains information about the advertisement service capabilities of a STA as shown in 3.

|  | Element ID (53) | Length | GAS Protocol #1 | GAS Protocol #2 (Optional) | … | GAS Protocol #N (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | … | 1 |

Figure u4— GAS Capability Information element format

The Element ID field is equal to the GAS Capability value in Table 26.

The Length field the length of the GAS Capabilities field. The Length field is 2 plus the number of GAS Protocol ID fields.

The GAS Capability information element shall contain at least one GAS Protocol field.  The GAS Protocol field is defined in Figure u4.

|  | B0 | B5 | B6 | B7 | B8 | B15 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GAS Protocol ID |  | Multicast Delivery | Unicast Delivery | Reserved |  |
| Bits: | 6 |  | 1 | 1 | 8 |  |

Figure u5— GAS Protocol field

- GAS Protocol ID is a 6-bit field identifying the advertising protocol supported by the AP.  The enumerated protocol identifiers are defined in Table u1.

- Bit 6 is set to 1 if the AP supports multicast delivery as described in clause 9.x.  Bit 6 is set to 0 if the AP does not support multicast delivery for this GAS Protocol ID.

- Bit 7 is set to 1 if the AP supports unicast delivery as described in clause 9.y.  Bit 7 is set to 0 if the AP does not support unicast delivery for this GAS Protocol ID.

- Bit 8 through Bit 15 reserved and will be used as needed by the protocol.

The STA shall either support multicast delivery or unicast delivery.  Optionally, an STA may support both delivery methods.

A STA using the GAS Protocol field requests the desired delivery method by setting either Bit6 or Bit7 as appropriate.

The lack of a GAS Capability element shall be interpreted as the STA having no advertised GAS Capability.

MIH Information Service is a mechanism defined in IEEE 802.21 to support information retrieval from a information repository in the back end network. MIH Command and Event Services capability discovery is a mechanism defined in IEEE 802.21 to support discovering capabilities of command service and event service entities in the back end network.

Table u1—GAS Protocol ID definitions

| Name | Value |
| --- | --- |
| MIH Information Service | 0 |
| MIH Command and Event Services Capability Discovery | 1 |
| Reserved | 2 – 63 |

Insert the following new clauses after 7.3.2.35

#### 7.3.2.37 GAS Request Information element

The GAS Request Information element contains information about the details of the GAS request. The format of the GAS Request element is shown in 5.

|  | Element ID | Length | GAS Protocol ID | Query |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | variable |

Figure u6—GAS Request Information element format

The Element ID field is equal to the GAS Request value in Table 26.

The Length field is variable. The minimum value of the Length field is 2.

The GAS Protocol ID is used by a STA to request a query using a specific GAS protocol.  This field is defined in clause 7.3.2.36.

The actual format of the Query depends on the GAS protocol ID value. For e.g. if the GAS protocol ID value is 0 (see 7.3.2.36), then the format is as per the IEEE 802.21 specification.

The GAS Request element is included in a GAS Initial Request action frame.

#### 7.3.2.38 GAS  Response Information element

The GAS Response Information element contains information about the details of the advertisement response. The format of the GAS Response element is shown in 6.

|  | Element ID | Length | GAS Protocol ID | Response Info |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | variable |

Figure u6— GAS Response Information element format

The Element ID field is equal to the GAS Response value in Table 26.

The Length field is variable. The minimum value of the Length field is 2.

The GAS Protocol ID field is equal to protocol ID which was specified in the preceeding request.

The GAS Response info contains the result of the preceeding query request. The actual format of the GAS Response Info depends on the GAS Protocol ID value. For e.g. if the GAS protocol ID value is 0 (see 7.3.2.36), then the format is as per the IEEE 802.21 specification.

GAS Response Element is sent by the BSS to the non-AP STA in the GAS Response or GAS Comeback frame to provide the information about one or more requested network services.

#### 7.3.2.39 GAS Traffic Indication Map Information Element

The GAS Traffic Indication Map (GASTIM) Information element shall be present in every beacon if dot11AdvertisementServiceEnabled is true and at least one of the Advertisement Protocols (cf. clause 7.3.2.36) is configured for multicast delivery method.  The format of the GASTIM element is shown in 7.

|  | Element ID | Length | GASTIM Count | GASTIM Period | Time to Suspend | Query ID(s) (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 2 | N |

Figure u7— GASTIM Information element format

The Length field is variable. The minimum value of the Length field is 4.

The GASTIM Count field indicates how many beacons (including the current frame) appear before the next

GASTIM. A GASDTIM Count of 0 indicates that the current beacon is a GASTIM and advertising frames will be scheduled for transmission after the cessation of the GASTIM beacon. The GASTIM count field is a single

octet.

The GASTIM Period field indicates the number of Beacon intervals between successive GASTIMs. If all beacons are GASTIMs, the GASTIM Period field has the value 1. The GASTIM Period value 0 is reserved. The GASTIM period field is a single octet.

The Time To Suspend field indicates the number of TUs after TBTT(GASTIM) after which the AP will not schedule for transmission any further multicast Advertising frames.  The Time to Suspend field is two octets.

The Query ID(s) field is a variable length field composed of a list of Query ID(s) (cf. clause 7.3.2.37).  Inclusion of a Querry ID in this list means the AP has scheduled the GAS Comeback Response(s) (cf. clause 7.4.6.1) for transmission subsequent to this GASTIM with the Query ID(s) as indicated in this field. This field can only be present when GASTIM count is zero.

#### 7.3.2.41 GAS Comeback Delay

The GAS Comeback Delay Information element contains information about the delay time in response to preceeding GAS request. The format of the GAS Comeback Delay element is shown in 8.

|  | Element ID | Length | GAS Comeback Delay |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure u8— GAS Comeback Delay Information element format

The Element ID field is equal to the GAS Response value in Table 26.

The Length field is 2.

The GAS Comeback Delay field specify the delay time value in milliseconds.

The GAS Comeback Delay is returned by the BSS in the GAS Response frame to indicate that a backend query is being carried out for the non-AP STA. The non-AP STA shall use the GAS Comeback Request to retrieve the response information after expiry of GAS Comeback delay.

#### 7.3.2.42 ESSID Information Element

The ESSID Information element contains description of ESSID. The format of the ESSID element is shown in 9.

|  | Element ID | Length | ESSID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure u8— ESSID Information element format

The Element ID field is equal to the ESSID value in Table 26.

The Length field is variable. The minimum value of the Length field is 6.

The ESSID field specifies the definition of ESSID.

In the infrastructure mode, the ESSID definition includes the BSSID value of one of the group of APs . The ESSID and the SSID together to provide is a unique value that is advertised in beacons and probe responses so that the non-AP STA is aware of continued applicability of previously discovered interworking and advertising services when moving from one AP to another within the scope of the ESSID definition. (there is a possibility to redefine this to concept normalize with other TG e.g. TGr).

  - Action frame format details

Insert the following after clause 7.4.5:

    - Interworking action details

Several Action frame formats are defined for Interworking purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Error: Reference source not found.

Table u58—Interworking Action field values

| Action field value | Description |
| --- | --- |
| 0 | GAS Initial Request |
| 1 | GAS Initial Response |
| 2 | GAS Comeback Request |
| 3 | GAS Comeback Response |
| 4-255 | Reserved |

      - GAS Initial Request Action frame format

The GAS Request frame uses the Action frame body format and is transmitted by a nonAP-STA to AP. The format of the GAS Request frame body is shown in Figure u11.  All GAS frames shall be transmitted using UP = 0 with the EDCA parameters for AC_BE.

|  | Category | Action | Dialog Token | GAS Request IE |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | Variable |

Figure u4— GAS Request frame body format

The Category field is set to the value indicating the Interworking category, as specified in Table 24.

The Action field is set to the value indicating GAS Request, as specified in Error: Reference source not found58 in Error: Reference source not found.

The Dialog Token field is set by the requesting STA for the action frame exchange.

GAS request element is defined in 7.3.2.39. There shall be only one element per request.

      - GAS Initial Response Action frame format

The GAS Response frame uses the Action frame body format and is transmitted by a nonAP-STA to AP. The format of the GAS Response frame body is shown in Figure u12.

|  | Category | Action | Dialog Token | Status Code | GAS Query ID | GAS multicast address (optional) | GAS Comeback Delay (optional) | GAS Response Element (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 1 | 6 | 1 | Variable |

Figure u12— GAS Initial Response frame body format

The Category field is set to the value indicating the Interworking category, as specified in Table 24.

The Action field is set to the value indicating GAS Initial Request, as specified in Error: Reference source not found58 in Error: Reference source not found.

The Dialog Token field is copied from the preceeding request action frame.

The Status Code values are defined in 7.3.1.9.

The GAS Query ID is defined in 7.3.1.18.. This value is set to 0 when there is no outstanding request to process.

The GAS Comeback Delay is defined in 7.3.2.38. If this element is not present then the non-AP STA shall not perform a GAS-Comeback frame exchange as defined in the unicast GAS mechanism.

The GAS multicast address is a muticast MAC address (6 octets) used for multicast GAS mechanism (section 11.1.x).

The GAS Response Element is defined in 7.3.2.40. There could be zero or one element per response.

      - GAS Comeback Request Action frame format

The GAS Comaback Response frame uses the Action frame body format and is transmitted by a nonAP-STA to AP. The format of the GAS Comaback Response frame body is shown in Figure u12.

|  | Category | Action | GAS Query ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure u12— GAS Comeback Request frame body format

The Category field is set to the value indicating the Interworking category, as specified in Table 24.

The Action field is set to the value indicating GAS Comeback Request, as specified in Error: Reference source not found58 in Error: Reference source not found.

The GAS Query ID is defined in 7.3.1.18. This value is copied from the earlier GAS Initial Response frame.

      - GAS Comeback Response Action frame format

The GAS Comeback Response frame uses the Action frame body format and is transmitted by a an  AP  to non AP.-STA. This action frame is used for both the unicast delivery and multicast delivery methods. The format of the GAS Comeback Response frame body is shown in Figure u12.

|  | Category | Action | GAS Query ID | Status Code | GAS Comeback Delay (optional) | GAS Request Element (optional) | GAS Response Element (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 | 1 | Variable | Variable |

Figure u12— GAS Comeback Response frame body format

The Category field is set to the value indicating the Interworking category, as specified in Table 24.

The Action field is set to the value indicating GAS Comeback Response, as specified in Error: Reference source not found58 in Error: Reference source not found.

The GAS Query ID field is copied from the preceeding GAS Initial request action frame exchange.

The Status Code values are defined in 7.3.1.9. When the multicast delivery method is used, the status code is set to zero.

The GAS Comeback Delay is defined in 7.3.2.38. This element is used in unicast GAS mechanism and is present when there is no GAS reponse element included in this frame. This is present and provides an indication for STA to make another GAS Comeback frame exchange.

The GAS Request Element is defined in 7.3.2.37. This information element is only included when multicast delivery method is used.

The GAS Response Element is defined in 7.3.2.40. There could be zero or only one element per response.

# 10. Layer Management

## MLME SAP interface

10.3.30.1 MLME-GAS-Initial-Request.request

10.3.30.1.1 Function

This primitive requests the information of a specific advertisement service from the network. It is valid at the non-AP STA and requests the AP.

10.3.30.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Initial-Request.request (

GAS Protocol ID,

Query,

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| Query | String | N/A | Query string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

10.3.30.1.3 When generated

This primitive is generated by the SME at a non-AP STA to request a specific advertisement service from the network side.

10.3.30.1.4 Effect of receipt

The non-AP STA operates according to the procedures defined in 11.y

10.3.30.2 MLME-GAS-Initial-Response.confirm

10.3.30.2.1 Function

This primitive reports the status code of the GAS request with an optional query ID, comeback delay information or multicast address and the response elements, if available at that point.

10.3.30.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Initial-Response.confirm (

StatusCode

GASQuery ID,

GASComeback Delay,

GASMulticastAddress,											GASProtocolID

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available | Indicates the result response to the GAS-initial-request from the peer MAC entity. |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASMulticastAddress | MACAddress | A multicast MAC address | Specifies the multicast MAC address to which the response info frame is addressed. A response info frame is a data frame with a query response. |
| GASProtocolID | Integer | N/A | Protocol ID identifying which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

10.3.30.2.3 When generated

This primitive is generated by the MLME as a response to the MLME-GAS-initial-Request.request primitive indicating the result of that request.

The primitive is generated when the non-AP STA receives a response in the form of a GAS Initial Response frame is received in the corresponding action frame from the AP.

10.3.30.2.4 Effect of receipt

The non-AP STA operates according to the procedures defined in 11.y

10.3.30.3 MLME-GAS-Initial-Request.indication

10.3.30.3.1 Function

This primitive reports to the AP SME about the GAS Request.

10.3.30.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Initial-Request.indication (

GAS Protocol ID,

Query

Non-APSTAAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| Query | String | N/A | Query string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which query message is received. |

10.3.30.3.3 When generated

This primitive is generated by the MLME as a result of receipt of a request for the GAS advertisement service from a non-AP STA.

10.3.30.3.4 Effect of receipt

The AP is notified of the request from the non-AP STA.

The SME should operate according to the procedures defined in 11.y.

The SME generates an MLME-GAS-Initial-Response.response primitive within a dot11GASResponseTimeout.

10.3.30.4 MLME-GAS-Initial-Response.response

10.3.30.4.1 Function

This primitive responds to the request for an advertisement service by a specified non-AP STA MAC entity.

10.3.30.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Initial-Response.response (

StatusCode,

Non-APSTA Address,

GASQueryID,

GASComebackDelay,

GASMulticastAddress,

GASProtocolID

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available | Indicates the result response to the GAS-initial-request from the peer MAC entity. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASMulticastAddress | MACAddress | A multicast MAC address | Specifies the multicast MAC address to which the response info frame is addressed. A response info frame is a data frame with a query response. |
| GASProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

10.3.30.4.3 When generated

This primitive is generated by the SME at the AP as a result of an MLME-GAS-Initial-Request.indication primitive.

10.3.30.4.4 Effect of receipt

The primitive causes the MAC entity at the AP to send a GAS response frame in the corresponding GAS response action management frame to the non-AP STA.

10.3.31.1 MLME-GAS-Comeback-Request.request

10.3.31.1.1 Function

This primitive requests the information of a specific advertisement service from the network in a comeback request. It is valid at the non-AP STA and requests the AP.

10.3.31.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.request (

GASQueryID

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |

10.3.31.1.3 When generated

This primitive is generated by the SME at a non-AP STA to request a specific GAS advertisement service from the network side using a comeback request.

10.3.31.1.4 Effect of receipt

The non-AP STA operates according to the procedures defined in 11.y

10.3.31.2 MLME-GAS-Comeback-Response.confirm

10.3.31.2.1 Function

This primitive reports the status code of the GAS comeback request with an optional query ID, comeback delay information and the response elements, if available at that point.

10.3.31.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.confirm (

StatusCode,

GASQuery ID,

GASComeback Delay,

GASProtocolID,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available | Indicates the result response to the GAS-Comeback-request from the peer MAC entity. |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

10.3.31.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-GAS-Comeback-Request.request primitive indicating the result of that request.

The primitive is generated when the non-AP STA receives a response in the form of a GAS Comeback Request frame in the corresponding action frame from the AP.

10.3.31.2.4 Effect of receipt

The non-AP STA operates according to the procedures defined in 11.y

10.3.31.3 MLME-GAS-Comeback-Request.indication

10.3.31.3.1 Function

This primitive reports to the AP SME the request GAS Comeback-Request.

10.3.31.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.indication (

GASQueryID,

Non-APSTAAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |

10.3.31.3.3 When generated

This primitive is generated by the MLME as a result of receipt of a request for the advertisement service from a non-AP STA.

10.3.31.3.4 Effect of receipt

The AP is notified of the request from the non-AP STA.

The SME should operate according to the procedures defined in 11.y.

The SME generates an MLME-GAS-Comeback-Response.response primitive within a dot11GASResponseTimeout.

10.3.31.4 MLME-GAS-Comeback-Response.response

10.3.31.4.1 Function

This primitive responds to the request for a advertisement service by a specified non-AP STA MAC entity.

10.3.31.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.response (

StatusCode,

Non-APSTA Address,

GASQueryID,

GASComebackDelay,

GASProtocolID,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available | Indicates the result response to the GAS-Comeback-request from the peer MAC entity. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| GASQueryID | Integer | As defined in frame format | used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in GASProtocolID. E.g, if the GASProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

10.3.31.4.3 When generated

This primitive is generated by the SME at the AP as a result of an MLME-GAS-Comeback-Request.indication primitive.

10.3.31.4.4 Effect of receipt

The primitive causes the MAC entity at the AP to send a GAS Comeback-response frame in the corresponding GAS Comeback-response action management frame to the non-AP STA.

# 11. MLME

Insert the following new clauses after section 11.10

## 11.11 Interworking Procedures : Generic Advertisement Services

This clause describes the actions and the procedures to enable seamless interworking of a multi-access capable STA with other non-802.11 access technologies or when other non-802.11 specific network protocols are involved. IEEE 802.11u GAS capability is included in the beacon and probe response frames.

The GAS inftomation delivery mechanism is supported using two options.

i) Multicast mechanism

ii) Unicast mechanism

The AP shows whether it supports multicast delivery method or unicast delivery method or both corresponding to the GAS Protocol Field as defined in clause 7.3.2.36.  If both delivery methods are supported, the non-AP STA requests the desired delivery  method by setting to one the corresponding bit in the GAS Protocol Field, which is part of the GAS Request information element (clause 7.3.2.37).  If the AP supports only one delivery method, the non-AP STA must choose this method by setting to one the corresponding bit in the GAS Protocol Field, which is part of the GAS Request information element.  If the non-AP STA does not set the same bit as advertised by the AP, an error status code is returned in the GAS Initial Response action frame.

### 11.11.1.2 Multicast Mechanism

The non-AP STA obtains the GAS capability information from beacons or Probe response messages. The capability fields indicate the GAS protocol ids supported in the BSS. The non-AP STA can send a GAS-Initial-Request with one of the supported protocol ids with specific query to obtain information about the specific interworking service. Upon receipt of the GAS-Intitial-Request frame AP generates a GAS-Initial-Response with a GAS Query ID and a GAS multicast address. The AP forwards or proxies the request to the backend GAS server to retrieve the query results. The response message from the GAS server is queued in the AP for multicast delivery at a later time.

In order to support multicast mechanism, a GASTIM IE is included in the beacons. The GASTIM count field is decremented from a configured count value to zero for every beacon and then restarted with the configured value for the following beacon. The GAS multicast delivery begins immediately after the beacon that includes the GASTIM count field value set to zero. The multicast delivery includes one or more GAS Comeback Response Action Frames that include the GAS query ID, corresponding GAS request element (optional), and GAS query results information.. The following figure shows the messaging sequence.

Figure 17 ― Example flow for GAS Multicast Query Mechanism

AP transmits multicast GAS Comeback Reponse Action Frames containing encapsulated query response. Each frame is transmitted several times to make the transmission more reliable as configured in the AP..

## 11.11.2 Unicast Mechanism

The non-AP STA obtains the GAS capability information from beacons or Probe response messages. The capability fields indicate the GAS protocol ids supported in the BSS. The non-AP STA can send a GAS-Initial-Request with one of the supported protocol ids with specific query to obtain information about the specific interworking service. Upon receipt of the GAS-Intitial-Request frame AP generates a GAS-Initial-Response with a GAS Query ID and a GAS Comeback delay. The AP forwards or proxies the request to the backend GAS server to retrieve the query results. The response message from the GAS server is queued in the AP for the comeback delay duration.

The non-AP STA shall send a GAS Comeback Request immediately after the expiry of the comeback delay including the GAS Query ID provided in the earlier GAS-Initial-Response. The AP shall provide the results in the GAS-Comeback-Response with the Query ID, corresponding GAS Request element (optional) and the query results. The AP can discard the query results data soon after.

If there were no Query results queued at the time of receiving the GAS-Comeback-Request then the AP can provide a new Comeback delay value in the GAS-Comeback-response with same Query ID.

Figure 17 ― Example flow for GAS Unicast Query Mechanism

The basic protocol concept for the unicast mechanism is as follows:

  - An AP receiving a GAS request from an STA with a for a certain advertisement service request can ensure that the specific service is supported in the backend and send an immediate response.

  - Within the GAS response, the AP can return a query identifier Query ID (whose value is unique for the AP) to be used by the STA to query the AP again without the need for indicating again the IEs required (this may be useful when more than one IEs are requested and are not available instantaneously at the AP, and performing a new query by providing again the list of IEs would imply a waste of radio resources), and by the AP to correlate the queries.

  - The AP may also return also a time value Comeback Delay determined by the AP and indicating how long the station shall wait before querying the AP again. The STA does not need to remain active while waiting to send the GAS Comeback request, therefore enabling power saving. In case the STA is connected to an AP1 and is actively exchanging data frames with AP1, but wants to retrieve information from a target AP2, the STA does not need to remain on the AP2 channel while waiting to send the GAS Comeback request.

  - The STA waits Comeback delay period and sends a GAS Combeback Request with the Query ID.

  - The AP returns the response corresponding to the Query ID if the response is returned by the server and queued at the AP. If the server has not returned the reponse, then the AP returns the same Query ID with the same or new Comeback Delay value.

# Annex A

# Amendments to the 802.11 draft

1.2 Purpose

…

Specifically, this standard

— Describes the functions and services required by an IEEE 802.11™-compliant device to operate

within ad hoc and infrastructure networks as well as the aspects of station mobility (transition)

within those networks.

— Defines the MAC procedures to support the asynchronous MAC service data unit (MSDU) delivery

services.

— Defines several PHY signaling techniques and interface functions that are controlled by the IEEE

802.11 MAC.

— Permits the operation of an IEEE 802.11-conformant device within a wireless local area network

(LAN) that may coexist with multiple overlapping IEEE 802.11 wireless LANs.

— Describes the requirements and procedures to provide data confidentiality of user information being

transferred over the wireless medium (WM) and authentication of IEEE 802.11-conformant devices.

— Defines mechanisms for dynamic frequency selection (DFS) and transmit power control (TPC) that

may be used to satisfy regulatory requirements for operation in the 5 GHz band. The regulations and

conformance tests are listed in Clause 2.

— Defines the medium access control (MAC) procedures to support local area network (LAN) applications

with quality of service (QoS) requirements, including the transport of voice, audio, and video

over IEEE 802.11 wireless LANs (WLANs).

Insert the following bullet below:

- Defines the functions and procedures required by an IEEE802.11TM- compliant device to access external network services within a IEEE802.11 wireless LAN.

3. Definitions

Insert the following new definitions in alphabetical order, renumbering as necessary:

[Note: This is taken from TGu Technical Requirement document]

3.xxx Subscription Service Provider (SSP): an organization (operator) offering connection to network services, usually for a fee.  The user has a contractual relationship with the service provider.

3.xxx Subscription Service Provider Network (SSPN): the network with which a STA has an established relationship with an SSP.  The network maintains user subscription information, and is always the same for a given user identity, or indeed multiple identities.

## A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition

### A.4.3 IUT configuration

Append this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFu | Interworking supported | 11.11 | O | Yes, No, N/A |

Insert this new clause after clause A.4.16:

### A.4.16 Interworking extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking capabilities supported? |  |  |  |
| IW1 | Advertisement support | 11.11.1 | CFu:M | Yes, No, N/A |

Add the following MIB attributes to Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11AdvertisementServicesEnabled TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11AdvertisementServicesEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting either non-802.11 Network

Access services or non-802.11 specific protocol functionalities in support of seamless interworking.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }