IEEE P802.11
Wireless LANs

| MIH Support – Normative text proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vivek Gupta | Intel Corporation | JF3-206 2111 NE 25 th Ave, Hillsboro, OR 97124 | +1 503 712 1754 | vivek.g.gupta@intel.com |
| Necati Canpolat | Intel Corporation | JF3-381 2111 NE 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Ronny Yongho Kim | LG Electronics, Inc. | 533 Hogye-ldong Dongan-gu Anyang-shi Kyongki-do Korea | +82 31 450 1879 | ronnykim@lge.com |
| Dave Stephenson | Cisco Systems | 170 W Tasman Dr, San Jose CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Eleanor Hepworth | Siemens Roke Manor Research | Roke Manor, Old Salisbury Ln, Romsey, Hampshire, SO51 0ZN, UK | +44 1794833146 | eleanor.hepworth@roke.co.uk |
| Andrew McDonald | Siemens Roke Manor Research | Roke Manor, Old Salisbury Ln Romsey, Hampshire, SO51 0ZN, UK | +44 1794833833 | andrew.mcdonald@roke.co.uk |
| Qiaobing Xie | Motorola | 600 North U.S. Hwy 45, Libertyville, IL 60048 | 847 523 0126 | qiaobing.xie@motorola.com |
| Ajoy Singh | Motorola | 600 North U.S. Hwy 45, Libertyville, IL 60048 | 847 523 0126 | asingh1@motorola.com |
| Jon Edney | Nokia | Cambridge, UK | +44 1353 648567 | EXT-jon.1.edney@nokia.com |
| Jari Jokela | Nokia | Tampere, Finland | +358 718060445 | jari.jokela@nokia.com |
| Dongwoon Hahn | LG Electronics, Inc. | 16 Woomyeon-dong Seocho-gu Seoul Korea | +82 2 526 4198 | dhahn@lge.com |
| Sabine Demel | T-Mobile | Vienna, Austria |  | sabine.demel@t-mobile.at |
| Jouni Korhonen | TeliaSonera | P.O.Box 970, 00051 Sonera, Finland | +358 40534445 | jouni.korhonen@teliasonera.com |
| Frans Hermodsson | TeliaSonera | Isbergs gata 2, SE-205 21 Malmo, Sweden | +46 406618658 | frans.hermodsson@teliasonera.com |
| Wolfgang Groeting | BenQ | Neutorplatz 3-4, 46395 Bocholt, Germany | +49 2842 95 2142 | wolfgang.groeting@benq.com |
| Stefan Berg | BenQ | Neutorplatz 3-4, 46395 Bocholt, Germany | +49 2842 95 1781 | stefan.berg@benq.com |
| Hong Cheng | Panasonic | BLK1022 TaiSeng Ave #06-3530 TaiSeng Ind. Est.Singapore 534415 | +65 6550 5477 | Hong.cheng@sg.panasonic.com |
| Sabine Demel | T-Mobile | Rennweg 97-99 1030 Vienna Austria | +43 1 795 85 7720 | Sabine.demel@t-mobile.at |
| Andrew Myers; Colin Blanchard | British Telecommunications | Orion Building, First Floor PP8 Adastral Park, Ipswich | +44 1473 605395 | Andrew.myers@bt.com Colin.blanchard@bt.com |
| Farooq Bari | Cingular Wireless | 7277 164th Avenue NE Redmond, WA 98052 | 425 580 5526 | farooq.bari@cingular.com |
| Marian Rudolf | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6258 | marian.rudolf@interdigital.com |
| Ulises Olvera-Hernandez | InterDigital | 1000 Sherbrooke W. Montreal, QC, Canada H3A 3G4 | +1 514 904 6282 | ulises.olvera-hernandez@interdigital.com |
| Joe Kwak | InterDigital | 482 Degas Bolingbrook, IL 60440 | +1-630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document provides a normative text proposal in support of requirement R11H1 - Media Independent Handover Cluster Requirements in 05/822r11.

- R11N1 “Provide a mechanism  to support Media Independent Handoff capability.”

This text proposal is based on P802.11-REVma-D7.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	4

10. Layer Management	5

10.3.2.2 MLME-SCAN.confirm	5

10.3.30 Media Independent Handover	5

10.3.30.1 Link Up	5

10.3.30.1.1 Function	5

10.3.30.1.2 Semantics of service primitve	5

10.3.30.1.3 When generated	5

10.3.30.1.4 Effect of receipt	5

10.3.30.2 Link down	6

10.3.30.2.1 Function	6

10.3.30.2.2 Semantics of service primitve	6

10.3.30.2.3 When generated	6

10.3.30.2.4 Effect of receipt	6

10.3.30.3 Link going down	6

10.3.30.3.1 Function	6

10.3.30.3.2 Semantics of service primitve	6

10.3.30.3.3 When generated	7

10.3.30.3.4 Effect of receipt	7

10.3.30.4 Link event rollback	7

10.3.30.4.1 Function	7

10.3.30.4.2 Semantics of service primitve	7

10.3.30.4.3 When generated	8

10.3.30.4.4 Effect of receipt	8

10.3.30.5 Link parameters change	8

10.3.30.5.1 Function	8

10.3.30.5.2 Semantics of service primitve	8

10.3.30.5.3 When generated	9

10.3.30.5.4 Effect of receipt	9

10.3.30.6 Link handover imminent	9

10.3.30.6.1 Function	9

10.3.30.6.2 Semantics of service primitve	9

10.3.30.6.3 When generated	9

10.3.30.6.4 Effect of receipt	9

10.3.30.7 Link handover complete	10

10.3.30.7.1 Function	10

10.3.30.7.2 Semantics of service primitve	10

10.3.30.7.3 When generated	10

10.3.30.7.4 Effect of receipt	10

11. MLME	11

11.11 Interworking Procedures	11

11.11.1 MIH Support	11

11.11.2 MIH Capability Indication Support	11

11.11.3 MIH Event/Command Service Support	12

11.11.4 MIH Information Services Support	12

11.11.5 MIH Protocol Request	12

11.11.6 MIH Protocol Indication (response)	12

Annex A	13

A3.1 Capability Field – Current state	13

A3.2 MIHF Frame Format	13

A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition	14

A.4.3 IUT configuration	14

A.4.16 Interworking extensions	15

Annex D	15

# 7. Frame formats

[Refer to Network Selection Proposal. MIH Cluster uses same parameters as Network Selection for this section.]

# 10. Layer Management

- 10.3.2.2 MLME-SCAN.confirm

Modify the second  row as shown below:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, NOT_SUPPORTED, No Available AP | Indicates the result of the MLMESCAN.confirm. |

Insert the following new clauses after section 10.3.29

- 10.3.30 Media Independent Handover

This subclause describes the management procedures associated with the Media Independent Handover.

#### 10.3.30.1 Link Up

#### 10.3.30.1.1 Function

Link Up is an L2 event which indicates that higher layers can now send packets over the link. All L2 activities required for configuring the link are expected to be completed at this point. Thus in case of 802.11 network with 802.11i security it would mean that not only has station associated with an AP, but the 4 way handshake is also complete.

#### 10.3.30.1.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkUp.indication (

MacMobileTerminal,

LinkIdentifier,

MacNewAP

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| LinkIdentifier | String | N/A | Unique 802.11 link identifier (SSID + BSSID) |
| MacNewAP | MAC Address | Any valid individual MAC Address | MAC Address of AP to which the STA is attached |

#### 10.3.30.1.3 When generated

Generated by MLME when all L2 connection set up activities are completed.

#### 10.3.30.1.4 Effect of receipt

Higher layers may initiate a new connection, router discovery, IP configuration, etc.

#### 10.3.30.2 Link down

#### 10.3.30.2.1 Function

This notification is delivered when a layer 2 connection is broken on the specified link and when no more packets can be sent on the link.

#### 10.3.30.2.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkDown.indication (

MacMobileTerminal,

MacOldAP,

ReasonCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| MacOldAP | MAC Address | Any valid individual MAC Address | MAC Address of AP to which the STA was attached before the link went down |
| ReasonCode | UNSIGNED BYTE | 0-255 | Reason for why the link went down. 1: Explicit Disconnnect 2:Packet Timeout 3:No resources 4:No broadcast reception 5-127: Reserved 128-255: Vendor Specific |

#### 10.3.30.2.3 When generated

This notification is generated when layer 2 connectivity is lost. For example, Layer 2 connectivity may be lost explicitly in cases where a STA initiates disassociate type procedures or is disassociated by a peer STA.

#### 10.3.30.2.4 Effect of receipt

Higher layers may take different actions on this notification. The handover policy may eliminate this link from list of active links for routing connections and may consider handing over any potential active connections to other more suitable links.

#### 10.3.30.3 Link going down

#### 10.3.30.3.1 Function

This notification is delivered when a Layer 2 connection is expected (predicted) to go down (Link Down) within a certain time interval. Link Going Down event may be the indication to initiate handover procedures. Different applications may have different requirements with respect to the time interval required to anticipate in advance a link going down and make adequate a priori preparation to handover a connection depending on Application Class. Hence upper layers such as a L3 mobility protocol may actually configure the time interval in which a Link Going Down event may be delivered. This would allow the flexibility for the time interval to be in msecs, 100s of msecs or even seconds.

#### 10.3.30.3.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkGoingDown.indication (

MacMobileTerminal,

TimeInterval,

ConfidenceLevel,

UniqueEventIdentifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| TimeInterval (msecs) | UNSIGNED INTEGER |  | Time Interval in msecs which the link is expected to go down. The link connectivity is expected to be available at least for time specified by TimeInterval |
| ConfidenceLevel (%) | UNSIGNED BYTE | 0-100 | The confidence level for link to go down within the specified time interval |
| UniqueEventIdentifier | UNSIGNED INTEGER |  | Used to uniquely identify the event. To be used in case of event rollback |

#### 10.3.30.3.3 When generated

A Link Going Down event implies that a Link_Down is imminent within a certain time interval. If Link Down is NOT received within specified time interval then actions due to previous Link Going Down may be rejected.

A 100% confidence would indicate certainty that the link is definitely going down within the specified time interval. For example an AP that may have decided to shut down for administrative reasons may send down a Link Going Down trigger with 100% confidence level. Predictions based on changes in link parameters such as signal strength values would typically have a lower confidence level. Different links would typically use implementation specific methods to predict future loss in link connectivity. For example, a STA may infer loss of link connectivity due to successive timeouts for acknowledgements of retransmitted packets along with loss of reception of broadcast frames.

#### 10.3.30.3.4 Effect of receipt

Higher layers may take different actions on this notification. Upper layers may prepare to initiate handovers based on the confidence level reported as part of the event.

#### 10.3.30.4 Link event rollback

#### 10.3.30.4.1 Function

Link_Event_Rollback is used in conjunction with Link_Going_Down. In case of Link_Going_Down if the link is no longer expected to go down in the specified time interval, then a Link_Event_Rollback message is sent to the Event destination. The destination should disregard or rollback the changes associated with the event identifier in such cases.

#### 10.3.30.4.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-EventRollback.indication (

MacMobileTerminal,

UniqueEventIdentifier

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| UniqueEventIdentifier | UNSIGNED INTEGER |  | Used to uniquely identify the event, obtained from Link Going Down. To be used to rollback the predictive event that did not occur. |

#### 10.3.30.4.3 When generated

This notification is generated when a predictive event expected to occur within a certain time interval is no longer expected to occur before the expiry of the indicated time interval.

#### 10.3.30.4.4 Effect of receipt

Higher layers may reevaluate the link under consideration by retrieving other link quality related parameters. Any handover procedures that may have already been initiated may need to be rescinded.

#### 10.3.30.5 Link parameters change

#### 10.3.30.5.1 Function

Link Parameters Change indicates changes in link parameters have crossed specified threshold levels. This may include link layer parameters such as Speed of the link, QoS, BER, etc. The threshold level for each such parameter may have to be configured through a separate command to link layer.

#### 10.3.30.5.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkParametersChange.indication (

MacMobileTerminal,

LinkParameterType,

OldValueOfLinkParameter,

NewValueofLinkParameter

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| LinkParameterType |  |  | Media specific parameter types that have crossed thresholds |
| oldValueOfLinkParameter |  |  | Old values of link parameters |
| newValueOfLinkParameter |  |  | New values of link parameters that have crossed thresholds |

Examples of link layer parameters that could trigger this event include:

- Link Speed

- Link Bit Error Rate

- Link Frame Loss Rate before Retransmission

- Link Received Signal Strength

#### 10.3.30.5.3 When generated

This notification is generated when a link layer parameter crosses a configured threshold.

#### 10.3.30.5.4 Effect of receipt

Higher layers may take different actions on this notification. If parameters related to link quality cross a certain threshold then that link may need to be evaluated for handing over current connections. The MIH Function may collectively evaluate different parameters and give appropriate indications to higher layers regarding suitability of different links.

#### 10.3.30.6 Link handover imminent

#### 10.3.30.6.1 Function

Link Handover Imminent is generated when a native link layer handover or switching decision has been made and its execution is imminent (as opposed to Link Going Down which only indicates that a link is losing connectivity due to a change in certain link condition such as signal strength, but does not guarantee that an autonomous link switchover has been decided by the link layer). It contains information about the new point of attachment of the mobile node. It also contains information about any application specific data that might be useful for the application running on the mobile node. Link Handover Imminent can be used by upper layers (e.g., transport ) to initiate handover specific adaptation and as an enabler for initiating effective make-before-break handovers thereby reducing packet loss due to link layer handovers. This is a link layer event that exists for intra-technology handovers defined in many media types (e.g., cellular, 802.16e). MIH can receive the Link Handover Imminent events from the link layer and pass them to upper layer as MIH events for both inter-technology and intra-technology handover cases.

#### 10.3.30.6.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkHandoverImminent.indication (

MacMobileTerminal,

MacNewAP

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| MacNewAP | MAC Address | Any valid individual MAC Address | MAC Address of candidate AP to which the STA may consider attaching |

#### 10.3.30.6.3 When generated

Depending on whether it is the Mobile node or the Network, it is generated when a native link layer handover or switching decision has been made and its execution is imminent.

#### 10.3.30.6.4 Effect of receipt

Application may take necessary actions to minimize the effect of the pending native link layer handover or switching on user data transfer. For example, to cover the handover period (when packets cannot be transferred), the network can proactively start sending the additional data as soon as it has received indication about the event and should expect a request from the mobile specifying how much data it needs to cover the handover period. This event may also be used as an indication to start buffering packets.

#### 10.3.30.7 Link handover complete

#### 10.3.30.7.1 Function

Link Handover Complete event is generated whenever a native link layer handover/switching has just been completed (as opposed to Link Up which only indicates that a link has been brought up for L2 connectivity, but does not indicate that a native link handover/switchover has just been completed by the link layer). Notifying the upper layer of this event can improve transport, session and application layer responsiveness to autonomous link changes. They can better adapt their data flows by resuming flows upon receiving this indication. The upper layer can also use this event to check whether its IP configuration needs to be updated. This is a link layer event that exists for intra-technology handovers defined in many media types (e.g., cellular, 802.16e). This event is applicable for terminal side only and is valid only for intra-technology handovers. MIH Function can receive the Link Handover Complete events from the link layer and pass them to upper layer as MIH events.

#### 10.3.30.7.2 Semantics of service primitve

The primitive parameters are as follows:

MLME-LinkHandoverComplete.indication (

MacMobileTerminal,

MacNewAP

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MacMobileTerminal | MAC Address | Any valid individual MAC Address | MAC Address of STA |
| MacNewAP | MAC Address | Any valid individual MAC Address | MAC Address of candidate AP to which the STA may consider attaching |

#### 10.3.30.7.3 When generated

This is generated whenever a L2 link layer handover or switching has just been completed.

#### 10.3.30.7.4 Effect of receipt

Upon reception of this event, an upper layer (e.g., transport) can stop any handover adaptation that it has engaged to cope with the just completed native link layer handover/switching and resume normal data transfer. This event may also be used as an indication that a re-verification of the IP parameter should be considered.

# 11. MLME

Insert the following new clauses after section 11.10

## 11.11 Interworking Procedures

This clause describes the actions and the procedures to enable seamless interworking of a multi-access capable STA with other non-802.11 access technologies or when other non-802.11 specific network protocols are involved.

### 11.11.1 MIH Support

The IEEE 802.21 (Media Independent Handover) standard supports handovers across heterogeneous networks. The key elements of this standard are as follows.

- The standard defines a framework that enables transparent service continuity when a mobile node (MN) switches between heterogeneous link-layer technologies. The framework relies on the identification of a mobility-management protocol stack within the network elements that support the handover. The standard defines a set of handover-enabling functions within the protocol stacks of the network elements and the creation therein of a new entity called the MIH Function. A media independent Service Access Point (called the MIH_SAP) and associated primitives are defined to provide MIH users with access to the services of the MIH Function. The MIH Function provides the following services.

- MIH capability discovery is discovering Event Service (ES) and/or Command Service (CS) capability of MIH function. In case of 802.11 network, STA can discover ES/CS MIH capability by sending MIH capability discovery query and receiving MIH capability discovery query response from the AP in State 1. In State 3, MIH function shall use data frame, MIH Ethertype Ethernet frame or layer 3 data frame.

- The Media Independent Event service detects events and delivers event messages from both local as well as remote interfaces. In case of 802.11 networks, events can originate both at the STA and the AP. Events can be delivered both from STA to AP and vice versa in State 3 only, using layer 3 capabilities and is outside the scope of 802.11.

- The Media independent Command service provides a set of commands for the MIH users to control handover related link states. In case of 802.11 networks, these commands can originate both at the STA and the AP. Commands can be delivered both from STA to AP and vice versa in State 3 only, using layer 3 capabilities and is outside the scope of 802.11.

- The Media Independent Information service provides an information repository, for query and response, thus enabling the making of more effective handover decisions across different heterogeneous networks. 802.11 STAs can query for information both in State 1 using management frames and State 3 using data frames.

### 11.11.2 MIH Capability Indication Support

Similar to the existing QoS Capability IE and the Fast BSS Transition Information IE, TGu will introduce a Generic Advertisement Service (GAS) Capabilities IE that can be extended to include various advertisement services. It will contain specific protocol ID for MIH IS. It will also provide protocol ID for MIH CS/ES capability discovery, indicating specific MIH capability sets and/or availability of particular MIH services that AP has connectivity. Connectivity includes AP is configured with MIH services. Interworking capability element will be used in beacons.

Amongst the MIH capabilities advertised through this TGu GAS Capability IE would be indications about availability to Information Service, and Command /Event Services capability discovery..

### 11.11.3 MIH Event/Command Service Support

The IEEE 802.11 specification shall support link layer events and commands as specified in the 802.21 specification. For each link layer event or command, this may result in definition of a new primitive, or change in semantics of an existing primitive or just identification of an existing primitive with appropriate semantics in the 802.11 specification. The 802.11 specification shall support primitives and L2 transport to discover MIH event and command services remotely in State 1 phase.

### 11.11.4 MIH Information Services Support

The 802.11 specification shall support primitives to query values of different Information Elements at L2. The STA shall be able to use the Information Service and query specific IEs. The 802.11 specification shall provide a suitable L2 transport that allows the STA to query the values of different Information Elements in State 1 phase.

### 11.11.5 MIH Protocol Request

When GAS Protocol ID of MIH Information Service and/or GAS Protocol ID of MIH Command and Event Services Capability Discovery are/is included in GAS Capability Information element, , STA shall use GAS Request/Response action frames in order to query Information Elements of Information Service and/or to discover MIH capabilities of Event Service and/or Command Service. The MIH function of STA generates query MIH frames, such as Information Service query and ES and CS capability discovery query and delivers using MLME-GAS-Initial-Request.request primitive. The 802.11 MAC of STA generaes GAS-Initial-Request action frame including delivered query MIH frame in order to transmit query MIH frame remotely to AP.

### 11.11.6 MIH Protocol Indication (response)

AP will transmit either GAS-Initial-Response or GAS-Comeback-Response action frame , including corresponding query response MIH frame for STA’s query, to STA. When the 802.11 MAC of STA receives query response MIH frame from AP, the 802.11 MAC generates MLME-GAS-Initial-Response.confirm or MLME-GAS-Comeback-Response.confirm primitive including received query response MIH frame and delivers it to MIH function of STA.

# Annex A

## A3.1 Capability Field – Current state

For purpose of understanding, an Annex has been added to mention the current state of the Capabilities field.

## A3.2 MIHF Frame Format

Figure below shows the current state of MIHF Frame as defined in the 802.21 draft P802-21-D00-05. This is just for information purposes only.

## A.4 PICS proforma–IEEE Std. 802.11, 2006 Edition

### A.4.3 IUT configuration

Append this entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFu | Interworking supported | 11.11 | O | Yes, No, N/A |

Insert this new clause after clause A.4.16:

### A.4.16 Interworking extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Interworking capabilities supported? |  |  |  |
| IW1 | Advertisement support | 11.11.1 | CFu:M | Yes, No, N/A |

Add the following MIB attributes to Annex D:

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11AdvertisementServicesEnabled TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11AdvertisementServicesEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting either non-802.11 Network

Access services or non-802.11 specific protocol functionalities in support of seamless interworking.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }