IEEE P802.11
Wireless LANs

| TGp Comment Resolution – Clause 20 to Clause 17 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wayne Fisher | ARINC, Inc | 2551 Riva Road, Annapolis, MD 21401 | 410-266-4958 | wfisher@arinc.com |
| Carl Kain | Mitretek Systems | 3150 Fairview Park Drive South, Falls Church, VA 22043 | +1 703 610 1788 | ckain@mitretek.org |

Abstract

This document presents what is left in Clause 20 after addressing most of the comments received for Clause 20.  The purpose of this document is to discuss the remaining parts and discuss the deposition of these remaining subclauses.  This includes those parts relocated to Annex I and Annex J.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  |  |  |  | LB81 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
|  |  | 20 | Wayne Fisher, Carl Kain |  | July 18, 2006 |

IEEE P802.11p/D1.x, July 2006

# Amendment  :  Wireless Access in Vehicular Environments (WAVE)

This document presents the remaining parts of Clause 20 that will need to be addressed, revised, and relocated IF  Clause 20 is deleted from the WAVE draft amendment.

Insert new Clause 20 after Clause 19, renumbering as needed:

# 20. Wireless access in vehicular environments (WAVE) PHY specification

Deleted.   N/A

# 20.1 Overview

Deleted.   In CL 17.X.X  OR  N/A  OR Moved to “CL XX.X  WAVE Performance Requirements.

# 20.1.1 Scope

Deleted.   In CL 17.X.X  OR  N/A

# 20.1.2 OFDM PHY functions

Deleted.   In CL 17.X.X  OR  N/A

# 20.1.2.1 PLCP sublayer

Deleted.   In CL 17.X.X  OR  N/A

# 20.1.2.2 WAVE PMD sublayer

Deleted.   In CL 17.X.X  OR  N/A

# 20.1.2.3 PLME

Deleted.   In CL 17.X.X  OR  N/A

# 20.1.2.4 Service specification method

# Deleted.   In CL 17.X.X  OR  N/A

# 20.2 WAVE OFDM PHY specific service parameter list

# 20.2.1 Introduction

Deleted.   In CL 17.X.X  OR  N/A

# 20.2.2 WAVE TXVECTOR Parameters

Deleted.   In CL 17.X.X  OR  N/A

# 20.2.2.1 WAVE TXVECTOR LENGTH

Deleted.   In CL 17.X.X  OR  N/A

# 20.2.2.2 WAVE TXVECTOR DATARATE

Deleted.   In CL 17.X.X  OR  N/A

# 20.2.2.3 WAVE TXVECTOR SERVICE

Deleted.   In CL 17.X.X  OR  N/A

# 20.2.2.4 WAVE TXVECTOR TXPWR_LEVEL

TBD  -  Review.   Relocate to CL 17.X.X  ????

The allowed values for the TXPWR_LEVEL parameter are in the range of 1 to 64.  This parameter  shall be used to indicate which of the available TxPowerLevel attributes defined in the MIB shall be used for the current transmission.

# 20.2.3 WAVE RXVECTOR parameters

Deleted.   In CL 17.X.X  OR  N/A  [ WRSS deleted, reverts to RCPI. ]

# 20.2.3.1 WAVE RXVECTOR LENGTH

Deleted.   In CL 17.X.X

# RXVECTOR RSSI

Deleted.   In CL 17.X.X

# 20.2.3.3 WAVE DATARATE

Deleted.   In CL 17.X.X

# 20.2.3.4 WAVE SERVICE

Deleted.   In CL 17.X.X

# 20.3 OFDM PLCP sublayer

Deleted.   In CL 17.X.X

# 20.3.1 Introduction

Deleted.   In CL 17.X.X

# 20.3.2 PLCP frame format

Deleted.   In CL 17.X.X

# 20.3.2.1 Overview of the PPDU encoding process

# Deleted.   In CL 17.X.X

# 20.3.2.2 WAVE Modulation-dependent parameters

Deleted.   In CL 17.X.X

# 20.3.2.3 WAVE Timing related parameters

Deleted.   In CL 17.X.X

# 20.3.2.4 Mathematical conventions in the signal descriptions

Deleted.   In CL 17.X.X

# 20.3.2.5 Discrete time implementation considerations

Deleted.   In CL 17.X.X

# 20.3.3 WAVE PLCP preamble (SYNC)

Deleted.   In CL 17.X.X

# 20.3.4 WAVE SIGNAL field

Deleted.   In CL 17.X.X

# 20.3.4.1 WAVE RATE field

Deleted.   In CL 17.X.X

# 20.3.4.2 PLCP LENGTH field

Deleted.   In CL 17.X.X

# 20.3.4.3 Parity (P), Reserved (R), and SIGNAL TAIL fields

Deleted.   In CL 17.X.X

# 20.3.5 DATA field

Deleted.   In CL 17.X.X

# 20.3.6 CCA

Deleted.   In CL 17.X.X

# 20.3.7 WAVE PLCP data modulation and modulation rate change

Deleted.   In CL 17.X.X

# 20.3.8 WAVE PMD operating specifications

Deleted.   In CL 17.X.X

# 20.3.8.1 WAVE Outline description

Deleted.   In CL 17.X.X

# 20.3.8.2 WAVE Regulatory requirements

Revised, Relocated to Annex I.

# 20.3.8.3 Operating channel frequencies

# 20.3.8.3.1 Operating frequency range

Deleted.   In CL 17.X.X

# 20.3.8.3.2 Channel numbering

Deleted.   In CL 17.X.X

# 20.3.8.3.3 WAVE channelization

# Revised, Relocated to Annex J.

# 20.3.8.4 WAVE transmit and receive in-band and out-of-band spurious emissions

# Revised, Relocated to Annex J.

# 20.3.8.5 WAVE TX RF delay

Deleted.   In CL 17.X.X

# 20.3.8.6 WAVE slot time

TBD, Review, Incorporate into CL 17.3.8.6 ????

See Clause 17.3.8.6  Slot time.  In addition, for WAVE the Slot Time shall be increased to 16 µs.

# 20.3.8.7 WAVE transmit and receive antenna requirements

TBD, Review, Incorporate into CL 17.3.8.7 ????

The WAVE transmit and receive antenna port(s) impedance shall be 50  if the port is exposed. The transmit and receive antennas shall be either right hand circularly or vertically polarized.

# 20.3.8.8 WAVE transmit and receive operating temperature range

TBD, Review, Incorporate into CL 17.3.8.8 ????

For WAVE operations, a fourth temperature range, Type 4, defined as from –40 to +85°C, is designated for automotive and outdoor environments. See Clause 17.3.8.8 for the Type 1, Type 2, and Type 3 temperature environments.

# 20.3.9 PMD Transmit Specifications

Deleted.   In CL 17.X.X

# 20.3.9.1 WAVE Transmit Power Levels

Revised, Relocated to Annex I.

# 20.3.9.2 WAVE transmit spectrum mask

Revised, Relocated to Annex I.

# 20.3.9.3 WAVE transmission spurious

Deleted.   In CL 17.X.X

# 20.3.9.4 WAVE transmit center frequency tolerance

Revised, Relocated to CL 17.3.9.4.

# 20.3.9.5 WAVE symbol clock frequency tolerance

Revised, Relocated to CL 17.3.9.5.

# 20.3.9.6 WAVE modulation accuracy

Deleted.   In CL 17.X.X

# 20.3.9.6.1 WAVE transmitter center frequency leakage

Deleted.   In CL 17.X.X

# 20.3.9.6.2 WAVE transmitter spectral flatness

Deleted.   In CL 17.X.X

# 20.3.9.6.3 WAVE transmitter constellation error

Deleted.   In CL 17.X.X

# 20.3.9.7 WAVE transmit modulation accuracy test

Deleted.   In CL 17.X.X

# 20.3.10 WAVE PMD receiver specifications

TBD, Review, Incorporate into CL 17.3.10 ????  [Note, Most is the same as CL 17.3.10  except   the addition of Category 2 RX Performance ]

The WAVE receive specifications associated with the PMD sublayer are described in 20.3.10.1 through 20.3.10.5.  Two categories of receivers are allowed for stations.  They are designated Category 1 and Category 2.  The Category 2 stations will have greater out-of-channel rejection to provide better system performance in a noisy signal environment.

# 20.3.10.1 WAVE receiver minimum input sensitivity

The packet error rate (PER) shall be less than 10 % at a PSDU length of 1000 bytes for rate-dependent input levels. The WAVE receiver minimum sensitivity levels shall be less than or equal to the numbers listed in Table p12. The minimum input levels are measured at the antenna connector (Noise figure of 10 dB and 5 dB implementation margins are assumed).

Table p12—WAVE Category 1 receiver performance requirements

| Data rate (Mbit/s) | Minimum sensitivity (dBm) 1 | Adjacent channel rejection (dB) | Nonadjacent channel rejection (dB) |
| --- | --- | --- | --- |
| 3 | -85 | 18 | 34 |
| 4.5 | -84 | 17 | 33 |
| 6 | -82 | 16 | 32 |
| 9 | -80 | 15 | 31 |
| 12 | -77 | 13 | 29 |
| 18 | -73 | 11 | 27 |
| 24 | -69 | 8 | 24 |
| 27 | -68 | 4 | 20 |

1  There is a 3 dB decrease in sensitivity when changing to 20 MHz channel operation.

# 20.3.10.2 WAVE adjacent channel rejection

The adjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate-dependent sensitivity specified in Table p12 and Table p13 and raising the power of the interfering signal, until 10 % PER is caused for a PSDU length of 1000 bytes. The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection. The interfering signal in the adjacent channel shall be an OFDM signal conforming to a Class A spectral mask, unsynchronized with the signal in the channel under test. For a compliant OFDM PHY, the corresponding rejection shall be no less than specified in Table p12 for a Category 1 station and Table p13 for a Category 2 station.

# 20.3.10.3 WAVE nonadjacent channel rejection

The nonadjacent channel rejection shall be measured by setting the desired signal's strength 3 dB above the rate-dependent sensitivity specified in Table p12 or Table p13  for the Category of station being tested, and raising the power of the interfering signal until a 10 % PER occurs for a PSDU length of 1000 bytes. The power difference between the interfering and the desired channel is the corresponding nonadjacent channel rejection. The interfering signal in the nonadjacent channel shall be an OFDM signal conforming to a Class A spectral mask, unsynchronized with the signal in the channel under test. For a conformed OFDM PHY, the corresponding rejection shall be no less than specified in Table p12 for a Category 1 station and Table p13 for a Category 2 station.

.

Table p13—WAVE Category 2 receiver performance requirements

| Data rate (Mbit/s) | Minimum sensitivity (dBm)1 | Adjacent channel rejection (dB) | Nonadjacent channel rejection (dB) |
| --- | --- | --- | --- |
| 3 | -85 | 37 | 44 |
| 4.5 | -84 | 36 | 43 |
| 6 | -82 | 35 | 42 |
| 9 | -80 | 34 | 41 |
| 12 | -77 | 32 | 39 |
| 18 | -73 | 30 | 37 |
| 24 | -69 | 27 | 34 |
| 27 | -68 | 23 | 30 |

1  There is a 3 dB decrease in sensitivity when changing to 20 MHz channel operation.

# 20.3.10.4 WAVE receiver maximum input level

Deleted.   In CL 17.X.X

# 20.3.10.5 WAVE CCA sensitivity

Deleted.   In CL 17.X.X

# 20.3.10.6 WAVE Performance in vehicular environments

TBD, Review, Incorporate into CL 17.3.10 ????  [ AND include WAVE Performance requirements defined in 20.1, para 5.]

Stations using WAVE functions are intended to be capable of operating in a mobile vehicular environment which consists of multiple classes of streets, roads, and highways in rural, suburban, and city settings. This environment imposes a continuously varying degree of multi-path propagation comprising both slow and fast fading, amplitude variations and signal drop-outs. WAVE stations shall be capable of operating in accordance with the channel models defined in Annex Q.  The channel models in Annex Q have been determined through actual data collection and model validation. These models prescribe the number of taps, and for each tap, a specified excess delay, a power-density spectrum (with a certain maximum Doppler shift), and a Rician channel K-factor.  Packet error rates shall be less than 10 % as specified in the Channel Model in Annex Q. For this verification the signal input level shall be 10 dB above the receiver sensitivities as specified in Table p12.

# 20.3.11 Transmit PLCP

Deleted.   In CL 17.X.X

# 20.3.12 Receive PLCP

Deleted.   In CL 17.X.X

# 20.4 WAVE OFDM PLME

# 20.4.1 WAVE PLME_SAP sublayer management primitives

TBD, Review, Incorporate into CL 17.4.1 ????

Table p14 lists the MIB attributes including those added for WAVE that may be accessed by the PHY entities and the intralayer of higher level LMEs.  These attributes shall be accessed via the PLME-GET, PLME-SET, PLME-RESET, and PLME-CHARACTERISTICS primitives for U-NII and WAVE band operations, as defined in Clause 10.4.

# 20.4.2 WAVE OFDM PHY management information base

All OFDM PHY management information base attributes are defined in Clause 13, with specific values for WAVE defined in Table p14. The column entitled “Operational semantics” in Table p14 contains two types: static and dynamic. Static MIB attributes are fixed and cannot be modified for a given PHY implementation. Dynamic MIB attributes can be modified by some management entity.

NOTE: Stations intended for use in one specific regulatory domain may implement relevant  static MIB values.

Table p14—MIB attribute default values/ranges

| Managed Object | Default Value/Range | Operational Semantics |
| --- | --- | --- |
| dot11 PHY Operation Table |  |  |
| dot11 PHY type | OFDM-5. (04) | static |
| dot11 current reg domain | implementation dependent | dynamic |
| dot11 current frequency band | implementation dependent | dynamic |
| dot11 temp type | implementation dependent | static |
| dot11 PHY Antenna Table |  |  |
| dot11 current Tx antenna | implementation dependent | dynamic |
| dot11 diversity support | implementation dependent | dynamic |
| dot11 current Rx antenna | implementation dependent | dynamic |
| dot11 PHY Tx Power Table |  |  |
| dot11 number supported power levels | implementation dependent | static |
| dot11 Tx power Level 1 | implementation dependent | static |
| dot11 Tx power Level 2 | implementation dependent | static |
| dot11 Tx power Level 3 | implementation dependent | static |
| dot11 Tx power Level 4 | implementation dependent | static |
| dot11 Tx power Level 5 | implementation dependent | static |
| dot11 Tx power Level 6 | implementation dependent | static |
| dot11 Tx power Level 7 | implementation dependent | static |
| dot11 Tx power Level 8 | implementation dependent | static |
| dot11 Tx power Level 9 | implementation dependent | static |
| dot11 Tx power Level 10 | implementation dependent | static |
| dot11 Tx power Level 11 | implementation dependent | static |
| dot11 Tx power Level 12 | implementation dependent | static |
| dot11 Tx power Level 13 | implementation dependent | static |
| dot11 Tx power Level 14 | implementation dependent | static |
| dot11 Tx power Level 15 | implementation dependent | static |
| dot11 Tx power Level 16 | implementation dependent | static |
| dot11 Tx power Level 17 | implementation dependent | static |
| dot11 Tx power Level 18 | implementation dependent | static |
| dot11 Tx power Level 19 | implementation dependent | static |
| dot11 Tx power Level 20 | implementation dependent | static |
| dot11 Tx power Level 21 | implementation dependent | static |
| dot11 Tx power Level 22 | implementation dependent | static |
| dot11 Tx power Level 23 | implementation dependent | static |
| dot11 Tx power Level 24 | implementation dependent | static |
| dot11 Tx power Level 25 | implementation dependent | static |
| dot11 Tx power Level 26 | implementation dependent | static |
| dot11 Tx power Level 27 | implementation dependent | static |
| dot11 Tx power Level 28 | implementation dependent | static |
| dot11 Tx power Level 29 | implementation dependent | static |
| dot11 Tx power Level 30 | implementation dependent | static |
| dot11 Tx power Level 31 | implementation dependent | static |
| dot11 Tx power Level 32 | implementation dependent | static |
| dot11 Tx power Level 33 | implementation dependent | static |
| dot11 Tx power Level 34 | implementation dependent | static |
| dot11 Tx power Level 35 | implementation dependent | static |
| dot11 Tx power Level 36 | implementation dependent | static |
| dot11 Tx power Level 37 | implementation dependent | static |
| dot11 Tx power Level 38 | implementation dependent | static |
| dot11 Tx power Level 39 | implementation dependent | static |
| dot11 Tx power Level 40 | implementation dependent | static |
| dot11 Tx power Level 41 | implementation dependent | static |
| dot11 Tx power Level 42 | implementation dependent | static |
| dot11 Tx power Level 43 | implementation dependent | static |
| dot11 Tx power Level 44 | implementation dependent | static |
| dot11 Tx power Level 45 | implementation dependent | static |
| dot11 Tx power Level 46 | implementation dependent | static |
| dot11 Tx power Level 47 | implementation dependent | static |
| dot11 Tx power Level 48 | implementation dependent | static |
| dot11 Tx power Level 49 | implementation dependent | static |
| dot11 Tx power Level 50 | implementation dependent | static |
| dot11 Tx power Level 51 | implementation dependent | static |
| dot11 Tx power Level 52 | implementation dependent | static |
| dot11 Tx power Level 53 | implementation dependent | static |
| dot11 Tx power Level 54 | implementation dependent | static |
| dot11 Tx power Level 55 | implementation dependent | static |
| dot11 Tx power Level 56 | implementation dependent | static |
| dot11 Tx power Level 57 | implementation dependent | static |
| dot11 Tx power Level 58 | implementation dependent | static |
| dot11 Tx power Level 59 | implementation dependent | static |
| dot11 Tx power Level 60 | implementation dependent | static |
| dot11 Tx power Level 61 | implementation dependent | static |
| dot11 Tx power Level 62 | implementation dependent | static |
| dot11 Tx power Level 63 | implementation dependent | static |
| dot11 Tx power Level 64 | implementation dependent | static |
| dot11 current Tx power level | implementation dependent | dynamic |
| dot11 Reg Domains supported Table |  |  |
| dot11 reg domains supported | implementation dependent | static |
| dot11 frequency bands supported | implementation dependent | dynamic |
| dot11 PHY Antennas List Table |  |  |
| dot 11 supported Tx antenna | implementation dependent | dynamic |
| dot11 supported Rx antenna | implementation dependent | dynamic |
| dot 11 diversity selection Rx | implementation dependent | dynamic |
| dot11 supported Data Rates Tx Table |  |  |
| dot11 supported rates Tx value | 3, 4.5, 6, 9, 12, 18, 24, 27, 36, 48, and 54 Mbit/s Mandatory Rates for WAVE: 3, 6, and 12 | static |
| dot11supportedDataRatesRxTable |  |  |
| dot11 supported rates Rx value | 3, 4.5, 6, 9, 12, 18, 24, 27, 36, 48, and 54 Mbit/s Mandatory Rates for WAVE: 3, 6, and 12 | static |
| dot11 PHY OFDM Table |  |  |
| dot11 current frequency | implementation dependent | dynamic |
| dot11 TI threshold | implementation dependent | dynamic |
| dot11 station class | implementation dependent | dynamic |
| dot11 ACR type | implementation dependent | dynamic |

The MIB attribute dot11StationClass is used to describe the specified spectrum mask. The MIB attribute dot11ACRType is used to describe the adjacent channel rejection performance category.

# 20.4.3 OFDM TXTIME calculation

Deleted.   In CL 17.X.X

# 20.4.4 WAVE OFDM PHY characteristics

TBD, Review, Incorporate into CL 17.4.4 ????  [ CH Sw Time, Air Propagation Time, and Slot time need to be addressed.]

The static WAVE OFDM PHY characteristics, provided through the PLME-CHARACTERISTICS service primitive, are shown in Table p15.  The definitions for these characteristics are given in Clause 10.4. The WAVE channel switching time (aChSwitchTime) of ≤ 2 TU has been added to the table. Note, for WAVE implementation the value for aAirPropagationTime is <4 us, not << 1us, because of the longer distances that can be used for WAVE communications. For WAVE the Slot Time shall also be increased to 16 us.

Table p15—WAVE OFDM PHY characteristics

| Characteristics | Values for WAVE |
| --- | --- |
| aSlotTime | 16 s |
| aSIFSTime | 32 s |
| aCCATime | < 8 s |
| aPHY-RX-START-Delay | 48 s |
| aRxTxTurnaroundTime | < 2 s |
| aTxPLCPDelay | Implementation dependent |
| aRxPLCPDelay | Implementation dependent |
| aRxTxSwitchTime | << 1 s |
| aCHSwitchTime | ≤ 2 TU |
| aTxRampOnTime | Implementation dependent |
| aTxRampOffTime | Implementation dependent |
| aTxRFDelay | Implementation dependent |
| aRxRFDelay | Implementation dependent |
| aAirPropagationTime | < 4 s |
| aMACProcessing Delay | < 2 s |
| aPreambleLength | 32 s |
| aPLCPHeaderLength | 8 s |
| aMPDUMaxLength | 4095 |
| aCWmin | 15 |
| aCWmax | 1023 |

# 20.5 OFDM PMD sublayer

Deleted.   In CL 17.X.X

# Annex I

(informative)

# Regulatory classes

## I.1 External regulatory references

In Table I.1- Regulatory requirement list, Add to the list of documents for the United States as shown:

Table I.1—Regulatory requirement list

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| Japan | Ministry of Internal Affairs and Communications (MIC) | MIC Equipment Ordinance (EO) for Regulating Radio Equipment Articles 7, 49.20, 49.21a | MIC |
| United States | Federal Communications Commission (FCC) | FCC CFR47 [B8], Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401–15.407, Section 90.210, Section 90.1201-90.1217 Section 90.371-383, Section 95.639, Section 95.1501-1511 | FCC |
| Europe | Europe European Conference of Postal and Telecommunications (CEPT) Administrations and its Electronic Communications Committee (ECC). Also, European Radiocommunications Office, European Telecommunications Standards Institute | ECC DEC (04) 08, ETSI EN301 893 | CEPT |

In Table I.2- Emissions limits sets, Add to the Emissions limits sets for WAVE as shown:

Table I.2—Emissions limits sets

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47 [B8], Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 interference-limited areas | Reserved | Reserved | MIC EO Article 49.21 |
| 3 other interference areas | Reserved | Reserved | MIC EO Article 49.21 |
| 4 | FCC CFR47 [B8], Section 15.247 | Reserved | Reserved |
| 5 public safety | FCC CFR47 [B8], Section 90.210 | Reserved | Reserved |
| 6 WAVE Mode Class A | FCC CFR 47, Sections 90.375, 90.377 ; FCC CFR 47, Sections 90.1511, 90.639 | Reserved | Reserved |
| 7 WAVE Mode Class B | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 8 WAVE Mode Class C | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 9 WAVE Mode Class D | FCC CFR 47, Sections 90.375, 90.377 and 95.1511 | Reserved | Reserved |
| 10-255 | Reserved | Reserved | Reserved |

In Table I.3- Behavior limits sets, Add to the Behavior limits sets for WAVE as shown:

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 10 Wave Mode RSU/OBU | FCC CFR 47, Sections 90.375, 90.377, 90.379, 90.383, and 95.1511 | Reserved | Reserved |
| 10-255 | Reserved | Reserved | Reserved |

# Annex J

(normative)

# Country information element and regulatory classes

Delete the last row (starting with 12 – 255) in Table J.1.

Insert the following statements after Table J.1.:

The regulatory classes specified for the 5.85 – 5.925 GHz band in the USA are enumerated in Table J.1A.  Regulatory classes 12 and 13 apply to both government (state/local) and non-government fixed and mobile station operations.  Regulatory class 14 applies to government fixed and mobile station operations.  Regulatory class 15 handles the corresponding non-government operations on this channel.  Regulatory class 16 applies to both government and non-government operations for fixed stations on channels 180 – 182 and class 17 handles operations on the same channels for mobile operations.  Regulatory class 18 applies to government fixed and mobile station operations on channel 184 and regulatory class 19 applies to non-government fixed and mobile station operations on this channel.

Add Table J.1A below:

Table J.1A—5.9 GHz USA Regulatory classes

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Max EIRP (dBm) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 5.860 | 10 | 172, 174, 176 | 28.8 | 33.0 | 6-9 | 10 |
| 13 | 5.875 | 10 | 175 | 10.0 | 23.0 | 6-9 | 10 |
| 14 | 5.890 | 10 | 178 | 28.8 | 44.8 | 6-9 | 10 |
| 15 | 5.890 | 10 | 178 | 28.8 | 33.0 | 6-8 | 10 |
| 16 | 5.900 | 10 | 180, 181, 182 | 10.0 | 23.0 | 6-9 | 10 |
| 17 | 5.900 | 10 | 180, 181, 182 | 20.0 | 23.0 | 6-9 | 10 |
| 18 | 5.920 | 10 | 184 | 28.8 | 40.0 | 6-9 | 10 |
| 19 | 5.920 | 10 | 184 | 28.8 | 33.0 | 6-9 | 10 |
| 20-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |