IEEE P802.11
Wireless LANs

| LB84-CID-873-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 21-09-2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Amit Bansal | Wipro-Newlogic | Electronic City, Bangalore, India | +91 80 30295128 | amit.bansal@wipro.com |
| Shravan Surineni | Qualcomm | 9, Damonmill Square Suite 2A, Concord, MA 01742 | +1 781 276 0926 | shravans@qualcomm.com |

Abstract

This document includes proposed text for modification of the TGn 1.03 draft. The text is intended to provide a solution to the issue raised by LB84 Comment ID (CID) 873 regarding clarity of RDG.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Interpretation of a Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction and headings are not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

Notes: R2 was presented to the group and the following two comments are being fixed in this revision.

1. Update the figure to show all frames have Imm-ACK.

2. Show a figure of an  aggregation of a BA with a data frame.

TGn Editor: Replace the figure in subclause 9.14.1, on page 98, lines 25-45 of TGn Draft 1.03 with the following:

References:

| CID | Name of Commenter(Ed) | Clause Number(C) | Page(Ed) | Line(Ed) | Clause Title(Ed) | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 873 | Emeott, Stephen | 9.19.1 | 98 | 48 | RD Exchange Sequences | Figure n39 is only tells part of the RDG strory by illustrating a single PPDU in the DLT including a BAR. It would be more informative to show an additional PPDU preceding the PPDU with the BAR, and for the additional PPDU to show that RDG=0. Also, what should More PPDU be for the additional PPDU and the PPDU already shown in the RDG figure? | Refine the figure and make it more readable and clear |