IEEE P802.11
Wireless LANs

| Management Frame Field for BIP MIC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2006-07-19 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Company | Address | Phone |  | email |
| Kevin Hayes | Atheros Communications | 5480 Great America Parkway, Santa Clara, Ca 95054 | 408-773-5275 | kevin@atheros.com |  |

Abstract

This submission contains instructions to the TGw technical editor in order to modify the layout of the mgmt frame IE, used with BIP..

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background:

When BIP is used in a hetergenous BSS of TGw-compliant STAs and non-compliant (legacy) STAs, broadcast management frames must be used that will be understandable by both kinds of clients.  Also, the MIC field should be the penultimate field in the frame, before the FCS.

We introduce a new container for the MIC such that legacy STAs will be able to perform general IE processing on the field.

## Detailed Editorial Instructions:

Change the clause 7.3.2.27 as follows:

7.3.2.27 Management MIC information element (MMIE) field

The Management MIC information element field (MMIE) (MMF) is used to protect broadcast management frames from forgery and replay. It provides message integrity for broadcast Deauthenticate and Disassociate Frames.

Figure 80a depicts the MMIE MMF format.

| (ANA value) | Length | Key ID | Replay | MIC |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 6 or 16 | 8 |

The value of the Element ID field shall be {edNOTE : TBD} ANA value shall be assigned by the Assigned Number Authority.

The MMF shall be inserted immediately preceding the FCS of the MMPDU.

The Length field denotes the number of octets in the information element MMF and shall have a value of either 16 or 26. Length 26 shall be used with protected Broadcast Disassociate frames and protected Broadcast Deauthenticate frames. Length 16 shall be used with other protected broadcast management frames.

Change Figure 104a to be:

| IEEE 802.11 Header | Management Frame Body | Management MIC IE Field | FCS |
| --- | --- | --- | --- |

Change clause 8.3.4.4 as follows:

BIP uses a different replay protection scheme, depending on the management frame type and format of the

MMIE MMF.

Change clause 8.3.4.4.1 as follows:

8.3.4.4.1 MMIE Replay field as sequence number

When the MMIE MMF Length field is 16, the MMIE MMF Replay field represents a sequence number. An MMIE MMF of length 16 shall only be used with broadcast action management frames.

In this case the transmitter shall insert a monotonically increasing value into the MMIE MMF Replay field. The receiver shall maintain a 48-bit replay counter for each IGTK. The replay counter shall be set to the value provided by the Authenticator. When a broadcast protected management frame subsequently arrives with an MMIE MMF Length field of 16, the receiver shall interpret the MMIE MMF Replay field as a 48 bit integer. It shall then compare this integer value against the replay counter for the IGTK identified by the MMIE MMF Key ID field. If the integer value from the received MMIE MMF Replay Field is less than or equal to the replay counter value for the IGTK, the receiver shall silently discard the frame and increment a MIB counter.

Change clause 8.3.4.4.2 as follows:

8.3.4.4.2 MMIE Replay field as current DGTK

When the MMIE MMF Length field is 26, the MMIE MMF Replay field contains the current DGTK of its sender. An MMIE MMF of length 26 shall only be used with broadcast disassociation or deauthentication frames.

In this case, the transmitter shall insert the DGTK into the MMIE MMF Replay field. Each receiver shall

maintain its own 128-bit Disconnect Hash Value (DHV). The transmitter distributes the DHV when the

pairwise security association is established. The DHV is derived by the transmitter as described in

Section 8.5.1.4.

The receiver identifies the DGTK and DHV using the transmitter's MAC address.

Upon receipt of a protected broadcast Disassociate or Deauthenticate management frame, indicated by an

MMIE MMF Length field of 26, the receiver shall interpret the MMIE MMF Replay field value as the DGTK. To determine if the received protected broadcast frame is a replay or forgery, the receiver shall compute

Change clause 8.3.4.5 as follows:

When a STA wishes to transmit a protected broadcast management frame it shall:

a) Select the appropriate key (IGTK or DGTK) for the frame and construct the MMIE MMF with the

MIC field muted to zero. Clause 7.3.2.26 defines the MMIE MMF.

b) Compute AAD for this management frame, as specified in 8.3.4.3.

c) Compute AES-CMAC-64 over the concatenation of (AAD||Management Frame Body||MMIE MMF)

d) Insert the left-most 64-bits of the result into the MMIE MMF MIC field.

e) Compose the broadcast protected management frame as the IEEE 802.11 header, Management

Frame Body, and MMIE MMF

f) Transmit the broadcast management frame.

Change clause 8.3.4.6 as follows:

When a STA receives a broadcast management frame protected by BIP, it shall

a) Identify the appropriate key and associated state based on the MMIE MMF Length field.

b) Execute the appropriate replay filtering scheme defined in 8.3.4.4.

c) If the replay filtering scheme succeeds, the receiver shall compute AAD for this management frame, as specified in 8.3.4.3 and compute the AES-CMAC-64 over the concatenation of

(AAD|| Management Frame Body|| MMIE MMF)

If the left-most 64-bits of the result do not match the value in the MMIE MMF, then the receiver shall silently discard the frame and increment a MIB counter {ednote: TBD}.