IEEE P802.11
Wireless LANs

| Signal and Pilot Definition for STBC Encoded MCS 32 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-07-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Deric Waters | Texas Instruments |  |  | deric@ti.com |
| Anuj Batra | Texas Instruments |  |  | batra@ti.com |
| Marc De Courville | Motorola |  |  | Marc.de.Courville@motorola.com |
| Stéphanie Rouquette-Léveil | Motorola |  |  | stephanie.rouquette@motorola.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 201

The changes marked in this document are based on TGn Draft version 1.0.

The signal definition of the HT-duplicate mode implicitly assumes that there will only be one space-time stream. However, the HT-duplicate can also be used in conjunction with STBC in which case there are two space-time streams. Therefore, the signal definition of the HT-duplicate mode in draft 1.0 of the IEEE 802.11n standard is inadequate. Specifically, the pilot sequence for the HT-Duplicate STBC mode is undefined, and the scaling is incorrect.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID 201

Issue: In Section 20.3.4.7.4 (D1.0), it states that the HT-duplicate mode can only be used with one spatial stream, but equation 20-53 is written as if the HT-duplicate mode can only be used with one space-time stream. Section 20.3.4.5.1 (D1.0) clearly states that the HT-duplicate mode can be encoded using STBC in which case there are two space-time streams. To resolve this inconsistency, the signal definition of the HT-duplicate mode in Section 20.3.4.7.4 (D1.0) needs to be updated to allow for two space-time streams; in particular, the pilot sequence for the second space-time streams needs to be defined in Section 20.3.4.6 (D1.0) and the scaling needs to be corrected.

Motion: Move to adopt the solution as described in the following.

- 20.3.4.6 Pilot Subcarriers

In the case of 20MHz transmissions 4 pilot tones are inserted, pilot signals are inserted in the same sub-carriers used in clause 17, i.e. in sub-carriers -21, -7, 7 and 21; the pilot sequence for the nth symbols and iSTSth space time stream is defined as follows:

(20-1)

TGn Editor: modify the following sentence as indicated.

In the case of 40MHz transmissions (excluding besides MCS 32) pilot signals are inserted in sub-carriers -53, -25, -11, 11, 25, 53; the pilot sequence for the nth symbols and iSTSth space time stream is defined as follows:

(20-2)

TGn Editor: insert the following sentence and equation.

For MCS = 32, the pilot sequence for the nth symbols and iSTSth space time stream is defined from the sequence:

where

indicates symbol number modulo 4;

Note that for each space time stream there is a different pilot pattern and the pilot patterns are cyclically rotated over symbols.

The basic patterns are also different according to the total number of space time streams for the packet.

TGn Editor: change the following sentence as indicated.

The patterns  are defined in the following tables  The first table (Table n1) defines the values for 20MHz transmissiond the second table (Table n2) defines the values for 40MHz transmissions besides(excluding MCS 32) HT-duplicate mode, and the third table (Table n77a) defines the values for the HT-duplicate modeMCS 32.

- Table n1 - pilot values for 20MHz transmission.

| NSTS | iSTS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | -1 |
| 2 | 0 | 1 | 1 | -1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 |
| 3 | 0 | 1 | 1 | -1 | -1 |
| 3 | 1 | 1 | -1 | 1 | -1 |
| 3 | 2 | -1 | 1 | 1 | -1 |
| 4 | 0 | 1 | 1 | 1 | -1 |
| 4 | 1 | 1 | 1 | -1 | 1 |
| 4 | 2 | 1 | -1 | 1 | 1 |
| 4 | 3 | -1 | 1 | 1 | 1 |

TGn Editor: change the following sentence as indicated.

- Table n2 - Pilots values for 40MHz transmission (excluding MCS 32).

| NSTS | iSTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 | -1 | -1 | 1 |
| 2 | 0 | 1 | 1 | -1 | -1 | -1 | -1 |
| 2 | 1 | 1 | 1 | 1 | -1 | 1 | 1 |
| 3 | 0 | 1 | 1 | -1 | -1 | -1 | -1 |
| 3 | 1 | 1 | 1 | 1 | -1 | 1 | 1 |
| 3 | 2 | 1 | -1 | 1 | -1 | -1 | 1 |
| 4 | 0 | 1 | 1 | -1 | -1 | -1 | -1 |
| 4 | 1 | 1 | 1 | 1 | -1 | 1 | 1 |
| 4 | 2 | 1 | -1 | 1 | -1 | -1 | 1 |
| 4 | 3 | -1 | 1 | 1 | 1 | -1 | 1 |

TGn Editor: insert the following Table.

Table n77a – Pilot Values for HT duplicate modeMCS 32.

| 1 | 0 | 1 | 1 | 1 | -1 |
| --- | --- | --- | --- | --- | --- |
| 2 | 0 | 1 | 1 | -1 | -1 |
| 2 | 1 | 1 | -1 | -1 | 1 |

20.3.4.7.4

Transmission in HT duplicate mode.

HT duplicate mode provides the lowest transmission rate in 40MHz.  It shall only be used for one spatial stream and only with BPSK coding and rate ½ coding.

Under HT duplicate mode, the following equation defines the signal:

TGn Editor: insert the equation in place of the current equation

rHT-DATAiTXt=1NSTSNDuplicateTonesn=0NSYM−1wTSYMt-nTSYM

(k=047iSTS=1NSTSQMk−32iTX,iSTSdk,iSTS,nexpj2πMk−32ΔFt−nTSYM−TGI−TCSiSTS+

jk=047iSTS=1NSTSQMk+32iTX,iSTSdk,iSTS,nexpj2πMk+32ΔFt−nTSYM−TGI−TCSiSTS+

(20-3)

where

z is defined in Error: Reference source not found (Ed: Error: Reference source not found ).

TGn Editor: modify the following sentence as indicated.

M(k) is defined in 17.3.5.9.

NSR has the value defined for non-HT 20MHz transmission.

is an element from a vector of length , which may be frequency dependent.

TGn Editor: insert the following:is defined in 20.3.4.6.

The rules of spatial expansion CSD limitation, as specified in )Error: Reference source not found, shall apply to