IEEE P802.11
Wireless LANs

| TGp Comment Resolution Clause 17 Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wayne Fisher | ARINC, Inc | 2551 Riva Road, Annapolis, MD 21401 | 410-266-4958 | wfisher@arinc.com |

Abstract

In response to the many comments received on Clause 20 of P802.11p D1.0 this document presents a draft of changes to Clause 17 in place of the information that was contained in Clause 20.  Since so much of Clause 20 has been included in Clause 17 already by other task group amendments or has been relocated to an Annex or has been deleted it is reasonable to create changes and additions to the latest Clause 17 instead of maintaing a separate clause.  This document presents a draft of these changes and is provided for discussion during the TGp teleconferences.

Note, this draft of changes to Clause 17 include the results of Comment Resolutions already addressed by documents:  802.11-06/  0627r0, 0628r1, 0735r2, 0874r0, 0876r1, 0878r0, 1038r0, and the motions agreed to during the formal TGp sessions at the 802.11 WG meetings.

Rev 2 Updates from 8/10/06 teleconference include: Keep 64 levels of TXPWR.  Update (mostly remove) references of Slot Time, aAirPropagation Time, and aChSwitchTime. Update Table p12 and Table 152. Note, some items/issues still need to be verified and finalized.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  |  |  |  | LB81 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| Multiple | Multiple | 20. | Wayne Fisher |  | July 27, 2006 |

- COMMENT:  [From Spreadsheet]

In response to all comments received for Clause 20.

- Commenter’s Suggested Remedy (If appropriate):  [From Spreadsheet]

Not included here. (Not directly applicable.) Note, many comments were incorporated into the draft for revisions to Clause 17.

- Background, Explanation, Discussion, etc.:

In response to the many comments received on Clause 20 of P802.11p D1.0 section 4 presents a draft of changes to Clause 17 in place of the information that was contained in Clause 20.  Since so much of Clause 20 has been included in Clause 17 already by other task group amendments or has been relocated to an Annex or has been deleted it is reasonable to create changes and additions to the latest Clause 17 instead of maintaing a separate clause.

In comparing the transmit spectrum masks and receiver rejection requirements for WAVE vs the present requirements in 802.11ma, please note that the higher power transmit capabilities of WAVE devices operating in a highly mobile environment require deeper skirts on the transmit spectrum masks (being relocated to Annex I) and the additional receiver rejection requirements being added to CL 17.3.10. Category 1 receiver rejection requirements are mandatory; Category 2 is optional.

Table 152, MIB attribute default values/ranges, has been expanded to include 64 levels of Tx Power levels. (There may be a way to reduce the size of this table by just stating there are 64 levels rather than 64 individual entries.)

- Recommended Resolution of the Comment:

A draft of changes/additions to Clause 17 is provided:

# 17. Orthogonal frequency division multiplexing (OFDM) PHY specification

# for the 5 GHz band

# 17.2.2.4 TXVECTOR TXPWR_LEVEL

Add the following statement to Clause 17.2.2.4 as shown:

***JPM to verify: ***

The allowed values for the TXPWR_LEVEL parameter are in the range from 1 to 8. For WAVE, the allowed values for the TXPWR_LEVEL parameter are in the range of 1 to 64.  This parameter is used to indicate which of the available TxPowerLevel attributes defined in the MIB shall be used for the current transmission.

# 17.3.8.6 Slot time

Add the following statement to Clause 17.3.8.6 as shown:

***JPM to verify: ***

The slot time for the OFDM PHY shall be 9 μs for 20 MHz channel spacing, and shall be 13 µs for 10 MHz channel spacing, and shall be 21 µs for 5 MHz channel spacing.  In addition, for WAVE the Slot Time shall be increased to 16 µs.

# 17.3.8.8 Transmit and receive operating temperature range

Change Clause 17.3.8.8 as shown:

Four temperature ranges for full operation compliance to the OFDM PHY are specified in Clause 13. Type 1, defined as 0 °C to 40 °C, is designated for office environments. Type 2, defined as –20 °C to 50 °C, and Type 3, defined as –30 °C to 70 °C, are designated for industrial environments.  For WAVE operations, a fourth temperature range, Type 4, defined as from –40 to +85°C, is designated for automotive and outdoor environments.

# 17.3.9.4 Transmit center frequency tolerance

Insert text to Clause 17.3.9.4 as shown:

The transmitted center frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz channels and 10 MHz channels operating in the WAVE mode . The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

EDIT NOTE:  See RSoranno doc # 0878r0

# 17.3.9.5 Symbol clock frequency tolerance

Insert text to Clause 17.3.9.5 as shown:

The symbol clock frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels not operating in the WAVE mode, and shall be ±10 ppm maximum for 5 MHz channels and 10 MHz channels operating in the WAVE mode. The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

# 17.3.10  PMD receiver specifications

Insert text to Clause 17.3.10 as shown:

The receive specifications associated with the PMD sublayer are described in 17.3.10.1 through 17.3.10.5.  Two categories of receivers are allowed for WAVE stations.  They are designated Category 1 and Category 2. Category 2 requirements are optional.  Category 2 stations will have greater out-of-channel rejection to provide better system performance in a noisy signal environment.

# 17.3.10.1 Receiver minimum input sensitivity

Insert text to Clause 17.3.10.1 as shown:

The packet error rate (PER) shall be less than 10% at a PSDU length of 1000 octets for rate-dependent input levels shall be the numbers listed in Table 151 or less. For the WAVE mode receiver minimum sensitivity levels shall be less than or equal to the numbers listed in Table p12.  The minimum input levels are measured at the antenna connector (noise factor of 10 dB and 5 dB implementation margins are assumed).

Insert a new figure, Figure p12, immediately following Figure 151, renumbering as necessary:

Table p12—WAVE receiver performance requirements

| Modulation | Coding Rate (R) | Minimum sensitivity (dBm) 1 | Category 1 |  | Category 2 (OPTIONAL) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) |
| BPSK | 1/2 | -85 | 18 | 34 | 37 | 44 |
| BPSK | 3/4 | -84 | 17 | 33 | 36 | 43 |
| QPSK | 1/2 | -82 | 16 | 32 | 35 | 42 |
| QPSK | 3/4 | -80 | 15 | 31 | 34 | 41 |
| 16-QAM | 1/2 | -77 | 13 | 29 | 32 | 39 |
| 16-QAM | 3/4 | -73 | 11 | 27 | 30 | 37 |
| 64-QAM | 2/3 | -69 | 8 | 24 | 27 | 34 |
| 64-QAM | 3/4 | -68 | 4 | 20 | 23 | 30 |

# 17.3.10.2 Adjacent channel rejection

Insert text to Clause 17.3.10.2 as shown:

The adjacent channel rejection shall be measured by setting the desired signal’s strength 3 dB above the rate-dependent sensitivity specified in Table 151 or Table p12 for WAVE operations, and raising the power of the interfering signal until 10% PER is caused for a PSDU length of 1000 octets. The power difference between the interfering and the desired channel is the corresponding adjacent channel rejection. The interfering signal in the adjacent channel shall be a conformant OFDM signal, unsynchronized with the signal in the channel under test. For a conformant OFDM PHY the corresponding rejection shall be no less than specified in Table 151 or Table p12 for WAVE operations.

# 17.3.10.3 Nonadjacent channel rejection

Insert text to Clause 17.3.10.3 as shown:

The nonadjacent channel rejection shall be measured by setting the desired signal’s strength 3 dB above the rate-dependent sensitivity specified in Table 151 or Table p12 for WAVE operations, and raising the power of the interfering signal until a 10% PER occurs for a PSDU length of 1000 octets. The power difference between the interfering and the desired channel is the corresponding nonadjacent channel rejection. The interfering signal in the nonadjacent channel shall be a conformant OFDM signal, unsynchronized with the signal in the channel under test. For a conformed OFDM PHY, the corresponding rejection shall be no less than specified in Table 151 or Table p12 for WAVE operations.

# 17.4 OFDM PLME

# 17.4.1 PLME_SAP sublayer management primitives

Change Table 152 as shown:

Table 152—MIB attribute default values/ranges

| Managed Object | Default Value/Range | Operational Semantics |
| --- | --- | --- |
| dot11 PHY Operation Table |  |  |
| dot11 PHY type | OFDM-5. (04) | Static |
| dot11 Current reg domain | Implementation dependent | Dynamic |
| dot11 Current frequency band | Implementation dependent | Dynamic |
| dot11Temp type | Implementation dependent | Static |
| dot11 PHY Antenna Table |  |  |
| dot11 Current Tx antenna | Implementation dependent | Dynamic |
| dot11 Diversity support | Implementation dependent | Static |
| dot11 Current Rx antenna | Implementation dependent | Dynamic |
| dot11 PHY Tx Power Table |  |  |
| dot11 Number supported power levels | Implementation dependent | Static |
| dot11 Tx power level 1 | Implementation dependent | Static |
| dot11 Tx power level 2 | Implementation dependent | Static |
| dot11 Tx power level 3 | Implementation dependent | Static |
| dot11 Tx power level 4 | Implementation dependent | Static |
| dot11 Tx power level 5 | Implementation dependent | Static |
| dot11 Tx power level 6 | Implementation dependent | Static |
| dot11 Tx power level 7 | Implementation dependent | Static |
| dot11 Tx power level 8 | Implementation dependent | Static |
| dot11 Tx power level 9 | Implementation dependent | Static |
| … | … | … |
| … | … | … |
| dot11 Tx power level 63 | Implementation dependent | Static |
| dot11 Tx power level 64 | Implementation dependent | Static |
| dot11 Current Tx power level | Implementation dependent | Static |
| dot11 Reg Domains supported Table |  |  |
| dot11Reg domains supported | Implementation dependent | Static |
| dot11 Frequency bands supported | Implementation dependent | Static |
| dot11 PHY Antennas List Table |  |  |
| dot 11 Supported Tx antenna | Implementation dependent | Static |
| dot11 Supported Rx antenna | Implementation dependent | Static |
| dot 11 Diversity selection Rx | Implementation dependent | Dynamic |
| dot11 supported Data Rates Tx Table |  |  |
| dot11 Supported data rates Tx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mb/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mb/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) | Static |
| dot11supportedDataRatesRxTable |  |  |
| dot11 Supported data rates Rx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mb/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mb/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) | Static |
| dot11 PHY OFDM Table |  |  |
| dot11 Current frequency | Implementation dependent | Dynamic |
| dot11 TI threshold | Implementation dependent | Dynamic |
| dot11 Channel starting factor | Implementation dependent | Dynamic |
| dot11 ACR type | Implementation dependent | Dynamic |

***JPM/BobSoranno to verify reason to keep dot11 ACR type:***

# 17.4.4 OFDM PHY characteristics

***JPM/CarlKain to verify that the following statement is not needed because these parameters for WAVE are covered in the MIB and elsewhere.***

For the WAVE mode, a channel switching time (aChSwitchTime) of ≤ 2 TU has been defined. For WAVE the value for aAirPropagationTime is <4 µs, not << 1µs, because of the longer distances that can be used for WAVE communications. For WAVE the Slot Time is increased to 16 µs.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the draft Clause 17 changes as defined in section 4 of this document (1136r1) as the replacement of Clause 20 in P802.11p D1.0

And instruct the editor to incorporate these changes into a new draft P802.11p amendment as appropriate.

Motion by: ____________________Date: _________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |