IEEE P802.11k™/D4.2, July 2006

Draft Standard for Information Technology—
Telecommunications and Information Exchange Between Systems—
Local and Metropolitan Area Networks—
Specific Requirements

# Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

# Amendment 1: Radio Resource Measurement

Sponsor 
LAN/MAN Committee
of the
IEEE Computer Society

Copyright © 2006 by the IEEE.
Three Park Avenue
New York, NY 10016-5997, USA
All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for IEEE Standards Committee participants to reproduce this document for purposes of international standardization consideration. Prior to adoption of this document, in whole or in part, by another standards development organization permission must first be obtained from the Manager, Standards Intellectual Property, IEEE Standards Activities Department. Other entities seeking permission to reproduce this document, in whole or in part, must obtain permission from the Manager, Standards Intellectual Property, IEEE Standards Activities Department.

IEEE Standards Activities Department
Manager, Standards Intellectual Property
445 Hoes Lane, P.O. Box 1331
Piscataway, NJ 08855-1331, USA

Abstract: This amendment specifies the extensions to IEEE Std 802.11® for Wireless Local Area Networks providing mechanisms for Radio Resource Measurement.

Keywords: local area network (LAN), radio, measurement, radio resource, network management

# Introduction

This introduction is not part of IEEE Std 802.11k-2006, IEEE Standard for Information Technology – Telecommunications and Information Exchange Between Systems – Local and Metropolitan Area Networks – Specific Requirements – Part 11:  Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications – Amendment 1: Radio Resource Measurements.

Radio Resource Measurement (RRM) is a key enabler to the next generation of WLANs. RRM addresses some of the existing issues in using unlicensed radio environments to meet the requirements of emerging technologies. In addition, RRM provides knowledge about the radio environment to improve performance and reliability.

The existing 802.11 measurements and information about them are inadequate to move ahead to the next generation of Wireless Local Area Network (WLAN). The proposed 11k approach, from the PAR and 5 criteria, is to add measurements that extend the capability, reliability, and maintainability of WLANs through measurements and provide that information to upper layers in the communications stack. In addition, there are new applications that require quantifiable radio environment measurements in order to attain the necessary performance. These applications include VOIP, Video Over IP, location based applications, as well as applications requiring mitigation of harsh radio environments (multifamily dwellings, airplanes, factories, municipalities, etc).

IEEE 802.11k addresses the MAC, the SME, the MLME, primitives, the MIB and the interface to upper layers. The 11k scope is: Define Radio Resource Measurement enhancements to provide mechanisms to higher layers for radio and network measurements. The group has provided the mechanisms using request/response queries and the MIB using an Object Identifier (OID) interface to upper layers.

The 11k specification adopted the TGh layer management request/response model to gather the information. Also, by recognizing the mobility requirements of the new technologies, such as VoIP and video streaming, the group specified measurements, the channel load report, and the neighbor report as a means of developing pre-handoff information gathering that drastically speeds up the ability to handoff between cells within the same ESS. By accessing and using this information, the STAs (either in the APs or in the individual devices) can make decisions about the most effective way to utilize the available spectrum, power, and bandwidth for its desired communications.

The Radio Resource Measurement specification:

- Enables better diagnostics of problems

- Using info that is easy and cheap to gather

- Enables better frequency planning, optimized network performance

- Enables automated frequency planning

- Enables new services

- Location based services

- Voice Over IP (VOIP)

- Video Over IP

The specification makes lists of known stations (STAs, both APs and devices) visible to the requestor; it includes link statistics including counts of MSDU/MPDU and channel utilization in total microseconds; it includes the data rates and modulation and link margin as seen by other stations. It includes optional events to notify management stations of authentication and association events and can provide TRAPS on association and authentication failures. It also provides reports on all the RSN pre-authentication and de-authentication events as well as normal association and de-association events.

Radio measurement, as offered by 11k, enables automatic radio configuration and enables radio-aware performance by offering the measurements to improve monitoring and transition. Radio measurements allow the 802.11 community to:

- Simplify and/or automate WLAN radio configuration

- Achieve better performance in dense BSS deployments

- Better utilize radio resources across client stations

- Alert WLAN administrator to problems

- Notify client station users of current radio status

- Vendors use measurements to add value

The request/response measurements are:

- Beacon

- Measurement Pilot

- Frame

- Channel Load

- Noise Histogram

- STA Statistics

- Location Configuration Information

- Neighbor Report

- Link Measurement

- QoS Metrics

The request-only mechanism is:

- Measurement Pause

The following list of measurements gives the reader an idea of the type of measurements obtained through the 11k mechanisms. Measurements include country, power constraints, Transmitter Power Control (TPC) report, Access Point (AP) Channel, power capability, neighbor report element ID, TSF Information, Received Channel Power Indicator (RCPI), RSNI, DS parameter set, capability information, channel number, regulatory class, randomization interval, measurement duration, Basic Service Set Identifier (BSSID), reporting condition, threshold/offset, measurement modes (passive/active or beacon table), group identity, STA counters, late bit, incapable bit, refused bit, location, RPI density, measurement start time, measurement duration, PHY type, parent TSF, target TSF, beacon interval, number of frames, transmitted fragment count, multicast transmitted frame count, failed count, retry count, multiple retry count, frame duplicate count, RTS success count, RTS failure count, ACK failure count, received fragment count, FCS error count, transmitted frame count, latitude resolution, latitude, longitude resolution, longitude, altitude type, altitude resolution, altitude, element ID, channel list, BSSID information, PHY options, TSF Information, reachability, security, key scope, capabilities unknown, capabilities, spectrum management capability, QoS capability, TSF offset, number of repetitions, frame restart delay, time unit, restart delay, neighbor TBTT offset type, transmit power, link margin,

The measurement pairs include the beacon request/report, the frame request/report, the channel load request/report, the noise histogram request/report, the STA statistics request/report, the Location Configuration Indication (LCI) request/report, the neighbor request/report, the link measurement request/report, and the QoS Metrics request/report.

Beacon

The Beacon request/report pair enables a STA to request from another STA a list of APs it can receive on a specified channel or channels. This measurement may be done by active mode (like active scan), passive mode (like passive scan), and beacon table modes. If the measurement request is accepted and is in passive mode, a duration timer is set and the measuring STA monitors the requested channel, measures beacon, probe response and measurement pilot power levels (RCPI), and logs all received beacons, probe responses and measurement pilots within the measurement duration. If the measurement request is in active mode, the measuring STA sends a probe request on the requested channel at the beginning of the measurement duration, then monitors the requested channel, measures beacon, probe response and measurement pilot power levels (RCPI), and logs all received beacons, probe responses and measurement pilots within the measurement duration. If the request is beacon table mode, then the measuring STA returns the current contents of any stored beacon information without monitoring any channel. At the end of the duration, the measuring STA prepares and sends a measurement report of all the beacon, probe response or measurement pilot information.

Measurement Pilot

The Measurement Pilot frame is a compact management frame periodically transmitted by an AP with a relatively small interval as compared to a Beacon Interval. The Measurement Pilot frame is designed to provide a minimal set of information as compared to a Beacon frame to allow for the required small interval. The purpose of the Measurement Pilot frame is to provide timely information to a STA.

Frame

The frame request/report pair returns a picture of all the channel traffic and a count of all the frames received at the measuring STA. For each unique Transmitter Address, the STA reports the Transmitter Address, number of frames received from this transmitter, average power level (RCPI) for these frames, and BSSID of the transmitter.

Channel Load

The channel load request/report pair returns the channel utilization measurement as observed by the measuring STA.

Noise Histogram

The noise histogram request/report pair returns a measurement of non-802.11 noise power by sampling the channel when CCA indicates idle.

STA Statistics

The STA statistics request/report pair returns groups of values for STA counters and for BSS load. The STA counter values include: transmitted fragment counts, multicast transmitted frame counts, failed counts, retry counts, multiple retry counts, frame duplicate counts, Request to Send (RTS) success counts, RTS failure counts, Acknowledgement (ACK) failure counts, received fragment counts, multicast received frame counts, FCS error counts, and transmitted frame counts. BSS load values include: AP service load, average access delay for each access category, associated STA count, and channel utilization

Location

The Location request/report pair returns a requested location in terms of latitude, longitude, and altitude. It includes types of altitude such as floors and permits various reporting resolutions. The requested location may be the location of the requestor (e.g. Where am I?) or the location of the reporting STA (e.g. Where are you?)

Measurement Pause

This measurement request is defined, but no report comes back from this request. The measurement pause permits the inclusion of a quantified delay between the execution of individual measurements which are provided in a series within a measurement request frame. The measurement pause used as the last measurement in a frame provides control of the measurement period when measurement request frames are to be repeated.

Neighbor

The neighbor report request/report pair is sent by an AP and returns information about known neighbor APs that are candidates for a BSS transition. Reports contain information from the table dot11RRMNeighborReportTable in the MIB concerning neighbor APs. This request/report pair enables a STA to gain information about the neighbors of the associated AP to be used as potential roaming candidates.

Link Measurement

The link measurement request/report pair is like an RF ping in which a STA requests another STA to measure and report the link path loss and estimation of the link margin. This enables understanding the instantaneous capabilities of a link for QoS-type requirements.

The 11k standard provides the ability of a STA to query its radio environment to make appropriate assessment about its health and efficiency. It is the first step in making the Wireless LAN (WLAN) smart and capable of making appropriate decisions for fast transition, for mesh connectivity, and for managing the radio environment for all wireless devices.

# Participants

At the time this supplement to the standard was submitted to Sponsor Ballot, the working group had the following membership:

Stuart J. Kerry, Chair		Al Petrick and Harry Worstell, Vice Chairs

Tim Godfrey, Secretary

Richard H. Paine, Chair Task Group k

Simon E. M. Barber, Editor, 802.11k

Paul Grey, Secretary, 802.11k

This list to be added upon conclusion of the sponsor ballot.

Major contributions were received from the following individuals:

Daryl Kaiser

Joseph Kwak

Dave Skellern

Steve Pope

Mike Moreton

Byoung-Jo “J” Kim

Simon Black

Hasse Sinivaara

Marty Lefkowitz

Zhun Zhong

Arun Ayyagari

Harry Worstell

Walter Johnson

Tim Olson

Lars Falk

Malik Audeh

Leo Monteban

Emily Qi

Bernard Aboba

Stephen Wang

Tim Wilson

Steve Emeott

Floyd Simpson

Peter Ecclesine

John Klein

Sudheer Matta

The following persons were on the balloting committee: (To be provided by IEEE editor at time of publication.)

Document History

| Richard Paine |  | 0.0 | Created empty template |
| --- | --- | --- | --- |
| Simon Barber |  | 0.1 | Added Daryl’s draft (xyz) |
| Simon Barber | June 2003 | 0.2 | Made changes according to the minutes of the May 2003 session (03/321r0) namely changes as specified in: Paragraph 1 - document 03/318r0, Paragraph 3 - document 03/318r1, 03/304r0, 03/174r3, 03/340r1, 03/317ro, 03/171r7, 03/372r0 |
| Simon Barber | July 8th 2003 | 0.3 | Removed changes from 03/174r1a that had been mistakenly applied, due to 03/174r3 being missing from documents on public website |
| Simon Barber | July 16th 2003 | 0.4 | Added changes from 03/174r3 |
| Simon Barber | August 5th 2003 | 0.5 | Made changes according to the minutes of the July 2003 session (03/xxxry) namely changes as specified in: - 03/608 - 03/582r2 (not done, due to unavailability of correct document), - Removal of preferred, trusted, rogue and subnet from clauses 7.3.2.22 and 11.12. |
| Simon Barber | August 25th 2003 | 0.6 | Made changes as specified in 03/582r0 (as confirmed as the correct document from July 2003) |
| Simon Barber | September 30th 2003 | 0.7 | Made changes according the minutes of the September 2003 session (03/780). Namely changes as specified in: - 03/605r2 – MIB AP service load, Joe Kwak - 03/733r1 – Beacon Periodic Measurements, Joe Kwak - 03/762r0 – Measurement Start Time, Tim Olson |
| Simon Barber | October 20th 2003 | 0.8 | Added to contributors list, corrected editing errors in adding Joe’s contributions to 0.7 |
| Simon Barber | November 24th 2003 | 0.9 | Made changes according to the minutes of the November 2003 session (document no 11-03-942r0). Changes are specified in documents: - 11-3-837-r3 – MIBs – Request/Report mechanism - 11-3-786r1 – 11h TPC mechanism for 11k, Joe Kwak - 11-3-789r2 – Extensions to SMTnotification table, Joe Kwak - 11-3-952r1 – Channel Information in Probe Request, Leo Monteban - 11-3-869r1 – Draft PICS Proforma, Simon Black |
| Simon Barber | January 12th 2004 | 0.10 | Corrected changes from Joe Kwak’s submission that were added to the 0.9 draft with the incorrect assumption that the base document did not include 11h. |
|  |  | 0.11 | No document issued with this number |
| Simon Barber | February 3rd 2004 | 0.12 | Applied most of the changes approved at the January 2004 Vancouver meeting – namely: - 11-04/123r0 – Revised AP Channel Report – Black / Sinivaara Comments resultions from 11-04/067r3 – 11k 0.9 comment review. Resolutions applied were: 3, 15, 18, 23, 25, 26, 28, 29, 30, 42, 43, 45, 47, 48, 49, 50, 51, 52, 53, 59, 63, 64, 67, 68, 70, 77, 78, 85, 94, 95, 96, 99, 100, 101, 102, 103, 106, 113, 122, 125, 126, 128, 130, 132, 135, 141, 142, 148, 149, 151, 162, 181, 187, 188, 189, 191, 201, 225, 233, 235, 236, 237, 239, 249, 250, 251, 253, 256, 260, 264, 266, 267, 296, 308, 309, 311, 312, 313, 315, 316 These approved resolutions have not been applied: 60, 133, 154, 156, 226, 298, 300, 310, 319 |
| Simon Barber | March 17th, 2004 | 0.13 | Applied changes from submissions approved by Tuesday Midday at the March 2004 Orlando meeting – namely: - 11-04/281r0 - Beacon Table Clarification - Olson - 11-04/252r0 - Correcting Medium Sensing Time Histogram Report Text - Zhong, Mangold, Soomro - 11-04/292r1 - Harmonisation of IEEE802.11k/D0.12 and IEEE802.11h - Black - 11-04/294r1 - Comment Resolution - Black - 11-04/296r1 - Comment Resolution for Periodic Measurements - Kwak |
| Simon Barber | March 19th, 2004 | 0.14 | Applied further changes approved at the March 2004 Orlando meeting – namely: - 11-04-251r2 - Revised site report text in Clause 11.12 - Zhun Zhong, Stefan Mangold, Amjad Soomro - 11-04-403r1 - Measurement Modes – Simon Black - 11-04-404r1 - Returning Measurement Reports – Simon Black - 11-04-108r1 - Text proposal on PICS for TPC - Marian Rudolf, Joe Kwak - 11-03-961r5 – adding RCPI information element in the probe response - Sudheer Matta Applied approved resolutions to comments: 6, 7, 294 Reordered 5.4.5, 5.4.4.1 and fixed editing instructions Reordered A.3-9, and fixed formatting 7.2.3.1 fixed editing instructions 7.3.1.11, 7.3.2 - fixed typos in editing instructions 7.3.2.21.4, 7.3.2.21.5 – fixed broken references |
| Simon Barber | June 3rd, 2004 | 0.15 | Applied approved resolutions to comments from 04-067r8. Comment numbers: 18, 23, 36, 37, 104, 105, 114, 115, 116, 119, 120, 121, 124, 141, 158, 199, 200, 242, 243, 244, 245, 252, 290, 292, 301, 304. Also a few small typo and layout corrections. |
| Simon Barber | June 3rd, 2004 | 0.16 | Applied changes approved at the May 2004 Garden Grove meeting: Motion by Edney and Black, re: measurements outside of the BSS Motion by Walker and Olson, re: channel 0 implying iterative measurements Motion by Qi and Kwak, 11-04-580r1 with changes. Motion to accept comment resolutions from teleconferences in 11-04-480-r6. Comment numbers: 3, 4, 5, 7, 8, 9, 10, 12, 21, 25, 27, 30, 31, 36, 38, 39, 40, 41, 42, 44, 46, 47, 48, 49, 50, 51, 55, 56, 57, 60 Motion to accept further comment resolutions from 11-04-480-r6. Comment numbers applied: 1, 21, 28, 29, 34, 45, 52, 69, 70, 71, 81, 85 (no renumbering), 87, 89, 92, 93, 94, 95, 102, 108, 109, 110, 111, 113, 114, 125, 127, 128, 131, 134, 135, 136, 138, 142, 144, 148, 149, 154, 156, 160, 191, 193, 194 (this undoes 191), 214 (this leaves other references to this dangling), 218, 239 Comment numbers not applied: 2 – need text 35 – unclear resolution 155 – partly done – no ref found on p29 l6 234,235 – need input from Terry Cole / SA |
| Simon Barber | July 14th, 2004 | 0.17 | Comments resolved: 35, 40, 41, 42, 72, 73, 78, 79, 80, 82, 83, 84, 86, 91, 98, 99, 100, 101, 103, 106, 112, 115, 116, 119, 120, 121, 122, 124, 126, 133, 139, 140, 146, 147, 150, 151, 152, 153, 155, 158, 162, 164, 166, 167, 169, 170, 171, 175, 177, 178, 180, 181, 183, 188, 189, 195, 196, 197, 199, 200, 201, 203, 204, 205, 206, 207, 209, 212, 236, 242, 243, Comments not resolved: 61 unclear 65 no text provided 159 (can’t find reference) 202 conflicts with 200 237 vague description 238 R2003 is not consistent, this is very hard to do in word. Consider during frame conversion 240 does not give resolution Motion by Black, 11-04-0757r0 Motion by Aboba, 11-04-735r4 |
| Simon Barber | July 27th, 2004 | 0.18 | Further work from the Portland, OR meeting: Motion by Edney, 11-04-0702r0 with changes Updated annex D with Portland comments applied, many thanks to Paul Gray for this work 11-04-0816r2 Motion by Black, 11-04-560r1 Comments 20, 43, 132, 137, 143 resolved Comment 16 could not be resolved, due to no clear resolution in the spreadsheet. Extra comments 1, 6 resolved Motion by Kwak, 11-04-0762r1 Motion by Kwak, 11-04-0763r1 |
| Simon Barber | July 27th, 2004 | 0.18a | On some versions of Word 0.18 seems to have lost it’s headers and footers. 0.18a is a version of 0.18 with headers and footers restored. |
| Simon Barber | July 29th, 2004 | 0.19 | Errors and Omissions corrected: Table 5, 11, 12 – font size corrected Table 12 – Incorrect editing instructions fixed Comment 96 from 04/757 Comment 132 from 04/480r19 Figure k15 – extra lines removed 10.3.2.2.2 table edited incorrectly from 04/757 |
| Simon Barber |  | 1.0 | Removed document history – created letter ballot document. |
| Simon Barber |  | 1.1 | Re-inserted document history. Processed editorial motions from Berlin Meeting: - Update figure 27 to be consistent with all ratified stds - Apply for ANA values and use them for Action Category, Capability Bits, Element IDs - Fix order values for Table 5,7,9,11,12 - Document 04/1095r0 - Replacement of Para 11.7.2/11.7.6 Processed comments ratified at Berlin meeting: 6, 17, 20, 23, 25, 27, 28, 31, 42, 43, 44, 45, 46, 47, 48, 56, 61, 67, 69, 79, 88, 97, 99, 105, 117, 118, 119, 125, 126, 128, 137, 138, 139, 178, 179, 203, 205, 215, 216, 217, 218, 219, 220, 240, 252, 329, 330, 332, 333, 336, 337, 339, 343, 344, 345, 347, 349, 350, 351, 352, 353, 354, 355, 356, 357, 358, 360, 361, 362, 363, 364, 365, 366, 367, 370, 371, 372, 373, 385, 775, 777, 942, 963, 983, 984 Did not process comments ratified at Berlin meeting: 14 – todo verify with SA correct style template 142 – want to verify change with group 156, 158 – resolution ambiguous 228, 230, 236 – need to check with SA |
| Simon Barber | January 2005 | 1.2 | Applied changes from editorial motions from San Antonio meeting: 04/1403r0 with changes – Neighbor TSF simplification 04/1410r0 – Neighbor Report in Association Response 04/1390r1 – Periodic Comment Resolutions 04/1440r1 – RCPI for TGg Extended rate PHY Addressed comments: 64, 120, 121, 122, 132, 376, 400, 435, 439, 441, 442, 444, 447, 448, 450, 451, 452, 453, 454, 455, 457, 458, 464, 467, 468, 470, 472, 473, 474, 475, 476, 477, 479, 994, 997 (via 1390r1) 831, 843 (via 1440r1) 5, 8, 21, 29, 30, 34, 53, 86, 89, 149, 167, 170, 180, 183, 197, 206, 207, 231, 261, 262, 264, 269, 274, 279, 282, 284, 286, 290, 291, 292, 293, 298, 305, 311, 312, 313, 314, 415, 417, 318, 319, 320, 321, 322, 324, 325, 326, 327, 334, 340, 342, 348, 374, 375, 378, 389, 393, 395, 402, 403, 416, 417, 418, 422, 428, 438, 483, 487, 489, 492, 494, 495, 500, 506, 514, 515, 545, 546, 554, 555, 556, 573, 574, 577, 595, 599, 600, 604, 605, 606, 609, 610, 612, 613, 616, 627, 628, 636, 637, 639, 640, 641, 647, 650, 652, 657, 665, 674, 679, 682, 693, 705, 717, 718, 735, 736, 743, 756, 758, 769, 776, 780, 783, 812, 813, 817, 830, 832, 834, 835, 836, 838, 840, 841, 842, 844, 847, 851, 853, 855, 911, 914, 935, 962, 972 (via 964r24) Not addressed: 50 - until TGe is ratified 11.7 is OK, no change to doc. 86 - does not state where inserted text should go. I guessed. 162,163,164 - no action for editor 169 - no text provided. changes unclear. 256,602,603 - no text provided. 261,394,515 - since TGe is not done yet no change needed. 264 - change left following para not making sense - I edited it a little. 265 - 2 different resolutions proposed - editor needs just 1! no change made. 304 - this changes base standard - are we sure? 334 - apparent no-op. I Did nothing. 346 - TGe is not yet issued, so clauses should not be changed yet. 359 - there is no TGe yet - cannot process comment. 368 - does not appear to resolve comment. 440 - not sufficiently clear resolution. 563 - resolution is truncated. 608 - unclear resolution, text must be provided. 666,675,681 - resolution truncated. 677,678,690,703,738,739,750,752,753 - resolution truncated - partially applied. need definition for capabilities unknown. 718 - need text for reference. 751,771 - resulution truncated - partially implemented. 756 - changed RSN to security to align with previous resolution. 770 - resolution not clear - text needed. 773,796 - resolution not clear - text needed. 779 - truncated. need more text. 839 - resolution unclear, text needed. |
| Simon Barber | January 2005 | 1.3 | 334, 505, 770, 856, 857, 858, 859, 860, 861, 862, 863, 866, 867, 868, 869, 876, 877, 878, 880, 881, 883, 888, 889, 890, 891, 893, 898, 900, 901, 912, 967, 1014, 1020, 1035 Not Addressed: 895 – Does not specify clear resolution (commenter agrees to withdraw) |
| Simon Barber | January 2005 | 1.4 | Applied normative text changes as requested by motions at the Monterey meeting: 05/0009r0 – CNPI-PHY interface 05/0041r0 – Neighbor Report SAP 04/1120r0 – TPC comment resolution 05/0038r0 – Parallel Category Comment Resolution Addressed comments: 1, 2, 3, 4, 13, 19, 22, 24, 26, 36, 37, 38, 40, 49, 52, 55, 57, 58, 59, 60, 62, 66, 68, 70, 72, 76, 78, 80, 81, 82, 84, 87, 90, 94, 95, 96, 98, 101, 103, 106, 107, 108, 109, 110, 113, 115, 116, 129, 140, 143, 144, 146, 147, 148, 156, 162, 163, 164, 166, 172, 175, 176, 181, 184, 185, 186, 187, 188, 189, 190, 192, 193, 196, 198, 199, 200, 201, 204, 209, 210, 211, 213, 222, 223, 225, 226, 232, 234, 235, 238, 243, 244, 245, 247, 250, 258, 265, 267, 273, 278, 285, 289, 299, 300, 302, 303, 310, 331, 335, 338, 368, 396, 398, 400, 408, 409, 410, 411, 419, 423, 426, 433, 434, 436, 437, 443, 445, 446, 460, 465, 491, 508, 512, 516, 517, 520, 524, 525, 537, 552, 561, 572, 585, 591, 597, 602, 603, 658, 662, 664, 671, 673, 683, 695, 697, 698, 699, 700, 701, 702, 704, 706, 707, 708, 711, 722, 723, 731, 751, 771, 774, 781, 786, 787, 821, 825, 826, 833, 845, 846, 886, 955, 957, 961, 969, 987, 996, 1004, 1027, 1034 |
| Simon Barber | February 2005 | 1.5 | Applied further normative changes from Monterey: 05/0071r0 – Repeated Measurement Request Frames Addressed Comments: 86, 124, 127, 133, 135, 141, 169, 228, 268, 283, 466, 507, 578, 608, 666, 669, 675, 677, 678, 681, 685, 690, 703, 725, 726, 727, 729, 730, 732, 738, 739, 750, 752, 753, 788, 789, 790, 791, 792, 793, 797, 798, 802, 804, 807, 815, 816, 818, 819, 823, 827, 828, 833, 845, 852, 854, 875, 884, 905, 906, 908, 913, 921, 924, 927, 937, 938, 939, 954, 959, 970, 974, 975, 976, 978, 979, 981, 992, 995, 1003, 1011, 1012, 1018, 1019, 1021, 1028, 1029, 1030, 1031, 1032, 1038, 1039, 1040 Updated contributor list as requested. |
| Simon Barber | February 2005 | 1.6 | Added MIB comment resolutions, many thanks to Paul Gray for his help with this work. |
| Simon Barber | February 2005 | 2.0 | Removed changebars, doc history for letter ballot draft |
| Simon Barber | July 2005 | 2.1 | Added doc history back after letter ballot. For alignment with 11e some clause renumbering has happened. 11.4→11.8; 11.5→11.9; 11.6→11.10; 11.7→11.11; 11.8→11.12. Added change from Atlanta: 05/1637r4 – QoS Metrics (done) 05/1599r2 (superceeded by instructions in 05/403r1) 05/0136r0 (done) Added changes from Cairns: 05/0353r0 – Frequency Bands (done) 05/0403r1, superceding 1599r2 (done) 05/476r0 – removal of BSSID info field (done) 05/0191r22 - editorial comment res (partially done, 18 comments left – picked up in 05/k9) |
| Simon Barber | July 2005 | 2.2 | Added further changes from Cairns: 05/k9 (done, except 24 comments – 19, 63, 123, 149, 174, 198, 241, 285, 296, 418, 597, 662, 665, 666, 667, 668, 909, 910, 1006, 1035, 1108, 1160, 1191, 1279) 05/k10 (done) 05/012r2 (done) 05/0458r1 (done) 05/475r1 (done) k17 and k18 (Barber – Hidden) (done) 05/472r1 (Black) (done) 05/409r0 (Klein) (done) 05/0252r4, exclude comment resolutions 472, 596, 887, 989, 1348 (done – comments 301, 402, 411, 412, 423, 426, 820, 1168, 1170, 1175, 1180). 05/438r0 (Kwak - Periodic) (done) |
| Simon Barber | September 2005 | 2.3 | Incorporated changes as moved in San Francisco: 11-05-655r0 – Removal of Neighbor Report from Association - resolves comments 80, 83, 199, 205, 242, 337, 348, 349, 364, 366, 370, 413, 461, 474, 475, 490, 508, 524, 561, 583, 602, 626, 637, 640, 670, 671, 687, 697, 741, 776, 798, 799, 878, 1043, 1104, 1110, 1144, 1249, 1271, 1324, 1325, 1327, 1328, 1346, 1357 11-05-685r0 – CNPI comments – resolves comments 28, 86, 760, 1120, 1296, 1332, 1489, 1490, 1491, 1492, 1503, 1504, 1505, 1535 11-05-744r0 – nothing for editor to do. 11-05-663r0 – Neighbor Report 11-05-664r1 – Neighbor Report for multiple ESSs 11-05-440r1 – RCPI - resolves comments 3, 12, 42, 65, 66, 67, 68, 69, 70, 94, 99, 100, 111, 112, 152, 165, 169, 170, 204, 266, 267, 268, 269, 273, 274, 279, 286, 294, 333, 340, 341, 368, 373, 463, 464, 477, 478, 480, 485, 487, 522, 587, 591, 610, 611, 612, 614, 649, 659, 674, 695, 739, 826, 827, 829, 862, 863, 865, 870, 871, 892, 893, 901, 919, 920, 972, 973, 974, 975, 976, 977, 982, 1105, 1106, 1117, 1130, 1158, 1209, 1210, 1256, 1270, 1313, 1326, 1331, 1362, 1363, 1412, 1415, 1440, 1444, 1454, 1472, 1488, 1509, 1510, 1519, 1523, 1524, 1525, 1540, 1541, 1542, 1543, 1544, 1545, 1587, 1598, 1600 11-05-434r4 – Advanced Antennas - resolves comments 299, 465, 756, 757, 868, 1013, 1233, 1452, 1502, 1558, 1594, 1601, 1602 11-05-679r0 – revised PICS – comments? 11-05-780r0 – SSID in Beacon Measurement Rqst – resolves comments 110, 690, 873, 969, 971, 1119 11-05-512r2 – Triggered QoS Measurements – resolves comments 11, 641, 755, 813, 816, 907 and 1187 |
| Simon Barber | September 2005 | 2.4 | Remaining comments from comment spreadsheet from SF resolved. Also: 05-579r1 – Location Comments |
| Simon Barber | September 2005 | 2.5 | Incorporated changes as moved in Garden Grove 05/948r0 - Beacon Measurement on Pilot Frames 05/762r1 - LB73 Statistics Comments - 344, 442, 443, 619, 903, 962, 964, 984, 1139, 1316, 1376, 1442, 1450, 1469, 1493, 1494, 1506, 1507, 1537, 1593 05/995r0 - RCPI Accuracy Change 05/1005r0 - Triggered QoS Measurements 05/1003r1 - Request-Report, Part 2 05/245r5 – MIB comments 90, 91, 345, 354, 356, 357, 494, 495, 496, 497, 498, 499, 500, 501, 502, 503, 521, 1081, 1082, 1220, 1221, 1226, 1281, 1284, 1287, 1288, 1289, 1290, 1299, 1300, 1301, 1302, 1303, 1304, 1305, 1308, 1554 1078, 1283, 296 141, 142, 143, 144, 355, 467, 630, 631, 632, 633, 661, 852, 856, 857, 1077, 1079, 1083, 1222, 1223, 1224, 1225, 1227, 1228, 1280, 1282, 1285, 1286, 1291, 1292, 1293, 1294, 1295, 1297, 1298, 1306, 1310, 1311, 1428, 1429, 1556 183, 226, 545, 718, 1080, 1091, 1229, 1307, 1309 05/976r0 – Misc LB73 comments - 119, 165. 480, 621, 763, 826, 842, 844, 1130, 1270, and 1587 05/941r1 – RSNI - 846, 1448 and 1592 11-05-664r1 – Neighbor Report for multiple ESSs – added clause 11 changes that had been omitted previously. |
| Simon Barber | November 2005 | 3.1 | Reintroduced document history section. Fixed editorial comments. Applied 05/1192r1 Applied 05/1173r0 |
| Simon Barber | January 2006 | 3.2 | Editorial Comments resolved Applied Resolution from document 06/17r0 – 12,14,15,16 05/1209r3 - 775,778,1558,1559 05/1238r0 - 299, 395, 396, 397, 452, 453, 513, 514, 515, 516, 517, 705, 1031, 1032, 1222, 1223, 1224, 1225, 1226, 1227, 1228, 1311, 1391 06/138r0 - 241, 242, 608, 609, 758, 759, 760, 1110, 1111, 1112, 1113, 1179, 1180, 1181, 1182, 1183 |
| Simon Barber | February 2006 | 3.3 | 06/15r1 - 90,256,779,1291 06/0022r0 (except comment 1532) 06/0021r0 - 16,17,96,97,127,130,145,168,208,285,382,390,783,784,842,864,887,1176,1177,1468,1532,1533 06/0023r2 and 06/0024r2 - 1169, , 757, 1170, 95, 1089, 1464, 1171, 1172, 1173, 1174, 1465 06/0047r3 & 06/0048r2 - 8, 83, 157, 328, 350, 472, 473, 619, 620, 621, 622, 623, 624, 625, 755, 885, 919, 955, 999, 1000, 1010, 1011, 1012, 1134, 1135, 1139, 1140, 1141, 1250, 1251, 1266, 1305, 1341, 1363, 1422, 1423 and 1424 05/1231r3 & 06/151r1 - 366, 367, 425, 743, 796, 797, 1283, 1379, 1380, 1517, 238, 957, 1092, 1104 and 1105 06/118r0 – 06/183r1 – |
| Simon Barber | March 2006 | 3.4 | 06/119r1 - MIB 06/28r0 – Introduction - 277, 498, 499, 834, 874, and 900 06/96r0 – comment 1026 06/97r0 – AP reachability comment resolution 06/100r0 – 06/101r0 – in-use antennas comment resolution 06/105r0 – 5.2.6. descriptive text update 06/106r0 – 5.2.7 descriptives 06/107r1 – 5.6 relationships between services edits |
| Simon Barber | March 2006 | 3.5 | 06/341r1 – TSF note removed 06/301r0 - 520, 521, 1230, 522, 1231, 1232, 1418, 304, 1317, 1556, 270, 455, 699, 1036, 1411, 1412, 1557 06/314r0 – 709 06/309r1 - 364, 940, 1377, 1437, 365, 1378, 89, 117, 368, 369, 426, 427, 582, 583, 744, 1069, 1284, 1285, 1381, 1382 05/1192r2 – 06/296r1 – 56, 105, 697, 888,and 1145 06/393r1 – remove beacon measurement for passive pilot / STA selected |
| Simon Barber | March 2006 | 3.6 | 06/307r5 – 115, 1302, 1260, 1502, 1434, 1463 06/182r1 - 158, 159, 212, 428, 494, 584, 585, 586, 587, 588, 745, 746, 941, 953, 954, 956, 1071, 1072, 1073, 1074, 1075, 1076, 1077, 1078, 1079, 1080, 1286, 1287, 1288, 1289, 1290, 1440, 1522, 1523 06/449r0 – 1102 06/319r1 – transmit QoS metrics 06/435r0 – 05/1252r1 – Comment 239 Comment 240 06/471r0 – 1223, 1225 06/176r4 – frame report Comment 1259 Comment 1458 |
| Simon Barber | March 2006 | 3.7 | 06/0486r1 – MIB comments 06/320r2 – 84, 576, 1274 |
| Simon Barber | July 2006 | 4.1 | Reintroduced document history after letter ballot Comment resolutions from 06/584r4: 713, 721, 588, 33, 568, 590, 589, 39, 431, 563, 681, 592, 557, 591, 42, 556, 46, 51, 52, 217 – not done, applied 52 instead 570, 496, 497, 498, 499, 55, 56, 57, 58, 59, 461, 559, 573, 599, 281, 60, 685, 61, 401, 600, 62, 63, 560, 574, 218, 219, 64, 65, 505, 667, 66, 436, 220 and all same as, 606, 67 & all same as, 68, 69, 512, 608, 609 and all same as, 71, 548, 610, 611, 612, 613, 617, 618, 619, 73, 566, 222, 77, 154, 80, 223, 84 and same as, 224, 577, 94, 99, 500, 103, 104, 225, 226, 228, 227, 229, 720, 105, 106, 107, 501, 108 315, 649, 517 – text I am asked to change is in the baseline – suggest commenter take comment to 11ma group 552, 376, 626, 472, 192, 230 231, 232, 492, 110, 111, 235, 112, 114, 116, 236, 237, 239, 240, 241, 242, 3, 710 9 – conflicts with comment 710, applied 710 only. 10, 11, 12, 711, 17, 212, 18, 19, 20, 21, 712, 244, 23, 213 Comment resolutions from 06/728r1: 167 168, 169 – change requested is to baseline text. Editor suggests commenter take change to 11ma. 202 – definition already in baseline 585, 680, 43, 558, 125, 92, 93 Text from document 643r2 applied - Information about AP’s Measurement Pilot transmission in Probe Response and Neighbor Report Comments resolutions from 06/772r2: 66, 505, 667, 666, 504, 107, 160, 113, 353, 413, 414, 138, 643 – need text |
| Simon Barber | July 2006 | 4.2 | Text from 702r0 excepting 170 & 171, resolves comments: 107, 113, 138, 139, 160, 163, 166, 297, 344, 353, 413, 414, 425, 643 and 666 Text from 725r1, resolves comments: 78, 79, 81, 142,179, 266, 285, 287, 288, 290, 355, 356, 370, 428, 462, 514, 525, 550, 638, 639, 640, 642, 717 and 718 Applied normative text from 06/727r0, resolving comments: 245, 246, 310, and 311 Applied normative text changes from 06/785r1, resolving comments: - all the "accepted" and "counter" comment resolutions assigned to Kwak, as described in 06/0784r1 Applied changes from 06/970r0, resolving comments: - all the "accepted" and "counter" comment resolutions assigned to Kwak, as described in 06/0969r0 Applied changes from 06/921r0, resolving comment 416. Applied text changes from 06/985r0, resolving comments: - all accepted and countered comments in document 0986r0 in the category Clause 7.3.2.21-22-7 and Clause 11.11.8.2 Applied text changes from 06/960r1, resolving comments: 31, 32, 34, 163, 164, 165, 188, 273, 274, 276, 277, 383, 437, 438, 447, 455, 456, 506, 507, 510, 546, 569, 572, 598, 622, 623, 625, 632, 633, 634, 636, 714 and 715 Applied changes from 06/509r3 – resolving comments from 06/988r1 Applied resolutions from 06/778r1 Applied resolutions from 06/980r3, except comment 2 Applied resolutions from 06/778r1 Applied resolutions from 06/986r0 |

Contents

Introduction	ii

2. Normative references	1

3. Definitions	1

4. Abbreviations and acronyms	3

5. General description	3

5.2 Components of the IEEE 802.11 architecture	3

5.2.7 Wireless LAN Radio Measurements	3

5.3 Logical Service Interfaces	3

5.3.1 Station service (SS)	3

5.4 Overview of the services	4

5.4.7 Radio Measurement Service	4

5.6 Relationships between services	4

7. Frame formats	5

7.1 MAC frame formats	5

7.1.3 Frame fields	5

7.1.3.1 Frame Control field	5

7.2 Format of individual frame types	5

7.2.3 Management frames	5

7.2.3.1 Beacon frame format	5

7.2.3.4 Association Request frame format	6

7.2.3.5 Association Response frame format	6

7.2.3.6 Reassociation Request frame format	7

7.2.3.7 Reassociation Response frame format	7

7.2.3.8 Probe Request frame format	7

7.2.3.9 Probe Response frame format	7

7.2.3.9A Measurement Pilot frame format	9

7.3 Management frame body components	9

7.3.1 Fixed fields	9

7.3.1.4 Capability Information field	9

7.3.1.11 Action field	10

7.3.1.18 Country String	10

7.3.1.19 Measurement Pilot Interval field	10

7.3.1.20 Max Regulatory Power field	11

7.3.1.21 Max Transmit Power field	11

7.3.1.22 Transmit Power Used field	11

7.3.1.23 Transceiver Noise Floor field	12

7.3.2 Information Elements	12

7.3.2.18 TPC Report Element	13

7.3.2.21 Measurement Request element	14

7.3.2.22 Measurement Report element	26

7.3.2.27 QBSS Load element	38

7.3.2.35 AP Channel Report element	39

7.3.2.36 Neighbor Report element	40

7.3.2.37 RCPI element	43

7.3.2.38 BSS Load element	43

7.3.2.39 Antenna Information element	44

7.3.2.40 RSNI element	44

7.4 Action frame format details	45

7.4.5 Radio Measurement action details	46

7.4.5.1 Radio Measurement Request frame format	47

7.4.5.2 Radio Measurement Report frame format	47

7.4.5.3 Link Measurement Request Frame Format	48

7.4.5.4 Link Measurement Report Frame Format	48

7.4.5.5 Neighbor Report Request frame format	49

7.4.5.6 Neighbor Report Response frame format	50

10. Layer management	50

10.3 MLME SAP Interface	50

10.3.2 Scan	50

10.3.2.2 MLME-SCAN.confirm	50

10.3.6 Associate	53

10.3.6.3 MLME-ASSOCIATE.indication	53

10.3.6.4 MLME-ASSOCIATE.response	54

10.3.7 Reassociate	55

10.3.7.3 MLME-REASSOCIATE.indication	55

10.3.7.4 MLME-REASSOCIATE.response	56

10.3.11 Spectrum Management pProtocol layer model for spectrum management & radio measurement	57

10.3.12	Measurement Request	59

10.3.12.1 MLME-MREQUEST.request	59

10.3.12.3 MLME-MREQUEST.indication	60

10.3.14 Measurement Report	61

10.3.14.1 MLME-MREPORT.request	61

10.3.14.3 MLME-MREPORT.indication	62

10.3.29 Neighbor Report Request	63

10.3.29.1 MLME-NEIGHBORREPREQ.request	63

10.3.29.2 MLME-NEIGHBORREPREQ.confirm	64

10.3.29.3 MLME-NEIGHBORREPREQ.indication	66

10.3.30 Neighbor Report Response	66

10.3.30.1 MLME-NEIGHBORREPRESP.request	66

10.3.30.2 MLME-NEIGHBORREPRESP.confirm	67

10.3.30.3 MLME-NEIGHBORREPRESP.indication	68

10.3.31 Link Measure Request	69

10.3.31.1 MLME-LINKMEASURE.request	69

10.3.31.2 MLME-LINKMEASURE.confirm	70

10.3.32 Measurement Pilot Link Margin Ceiling request	71

10.3.32.1 MLME-MPILOTLMCEILING.request	72

10.3.32.2 MLME-MPILOTLMCEILING.confirm	72

11. MAC sublayer management entity	73

11.1 Synchronization	73

11.1.3 Acquiring synchronization, scanning	73

11.1.3.2 Active scanning	73

11.9 TPC procedures	74

11.9.2 Specification of regulatory and local maximum transmit power levels	75

11.11 Radio Measurement Procedures	76

11.11.1 Measurement on serving and non-serving channels	76

11.11.2 Measurement start time	76

11.11.3 Measurement Duration	77

11.11.4 Station responsibility for conducting measurements	77

11.11.5 Requesting and reporting of measurements	77

11.11.6 Repeated measurement request frames	79

11.11.7 Triggered Autonomous Reporting	80

11.11.8 Specific measurement usage	81

11.11.8.1 Beacon Report	81

11.11.8.2 Frame Report	83

11.11.8.3 Channel Load Report	84

11.11.8.4 Noise Histogram Report	84

11.11.8.5 STA Statistics Report	84

11.11.8.6 Location Configuration Information Report	85

11.11.8.7 Measurement Pause	85

11.11.8.8 QoS Metrics Report	86

11.12 Usage of the Neighbor Report	87

11.12.1 Requesting a Neighbor Report	87

11.12.2 Receiving a Neighbor Report	87

11.13 Link Measurement	88

11.14 Measurement Pilot generation and usage	88

11.14.1 Measurement Pilot generation by an AP	88

11.14.2 Measurement Pilot Link margin ceiling calculation	88

12. PHY service specification	89

12.3 Detailed PHY service specifications	89

12.3.4 Basic service and options	89

12.3.4.3 PHY-SAP service primitives parameters	89

12.3.5 PHY-SAP detailed service specification	89

12.3.5.8 PHY-CCARESET.request	89

12.3.5.9 PHY-CCARESET.confirm	90

12.3.5.10 PHY-CCA.indication	90

12.3.5.11 PHY-RXSTART.indication	91

12.3.5.12 PHY-RXEND.indication	91

15. DSSS PHY specification for the 2.4 GHz band designated for ISM applications	91

15.2 DSSS PLCP sublayer	91

15.2.7 Receive PLCP	91

15.4 DSSS PMD sublayer	92

15.4.4 Basic service and options	92

15.4.4.2 PMD_SAP peer-to-peer service primitive parameters	92

15.4.4.3 PMD_SAP sublayer-to-sublayer service primitives	93

15.4.4.4 PMD_SAP service primitive parameters	93

15.4.5 PMD_SAP detailed service specification	93

15.4.5.16 PMD_RCPI.indicate	93

15.4.8 PMD receiver specifications	94

15.4.8.5 Received Channel Power Indicator (RCPI) Measurement	94

17. Orthogonal frequency division multiplexing (OFDM) PHY specification for the 5 GHz band	94

17.2 OFDM PHY specific service parameter list	94

17.2.3 RXVECTOR parameters	94

17.2.3.5 RXVECTOR RCPI	95

17.3 OFDM PLCP sublayer	95

17.3.10 PMD receiver specifications	95

17.3.10.6 Received Channel Power Indicator (RCPI) Measurement	95

17.3.12 Receive PLCP	95

17.5 OFDM PMD sublayer	96

17.5.4 Basic service and options	96

17.5.4.2 PMD_SAP sublayer-to-sublayer service primitives	96

17.5.4.3 PMD_SAP service primitive parameters	97

17.5.5 PMD_SAP detailed service specification	97

17.5.5.8 PMD_RCPI.indicate	97

18. High Rate direct sequence spread spectrum (HR/DSSS) PHY specification	98

18.2 High Rate PLCP sublayer	98

18.2.6 Receive PLCP	98

18.3 High Rate PLME	100

18.3.5 Vector descriptions	100

18.4 High Rate PMD sublayer	100

18.4.4 Basic service and options	100

18.4.4.2 PMD_SAP sublayer-to-sublayer service primitives	100

18.4.5 PMD_SAP detailed service specification	100

18.4.5.16 PMD_RCPI.indicate	100

18.4.8 PMD receiver specifications	101

18.4.8.5 Received Channel Power Indicator (RCPI) Measurement	101

19. Extended Rate PHY specification	102

19.2 PHY specific service parameter list	102

19.9 Extended Rate PMD sublayer	102

19.9.4 Basic service and options	102

19.9.4.2 PMD_SAP sublayer-to-sublayer service primitives	102

19.9.4.3 PMD_SAP service primitive parameters	102

19.9.5 PMD_SAP detailed service specification	103

19.9.5.14 PMD_RCPI.indicate	103

Annex A	103

A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9	103

A.4.3 IUT configuration	103

A.4.13 Radio Resource Measurement extensions	103

Annex D	108

Annex I	157

I.1 External regulatory references	157

Annex J	160

# Draft Standard for
Information Technology—
Telecommunications and information exchange between systems—
Local and metropolitan area networks—
Specific Requirements

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

Amendment 1: Radio Resource Measurement

[This amendment is based on IEEE Std 802.11™, 2006 Revision Draft 7.0.]

NOTE—The editing instructions contained in this amendment define how to merge the material contained herein into the existing base standard to form the new comprehensive standard.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert and replace. Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make large changes in existing text, subclauses, tables or figures by removing existing material and replacing it with new material. Editorial notes will not be carried over into future editions.

# 2. Normative references

Insert the following new definition in alphabetical order:

IETF RFC 3825, Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information, Polk, J., Schnizlein, J., Linsner, M., July 2004

# 3. Definitions

Insert the following new definitions:

3.7A AP reachablility: An AP is reachable by a STA if pre-authentication messages as defined in  8.4.6.1 can be exchanged between the STA and the target AP via the DS.

3.15A average noise power indicator (ANPI):  An indication of the average noise plus interference power measured when the channel is idle, i.e. when virtual CS mechanism indicates idle channel but excluding periods of frame transmission or reception.

3.15B azimuth: The horizontal orientation of the front surface of a station or of a radio’s main beam measured clockwise from True North.

3.21A idle power indicator: An indication of the total channel power (noise and interference) as measured in the channel at the receiving antenna connector while the STA is idle, i.e. neither transmitting nor receiving a frame.

3.31A antenna connector: the logical measurement point of reference for RF measurements in a STA. The antenna connector is the point in the STA architecture representing the input of the receiver (output of the antenna) for radio reception and the input of the antenna (output of the transmitter) for radio transmission. For radio reception, the receive power level at the antenna connector is amplified by the gain of the antenna with respect to a 0db omnidirectional reference antenna. In systems using multiple antennas or antenna arrays, the antenna connector is a virtual point representing the aggregate of the multiple antennas. In systems using antenna arrays with active array processing, the antenna connector is the output of the active array which includes any processing gain of the active antenna subsystem.

3.75A LCI subject local: The LCI request is for the location of the requesting STA, asking “Where am I?”

3.75B LCI subject remote: The LCI request is for the location of the reporting STA, asking “Where are you?”

3.79A location configuration information (LCI): As defined in IETF RFC 3825, includes latitude, longitude, and altitude, with resolution indicators for each.

3.86A neighbor AP: Any AP that is a potential transition candidate.

3.88A non-operating channel: A channel that is not the operating channel of the BSS of which the STA is a member.

3.92A operating channel: The operating channel is the channel specified by the current channel value in the DS Parameter Set element of the MLME-JOIN.request or MLME-START.request primitives. If neither of these primitives have been issued by the SME then the operating channel value is 0.

3.120A received channel power indicator (RCPI): An indication of the total channel power (signal, noise, and interference) of a received IEEE 802.11 frame measured on a single channel and at the antenna connector used to receive the frame.

3.121A received signal to noise indicator (RSNI): An indication of the signal to noise plus interference ratio of a received IEEE 802.11 frame. RSNI is defined by the ratio of the received signal power (RCPI-ANPI) to the noise plus interference power (ANPI) as measured on a single channel and at the antenna connector used to receive the frame.

3.131A serving AP: An AP which transmits beacons on the operating channel.

3.161A validated AP: an AP that has either been explicitly configured as a Neighbor in the MIB, or learned through a mechanism like the Beacon Report and confirmed through a trusted mechanism. The specification of a trusted mechanism is outside the scope of this standard.

3.166A wildcard SSID: A special value (null) for SSID used to represent all SSIDs.

NOTE—See 7.3.2.1.

# 4. Abbreviations and acronyms

Insert the following new acronyms in alphabetical order:

ANPI		average noise power indicator
IPI		idle power indicator
LCI		location configuration information
TMPTT		target measurement pilot transmission time
RCPI		received channel power indicator
RSNI		received signal to noise indicator
RFC		request for comments

# 5. General description

## 5.2 Components of the IEEE 802.11 architecture

Insert the following new clause after 5.2.6

### 5.2.7 Wireless LAN Radio Measurements

Wireless LAN radio measurements enable applications in the stations to understand the radio environment in which they exist. Wireless LAN radio measurements enable stations to observe and gather data on radio link performance and on the radio environment. A station may choose to make measurements locally, or may be requested by another station to make one or more measurements and return the results. Radio measurement data is made available to station management and upper protocol layers where it may be used for a range of applications, for example, to adjust station operation to better suit the radio environment.

## 5.3 Logical Service Interfaces

Insert the item at the end of the list of architectural services in 5.3 as follows:

a)	Authentication

b)	Association

c)	Deauthentication

d)	Disassociation

e)	Distribution

f)	Integration

g)	Confidentiality

h)	Reassociation

i)	MSDU delivery

j)	DFS

k)	TPC
l)	Higher-layer timer synchronization (QoS facility only)

m)	QoS traffic scheduling (QoS facility only)

n)	Radio measurement

### 5.3.1 Station service (SS)

Insert the item at the end of the list of station services (SSs) in 5.3.1 as follows:

a)	Authentication
b)	Deauthentication
c)	Confidentiality
d)	MSDU delivery 
e)	DFS
f)	TPC
g)	Higher-layer timer synchronization (QoS facility only)
h)	QoS traffic scheduling (QoS facility only)
i)	Radio measurement

## 5.4 Overview of the services

Insert the following sentence at the end of the first paragraph:

One of the services is used for radio measurement.

Insert the following new clause after  5.4.6:

### 5.4.7 Radio Measurement Service

The Radio Measurement Service provides:

- The ability to request and report radio measurements in supported channels.

- The ability to perform radio measurements in supported channels.

- An interface for upper layer applications to retrieve radio measurements using MLME primitives and/or MIB access.

- Information about neighbor APs.

## 5.6 Relationships between services

Add items to the list as shown below:

a)	Class 1 frames (permitted from within States 1, 2, and 3):

2)	Management frames

i)	Probe request/response

ii)	Beacon

iii)	Authentication:  Successful authentication enables a STA to exchange Class 2 frames. Unsuccessful authentication leaves the STA in State 1.

iv)	Deauthentication:  Deauthentication notification when in State 2 or State 3 changes the STA’s state to State 1. The STA shall become authenticated again prior to sending Class 2 frames.

v)	Announcement traffic indication message (ATIM)

vi)	Spectrum Management Action
Within an IBSS, action frames are class 1.

- Measurement Pilot

Change the list to add item c.2.ii as shown below:

c)	Class 3 frames (if and only if associated; allowed only from within State 3):

2)	Management frames

i)	QoS, DLS and Block Ack Action

ii)	Radio Measurement Action outside of an IBSS

# 7. Frame formats

## 7.1 MAC frame formats

### 7.1.3 Frame fields

#### 7.1.3.1 Frame Control field

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as shown:

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0101 | Probe response |
| 00 | Management | 0110-0111 | ReservedMeasurement Pilot |
| 00 | Management | 0111 | Reserved |
| 00 | Management | 1000 | Beacon |

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the first paragraph, and table 8 as shown:

The frame body of a management frame of subtype Beacon contains the information shown in Table 8. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall include a Country information element in the transmission of Beacon frames. Optionally, the Beacon frame format may also include the information described in either or both of FH Parameters and FH Pattern Table elements. If both FH Parameters and FH Pattern Table elements are sent, they shall describe the same hopping pattern. Note that the information described in FH Parameters and FH Pattern Table elements also may be contained in the Probe Response frame.

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | The Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 14 | Power Constraint | The Power Constraint element shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 16 | Quiet | Quiet element may be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 26 | AP Channel Report | The AP Channel Report element shall be present if dot11RadioMeasurementEnabled is true and there is at least 1 channel to report. |
| 27 | BSS Load | The BSS Load information element shall be present only in a non-QAP and only if dot11RadioMeasurementEnabled is true. |
| 28 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |
| 29 | QBSS Available Admission Capacity | The QBSS AAC information element shall be present only in a QAP and if dot11RadioMeasurementEnabled is true. |
| 30 | QBSS Access Delay | The QBSS Access Delay element shall be present only in a QAP and if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.4 Association Request frame format

Change the order 6 information field as follows:

Table 10—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |

#### 7.2.3.5 Association Response frame format

Insert new rows into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 9 | RSNI | The RSNI information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.6 Reassociation Request frame format

Change the order 7 information field as follows:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Power Capability | The Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new rows into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 9 | RSNI | The RSNI information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.8 Probe Request frame format

Insert new row 6 as follows:

Table 14—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | DS Parameter Set | The DS Parameter Set information element shall be present within Probe Request frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs if dot11RadioMeasurementEnabled set to true. The DS Parameter Set information element may be present within Probe Request frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs if dot11RadioMeasurementEnabled set to false. |

#### 7.2.3.9 Probe Response frame format

Change the first paragraph as follows:

The frame body of a management frame of subtype Probe Response contains the information shown in Table 15. If the dot11MultiDomainCapabilityEnabled attribute is true, the Probe Response frame contains a Country information element and all information elements identified by the Requested Element IDs of a Request information element. Note that the information returned as a result of a Probe Request frame with a Request information element may include the FH parameters and/or the FH Pattern Table possibly replicating optional elements identified by orders 12 and 13. When a Probe Response frame is returned in response to a Probe Request frame which contained a Request information element, any of the requested elements that are listed in table 12 with a specific ordering shall be included in the Probe Response both in their listed ordered location as specified in table 12 and in the ordered location reserved for the list of requested elements, where the requested elements appear in the same order as requested in the Request information element.

A STA shall return only the information elements that it supports. In an improperly formed Request information element, a STA may ignore the first information element requested that is not ordered properly and all subsequent information elements requested. In the probe response frame, the STA shall return the requested information elements in the same order as requested in the Request information element.

Change the order 10 and up information fields, insert the order 24 and up information fields and change the starting order of the Requested information elements field in Table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | Shall be included if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 15 | Quiet | May be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | Shall be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 24 | AP Channel Report | Shall be included if dot11RadioMeasurementEnabled is true and there is at least 1 channel to report. |
| 25 | BSS Load | Shall be included only in a non-QAP and only if dot11RadioMeasurementEnabled is true. |
| 26 | Antenna Information | Shall be included if dot11RadioMeasurementEnabled is true. |
| 27 | Measurement Pilot Transmission Information | May be included if dot11RadioMeasurementEnabled is true. |
| 28 | QBSS Available Admission Capacity | The QBSS AAC information element shall be present only in a QAP and if dot11RadioMeasurementEnabled is true. |
| 29 | QBSS Access Delay | The QBSS Access Delay element shall be present only in a QAP and only if dot11RadioMeasurementEnabled is true. |
| 2430-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

Insert the following clause after  7.2.3.9:

#### 7.2.3.9A Measurement Pilot frame format

The frame body of a management frame of subtype Measurement Pilot contains the information shown in Table 15A.

Table 15A—Measurement Pilot frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Timestamp |  |
| 2 | Measurement Pilot Interval |  |
| 3 | Beacon Interval |  |
| 4 | Capability |  |
| 5 | Country String |  |
| 6 | Max Regulatory Power |  |
| 7 | Max Transmit Power |  |
| 8 | Transmit Power Used |  |
| 9 | Transceiver Noise Floor |  |
| 10 | DS Parameter Set | The DS Parameter Set information element is present within Measurement Pilot frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs. |
| 11 | Vendor Specific | One or more vendor specific information elements may appear in this frame. |

## 7.3 Management frame body components

### 7.3.1 Fixed fields

#### 7.3.1.4 Capability Information field

Change the contents of Figure 27 as follows:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| Spectrum Mgmt | QoS | Short Slot Time | APSD | Reserved Radio Measurement | DSSS-OFDM | Delayed Block Ack | Immediate Block Ack |

Figure 27—Capability Information fixed field

Insert the following text at the end of 7.3.1.4:

A STA shall set the Radio Measurement subfield in the Capability Information field to 1 when the MIB attribute dot11RadioMeasurementEnabled is true, otherwise it shall be set to 0.

#### 7.3.1.11 Action field

Insert the following new row into Table 24 and change the reserved value as shown:

Table 24—Category values

| Name | Value | See clause |
| --- | --- | --- |
| Radio measurement | 4 | 7.4.5 |
| Reserved | 45-127 | - |

Insert the following new clauses:

#### 7.3.1.18 Country String

The Country String field is 3 octets in length. The STA shall set this field to the value contained in the dot11CountryString attribute. The Country String field is illustrated in Figure 54A.

|  |  | B0 | B23 |
| --- | --- | --- | --- |
|  | Country String |  |  |
| Octets: | 3 |  |  |

Figure 54A—Country String fixed field

#### 7.3.1.19 Measurement Pilot Interval field

The Measurement Pilot Interval field represents the number of time units (TUs) between target measurement pilot transmission times (TMPTTs). The length of the Measurement Pilot Interval field is 2 octets. The Measurement Pilot Interval field is illustrated in Figure 54B.

|  |  | B0 | B15 |
| --- | --- | --- | --- |
|  | Measurement Pilot Interval |  |  |
| Octets: | 2 |  |  |

Figure 54B—Measurement Pilot Interval fixed field

#### 7.3.1.20 Max Regulatory Power field

The Max Regulatory Power field is a signed integer and is 1 octet in length. It shall indicate the maximum power, in units of dBm, that a STA is allowed to transmit on the current channel, as permitted by radio regulations for the domain identified by the Country String. As the method of measurement for maximum transmit power level differs by regulatory domain, the value in this field shall be interpreted according to the applicable regulations. The Max Regulatory Power field is illustrated in Figure 54C.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Max Regulatory Power |  |  |
| Octets: | 1 |  |  |

Figure 54C—Max Regulatory Power fixed field

#### 7.3.1.21 Max Transmit Power field

The Max Transmit Power field is a signed integer and is 1 octet in length. When set by a STA, it provides an upper limit, in units of dBm, on the transmit power as measured at the output of the antenna connector to be used by that STA on the current channel. The maximum tolerance for the value reported in Max Transmit Power field shall be ± 5 dB. The value of the Max Transmit Power field shall be less than or equal to the Max Regulatory Power value for the current channel. The Max Transmit Power field is illustrated in Figure 54D.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Max Transmit Power |  |  |
| Octets: | 1 |  |  |

Figure 54D—Max Transmit Power fixed field

#### 7.3.1.22 Transmit Power Used field

The Transmit Power Used field is a signed integer and is 1 octet in length. It shall be less than or equal to the Max Transmit Power and indicates the actual power used as measured at the output of the antenna connector, in units of dBm, by a STA when transmitting the frame containing the Transmit Power Used field. The Transmit Power Used value is determined anytime prior to sending the frame in which it is contained and has a tolerance of +/-5dB. The Transmit Power Used field is illustrated in Figure 54E.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Transmit Power Used |  |  |
| Octets: | 1 |  |  |

Figure 54E—Transmit Power Used fixed field

#### 7.3.1.23 Transceiver Noise Floor field

The Transceiver Noise Floor field is a signed integer and is 1 octet in length. It shall indicate the noise floor of the receiver used by the STA transmitting the measurement pilot frame in units of dBm with a tolerance of +/-5dB. The transceiver noise floor is referenced to the connector of the receiver antenna of the STA transmitting the frame that contains the Transceiver Noise Floor field. If the STA has multiple receiver antennas then the Transceiver Noise Floor is the lowest value from all the receiving antennas. The Transceiver Noise Floor field is illustrated in Figure 54F.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Transceiver Noise Floor |  |  |
| Octets: | 1 |  |  |

Figure 54F—Transceiver Noise Floor fixed field

### 7.3.2 Information Elements

Insert new Element IDs into Table 26 and change the reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| Reserved | 54-62 |
| RSNI | 63 |
| BSS Load | 64 |
| Antenna Information | 65 |
| Measurement Pilot Transmission Information | 66 |
| QBSS Available Admission Capacity | 67 |
| QBSS Access Delay | 68 |
| Reserved | 5169-126 |

#### 7.3.2.18 TPC Report Element

Change this clause as shown:

The TPC Report element contains transmit power and link margin information sent in response to a TPC Request element or a Link Measurement Request frame. A TPC Report element is included in a Beacon frame or Probe Response frame without a corresponding request. The format of the TPC Report element is shown in Figure 73.

|  | Element ID | Length | Transmit Power | Link Margin |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 73 - TPC Report element format

The Length field shall be set to 2.

The Transmit Power field shall be set to the transmit power used to transmit the frame containing the TPC Report element. The field is coded as a signed integer in units of decibels relative to 1 mW. The maximum tolerance for the transmit power value reported in the TPC Response element shall be +/- 5 dB. This tolerance is defined as the difference, in decibels, between the reported power value and the actual EIRP of the STA (measured when transmitting 1500 octet frames).

The Link Margin field contains the link margin at the time and for the rate at which the Link Measurement Request frame or the frame containing the TPC Request element was received. The field is coded as a signed integer in units of decibels. The Link Margin field shall be set 0 and shall be ignored when a TPC Report element is included in a Beacon frame or Probe Response frame. The measurement method of Link Margin is beyond the scope of this standard.

The TPC Report element is included in TPC Report frames, as described in 7.4.1.4; Link Measurement Report frames as described in 7.4.5.4; Beacon frames, as described in 7.2.3.1; and Probe Response frames, as described in 7.2.3.9. The use of TPC Report elements and frames is described in 11.9.4.

#### 7.3.2.21 Measurement Request element

Change  7.3.2.21 as follows:

The Measurement Request element contains a request that the receiving STA undertake the specified measurement action. The Measurement Request element is included in Spectrum Management Measurement Request frames as described in 7.4.1.1, or Radio Measurement Request frames as described in 7.4.5.1. Measurement Types 0, 1 and 2 are defined for spectrum management and shall only be included in Spectrum Management Measurement Request frames. The use of Measurement Request elements for spectrum management is described in 11.10.6. Measurement Types 3 through 9 and 255 are defined for radio measurement and shall only be included in Radio Measurement Request frames. The use of Measurement Request elements for radio measurement is described in 11.11.

The format of the Measurement Request element is shown in Figure 76.

|  | Element ID | Length | Measurement Token | Measurement Request Mode (see Figure 77) | Measurement Type | Measurement Request |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure 76—Measurement Request element format

|  | Reserved (0) Parallel | Enable | Request | Report | Duration Mandatory | Reserved (0) |
| --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4 | 45-7 |

Figure 77—Measurement Request Mode field

The value of the Length field is variable and depends on the length of the Measurement Request field. The minimum value of the Length field is 3 (based on a minimum length for the Measurement Request field of 0 octets).

The Measurement Token shall be set to a nonzero number that is unique among the Measurement Request elements sent in a particular Measurement Request frame.

The Measurement Request Mode field (shown in Figure 77) is a bit field with the following bits defined:

- The Parallel bit (bit 0) is used to request that more than one measurement is to be started in parallel. Parallel is set to 1 to request that the measurement is to start at the same time as the measurement described by the next Measurement Request element in the same Measurement Request frame. Parallel is set to 0 if the measurements are to be performed in sequence. The Parallel bit is reserved when Enable is set to 1, in the last or only measurement request element in the frame or when the value of the Measurement Type field is 0, 1, or 2 (Spectrum Management measurements). See 11.11.6.

- The Enable bit (bit 1) is used to differentiate between
a request to make a measurement and
a request to control the measurement requests and triggered or autonomous reports generated by the destination STA.

- Enable bit (bit 1) indicates whether this element is used to request the destination STA to enable or disable the sending of measurement requests and autonomous measurement reports of a specified type to this STA. The Enable bit shall be set to 1 when the Request bit and Report bit are valid. The Enable bit shall be set to 0 when the Request bit and Report bit are invalid.

- Request bit (bit 2) indicates whether the STA receiving the request shall enable or disable measurement requests of the type specified in the Measurement Type field. The Request bit shall be set to 1 when enabling a measurement request. The Request bit shall be set to 0 when disabling a measurement request or when the Request bit is invalid (i.e. when Enable bit is set to 0 or when the Measurement Type field contains a reserved measurement request type value).

- Report bit (bit 3) indicates whether the STA receiving the request shall enable or disable autonomous measurement reports of the type specified in the Measurement Type field. The Report bit shall be set to 1 when enabling an autonomous measurement report. The Report bit shall be set to 0 when disabling an autonomous measurement report or when the Report bit is invalid (i.e. when Enable bit is set to 0 or when the Measurement Type field contains a reserved measurement report type value).

- The Duration Mandatory bit (bit 4) indicates whether the measurement duration contained within the Measurement Request should be interpreted as mandatory by the STA receiving the request. A value of 0 shall indicate that the duration requested is a target duration, but the requesting STA shall accept measurement results taken over a shorter duration. A value of 1 shall indicate that the duration requested is a mandatory duration. The Duration Mandatory bit is reserved when the Enable bit is set to 1, or when the value of the Measurement Type field is 0, 1, 2, 8 or 255. See 11.11.2.

- All other bits are reserved and shall be set to 0.

The use of the Enable, Request and Report bits is also summarized in Table 28. See 11.10.6 and 11.11.5 for the description of how a STA shall handles requests to enable or disable measurement requests and autonomous reports. See 11.11.7 for a description of the use of the Enable and Report bits in triggered reporting.

Table 28—Summary of use of Enable, Request and Report bits

| Bits |  |  | Meaning of bitsMeasurement request meaning |
| --- | --- | --- | --- |
| Enable | Request | Report |  |
| 0 | 0Reserved | 0Reserved | The transmitting STA is requesting that the destination STA make a Measurement of type indicated in the Measurement Type field. When Enable is set to 0, Request and Report bits are reserved invalid and shall be set to 0. |
| 0 | 0 | 1 | Not allowed |
| 0 | 1 | 0 | Not allowed |
| 0 | 1 | 1 | Not allowed |
| 1 | 0 | 0 | The transmitting STA is requesting that the destination STA sendsit is sent neither measurement requests nor triggered or autonomous measurement reports of the types indicated in the Measurement Type field. |
| 1 | 1 | 0 | The transmitting STA is indicating to the destination STA that it will may accept measurement requests and requesting it is not be sent triggered or autonomous measurement reports of the types indicated in the Measurement Type field. Note: This setting corresponds to the default STA behavior. |
| 1 | 0 | 1 | The transmitting STA is requesting that the destination STA it not sendbe sent measurement requests and indicating it will accept autonomous measurement reports of the types indicated in the Measurement Type field or is requesting triggered reporting. |
| 1 | 1 | 1 | The transmitting STA is indicating to the destination STA that it will may accept measurement requests and either will accept autonomous measurement reports of the type indicated in the Measurement Type field or is requesting triggered reporting. |

The Measurement Type field shall be set to a number that identifies a type of measurement request or a measurement report. Those Measurement Types that have been allocated for measurement requests are shown in Table 29 and measurement reports are shown in Table 30 (in 7.3.2.22). A measurement report type is used when enabling, or disabling an autonomous report.

Table 29—Measurement Type definitions for measurement requests

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic Request | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) request | 1 |  |
| Receive power indication (RPI) histogram request | 2 |  |
| Channel load request | 3 | Radio Resource Measurement |
| Noise histogram request | 4 |  |
| Beacon request | 5 |  |
| Frame request | 6 |  |
| STA statistics request | 7 |  |
| LCI request | 8 |  |
| QoS metrics request | 9 |  |
| Reserved | 310-254 | N/A |
| Measurement Pause request | 255 | Radio Resource Measurement |

The Measurement Request field shall be null when the Enable bit is set to 1 and shall contain the specification of the measurement request, as described in 7.3.2.19.1 through 7.3.2.19.3, when the Enable bit is set to 0.

When the Enable bit is set to 0 the Measurement Request field contains the specification of a single measurement request corresponding to the Measurement Type as described in 7.3.2.21.1 through 7.3.2.21.11. When the Enable bit is set to 1, the Measurement Request field is only present when requesting a triggered measurement.

The Measurement Request element is included in a Measurement Request frame as described in 7.4.1.1. The use of Measurement Request elements and frames is described in 11.10.6.

Insert the following new clauses after  7.3.2.21.3:

##### 7.3.2.21.4 Channel Load Request

The Measurement Request field corresponding to a Channel Load Request is shown in Figure 80A.

|  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 |

Figure 80A—Measurement Request field format for a Channel Load Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TUs. See 11.11.2.

The Measurement Duration field shall be set to the preferred or mandatory duration of the requested measurement, expressed in TUs. See 11.11.3.

##### 7.3.2.21.5 Noise Histogram Request

The Measurement Request field corresponding to a Noise Histogram Request is shown in Figure 80B.

|  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 |

Figure 80B—Measurement Request field format for a Noise Histogram Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TUs. See 11.11.2.

The Measurement Duration field shall be set to the preferred or mandatory duration of the requested measurement, expressed in TUs. See 11.11.3.

##### 7.3.2.21.6 Beacon Request

The Measurement Request field corresponding to a Beacon Request is shown in Figure 80C.

|  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration | Measurement Mode |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 | 1 |

|  | BSSID |  | Reporting Condition |  | Threshold/Offset (optional) |  | SSID element |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 6 |  | 1 |  | 1 |  | 2-34 |

Figure 80C—Measurement Request field format for a Beacon Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Annex J. A Channel Number of 0 indicates a request to make iterative measurements for all supported channels in the Regulatory Class where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. A Channel Number of 255 indicates a request to make iterative measurements for all supported channels listed in the AP Channel

Report for the Regulatory Class. The procedures for iterative measurements on multiple channels are described in 11.11.8.1.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TUs. See 11.11.2.

The Measurement Duration field shall be set to the preferred or mandatory duration of the requested measurement, expressed in TUs. See 11.11.3.

Measurement Mode indicates the mode to be used for the measurement. The valid measurement modes are listed in Table 29A. The procedures for each mode are described in 11.11.8.1.

Table 29A—Measurement Mode definitions for Beacon Request element

| Mode | Value |
| --- | --- |
| Passive | 0 |
| Active | 1 |
| Beacon Table | 2 |
| Reserved | 3-255 |

The BSSID field indicates the BSSID of the BSS(s) for which a beacon report is requested. When requesting beacon reports for all BSSs on the channel, the BSSID field contains the wildcard BSSID; otherwise the BSSID field contains a specific BSSID for a single BSS.

The Reporting Condition defines when the measured results are to be reported to the requesting STA. The Reporting Condition values are defined in Table 29B. Non-zero values for Reporting Condition may be used only for repeated measurements. Reporting Condition shall be set to 0 for single measurements or when measurement mode is Beacon Table. Reporting Condition shall be limited to the range 0 to 4 inclusive when requesting measurements in an IBSS. Procedures for Reporting Conditions are described in 11.11.8.1.

Table 29B—Reporting Condition definitions for Beacon Request element

| Condition for report to be issued in Repeated Measurement | Reporting Condition |
| --- | --- |
| Report to be issued after each measurement. | 0 |
| The measured RCPI level is greater than an absolute threshold. | 1 |
| The measured RCPI level is less than an absolute threshold. | 2 |
| The measured RSNI level is greater than an absolute threshold. | 3 |
| The measured RSNI level is less than an absolute threshold. | 4 |
| The measured RCPI level is greater than a threshold defined by an offset from the serving AP's reference RCPI. | 5 |
| The measured RCPI level is less than a threshold defined by an offset from the serving AP’s reference RCPI. | 6 |
| The measured RSNI level is greater than a threshold defined by an offset from the serving AP’s reference RSNI. | 7 |
| The measured RSNI level is less than a threshold defined by an offset from the serving AP’s reference RSNI. | 8 |
| The measured RCPI level is in a range bound by the serving AP’s reference RCPI and an offset from the serving AP’s reference RCPI. | 9 |
| The measured RSNI level is in a range bound by the serving AP’s reference RSNI and an offset from the serving AP’s reference RSNI. | 10 |
| Reserved | 11-255 |

Threshold/Offset provides either the threshold value or the offset value to be used for conditional reporting. Threshold/Offset is not included when the Reporting Condition is 0. Threshold/Offset is always included when the Reporting Condition is non-zero. For Reporting Conditions 1 to 4, the threshold value is an unsigned 8 bit integer in units of 0.5 dBm. For Reporting Conditions 5, 6, and 9, the offset value is an 8 bit two’s complement integer in units of 0.5 dBm. For Reporting Conditions 7, 8 and 10 the offset value is an 8 bit two’s complement integer in units of 0.5 dB.

The SSID element indicates the ESS(s), or IBSS(s) for which a beacon report is requested. When requesting beacon reports for all ESSs on the channel, the SSID field contains the 'wildcard SSID'; otherwise the SSID field contains a specific SSID for a single ESS or IBSS. The 'wildcard SSID' is defined as a zero length (null) SSID used to represent all possible SSIDs. The SSID element is described in 7.3.2.1.

##### 7.3.2.21.7 Frame Request

The Measurement Request field corresponding to a Frame Request is shown Figure 80D.

|  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration | MAC Address (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 | 6 |

Figure 80D—Measurement Request field format for a Frame Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TUs. See 11.11.2.

The Measurement Duration field shall be set to the preferred or mandatory duration of the requested measurement, expressed in TUs. See 11.11.3.

If the MAC Address field is included only frames matching this MAC address as the Transmitter Address, shall be counted towards the Frame Report generated in response to this Frame Request.

##### 7.3.2.21.8 STA Statistics Request

The Measurement Request field corresponding to a STA Statistics Request is shown in Figure 80E.

|  | Randomization Interval | Measurement Duration | Group Identity |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure 80E—Measurement Request field format for a STA Statistics Request

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TUs. See 11.11.2.

The Measurement Duration field shall be set to the duration of the requested measurement in TUs.

Group Identity indicates the requested statistics group according to Table 29C.

Table 29C—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| STA Counters from dot11MacStatistics group | 1 |
| QSTA Counters from dot11QosCountersTable | 2 |
| BSS Load as described in 7.3.2.38 and 7.3.2.42 | 3 |
| Reserved | 4 – 255 |

##### 7.3.2.21.9 Location Configuration Indication (LCI) Request

The Measurement Request field corresponding to an LCI request is shown in Figure 80F.

|  | Location Subject | Latitude Requested Resolution | Longitude Requested Resolution | Altitude Requested Resolution | Azimuth Request (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure 80F—LCI Request field format

The Location Subject field of a LCI request is a single octet. See Table 29D.

Table 29D—Location subject definition

| Value | Location Subject |
| --- | --- |
| 0 | LCI Subject Local |
| 1 | LCI Subject Remote |
| 2-255 | Reserved |

The term ‘Local’ refers to the location of the requesting STA, and ‘Remote’ refers to the location of the reporting STA.

NOTE—Local LCI Measurement Request is used by requesting STA to obtain its own location, asking "Where am I?". Remote LCI Measurement Request is used by requesting STA to obtain the location of the reporting STA, asking "Where are you?".

Latitude Requested Resolution is the number of valid most significant bits requested for the fixed-point value of Latitude in degrees. Values above 34 (decimal), the specified maximum number of bits of Latitude, are reserved.

Longitude Requested Resolution is the number of valid most significant bits requested for the fixed-point value of Longitude in degrees. Values above 34 (decimal), the specified maximum number of bits of Longitude, are reserved.

Altitude Requested Resolution is the number of valid most significant bits requested for the Altitude, which has either of two types, as described in 7.3.2.22.9. Values above 30 (decimal), the specified maximum number of bits of Altitude, are reserved.

The Azimuth Request field of a LCI request is a single octet. See Figure 80G.

|  | B0-B3 | B4 | B5 | B7 |
| --- | --- | --- | --- | --- |
|  | Azimuth Resolution Requested | Azimuth Type | Reserved |  |
| Bits: | 4 | 1 | 3 |  |

Figure 80G—Azimuth Request field

Azimuth Resolution Requested is the number of valid most significant bits requested for the fixed-point value of Azimuth, reported in integer degrees. Values above 9 are reserved.

Azimuth Type (bit 4) is set to 1 to request a report of the Azimuth of radio reception, and is set to 0 to request a report of the Azimuth of front face of the reporting STA.

NOTE—A geographic feature is "an abstraction of a real world phenomenon; it is a geographic feature if it is associated with a location relative to the Earth”. The designation of a horizontal plane is relative to the Earth. The designation of the ‘front surface’ of a station is arbitrary, but refers to an orientable surface (possessing a centerline) of the station. It is common to use a direction cosine matrix to convert from one coordinate system to another, i.e. body-centered coordinates to earth-centered coordinates.

##### 7.3.2.21.10 QoS Metrics Request

The Measurement Request field corresponding to a QoS Metrics Request is shown in Figure 80H. A response to a QoS Metrics Request is a QoS Metrics Report.

|  | Randomization Interval | Measurement Duration | Peer QSTA Address | Traffic Identifier | Bin 0 Range | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 1 | 1 | 6 |

Figure 80H—Measurement Request field format for a QoS Metrics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.11.2. When requesting a triggered QoS metrics measurement Randomization Interval is not used and shall be set to 0 – see 11.11.8.8.

The Measurement Duration shall be set to the duration of the requested measurement, expressed in TUs except when setting up a triggered QoS measurement, when it is not used and shall be set to 0.

The Peer QSTA Address contains a MAC address indicating the RA address in the measured MSDUs.

The Traffic Identifier field contains the TID subfield as shown in Figure 80I.

|  | B0 | B3 | B4 | B7 |
| --- | --- | --- | --- | --- |
|  | Reserved |  | TID |  |
| Bits: | 4 |  | 4 |  |

Figure 80I—Traffic Identifier field

The TID subfield indicates the TC or TS for which traffic is to be measured.

Bin 0 Range shall indicate the delay range of the first bin (Bin 0) of the Transmit Delay Histogram, expressed in TUs. The Bin 0 Range value is used to calculate the delay ranges of the other 5 bins making up the histogram. The delay range for each bin shall increase in a binary exponential fashion as described in 7.3.2.22.10.

The Triggered Reporting field is used to specify measurement trigger thresholds. It is only present if requesting triggered QoS metrics reporting. The Triggered Reporting field is as shown in Figure 80J.

|  | Trigger Conditions | Average Error Threshold | Consecutive Error Threshold | Delay Threshold | Measurement Count | Trigger Timeout |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 80J—Triggered Reporting Field

Trigger Conditions is a bit-field that specifies reporting triggers when requesting a triggered QoS metrics measurement. The format of the Trigger Conditions bit-field is shown in Figure 80K.

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average | Consecutive | Delay | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure 80K—Trigger Conditions bit-field

- Average is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the TID that are discarded out of the number of preceding MSDUs specified in Measurement Count is greater than or equal to the value given in Average Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit, or due to the MSDU lifetime having been reached shall be counted.

- Consecutive is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the TID that are discarded in succession is greater than or equal to the value given in Consecutive Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit, or due to the MSDU lifetime having been reached shall be counted.

- Delay is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the TID that experience a transmit delay greater than or equal to the value specified in the Delay Threshold subfield is greater than or equal to the value given in Delayed MSDU Count. Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the entire MSDU has been successfully transmitted, including receipt of the final ACK from the peer QSTA if the QoSAck service class is being used.

The Average Error Threshold field contains a value representing the number of discarded MSDUs to be used as the threshold value for the Average trigger condition.

The Consecutive Error Threshold field contains a value representing the number of discarded MSDUs to be used as the threshold value for the Consecutive trigger condition.

The Delay Threshold field contains two subfields as shown in Figure 80L.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Delayed MSDU Range |  | Delayed MSDU Count |  |
| Bits: | 2 |  | 6 |  |

Figure 80L—Delay Threshold subfield

Delayed MSDU Range contains a value representing the MSDU transmit delay at or above which an MSDU will be counted towards the Delayed MSDU Count threshold. Delayed MSDU Range is encoded as a value representing the lower bound of a bin in the Transmit Delay Histogram as shown in Table 29E. The Transmit Delay Histogram is defined in 7.3.2.22.10.

Table 29E—Delayed MSDU Range Definitions

| Delayed MSDU Range | Condition |
| --- | --- |
| 0 | Transmit Delay ≥ Lower Bound of Bin 2 |
| 1 | Transmit Delay ≥ Lower Bound of Bin 3 |
| 2 | Transmit Delay ≥ Lower Bound of Bin 4 |
| 3 | Transmit Delay ≥ Lower Bound of Bin 5 |

Delayed MSDU Count contains a value representing the number of MSDUs to be used as the threshold value for the Delay trigger condition.

The Measurement Count field contains a number of MSDUs. This value is used to calculate an average discard count for the Average trigger condition. It is also used in place of measurement duration in determining the scope of the reported results when a report is triggered – see 11.11.8.8.

The Trigger Timeout field contains a value in units of 100TU during which a measuring STA shall not generate further triggered QoS metrics reports after a trigger condition has been met. See 11.11.8.8.

##### 7.3.2.21.11 Measurement Pause Request

The Measurement Request field corresponding to a Measurement Pause Request is shown in Figure 80M. The Measurement Pause Request cannot be processed in parallel with any other Measurement Request. See 11.11.8.7.

|  | Pause Time |
| --- | --- |
| Octets: | 2 |

Figure 80M—Measurement Pause Request field format

The Pause Time field contains a number between 1 and 65535 representing the time period for which measurements are suspended or paused. The time unit for the Pause Time is 10 TUs. The Pause Time value 0 is reserved. Measurement Pause Requests are used to provide time delays between the execution times of measurement request elements in a Measurement Request frame.

#### 7.3.2.22 Measurement Report element

Change  7.3.2.22 as follows:

The Measurement Report element contains a measurement report. The format of the Measurement Report element is shown in Figure 81. The Measurement Report element is included in Spectrum Management Measurement Report frames as described in 7.4.1.2, or Radio Resource Measurement Report frames as described in 7.4.5.2. Measurement Types 0, 1 and 2 are used for spectrum management and shall only be included in spectrum management Measurement Report frames. All other Measurement Types are used for radio resource measurement and shall only be included in Radio Resource Measurement Report frames. The use of Measurement Report elements and frames for spectrum management is described in 11.10.6. The use of Measurement Report elements and frames for radio measurement is described in 11.11.

|  | Element ID | Length | Measurement Token | Measurement Report Mode (see Figure 82) | Measurement Type | Measurement Report |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure 81—Measurement Report element format

|  | Late | Incapable | Refused | Reserved |
| --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3-7 |

Figure 82—Measurement Report Mode field

The value of the Length field is variable and depends on the length of the Measurement Report field. The minimum value of the Length field is 3.

The Measurement Token field shall be set to the Measurement Token in the corresponding Measurement Request element. If the Measurement Report element is being sent autonomously then the Measurement Token shall be set to 0.

The Measurement Report Mode field (shown in Figure 82) is used to indicate the reason for a failed or rejected measurement request. The Measurement Report Mode is a bit field with the following bits defined:

- Late bit (bit 0) indicates whether this STA is unable to carry out a measurement request because it received the request after the requested measurement time. The Late bit shall be set to 1 to indicate the request was too late. The Late bit shall be set to 0 to indicate the request was received in time for the measurement to be executed. The Late bit only applies to spectrum management measurement and shall be set to 0 in all measurement report elements for radio resource measurement types (see Table 30).

- Incapable bit (bit 1) indicates whether this STA is incapable of generating a report of the type specified in the Measurement Type field that was previously requested by the destination STA of this Measurement Report element. The Incapable bit shall be set to 1 to indicate the STA is incapable. The Incapable bit shall be set to 0 to indicate the STA is capable or the report is autonomous.

- Refused bit (bit 2) indicates whether this STA is refusing to generate a report of the type specified in the Measurement Type field that was previously requested by the destination STA of this Measurement Report element. The Refused bit shall be set to 1 to indicate the STA is refusing. The Refused bit shall be set to 0 to indicate the STA is not refusing or the report is autonomous.

- All other bits are reserved and shall be set to 0.

Not more than one bit shall be set within a Measurement Report Mode field. All bits within the Measurement Mode field shall be set to 0 if the results of a successful measurement request, or an autonomous measurement are being reported.

The Measurement Type field shall be set to a number that identifies the measurement report. Those Measurement Types that have been allocated are shown in Table 30.

The Measurement Report field shall be null not be present when the Late bit is set to 1, the Incapable bit is set to 1 or the Refused bit is set to 1. Otherwise, it shall contain the specification of thea single measurement report, as described in 7.3.2.20.17.3.2.22.1 through 7.3.2.20.37.3.2.22.10.

Table 30—Measurement Type definitions for measurement reports

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic report | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) report | 1 |  |
| Receive power indication (RPI) histogram report | 2 |  |
| Channel load report | 3 | Radio Resource Measurement |
| Noise histogram report | 4 |  |
| Beacon report | 5 |  |
| Frame report | 6 |  |
| STA statistics report | 7 |  |
| LCI report | 8 |  |
| QoS metrics report | 9 |  |
| Reserved | 310-255 | N/A |

The Measurement Report element is included in a Measurement Report frame as described in 7.4.1.2. The use of Measurement Report elements and frames is described in 11.6.6.

Insert the following new clauses after  7.3.2.22.3:

##### 7.3.2.22.4 Channel Load Report

The format of the Measurement Report field corresponding to a Channel Load Report is shown in Figure 86A.

|  | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Channel Load |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 1 |

Figure 86A—Measurement Report field format for a Channel Load Report

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set to the duration over which the Channel Load Report was measured, expressed in TUs.

Channel Load shall contain the proportion of measurement duration for which the measuring STA determined the channel to be busy. Procedure for Channel Load measurement and definition of channel load values are found in 11.11.8.3.

##### 7.3.2.22.5 Noise Histogram Report

The format of the Measurement Report field of a Noise Histogram Report is shown in Figure 86B.

|  | Regulatory Class |  |  | Channel Number |  | Actual Measurement Start Time |  |  | Measurement Duration |  |  | Antenna ID |  | ANPI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  |  | 1 |  | 8 |  |  | 2 |  |  | 1 |  | 1 |  |
|  |  | IPI 0 Density | IPI 1 Density |  | IPI 2 Density |  | ………………… |  |  | IPI 8 Density | IPI 9 Density |  | IPI 10 Density |  |  |
| Octets: |  | 1 | 1 |  | 1 |  |  |  |  | 1 | 1 |  | 1 |  |  |

Figure 86B—Measurement Report field format for a Noise Histogram Report

hRegulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set to the duration over which the Noise Histogram Report was measured, expressed in TUs.

Antenna ID shall be set to the identifying number for the antennas used for this measurement. Antenna ID is defined in 7.3.2.39.

ANPI shall be set to the average noise plus interference power value measured during the indicated Measurement Duration while the indicated channel is idle as described in 11.11.8.4.

The Noise Histogram Report shall contain the IPI densities, as defined in 11.11.8.4, observed in the channel for the eleven IPI levels defined in Table 31H.

Table 31H—IPI Definitions for a Noise Histogram Report

| IPI Level | IPI Measured Power (dBm) |
| --- | --- |
| 0 | IPI  -92 |
| 1 | -92 < IPI -89 |
| 2 | -89  IPI -86 |
| 3 | -86  IPI -83 |
| 4 | -83 IPI -80 |
| 5 | -80 IPI -75 |
| 6 | -75 IPI -70 |
| 7 | -70 IPI -65 |
| 8 | -65 IPI -60 |
| 9 | -60 IPI -55 |
| 10 | -55 IPI |

##### 7.3.2.22.6 Beacon Report

The format of the Measurement Report field corresponding to a Beacon Report is shown in Figure 86C.

|  |  | Regulatory Class | Channel Number |  |  | Actual Measurement Start Time |  |  |  | Measurement Duration |  | Reported Frame Information | RCPI | RSNI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 |  |  | 8 |  |  |  | 2 |  | 1 | 1 | 1 |
|  | BSSID |  |  | Antenna ID | Parent TSF |  |  | Reported Frame Body |  |  |  |  |  |  |
| Octets: | 6 |  |  | 1 | 4 |  |  | variable |  |  |  |  |  |  |

Figure 86C—Measurement Report field format for a Beacon Report

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set to the duration over which the Beacon Report was measured, expressed in TUs.

The Reported Frame Information field contains two subfields as shown in Figure 86D.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | Reported Frame Type |
| Bits: | 7 | 1 |

Figure 86D—Reported Frame Information Field

Condensed PHY Type indicates the physical medium type on which the Beacon, Measurement Pilot or Probe Response frame being reported was received. It shall have an integer value between 0 and 127 coded according to the value of dot11PHYType.

Reported Frame Type indicates the type of frame reported. A value of 0 indicates a Beacon, or Probe Response frame; a value of 1 indicates a Measurement Pilot frame.

RCPI indicates the received channel power of the Beacon, Measurement Pilot or Probe Response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

RSNI indicates the received signal to noise indication for the Beacon, Measurement Pilot or Probe Response frame in dB, as described in 7.3.2.40.

The BSSID field contains the BSSID from the Beacon, Measurement Pilot or Probe Response frame being reported.

The Antenna ID field contains the identifying number for the antennas used for this measurement. If different antennas are used to receive the beacon preamble and the beacon frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the beacon frame body. Antenna ID is defined in 7.3.2.39.

The Parent TSF field shall contain the lower 4-octets of the measuring STA's TSF timer value at the start of reception of the first octet of the timestamp field of the reported Beacon, Measurement Pilot, or Probe Response frame at the time the Beacon, Measurement Pilot or Probe Response frame being reported was received.

The Reported Frame Body field contains the frame body of the reported Beacon, Measurement Pilot, or Probe Response frame. All fixed fields and information elements are included in the order they appeared in the reported frame. Reported TIM elements shall be truncated such that only the first 4 octets of the element are reported and the element length field is modified to indicate the truncated length of 4. If the Reported Frame Body would cause the Measurement Report element to exceed the maximum information element size then the Reported Frame Body shall be truncated so that the last information element in the Reported Frame Body field shall be a complete information element.

##### 7.3.2.22.7 Frame Report

The format of the Measurement Report field corresponding to a Frame Report is shown in Figure 86E.

|  |  |  |  |  | Zero or more entries |
| --- | --- | --- | --- | --- | --- |
|  | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Frame Report Entry |
| Octets: | 1 | 1 | 8 | 2 | n x 18 |

Figure 86E—Measurement Report field format for a Frame Report

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Annex J.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Annex J.

Actual Measurement Start Time shall be set to value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set to the duration over which the Frame Report was measured, expressed in TUs.

The format of the Frame Report Entry is shown in Figure 86F.

|  | Transmit Address | BSSID | PHY Type | Average RCPI | Last RSNI | Last RCPI | Antenna ID | Frame Count |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 1 | 1 | 1 | 2 |

Figure 86F—Frame Report Entry format

The Transmit Address field contains the Transmitter Address (TA) from the frames being reported.

The BSSID field contains the BSSID from the frames being reported.

PHY Type indicates the physical medium type for the frame(s) being reported. Valid entries are coded according to the value of dot11PHYType.

Average RCPI indicates the average value for the received channel power of frames received and counted in this Frame Report Entry as described in 11.11.8.2. Average RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

Last RSNI indicates the received signal to noise indication of the most recently measured frame counted in this Frame Report Entry in dB, as described in 7.3.2.40.

Last RCPI indicates the received channel power of the most recently measured frame counted in this Frame Report entry. Last RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Antenna ID field contains the identifying number for the antennas used to receive the most recently measured frame counted in this Frame Report entry. Antenna ID is defined in 7.3.2.39.

Frame Count is a count of the data and management frames received with the indicated Transmit Address and BSSID during the measurement duration. The value 65535 shall indicate a count of 65535 or more.

##### 7.3.2.22.8 STA Statistics Report

The format of the Measurement Report field of a STA Statistics Report is shown in Figure 86G.

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |
| Octets: | 2 | Variable |

Figure 86G—Measurement Report field format for a STA Statistics Report

The STA Statistics Report reports the change in the requested Statistics Group Data values measured within the Measurement Duration. When the Measurement Duration is set to 0 the current values of the requested Statistics Group Data shall be reported, rather than the change.

The Measurement Duration shall be set to the duration over which the change in Statistics Group Data was measured and reported, expressed in TUs. A Measurement Duration value of 0 indicates a report of the current values of the Statistics Group Data.

Statistics Group Data shall contain the requested statistics from the MIB in Annex D related to the interface on which the request was received according to Table 31I. Units used for reporting a statistic or change in statistic shall be the units used to defined the statistic in Annex B. When Measurement Duration value is non-zero, the reported data values for statistics which are not counters shall be 2's complement integers representing both positive and negative changes in the statistics data.

Table 31I—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 0 | dot11Counters Group for the Interface on which the STA Statistics Request was received (with the exception of WEPUndecryptableCount and those counters listed in group 1) dot11TransmittedFragmentCount (Counter32), dot11MulticastTransmittedFrameCount (Counter32), dot11FailedCount (Counter32), dot11ReceivedFragmentCount (Counter32), dot11MulticastReceivedFrameCount (Counter32), dot11FCSErrorCount (Counter32), dot11TransmittedFrameCount (Counter32) |
| 1 | dot11MACStatistics Group for the Interface on which the STA Statistics Request was received: dot11RetryCount (Counter32), dot11MultipleRetryCount (Counter32), dot11FrameDuplicateCount (Counter32), dot11RTSSuccessCount (Counter32), dot11RTSFailureCount (Counter32), dot11ACKFailureCount (Counter32) |
| 2 | dot11QosCounters Group for the Interface on which the STA Statistics Request was received: dot11QosTransmittedFragmentCount, dot11QosFailedCount, dot11QosRetryCount, dot11QosMultipleRetryCount, dot11QosFrameDuplicateCount, dot11QosRTSSuccessCount, dot11QosRTSFailureCount, dot11QosACKFailureCount, dot11QosReceivedFragmentCount, dot11QosTransmittedFrameCount, dot11QosDiscardedFrameCount, dot11QosMPDUsReceivedCount, dot11QosRetriesReceivedCount |
| 3 | dot11BSS Load Group (only available at an AP): dot11STAStatisticsAPServiceLoad (INTEGER), dot11STAStatisticsAverageAccessDelayBestEffort (INTEGER), dot11STAStatisticsAverageAccessDelayBackGround (INTEGER), dot11STAStatisticsAverageAccessDelayVideo (INTEGER), dot11STAStatisticsAverageAccessDelayVoice (INTEGER), dot11STAStatisticsStationCount (INTEGER), dot11STAStatisticsChannelUtilization (INTEGER) |
| 4–255 | None |

The format of the Measurement Report field for dot11Counters Group is shown in Figure 86H.

|  | dot11Transmitted FragmentCount | dot11Multicast TransmittedFrame Count | dot11FailedCount | dot11Received FragmentCount | dot11Multicast ReceivedFrame Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11FCSErrorCount | dot11Transmitted FrameCount |  |  |  |
| Octets: | 4 | 4 |  |  |  |

Figure 86H—Measurement Report field format for dot11Counters Group

The format of the Measurement Report field for dot11MACStatistics is shown in Figure 86I

|  | dot11RetryCount | dot11MultipleRetry Count | Dot11FrameDuplicateCount | dot11RTSSuccess Count | dot11RTSFailure Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11ACKFailure Count |  |  |  |  |
| Octets: | 4 |  |  |  |  |

Figure 86I—Measurement Report field format for dot11MACStatistics Group

The format of the Measurement Report field for dot11QosCounters Group is shown in Figure 86J.

|  | dot11QosTransmitted FragmentCount | dot11QosFailed Countt | dot11QosRetry Count | dot11QosMultiple RetryCount | dot11QosFrame DuplicateCount |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11QosRTS SuccessCount | dot11QosRTS FailureCount | dot11QosACK FailureCount | dot11QosReceived FragmentCount | dot11Qos TransmittedFrame Count |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11QosDiscardedFrameCount | dot11QosMPDUsReceivedCount | dot11QosRetriesReceivedCount |  |  |
| Octets: | 4 | 4 | 4 |  |  |

Figure 86J—Measurement Report field format for dot11QosCounters Group

The format of the Measurement Report field for dot11BSSLoad Group is shown in Figure 86K.

|  | dot11STAStatisticsAPServiceLoad | dot11STAStatisticsAverageAccess DelayBestEffort | dot11STAStatisticsAverageAccess DelayBackGround | dot11STAStatisticsAverageAccess DelayVideo | dot11STAStatisticsAverageAccess DelayVoice |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |
|  | dot11STAStatisticsStationCount | dot11STAStatisticsChannelUtilization |  |  |  |
| Octets: | 2 | 1 |  |  |  |

Figure 86K—Measurement Report field format for dot11BSSLoad Group

##### 7.3.2.22.9 Location Configuration Information Report element

A Location Configuration Information report includes Latitude, Longitude, Altitude and optional Azimuth information. The LCI report format is shown in Figure 86L.

Figure 86L—Location Configuration Information

This structure and information fields are little-endian, per conventions defined in 7.1.1, and are based on the LCI format described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”.

The definition of elements within the LCI report shall be as defined in IETF RFC3825 section 2.1 or as defined herein.

The Azimuth Report field contains three subfields as shown in Figure 86M.

|  | B0 | B1 | B2 | B3 | B6 | B7 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reserved |  | Azimuth Type | Azimuth Resolution |  | Azimuth |  |
| Bits: | 2 |  | 1 | 4 |  | 9 |  |

Figure 86M—Azimuth Report subfield

Azimuth Type is set to 1 to report the Azimuth of radio reception, and is set to 0 to report the Azimuth of front face of the reporting STA.

Azimuth Resolution is 4 bits, indicating the number of valid most significant bits in the Azimuth.

Azimuth is a 9-bit unsigned integer value in degrees from True North, of the type defined by the Azimuth Type field.

##### 7.3.2.22.10 QoS Metrics Report

The format of the Measurement Report field corresponding to a QoS Metrics Report is shown in Figure 86N.

|  | Actual Measurement Start Time | Measurement Duration | Peer QSTA Address | Traffic Identifier | Reporting Reason |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 2 | 6 | 1 | 1 |

|  | Transmitted MSDU Count | MSDU Discarded Count | MSDU Failed Count | MSDU Multiple Retry Count | QoS CF-Polls Lost Count | Average Queue Delay | Average Transmit Delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

|  | Bin 0 Range | Bin 0 Count | Bin 1 Count | Bin 2 Count | Bin 3 Count | Bin 4 Count | Bin 5 Count |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 4 | 4 | 4 | 4 | 4 | 4 |

Figure 86N—Measurement Report field format for QoS Metrics Report

Actual Measurement Start Time shall be set to the TSF at the time at which the measurement started, or for a triggered QoS metrics report the TSF value at the reporting QSTA when the trigger condition was met.

Measurement Duration shall be set to the duration over which the QoS Metrics Report was measured, expressed in TUs. For a triggered QoS Metrics Report, metrics are reported over a number of transmitted MSDUs rather than a duration, hence Measurement Duration shall be set to 0 – see 11.11.8.8.

The Peer QSTA Address shall contain a MAC address indicating the RA for the measured frames.

The Traffic Identifier field contains the TID subfield as shown in Figure 80. The TID subfield indicates the TC or TS for which traffic was measured.

The Reporting Reason field is a bit field indicating the reason that the measuring QSTA sent the QoS metrics report. The Reporting Reason field is shown in Figure 86O.

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average Trigger | Consecutive Trigger | Delay Trigger | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure 86O—Reporting Reason field

- The Average Trigger bit set to 1 indicates that the QoS Metrics Report was generated as a triggered report due to the Average Error trigger.

- The Consecutive Trigger bit set to 1 indicates that the QoS Metrics Report was generated as a triggered report due to the Consecutive Error trigger.

- The Delay Trigger bit set to 1 indicates that the QoS Metrics Report was generated as a triggered report due to the Delay Error trigger.

When a QoS Metrics Report is sent as a direct response to a QoS Metrics Request and not as a triggered QoS Metrics Report, all bit fields in the Reporting Reason field are set to 0. This is termed a requested QoS Metrics report. Within a triggered QoS metrics report, more than one bit field in the Reporting Reason field may be set to 1 if more than one trigger condition was met.

The Transmitted MSDU Count, MSDU Failed Count, MSDU Discarded Count, MSDU Multiple Retry Count, QoS CF-Polls Lost Count, Average Queue Delay, Average Transmit Delay, and delay histogram fields relate to transmissions to the QSTA given in the Peer QSTA Address field. Metrics shall be reported over the Measurement Duration, or for triggered QoS metrics, over the Measurement Count.

The Transmitted MSDU Count field contains the number of MSDUs for the TC, or the TS specified by the TID that were successfully transmitted.

The MSDU Discarded Count field contains the number of MSDUs for the TC, or the TS specified by the TID that were discarded due either to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit (as appropriate), or due to the MSDU lifetime having been reached.

The MSDU Failed Count field contains the number of MSDUs for the TC, or the TS specified by the TID that were discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit (as appropriate).

The MSDU Multiple Retry Count field contains the number of MSDUs for the TC, or the TS specified by the TID that were successfully transmitted after more than one retransmission attempt.

The QoS CF-Polls Lost Count field contains the number of QoS (+)CF-Poll frames that were transmitted where there was no response from the QSTA. QoS CF-Polls Lost Count shall only be returned if the reporting QSTA is a QAP and the TID is for a TS. This field shall be set to 0 when QoS CF-Polls Lost Count is not returned.

Average Queue Delay shall be the average queuing delay of the frames (MSDUs) that are passed to the MAC for the indicated Peer QSTA Address and the indicated Traffic Identifier. Queue Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the first, or only fragment is ready for transmission and shall be expressed in TUs.

Average Transmit Delay shall be the average delay of the frames (MSDUs) that are successfully transmitted for the indicated Peer QSTA Address and TID. Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the entire MSDU has been successfully transmitted, including receipt of the final ACK from the peer QSTA if the QoSAck service class is being used. Average Transmit delay shall be expressed in TUs.

Bin 0 Range shall indicate the delay range of the first bin (Bin 0) of the Transmit Delay Histogram, expressed in TUs. It is also used to calculate the delay ranges of the other 5 bins making up the histogram. The delay range for each bin shall increase in a binary exponential fashion as follows:

B0 duration:   	 0   Delay <B0, 		for i = 0;

Bi duration:  	2i−1*B0  Delay < 2i*B0     	for 1  i  5

For example if Bin 0 Range is 10TUs, the bin durations should be as defined in Table 31J.

Table 31J—Delay Definitions for a QoS Metrics Report for a bin range of 10TU

| Bin | Measured Delay (TUs) |
| --- | --- |
| 0 | Delay <10 |
| 1 | 10  Delay < 20 |
| 2 | 20  Delay <40 |
| 3 | 40  Delay <80 |
| 4 | 80  Delay <160 |
| 5 | Delay  160 |

To compute the value reported in Bin i, Bi, 0 ≤ i ≤ 5, of the Transmit Delay Histogram, the STA initializes all bin counts to zero. For each MSDU successfully transmitted, the measured Transmit Delay determines which bin count is to be incremented. If the measured delay has a duration time t within Bin i, then the frame count in Bin i is increased by one, up to a ceiling value of 232-1. Transmit Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the entire MSDU has been successfully transmitted, including receipt of the final ACK from the peer QSTA if the QoSAck service class is being used. The sum of the counts in all six bins shall be equal to the value reported in the Transmitted MSDU Count.

Insert the following new clauses:

#### 7.3.2.35 AP Channel Report element

The AP Channel Report element contains a list of channels where a STA is likely to find an AP. The format of the AP Channel Report element is shown in Figure 112A.

|  | Element ID | Length | Regulatory Class | Channel List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 112A—AP Channel Report element format

The Element ID field shall be equal to the AP Channel Report value in Table 26.

The Length field is dependent on the number of channels reported in the Channel List. The minimum value of the length field is 1 (based on a minimum length for the channel list field of 0 octets).

Regulatory Class shall contain an enumerated value from Annex J, specifying the regulatory class in which the Channel List is valid. An AP Channel Report shall only report channels for a single regulatory class. Multiple AP Channel Report elements may be used to report channels in more than one regulatory class.

The Channel List shall contain a variable number of octets, where each octet describes a single channel number. Channel numbering shall be dependent on Regulatory Class according to Annex J.

An AP Channel report shall only include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with the Country element in the frame in which it appears.

#### 7.3.2.36 Neighbor Report element

The format of the Neighbor Report element is shown in Figure 112B.

|  |  |  |  |  |  |  |  |  | Zero or more optional extensions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | BSSID | BSSID Information | Regulatory Class | Channel Number | PHY Type | Measurement Pilot Interval | Extensions |
| Octets: | 1 | 1 | 6 | 4 | 1 | 1 | 1 | 2 | variable |

Figure 112B—Neighbor Report element format

The Element ID field is equal to the Neighbor Report value in Table 26.

The value of Length field is dependent on the number and length of optional extensions. Each Report element describes an AP and consists of BSSID, BSSID Information, Channel Number, Regulatory Class, PHY Type, and may include extensions. The minimum value of the Length field is 13 (i.e., with no optional extensions in the Neighbor Report element).

The BSSID is the BSSID of the BSS being reported. The subsequent fields in the Neighbor Report Element pertain to this BSS.

The BSSID Information field may be used to help determine neighbor BSS transition candidates. It shall be two octets in length and shall contain the subfields as shown in Figure 112C.

|  | B0 | B1 | B2 | B3 | B4 | B9 | B10 | B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AP Reachability |  | Security | Key Scope | Capabilities |  | Reserved |  |
| Bits: | 2 |  | 1 | 1 | 6 |  | 22 |  |

Figure 112C—BSSID information field

The AP Reachability field indicates whether the AP identified by this BSSID is reachable by the STA that requested the Neighbor Report. For example the AP identified by this BSSID is reachable for the exchange of preauthentication frames as described in  8.4.6.1. The values are shown in Table 43A.

Table 43A—Reachability field

| Value | Reachability | Usage |
| --- | --- | --- |
| 0 | Reserved | Not used. |
| 1 | Not Reachable | A station sending a preauthentication frame to the BSSID will not receive a response even if the AP represented by the BSSID is capable of preauthentication. |
| 2 | Unknown | The AP is unable to determine if the value Reachable or Not Reachable is to be returned. |
| 3 | Reachable | The station sending a preauthentication frame to the BSSID can receive a response from an AP that is capable of preauthentication. |

The Security bit if set to 1 indicates that the AP identified by this BSSID supports the same level of security as used by the STA in its current association. If the bit is set to 0 it indicates that either the AP can only provide less security than the current association, as defined by clause 8 or the information is not available to the AP at this time.

The Key Scope bit, when set, indicates the AP represented by this BSSID has the same authenticator as the AP sending the report. If this bit is set to 0 it indicates a distinct authenticator or the information is not available.

The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set to the equivalent bits within the Capability Information field being sent in the beacons by the AP being reported. The format of the Capabilities subfield is as in Figure 112D.

|  | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Delayed Block Ack | Immediate Block Ack |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure 112D—Capabilities Subfield

Bits 10 – 31 are reserved and shall be set to 0 on transmission and are ignored on reception.

Regulatory Class contains an enumerated value from Annex J specifying the regulatory class in which the Current Channel is valid.

Channel Number indicates the last known operating channel of the AP represented by the BSSID in this neighbor list entry.

The PHY Type field indicates the PHY type of the AP being reported. It shall have an integer value coded according to the value of the dot11PHYType.

The Measurement Pilot Interval field is as specified in 7.3.1.19. The Measurement Pilot Interval field is set to zeroes if the reported AP is not transmitting the Measurement Pilots frames or the Measurement Pilot interval is not known by the reporting AP.

The optional extensions shall be ordered by increasing Sub-Element ID.

The format of the optional extensions is shown in Figure 112E.

|  | Sub-Element ID | Length | Data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure 112E—Sub-Element format

The values of the Sub-Element ID field are shown in Table 43B.

The value of Length field is dependent on the length of the sub-element.

Table 43B—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 0 | Reserved |
| 1 | TSF Information |
| 2-220 | Reserved |
| 221 | Vendor Specific |
| 222-255 | Reserved |

The TSF information sub-element contains TSF Offset and Beacon Interval subfields.

|  | TSF Offset | Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure 112F—TSF Information sub-element format

The value of the TSF Information sub-element length field is 4.

The TSF Offset subfield is 2 octets long and contains the neighbor AP’s TSF timer offset. This is the time difference, in TU units, between the serving AP and a neighbor AP. This offset is given modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Beacon Interval field is the beacon interval of the Neighbor AP being reported. This field is defined in  7.3.1.3 and illustrated in Figure 26.

The Vendor Specific sub-element is defined to be the same as the Vendor Specific element (see 7.3.2.26).

#### 7.3.2.37 RCPI element

The RCPI element indicates the received frame power level at the receiving STA.

|  | Element ID | Length | RCPI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 112G—RCPI element format

The length field shall be set to 1.

#### 7.3.2.38 BSS Load element

The BSS Load element contains information on the current service level in the BSS. The element information field is defined in Error: Reference source not foundFigure 112H.

|  | Element ID | Length | AP Service Load | Station Count | Channel Utilization |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 1 |

Figure 112H—BSS Load element format

The Length field shall be set to 1.

The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value indicates more available service capacity than a higher value. The value 0 indicates that this AP is not currently serving any STA. The value 255 indicates that the AP Service Load is not available. The values between 1 and 247 are a logarithmically scaled representation of the average medium access delay for DCF or EDCF transmitted frames measured from the time the DCF or EDCF MPDU is ready for transmission (i.e. begins CSMA/CA access) until the actual frame transmission start time. The access delay time values are scaled as follows:

0:                        Access Delay  <  50 µsec

1:     50 µsec <=  Access Delay  <  51 µsec

2:     51 µsec <=  Access Delay  <  52 µsec

and so on where

n:     50*(10^ ((n-1)*0.0081))  <=  Access Delay  <  50*(10^ (n*0.0081))

in which the calculated range limit values are rounded to nearest integer and where n is the integer value (step) used to indicate the measured Access Delays.

247:     4916 usec <=  Access Delay  <  5008 usec

248:     5008 usec <=  Access Delay  <  8000 usec

249:     8000 usec <=  Access Delay  <  11000 usec

250:     11000 usec <=  Access Delay  <  14000 usec

251:     14000 µsec <=  Access Delay  <  17000 µsec

252:     17000 µsec <=  Access Delay  <  20000 µsec

253:     20000 µsec <=  Access Delay

254:      Service not available (blocked)

255:      Measurement not available

A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 20ms delay or any delay greater than 20ms. The value 254 shall indicate that DCF services are currently not available (blocked). The AP shall measure and average the medium access delay for all transmit frames using the DCF or EDCA over a continuous thirty second measurement window. The accuracy for the average medium access delay shall be +/- 100 µsec or better when averaged over at least 200 frames.

The STA Count field is interpreted as an unsigned integer that indicates the total number of STAs currently associated with this QBSS.

The Channel Utilization field is defined as the percentage of time, normalized to 255, the QAP sensed the medium was busy, as indicated by either the physical or virtual carrier sense (CS) mechanism. This percentage is computed using the formula:

((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11BeaconPeriod * 1024)) * 255)

where channel busy time is defined to be the number of microseconds during which the CS mechanism, as defined in 9.2.1, has indicated a channel busy indication, and the MIB attribute dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the channel busy time is measured. The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex D.

#### 7.3.2.39 Antenna Information element

The Antenna Information element contains the Antenna ID field as shown in Figure 112I.

|  | Element ID | Length | Antenna ID |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |

Figure 112I—Antenna Information element format

The length field shall be set to 1.

The Antenna ID field contains the identifying number for the relevant antennas. When included in a Beacon or Probe Response, the Antenna ID identifies the antennas used to transmit the Beacon or Probe Response frame. When included in a measurement report, the Antenna ID identifies the antennas used for the reported measurement. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas, i.e. antennas were switched during the measurement. The value 1 is the only value used for a STA with only one antenna. STAs with more than one antenna shall assign Antenna IDs to each antenna and each antenna configuration as consecutive, positive integers starting with 1. Each Antenna ID number represents a unique antenna or unique configuration of multiple antennas characterized by a fixed relative position, a fixed relative direction and a fixed peak gain for that position and direction.

#### 7.3.2.40 RSNI element

The RSNI element contains a RSNI value, as shown in Figure 112J.

|  | Element ID | Length | RSNI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 112J—RSNI element format

The length field shall be set to 1.

RSNI is in units of 0.5dB. RSNI is calculated by the ratio of the received signal power (RCPI - ANPI) to the noise plus interference power (ANPI) where RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to +117dB. The Value 255 indicates that RSNI is not available. See 11.11.8.4 for details and procedures for measuring ANPI.

#### 7.3.2.41 Measurement Pilot Transmission Information element

The Measurement Pilot Transmission Information element contains a Measurement Pilot Transmission field as shown in Figure 112K.

|  | Element ID | Length | Measurement Pilot Transmission |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |  |

Figure 112K—Measurement Pilot Transmission Information element format

The length field shall be set to 2.

The Measurement Pilot Transmission Field contains the Measurement Pilot Interval as specified in 7.3.1.19, if the Measurement Pilot frames are transmitted. In Probe Responses the field is set to all zeroes if the Measurement Pilot frames are not transmitted. In Neighbor Reports the field is set to all zeroes reported AP is not transmitting the Measurement Pilots frames or the Measurement Pilot interval is not known by the reporting AP.

#### 7.3.2.42 QBSS Available Admission Capacity element

QBSS Available Admission Capacity element contains a list of Available Admission Capacity at different User Priorities and Accesss Categories as shown in Figure 112L.

|  | Element ID | Available Admission Capacity Bitmask | Available Admission Capacity List |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 x (total number of non-zero bits in Available Admission Capacity Bitmask) |  |

Figure 112L—QBSS Available Admission Capacity element Data field format

The value of the QBSS Available Admission Capacity element Length field is set to 2 + 2 x (total number of non-zero bits in Available Admission Capacity Bitmask).

Available Admission Capacity Bitmask field indicates the UP traffics that have Available Admission Capacity specified in the following Available Admission Capacity List. The format of Available Admission Capacity Bitmask is defined in Table 43C.

Table 43C: Available Admission Capacity Bitmask Definition

| Bit (s) | Available Admission Capacity Reported? |
| --- | --- |
| 0 | UP 0 |
| 1 | UP 1 |
| 2 | UP 2 |
| 3 | UP 3 |
| 4 | UP 4 |
| 5 | UP 5 |
| 6 | UP 6 |
| 7 | UP 7 |
| 8 | AC 0 |
| 9 | AC 1 |
| 10 | AC 2 |
| 11 | AC3 |
| 12-15 | Reserved |

The bit is set to 1 to indicate that the Available Admission Capacity for the corresponding traffic type is present in the Available Admission Capacity List field. The bit is set to 0 to indicate that the Available Admission Capacity for the corresponding traffic type is not present in the Available Admission Capacity List field.

The Available Admission Capacity List comprises a sequence of Available Admission Capacity fields corresponding respectively to the non-zero bits in the Available Admission Capacity Bitmask field.

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the remaining amount of medium time available via explicit admission control for the corresponding UP or AC traffic, in units of 32 microsecond periods per 1 second. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests.

#### 7.3.2.43 QBSS Access Delay element

The QBSS Acess Delay element contains an Access Category Service Load field, as shown in Figure 112M.

|  | Element ID | Length | Access Category Service Load |
| --- | --- | --- | --- |
| Octets | 1 | 1 | 4 |

Figure 112M—QBSS Access Delay element format

The AC Service load field is formatted as four subfields as shown in Figure 112R. The AC Service Load shall be a scalar indication of the average access delay at a QAP for services for each of the indicated Access Categories. If the QAP is not currently transmitting any traffic using the indicated AC, the Average Access Delay for that AC is set to 255. The values between 1 and 247 are a logarithmically scaled representation of the average medium access delay for transmitted frames in the indicated AC measured from the time the EDCF MPDU is ready for transmission (i.e. begins CSMA/CA access) until the actual frame transmission start time. The access delay time values are scaled as follows:

0:                        Access Delay  <  50 usec

1:     50 usec <=  Access Delay  <  51 usec

2:     51 usec <=  Access Delay  <  52 usec

and so on where

n:     50*(10^ ((n-1)*0.0081))  <=  Access Delay  <  50*(10^ (n*0.0081))

in which the calculated range limit values are rounded to nearest integer and where n is the integer value (step) used to indicate the measured Access Delays.

247:     4916 usec <=  Access Delay  <  5008 usec

248:     5008 usec <=  Access Delay  <  8000 usec

249:     8000 usec <=  Access Delay  <  11000 usec

250:     11000 usec <=  Access Delay  <  14000 usec

251:     14000 usec <=  Access Delay  <  17000 usec

252:     17000 usec <=  Access Delay  <  20000 usec

253:     20000 usec <=  Access Delay

254:      Service not available (blocked)

255:      Measurement not available

The value 254 indicates that services at the indicated AC are currently not available (blocked). The value 255 indicates that the AC Service Load is not available. The QAP shall measure and average the medium access delay for all transmit frames of the indicated AC using EDCF access mechanism over a continuous thirty second measurement window. The accuracy for the average medium access delay shall be +/- 100 usec or better when averaged over at least 200 frames.

|  | Average Access Delay for Best Effort (AADBE) | Average Access Delay for Background (AADBG) | Average Access Delay for Video (AADVI) | Average Access Delay for Voice (AADVO) |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure 112R—Access Category Service Load subfields

## 7.4 Action frame format details

Insert the following new clauses after the end of  7.4.4:

### 7.4.5 Radio Measurement action details

Several Action frame formats are defined for Radio Measurement purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table 57A.

Table 57A — Radio Measurement Action field values

| Action field value | Description |
| --- | --- |
| 0 | Radio Measurement Request |
| 1 | Radio Measurement Report |
| 2 | Link Measurement Request |
| 3 | Link Measurement Report |
| 4 | Neighbor Report Request |
| 5 | Neighbor Report Response |
| 6-255 | Reserved |

#### 7.4.5.1 Radio Measurement Request frame format

The Radio Measurement Request frame uses the Action frame body format. It is transmitted by a STA requesting another STA to make one or more measurements on one or more channels. The format of the frame is shown in Figure 117A.

|  | Category | Action | Dialog Token | Number of Repetitions | Measurement Request Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | variable |

Figure 117A—Radio Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Measurement Request according to Table 57A in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the radio measurement request to identify the request/report transaction.

The Number of Repetitions field contains the requested number of repetitions for all the Measurement Request elements in this frame. A value of zero in the Number of Repetitions field indicates Measurement Request elements are executed once without repetition. A value of 65535 in the Number of Repetitions field indicates Measurement Request elements are repeated until the measurement is cancelled.

The Measurement Request Elements field shall contain zero or more of the Measurement Request elements described in 7.3.2.21. The number and length of the Measurement Request elements in a Measurement Request frame is limited by the maximum allowed MMPDU size.

#### 7.4.5.2 Radio Measurement Report frame format

The Measurement Report frame uses the Action frame body format. It is transmitted by a STA in response to a Measurement Request frame or by a STA providing a triggered autonomous measurement report. The format of the frame is shown in Figure 117B.

|  | Category | Action | Dialog Token | Measurement Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 117B—Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a.

The Action field shall be set to indicate a Measurement Report according to Table 57A.

The Dialog Token field shall be set to the value in the corresponding Measurement Request frame. If the Radio Measurement Report frame is not being transmitted in response to a Radio Measurement Request frame then the Dialog token shall be set to zero.

The Measurement Report Elements field shall contain one or more Measurement Report elements described in 7.3.2.22. The number and length of the Measurement Report elements in a single Radio Measurement Report frame is limited by the maximum allowed MMPDU size. Section 11.11.5 describes the required behavior for multi-frame Radio Measurement Report frame responses.

#### 7.4.5.3 Link Measurement Request Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA to request another STA to respond with a Link Measurement Report frame to enable measurement of link path loss and estimation of link margin. The format of the frame is shown in Figure 117C.

|  | Category | Action | Dialog Token | Transmit Power | Max Transmit Power |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure 117C—Link Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Request according to Table 57A in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the request to identify the transaction.

The Transmit Power element shall be set to the transmit power used to transmit the frame containing the Link Measurement Request, as described in 7.3.1.22.

The Max Transmit Power field shall provide the upper limit on the transmit power as measured at the output of the antenna connector to be used by the transmitting STA on the current channel. This field is described in 7.3.1.21.

#### 7.4.5.4 Link Measurement Report Frame Format

The Link Measurement Report frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame. The format of the frame is shown in Figure 117D.

|  | Category | Action | Dialog Token | TPC Report element | Receive Antenna ID | Transmit Antenna ID | RCPI | RSNI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 1 | 1 | 1 | 1 |

Figure 117D—Link Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Report according to Table 57A in 7.4.5.

The Dialog Token field shall be set to the Dialog Token value in the corresponding Link Measurement Request frame.

The TPC Report element shall be set as described in 7.3.2.18.

The Receive Antenna ID field contains the identifying number for the antenna used to receive the corresponding Link Measurement Request frame. Antenna ID is defined in 7.3.2.30.

The Transmit Antenna ID field contains the identifying number for the antenna used to transmit this Link Measurement Report frame. Antenna ID is defined in 7.3.2.30.

RCPI indicates the received channel power of the corresponding Link Measurement Request frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type, as described in 7.3.2.37.

RSNI indicates the received signal to noise indication for the corresponding Link Measurement Request frame in dB, as described in 7.3.2.40.

#### 7.4.5.5 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by a STA requesting information in the Neighbor Report about neighboring AP’s. The format of the Neighbor Report Request frame body is shown in Figure 117E.

|  | Category | Action | Dialog Token | SSID element (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2-34 |

Figure 117E—Neighbor Report Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Request, as specified in Table 57A in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The SSID element is defined in 7.3.2.1. An SSID may optionally be included in a Neighbor Report Request frame to indicate a request for a neighbor list for the specified SSID in the SSID Element. The absence of a SSID element indicates neighbor report for the current ESS. Inclusion of the SSID element is determined by the evaluating the length of the Neighbor Report Request frame.

#### 7.4.5.6 Neighbor Report Response frame format

The Neighbor Report Response frame uses the Action frame body format and is transmitted by a STA in response to a Neighbor Report Request frame. The format of the Neighbor Report Response frame body is shown in Figure 117F.

|  | Category | Action | Dialog Token | Neighbor Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 117F—Neighbor Report Response frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Response, as specified in Table 57A in 7.4.5.

The Dialog Token field shall be set to the value in the corresponding Neighbor Report Request frame. If the Neighbor Report Response frame is not being transmitted in response to a Neighbor Report Request frame then the Dialog token shall be set to zero.

The Neighbor Report Elements field shall contain the Neighbor Report elements described in 7.3.2.36. If the STA has no information in response to the Neighbor Report Request, the Neighbor Report elements shall be omitted. The number and length of the Neighbor Report Elements in a Neighbor Report frame is limited by the maximum allowed MMPDU size.

# 10. Layer management

## 10.3 MLME SAP Interface

### 10.3.2 Scan

#### 10.3.2.2 MLME-SCAN.confirm

##### 10.3.2.2.2 Semantics of the Service Primitive

Change the following row of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Country | As defined in the Country element | As defined in the Country element | The information required to identify the regulatory domain in which the STA is located and to configure its PHY for operation in that regulatory domain. Present only when TPC functionality is required, as specified in 11.9, or when dot11MultiDomainCapabilityEnabled is true or when dot11RadioMeasurementEnabled is true. |

Insert the following rows at the end of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| RCPIMeasurement | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI of the Probe Request frame measured during the active scan procedure. This parameter shall be present within a BSSDescription returned in an MLME-SCAN.confirm primitive when an RCPI element was present in the received Probe Response. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| PowerConstraint | As defined in frame format | As defined in frame format | The contents of the Power Constraint element when received. Present only when the MIB attribute dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| TPCReport | As defined in frame format | As defined in frame format | The contents of the TPC Report element when received. Present only when either the MIB attribute dot11SpectrumManagementRequired or dot11RadioMeasurementEnabled is true. |
| APChannelReportSet | Set of AP Channel Reports each as defined in the AP Channel Report element format | Set of AP Channel Reports each as defined in the AP Channel Report element format | The contents of any AP Channel Report elements received. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| BSS Load | As defined in frame format | As defined in frame format | The values from the BSS Load information element if such an element was present in the probe response or beacon, else null. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| Antenna Information | 0-255 | 0-255 | The value of Antenna ID from the Antenna Information element when received. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| Measurement Pilot Transmission | As defined in frame format | As defined in frame format | The value of Measurement Pilot Transmission from the Measurement Pilot Transmission Information element if such an element was present in the Probe Response. The element may be present only when the MIB attribute dot11RadioMeasurementEnabled is true. |
| QBSS Admission Available Capacity | As defined in frame format | As defined in frame format | The values from the QBSS Admission Available Capacity information element if such an element was present in the probe response or beacon, else null. Present only when the MIB attribute dot11RadioMeasurementEnabled is true. |

Change the following clauses as shown:

### 10.3.6 Associate

#### 10.3.6.3 MLME-ASSOCIATE.indication

##### 10.3.6.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.indication	(
PeerSTAAddress, 
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association was received. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by the peer MAC entity as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MAC entity as part of the association request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting association. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. Only one pairwise cipher suite and only one authenticated key suite are allowed in the RSN information element. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. |

#### 10.3.6.4 MLME-ASSOCIATE.response

##### 10.3.6.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the association request was received. |
| ResultCode | Enumeration | SUCCESS, REFUSED_REASON_UNSPECIFIED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result response to the association request from the peer MAC entity. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the association request. |
| AssociationID | Integer | 1-2007 inclusive | If the association request result was SUCCESS, then AssociationID specifies the association ID value assigned to the peer MAC entity by the AP. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Association Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Association Request frame was received. |

### 10.3.7 Reassociate

#### 10.3.7.3 MLME-REASSOCIATE.indication

##### 10.3.7.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.indication	(
PeerSTAAddress, 
CurrentAPAddress,
CapabilityInformation, 
ListenInterval, 
SSID, 
SupportedRates,
RSN
QoSCapability,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the reassociation was received. |
| CurrentAPAddress | MACAddress | Any valid individual MAC address | Specifies the address of the AP with which the peer station is currently associated. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capability definitions provided by the peer MAC entity as part of the reassociation request. |
| ListenInterval | Integer | >=0 | Specifies the listen interval value provided by the peer MAC entity as part of the association request. |
| SSID | Octet string | 0-32 octets | Specifies the SSID provided by the peer MACentity as part of the reassociation request. |
| SupportedRates | Set of integers | 2-127 inclusive (for each integer in the set) | The set of data rates (in units of 500 kb/s) that are supported by the STA that is requesting reassociation. |
| RSN | RSN information element | As defined in frame format | A description of the cipher suites and AKM suites supported in the BSS. |
| QoSCapability | As defined in frame format | As defined in frame format | Specifies the parameters within the QoS Capability that are supported by the peer MAC entity. The parameter may be present only if the MIB attribute dot11QosOptionImplemented is true. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding Reassociation Request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding Reassociation Request frame was received. |

#### 10.3.7.4 MLME-REASSOCIATE.response

##### 10.3.7.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
RCPI,
RSNI
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which the reassociation request was received. |
| ResultCode | Enumeration | SUCCESS, REFUSED_REASON_UNSPECIFIED, REFUSED_CAPABILITIES_MISMATCH, REFUSED_EXTERNAL_REASON, REFUSED_AP_OUT_OF_MEMORY, REFUSED_BASIC_RATES_MISMATCH | Indicates the result response to the reassociation request from the peer MAC entity. |
| Capability-Information | As defined in frame format | As defined in frame format | Specifies the operational capabilities advertised by the AP. |
| AssociationID | Integer | 1-2007 inclusive | If the reassociation request result was SUCCESS, then AssociationID specifies the association ID value assigned to the peer MAC entity by the AP. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI value represents the measured RCPI of the corresponding reassociation request frame. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI value represents the measured RSNI at the time the corresponding reassociation request frame was received. |

Change the title, first and second paragraphs of 10.3.11 as follows:

### 10.3.11 Spectrum Management pProtocol layer model for spectrum management & radio measurement

The layer management extensions for measurement, TPC and channel switching assume a certain partition of spectrum management functionality between the MLME and station management entity (SME). This partitioning assumes that policy decisions (e.g., regarding measurement and channel switching) reside in the SME, while the protocol for measurement, switch timing and the associated frame exchanges resides within the MLME (see Figure 184).

The informative diagrams within this clause further illustrate the spectrum management protocol layer model adopted. Figure 185 and Figure 186 depict the measurement process for a peer STA to accept and reject a measurement request respectively. Figure 187 illustrates the TPC adaptation process. Lastly, Figure 188 depicts the management process for a channel switch using a Channel Switch Announcement frame.

Insert a new paragraph after the second paragraph in 10.3.11 as follows:

It should be noted that these diagrams are intended as examples and do not depict all possible protocol scenarios, e.g., a measurement request may result in more than one measurement report frame as described in 11.10.6 and 11.11. Note that when rejecting a radio measurement category request it is mandatory for a STA to send a measurement report with the Incapable bit set if the STA is permanently unable to make the requested measurement or with the Refused bit set if the STA is refusing the request.

Change Figure 185 and 186 as follows:

MLME-MEASURE.cfm

Measurement/Radio Measurement

Report frame

Measurement/Radio Measurement

Request frame

MLME-MREQUEST.ind

MLME-MREQUEST.cfm

MLME-MREQUEST.req

Decision to request measurement from peer STA

Decision to accept measurement request

from peer STA

SME

MLME

MLME

SME

Measurement

process

MLME-MEASURE.req

Compile

measurement

report

MLME-MREPORT.req

MLME-MREPORT.cfm

MLME-MREPORT.ind

Measurement

request

completed

IEEE802.11 STA

IEEE802.11 STA

Figure 185—Measurement request – accepted

Measurement/Radio Measurement

Request frame

Measurement/Radio Measurement

Report frame

MLME-MREQUEST.ind

MLME-MREQUEST.cfm

MLME-MREQUEST.req

Decision to request measurement from peer STA

Decision to reject measurement request

from peer STA

as refuse or incapable

SME

MLME

MLME

SME

MLME-MREPORT.req

MLME-MREPORT.cfm

MLME-MREPORT.ind

Measurement

request

completed

IEEE802.11 STA

IEEE802.11 STA

Figure 186—Measurement Request – rejected

Change the primitive definitions in 10.3.12.1.2, 10.3.12.3.2, 10.3.14.1.2 and 10.3.14.3.2 as follows:

### 10.3.12	Measurement Request

#### 10.3.12.1 MLME-MREQUEST.request

##### 10.3.12.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MREQUEST.request	(
Peer MAC Address,
Dialog Token,
Measurement Request Set,
Number of Repetitions,
Measurement Category
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group MAC Address | The address of the peer MAC entity to which the measurement request shall be setsent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the measurement transaction. |
| Measurement Request Set | Set of measurement requests each as defined in the Measurement Request element format | Set of measurement requests each as defined in the Measurement Request element format | A set of measurement requests each containing a Measurement Token, Measurement Request Mode, Measurement Type and a Measurement Request. If dot11RadioMeasurementEnabled is true, the Measurement Request Set includes the Number of Repetitions for the Measurement Request Set. |
| Number of Repetitions | Integer | 0 – 65535 | The number of times the Measurement Request Set is to be repeated. Shall only be present if Measurement Category is RADIO MEASUREMENT. |
| Measurement Category | Enumeration | SPECTRUM MANAGEMENT, or RADIO MEASURE-MENT | Indicates whether the Measurement Report Set is a set of Spectrum Management or Radio Measurement measurement requests. Shall only be present if dot11RadioMeasurementEnabled is true. |

#### 10.3.12.3 MLME-MREQUEST.indication

##### 10.3.12.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Measurement Request Set,
Number of Repetitions,
Measurement Category
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual Address | The address of the peer MAC entity from which the measurement request was received. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the measurement transaction. |
| Measurement Request Set | Set of measurement requests each as defined in the Measurement Request element format | Set of measurement requests each as defined in the Measurement Request element format | A set of measurement requests each containing a Measurement Token, Measurement Request Mode, Measurement Type and a Measurement Request. If dot11RadioMeasurementEnabled is true, the Measurement Request Set includes the Number of Repetitions for the Measurement Request Set. |
| Number of Repetitions | Integer | 0 – 65535 | The number of times the Measurement Request Set is to be repeated. Shall only be present if Measurement Category is RADIO MEASUREMENT. |
| Measurement Category | Enumeration | SPECTRUM MANAGEMENT, or RADIO MEASURE-MENT | Indicates whether the Measurement Report Set is a set of Spectrum Management or Radio Measurement measurement requests. Shall only be present if dot11RadioMeasurementEnabled is true. |

### 10.3.14 Measurement Report

#### 10.3.14.1 MLME-MREPORT.request

##### 10.3.14.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MREPORT.request	(
Peer MAC Address,
Dialog Token,
Measurement Report Set,
Measurement Category
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the measurement report shall be sent. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the measurement transaction. Set to 0 for an autonomous report. |
| Measurement Report Set | Set of measurement reports each as defined in the Measurement Report element format | Set of measurement reports each as defined in the Measurement Report element format | A set of measurement reports each containing a Measurement Token, Measurement Report Mode, Measurement Type and a Measurement Report. |
| Measurement Category | Enumeration | SPECTRUM MANAGEMENT, or RADIO MEASURE-MENT | Indicates whether the Measurement Report Set is a set of Spectrum Management or Radio Measurement reports. Shall only be present if dot11RadioMeasurementEnabled is true. |

#### 10.3.14.3 MLME-MREPORT.indication

##### 10.3.14.3.1 Function

This primitive indicates that a Measurement Report or Radio Measurement Report frame has been received from a peer entity. This may be in response to an earlier Measurement request (MLME-MREQUEST.request), or may be an autonomous report.

##### 10.3.14.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MREPORT.indication	(
Peer MAC Address,
Dialog Token,
Measurement Report Set,
Measurement Category
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Measurement Report frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the measurement transaction. Set to 0 for an autonomous report. |
| Measurement Report Set | Set of measurement reports each as defined in the Measurement Report element format | Set of measurement reports each as defined in the Measurement Report element format | A set of measurement reports each containing a Measurement Token, Measurement Report Mode, Measurement Type and a Measurement Report. |
| Measurement Category | Enumeration | SPECTRUM MANAGEMENT, or RADIO MEASURE-MENT | Indicates whether the Measurement Report Set is a set of Spectrum Management or Radio Measurement reports. Shall only be present if dot11RadioMeasurementEnabled is true. |

Insert the following new clauses after  10.3.28:

### 10.3.29 Neighbor Report Request

The following MLME primitives support the signaling of Neighbor Report Requests.

#### 10.3.29.1 MLME-NEIGHBORREPREQ.request

##### 10.3.29.1.1 Function

This primitive requests that a Neighbor Report Request frame be sent to the AP with which the STA is associated. It is valid only at a Radio Measurement capable non-AP STA.

##### 10.3.29.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPREQ.request	(
DialogToken,
SSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the neighbor report transaction. |
| SSID | As defined in the SSID element | As defined in the SSID element | Optional SSID element to request a neighbor list for a specific SSID. |

##### 10.3.29.1.3 When Generated

This primitive is generated by the SME to request that a Neighbor Report Request frame be sent to the AP with which the STA is associated to request a Neighbor Report.

##### 10.3.29.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Neighbor Report Request Action management frame. The STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.29.2 MLME-NEIGHBORREPREQ.confirm

##### 10.3.29.2.1 Function

This primitive reports the result of an attempt to send a Neighbor Report Request frame. It is valid only at a Radio Measurement capable non-AP STA.

##### 10.3.29.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPREQ.confirm	(
ResultCode,
DialogToken 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-NEIGHBORREP.request. |
| DialogToken | Integer | 1-255 | The Dialog Token to identify the neighbor report transaction. Set to the value in the corresponding MLME-NEIGHBORREPREQ.request. |

##### 10.3.29.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-NEIGHBORREPREQ.request and indicates the result of the request to send a Neighbor Report Request frame.

This primitive is generated when the Neighbor Report Request frame is successfully transmitted, the MLME-NEIGHBORREPREQ.request primitive contains invalid parameters, or when a timeout or other failure occurs.

##### 10.3.29.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

#### 10.3.29.3 MLME-NEIGHBORREPREQ.indication

##### 10.3.29.3.1 Function

This primitive indicates that a Neighbor Report Request frame was received from a non-AP STA. It is valid only at a Radio Measurement capable AP.

##### 10.3.29.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPREQ.indication	(
PeerSTAAddress,
DialogToken,
SSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Neighbor Report Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token in the Neighbor Report Request frame that was received. |
| SSID | As defined in the SSID element | As defined in the SSID element | Optional SSID element to request a neighbor list for a specific SSID. |

##### 10.3.29.3.3 When Generated

This primitive is generated by the MLME when a valid Neighbor Report Request frame is received.

##### 10.3.29.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.12.2.

### 10.3.30 Neighbor Report Response

The following MLME primitives support the signaling of Neighbor Report Responses.

#### 10.3.30.1 MLME-NEIGHBORREPRESP.request

##### 10.3.30.1.1 Function

This primitive requests that a neighbor report response be sent to a non-AP STA. This may be in response to an MLME-NEIGHBORREPREQ.indication primitive, or an autonomous request. It is valid only at a Radio Measurement capable AP.

##### 10.3.30.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPRESP.request	(
PeerSTAAddress,

DialogToken,
NeighborListSet,
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity to which a Neighbor Report Response frame is to be sent. |
| DialogToken | Integer | 0-255 | The Dialog Token to identify the neighbor report transaction. Set to the value received in the corresponding MLME-NEIGHBORREPORTREQ.indication or to 0 for an autonomous report. |
| NeighborListSet | Set of Neighbor List elements each as defined in the Neighbor Report element format | Set of Neighbor List elements each as defined in the Neighbor Report element format | A set of Neighbor List elements derived from the MIB table dot11RRMNeighborReportTable each representing a neighboring AP being reported as defined in the Neighbor Report element format. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.36 | Zero or more information elements. |

##### 10.3.29.1.3 When Generated

This primitive is generated by the SME to request a neighbor report be sent to a non-AP STA. This may be in response to an earlier MLME-NEIGHBORREPREQ.indication, or a request to transmit an autonomous report.

##### 10.3.29.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Neighbor Report Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

#### 10.3.30.2 MLME-NEIGHBORREPRESP.confirm

##### 10.3.30.2.1 Function

This primitive reports the result of an attempt to send a Neighbor Report Response frame to a peer STA. It is valid only at a Radio Measurement capable AP.

##### 10.3.30.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPRESP.confirm	(
ResultCode,
DialogToken,
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-NEIGHBORREPRESP.request. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the neighbor report transaction. Set to the value in the corresponding MLME-NEIGHBORREPRESP.request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.36 | Zero or more information elements. |

##### 10.3.30.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-NEIGHBORREPRESP.request and indicates the result of the request to send a Neighbor Report Response frame.

This primitive is generated when the Neighbor Report Response frame is successfully transmitted, the MLME-NEIGHBORREPRESP.request primitive contains invalid parameters, or when a timeout or other failure occurs.

##### 10.3.30.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.30.3 MLME-NEIGHBORREPRESP.indication

##### 10.3.30.3.1 Function

This primitive indicates that a neighbor report response has been received. This may be in response to an earlier neighbor report request (MLME-NEIGHBORREPORTREQ.req), or an autonomous report. It is valid only at a Radio Measurement capable non-AP STA.

##### 10.3.30.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NEIGHBORREPRESP.indication	(
PeerSTAAddress, 
DialogToken,
NeighborListSet,
VendorSpecificInfo
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the AP from which the Neighbor Report Response frame was received. |
| DialogToken | Integer | 0-255 | The Dialog Token received in the Neighbor Report Response frame to identify the neighbor report transaction. |
| NeighborListSet | Set of Neighbor List elements each as defined in the Neighbor Report information element format | Set of Neighbor List elements each as defined in the Neighbor Report information element format | A set of Neighbor List elements derived from the MIB table dot11RRMNeighborReportTable, each representing a neighboring AP being reported as defined in the Neighbor Report information element format. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.36 | Zero or more information elements. |

##### 10.3.30.3.3 When Generated

This primitive is generated by the MLME when a valid Neighbor Report Response Frame is received.

##### 10.3.30.3.4 Effect of Receipt

On receipt of this primitive, neighbor report data may be available to the SME.

### 10.3.31 Link Measure Request

The following primitives support the measurement of link path loss, and the estimation of link margin between peer entities for Radio Resource Measurement.

#### 10.3.31.1 MLME-LINKMEASURE.request

##### 10.3.31.1.1 Function

This primitive supports the measurement of link path loss, the estimation of link margin between peer entities.

Note—The layer management model used assumes that the handling of a received Link Measurement Request frame is entirely within the MLME. Correspondingly there are no MLME-SME primitives specified for the peer side of a link measurement request transaction.

##### 10.3.31.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMEASURE.request	(
PeerMACAddress,
DialogToken,
Transmit Power,
Max Transmit Power 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Link Measure Request shall be sent. |
| DialogToken | Integer | 1 – 255 | The dialog token to identify the Link Measure transaction. |
| Transmit Power | Integer | As defined in the Link Measurement Request frame format | The transmit power to be used when transmitting the Link Measurement Request frame and included in the frame body. See 7.4.5.3. |
| Max Transmit Power | Integer | As defined in the Link Measurement Request frame format | The maximum transmit power to be used by the transmitting STA on the current channel. See 7.4.5.4. |

##### 10.3.31.1.3 When Generated

This primitive is generated by the SME to request that a Link Measurement Request frame be sent to the peer entity to request that entity to report transmit power and link margin information.

##### 10.3.31.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Link Measurement Request frame. The STA then attempts to transmit this to the STA indicated in the PeerMACAddress parameter.

#### 10.3.31.2 MLME-LINKMEASURE.confirm

##### 10.3.31.2.1 Function

This primitive reports the result of a Link Measurement request.

##### 10.3.31.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMEASURE.confirm	(
ResultCode, 
DialogToken, 
TransmitPower, 
LinkMargin, 
RCPI,
RSNI,
ReceiveAntennaID,
TransmitAntennaID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, TRANSMISSION_FAILURE, UNSPECIFIED FAILURE | Indicates the result of the corresponding MLME-LINKMEASURE.request. |
| DialogToken | Integer | As defined in the corresponding MLME-LINKMEASURE.request | The Dialog Token to identify the link measurement transaction. |
| TransmitPower | As defined in the TPC Report element | As defined in the TPC Report element | The contents of the Transmit Power field of the received Link Measure Report frame. Only present if ResultCode = SUCCESS. |
| LinkMargin | As defined in the TPC Report element | As defined in the TPC Report element | The contents of the Link Margin field of the received Link Measure Report frame. Only present if ResultCode = SUCCESS. |
| RCPI | Integer | As defined in 15.4.8.5, or 17.3.10.6, or 18.4.8.5 | The RCPI level of the received Link Measurement Report frame. Only present if ResultCode = SUCCESS. |
| RSNI | Integer | As defined in 7.3.2.40 | The RSNI of the received Measurement Report frame. Only present if ResultCode=SUCCESS |
| Receive Antenna ID | Integer | 0-255 | The Antenna ID corresponding to the antenna on which the Link Measurement Request frame was received at the reporting STA. Antenna ID is defined in 7.3.2.29. |
| Transmit Antenna ID | Integer | 0-255 | The Antenna ID corresponding to the antenna used to transmit the Link Measurement Report frame. Antenna ID is defined in 7.3.2.29. |

##### 10.3.31.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-LINKMEASURE.request and indicates the results of the request.

This primitive is generated when the MLME-LINKMEASURE.request contains invalid parameters, when a failure occurs or when the STA receives a Link Measurement Response frame from the requested STA.

##### 10.3.31.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

### 10.3.32 Measurement Pilot Link SNR Ceiling request

This set of primitives allows a STA to retrieve uplink and downlink link SNR ceiling computed from a received Measurement Pilot.

#### 10.3.32.1 MLME-MPILOTLSCEILING.request

##### 10.3.32.1.1 Function

This set of primitives allows STA to retrieve uplink and downlink link SNR ceiling computed from a received Measurement Pilot.

##### 10.3.32.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MPILOTLSCEILING.request 	(
Link SNR Ceiling Direction
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link SNR Ceiling Direction | Enumeration | ULSC or DLSC or ALL | Differentiates between uplink link SNR ceiling (ULSC) and downlink link SNR ceiling (DLSC) request. Shall only be present if dot11MeasurementPilotEnabled is true. |

##### 10.3.32.1.3 When generated

This primitive is generated by the SME to request that a Link SNR Ceiling report be sent to the SME upper layer application, containing link SNR ceiling values computed from information received in a Measurement Pilot frame.

##### 10.3.32.1.4 Effect of receipt

On receipt of this primitive, the MLME shall determine link SNR ceiling based on information received in and measurement of the Measurement Pilot and report this back to the SME in a MLME-MPILOTLSCEILING.confirm primitive.

#### 10.3.32.2 MLME-MPILOTLSCEILING.confirm

##### 10.3.32.2.1 Function

This primitive reports the result of the Measurement Pilot Link SNR Ceiling request.

##### 10.3.32.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MEASPILOTLSCEILING.confirm 	(
ResultCode,
Link SNR Ceiling Direction,
Uplink Link SNR Ceiling,
Downlink Link SNR Ceiling
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, LM INFO UNAVAILABLE | Reports the outcome of a Link SNR request |
| Link SNR Ceiling Direction | Enumeration | ULSC or DLSC or ALL | Differentiates between uplink link SNR ceiling (ULSC) and downlink link SNR ceiling (DLSC) computation. Shall only be present if dot11MeasurementPilotEnabled is true. |
| Uplink Link SNR Ceiling | Integer | 0 – 255 | Uplink link SNR ceiling in dBm computed based on information received in Measurement Pilot frame. Shall only be present if dot11MeasurementPilotEnabled is true. |
| Downlink Link SNR Ceiling | Integer | 0 – 255 | Downlink link SNR ceiling in dBm computed based on information received in Measurement Pilot frame. Shall only be present if dot11MeasurementPilotEnabled is true. |

##### 10.3.32.2.3 When generated

This primitive is generated by the MLME when the Link SNR Ceiling report is ready.

##### 10.3.32.2.4 Effect of receipt

The SME upper layer application receives the Link SNR Ceiling report it has requested.

# 11. MAC sublayer management entity

## 11.1 Synchronization

### 11.1.3 Acquiring synchronization, scanning

Change the 5th paragraph as shown below:

Upon receipt of an MLME-SCAN.request with the SSID parameter set to the wildcard SSID, the STA shall passively scan for any Beacon or Measurement Pilot frames, or actively transmit Probe request frames containing the wildcard SSID, as appropriate depending upon the value of ScanMode. Upon completion of scanning, an MLME-SCAN.confirm is issued by the MLME indicating all of the BSS information received.

#### 11.1.3.2 Active scanning

##### 11.1.3.2.1 Sending a probe response

Change the fourth paragraph as shown below:

STAs receiving Probe Request frames shall respond with a probe response only if the SSID in the probe request is the wildcard SSID or matches the specific SSID of the STA. Furthermore, a STA receiving a probe request with a DS Parameter Set element containing a Current Channel field value that is not the same as the value of dot11CurrentChannelNumber shall not respond with a probe response. Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last beacon shall be the STA that responds to a probe request.

Insert the following paragraphs at the end of  11.1.3.2.1:

When a probe response frame is returned in response to a probe request frame which contains Requested information elements, any of the requested elements which appear as individual items in the ordering list of Table 15 shall appear both in their individual ordered location as specified in Table 15 and in the ordered location reserved for the list of requested elements, where the requested elements appear in the same order as requested in the Request information element.

If dot11RadioMeasurementEnabled is true and if the Request Information element of the Probe Request includes the RCPI element ID, the STA shall include in the Probe Response an RCPI element containing the measured RCPI value of the received Probe Request frame. If no measurement result is available the RCPI value shall be set to indicate that a measurement is not available.

##### 11.1.3.2.2 Active scanning procedure

Insert the following text at the end of this clause:

If dot11RadioMeasurementEnabled is true and if the Request Information element of the Probe Request includes the RCPI element ID, an AP shall include a RCPI element in the Probe Response. An AP may measure RCPI on the received Probe Request frame and include the result in the RCPI element of the Probe Response. If no measurement result is available the RCPI value shall be set to indicate that a measurement is not available.

Change  11.9 as follows:

## 11.9 TPC procedures

ERC/DEC/(99)23 requires RLANs operating in the 5 GHz band to use transmitter power control, involving specification of a regulatory maximum transmit power and a mitigation requirement for each allowed channel, to reduce interference with satellite services. This standard describes such a mechanism, referred to as transmit power control (TPC).

This subclause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and be useful in other frequency bands and may be useful for other purposes (e.g., reduction of interference, range control, reduction of power consumption).

STAs shall use the TPC procedures defined in this subclause if dot11SpectrumManagementRequired is true. dot11SpectrumManagementRequired shall be set to TRUE when regulatory authorities require TPC. It may also be set to TRUE in other circumstances. The TPC procedures provide for the following:

- Association of STAs with an AP in a BSS based on the STAs’ power capability (see 11.9.1).

- Specification of regulatory and local maximum transmit power levels for the current channel (see 11.9.2).

- Selection of a transmit power for each transmission in a channel within constraints imposed by regulatory and local requirements (see 11.9.3).

- Adaptation of transmit power based on a range of information, including path loss and link margin estimates (see 11.9.4).

For the purposes of TPC, the following statements apply:

Convert the following bullet point into a paragraph as shown below:

- A STA with dot11SpectrumManagementRequired set to TRUE shall not operate in a BSS or IBSS unless the Spectrum Management bit is set to 1 in the Capability Information field in Beacon frames and Probe Response frames received from other STAs in the BSS or IBSS, with the following exception.

A STA with dot11SpectrumManagementRequired set to TRUE shall not join a BSS or IBSS unless the Spectrum Management bit is set to 1 in the Capability Information field in Beacons, Measurement Pilot frames and Probe Response frames received from other STAs in the BSS or IBSS, with the following exceptions:.

- A STA may join operate when the Spectrum Management bit is set to 0 if the STA can determine that it is in a regulatory domain that does not require TPC or can ensure that it will meet regulatory requirements even if TPC is not employed. Potential methods for determining the regulatory domain include receiving a country indication in the beacon, Measurement Pilot frame, user confirmation, or configuration information within the device. Potential methods to ensure regulations are met even if TPC is not employed include using a transmit power that is below the legal maximum (including any mitigation factor).

- A STA shall set dot11SpectrumManagementRequired to TRUE before associating with a BSS or IBSS in which the Spectrum Management bit is set to 1 in the Capability Information field in Beacon, Measurement Pilot frames and Probe Response frames received from the BSS or IBSS.

- APs may allow association of devices that do not have the Spectrum Management bit set to 1 in the Capability Information field in Association Request frames and Reassociation Request frames received from the STA to account for the existence of legacy devices that do not support TPC, but do meet regulatory requirements.

Change  11.9.2 as follows:

### 11.9.2 Specification of regulatory and local maximum transmit power levels

A STA shall determine a regulatory maximum transmit power for the current channel. The STA shall by selecting use the minimum of the following:

- Any regulatory maximum transmit power received in a Country element from the AP in its BSS or another STA in its IBSS and

- Any regulatory maximum transmit power for the channel in the current regulatory domain received in the Max Regulatory Power field from the AP in its BSS.

- Any regulatory maximum transmit power for the channel in the current regulatory domain known by the STA from other sources.

A STA shall determine a local maximum transmit power for the current channel. The STA shall by selecting use the minimum of the following:

- Any local maximum transmit power received in the combination of a Country element and a Power Constraint element from the AP in its BSS or another STA in its IBSS and

- Any local maximum transmit power for the channel regulatory domain known by the STA from other sources.

Any calculation of the local maximum transmit power for the channel shall ensure the mitigation requirements for the channel in the current regulatory domain can be satisfied. The conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the mitigation requirement. However, it may be possible to satisfy the mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes (e.g., range control, reduction of interference).

The regulatory and local maximum transmit powers may change in a STA during the life of a BSS. However, network stability should be considered when deciding how often or by how much these maximums are changed. It is the responsibility of the STA receiving the Power Constraint element to determine the authenticity of the element. As such, the receiving STA may need to determine how much to reduce its transmit power beyond the local mitigation requirements in order to prevent adversely affecting normal operation. The regulatory and local maximum transmit powers shall not change during the life of an IBSS.

An AP in a BSS and a STA in an IBSS shall advertise the regulatory maximum transmit power for the current channel in Beacon frames and Probe Response frames using a Country element. An AP in a BSS and a STA in an IBSS shall advertise the local maximum transmit power for the current channel in Beacon frames and Probe Response frames using the combination of a Country element and a Power Constraint element.

When dot11MeasurementPilotEnabled is set to true, an AP in a BSS shall advertise the regulatory maximum transmit power for the current channel in Measurement Pilot frames using a Max Regulatory Power field.

Where TPC is being used for radio resource measurement without spectrum management, the inclusion of a Power Constraint element in Beacon and Probe Response frames shall be optional.

Insert the following new clauses after  11.10:

## 11.11 Radio Measurement Procedures

This clause describes the radio measurements and the procedures for requesting and reporting radio measurements between STAs.

### 11.11.1 Measurement on operating and non-operating channels

Measurements on non-operating channels may require the measuring STA to interrupt its data services on the operating channel, switch channels and make measurements. Measurements on the operating channel may not require the STA to interrupt its data services.

All stations are responsible for maintaining data services and an association or membership with the BSS or IBSS respectively, on the operating channel while performing measurements on non-operating channels.

A STA shall determine the time between successive non-operating channel measurements. This time may be a fixed length, or it may be determined by the STA using application-specific (or other) knowledge.

### 11.11.2 Measurement start time

A Radio Measurement Request frame may contain a single Measurement Request element or a sequence of Measurement Request elements. A STA that accepts the first, or only measurement request within a Radio Measurement Request frame shall start the measurement as soon as practical after receiving the request. Subsequent measurement requests in the Radio Measurement Request frame that are accepted shall start as soon as practical after processing the previous request in the frame. Such measurement start times shall be subject to any specified Randomization Interval.

The Radio Measurement category permits a randomization interval to be specified for measurement start times. The intent of this is to avoid traffic storms that could arise with synchronized broadcast and multicast measurements. Prior to making each measurement in the requested sequence, the STA shall calculate a random delay distributed uniformly in the range 0 to the Randomization Interval specified in the measurement request. The STA shall not start the measurement until this delay has expired. Randomization Interval shall be specified in units of TUs. A Randomization Interval of 0 in a measurement request shall mean that no random delay is to be used.

NOTE—It is important that designers recognize the need for statistical independence among the pseudo random number streams among STAs.

A number of repetitions may be specified in the Radio Measurement Request frame. In this case, the measurements in the frame are repeated as detailed further in  11.11.6. Each time a measurement is repeated, the STA shall recalculate the random delay as described above.

When a Measurement Start Time field is present in a measurement report, the measuring STA shall report the value of its TSF timer at the time the measurement started to an accuracy of +/- 1 TU.

### 11.11.3 Measurement Duration

If the Duration Mandatory bit is set to 1 in the Measurement Request mode field of a measurement request, the requested STA, if it accepts the request, shall perform the measurement over the Measurement Duration specified in the request. If the STA is unable to commit to making the measurement over the requested duration it shall refuse the request by sending a measurement report with the refused bit set in the Measurement Report Mode field. The measurement duration in the measurement report shall be equal to the requested measurement duration.

If the Duration Mandatory bit is set to 0 in the Measurement Request mode field of a measurement request, the requested STA, if it accepts the request, shall attempt a measurement using the requested duration as a target measurement duration, and may report results with an actual measurement duration less than the requested duration. The duration over which the measurement was made shall be included in the measurement duration field of the measurement report.

Each separate measurement within the Radio Measurement Request frame shall be performed over a continuous measurement duration time period. In Measurement Request frames, the requested Measurement Duration value shall not be set to 0 except for Beacon Request with Measurement Mode set to Beacon Table mode and Statistics Request measurements.

A Measurement Duration value of 0 shall only be used in triggered autonomous radio measurement requests.

### 11.11.4 Station responsibility for conducting measurements

A Radio Measurement-capable STA shall decode and interpret each Radio Measurement Request frame that it receives and shall assess the contents against its capabilities and the impact on its own performance. A measurement request can be refused by the receiving STA by sending a Radio Measurement Report with the refused bit set in the Measurement Report Mode field. The reasons for refusing a measurement request are outside the scope of this standard but may include reduced quality of service, unacceptable power consumption, measurement scheduling conflicts, or other significant factors.

In assessing the performance impact of each measurement request element, a STA may use application-specific knowledge or other knowledge to limit the time it spends away from the operating channel. In doing so, the STA may either:

- reject any Measurement Request element that has the Duration Mandatory bit set to 1 and has a mandatory measurement duration exceeding the maximum allowed off-operating channel time, or

- measure for a reduced duration if the Duration Mandatory bit is set to 0.

A STA shall cancel all in-process radio measurements and shall delete all pending, unprocessed radio measurement requests upon receipt of a Disassociation message or upon association or reassociation with a BSSID different from its most recent association.

### 11.11.5 Requesting and reporting of measurements

A STA may measure one or more channels itself or a STA may request peer STAs in the same BSS to measure one or more channels.

When requesting other STAs to measure one or more channels, a STA shall use a Radio Measurement Request frame containing one or more Measurement Request elements. The measurement request may be sent to an individual or group destination address. The permitted measurement requests are shown in Table 82A.

Table 82A—Allowed measurement requests

| Service Set | Source of Request | Destination of Request | Receiver Address of Radio Measurement Request Frame |
| --- | --- | --- | --- |
| Infrastructure BSS | AP | Non-AP STA | Individual or group |
|  | Non-AP STA | AP | Individual only |
|  | Non-AP STA | Non-AP STA | Individual only for Direct Link within a QBSS, otherwise not allowed |
| IBSS | Non-AP STA | Non-AP STA | Individual or group |

The source and destination of a measurement request shall both be a member of the same infrastructure BSS or a member of the same IBSS. Measurement requests with an individual Receiver Address shall only be sent to STAs that have indicated Radio Measurement capability.

The set of requested measurements received in the most recently received Radio Measurement Request frame of highest precedence is active at a STA. The precedence order for measurement requests shall be as follows (highest precedence first):

- Measurement requests received in individually addressed Radio Measurement Request frames

- Measurement requests received in Multicast-group addressed Radio Measurement Request frames

- Measurement requests received in Broadcast addressed Radio Measurement Request frames

The Measurement Request elements shall be processed in sequence by default, with certain Measurement Request elements processed in parallel according to the parallel bit field setting – see 7.3.2.21. If measurement resources are available, the STA processes each element by setting up and making the specified measurement.

The Measurement Request elements within a Radio Measurement Request frame may specify multiple measurement types across multiple channels.

A STA may receive another Radio Measurement Request frame while the measurements requested in a previous Radio Measurement Request frame are pending or in progress. If this request is accepted, the set of measurement requests in the new frame supersedes any previous requests received in a Radio Measurement Request frame of the same or lower precedence. The measuring STA shall report the results of any completed measurements and terminate any pending or in-progress measurements. Results from a terminated in-progress measurement may be valid and reported if Duration Mandatory was not set to 1 in the corresponding request. It is permissible for the superceding Radio Measurement Request frame to contain no new measurement requests. If a station receives a Radio Measurement Request frame with lower precedence than the currently active Radio Measurement Request frame, the station shall discard the measurement requests in the new Radio Measurement Request frame. Measurement Request elements that have the Enable bit set to 1 shall be processed in all received Radio Measurement Request frames regardless of these precedence rules.

If a STA receives a spectrum management Measurement Request with Measurement Type equal to 0 (Basic Request), this shall always take priority over any pending or in-progress radio measurements.

A STA that issues a radio measurement request to another STA to perform a measurement on the operating channel may continue to transmit MPDUs and MMPDUs to that STA while the measurement is being processed.

A STA that issues a radio measurement request to another STA to perform a measurement on a non-operating channel is not required to take any special action to suspend traffic to that STA. All stations shall maintain state information such that data services and association or membership with the BSS or IBSS respectively can continue when returning from a non-operating channel measurement.

The result of each measurement requested in a Measurement Request element shall be reported in one or more Measurement Report elements of type corresponding to the request. Each Measurement Report element returned shall have the same Measurement Token as in the corresponding Measurement Request element. The results of each measurement should be returned without undue delay to the requesting STA.

Measurement Report elements shall be returned to the requesting STA in one or more Radio Measurement Report frames. Each Radio Measurement Report frame shall contain the same Dialog Token field value as the corresponding Radio Measurement Request frame.

When a STA is permanently unable to make a requested measurement the STA shall respond to such a measurement request received within a unicast Radio Measurement Request frame with a measurement report indicating that it is incapable of completing the measurement request. A STA shall not respond to requests received in broadcast and multicast frames in this manner. Examples of when an incapable response is appropriate are:

—	The requested measurement type is not supported.

—	The measuring STA cannot support requested parallel measurements due to the requests relating to different channels.

A STA that receives a response with an incapable indication shall not make the same request to the responding STA during the lifetime of the current association, or IBSS membership. This is logically the same as the responding STA using the Enable and Request bits in a measurement request to indicate that it will not accept measurement requests of a certain type. A STA that has indicated an incapable response to a requesting STA may discard further requests of the same type from that STA.

A STA may refuse to make any requested measurement. A STA refusing a measurement request within a unicast Radio Measurement Request frame shall respond with a measurement report indicating that it is refusing the measurement request. A STA shall not respond to measurement requests received in broadcast or multicast Radio Measurement Request frames in this manner.

By default, a STA may send a radio measurement request of any defined measurement type. A STA that receives a Measurement Request element with the Enable bit set to 1 and the Request bit set to 0 shall not issue measurement requests of the Measurement Type type in the request to the STA from which the element was received.

Note—Since measurements on non-operating channels could potentially degrade a station’s performance, non-operating channel measurements should be requested sparingly and for short durations. Since measurements on the operating channel execute concurrently with normal traffic processing, operating channel measurements may be requested more frequently and for longer durations.

### 11.11.6 Repeated measurement request frames

Radio Measurement Request frames contain a field specifying the number of repetitions for the Radio Measurement Request frame. If the Radio Measurement Request frame includes a non-zero value for the Number of Repetitions, the STA shall iterate (repeat) the processing of all the Measurement Request elements in the frame as specified by the value in the Number of Repetitions field. When completing the processing of the last Measurement Request element in the frame, the STA shall begin processing of the first Measurement Request element in the frame to repeat the frame until the number of iterations reaches the value in the Number of Repetitions field. Measurement Request elements with the Enable bit set to 1 shall be processed once regardless of the number of repetitions.

Each repeated measurement result shall include the Measurement Token value as in the corresponding Measurement request element and the Dialog Token value as in the corresponding Radio Measurement Request frame.

Measurement results shall be reported for each repetition of a repeated measurement request subject to any conditional reporting requirement.

STAs responding with incapable or refused indications to measurement requests within a Radio Measurement Request frame with a non-zero value for Number of Repetitions shall only respond once.

### 11.11.7 Triggered Autonomous Reporting

Autonomous reporting is defined for Spectrum Management measurements supporting DFS – see 11.6.6. It allows a STA to report the results of measurements to a peer STA for which there was no explicit measurement request. In this case, the transmission of autonomous reports shall be entirely the decision of the STA at which such reporting has been enabled. An example of this use would be to report a change in conditions at the STA observed as a result of background measurement, e.g. the presence of a radar signal.

In radio measurement, triggered autonomous reporting shall be subject to trigger conditions set by the enabling STA that determine when measurement reports are issued. This is termed triggered autonomous reporting and provides a method for reporting during continuous background measurement. An example of the use of triggered autonomous measurement is for reporting problem conditions in continuous, non-invasive statistical monitoring.

Triggered autonomous reporting is defined only for the QoS Metrics measurement type – see 11.11.8.8.

A STA indicates that it wishes to accept triggered autonomous reports by sending a Measurement Request element with the Enable and Report bits set to 1 – see 7.3.2.21. The type of measurement is indicated in the Measurement Type field. Trigger conditions that determine when measurement reports are to be generated shall be specified in the Measurement Request field. A Measurement Request element that is being used to control triggered autonomous reporting shall be sent within a Radio Measurement Request frame. Measurement Request elements being used to request measurements may also appear in the same Radio Measurement Request frame. The Radio Measurement Request frame may be sent to a group receiver address to enable triggered autonomous reports at more than one STA.

A STA shall not send autonomous reports for radio measurement types having triggered autonomous reporting enabled without the trigger conditions having been met.

If a request to enable triggered autonomous reporting is sent to an individual address and the recipient STA does not support measurements of the type indicated, a Measurement Report element shall be returned to the requesting STA with the Incapable bit set. A STA may also refuse to enable triggered autonomous reporting. In this case a Measurement Report element shall be returned to the requesting STA with the refused bit set. Such responses shall not be issued if the request to enable triggered autonomous reporting was sent to a group address.

A STA receiving a request to enable triggered autonomous reporting from another STA may send reports of the appropriate type, addressed to the individual address of the STA that sent the enable request. Autonomous reports shall only be sent to the individual addresses of STAs from which a valid enable request has been received and shall only be issued when the trigger conditions have been met. The Measurement Token in each Measurement Report element and the Dialog Token value in the Measurement Report frame shall both be set to 0 in a triggered autonomous report.

A STA may update the trigger conditions set for triggered autonomous reports by issuing a new Measurement Request element with the Enable and Report bits both set to 1, the Measurement Type field set to the appropriate type and the Measurement Request field indicating the new trigger conditions. A STA disables all triggered autonomous measurement reports by sending a Measurement Request element with the Enable bit set to 1 and the Report bit set to 0 – see 7.2.3.21.

A STA in an infrastructure BSS shall cease all triggered autonomous reporting if it disassociates, or re-associates to a different BSS (re-association to the same BSS shall not affect triggered reporting). A STA in an independent BSS shall cease all triggered autonomous reporting if it leaves the BSS.

Triggered autonomous reporting and requested measurements are independent: a STA may request measurements from another STA even if it has enabled triggered autonomous reporting from that STA. All Measurement Request elements received in Radio Measurement Request frames that have the Enable bit set shall be processed without regard for the measurement precedence rules for requested measurements in 11.11.5.

### 11.11.8 Specific measurement usage

#### 11.11.8.1 Beacon Report

If a STA accepts a Beacon Request it shall respond with a Radio Measurement Report frame containing Beacon Measurement Reports for all observed BSSs matching the BSSID and SSID in the Beacon Measurement Request. The RCPI in the Beacon Report indicates the power level of the received Beacon, Measurement Pilot or Probe Response frame. RCPI in a Beacon Report element may be used with other Beacon Report elements to determine the most suitable AP target for BSS transitions. For repeated measurements (when the Measurement Request frame contains a non zero value for the Number of Repetitions field), the transmission of the Beacon Report element may be conditional on the measured RCPI or RSNI value. When the Measurement Request frame contains a 0 value for the Number of Repetitions field, the Reporting Condition field in all Beacon Requests in that frame shall be set to 0. Table 29B lists the reporting conditions that are based on the measured RCPI or RSNI levels.

If the Measurement Mode in the measurement request is Passive, the measuring STA shall perform the following procedure (or an equivalent procedure) on the requested channel:

- Set a measurement duration timer.

- At the end of the measurement duration, process all received Beacon, or Probe Response management frames with the requested SSID and BSSID to compile the measurement report. If no Beacons or Probe Responses with the requested SSID and BSSID were received in the measurement duration process all Measurement Pilot Frames with the requested BSSID to compile the measurement report. Otherwise, compile an empty Beacon measurement report.

If the Measurement Mode in the measurement request is Active, the measuring STA shall perform the following procedure (or an equivalent procedure) on the requested channel:

- If the channel is not the operating channel, wait for dot11RadioProbeDelay, or until a PHY-RXSTART.indication has been received.

- Using the basic access protocol in 9.2.5.1, send a Probe Request management frame to the broadcast destination address (DA). The BSSID field in the Probe Request shall be set to the BSSID field in the measurement request. The SSID element in the Probe Request shall be set to the SSID element in the measurement request.

- Set a measurement duration timer.

- At the end of the measurement duration, process all received Probe Response and Beacon management frames with the requested SSID and BSSID to compile the measurement report. If no Beacons or Probe Response frames were received in the measurement duration and Measurement Pilot frames with the requested BSSID were received in the measurement duration then process all these Measurement Pilot Frames to compile the measurement report. Otherwise, compile an empty Beacon measurement report.

When more than one Beacon or Probe Response from a BSS is received in the measurement duration, the contents of the Beacon Report shall be based on the latest received. If only Measurement Pilot frames were received in the measurement duration the contents of the Beacon Report shall be based on the latest Measurement Pilot frame received.

If the BSSID field in the Measurement Request contains a broadcast BSSID, all observed BSSs with the requested SSID shall be reported in a separate Beacon Report element for each BSSID. If the SSID element in the Measurement Request contains the wildcard SSID, all observed BSSs shall be reported in a separate Beacon Report element for each BSSID. In Active mode, Probe Response frames shall be evaluated regardless of whether or not the Probe Response frame was triggered by the measuring STA's Probe Request.

On accepting an active or passive mode Beacon measurement request with Channel Number set to 0 a STA shall conduct iterative measurements on all supported channels in the specified Regulatory Class where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. Measurements shall be made using the specified Measurement Duration with the time between each consecutive measurement as defined in 11.11.1. Iterative measurements shall cease when all supported channels have been measured. While the STA is processing a Beacon measurement request for interative channel measurements, the STA may not begin processing the next measurement request in the measurement request frame.

On accepting an active or passive mode Beacon measurement request with Channel Number set to 255 a STA shall conduct iterative measurements on all supported channels listed in the AP Channel Report where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. For iterative beacon measurements, the measurement duration applies to the measurement on each channel. Measurements shall be made using the specified Measurement Duration with the time between each consecutive measurement as defined in 11.11.1. Iterative measurements shall cease when all supported channels have been measured. A Beacon measurement request with Channel Number set to 255 shall only specify a specific BSSID (broadcast BSSID not allowed). If an AP Channel Report for the specific BSSID is not available in the STA, the STA shall iteratively conduct measurements on all supported channels in the specified Regulatory Class that are valid for the current regulatory domain. While the STA is processing a Beacon measurement request for interative channel measurements, the STA may not begin processing the next measurement request in the measurement request frame.

If the Measurement Mode in the measurement request is Beacon Table, the measuring STA shall return a Beacon Report containing the current contents of any stored beacon information for any supported channel with the requested SSID and BSSID without performing additional measurements. The receiving STA shall ignore the channel and measurement duration specified in the Beacon Request when Beacon Table mode is selected. The beacon information accumulated may be the result of any operation that caused the STA to acquire these results. If the stored beacon information is based on a measurement made by the reporting STA, and if the actual measurement start time, measurement duration and Parent TSF are available for this measurement, then the beacon report shall include the actual measurement start time, measurement duration and Parent TSF; otherwise the actual measurement start time, measurement duration, and Parent TSF shall be set to 0. The RCPI and RSNI for that stored beacon measurement may be included in the beacon report; otherwise the beacon report shall indicate that RCPI and RSNI measurements are not available. The channel number, regulatory class and reported frame information for that stored measurement may be included in the beacon report; otherwise these fields shall be set to 255 in the beacon report. If the STA has no beacon information available then the STA may either refuse the request or send an empty Beacon Report.

For repeated measurements, the Beacon Request element may specify a reporting condition that determines when the measuring STA is to send a Beacon Report element for a measured Beacon, Measurement Pilot or Probe Response frame with the requested BSSID. When the requested Reporting Condition value is non zero, the STA shall create and transmit a Beacon Report element for that measured frame only if the condition indicated in Table 29B is true. Otherwise, a Beacon Report element is not created for that measured frame. If multiple Beacons, Measurement Pilots or Probe Response frames with the requested specific BSSID are received during the measurement duration, the reporting condition shall only be applied to the latest received Beacon, Measurement Pilot or Probe Response. If multiple Beacons, Measurement Pilots or Probe Response frames are received during the measurement duration when a broadcast BSSID is requested, the STA shall generate one Beacon Report element for each BSSID ocurring in frames which satisfy the reporting condition; the Beacon Report element shall be based on the latest received Beacon, Measurement Pilot or Probe Response for that specific BSSID. For reporting conditions 5-10, the serving AP's reference RCPI level and the serving AP's reference RSNI level referred to in Table 29B are average values of the RCPI or RSNI of the 10 most recent Beacon frames received from the measuring STA's serving AP. The serving AP's reference RCPI level and the serving AP's reference RSNI level are so averaged to provide a more accurate and stable indication of the signal level from the serving AP. For reporting conditions 5-10, the STA shall use the serving AP's reference RCPI level or reference RSNI level (with offset, if any) to test the measured RCPI oI RSNI to determine whether to create and send a Beacon Report element for this measured Beacon, Measurement Pilot or Probe Response frame.

11.11.8.2 Frame Report

If a station accepts a Frame Request it shall respond with a Radio Measurement Report frame containing one or more Measurement (Frame) Report elements.

If the MAC Address field was included in the Frame Request, a Frame Report Entry where Transmitter Address (TA) matches the MAC address in the Frame Request shall be included in the Frame Report if at least one data or management frame was received with this Transmitter Address during the measurement duration. If the MAC address field was not included in the Frame Request in response of which this Frame Report is being generated, the measuring station shall receive all observable traffic during the measurement duration and shall summarize this traffic in one or more Frame Report elements.

Each Frame Report element contains one or more Frame Report Entries. The measuring station shall count the number of unicast data and management frames received from one transmit address during the measurement duration and shall summarize this traffic in a Frame Report Entry.

Each Frame Report Entry contains the Transmit Address, BSSID, PHY Type, Average RCPI, Last RCPI, Last RSNI, Antenna ID and Frame Count for the frames counted in this Frame Report Entry.

The reported Average RCPI shall be an average of the RCPI values of frames received and counted in the Frame Report Entry. If there are up to 128 frames, then the Average RCPI indicates the mean of the RCPI of each of the frames. If there are more than 128 frames, then the Average RCPI indicates an exponentially weighted average, initialized by the mean RCPI of the first 128 frames and exponentially updated by new RCPI values. The averaging is calculated as depicted below.

If the number of frames is less than or equal to 128:

Average RCPI = Sum of RCPI values / Number of frames

For 129 frames and above:

Average RCPI = (Last Average RCPI * (N-1) / N) + (Current frame RCPI / N), where N = 128.

The Last RCPI shall be the RCPI value of the most recently received frame counted in the Frame Report Entry. The Antenna ID field contains the identifying number for the antenna used to receive the most recently received frame included in this Frame Report Entry as defined in section 7.3.2.39. If different antennas are used to receive the frame preamble and the frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the frame body.

#### 11.11.8.3 Channel Load Report

If a station accepts a Channel Load Request it shall respond with a Radio Measurement Report frame containing one Measurement (Channel Load) Report element. The Channel Load value is defined as Integer(256*[channel busy time (microseconds)]/(1024 * [measurement duration (TU)])). Channel busy time shall be the time during which either the physical carrier sense or the virtual carrier sense (NAV) or both indicates that the channel is busy. See 9.2.1 for description of carrier sense mechanism.

#### 11.11.8.4 Noise Histogram Report

If a station accepts a Noise Histogram Request it shall respond with a Radio Measurement Report frame containing one Measurement (Noise Histogram) Report element. The Noise Histogram Report shall contain the IPI densities observed in the channel for the nine IPI levels defined in Table 31H.

To compute the IPI densities, the STA shall measure the IPI in the specified channel as a function of time over the measurement duration when NAV is equal to 0 (when virtual CS mechanism indicates idle channel) except during frame transmission or reception. The time resolution of the IPI measurements shall be in microseconds. The IPI densities are then computed for each of the nine possible IPI values Integer (256 * ([Duration receiving at IPI value (microseconds)] / ((1024 * [Measurement Duration (TU)]) – [NAVBUSY (microseconds)] - [Ttx (microseconds)] - [Trx (microseconds)]))). Ttx is the frame transmission time during the Measurement Duration. Trx is the frame reception time during the Measurement Duration. NAVBUSY is the total time in microseconds that NAV is non-zero during the Measurement Duration. The sum of the IPI densities will be approximately 256. If either NAV is equal to one or if frame transmission or reception covers the entire measurement duration period, no reportable IPI values will be measured, and all IPI Densities shall be set to 0 in the Measurement Report element.

A STA shall include in the Noise Histogram Report an Average Noise Power Indicator (ANPI) value representing the average noise plus interference power on the measured channel at the antenna connector during the measurement duration. The STA may use Noise Histogram IPI density values to calculate ANPI. The IPI densities in the Noise Histogram Report may be used to calculate an average noise power for the channel during the measurement duration. This calculated average IPI power value may be reported as the value for ANPI. Any equivalent method to measure ANPI may also be used. ANPI power is defined in dBm using the same units and accuracy as defined for RCPI.

ANPI may be calculated over any period and for any received frame. ANPI may be calculated in any period using all IPI values reported by PHY during an idle channel (when NAV is equal to 0). Furthermore, ANPI may be calculated at any time using IPI values stored a first-in-first-out buffer of all IPI values reported by PHY during an idle channel (when NAV is equal to 0). ANPI may be so calculated upon receipt of any frame and may be used with RCPI to calculate RSNI for any received frame. Any equivalent method to measure ANPI may also be used to calculate RSNI for any received frame.

#### 11.11.8.5 STA Statistics Report

If a station accepts a STA Statistics Request it shall respond with a Radio Measurement Report frame including one STA Statistics Report element. If the Requested Measurement Duration value is 0, the STA shall report the current values for the requested Statistics Group Data. If the Requested Measurement Duration value is greater than 0, the STA Statistics Report reports the change in the requested Statistics Group Data measured within that non-zero Measurement Duration. The reported change in data value shall be the value of the data at the end of the actual Measurement Duration minus the value of the data at the beginning of the actual Measurement Duration. If a STA accepts a Statistics Request measurement with non-zero, positive Measurement Duration, the STA shall perform the measurement over ther requested Measurement Duration without regard to the Duration Mandatory bit in the Measurement Request Mode field. If a STA cannot measure over the requested Measurement Duration, the STA shall refuse the Statistics Request measurement.

#### 11.11.8.6 Location Configuration Information Report

If a STA accepts an LCI request, it shall respond with a Radio Measurement Report frame including one Location Configuration Information element (LCI Report).

NOTE—IETF RFC 3825, section 2.1, defines formats and information fields for reporting physical location to sub-centimeter resolution. The fixed-point values have integer and fractional parts, which together represent Latitude, Longitude or Altitude to 34-, 34-, or 30-bits. A Latitude report with 24-bit resolution would be reporting with a precision of about 3.18 m in Latitude at the equator. The physical location and azimuth MIB information of the STA may be set by administrative means.

The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation in the regulatory domain.

If the Altitude Type is 2 (Floors of Altitude), the value reported shall be as required for operation in the regulatory domain.

An LCI request shall indicate a location request for the requesting STA or the reporting STA by setting the LCI request Location Subject octet to indicate a Local or Remote request respectively. Local LCI Measurement Request is used by requesting STA to obtain its own location by asking "Where am I?". Remote LCI Measurement Request is used by requesting STA to obtain location of reporting STA by asking "Where are you?".

If the STA receiving an LCI request lacks the means to report the requested location to the requested resolution, then the LCI Report shall have that corresponding Latitude, Longitude, Altitude or Azimuth resolution set to zero, otherwise Latitude, Longitude, Altitude and Azimuth fields shall be reported to their full resolutions, which may exceed the requested resolutions.

If the STA receiving an LCI request has no location information about the requested LCI Subject physical location or requested Azimuth, it shall set the Incapable bit in the Measurement Report Mode field. The method by which the physical location and azimuth information in the LCI Report is generated is outside the scope of this standard.

NOTE—A STA that requested a ‘Local’ LCI and received an LCI Report with the Incapable bit set, may alternatively make a ‘Remote’ LCI request to obtain the reporting STA’s physical location. A STA that requested an LCI including an Azimuth Request, and received an LCI Report with the Incapable bit set may alternatively request the LCI with no Azimuth Requested.

#### 11.11.8.7 Measurement Pause

A Measurement Pause is used within a Measurement Request frame to provide a time delay between the processing of two other Measurement Request Elements within the sequence of Measurement Request Elements in that frame.

If a STA accepts a Measurement Pause Request it shall delay processing of the next measurement request in the Measurement Request frame. If the Measurement Pause Request is the last Request Element in a repeated Measurement Request frame, the STA shall delay processing the first Request Element in the Measurement Request frame for the next repeat. In each case the delay shall be no less than the Pause Time value specified in the Measurement Pause Request.

A Measurement Pause shall not be sent as the only Request Element in a Measurement Request frame. A Measurement Pause shall not be included as the last Request Element in a Measurement Request frame which has the Number of Repetitions field set to 0.

A measurement pause cannot be processed in parallel to other measurements. If the Parallel bit is set in the Measurement Request element immediately prior to a Measurement Pause an incapable response shall be returned.

There is no measurement report associated with a Measurement Pause Request.

#### 11.11.8.8 QoS Metrics Report

A QSTA receiving a QoS Metrics Request shall respond with a Radio Measurement Report frame containing one Measurement (QoS Metrics) Report element. If the traffic stream (TS) that is corresponding to the Traffic Identifier is deleted, either by a DELTS Action Frame or by disassociation, the STA shall cease sending Radio Measurement Reports.

The QoS Metrics measurement shall be made on traffic that is transmitted from the measuring QSTA to the peer QSTA and TID indicated in the request. The Peer QSTA Address may be the MAC address of the QSTA from which the Measurement Request was sent, the MAC address of another QSTA within the QBSS, or the broadcast address. This enables a QAP to query QoS Metrics for DLS links. A broadcast address shall only be used with a TID corresponding to a TC and shall mean that measurement is to be made on all traffic for the specified TC. Depending on policy, a QAP may not permit QoS Metrics measurement requests for traffic to other QSTAs in the BSS. In this case the QAP shall respond with an incapable indication.

If the measurement request included multiple QoS Metrics requests each for a unique TID, the corresponding measurement report shall include a QoS Metrics report for every TID in the request which has been admitted. If the measurement request is for a TID that has not been admitted yet, a report is generated only after the TID becomes admitted.

The requesting and reporting STAs must be QSTAs. A non-QSTA receiving a QoS Metrics Measurement Request shall reject the request with indication of “incapable”.

A QSTA may request that a measuring QSTA send a QoS metrics report when the number of MSDUs for a specified TID are discarded or delayed reaches a specified threshold. This is termed a triggered QoS metrics measurement and shall be requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the QoS Metrics Measurement Type. The Measurement Request field shall contain a QoS Metrics Request with the trigger conditions specified in the Triggered Reporting field. One or more trigger conditions may be set with specified thresholds. See 7.3.2.21.10.

Depending on policy, a QAP may not permit the establishment of triggered QoS metrics measurement. Such a QAP receiving a triggered QoS metrics request shall give an incapable indication. The number of simultaneous triggered QoS metrics measurements supported at a QSTA is outside the scope of the standard. A STA shall respond to further requests with a refused indication if the number of simultaneous triggered QoS measurements supported by the STA is reached.

A QSTA accepting a triggered QoS measurement shall measure the requested TC, or TS. If a trigger condition occurs, the measuring QSTA shall send a QoS metrics measurement report to the requesting QSTA. The measuring QSTA shall not send further triggered QoS reports until the Trigger Timeout period specified in the request has expired, or new trigger conditions have been requested. Measurement of QoS Metrics shall continue during the reporting timeout period. Reporting shall resume following the Trigger Timeout period, or immediately following the acceptance of new trigger conditions.

If a non-AP QSTA receives a requested QoS metrics measurement for a TC, or TS that is already being measured using a triggered QoS metrics measurement, the triggered QoS measurement shall be suspended for the duration of the requested QoS measurement. When triggered measurement resumes the QoS metrics shall be reset.

QoS metrics reported in a triggered QoS metrics report shall be the values accumulated over the number of transmitted MSDUs prior to the trigger event given in the Measurement Count field of the QoS metrics measurement request that established the trigger condition. It is possible that a consecutive or delay trigger event occurs after acceptance of a triggered QoS metrics measurement but before the number of MSDUs in Measurement Count have been transmitted. In this case the report shall be the values accumulated since measurement started. The measurement count value appears in the Transmitted MSDU Count field of a triggered QoS metrics measurement report. Measurement duration shall not be used in triggered QoS measurement and the Measurement Duration field in both the Measurement Request and any Measurement Report shall be set to 0.

The Measurement Start Time field of a triggered QoS metrics report shall contain the value of the QSTA TSF timer at the time the trigger condition occurred to an accuracy of ±1TU.

Once accepted by a measuring QSTA, a triggered QoS measurement continues to be active until:

- The relevant TS is deleted

- The measuring non-AP QSTA, or non-AP QSTA that requested the measurement disassociates or successfully reassociates, or

- The measurement is terminated by the requesting QSTA.

All triggered QoS measurements shall be terminated at a measuring QSTA by receiving a triggered QoS metrics measurement request with the Enable bit set to 1 and the Report bit set to 0. A triggered QoS measurement request with no trigger conditions specified in the Trigger Conditions field shall terminate a triggered QoS measurement for the TC, or TS specified in the request. A QSTA requesting a triggered QoS measurement may update the trigger conditions by sending a triggered QoS metrics measurement request specifying the new trigger conditions.

## 11.12 Usage of the Neighbor Report

A Neighbor Report is sent by an AP and it contains information on validated AP’s that are members of ESSs requested in the Neighbor Report Request. A Neighbor Report may not be exhaustive either by choice, or due to the fact that there may be neighbor APs not known to the AP. The Neighbor Report contents are derived from the NeighborListSet parameter of the MLME-NEIGHBORREPRESP.request. The mechanism by which the contents of this table are determined is outside the scope of this amendment, but it may include information from measurement reports received from the STA’s within the BSS, information obtained via a management interface, or the DS.

NOTE—The purpose of the Neighbor Report is to enable the STA to optimize aspects of neighbor BSS transition and ESS operation. A Neighbor Report element contains information on APs which the STA may use as candidates for a BSS transition. Since the information in the Neighbor Report may be stale, it should be considered advisory; information obtained by the report recipient through a scan or other sources may also be considered, possibly overriding information in the Neighbor Report. For example, where information contained within a Neighbor Report is contradicted by information in the Measurement Pilot, Beacon or Probe Response, that response information should take precedence.

### 11.12.1 Requesting a Neighbor Report

An associated STA requesting a Neighbor Report shall send a Neighbor Report Request frame to its associated AP.

### 11.12.2 Receiving a Neighbor Report

An AP receiving a Neighbor Report Request shall respond with a Neighbor Report Response frame containing zero or more Neighbor Report elements. If an SSID element is specified in the corresponding Neighbor Report Request frame, the Neighbor Report element shall only contain information concerning neighbor APs that are members of the current ESS identified by the SSID element contained within the Neighbor Report Request. If the SSID element is omitted the Neighbor Report elements shall contain information concerning neighbor APs that belong to the same ESS as the requesting STA. The wildcard SSID will return all known Neighbors in the Neighbor Report. If there are no neighbor APs available the AP shall send a Neighbor Report Response frame with no Neighbor Report elements.

A serving AP shall include a TSF information field in the Neighbor Report element only if it is able to guarantee an accumulated error of ±1.5 TU or better on the TSF Offset subfield.

## 11.13 Link Measurement

A STA may use a Link Measurement Request frame to request another STA to respond with a Link Measurement Report frame containing a TPC report element. A STA receiving a Link Measurement Request frame shall respond with a Link Measurement Report frame containing a TPC Report element indicating the power used to transmit the Link Measurement Report. The Link Measurement Report also contains antenna ID and signal quality (RCPI and RSNI).

## 11.14 Measurement Pilot generation and usage

The Measurement Pilot frame is a compact management frame periodically transmitted by an AP with a relatively small interval as compared to a Beacon Interval. The Measurement Pilot frame is designed to provide a minimal set of information as compared to a Beacon frame to allow for the required small interval. The purpose of the Measurement Pilot frame is to provide timely information to a STA for the following functions:

- Rapid discovery of a BSS via passive scanning.

- Rapid collection of neighbor measurements via passive scanning.

- Link SNR calculation for various purposes such as transition decisions or initial rate selection.

### 11.14.1 Measurement Pilot generation by an AP

If dot11MeasurementPilotEnabled is true, an AP shall maintain a Measurement Pilot generation function, which transmits Measurement Pilot frames at the lowest basic rate according to the dot11MeasurementPilotPeriod attribute within the AP and according to the following rules:

- The AP defines a series of TMPTTs exactly dot11MeasurementPilotPeriod apart. A TMPTT arrives when the AP’s local TSF timer (in s) modulo the Measurement Pilot Interval equals zero.

- At each TMPTT, the AP shall schedule a Measurement Pilot as the next frame for transmission unless the TMPTT is coincident with a TBTT, in which case the AP shall not generate the Measurement Pilot.

- The QAP shall schedule and transmit Measurement Pilots using the AC specified in the dot11MeasurementPilotTransmitPriority attribute.

In case the medium is determined by the carrier-sense mechanism (see 9.2.1) to be unavailable at the TMPTT, the AP shall delay the actual transmission of a Measurement Pilot according to the basic medium access rules specified in Clause 9 for a maximum period of one dot11MeasurementPilotPeriod and drop the delayed Measurement Pilot at the next TMPTT. In this way, a continuously busy medium will cause multiple successive Measurement Pilots to be delayed, then dropped.

An AP shall transmit Measurement Pilots to the broadcast address. An AP shall not retransmit or buffer Measurement Pilots as part of the PSP mechanisms.

### 11.14.2 Measurement Pilot Link SNR ceiling calculation

A STA may calculate link SNR ceiling with information received in a Measurement Pilot, and may use it to assess the current link condition, and to assist in roaming decisions. SNR ceiling represents the maximum SNR expected on this link under the current conditions for any frames transmitted from the AP at the Max Transmit Power indicated in the Measurement Pilot frame. Link SNR ceiling is calculated as follows:

Let RCPIMaxPwr representing the RCPI that would have been received by a STA in a downlink frame if the frame were sent with the Max Transmit Power. Furthermore assume that a STA knows it’s own “STA Noise Floor” (in dBm) and “STA Max Transmit Power”. Then:

RCPIMaxPwr = RCPI + (Max Transmit Power – Transmit Power Used)

Downlink Link SNR Ceiling (DLSC) = RCPIMaxPwr – STA Noise Floor

Uplink Link SNR Ceiling (ULSC) = RCPIMaxPwr – (Max Transmit Power – STA Max Transmit Power) – Transceiver Noise Floor

# 12. PHY service specification

## 12.3 Detailed PHY service specifications

### 12.3.4 Basic service and options

#### 12.3.4.3 PHY-SAP service primitives parameters

Change row 4 and insert 2 new rows into Table 85 as shown:

Table 85—PHY-SAP service primitive parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RXVECTOR | PHY-RXSTART.indication PHY-RXEND.indication | A set of parameters |
| IPI-STATE | PHY-CCARESET.request PHY-CCARESET.confirm | IPI-ON, IPI-OFF |
| IPI-REPORT | PHY-CCA.indication PHY-CCARESET.confirm | A set of IPI values for the preceding time interval |

### 12.3.5 PHY-SAP detailed service specification

#### 12.3.5.8 PHY-CCARESET.request

Change the following clauses as shown:

##### 12.3.5.8.1 Function

This primitive is a request by the MAC sublayer to the local PHY entity to reset the CCA state machine and to turn IPI reporting on and off by means of the IPI-STATE parameter.

##### 12.3.5.8.2 Semantics of the service primitive

The semantics of the primitives are as follows:

PHY-CCARESET.request(IPI-STATE)

This primitive has no parameters.

The IPI-STATE parameter shall be present if dot11RadioMeasurementEnabled is true. The IPI-STATE parameter can be one of two values: IPI-ON or IPI-OFF. The parameter value is IPI-ON when the MAC sublayer is requesting the PHY entity to report IPI values when the PHY is neither receiving nor transmitting an MPDU. IPI-ON turns on IPI reporting in the PHY entity. IPI-OFF turns off IPI reporting in the PHY entity.

##### 12.3.5.8.3 When generated

This primitive is generated by the MAC sublayer for the local PHY entity at the end of a NAV timer. This request can be used by some PHY implementations that may synchronize antenna diversity with slot timings.

##### 12.3.5.8.4 Effect of receipt

The effect of receipt of this primitive by the PHY entity is to reset the PLCP CS/CCA timers to the state appropriate for the end of a received frame. If IPI-STATE parameter is set to IPI-ON, the PHY entity collects IPI values when it is not transmitting or receiving and provides those values to the MAC sublayer using the IPI-REPORT parameter.

#### 12.3.5.9 PHY-CCARESET.confirm

Change the following clauses as shown:

##### 12.3.5.9.1 Function

This primitive is issued by the PHY to the local MAC entity to confirm that the PHY has reset the CCA state Machine and to provide observed IPI values when IPI reporting is turned on.

##### 12.3.5.9.2 Semantics of the service primitive

The semantics of the primitives are as follows:

PHY-CCARESET.requestconfirm(IPI-STATE, IPI-REPORT)

This primitive has no parameters.

The IPI-STATE parameter shall be present if dot11RadioMeasurementEnabled is true. The IPI-STATE parameter can be one of two values: IPI-ON or IPI-OFF. The IPI-STATE value shall be set to the value of IPI-STATE received by the PHY entity in the most recent PHY-CCARESET.request.

The IPI-REPORT parameter shall be present if dot11RadioMeasurementEnabled is true and if IPI reporting was turned on prior to the receipt of the latest PHY-CCARESET.request. The IPI-REPORT parameter provides a set of IPI values for a time interval. The set of IPI values are recent values observed by the PHY entity since the generation of the most recent PHY-TXEND.confirm, PHY-RXEND.indication, PHY-CCARESET.confirm, or PHY_CCA.indication, whichever occurred latest.

#### 12.3.5.10 PHY-CCA.indication

Change the following clauses as shown:

##### 12.3.5.10.1 Function

This primitive is an indication by the PHY to the local MAC entity of the current state of the medium and to provide observed IPI values when IPI reporting is turned on.

##### 12.3.5.10.2 Semantics of the service primitive

The primitive provides the following parameter:

PHY-CCA.indication (STATE, IPI-REPORT)

The STATE parameter can be one of two values: BUSY or IDLE. The parameter value is BUSY if the channel assessment by the PHY determines that the channel is not available. Otherwise, the value of the parameter is IDLE.

The IPI-REPORT parameter shall be present if dot11RadioMeasurementEnabled is true and if IPI reporting has been turned on by the IPI-STATE parameter. The IPI-REPORT parameter provides a set of IPI values for a time interval. The set of IPI values may be used by the MAC sublayer for Radio Measurement purposes. The set of IPI values are recent values observed by the PHY entity since the generation of the most recent PHY-TXEND.confirm, PHY-RXEND.indication, PHY-CCARESET.confirm, or PHY_CCA.indication, whichever occurred latest.

#### 12.3.5.11 PHY-RXSTART.indication

##### 12.3.5.11.2 Semantics of the service primitive

Change the third paragraph as shown:

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. This vector may contain both MAC and MAC management parameters. The required parameters are listed in 12.3.4.4.

#### 12.3.5.12 PHY-RXEND.indication

##### 12.3.5.12.2 Semantics of the service primitive

Change the first two paragraphs as shown:

The primitive provides the following parameters:

PHY-RXEND.indication (RXERROR, RXVECTOR)

Insert the following text after the third paragraph:

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. RXVECTOR is an included parameter only when dot11RadioMeasurementEnabled is true. This vector may contain both MAC and MAC management parameters. The required parameters are listed in 12.3.4.4.

# 15. DSSS PHY specification for the 2.4 GHz band designated for ISM applications

## 15.2 DSSS PLCP sublayer

### 15.2.7 Receive PLCP

Change the second and fifth paragraph of 15.2.7 as shown:

The receive PLCP is shown in Figure 230.

In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management via the PLME, the PHY is set to the appropriate channel and the CCA method is chosen. Other receive parameters such as RSSI, RCPI, signal quality (SQ), and indicated DATARATE may be accessed via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled (according to 15.4.8.4) as the RSSI reaches the ED_THRESHOLD and/or PMD_CS shall be enabled after code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate according to 15.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for energy detection (ED) and/or code lock prior to correct reception of the PLCP frame. The PMD primitives PMD_SQ and PMD_RSSI are issued to update the RSSI and SQ parameters reported to the MAC.

After PHY-CCA.indicate is issued, the PHY entity shall begin searching for the SFD field. Once the SFD field is detected, CRC-16 processing shall be initiated and the PLCP IEEE 802.11® SIGNAL, IEEE 802.11® SERVICE and LENGTH fields are received. The CRC-16 FCS shall be processed. If the CRC-16 FCS check fails, the PHY receiver shall return to the RX IDLE state as depicted in Figure 98. Should the status of CCA return to the IDLE state during reception prior to completion of the full PLCP processing, the PHY receiver shall return to the RX IDLE state.

If the PLCP Header reception is successful (and the SIGNAL field is completely recognizable and supported), a PHY-RXSTART.indicate(RXVECTOR) shall be issued. The RXVECTOR associated with this primitive includes the SIGNAL field, the SERVICE field, the MPDU length in octets (calculated from the LENGTH field in microseconds), the antenna used for receive (RX_ANTENNA), RSSI, RCPI, and SQ.

Change Figure 230 as follows:

Figure 230—Receive PLCP

## 15.4 DSSS PMD sublayer

### 15.4.4 Basic service and options

#### 15.4.4.2 PMD_SAP peer-to-peer service primitive parameters

Insert a new row into Table 121 as follows:

Table 121—DSSS PMD_SAP peer-to-peer service primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | RXVECTOR | 0-255. RCPI is present only when dot11RadioMeasurementEnabled is true. |

#### 15.4.4.3 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 122 as follows:

Table 122—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI |  | X | -- | -- |

#### 15.4.4.4 PMD_SAP service primitive parameters

Insert new row into Table 123 as follows:

Table 123—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PMD-RCPI.indicate | 0-255 |

### 15.4.5 PMD_SAP detailed service specification

Insert the following clause (15.4.5.16) after 15.4.5.15:

#### 15.4.5.16 PMD_RCPI.indicate

##### 15.4.5.16.1 Function

This optional primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC.

##### 15.4.5.16.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_RCPI.indicate(RCPI).

The RCPI shall be a measure of the channel power received by the DSSS PHY. RCPI indications are supported as defined in 15.4.8.5.

##### 15.4.5.16.3 When generated

This primitive shall be generated by the PMD when the DSSS PHY is in the receive state. It shall be continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 15.4.5.16.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSNI to measure input signal quality.

### 15.4.8 PMD receiver specifications

Insert the following clause after 15.4.8.4:

#### 15.4.8.5 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI = int{(Power in dBm +110)*2} for 0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

RCPI shall equal the received RF power within an accuracy of +/-5 dB (95% confidence interval) within the specified dynamic range of the receiver. The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

# 17. Orthogonal frequency division multiplexing (OFDM) PHY specification for the 5 GHz band

## 17.2 OFDM PHY specific service parameter list

### 17.2.3 RXVECTOR parameters

Insert new row into Table 137 as follows:

Table 137—RXVECTOR parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PHY-RXSTART.indicate (RXVECTOR) PHY-RXEND.indicate (RXVECTOR) | 0 to 255. RCPI is present only when dot11RadioMeasurementEnabled is true. |

Insert the following new clause after 17.2.3.4:

#### 17.2.3.5 RXVECTOR RCPI

The allowed values for the RCPI parameter are in the range from 0 through 255, as defined in 17.3.10.6. This parameter is a measure by the PHY of the received channel power. RCPI indications of 8 bits are supported. RCPI shall be measured over the entire received frame or by other equivalent means which meet the specified accuracy.

## 17.3 OFDM PLCP sublayer

### 17.3.10 PMD receiver specifications

Insert the following new clause after 17.3.10.5:

#### 17.3.10.6 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI = int{(Power in dBm +110)*2} for  0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

### 17.3.12 Receive PLCP

Change the first paragraph of 17.3.12 as follows:

The receive PLCP is shown in Figure 259. In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management (via the PLME) the PHY is set to the appropriate frequency. Other receive parameters, such as RSSI, RCPI, and indicated DATARATE, may be accessed via the PHY-SAP.

Change Figure 259 to insert new parameter as follows:

Figure 259—Receive PLCP

## 17.5 OFDM PMD sublayer

### 17.5.4 Basic service and options

#### 17.5.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 152 as follows:

Table 152—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

#### 17.5.4.3 PMD_SAP service primitive parameters

Insert new row into Table 153 as follows:

Table 153—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 |

### 17.5.5 PMD_SAP detailed service specification

Insert the following clause (17.5.5.8) after 17.5.5.7:

#### 17.5.5.8 PMD_RCPI.indicate

##### 17.5.5.8.1 Function

This primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC entity.

##### 17.5.5.8.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_RCPI.indicate(RCPI).

The RCPI shall be a measure of the channel power received by the OFDM PHY. RCPI indications of 8 bits are supported, as defined in 17.3.10.6.

##### 17.5.5.8.3 When generated

This primitive shall be generated by the PMD when the OFDM PHY is in the receive state. It shall be continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 17.5.5.8.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSSI to measure input signal quality.

# 18. High Rate direct sequence spread spectrum (HR/DSSS) PHY specification

## 18.2 High Rate PLCP sublayer

### 18.2.6 Receive PLCP

Change the third paragraph of 18.2.6 as shown:

The receive PLCP is shown in Figure 270 In order to receive data, the PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management via the PLME, the PHY shall be set to the appropriate channel and the CCA method chosen. Other receive parameters, such as RSSI, RCPI, SQ, and indicated DATARATE, may be accessed via the PHY-SAP.

Change the fourth paragraph of 18.2.6 as shown:

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled (according to 18.4.8.4) as the RSSI reaches the ED_THRESHOLD, and/or PMD_CS shall be enabled after code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate, according to 18.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for ED and/or code lock prior to correct reception of the PLCP header. The PMD primitives, PMD_SQ, and PMD_RSSI, and PMD_RCPI, are issued to update the SQ, RSSI and SQRCPI parameters reported to the MAC.

Change the list following the sixth paragraph of 18.2.6 as shown:

- The SIGNAL field;

- The SERVICE field;

- The PSDU length in octets (calculated from the LENGTH field in microseconds and the DATARATE in Mbit/s, in accordance with the formula in 18.2.3.5);

- RXPREAMBLE_TYPE (which is an enumerated type taking on values SHORTPREAMBLE or LONGPREAMBLE);

- The antenna used for receive (RX_ANTENNA)ANT_STATE (the antenna used for receive), RSSI, RCPI, and SQ.

Change Figure 270 to insert new parameter as follows:

Figure 270—Receive PLCP

## 18.3 High Rate PLME

### 18.3.5 Vector descriptions

Insert new rows at the end of Table 159 as follows:

Table 159—Parameter vectors

| Parameter | Associated vector | Value |
| --- | --- | --- |
| ANT_STATE | RXVECTOR | 1 to 256 |
| RSSI | RXVECTOR | 0-8 bits of RSSI |
| RCPI | RXVECTOR | 0-255. RCPI is present only when dot11RadioMeasurementEnabled is true. |
| SQ | RXVECTOR | 0-8 bits of SQ |

## 18.4 High Rate PMD sublayer

### 18.4.4 Basic service and options

#### 18.4.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 161 as follows:

Table 161—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

### 18.4.5 PMD_SAP detailed service specification

Insert the following clause (18.4.5.16) at the end of 18.4.5:

#### 18.4.5.16 PMD_RCPI.indicate

##### 18.4.5.16.1 Function

This optional primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC.

##### 18.4.5.16.2 Semantics of the service primitive

The primitive shall provide the following parameter:

| Parameter | Associated primitive | Value | Description |
| --- | --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 | The RCPI is a measure of the received power by the High Rate PHY as defined in 18.4.8.5. |

##### 18.4.5.16.3 When generated

This primitive shall be generated by the PMD when the High Rate PHY is in the receive state when dot11RadioMeasurementEnabled is true. It is continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 18.4.5.16.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSNI to measure input signal quality.

### 18.4.8 PMD receiver specifications

Insert the following clause (18.4.8.5) at the end of 18.4.8:

#### 18.4.8.5 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI   =  int{(Power in dBm +110)*2} for  0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

# 19. Extended Rate PHY specification

## 19.2 PHY specific service parameter list

Insert new row into Table 173 as follows:

Table 173—RXVECTOR parameters

| Parameter | Value |
| --- | --- |
| RCPI | The RCPI is a measure of the received channel power and is included when dot11RadioMeasurementEnabled is true. The 8-bit RCPI value is described in 17.2.3.5 and 18.4.5.16. |

## 19.9 Extended Rate PMD sublayer

### 19.9.4 Basic service and options

#### 19.9.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 180 as follows:

Table 180—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

#### 19.9.4.3 PMD_SAP service primitive parameters

Insert new row into Table 181 as follows:

Table 181—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value | Description |
| --- | --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 | The RCPI is a measure of the received channel power. See 19.9.5.14. |

### 19.9.5 PMD_SAP detailed service specification

Insert the following clause (19.9.5.14) after 19.9.5.13:

#### 19.9.5.14 PMD_RCPI.indicate

This primitive is the same as that defined in 17.5.5.8 and 18.4.5.16, including the parameter RCPI. This primitive is used for radio measurement purposes and to aid in link optimization algorithms such as roaming decisions.

# Annex A

## A.4 PICS proforma–IEEE Std 802.11, 2006 Edition34

### A.4.3 IUT configuration

Append this entry to the end of this table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF13 | Is Radio Resource Measurement supported? | 7.3.1.4 | O | Yes, No |

Insert this new clause after  A.4.12:

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Radio Resource Measurement capabilities supported? |  |  |  |
| RRM1 | Radio Measurement Capability | 7.3.1.4 | CF13:M | Yes, No, N/A |
| RRM2 | Action frame protocol for measurements | 7.4 | CF13:M | Yes, No, N/A |
| RRM2.1 | Radio Measurement Request frame | 7.4.5.1 | CF13:M | Yes, No, N/A |
| RRM2.2 | Radio Measurement Report frame | 7.4.5.2 | CF13:M | Yes, No, N/A |
| RRM2.3 | Link Measurement Request frame | 7.4.5.3 | CF13:M | Yes, No, N/A |
| RRM2.4 | Link Measurement Report frame | 7.4.5.4 | CF13:M | Yes, No, N/A |
| RRM2.5 | Neighbor Report Request |  |  |  |
| RRM2.5.1 | Generate and transmit Neighbor Report Request | 7.4.5.5 | (CF13 AND CF2):M | Yes, No, N/A |
| RRM2.5.2 | Receive and process Neighbor Report Request | 7.4.5.5 | (CF13 AND CF1):M | Yes, No, N/A |
| RRM2.6 | Neighbor Report Response |  |  |  |
| RRM2.6.1 | Generate and transmit Neighbor Report Response | 7.4.5.6, 7.3.2.36 | (CF13 AND CF1):M | Yes, No, N/A |
| RRM2.6.2 | Receive and process Neighbor Report Response | 7.4.5.6, 7.3.2.36 | (CF13 AND CF2):M | Yes, No, N/A |
| RRM3 | General protocol for requesting and reporting of measurements | 7.3.2.21, 7.3.2.22, 11.11, 11.11.5 | CF13:M | Yes, No, N/A |
| RRM3.1 | Parallel Measurements | 7.3.2.21, 11.11.5, 7.3.2.22 | CF13:M | Yes, No, N/A |
| RRM3.2 | Use of Enable, Request & Report bits to enable/disable measurement requests and triggered autonomous reports Measurement Requests | 7.3.2.21, 11.11.7 | CF13:M | Yes, No, N/A |
| RRM3.3 | Enable Autonomous Report | 7.3.2.21, 11.11.7 | CF13:M | Yes, No, N/A |
| RRM3.4 | Duration Mandatory | 7.3.2.21, 11.11.3 | CF13:M | Yes, No, N/A |
| RRM3.5 | Incapable Indication | 7.3.2.22 | CF13:M | Yes, No, N/A |
| RRM3.6 | Refused Indication | 7.3.2.22 | CF13:M | Yes, No, N/A |
| RRM3.7 | Repeated Measurement | 7.4.5.1, 11.11.6 | CF13:M | Yes, No, N/A |
| RRM3.8 | Measurement Pause | 7.3.2.21.11, 11.11.8.7 | CF13:M | Yes, No, N/A |
| RRM4 | Beacon Measurement Type | 11.11, 11.11.8.1 | CF13:M | Yes, No, N/A |
| RRM4.1 | Beacon Request | 7.3.2.21.6 | CF13:M | Yes, No, N/A |
| RRM4.2 | Passive Measurement mode | 7.3.2.21.6 11.11.8.1 | CF13:M | Yes, No, N/A |
| RRM4.3 | Active Measurement mode | 7.3.2.21.6 11.11.8.1 | CF13:M | Yes, No, N/A |
| RRM4.4 | Beacon table mode | 7.3.2.21.6 11.11.8.1 | CF13:M | Yes, No, N/A |
| RRM4.5 | Reporting Conditions | 7.3.2.21.6 | CF13:O | Yes, No, N/A |
| RRM4.6 | Beacon Report | 7.3.2.22.6 | CF13:M | Yes, No, N/A |
| * RRM5 | Frame Measurement Type | 11.11, 11.11.8.2 | CF13:O | Yes, No, N/A |
| RRM5.1 | Frame Request | 7.3.2.21.7 | (CF13 AND RRM5):M | Yes, No, N/A |
| RRM5.2 | Frame Report | 7.3.2.22.7 | (CF13 AND RRM5):M | Yes, No, N/A |
| RRM6 | Channel Load Measurement Type | 11.11, 11.11.8.3 | CF13:M | Yes, No, N/A |
| RRM6.1 | Channel Load Request | 7.3.2.21.4 | CF13:M | Yes, No, N/A |
| RRM6.2 | Channel Load Report | 7.3.2.22.4 | CF13:M | Yes, No, N/A |
| * RRM7 | Noise Histogram Measurement Type | 11.11, 11.11.8.4 | CF13:M | Yes, No, N/A |
| RRM7.1 | Noise Histogram Request | 7.3.2.21.5 | CF13:M | Yes, No, N/A |
| RRM7.2 | Noise Histogram Report | 7.3.2.22.5 | CF13:M | Yes, No, N/A |
| RRM8 | STA Statistics Measurement Type | 11.11, 11.11.8.5 | CF13:M | Yes, No, N/A |
| RRM8.1 | STA Statistics Request | 7.3.2.21.8 | CF13:M | Yes, No, N/A |
| RRM8.2 | STA Statistics Report | 7.3.2.22.8 | CF13:M | Yes, No, N/A |
| RRM9 | LCI Measurement Type | 11.11, 11.11.8.6 | CF13:M | Yes, No, N/A |
| RRM9.1 | LCI Request | 7.3.2.21.9 | CF13:M | Yes, No, N/A |
| RRM9.1.1 | Location Subject | 7.3.2.21.9 | CF13:M | Yes, No, N/A |
| RRM9.1.2 | Latitude Requested Resolution | 7.3.2.21.9 | CF13:M | Yes, No, N/A |
| RRM9.1.3 | Longitude Requested Resolution | 7.3.2.21.9 | CF13:M | Yes, No, N/A |
| RRM9.1.4 | Altitude Requested Resolution | 7.3.2.21.9 | CF13:M | Yes, No, N/A |
| RRM9.2 | LCI Report | 7.3.2.22.9 | CF13:M | Yes, No, N/A |
| RRM9.3 | Azimuth | 11.11, 11.11.8.6 | CF13:O | Yes, No, N/A |
| RRM9.3.1 | Azimuth Request | 7.3.2.21.9 | CF13:O | Yes, No, N/A |
| RRM9.3.2 | Azimuth Response | 7.3.2.22.9 | CF13:O | Yes, No, N/A |
| *RRM10 | QoS Metrics Type | 11.11, 11.11.8.8 | (CF13 AND CF12):O | Yes, No, N/A |
| RRM10.1 | QoS Metrics Type Request | 7.3.2.21.10 | RRM10:M | Yes, No, N/A |
| RRM10.2 | QoS Metrics Type Report | 7.3.2.22.10 | RRM10:M | Yes, No, N/A |
| RRM10.3 | Triggered QoS Metrics Report | 7.3.2. 22.10, 11.11.8.8 | RRM10:O | Yes, No, N/A |
| RRM11 | AP Channel Report | 7.3.2.9, 7.3.2.35 | (CF13 AND CF1):M | Yes, No, N/A |
| RRM11.1 | Generate and transmit AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.35 | (CF13 AND CF1):M | Yes, No, N/A |
| RRM11.2 | Receive and process AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.35 | (CF13 AND CF2):M | Yes, No, N/A |
| RRM12 | Neighbor ReportProcedure | 11.12.2, 11.12.3 | CF13:M | Yes, No, N/A |
| RRM12.1 | Neighbor Report Procedure | 11.12.2, 11.12.3 | CF13:M | Yes, No, N/A |
| RRM12.2 | TBTT Offset in Neighbor Report | 7.4.5.5, 11.12.3 | CF13:O | Yes, No, N/A |
| RRM12.3 | Measurement Pilot Transmission Information in Neighbor Report | 7.3.2.36, 11.12.2 | CF13:M | Yes, No, N/A |
| RRM13 | RCPI Measurement |  |  |  |
|  | RCPI Measurement for DSSS PHY at 2.4GHz | 15.4.8.5 | (CF13 AND CF4):M | Yes, No, N/A |
|  | RCPI Measurement for OFDM PHY at 5GHz | 17.2.3.5, 17.3.10.6, 17.5.4.3 | (CF13 AND CF6):M | Yes, No, N/A |
|  | RCPI Measurement for HR DSSS PHY at 2.4GHz | 18.4.8.5 | (CF13 AND CF7):M | Yes, No, N/A |
|  | RCPI Measurement for Extended Rate PHY at 2.4Ghz | 19.9.5.14 | (CF13 AND CF9):M | Yes, No, N/A |
| RRM14 | RCPI Measurement during Active Scanning |  |  |  |
| RRM14.1 | Respond with RCPI information element when requested | 11.1.3.2.2 | (CF13 AND CF12 AND CF1):M | Yes, No, N/A |
| RRM14.2 | Measurement of RCPI on Probe Request frames | 11.1.3.2.2 | (CF13 AND CF12 AND CF1):O | Yes, No, N/A |
| RRM15 | RSNI Measurement | 7.3.2.40 | CF13:M | Yes, No, N/A |
| RRM16 | TPC Information in Beacon and Probe Response frames |  |  |  |
| RRM16.1 | Country and Transmit Power Control (TPC) Report elements included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.9, 7.3.2.18, 11.13 | CF13:M | Yes, No, N/A |
| RRM16.2 | Power Constraint element included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.15 | CF13:O | Yes, No, N/A |
| RRM17 | Power Capability elements in Association and Reassociation frames | 7.2.3.4, 7.2.3.5, 11.10.1 | CF13:M | Yes, No, N/A |
| RRM18 | Management Information Base |  |  |  |
| RRM18.1 | dot11SMTRRMRequest | Annex D | (CF13 AND CF1):M | Yes, No, N/A |
| RRM18.2 | dot11SMTRRMReport | Annex D | (CF13 AND CF1):M | Yes, No, N/A |
| RRM18.3 | dot11SMTRRMConfig | Annex D | (CF13 AND CF1):M | Yes, No, N/A |
| RRM19 | Measurement Pilot Frame | 7.1.3.1.2, 7.2.3.9A, 7.3.1.18, 7.3.1.19, 7.3.1.20, 7.3.1.21, 7.3.1.22, 7.3.1.23, 10.3.29, 11.9, 11.14 | CF13:O | Yes, No, N/A |
| RRM20 | BSS/QBSS Load elements included in Beacon and Probe Response frames |  |  |  |
| RRM20.1 | BSS Load elements | 7.2.3.1, 7.2.3.9, 7.3.2.38 | (CF1 AND NOT CF12 AND CF13):M | Yes, No, N/A |
| RRM20.2 | QBSS Load elements | 7.2.3.1, 7.2.3.9, 7.3.2.27 | (CF1 AND CF12 AND CF13):M | Yes, No, N/A |
| RRM21 | Antenna Information elements included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.39 | CF13:M | Yes, No, N/A |
| RRM22 | Measurement Pilot Transmission Information element included in Probe Response frame | 7.2.3.9, 7.3.2.41 | CF13:O | Yes, No, N/A |
| RRM23 | Quiet interval |  |  |  |
| RRM23.1 | AP-defined Quiet Interval | 7.2.3.1, 7.2.3.9, 7.3.2.21, 11.6.2 | (CF1 AND CF13):M | Yes, No, N/A |
| RRM23.2 | STA-defined Quiet Interval | 7.2.3.1, 7.2.3.9, 7.3.2.21, 11.6.2 | (CF2 AND CF13):M | Yes, No, N/A |
| RRM23.3 | STA support for Quiet Interval | 7.2.3.1, 7.2.3.9, 7.3.2.21, 11.6.2 | CF13:M | Yes, No, N/A |
| *RRM24 | QBSS Available Admission Capacity | 7.3.2.42 | (CF1 AND CF12 AND CF13):M | Yes, No, N/A |
| RRM24.1 | Generate and transmit AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.41 | RRM24:M | Yes, No, N/A |
| RRM24.2 | Receive and process AP Channel Report | 7.2.3.1, 7.2.3.9, 7.3.2.41 | RRM24:M | Yes, No, N/A |

# Annex D

In the IMPORT Section of Annex D, insert the following import statement to the end behind the ifIndex statement:

InterfaceIndex FROM IF-MIB;

Change the “Station Management (SMT) Attributes” of “Major Sections” of the MIB as follows:

--**********************************************************************

--* Major sections

--**********************************************************************

--

-- Station Management (SMT) Attributes

-- DEFINED AS “The SMT object class provides the necessary support

-- at the station to manage the processes in the station such that

-- the station may work cooperatively as part of an IEEE 802.11

-- network.

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

-- dot11smt GROUPS

-- dot11StationConfigTable 			::= { dot11smt 1 }

-- dot11AuthenticationAlgorithmTable 		::= { dot11smt 2 }

-- dot11WEPDefaultKeysTable 			::= { dot11smt 3 }

-- dot11WEPKEYMappingsTable 			::= { dot11smt 4 }

-- dot11PrivacyTable 			::= { dot11smt 5 }

-- dot11SMTnotification 			::= { dot11smt 6 }

-- dot11MultiDomainCapabilityTable 		::= { dot11smt 7 }

-- dot11SpectrummanagementTable 		::= { dot11smt 8 }

-- dot11RSNAConfigTable 			::= { dot11smt 9 }

-- dot11RSNAConfigPairwiseCiphersTable 		::= { dot11smt 10 }

-- dot11RSNAConfigAuthenticationSuitesTable 	::= { dot11smt 11 }

-- dot11RSNAStatsTable 			::= { dot11smt 12 }

-- dot11RegulatoryClassesTable 		::= { dot11smt 13 }

-- dot11RadioResourceManagement 		::= { dot11smt 14 }

Change the “MAC Attributes” of “Major Sections” of the MIB as follows:

-- MAC Attributes

-- DEFINED AS "The MAC object class provides the necessary support

-- for the access control, generation, and verification of frame

-- check sequences (FCSs), and proper delivery of valid data to

-- upper layers."

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

-- MAC GROUPS

-- reference IEEE Std 802.1f-1993

-- dot11OperationTable 			::= { dot11mac 1 }

-- dot11CountersTable 			::= { dot11mac 2 }

-- dot11GroupAddressesTable 		::= { dot11mac 3 }

-- dot11EDCATable 			::= { dot11mac 4 }

-- dot11QAPEDCATable 			::= { dot11mac 5 }

-- dot11QosCountersTable 			::= { dot11mac 6 }

-- dot11PeerStatsTable 			::= { dot11mac 7 }

Change the “Textual conventions” of “Major Sections” of the MIB as follows:

-- **********************************************************************

-- * Textual conventions from 802 definitions

-- **********************************************************************

WEPKeytype ::= OCTET STRING (SIZE (5))

TSFType ::= OCTET STRING (SIZE (8))

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID 			MacAddress,

dot11MediumOccupancyLimit 		INTEGER,

dot11CFPollable 			TruthValue,

dot11CFPPeriod 			INTEGER,

dot11CFPMaxDuration 			INTEGER,

dot11AuthenticationResponseTimeOut 	Unsigned32,

dot11PrivacyOptionImplemented 		TruthValue,

dot11PowerManagementMode 		INTEGER,

dot11DesiredSSID 			OCTET STRING,

dot11DesiredBSSType 			INTEGER,

dot11OperationalRateSet 		OCTET STRING,

dot11BeaconPeriod 			INTEGER,

dot11DTIMPeriod 			INTEGER,

dot11AssociationResponseTimeOut 		Unsigned32,

dot11DisassociateReason 		INTEGER,

dot11DisassociateStation 		MacAddress,

dot11DeauthenticateReason 		INTEGER,

dot11DeauthenticateStation 		MacAddress,

dot11AuthenticateFailStatus 		INTEGER,

dot11AuthenticateFailStation 		MacAddress,

dot11SpectrumManagementImplemented 	TruthValue,

dot11SpectrumManagementRequired 		TruthValue,

dot11MultiDomainCapabilityImplemented 	TruthValue,

dot11MultiDomainCapabilityEnabled 	TruthValue,

dot11CountryString 			OCTET STRING,

dot11RSNAOptionImplemented 		TruthValue,

dot11RSNAPreauthenticationImplemented 	TruthValue

dot11QosOptionImplemented 		TruthValue,

dot11ImmediateBlockAckOptionImplemented 	TruthValue,

dot11DelayedBlockAckOptionImplemented 	TruthValue,

dot11DirectOptionImplemented 		TruthValue,

dot11APSDOptionImplemented 		TruthValue,

dot11QAckOptionImplemented 		TruthValue,

dot11QBSSLoadOptionImplemented 		TruthValue,

dot11QueueRequestOptionImplemented 	TruthValue,

dot11TXOPRequestOptionImplemented 	TruthValue,

dot11MoreDataAckOptionImplemented 	TruthValue,

dot11AssociateinNQBSS 			TruthValue,

dot11DLSAllowedInQBSS 			TruthValue,

dot11DLSAllowed			TRuthValue,

dot11AssociateStation 			MacAddress,

dot11AssociateID 			INTEGER,

dot11AssociateFailStation 		MacAddress,

dot11AssociateFailStatus I		INTEGER,

dot11ReassociateStation 		MacAddress,

dot11ReassociateID 			INTEGER,

dot11ReassociateFailStation 		MacAddress,

dot11ReassociateFailStatus 		INTEGER,

dot11RadioMeasurementCapable 		TruthValue,

dot11RadioMeasurementEnabled 		TruthValue,

dot11RadioMeasurementProbeDelay 		INTEGER,

dot11MeasurementPilotEnabled 		TruthValue,

dot11MeasurementPilotPeriod 		INTEGER,

dot11MeasurementPilotTransmitPriority 	INTEGER }

Update the dot11BeaconPeriod element as shown:

dot11BeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that a station shall use for scheduling Beacon transmissions. This value is transmitted in Beacon, Probe Response, and Measurement Pilot frames."

::= { dot11StationConfigEntry 12 }

Insert the following elements to the end of dot11StationConfigTable element definitions after dot11DLSAllowed:

dot11AssociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

association response frame. If no association response

frame has been transmitted, the value of this attribute

shall be 0."

::= { dot11StationConfigEntry 43 }

dot11AssociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the Association ID from the

most recently transmitted association response frame.

If no association response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 44 }

dot11AssociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed association response frame. If no failed

association response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 45 }

dot11AssociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted Status

Code in a failed association response frame. If no failed

association response frame has been transmitted, the

value of this attribute shall be 0."

::= { dot11StationConfigEntry 46 }

dot11ReassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

reassociation response frame. If no reassociation response

frame has been transmitted, the value of this attribute

shall be 0."

::= { dot11StationConfigEntry 47 }

dot11ReassociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the Association ID from the

most recently transmitted reassociation response frame.

If no reassociation response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 48 }

dot11ReassociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed reassociation response frame. If no failed

reassociation response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 49 }

dot11ReassociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted

Status Code in a failed reassociation response frame.

If no failed reassociation response frame has been

transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 50 }

dot11RadioMeasurementCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Radio

Measurement. Otherwise it is not capable of performing

Radio Measurement.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 51 }

dot11RadioMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA may use the defined Radio Measurement procedures if

this attribute is TRUE. The default value of this attribute

is FALSE."

::= { dot11StationConfigEntry 52 }

dot11RadioMeasurementProbeDelay OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The value of ProbeDelay to be used when making a beacon

type measurement with measurement mode active."

::= { dot11StationConfigEntry 53 }

dot11MeasurementPilotEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA shall transmit Measurement Pilot frames if this attribute is TRUE.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 54 }

dot11MeasurementPilotPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that a station shall use for

scheduling Measurement Pilot transmissions. This value is transmitted in

Measurement Pilot frames."

::= { dot11StationConfigEntry 55 }

dot11MeasurementPilotTransmitPriority OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the Access Category (AC), at which Measurement Pilot

frames are transmitted."

::= { dot11StationConfigEntry 56 }

In SMT Notification Objects Section of Annex D, insert the following notifications following dot11AuthenticateFail:

dot11Associate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateStation, dot11AssociateID}

STATUS current

DESCRIPTION

"The associate notification shall be sent when the STA sends

an Association Response frame with a status code equal to ‘successful.’

The value of the notification shall include the MAC address of the

MAC to which the Association Response frame was sent and the

Association ID.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 4 }

dot11AssociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateFailStatus,

dot11AssociateFailStation }

STATUS current

DESCRIPTION

"The associate failed notification shall be sent when the

STA sends an Association Response frame with a status code

other than ‘successful.’ The value of the notification shall

include the MAC address of the MAC to which the Association

Response frame was sent and the reason for the association failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 5 }

dot11Reassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateStation, dot11ReassociateID}

STATUS current

DESCRIPTION

"The reassociate notification shall be sent

when the STA sends an Reassociation Response frame with a

status code equal to ‘successful.’ The value of

the notification shall include the MAC address of the

MAC to which the Reassociation Response frame was sent and the

Reassociation ID.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 6 }

dot11ReassociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateFailStatus,

dot11ReassociateStation }

STATUS current

DESCRIPTION

"The reassociate failed notification shall be sent

when the STA sends an Reassociation Response frame with a

status code other than ‘successful.’ The value of

the notification shall include the MAC address of the

MAC to which the Reassociation Response frame was sent and

the reason for the reassociation failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 7 }

In dot11CountersTable change Dot11CountersEntry as follows:

Dot11CountersEntry ::=

SEQUENCE {	dot11TransmittedFragmentCount	Counter32,

dot11MulticastTransmittedFrameCount	Counter32,

dot11FailedCount	Counter32,

dot11RetryCount	Counter32,

dot11MultipleRetryCount	Counter32,

dot11FrameDuplicateCount	Counter32,

dot11RTSSuccessCount	Counter32,

dot11RTSFailureCount	Counter32,

dot11ACKFailureCount	Counter32,

dot11ReceivedFragmentCount	Counter32,

dot11MulticastReceivedFrameCount	Counter32,

dot11FCSErrorCount	Counter32,

dot11TransmittedFrameCount	Counter32,

dot11WEPUndecryptableCount	Counter32,

dot11QoSDiscardedFragmentCount	Counter32,

dot11AssociatedStationCount	Counter32,

dot11QoSCFPollsReceivedCount	Counter32,

dot11QoSCFPollsUnusedCount	Counter32,

dot11QoSCFPollsUnusableCount	Counter32,

dot11QoSCFPollsLostCount	Counter32 }

Insert the following attribute definition to the end of the dot11CountersEntry attributes:

dot11QoSCFPollsLostCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment for each QoS (+)CF-Poll that has been issued by the HC where there was no response from the QSTA indicated in the RA field.”

::= { dot11CountersEntry 20 }

In SMT MIB of Annex D, insert the following Radio Resource Measurement MIB at the after the Robust Security Network Association (RSNA and TSN) MIB

-- ********************************************************************

-- * Radio Resource Measurement

-- ********************************************************************

dot11RadioResourceMeasurement OBJECT IDENTIFIER ::= { dot11smt 14 }

-- **********************************************************************

-- *  dot11RRMRequest and dot11RRMReport Usage

-- *

-- *  The dot11RRMRequest and dot11RRMReport portions of the SMT MIB

-- *  provide access to the Radio Measurement service.  By performing

-- *  SET operations on the various dot11RRMRequest MIB objects,

-- *  radio measurements may be initiated directly on the local STA or

-- *  on any peer station within the same BSS.  Subsequently, by

-- *  performing GET operations on the various dot11RRMReport MIB

-- *  objects the results of the requested measurements may be

-- *  retrieved.

-- *

-- *  In the diagram below, a radio measurement could be initiated

-- *  for STA x by performing a MIB.set operation on the SMT MIB of

-- *  STA x and specifying the MAC address of STA x in

-- *  dot11RRMRqstTargetAdd.  Additionally, it is possible to have STA x

-- *  request a measurement from STA y by performing a MIB.set operation

-- *  on the SMT MIB of STA x and specifying the MAC address of STA y in

-- *  dot11RRMRqstTargetAdd.  In both cases the result of the measurements

-- *  can be retrieved by performing a MIB.get operation on the SMT MIB

-- *  of STA x upon completion of the measurement.

-- *

-- *          MIB.Set                                        MIB.Set

-- *            or                                              or

-- *          MIB.Get                                        MIB.Get

-- *    +--------|---------+                           +--------|---------+

-- *    | SME    |         |                           | SME    |         |

-- *    |       \ /        |                           |       \ /        |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |   | SMT MIB |    |                           |   | SMT MIB |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |                  |                           |                  |

-- *    |   / \            |                           |   / \            |

-- *    |    |   MREQUEST  |                           |    |   MREQUEST  |

-- *    +----+-------------+                           +----+-------------+

-- *    |    |   MREPORT   |                           |    |   MREPORT   |

-- *    |   \ /  MEASURE   |     Action Frames         |   \ /  MEASURE   |

-- *    |                  | <--Measurement Request--> |                  |

-- *    |                  | <--Measurement Report---> |                  |

-- *    | MLME             |                           | MLME             |

-- *    +------------------+                           +------------------+

-- *          STA x                                           STA y

-- *

-- *

-- *  Each STA maintains a single dot11RRMRequestTable in the SMT MIB

-- *  used to initiate RM Measurement Requests.  Each dot11RRMRequestEntry

-- *  in the table represents an individual Measurement Request that

-- *  makes up a complete Measurement Request Action frame.

-- *  Multiple Measurement Requests may be concatenated into a single

-- *  Measurement Request Action frame by setting the same

-- *  dot11RRMRqstToken value into multiple dot11RRMRequestEntrys.

-- *

-- *  Each row, dot11RRMRequestEntry, of the dot11RRMRequestTable

-- *  provides read-create access for the initiation of a measurement

-- *  request.  The dot11RRMRequestNextIndex object can be used to

-- *  determine which is the next row available.  Each row corresponding to

-- *  one measurement in the sequence is created with a dot11RRMRqstRowStatus

-- *  set to notInService.  Once the dot11RRMRequestEntry(s) have been

-- *  created for a desired measurement sequence the corresponding

-- *  dot11RRMRqstRowStatus(s) objects are set to active to indicate that

-- *  the SME can trigger the appropriate MLME primitives.  Upon processing

-- *  the request, the SME returns the corresponding dot11RRMRqstRowStatus(s)

-- *  object to notInService and are now available for additional

-- *  measurement requests.

-- *

-- *  After a radio measurement is complete the SME populates the RRMReport

-- *  objects with the results of the measurement.  Each STA maintains a set

-- *  of RRMReport tables, one for each corresponding measurement type.  The

-- *  results of the entire measurement sequence are spread across the tables

-- *  based on what types of measurements were requested.  Each xxxReportEntry

-- *  within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- *  to the original dot11RRMRqstToken in the measurement request.  So the

-- *  results of the measurement can be collected by searching the appropriate

-- *  xxxReportTables and retrieve any reports with the matching request

-- *  token.

-- **********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Requests

-- ********************************************************************

dot11RRMRequest OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 1 }

-- ********************************************************************

-- * dot11RRMRequest TABLE

-- ********************************************************************

dot11RRMRequestNextIndex OBJECT-TYPE

SYNTAX Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies a hint for the next value of dot11RRMRqstIndex to be used in a

row creation attempt for dot11RRMRequestTable. If no new rows can be created

for some reason, such as memory, processing requirements, etc, the SME shall

set this attribute to 0. It shall update this attribute to a proper value

other than 0 as soon as it is capable of receiving new measurement requests.

The nextIndex is not necessarily sequential nor monotonically increasing."

::= { dot11RRMRequest 1 }

dot11RRMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This group contains the current list of requests for RRM

reports to be issued and have been issued until removed. A network manager

adds a RRM request by creating a row with createAndWait row status and then

filling in the request parameters/attributes.

The request becomes active to be issued when the row status

is set to Active. The columnar objects or attributes other than the

rowStatus shall not be written if the rowStatus is Active. The request rows

can be deleted, if commanded by a network manager via changing the value of

dot11RRMRqstRowStatus to Destroy. This may leave orphaned rows if a manager

crashes and forgets which rows are being used by it. One recommended way to

manage orphaned or finished rows is to delete rows if their

dot11RRMRqstRowStatus remains other than Active for longer than a

period (recommend at least 5 minutes, RFC 2579). Or another recommended

way is to delete older rows as needed based on their dot11RRMRqstTimeStamp

values. This can be done by the agent as well as the manager. "

::= { dot11RRMRequest 2 }

dot11RRMRequestEntry OBJECT-TYPE

SYNTAX Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMRequestTable Indexed by dot11RRMRqstIndex."

INDEX { dot11RRMRqstIndex }

::= { dot11RRMRequestTable 1 }

Dot11RRMRequestEntry ::=

SEQUENCE {

dot11RRMRqstIndex Unsigned32,

dot11RRMRqstRowStatus RowStatus,

dot11RRMRqstToken OCTET STRING,

dot11RRMRqstRepetitions INTEGER,

dot11RRMRqstIfIndex InterfaceIndex,

dot11RRMRqstType INTEGER,

dot11RRMRqstTargetAdd MacAddress,

dot11RRMRqstTimeStamp TimeTicks,

dot11RRMRqstChanNumber INTEGER,

dot11RRMRqstRegulatoryClass INTEGER,

dot11RRMRqstRndInterval Unsigned32,

dot11RRMRqstDuration Unsigned32,

dot11RRMRqstParallel TruthValue,

dot11RRMRqstEnable TruthValue,

dot11RRMRqstRequest TruthValue,

dot11RRMRqstReport TruthValue,

dot11RRMRqstDurationMandatory TruthValue,

dot11RRMRqstBeaconRqstMode INTEGER,

dot11RRMRqstBssid MacAddress,

dot11RRMRqstSSID OCTET STRING,

dot11RRMRqstReportingCondition INTEGER,

dot11RRMRqstThresholdOffset INTEGER,

dot11RRMRqstSTAStatRqstGroupID INTEGER,

dot11RRMRqstLCIRqstOctet INTEGER,

dot11RRMRqstLCILatitudeResolution INTEGER,

dot11RRMRqstLCILongitudeResolution INTEGER,

dot11RRMRqstLCIAltitudeResolution INTEGER,

dot11RRMRqstPauseTime INTEGER,

dot11RRMRqstQoSMetricsPeerQSTAAddress MacAddress,

dot11RRMRqstQoSMetricsTrafficIdentifier INTEGER,

dot11RRMRqstQoSMetricsBin0Range INTEGER,

dot11RRMRqstTrigdQoSAverageCondition TruthValue,

dot11RRMRqstTrigdQoSConsecutiveCondition TruthValue,

dot11RRMRqstTrigdQoSDelayCondition TruthValue,

dot11RRMRqstTrigdQoSAverageThreshold INTEGER,

dot11RRMRqstTrigdQoSConsecutiveThreshold INTEGER,

dot11RRMRqstTrigdQoSDelayThresholdRange INTEGER,

dot11RRMRqstTrigdQoSDelayThreshold INTEGER,

dot11RRMRqstTrigdQoSMeasurementCount INTEGER,

dot11RRMRqstTrigdQoSTimeout INTEGER }

dot11RRMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for RRM Request elements in dot11RRMRequestTable, greater than 0."

::= { dot11RRMRequestEntry 1 }

dot11RRMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Row Status column of the current row, used for

tracking status of an individual request. When this attribute is set to

Active, AND a measurement request can be unambiguously created based on the

parameters in the row, then the

MLME may proceed to issue the request to its intended targets when

appropriate. If not, this attribute may be set to Not-ready immediately to

indicate parametric errors. However, it is the network managers

responsibility to correct the error. If the request is successfully issued

to the target STA, then the rowStatus is set to notInService."

REFERENCE

"IEEE 802.11, Clause 7.3.2.21"

::= { dot11RRMRequestEntry 2 }

dot11RRMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A unique string to identify a group of rows to be issued as parallel or backto back measurements. To guarantee the uniqueness of this token acrossmultiple network managers, it is recommended that this token be prefixed with the IP address of the network manager creating this row. This token isnot necessarily equivalent to the measurement tokens in RRM request frames.If this attribute is an empty string, then this row of request is independentfrom other requests."

DEFVAL { "" }

::= { dot11RRMRequestEntry 3 }

dot11RRMRqstRepetitions OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Number of Repetitions field contains the requested number of repetitions for all the measurement request elements in this frame.  A value of zero in the Number of Repetitions field indicates measurement request elements are executed once without repetition."

::= { dot11RRMRequestEntry 4 }

dot11RRMRqstIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of RRM Request to be issued on."

::= { dot11RRMRequestEntry 5 }

dot11RRMRqstType OBJECT-TYPE

SYNTAX INTEGER {

channelLoad(3),

noiseHistogram(4),

beacon(5),

frame(6),

staStatistics(7),

lci(8),

qosMetrics(9),

pause(255)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The measurement type of this RRM request row."

::= { dot11RRMRequestEntry 6 }

dot11RRMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of STA for this row of RRM Request is to be issued to. If

this attribute matches the MAC address of the dot11RRMRqstIfIndex, then

measurement request is for this STA itself to carry out."

::= { dot11RRMRequestEntry 7 }

dot11RRMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The SysUpTime Value the last time when the dot11RRMRqstRowStatus is set to

active or when this row is created the first time. This attribute shall be

set by this STA or AP automatically, not by an SNMP manager."

::= { dot11RRMRequestEntry 8 }

dot11RRMRqstChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The channel number on which this request instructs the target STA to perform measurements and report. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request, QoS Metrics, or Measurement Pause. However, even in that case, the manager should set this attribute to the current channel for this interface, so that the row can be set to active when ready with all attributes specified."

::= { dot11RRMRequestEntry 9 }

dot11RRMRqstRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which the receiving STA shall conduct its measurement. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request, QoS Metrics or Measurement Pause. However, even in that case, this value should set to the current Regulatory Class for this interface, so that the row can be set to active when ready with all attributes specified."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11RRMRequestEntry 10 }

dot11RRMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute sets the Randomization Interval in the measurement Request Frame for this RRM Request. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request, QoS Metrics or Measurement Pause."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 11 }

dot11RRMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute sets the Measurement Duration in the measurement Request

Element for this Measurement Request. This attribute is ignored if

dot11RRMRqstType = LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 12 }

dot11RRMRqstParallel OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the parallel bit of the request frame that

contains this row of request. Default is false which means the measurement

shall be preformed in sequence. If this value is true, then the

measurement should start at the same time as the measurement described by

the next Measurement Request element in the same Measurement Request frame."

DEFVAL { false }

::= { dot11RRMRequestEntry 13 }

dot11RRMRqstEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the enable bit of the request frame that

contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 14 }

dot11RRMRqstRequest OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the request bit of the request element that

contains this row of request. This attribute indicates whether this STA

shall accept measurement requests from the target STA. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 15 }

dot11RRMRqstReport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the report bit of the request element that

contains this row of request. This bit controls whether the target STA shall

enable autonomous measurement report, and does not affect directed requests.

Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 16 }

dot11RRMRqstDurationMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the duration mandatory bit of the request frame that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 17 }

dot11RRMRqstBeaconRqstMode OBJECT-TYPE

SYNTAX INTEGER {

passive(0),

active(1),

beaconTable(2)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"dot11RRMRqstBeaconRqstMode corresponds to the Measurement Mode for

Beacon Request element. This attribute is only valid if the

dot11RRMRqstType is 5, indicating a beacon report. Otherwise this

attribute is ignored."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 18 }

dot11RRMRqstBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" BSSID indicates the BSSID of the particular AP for which this measurement

is requested. The BSSID shall be set to the broadcast BSSID when the

measurement is to be performed on any AP(s) on the specified channel."

DEFVAL { 'FFFFFFFF'H }

::= { dot11RRMRequestEntry 19 }

dot11RRMRqstSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the SSID for the measurement. Zero length MIB element for SSID indicates the wildcard SSID. The SSID shall be set to the wildcard SSID when the measurement is to be performed on all ESSs/IBSSs on the specified channel."

DEFVAL { ''H }

::= { dot11RRMRequestEntry 20 }

dot11RRMRqstReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

rcpiAboveAbsoluteThreshold(1),

rcpiBelowAbsoluteThreshold(2),

rssiAboveAbsoluteThreshold(3),

rssiBelowAbsoluteThreshold(4),

rcpiAboveOffsetThreshold(5),

rcpiBelowOffsetThreshold(6),

rssiAboveOffsetThreshold(7),

rssiBelowOffsetThreshold(8),

rcpiInBound(9),

rssiInBound(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Reporting Condition defines when the measured results are to be

reported to the requesting STA."

REFERENCE

"IEEE 802.11, Table 29B—Reporting Condition definitions for Beacon Request element"

DEFVAL {0}

::= { dot11RRMRequestEntry 21 }

dot11RRMRqstThresholdOffset OBJECT-TYPE

SYNTAX INTEGER (-127..127)

UNITS "dB"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Threshold/Offset provides either the threshold value or the offset value

to be used for conditional reporting."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 22 }

dot11RRMRqstSTAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER {

staCounters(0),

bssLoad(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the group identity if dot11RRMRqstType =

STA statistics Request. This attribute shall be ignored otherwise.

0 = STA Counters from dot11CountersTable

1 = BSS Load as defined in 7.3.2.38"

DEFVAL { 0 }

::= { dot11RRMRequestEntry 23 }

dot11RRMRqstLCIRqstOctet OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the subject of the LCI measurement request."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 24 }

dot11RRMRqstLCILatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude of the LCI measurement

request."

::= { dot11RRMRequestEntry 25 }

dot11RRMRqstLCILongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude of the LCI measurement

request."

::= { dot11RRMRequestEntry 26 }

dot11RRMRqstLCIAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Altitude of the LCI measurement

request."

::= { dot11RRMRequestEntry 27 }

dot11RRMRqstPauseTime OBJECT-TYPE

SYNTAX INTEGER (0..65535)

UNITS "10 TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Pause Time consists of a 16 bit unsigned integer number

representing the time period for which measurements are

suspended or paused. Measurement Pause Requests are used to

provide time delays between the execution times of measurement

request elements in a Measurement Request Frame"

DEFVAL { 0 }

::= { dot11RRMRequestEntry 28 }

dot11RRMRqstQoSMetricsPeerQSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the peer QSTA address to be measured for a QoS Metrics measurement"

::= { dot11RRMRequestEntry 29 }

dot11RRMRqstQoSMetricsTrafficIdentifier OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the TC, or TS to be measured for a QoS Metrics measurement"

::= { dot11RRMRequestEntry 30 }

dot11RRMRqstQoSMetricsBin0Range OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the delay range for bin 0 of the transmit delay histogram."

::= { dot11RRMRequestEntry 31 }

dot11RRMRqstTrigdQoSAverageCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Average bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the number of MSDUs discarded averaged over dot11RRMRqstTrigdQoSMEasurementCount reaching dot11RRMRqstTrigdQoSAverageThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 32 }

dot11RRMRqstTrigdQoSConsecutiveCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Consecutive bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the consecutive number of MSDUs discarded reaching dot11RRMRqstTrigdQoSConsecutiveThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 33 }

dot11RRMRqstTrigdQoSDelayCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Delay bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the consecutive number of MSDUs that experience a transmit delay greater than dot11RRMRqstTrigdQoSDelayThresholdRange reaching dot11RRMRqstTrigdQoSDelayThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 34 }

dot11RRMRqstTrigdQoSAverageThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The trigger threshold for triggered QoS Metrics based on average MSDUs discarded. Triggering occurs if the number of MSDUs discarded over the moving average number of transmitted MSDUs in dot11RRMRqstTrigdQoSMeasurementCount reaches this threshold."

DEFVAL { 10 }

::= { dot11RRMRequestEntry 35 }

dot11RRMRqstTrigdQoSConsecutiveThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The trigger threshold for triggered QoS Metrics based on consecutive MSDUs discarded. Triggering occurs if the consecutive number of MSDUs discarded reaches this threshold."

DEFVAL { 5 }

::= { dot11RRMRequestEntry 36 }

dot11RRMRqstTrigdQoSDelayThresholdRange OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The first part of the trigger threshold for triggered QoS Metrics based on consecutive MSDUs delayed. Triggering occurs if the a consecutive number of MSDUs experience a transmit delay greater than or equal to the lower bound of the bin of the Transmit Delay Histogram given by the value of this attribute + 2, e.g. if this attribute is 1 the lower bound of bin 3."

DEFVAL { 1 }

::= { dot11RRMRequestEntry 37 }

dot11RRMRqstTrigdQoSDelayThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The second part of the trigger threshold for triggered QoS Metrics based on consecutive MSDUs delayed. Triggering occurs if the consecutive number of MSDUs that experience a transmit delay greater than dot11RRMRqstQoSDelayThresholdRange reaches this value."

DEFVAL { 20 }

::= { dot11RRMRequestEntry 38 }

dot11RRMRqstTrigdQoSMeasurementCount OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains a number of MSDUs to be used as a moving average count in the average error threshold and in determining the scope of the reported QoS metrics in a triggered measurement report."

DEFVAL { 100 }

::= { dot11RRMRequestEntry 39 }

dot11RRMRqstTrigdQoSTimeout OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains a value in units of 100TU during which a measuring STA shall not generate further triggered QoS metrics reports after a trigger condition has been met and a report generated."

DEFVAL { 20 }

::= { dot11RRMRequestEntry 40 }

-- **********************************************************************

-- * End of dot11RRMRequest TABLE

-- **********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Reports

-- * Report tables contain measurement reports received by this STA or

-- * results of measurements performed by this STA.

-- ********************************************************************

dot11RRMReport OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 2 }

-- ********************************************************************

-- * dot11ChannelLoadReport TABLE

-- ********************************************************************

dot11ChannelLoadReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Channel Load reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 1 }

dot11ChannelLoadReportEntry OBJECT-TYPE

SYNTAX Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ChannelLoadReportTable Indexed by

dot11ChannelLoadRprtIndex."

INDEX { dot11ChannelLoadRprtIndex }

::= { dot11ChannelLoadReportTable 1 }

Dot11ChannelLoadReportEntry ::=

SEQUENCE {

dot11ChannelLoadRprtIndex Unsigned32,

dot11ChannelLoadRprtRqstToken OCTET STRING,

dot11ChannelLoadRprtIfIndex InterfaceIndex,

dot11ChannelLoadMeasuringSTAAddr MacAddress,

dot11ChannelLoadRprtChanNumber INTEGER,

dot11ChannelLoadRprtRegulatoryClass INTEGER,

dot11ChannelLoadRprtActualStartTime TSFType,

dot11ChannelLoadRprtMeasurementDuration Unsigned32,

dot11ChannelLoadRprtChannelLoad INTEGER,

dot11ChannelLoadRprtMeasurementMode INTEGER }

dot11ChannelLoadRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Channel Load Report elements in

dot11ChannelLoadReportTable, greater than 0."

::= { dot11ChannelLoadReportEntry 1 }

dot11ChannelLoadRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

Measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be  multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11ChannelLoadReportEntry 2 }

dot11ChannelLoadRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of ChannelLoad Report has been received on."

::= { dot11ChannelLoadReportEntry 3 }

dot11ChannelLoadMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Channel Load report."

::= { dot11ChannelLoadReportEntry 4 }

dot11ChannelLoadRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number for this row of Channel Load Report Table."

::= { dot11ChannelLoadReportEntry 5 }

dot11ChannelLoadRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

ChannelLoad report has been conducted."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11ChannelLoadReportEntry 6 }

dot11ChannelLoadRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11ChannelLoadReportEntry 7 }

dot11ChannelLoadRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the ChannelLoad

Report was measured."

::= { dot11ChannelLoadReportEntry 8 }

dot11ChannelLoadRprtChannelLoad OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Load shall contain the fractional duration over which the

measuring STA determined the channel to be busy during the

measurement duration.”

REFERENCE

"IEEE 802.11, Clause 7.3.2.22.4"

::= { dot11ChannelLoadReportEntry 9 }

dot11ChannelLoadRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 2 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11ChannelLoadReportEntry 10 }

-- ********************************************************************

-- * End of dot11ChannelLoadReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11NoiseHistogramReport TABLE

-- ********************************************************************

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Noise Histogram reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 2 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable

Indexed by dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex Unsigned32,

dot11NoiseHistogramRprtRqstToken OCTET STRING,

dot11NoiseHistogramRprtIfIndex InterfaceIndex,

dot11NoiseHistogramMeasuringSTAAddr MacAddress,

dot11NoiseHistogramRprtChanNumber INTEGER,

dot11NoiseHistogramRprtRegulatoryClass INTEGER,

dot11NoiseHistogramRprtActualStartTime TSFType,

dot11NoiseHistogramRprtAntennaID INTEGER,

dot11NoiseHistogramRprtANPI INTEGER,

dot11NoiseHistogramRprtANPI INTEGER,

dot11NoiseHistogramRprtMeasurementDuration Unsigned32,

dot11NoiseHistogramRprtIPIDensity0 INTEGER,

dot11NoiseHistogramRprtIPIDensity1 INTEGER,

dot11NoiseHistogramRprtIPIDensity2 INTEGER,

dot11NoiseHistogramRprtIPIDensity3 INTEGER,

dot11NoiseHistogramRprtIPIDensity4 INTEGER,

dot11NoiseHistogramRprtIPIDensity5 INTEGER,

dot11NoiseHistogramRprtIPIDensity6 INTEGER,

dot11NoiseHistogramRprtIPIDensity7 INTEGER,

dot11NoiseHistogramRprtIPIDensity8 INTEGER,

dot11NoiseHistogramRprtMeasurementMode INTEGER }

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Noise Histogram elements in dot11NoiseHistogramReportTable,

greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Noise Histogram Report has been received

on. "

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Noise Histogram

report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this measurement report was made."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

noise histogram report has been conducted."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Noise Histogram

Report was measured."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used for

this measurement. The value 0 indicates that the antenna identifier is

unknown.  'The value 255 indicates that the measurement was made with

multiple antennas or that the antenna ID is unknown. that the antenna

identifier is unknown. The value 255 indicates that this measurement was

made with multiple antennas. The value 1 is used for a STA with only one

antenna.  STAs with more than one antenna shall assign Antenna IDs to

each antenna as consecutive, ascending numbers. Each  Antenna ID number

represents a unique antenna characterized by a fixed relative position,

a fixed relative direction and a peak gain for that position and

direction."

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtANPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the ANPI for this Noise Histogram measurement.  Average Noise Power Indicator (ANPI) value represents

the average noise plus interference power on the measured channel at the antenna connector during the measurement duration To calculate ANPI, the

STA shall measure and use IPI in the specified channel when NAV is equal

to 0 (when virtual CS mechanism indicates idle channel) except during

frame transmission or reception."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtIPIDensity0 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from Power <= -92dBm."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtIPIDensity1 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -92dBm < Power <= -87dBm."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtIPIDensity2 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -87dBm < Power <= -82dBm."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtIPIDensity3 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -82dBm < Power <= -77dBm."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtIPIDensity4 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -77dBm < Power <= -72dBm."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtIPIDensity5 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -72dBm < Power <= -67dBm."

::= { dot11NoiseHistogramReportEntry 16 }

dot11NoiseHistogramRprtIPIDensity6 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -67dBm < Power <= -62dBm."

::= { dot11NoiseHistogramReportEntry 17 }

dot11NoiseHistogramRprtIPIDensity7 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from -62dBm < Power <= -57dBm."

::= { dot11NoiseHistogramReportEntry 18 }

dot11NoiseHistogramRprtIPIDensity8 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured IPI density for non-802.11 signals

from Power -57dBm <= Power."

::= { dot11NoiseHistogramReportEntry 19 }

dot11NoiseHistogramRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 3 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11NoiseHistogramReportEntry 20 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex Unsigned32,

dot11BeaconRprtRqstToken OCTET STRING,

dot11BeaconRprtIfIndex InterfaceIndex,

dot11BeaconMeasuringSTAAddr MacAddress,

dot11BeaconRprtChanNumber INTEGER,

dot11BeaconRprtRegulatoryClass INTEGER,

dot11BeaconRprtActualStartTime TSFType,

dot11BeaconRprtMeasurementDuration Unsigned32,

dot11BeaconRprtPhyType INTEGER,

dot11BeaconRprtReportedFrameType INTEGER,

dot11BeaconRprtRCPI INTEGER,

dot11BeaconRprtRSNI INTEGER,

dot11BeaconRprtBSSID MacAddress,

dot11BeaconRprtAntennaID INTEGER,

dot11BeaconRprtParentTSF TSFType,

dot11BeaconRprtReportedFrameBody OCTET STRING,

dot11BeaconRptMeasurementMode INTEGER }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable, greater than

0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Beacon Report has been received on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected beacons

and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row

of beacon report has been conducted."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report was

measured."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the PHY Type for this row of

Beacon Report."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtReportedFrameType OBJECT-TYPE

SYNTAX INTEGER {

beaconOrProbeResponse(0),

measurementPilot(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the frame type reported in
dot11BeaconRprtReportedFrameBody"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or

probe response frame in dBm, as defined in the RCPI measurement clause

for the indicated PHY Type."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise ratio of the beacon or probe response frame in dB. RSNI is the received signal to noise plus interference ratio derived from the measured RCPI for the received frame and from the measured ANPI for the channel used to receive the frame. RSNI is calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI) where RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to +118dB.

"

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used

for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving measuring STA’s

TSF value at the time the measuring STA received the beacon or probe

response frame."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtReportedFrameBody OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the fixed fields and information elements from the

frame body of the Beacon, Measurement Pilot or Probe Response frame being received.

All reported TIM elements are truncated to 4 octets."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 2 indicates this STA is refusing to generate

the report.   0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11BeaconReportEntry 17 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex Unsigned32,

dot11FrameRprtIfIndex InterfaceIndex,

dot11FrameRprtRqstToken Unsigned32,

dot11FrameRprtChanNumber INTEGER,

dot11FrameRprtRegulatoryClass INTEGER,

dot11FrameRprtActualStartTime TSFType,

dot11FrameRprtMeasurementDuration Unsigned32,

dot11FrameRprtTransmitSTAAddress MacAddress,

dot11FrameRprtBSSID MacAddress,

dot11FrameRprtPhyType INTEGER,

dot11FrameRprtAvgRCPI INTEGER,

dot11FrameRprtRSNI INTEGER,

dot11FrameRprtLastRCPI INTEGER,

dot11FrameRprtAntennaID INTEGER,

dot11FrameRprtNumberFrames INTEGER,

dot11FrameRptMeasurementMode INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Report elements in dot11FrameReportTable, greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Frame Report has been received on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that corresponds

to this row of frame report. Since a single frame request can generate

multiple rows in the frame report table, one per BSSID, this

dot11FrameRprtRqstToken indicates which request this particular row

corresponds to. If this row of report is received without a particular

request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected frame request

responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

frame report has been conducted."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when measurement

started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Frame Report

was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtTransmitSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has been

received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that transmitted this

frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtPhyType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"PHY Type indicates the PHY in which this row of frame report has been

conducted."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtAvgRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The average value for the received channel power of all the frames received and counted in this Frame Report Entry."

::= { dot11FrameReportEntry 11 }

dot11FrameRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "0.5 dB"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

This attribute indicates the received signal to noise ratio of the received frame in dBm. RSNI is the received signal to noise plus interference ratio derived from the RCPI for the received frame and from the most recent ANPI value measured on the channel used to receive the frame. RSNI may be calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI) where RSNI = [(ratio(dB) + 10) * 2],  for  ratios in the range -10dB to +118dB.  Other measurement techniques are allowed."

::= { dot11FrameReportEntry 12 }

dot11FrameRprtLastRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the most

recently measured frame in this Frame Report entry. Last RCPI is

reported in dBm, as defined in the RCPI measurement clause for the

PHY Type."

::= { dot11FrameReportEntry 13 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11FrameReportEntry 14 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report."

::= { dot11FrameReportEntry 15 }

dot11FrameRptMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 2 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11FrameReportEntry 14 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport TABLE

-- ********************************************************************

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of STA Statistics reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 5 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable

Indexed by dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex Unsigned32,

dot11STAStatisticsReportToken OCTET STRING,

dot11STAStatisticsIfIndex InterfaceIndex,

dot11STAStatisticsSTAAddress MacAddress,

dot11STAStatisticsMeasurementDuration Unsigned32,

dot11STAStatisticsGroupID Unsigned32,

dot11STAStatisticsTransmittedFragmentCount Counter32,

dot11STAStatisticsMulticastTransmittedFrameCount Counter32,

dot11STAStatisticsFailedCount Counter32,

dot11STAStatisticsRetryCount Counter32,

dot11STAStatisticsMultipleRetryCount Counter32,

dot11STAStatisticsFrameDuplicateCount Counter32,

dot11STAStatisticsRTSSuccessCount Counter32,

dot11STAStatisticsRTSFailureCount Counter32,

dot11STAStatisticsACKFailureCount Counter32,

dot11STAStatisticsReceivedFragmentCount Counter32,

dot11STAStatisticsMulticastReceivedFrameCount Counter32,

dot11STAStatisticsFCSErrorCount Counter32,

dot11STAStatisticsTransmittedFrameCount Counter32,

dot11STAStatisticsAPServiceLoad INTEGER,

dot11STAStatisticsAverageAccessDelayBestEffort INTEGER,

dot11STAStatisticsAverageAccessDelayBackground INTEGER,

dot11STAStatisticsAverageAccessDelayVideo INTEGER,

dot11STAStatisticsAverageAccessDelayVoice INTEGER,

dot11STAStatisticsStationCount INTEGER,

dot11STAStatisticsChannelUtilization INTEGER,

dot11STAStatisticsRprtMeasurementMode INTEGER }

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA Statistics Report elements in dot11STAStatisticsReportTable,

greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of STA Statistics Report has been

received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this STA Statistics report"

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNIT "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the STA Statistics

was measured."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsGroupID OBJECT-TYPE

SYNTAX INTEGER {

staCounters(0),

bssLoad(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RRMRqstSTAStatRqstGroupID returned from the STA in

STA in this STA Statistics Report."

DEFVAL { 0 }

::= { dot11FrameReportEntry 6 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFragmentCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastTransmittedFrameCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FailedCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RetryCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MultipleRetryCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FrameDuplicateCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSSuccessCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ACKFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ReceivedFragmentCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastReceivedFrameCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsFCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FCSErrorCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 18 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFrameCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsAPServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The AP Service Load is a scalar indication of the relative level

of service loading at an AP.

"

REFERENCE

"IEEE 802.11 – Clause 7.3.2.38"

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsAverageAccessDelayBestEffort OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay Best Effort element shall consist of an Average

Access Delay (AAD) for the Best Effort Access Category.

"

REFERENCE

"IEEE 802.11 – Clause 7.3.2.43"

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsAverageAccessDelayBackground OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay Background element shall consist of an

Average Access Delay (AAD) for the Background Access Category.

"

REFERENCE

"IEEE 802.11 – Clause 7.3.2.43"

::= { dot11STAStatisticsReportEntry 22 }

dot11STAStatisticsAverageAccessDelayVideo OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay Video element shall consist of an Average

Access Delay (AAD) for the Video Access Category.

"

REFERENCE

"IEEE 802.11 – Clause 7.3.2.43"

::= { dot11STAStatisticsReportEntry 23 }

dot11STAStatisticsAverageAccessDelayVoice OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay Voice element shall consist of an Average

Access Delay (AAD) for the Voice Access Category.

"

REFERENCE

"IEEE 802.11 – Clause 7.3.2.43"

::= { dot11STAStatisticsReportEntry 24 }

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Station Count field is an unsigned integer that indicates the total number of STAs currently associated with this BSS."

::= { dot11STAStatisticsReportEntry 25 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The Channel Utilization field is the percentage of time the AP sensed

the medium busy, as indicated by either the physical or virtual

carrier sense mechanism. This percentage is represented as a moving

average of ((channel busy time/(dot11ChannelUtilizationBeaconIntervals

* dot11BeaconPeriod * 1024)) *255), where ‘channel busy time’ is

defined to be the number of microseconds during which the carrier

sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and dot11ChannelUtilizationBeaconIntervals represents the

number of consecutive beacon intervals during which the average should

be calculated."

REFERENCE

"IEEE 802.11 – Clause 9.2.1"

::= { dot11STAStatisticsReportEntry 26 }

dot11STAStatisticsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 2 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 27 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11LCIReport TABLE

-- ********************************************************************

dot11LCIReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of LCI reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 6 }

dot11LCIReportEntry OBJECT-TYPE

SYNTAX Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIReportTable

Indexed by dot11LCIReportIndex."

INDEX { dot11LCIReportIndex }

::= { dot11LCIReportTable 1 }

Dot11LCIReportEntry ::=

SEQUENCE {

dot11LCIReportIndex Unsigned32,

dot11LCIReportToken OCTET STRING,

dot11LCIIfIndex InterfaceIndex,

dot11LCISTAAddress MacAddress,

dot11LCILatitudeResolution INTEGER,

dot11LCILatitudeInteger INTEGER,

dot11LCILatitudeFraction Unsigned32,

dot11LCILongitudeResolution INTEGER,

dot11LCILongitudeInteger INTEGER,

dot11LCILongitudeFraction Unsigned32,

dot11LCIAltitudeType INTEGER,

dot11LCIAltitudeResolution INTEGER,

dot11LCIAltitudeInteger Integer32,

dot11LCIAltitudeFraction Unsigned32,

dot11LCIDatum INTEGER,

dot11AzimuthType INTEGER,

dot11AzimuthResolution INTEGER,

dot11Azimuth Integer32,

dot11LCIRprtMeasurementMode INTEGER }

dot11LCIReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI Report elements in dot11LCIReportTable,

greater than 0."

::= { dot11LCIReportEntry 1 }

dot11LCIReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11LCIReportEntry 2 }

dot11LCIIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of LCI Report has been

received on"

::= { dot11LCIReportEntry 3 }

dot11LCISTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this LCI report"

::= { dot11LCIReportEntry 4 }

dot11LCILatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude."

::= { dot11LCIReportEntry 5 }

dot11LCILatitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

9 bits of integer portion of Latitude."

::= { dot11LCIReportEntry 6 }

dot11LCILatitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

25 bits of fraction portion of Latitude."

::= { dot11LCIReportEntry 7 }

dot11LCILongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude."

::= { dot11LCIReportEntry 8 }

dot11LCILongitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

9 bits of integer portion of Longitude."

::= { dot11LCIReportEntry 9 }

dot11LCILongitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

25 bits of fraction portion of Longitude."

::= { dot11LCIReportEntry 10 }

dot11LCIAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude Type is four bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

floors : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction"

::= { dot11LCIReportEntry 11 }

dot11LCIAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid

bits in the altitude."

::= { dot11LCIReportEntry 12 }

dot11LCIAltitudeInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction.  This field contains the fixed-point

Part of Altitude."

::= { dot11LCIReportEntry 13 }

dot11LCIAltitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction.  This field contains the fraction part

of Altitude."

::= { dot11LCIReportEntry 14 }

dot11LCIDatum OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Datum is an eight-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI."

::= { dot11LCIReportEntry 15 }

dot11AzimuthType OBJECT-TYPE

SYNTAX INTEGER {

frontSurfaceOfSTA(0),

radioBeam(1) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth Type is a one bit attribute encoding the type of Azimuth.

Codes defined are:

front surface of STA : in 2s-complement fixed-point 9-bit integer

radio beam : in 2s-complement fixed-point 9-bit integer"

::= { dot11LCIReportEntry 16 }

dot11AzimuthResolution OBJECT-TYPE

SYNTAX INTEGER (0..15)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth Resolution is 4 bits indicating the number of valid

bits in the azimuth."

::= { dot11LCIReportEntry 17 }

dot11Azimuth OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Azimuth is a 9 bit value defined by the Azimuth Type field.

The field is encoded as a 2s-complement fixed-point 9-bit integer

horizontal angle in degrees from True North."

::= { dot11LCIReportEntry 18 }

dot11LCIRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 2 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11LCIReportEntry 19 }

-- ********************************************************************

-- * End of dot11LCIReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11QoSMetricsReport TABLE

-- ********************************************************************

dot11QoSMetricsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QoSMetricsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Transmit Delay Metrics reports that

have been received by the MLME. The report tables shall be maintained as

FIFO to preserve freshness, thus the rows in this table can be deleted

for memory constraints or other implementation constraints determined by

the vendor. New rows shall have different RprtIndex values than those

deleted within the range limitation of the index. One easy way is to

monotonically increase RprtIndex for new reports being written in the

table."

::= { dot11RRMReport 7 }

dot11QoSMetricsReportEntry OBJECT-TYPE

SYNTAX Dot11QoSMetricsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11QoSMetricsReportTable Indexed by dot11QoSMetricsRprtIndex."

INDEX { dot11QoSMetricsRprtIndex }

::= { dot11QoSMetricsReportTable 1 }

Dot11QoSMetricsReportEntry ::=

SEQUENCE {

dot11QoSMetricsRprtIndex Unsigned32,

dot11QoSMetricsRprtRqstToken OCTET STRING,

dot11QoSMetricsRprtIfIndex InterfaceIndex,

dot11QoSMetricsMeasuringSTAAddr MacAddress,

dot11QoSMetricsRprtActualStartTime TSFType,

dot11QoSMetricsRprtMeasurementDuration Unsigned32,

dot11QoSMetricsRprtPeerSTAAddress MacAddress,

dot11QoSMetricsRprtTID INTEGER,

dot11QoSMetricsRprtAverageQueueDelay Integer32,

dot11QoSMetricsRprtAverageTransmitDelay Integer32,

dot11QoSMetricsRprtTransmittedMSDUCount Integer32,

dot11QoSMetricsRprtMSDUDiscardedCount Integer32,

dot11QoSMetricsRprtMSDUFailedCount Integer32,

dot11QoSMetricsRprtMultipleRetryCount Integer32,

dot11QoSMetricsRprtCFPollsLostCount Integer32,

dot11QoSMetricsRprtBin0Range INTEGER,

dot11QoSMetricsRprtDelayHistogram OCTET STRING,

dot11QoSMetricsRprtMeasurementMode INTEGER,

dot11QoSMetricsRprtReason INTEGER }

dot11QoSMetricsRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Transmit Delay Metrics Report elements in

dot11QoSMetricsReportTable, greater than 0."

::= { dot11QoSMetricsReportEntry 1 }

dot11QoSMetricsRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken attribute.

Note that there may be multiple entries in the table that match this

value since a single request may generate multiple measurement

reports."

::= { dot11QoSMetricsReportEntry 2 }

dot11QoSMetricsRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of TransmitQoSMetrics Report has

been received on."

::= { dot11QoSMetricsReportEntry 3 }

dot11QoSMetricsMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Transmit

Delay Metrics report."

::= { dot11QoSMetricsReportEntry 4 }

dot11QoSMetricsRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when

the measurement started or for a triggered QoS metrics report the

TSF value at the reporting QSTA when the trigger condition was met."

::= { dot11QoSMetricsReportEntry 5 }

dot11QoSMetricsRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Transmit

Delay Metrics Report was measured. For a triggered QoS Metrics Report, metrics are reported over a number of transmitted MSDUs rather than a duration, hence Measurement Duration shall be set to 0."

::= { dot11QoSMetricsReportEntry 6 }

dot11QoSMetricsRprtPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address present in the Address 1 filed of the measured

data frames for this row of Transmit QoS Metrics report."

::= { dot11QoSMetricsReportEntry 7 }

dot11QoSMetricsRprtTID OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Traffic Identifier shall indicate the TC or TS for which traffic is to

be measured. Values 0 through 15 are defined. Values 16-255 are reserved. .”

::= { dot11QoSMetricsReportEntry 8 }

dot11QoSMetricsRprtAverageQueueDelay OBJECT-TYPE

SYNTAX Integer32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the average delay of the frames (MSDUs) that are

passed to the MAC during the measurement duration for the specified

destination and the specified Traffic Identifier. Queue Delay shall be

measured from the time the MSDU is passed to the MAC until the transmission

starts and shall be expressed in TUs."

::= { dot11QoSMetricsReportEntry 9 }

dot11QoSMetricsRprtAverageTransmitDelay OBJECT-TYPE

SYNTAX Integer32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the average delay of the frames (MSDUs) that are

successfully transmitted during the measurement duration for the

specified destination and the specified Traffic Identifier. Delay shall

be measured from the time the MSDU is passed to the MAC until ACK is

received from the intermediate destination."

::= { dot11QoSMetricsReportEntry 10}

dot11QoSMetricsRprtTransmittedMSDUCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or TS

given by the Traffic Identifier successfully transmitted in

the measurement duration”

::= {dot11QoSMetricsReportEntry 11}

dot11QoSMetricsRprtMSDUDiscardedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or

TS given by the Traffic Identifier discarded due either to the

number of transmit attempts exceeding dot11ShortRetryLimit or

dot11LongRetryLimit as appropriate, or due to the MSDU lifetime

having been reached”

::= {dot11QoSMetricsReportEntry 12}

dot11QoSMetricsRprtMSDUFailedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or TS

given by the Traffic Identifier discarded during the measurement

duration due to the number of transmit attempts exceeding

dot11ShortRetryLimit or dot11LongRetryLimit as appropriate.”

::= {dot11QoSMetricsReportEntry 13}

dot11QoSMetricsRprtMultipleRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs for the TC, or TS given by the

Traffic Identifier that are successfully transmitted after

more than one retransmission attempt.”

::= {dot11QoSMetricsReportEntry 14}

dot11QoSMetricsRprtCFPollsLostCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of QoS (+)CF-Poll frames transmitted to the

peer QSTA where there was no response from the QSTA. ”

::= {dot11QoSMetricsReportEntry 15}

dot11QoSMetricsRprtBin0Range OBJECT-TYPE

SYNTAX INTEGER

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the delay range for Bin 0 of the delay histogram.”

::= { dot11QoSMetricsReportEntry 16 }

dot11QoSMetricsRprtDelayHistogram OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the histogram of delay of the frames (MSDUs)

that are successfully transmitted during the measurement duration

for the specified Traffic Identifier and the specified destination.

Delay shall be measured from the time the MSDU is passed to the MAC

until the ACK is received from the intermediate destination and shall

be expressed in TUs. "

::= { dot11QoSMetricsReportEntry 17 }

dot11QoSMetricsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

success(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 1 indicates this STA is incapable of

generating the report. 3 indicates this STA is refusing to generate

the report.  0 indicates the STA successfully carried out a

measurement request."

DEFVAL { 0 }

::= { dot11QoSMetricsReportEntry 18 }

dot11QoSMetricsRprtReason OBJECT-TYPE

- SYNTAX INTEGER {

averageError(0),

consecutiveError(1),

delayError(2),

}

- MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Reason corresponds to the Reason field indicating the reason that

the measuring QSTA sent the TransmitQoS metrics report.”

- DEFVAL { 0 }

::= { dot11QoSMetricsReportEntry 19 }

-- ********************************************************************

-- * End of dot11QoSMetricsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Configuration Information

-- ********************************************************************

dot11RRMConfig OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 3 }

-- ********************************************************************

-- * dot11APChannelReport TABLE

-- ********************************************************************

dot11APChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"AP Channel Report information, in tabular form."

::= { dot11RRMConfig 1 }

dot11APChannelReportEntry OBJECT-TYPE

SYNTAX Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11APChannelReportTable. Each entry in the table is

indexed by dot11APChannelReportIndex."

INDEX { dot11APChannelReportIndex }

::= { dot11APChannelReportTable 1 }

Dot11APChannelReportEntry ::=

SEQUENCE {

dot11APChannelReportIndex Unsigned32,

dot11APChannelReportIfIndex InterfaceIndex,

dot11APChannelReportRegulatoryClass INTEGER,

dot11APChannelReportChannelList OCTET STRING }

dot11APChannelReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for AP channel report entry in dot11APChannelReportTable, greater

than 0."

::= { dot11APChannelReportEntry 1 }

dot11APChannelReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex this row of AP channel report entry belongs to."

::= { dot11APChannelReportEntry 2 }

dot11APChannelReportRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list

being reported."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11APChannelReportEntry 3 }

dot11APChannelReportChannelList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the specific channels being reported. Each

octet corresponds to a different channel within the specified Regulatory Class. This list of channels is the Channel List in the AP Channel Report element described in 7.3.2.26."

::= { dot11APChannelReportEntry 4 }

-- ********************************************************************

-- * End of dot11APChannelReportTable TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RRMNeighborReport TABLE

-- ********************************************************************

dot11RRMNeighborReportNextIndex OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Identifies the next available index for managing the Neighbor Report table.

If this attribute is 0, it indicates that the Neighbor Report feature is not

configurable via SNMP, or the table is full and new rows cannot be

accepted."

::= { dot11RRMConfig 2 }

dot11RRMNeighborReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains pertinent information on a collection of BSSID’s that are

candidates to which STA’s can roam. The rows are created using createAndWait

method and fill in the attributes. When the rowStatus is set to active, the

row can be included in Neighbor Report IEs. If there is an error, the

rowStatus shall be set to notReady by SME. Since this table contains all

Neighbor Report IE entries for all interfaces enabled with the Neighbor

Report feature, it is possible to have too many entries for one interface,

while still remaining under the MaxTableSize. In that situation, SME shall

only include Neighbor Report entries with lower

dot11RRMNeighborReportIFIndex up to the maximum possible number of entries

for a particular interface identified by ifIndex.

SME shall set the rowStatus to notInService for those rows that cannot be

included in the Neighbor Report element for that interface."

::= { dot11RRMConfig 3 }

dot11RRMNeighborReportEntry OBJECT-TYPE

SYNTAX Dot11RRMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMNeighborReportTable"

INDEX { dot11RRMNeighborReportIndex }

::= { dot11RRMNeighborReportTable 1 }

Dot11RRMNeighborReportEntry ::=

SEQUENCE {

dot11RRMNeighborReportIndex Unsigned32,

dot11RRMNeighborReportIfIndex InterfaceIndex,

dot11RRMNeighborReportBSSID MacAddress,

dot11RRMNeighborReportAPReachability INTEGER,

dot11RRMNeighborReportSecurity TruthValue,

dot11RRMNeighborReportCapSpectrumMgmt TruthValue,

dot11RRMNeighborReportCapQoS TruthValue,

dot11RRMNeighborReportCapAPSD TruthValue,

dot11RRMNeighborReportCapRRM TruthValue,

dot11RRMNeighborReportCapDelayBlockAck TruthValue,

dot11RRMNeighborReportCapImmediateBlockAck TruthValue,

dot11RRMNeighborReportKeyScope TruthValue,

dot11RRMNeighborReportChannelNumber INTEGER,

dot11RRMNeighborReportRegulatoryClass INTEGER,

dot11RRMNeighborReportPhyType INTEGER,

dot11RRMNeighborReportNeighborTSFInfo Unsigned32,

dot11RRMNeighborReportTSFInfoFlag TruthValue,

dot11RRMNeighborReportBeaconInterval Unsigned32,

dot11RRMNeighborReportSSID OCTET STRING,

dot11RRMNeighborReportRowStatus RowStatus }

dot11RRMNeighborReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Neighbor Report configuration table in

dot11RRMNeighborReportTable, greater than 0."

::= { dot11RRMNeighborReportEntry 1 }

dot11RRMNeighborReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of Neighbor Report entry belongs to."

::= { dot11RRMNeighborReportEntry 2 }

dot11RRMNeighborReportBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The BSSID of the STA to which this row of Neighbor Report applies."

::= { dot11RRMNeighborReportEntry 3 }

dot11RRMNeighborReportAPReachability OBJECT-TYPE

SYNTAX INTEGER {

notReachable(1),

unknown(2),

reachable(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the reachability of the AP represented by

the dot11NeighborReportBSSID."

::= { dot11RRMNeighborReportEntry 4 }

dot11RRMNeighborReportSecurity OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the RSN capability of the AP

represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 5 }

dot11RRMNeighborReportCapSpectrumMgmt OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the spectrum management capability of the AP

represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 6 }

dot11RRMNeighborReportCapQoS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Indicates the QoS capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 7 }

dot11RRMNeighborReportCapAPSD OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the APSD capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 8 }

dot11RRMNeighborReportCapRRM OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the RRM capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 9 }

dot11RRMNeighborReportCapDelayBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the Delayed BlockAck capability of the AP represented

by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 10 }

dot11RRMNeighborReportCapImmediateBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the Immediate BlockAck capability of the AP represented

by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 11 }

dot11RRMNeighborReportKeyScope OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates a shared authenticator identity for the AP represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 12 }

dot11RRMNeighborReportChannelNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Channel Number of the current operating channel of

the AP represented by the dot11NeighborReportBSSID."

::= { dot11RRMNeighborReportEntry 13 }

dot11RRMNeighborReportRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list being

reported."

REFERENCE

"IEEE 802.11, Annex J"

::= { dot11RRMNeighborReportEntry 14 }

dot11RRMNeighborReportPhyType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The PHY Type of the Neighbor AP Entry."

::= { dot11RRMNeighborReportEntry 15 }

dot11RRMNeighborReportNeighborTSFInfo OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Neighbor TBTT info sub-element is as defined in clause 7.3.2.35."

::= { dot11RRMNeighborReportEntry 16 }

dot11RRMNeighborReportTSFInfoFlag OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates whether the TSF Info sub-element is included in the Neighbor Report element.”

::= { dot11RRMNeighborReportEntry 17 }

dot11RRMNeighborReportBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Beacon Interval field is as defined in clause 7.3.2.36."

::= { dot11RRMNeighborReportEntry 18 }

dot11RRMNeighborReportSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" Indicates the SSID of the AP represented by the

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 19 }

dot11RRMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains the row status of the Neighbor Report, essentially used for

indicating whether the row has all valid attributes filled in. Then set to

active to be used in Neighbor Report information elements. If any parameter

is invalid, the SME shall set this attribute back to notReady. It is the

responsibility of the manager to correct the parameters."

::= { dot11RRMNeighborReportEntry 20 }

-- ********************************************************************

-- * End of dot11RRMNeighborReport TABLE

-- ********************************************************************

In dot11OperationTable of Annex D, insert the following text to the end of Dot11OperationEntry sequence list:

dot11PeerStatsTableBSSaging Unsigned32,

dot11PeerStatsTableIBSSaging Unsigned32 }

Insert the following elements to the end of dot11OperationTable element definition behind dot11ProductID:

dot11PeerStatsTableBSSaging OBJECT-TYPE

SYNTAX Unsigned32

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The peerStatsTableBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was authenticated or associated with the

local STA shall be kept in the table after the peer STA

becomes disassociated. The peer STA entry may be kept in the

peerStatsTable for longer than this time."

DEFVAL { 300 }

::= { dot11OperationEntry 10 }

dot11PeerStatsTableIBSSaging OBJECT-TYPE

SYNTAX Unsigned32

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The peerStatsTableIBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was in an IBSS that the local STA was

joined to, and send a data frame to the local STA, or the

local STA attempted to send any frame to the peer STA shall

be kept in the table after the peer STA last communicated

with the local STA. The peer STA entry may be kept in the

peerStatsTable for longer than this time."

DEFVAL { 300 }

::= { dot11OperationEntry 11 }

In dot11MAC MIB of Annex D, insert the peerStatsTable behind GroupAddressesTable:

--**********************************************************************

--* dot11PeerStatsTable TABLE

--**********************************************************************

dot11PeerStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PeerStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains statistics for peer STAs. The following peer STAs

appear in this table:

1. Authenticated peer STAs must appear in this table. Their

entries must not be removed until at least

dot11PeerStatsTableBSSaging seconds have passed after they are

marked as deauthenticated.

2. Peer STAs joined to the same IBSS as this STA that have

sent any data frame to this STA must appear in this table.

Entries for these peer STAs may be removed when this STA is

no longer joined to the same IBSS, or if dot11PeerStatsTableIBSSaging

seconds have passed since a data frame was exchanged with the peer STA.

3. Other peer STAs may be included in this table."

::= { dot11mac 4 }

dot11PeerStatsEntry OBJECT-TYPE

SYNTAX Dot11PeerStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the Dot11PeerStaStatsEntry Table.

Indexed by ifIndex and the MAC address of the remote

STA dot11PeerStaAddress"

INDEX { ifIndex, dot11PeerAddress }

::= { dot11PeerStatsTable 1 }

Dot11PeerStatsEntry ::=

SEQUENCE {

dot11PeerAddress MacAddress,

dot11PeerTransmittedFragmentCount Counter32,

dot11PeerMulticastTransmittedFrameCount Counter32,

dot11PeerFailedCount Counter32,

dot11PeerRetryCount Counter32,

dot11PeerMultipleRetryCount Counter32,

dot11PeerFrameDuplicateCount Counter32,

dot11PeerRTSSuccessCount Counter32,

dot11PeerRTSFailureCount Counter32,

dot11PeerACKFailureCount Counter32,

dot11PeerReceivedFragmentCount Counter32,

dot11PeerMulticastReceivedFrameCount Counter32,

dot11PeerTransmittedFrameCount Counter32,

dot11PeerWEPUndecryptableCount Counter32,

dot11PeerWEPICVErrorCount Counter32,

dot11PeerReceivedOctetCount Counter32,

dot11PeerTransmittedOctetCount Counter32,

dot11PeerReceivedBeaconCount Counter32,

dot11PeerLastReceivedRCPI INTEGER }

dot11PeerAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MAC address identifying a peer STA."

::= { dot11PeerStatsEntry 1 }

dot11PeerTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU

with an individual address in the address 1 field or an MPDU

with a multicast address in the address 1 field of type Data

or Management."

::= { dot11PeerStatsEntry 2 }

dot11PeerMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment only when the multicast bit

is set in the destination MAC address of a successfully

transmitted MSDU. When operating as a non-AP STA in an ESS, where

these frames are directed to the AP, this implies having

received an acknowledgment to all associated MPDUs."

::= { dot11PeerStatsEntry 3 }

dot11PeerFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is not transmitted

successfully due to the number of transmit attempts exceeding

either the dot11ShortRetryLimit or dot11LongRetryLimit."

::= { dot11PeerStatsEntry 4 }

dot11PeerRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after one or more retransmissions."

::= { dot11PeerStatsEntry 5 }

dot11PeerMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission."

::= { dot11PeerStatsEntry 6 }

dot11PeerFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

that the Sequence Control field indicates is a duplicate."

::= { dot11PeerStatsEntry 7 }

dot11PeerRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS."

::= { dot11PeerStatsEntry 8 }

dot11PeerRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS."

::= { dot11PeerStatsEntry 9 }

dot11PeerACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected."

::= { dot11PeerStatsEntry 10 }

dot11PeerReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management."

::= { dot11PeerStatsEntry 11 }

dot11PeerMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination MAC address."

::= { dot11PeerStatsEntry 12 }

dot11PeerTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully

transmitted MSDU."

::= { dot11PeerStatsEntry 13 }

dot11PeerWEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with

the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted

that frame is discarded due to the receiving STA not

implementing the privacy option."

::= { dot11PeerStatsEntry 14 }

dot11PeerWEPICVErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the

WEP subfield of the Frame Control field set to one and the value

of the ICV as received in the frame does not match the ICV value

that is calculated for the contents of the received frame."

::= { dot11PeerStatsEntry 15 }

dot11PeerReceivedOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Total number of octets of data in MSDUs received from this peer."

::= { dot11PeerStatsEntry 16 }

dot11PeerTransmittedOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Total number of octets of data in unicast MSDUs successfully transmitted

to this peer."

::= { dot11PeerStatsEntry 17 }

dot11PeerReceivedBeaconCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of beacon frames received from this peer."

::= { dot11PeerStatsEntry 18 }

dot11PeerLastReceivedRCPI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The RCPI from the most recently received frame, or 0 if no

frames have been received."

::= { dot11PeerStatsEntry 19 }

-- ********************************************************************

-- * End of dot11PeerStatsTable TABLE

-- ********************************************************************

Change the “dot11Compliance MODULE-COMPLIANCE” of the “Compliance Statements” as follows:

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement

The IEEE 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase7, dot11MACbase2, dot11CountersGroup2,

dot11SmtAuthenticationAlgorithms, dot11ResourceTypeID,

dot11PhyOperationComplianceGroup }

Change the “OPTIONAL-GROUPS” of the “Compliance Statements” as follows:

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

--    dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

--    dot11PhyRegDomainsSupportGroup,

--    dot11PhyAntennasListGroup, dot11PhyRateGroup,

--    dot11SMTbase3, dot11MultiDomainCapabilityGroup,

--    dot11PhyFHSSComplianceGroup2, dot11RSNAadditions,

--    dot11RegulatoryClassesGroup, dot11Qosadditions,

--    dot11RRMCompliance }

--

::= { dot11Compliances 1 }

Insert at the end of the “Groups - units of conformance” as follows:

-- ********************************************************************
-- * Compliance Statements - RRM
-- ********************************************************************

dot11RRMCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB for Measurement Services."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTRRMRequest,

dot11SMTRRMReport,

dot11SMTRRMConfig

}

-- OPTIONAL-GROUPS { }

::= { dot11Compliances 3 }

Deprecate the current SMTbase and add new SMTbase in the “Groups – units of conformance” clause

dot11SMTbase6 OBJECT-GROUP

OBJECTS{	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedinQBSS,

dot11DLSAllowed }

STATUS currentdeprecated

DESCRIPTION

"The SMTbase6 object class provides the necessary support

at the STA to manage the processes in the STA such that the

STA may work cooperatively as a part of an IEEE 802.11 net-

work, when the STA is capable of multi-domain operation.

This object group should be implemented when the multi-

domain capability option is implemented."

::= { dot11Groups 32 }

dot11SMTbase7 OBJECT-GROUP

OBJECTS{	dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID,

dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod,

dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11QosOptionImplemented,

dot11ImmediateBlockAckOptionImplemented,

dot11DelayedBlockAckOptionImplemented,

dot11DirectOptionImplemented,

dot11APSDOptionImplemented,

dot11QAckOptionImplemented,

dot11QBSSLoadOptionImplemented,

dot11QueueRequestOptionImplemented,

dot11TXOPRequestOptionImplemented,

dot11MoreDataAckOptionImplemented,

dot11AssociateinNQBSS,

dot11DLSAllowedinQBSS,

dot11DLSAllowed,

dot11SMTRRMRequest,

dot11SMTRRMReport,

dot11SMTRRMConfig }

STATUS current

DESCRIPTION

"The SMTbase7 object class provides the necessary support

at the STA to manage the processes in the STA such that the

STA may work cooperatively as a part of an IEEE 802.11 net-

work, when the STA is capable of multi-domain operation.

This object group should be implemented when the multi-

domain capability option is implemented."

::= { dot11Groups 38 }

Insert the following at the end of the section “Groups – units of conformance”:

dot11SMTRRMRequest OBJECT-GROUP

OBJECTS {   dot11RRMRqstIndex,

dot11RRMRqstRowStatus,

dot11RRMRqstToken,

dot11RRMRqstRepetitions,

dot11RRMRqstIfIndex,

dot11RRMRqstType,

dot11RRMRqstTargetAdd,

dot11RRMRqstTimeStamp,

dot11RRMRqstChanNumber,

dot11RRMRqstRegulatoryClass,

dot11RRMRqstRndInterval,

dot11RRMRqstDuration,

dot11RRMRqstParallel,

dot11RRMRqstEnable,

dot11RRMRqstRequest,

dot11RRMRqstReport,

dot11RRMRqstDurationMandatory,

dot11RRMRqstBeaconRqstMode,

dot11RRMRqstBssid,

dot11RRMRqstReportingCondition,

dot11RRMRqstThresholdOffset,

dot11RRMRqstSTAStatRqstGroupID,

dot11RRMRqstLCIRqstOctet,

dot11RRMRqstLCILatitudeResolution,

dot11RRMRqstLCILongitudeResolution,

dot11RRMRqstLCIAltitudeResolution,

dot11RRMRqstPauseTime,

dot11RRMRqstQoSMetricsPeerQSTAAddress,

dot11RRMRqstQoSMetricsTrafficIdentifier,

dot11RRMRqstQoSMetricsBin0Range,

dot11RRMRqstTrigdQoSAverageCondition,

dot11RRMRqstTrigdQoSConsecutiveCondition,

dot11RRMRqstTrigdQoSDelayCondition,

dot11RRMRqstTrigdQoSAverageThreshold,

dot11RRMRqstTrigdQoSConsecutiveThreshold,

dot11RRMRqstTrigdQoSDelayThresholdRange,

dot11RRMRqstTrigdQoSDelayThreshold,

dot11RRMRqstTrigdQoSMeasurementCount,

dot11RRMRqstTrigdQoSTimeout }

STATUS current

DESCRIPTION

"The SMTRRMRequest package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 35 }

dot11SMTRRMReport OBJECT-GROUP

OBJECTS {   dot11ChannelLoadRprtIndex,

dot11ChannelLoadRprtRqstToken,

dot11ChannelLoadRprtIfIndex,

dot11ChannelLoadMeasuringSTAAddr,

dot11ChannelLoadRprtChanNumber,

dot11ChannelLoadRprtRegulatoryClass,

dot11ChannelLoadRprtActualStartTime,

dot11ChannelLoadRprtMeasurementDuration,

dot11ChannelLoadRprtChannelLoad,

dot11ChannelLoadRprtMeasurementMode,

dot11NoiseHistogramRprtIndex,

dot11NoiseHistogramRprtRqstToken,

dot11NoiseHistogramRprtIfIndex,

dot11NoiseHistogramMeasuringSTAAddr,

dot11NoiseHistogramRprtChanNumber,

dot11NoiseHistogramRprtRegulatoryClass,

dot11NoiseHistogramRprtActualStartTime,

dot11NoiseHistogramRprtMeasurementDuration,

dot11NoiseHistogramRprtAntennaID,

dot11NoiseHistogramRprtANPI,

dot11NoiseHistogramRprtIPIDensity0,

dot11NoiseHistogramRprtIPIDensity1,

dot11NoiseHistogramRprtIPIDensity2,

dot11NoiseHistogramRprtIPIDensity3,

dot11NoiseHistogramRprtIPIDensity4,

dot11NoiseHistogramRprtIPIDensity5,

dot11NoiseHistogramRprtIPIDensity6,

dot11NoiseHistogramRprtIPIDensity7,

dot11NoiseHistogramRprtIPIDensity8,

dot11NoiseHistogramRprtMeasurementMode,

dot11BeaconRprtIndex,

dot11BeaconRprtRqstToken,

dot11BeaconRprtIfIndex,

dot11BeaconMeasuringSTAAddr,

dot11BeaconRprtChanNumber,

dot11BeaconRprtRegulatoryClass,

dot11BeaconRprtActualStartTime,

dot11BeaconRprtMeasurementDuration,

dot11BeaconRprtPhyType,

dot11BeaconRprtReportedFrameType,

dot11BeaconRprtRCPI,

dot11BeaconRprtRSNI,

dot11BeaconRprtBSSID,

dot11BeaconRprtAntennaID,

dot11BeaconRprtParentTSF,

dot11BeaconRprtReportedFrameBody,

dot11BeaconRprtMeasurementMode,

dot11FrameRprtIndex,

dot11FrameRprtIfIndex,

dot11FrameRprtRqstToken,

dot11FrameRprtChanNumber,

dot11FrameRprtRegulatoryClass,

dot11FrameRprtActualStartTime,

dot11FrameRprtMeasurementDuration,

dot11FrameRprtTransmitSTAAddress,

dot11FrameRprtBSSID,

dot11FrameRprtPhyType,

dot11FrameRprtAvgRCPI,

dot11FrameRprtRSNI,

dot11FrameRprtLastRCPI,

dot11FrameRprtAntennaID,

dot11FrameRprtNumberFrames,

dot11FrameRptMeasurementMode,

dot11STAStatisticsReportIndex,

dot11STAStatisticsReportToken,

dot11STAStatisticsIfIndex,

dot11STAStatisticsSTAAddress,

dot11STAStatisticsMeasurementDuration,

dot11STAStatisticsGroupID,

dot11STAStatisticsTransmittedFragmentCount,

dot11STAStatisticsMulticastTransmittedFrameCount,

dot11STAStatisticsFailedCount,

dot11STAStatisticsRetryCount,

dot11STAStatisticsMultipleRetryCount,

dot11STAStatisticsFrameDuplicateCount,

dot11STAStatisticsRTSSuccessCount,

dot11STAStatisticsRTSFailureCount,

dot11STAStatisticsACKFailureCount,

dot11STAStatisticsReceivedFragmentCount,

dot11STAStatisticsMulticastReceivedFrameCount,

dot11STAStatisticsFCSErrorCount,

dot11STAStatisticsTransmittedFrameCount,

dot11STAStatisticsAPServiceLoad,

dot11STAStatisticsAverageAccessDelayBestEffort,

dot11STAStatisticsAverageAccessDelayBackGround,

dot11STAStatisticsAverageAccessDelayVIdeo,

dot11STAStatisticsAverageAccessDelayVOice,

dot11STAStatisticsStationCount,

dot11STAStatisticsChannelUtilization,

dot11STAStatisticsRprtMeasurementMode,

dot11LCIReportIndex,

dot11LCIReportToken,

dot11LCIIfIndex,

dot11LCISTAAddress,

dot11LCILatitudeResolution,

dot11LCILatitude,

dot11LCILatitudeFraction,

dot11LCILongitudeResolution,

dot11LCILongitude,

dot11LCILongitudeFraction,

dot11LCIAltitudeType,

dot11LCIAltitudeResolution,

dot11LCIAltitude,

dot11LCIAltitudeFraction,

dot11LCIDatum,

dot11LCIRprtMeasurementMode,

dot11QoSMetricsRprtIndex,

dot11QoSMetricsRprtRqstToken,

dot11QoSMetricsRprtIfIndex,

dot11QoSMetricsMeasuringSTAAddr,

dot11QoSMetricsRprtActualStartTime,

dot11QoSMetricsRprtMeasurementDuration,

dot11QoSMetricsRprtPeerSTAAddress,

dot11QoSMetricsRprtTID,

dot11QoSMetricsRprtAverageQueueDelay,

dot11QoSMetricsRprtAverageTransmitDelay,

dot11QoSMetricsRprtTransmittedMSDUCount,

dot11QoSMetricsRprtMSDUDiscardedCount,

dot11QoSMetricsRprtMSDUFailedCount,

dot11QoSMetricsRprtMultipleRetryCount,

dot11QoSMetricsRprtCFPollsLostCount,

dot11QoSMetricsRprtBin0Range,

dot11QoSMetricsRprtDelayHistogram,

dot11QoSMetricsRprtMeasurementMode,

dot11QoSMetricsReportEntry }

STATUS current

DESCRIPTION

"The SMTRRMReport package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 36 }

dot11SMTRRMConfig OBJECT-GROUP

OBJECTS {   dot11APChannelReportIndex,

dot11APChannelReportIfIndex,

dot11APChannelReportRegulatoryClass,

dot11APChannelReportChannelList,

dot11RRMNeighborReportIndex,

dot11RRMNeighborReportIfIndex,

dot11RRMNeighborReportBSSID,

dot11RRMNeighborReportReachability,

dot11RRMNeighborReportSecurity,

dot11RRMNeighborReportCapSpectrumMgmt,

dot11RRMNeighborReportCapQoS,

dot11RRMNeighborReportCapAPSD,

dot11RRMNeighborReportCapRRM,

dot11RRMNeighborReportCapDelayBlockAck,

dot11RRMNeighborReportCapImmediateBlockAck,

dot11RRMNeighborReportKeyScope,

dot11RRMNeighborReportChannelNumber,

dot11RRMNeighborReportRegulatoryClass,

dot11RRMNeighborReportPhyType,

dot11RRMNeighborReportNeighborTSFInfo,

dot11RRMNeighborReportTSFOffsetFlag,

dot11RRMNeighborReportBeaconInterval,

dot11RRMNeighborReportSSID,

dot11RRMNeighborReportRowStatus }

STATUS current

DESCRIPTION

"The SMTRRMConfig package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 37 }

Annex I

## I.1 External regulatory references

Change the first paragraph as shown:

This annex and Annex J provide information and specifications for operation in many regulatory domains requirements extending the OFDM PHY specification in Clause 17 for operation in the USA, Europe, and the 4.9 GHz and 5 GHz bands in Japan. The OFDM system provides a WLAN with data payload communication capabilities of 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s in 20 MHz channels and 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s in 10 MHz channels.

Change the documents column in Table I.1 as shown:

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| Europe | European Conference of Postal and Telecommunications (CEPT) Administrations and its Electronic Communications Committee (ECC). Also, European Radiocommunications Office, European Telecommunications Standards Institute (ETSI) | ECC DEC (04) 08, ETSI EN 301 893, ETS 300-328 | CEPT |

Change Table I.2 as shown:

| Emissions limits set |  | USA | Europe |  | Japan |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4 ISM bands |  | FCC CFR47 [B8], Section 15.247 | Reserved ETS 300-328 |  | Reserved MPHPT EO Articles 7, 49.20 |  |

Insert a new row after the ninth Behavior limits set row, and change the last row of Table I.3 as shown:

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 10 ISM bands | FCC CFR47 [B8], Section 15.247 | ETS 300-328 | MPHPT EO Article 49.20 |
| 10 11-255 | Reserved | Reserved | Reserved |

Insert a new row in Table I.4 as shown:

| Frequency band (GHz) | USA (Maximum output power with up to 6 dBi antenna gain) (mW) | Europe (EIRP) |
| --- | --- | --- |
| 2.400-2.4835 | 1 000 with antenna gain per FCC 47 CFR 15.247 (b)(4)(ii)(iii) | 100 mW |

Insert a new second row in Table I.6 as shown:

| Frequency band (GHz) | Regulatory type | Japan |
| --- | --- | --- |
| 2.400-2.497 | Nomadic access, unlicensed | < 10 mW/MHz EIRP |

# Annex J

Country information element and regulatory classes

Change the title, change row 5, insert a new penultimate row and change the last row of Table J.1 as shown:

Table J.1—Regulatory classes for 4.9 Ghz and 5 GHz bands in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 20 | 149, 153, 157, 161, 165 | 1 000 | 4 | 10 |
| 12 | 2.407 | 25 | 1-11 | 1 000 | 4 | 10 |
| 1213-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change the title, insert a new row and change the last row of Table J.2 as shown:

Table J.2—Regulatory classes for 5 GHz bands in Europe

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 2.407 | 25 | 1-13 | 100 | 4 | 10 |
| 45-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change the title, insert two new rows and change the last row of Table J.3 as shown:

Table J.3—Regulatory classes for 4.9 GHz and 5 GHz bands in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (dBm) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 2.407 | 25 | 1-13 | 10 mW/MHz | 4 | 6, 10 |
| 31 | 2.414 | 25 | 14 | 10 mW/MHz | 4 | 6, 10 |
| 30 32-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |