IEEE P802.11
Wireless LANs

| Local Congestion Monitoring |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Junping Zhang | Huawei Technologies Co., Ltd. | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-135-9023-1349 | zhjunping@huawei.com |

Abstract

Proposed resolution text for Comments 66

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- This text provides a resolution to Comments 66

- CID: 66

Comment: Local Congestion Monitoring (Informative)

What are the criteria for sending a congestion control request or a neighborhood congestion announcement. There must be guidelines.

- Proposed Change:

- Resolution: Defer-RPS

Proposed resolution:

Insert the following at the end of clause11A.7.2 “Local Congestion Monitoring(Informative)” to P802.11s/ D0.03:

The following table indicate the criteria for sending a congestion control request or a neighbourhood congestion announcement

| STR_AC L_AC | Low | Medium | High |
| --- | --- | --- | --- |
| Low | Sending Congestion Control Request | NA | Sending Congestion Control Request |
| High | NA | Sending Neighborhood Congestion Announcement | Sending Neighborhood Congestion Announcement |

All the following description limited to a certain single AC in the period, the period is T(its value is related to the network condition)

a) L_AC denotes the queue size of  the corresponding AC,

The “Low” state denotes queue size is small, and the “High” state denotes queue size is lagrge

b) STR_AC denotes the difference between the receive rate and transmit rate of the corresponding AC

The “Low” state denotes receive rate is less than transmit rate

The “Medium” state denotes: receive rate nearly equals transmit rate

The “High” state denotes receive rate is more than transmit rate

The specific threshold for Low ,medium and  high is a local implementation mater and out of the scope of the standard.

- An example choice for the threold of  Low ,medium and high  is following.

- a) Short Term Transmitting Rate (STTR_AC): The rate is based on a statistical time period T.

STTR_AC = NtP_AC/T                           (1)

In Eq.1, Nt denotes the transmitted packets of the corresponding AC of the local node during the period; and P_AC denotes the packet size of the corresponding AC.

- b) Short Term Receiving Rate (STRR_AC): The rate is based on the statistical time period T.

STRR_AC = NrP_AC/T                          (2)

In Eq.2, Nr denotes the received packets of the corresponding AC of the local node during the period.

The normal distribution is choosed as the membership functions of STR_AC and L_AC. STR_AC includes three states: {“Low”, “Medium”, “High”}. L_AC includes two states:{“Low”, “High”}. The normal distribution formula as follws:

u(x)=e−k|x−a|2,                           (3)

- Table1: the experimental parameter of the distributition for STR_AC

| Low | k=0,x<0 |
| --- | --- |
| Medium | a=0, k=1.2 |
| High | a=7, k=0.1 |

- Table1: the experimental parameter of the distributition for L_AC

| Low | a=0, k=0.004 |
| --- | --- |
| High | a=41,k=0.06 |

At any time, the state of the STR_AC and L_AC should be the state which has the largest value according to Eq.1 , Table1 ans Table2

References: