IEEE P802.11
Wireless LANs

| Comment Resolutions for Subclause 21.3.12.3 (D1.03) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Ketchum | Qualcomm Incorporated | 9 Damonmill Square Ste 2A Concord MA 01451 | 781-276-0915 | johnk@qualcomm.com |
| Krishna Pillai | Wipro Technologies | #70/1234 & 84/1234, Keonics Electronic City, Bangalore, 560100 | +91-80-3029-534 | krishna.pillai@wipro.com |
| Deric Waters | Texas Instruments | 12500 TI blvd; Dallas, TX 75243 | 214-480-6026 | deric@ti.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 263, 5073, 5074, 5075, 5076, 5077, 970

- Resolution also effects 969

The changes marked in this document are based on TGn Draft version D1.03.

A supporting PowerPoint file reviews the reasons for the resolution given in this document:

11-06-1310-00-000n-backup-21-3-12-3-resoludtion.ppt

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

# CID 263, 5073, 5074, 5075, 5076, 5077, 970

TGn Editor:  Replace the text currently in 21.3.12.3 with the following text.

In response to the reception of an MCS Request (MRQ), sent by STA A to STA B, the responding STA B returns to the initiating STA A an MCS selection that the responding STA B determines to be a suitable MCS for the initiating STA A to use in subsequent transmissions to the responding STA B.  In determining the MCS, the responding STA B performs a channel quality assessment, which entails using whatever information the responding STA B has about the channel, such as an estimate of the MIMO channel derived from the sounding PPDU that carries the MRQ.  To enable this calculation, the MRQ should be sent in conjunction with a sounding PPDU (either in a sounding PPDU, or in a PPDU in which the NDP announcement bit in the HT Control field is set, and which is followed by an NDP.)

The STA sending the MRQ (STA A) determines how many HT-LTFs to send, and whether to use extension HT-LTFs or an NDP, based on the number of space time streams used in the PPDU carrying the MRQ, the number of transmit chains it is using (), whether or not the transmit and receive stations support STBC, and in some cases, the number of receive chains at the responding STA ().

The maximum number of available space-time streams is set by the number of transmit and receive chains, and the STBC capabilities of the transmitter and receiver, as is shown in Table nxx.  While the number of receive chains at a STA is not communicated in a capabilities indicator, the maximum number of space time streams supported may be inferred from the MCS capabilities and the STBC capabilities of the receiveing STA.  When the number of receive chains is known at the transmitter, the number of HT-LTFs sent to obtain a full channel quality assessment is determined according to the maximum number of space-time streams indicated in Table nxx. The number of HT-LTFs to use in conjunction with the indicated number of space time streams is determined according to 21.3.9.4.6 The HT-LTF long training field. (Ed:  update subclause reference)

Submission note: If a capability indication is added that restricts the  maximum number of space time streams to be trained, this may be used in addition to MCS capabilities and STBC capabilities to infer the maximum number of space time streams supported.

Table nxx:  Maximum available space-time streams

|  |  | without STBC | with STBC |
| --- | --- | --- | --- |
| 1 | ≥1 | 1 | N/A |
| 2 | ≥1 | min(,) | 2 |
| 3 | 1 | 1 | 2 |
| 3 | ≥2 | min(,) | 3 |
| ≥4 | 1 | 1 | 2 |
| ≥4 | ≥2 | min(,,4) | 4 |

If the initiating STA A sends an MRQ in a PPDU that uses fewer space time streams in the data portion than are available on the channel, the channel quality assessment made by responder STA B may be based on the Data HT-LTFs alone, but in this case the MCS feedback will be limited to MCSs using the number of streams used in the data portion of the PPDU, or fewer.  To determine whether an MCS should be chosen that uses more spatial streams than the PPDU containing the MRQ, it is necessary for the initiating STA A to either use extension HT-LTFs (send the MRQ in a staggered sounding PPDU), or use an NDP (send the MRQ in a PPDU in which the NDP announcement bit in the HT Control field is set, and which is followed by an NDP.)

References:  11-06-1310-01-000n-backup-21-3-12-3-resoludtion.ppt