IEEE P802.11
Wireless LANs

| Proposal to change the text in subclause 21.3.11.2.3 for compressed steering matrix feedback in TGn Draft D1.03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom Corporation | 190 Mathilda place Sunnyvale, CA 94086 | 1-408-543-3455 | joonsuk@broadcom.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 963, CID 3396, CID 3397, CID 3398, CID 4487, CID 4699, CID 4702, CID 7106, CID 7110,  CID 7111,  CID 7112, CID 7113,  CID 965, CID 967,  CID 11900, CID 11901, CID 11902

The changes marked in this document are based on TGn Draft version D1.03

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

When we agreed on the joint proposal at Hawaii, January 2006, the last minute change on this topic was agreed unanimously. However, due to the limited time to prepare for the contribution, the joint proposal document, i.e., 05-1095r5 and 05-1102r4, did not pick up the complete fixes in the text, which are mostly editorial changes. In order to make it clear, especially for the editorial point of view, we decided to have a submission to change the subclause 21.3.11.2.3, instead of changing words and equations line by line.

## CIDs to cover

CID 963, 3396, 3397, 3398, 4487, 4699: Editorial changes on the example of 4x2 V expression.

CID 4702: Rephrase the last sentence of the subclause.

CID 7106, 7110: Definition of 1_(i-1)

CID 7111, 7112, 7113, 965, 967, 11900, 11901: Editorial changes on the process of finding V matrix.

CID 11902: Refer to subclause 7.4.7.8 for quantization process of angles.

## Proposal for the change of subclause 21.3.11.2.3

21.3.11.2.3 Compressed Steering Matrix Feedback

In compressed steering matrix feedback, the receiving station computes a set of compressed unitary matrices for feedback to the transmitter.  These compressed matrices are assembled into an action frame as described in 7.4.8.10. In compressed steering matrix feedback, the receiving STA removes the CSD in table n70 from the measured channel before computing a set of matrices for feedback to the transmitter. These matrices are assembled into an action frame as described in 7.4.8.10. The transmitter can use these matrices to determine the steering matrices.

The transmitter can then apply these matrices, or a function of them, as the spatial mapping matrices. The matrix compression is defined as follows.  The unitary matrix V shall be represented as:

V=i=1min(Nc,Nr−1)Di1i−1ejφi,i...ejφNr−1,i1l=i+1NGliTψliINr×Nc,   <21-64>

where, Thethe matrixD1x1...xNr is an n by n Nr x Nr diagonal matrix with elements to xNron the diagonal.  The first i−1 elements of , e.g.,1i−1 in (21-64), and the last element of are all ones. The matrix  is an n by n Nr x Nr Givens rotation matrix:

Gli(ψ)=Ii−100000cos(ψ)0sin(ψ)000Il−i−1000−sin(ψ)0cos(ψ)00000IN−l

where each is an m x m identity matrix, and  and  are located at lth and ith row and column. INrxNcis an identity matrix padded with zeros to fill the additional rows or columns when Nr≠Nc.

For example, a 4x2 V matrix has the following representation:

V=ejφ110000ejφ210000ejφ3100001cosψ21sinψ2100−sinψ21cosψ210000100001Tcosψ310sinψ3100100−sinψ310cosψ3100001Tcosψ4100sinψ4101000010−sinψ4100cosψ41T10000ejφ220000ejφ320000110000cosψ32sinψ3200−sinψ32cosψ3200001T10000cosψ420sinψ4200100−sinψ420cosψ42T10010000

This is a description of a method for finding a compressed V matrix using Givens Rotation.

V=i=1min(M,N−1)Di1i−1ejφi,i...ejφN−1,i1l=i+1NGliTψliIN×M

The procedure of finding a compressed V matrix is decribed as follows.

A unitary NxM Nr x Nc beamforming matrix V is column-wise phase invariant because the steering matrix needs a reference in phase per each column. This means V may be equivalent to VD, where D is a column-wise phase shift matrix such as D=diag(ejθ1,.ejθ2,..,ejθNc). When the beamformee estimates the channel, it may find V for the beamforming matrix for the beamformer, but it may should send VD back to the beamformer, where V=VD. The angle, θi, in D is found to make the last row of VD to be non-negative real numbers.

The angles 1,1,  …, N-1,1φ1,1,...,φNr−1,1in the  diagonal matrix D1ejφ11...ejφNr−1,11 may be found to make the first column of D1Vto all be non-negative real numbers. Now, the first column of GNr1...G31G21D1V can be [1 0 … 0]T by Givens rotation Gl1‘s such as

cosψNr100sinψNr101000010−sinψNr100cosψNr1...cosψ310sinψ3100100−sinψ310cosψ3100001cosψ21sinψ2100−sinψ21cosψ210000100001ejφ110000⋱0000ejφN−1,100001V=1000000V2

For new (Nr-1) x (Nc-1) submatrix V2 ,  this process may be applied in the same way. Then, the angles 2,2,  …, N-1.2φ2,2,...,φNr−1,2in the  diagonal matrix D21ejφ22...ejφNr−1,21 may be found to make the first second column of V2D2×diag(1,V2)to be all non-negative real numbers. Now, the first two columns of GNr2...G32D2GNr1...G31G21D1V can be INrx2by Givens rotation Gl2‘s such as

10000cosψNr20sinψNr200100−sinψNr20cosψNr2...10000cosψ32sinψ3200−sinψ32cosψ320000110000ejφ220000ejφNr−1,200001GNr1...G31G21D1V=100100000000V3

This process may keep going until the first M Nc columns of right side matrix becomes INrxNc. When M < N Nc < Nr, this process does not need to continue because VM+1  VNc+1will be nulled out by  INrxNc. Then, by multiplying the complex conjugate transpose of products of series of Di‘s and Gli‘s on the left, V can be expressed as

V=i=1min(M,N−1)Di1i−1ejφi,i...ejφN−1,i1l=i+1NGliTψliIN×M,

V=D1G21TG31T...GNr1TD2G32TG42T...GNr2T...×DpGp+1,pTGp+2,pT...GNrpTINr×Nc,

wherep=min(Nc,Nr−1) , which can be written in the short form as in (21-64).

The angles found from the decomposition process above, e.g.,φ’s and ψ’s , shall be quantized as described in 7.4.8.10.

Columns 1..NSS of the steering matrix correspond to spatial streams 1..NSS, respectively. Spatial stream to modulation mapping is defined in the MCS Tables in 21.6. A transmitter shall not re-order the columns of the steering matrices.

## Note to the editor

The index for the equation “(21-64)” is a temporary number, which can be changed during the editorial process for the draft.