IEEE P802.11
Wireless LANs

| Proposal to change the text for the explicit feedback with sounding packet |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom | 190 Mathilda place Sunnyvale, CA 94086 | 1-408-543-3455 | joonsuk@broadcom.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | 1 858 521 5885 | verceg@broadcom.com |
| Rohit Nabar | Marvell | 5488 Marvell Lane, Santa Clara, CA 95054 | 1 408 222 0865 | rnabar@marvell.com |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |
| Gal Basson | Intel | POB 1659, Haifa 31015, Israel | 972547885404 | gal.basson@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- CID 968, CID 969, CID 1073, CID 4161, CID 11959, CID 947, CID 948, CID 949, CID 950, CID 951, CID 952, CID 954,  CID 1072, CID 4144, CID 10286, CID 973, CID 974, CID 975

The changes marked in this document are based on TGn Draft version D1.03

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Introduction

For CSI feedback, the channel response H to be fedback is not properly defined. In addition, if The spatial mapping matrix Qk used for sounding must be accounted for in the steering matrix computation for the subsequent beamformed transmission. This is missing in the current draft. We have a presentation, 06/1032r4, which describes the problem in more detail. With this contribution, we propose appropriate changes in the current text to accommodate the problems described in 06/1032r4.

## CIDs to cover

## CID 973, 974, 975, 11959:

TGn Editor:  Change the following text in 7.4.8.8 in the line 65, page 54 and Table n34 in page 55.

A CSI matrix H for a single carrier has the structure defined in Table n34. The encoding rules for the elements of the H matrix are given in 21.3.11.2.1.

A CSI matrix for the feedback, Heff,  is defined as a response from space-time stream CSD input at the transmitter to FFT output at the receiver, such that Heff = HQCcsd, where H is a frequency response from IFFT input at the transmitter to FFT output at the receiver. Heff for a single carrier has the structure defined in Table n34. The encoding rules for the elements of the Heff matrix are given in 21.3.11.2.1.

Change Table n34—Matrix coding  in clause  7.4.8.8, with the figure  as follows (Change H to Heff from submission 06/1367r0 which was already accepted)

For each subcarrier include { Carrier Matrix Amplitude of 3 bits For each of Nr rows in each CSI matrix in order: (1, …, Nr) { Include Nc complex coefficients of CSI matrix H Heff in order: (1, …, Nc ); each element of H Heff includes the real part of the element (Nb bits) and imaginary part of element (Nb bits) in that order } }

Figure nXX — Matrix coding

## CID 947:

TGn Editor:  Change the following text in 21.3.11 at the line 1, page 204.

Replace “a spatial mapping matrix Qk” with “a steering matrix Qk”.

## CID 948, 949, 950:

TGn Editor:  Replace the following text in 21.3.11 from the line 40 to line 46, page 204.

When beamforming is used, the transmitter replaces xk  with  Qkxk so that the received vector is

yk=HkQkxk+n (21-61)

There are several methods of beamforming, differing in the way the transmitter acquires the knowledge of

the channel matrices Hk and on whether the transmitter or the receiver generates Qk  .

When beamforming is used, the transmitter replaces xk  with  Qkxk  so that the received vector is

yk=HkQkxk+n                                                             (21-61)

The beamforming steering matrix that is computed (or updated) from a new channel measurement replaces the existing Qk for the next beamformed data transmission. There are several methods of beamforming, differing in the way the transmitter acquires the knowledge of the channel matrices and on whether the transmitter or receiver generates steering matrices.

## CID 951, 1072, 4144:

TGn Editor:  Replace the following text in 21.3.11.2 from the line 9 to line 11, page 205.

In explicit beamforming, in order for STA A to transmit a beamformed packet to STA B, STA B measures

the channel matrices and sends STA A either the mapping matrices Qk to use, or the channel matrices Hk.

In explicit beamforming, in order for STA A to transmit a beamformed packet to STA B, STA B measures the channel matrices and sends STA A either the effective channel Heff,k= HkQkCcsd for STA A to determine a steering matrix Qsteer_CSI,k = QkCcsdVk with Vk found from Heff,k, or the beamforming feedback matrix Vk for STA A to determine a steering matrix Qsteer_V,k = QkVk with Vk found from HkQk, where Qk is the orthonormal spatial mapping matrix which was used to transmit the sounding packet that elicited the Vk feedback. When new steering matrix Qsteer_CSI,k or Qsteer_V,k is found, Qsteer_CSI,k or Qsteer_V,k may replace Qk for the next beamformed data transmission.

## CID 952, 10286:

TGn Editor:  Replace the following text in 21.3.11.2.1 from the line 19 to line 21,  page 205.

In CSI Matrices feedback the transmitting STA receives the quantized MIMO channel matrix,H, from the

receiving STA. The transmitting STA then may use this matrix to compute a set of transmit steering matrices,Q.

In CSI Matrices feedback the transmitting STA receives the quantized MIMO channel matrix from the

receiving STA, where the MIMO channel matrix is defined a response from the transmitter space-time stream CSD input to the receiver FFT output. The transmitting STA then may use this matrix to compute a set of transmit steering matrices, Qk.

## CID 954:

TGn Editor:  Change the following text in 21.3.11.2.2 from the line 59 to line 62, page 205.

In non compressed steering matrix feedback, the receiving STA removes the space-time stream CSD in table n70 from the measured channel before computing a set of matrices computes a set of matrices for feedback to the transmitter. These matrices are assembled into an action frame as described in 7.4.8.9. The transmitter can then apply these matrices directly as the spatial mapping matrices Qk. The transmitter can use these matrices to determine the steering matrices Qk.

TGn Editor:  Change the following text in 21.3.11.2.3 (the first paragraph in 06/1334r4 which was already accepted).

In compressed steering matrix feedback, the receiving station computes a set of compressed unitary matrices for feedback to the transmitter. In compressed steering matrix feedback, the receiving STA removes the space-time stream CSD in table n70 from the measured channel before computing a set of matrices for feedback to the transmitter. These matrices are compressed in the form of angles, which the transmitter can use to de-compress the matrices and determine the steering matrices. These matrices are compressed in the form of angles which are assembled into an action frame as described in 7.4.8.10. The transmitter can use these angles to de-compress matrices and determine the steering matrices.

## CID 969:

TGn Editor:  Withdrawn

## CID 968, 1073, 4161:

TGn Editor:  Add the following text at the end of 21.3.12.3 in page 210.

The sounding PPDU may have non-identity spatial mapping matrix Qk. Over different RAs, Qk may vary.