IEEE P802.11
Wireless LANs

| LB84-CID-1250-text-proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-06-01 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | 1 408 543 3370 | mfischer@broadcom.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | 1 408 543 3316 | henryp@broadcom.com |

Abstract

This document includes proposed text for modification of the TGn 1.0 draft. The text is intended to provide a resolution for CID 1250, 6947, 496, 10395, 2391, 3987, 4759, 6776, 10299, 10400, 11991, and 1251 from LB84. These CIDs raise questions regarding the restriction in the use of security algorithms.

REVma 7.0 includes the following subclauses:

8.2 Pre-RSNA security methods

Except for Open System authentication, all pre-RSNA security mechanisms have been deprecated, as they fail to meet their security goals. New implementations should support pre-RSNA methods only to aid migration to RSNA methods.

8.3 RSNA data confidentiality protocols

8.3.1 Overview

NOTE—Use of any of the data confidentiality algorithms depends on local policies. The data confidentiality and integrity mechanisms of TKIP are not as robust as those of CCMP. TKIP is designed to operate within the hardware limitations of a broad class of pre-RSNA devices. TKIP is suitable for firmware-only, hardware-compatible upgrade of fielded equipment. RSNA devices should only use TKIP when communicating with devices that are unable or not configured to communicate using CCMP.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

CID 1250 (A), 6947 (C), 496 (A), 10395 (C), 2391 (C), 3987 (A), 4759 (A), 6776 (A), 10299 (A), 10400 (C), 11991 (A), 1251 (C)

TGn Editor: On page 72 of TGn draft 1.01, near line 4, delete the text beginning with “Insert the following new subclause 8.8 Security for HT STA” through and ending with the sentence which begins “An HT STA shall not use WEP”

TGn Editor: Insert, on page 72 of draft 1.01, near line 4, the following text and editor instructions:

8.2 Pre-RSNA Security methods

Insert the following text at the end of the clause:

The TKIP security mechanism was intended for short-term deployment pending the development of stronger ciphers. The development of the definition of the HT STA coincides with the existence of stronger ciphers and hence, HT STA encryption algorithm use is restricted.

It is noted that when keys are negotiated, only one pairwise key is negotiated for each pairing of STA. It is also noted that in order to insure the greatest possible security of exchanged data, each key may be utilized by a maximum of one encryption algorithm. These facts contribute to the definition of the restrictions on security algorithm usage.

The use of WEP and TKIP by an HT STA are strongly discouraged for communication with other STA which support stronger ciphers, and forbidden in HT STA to HT STA communication.

Note that frames exchanged between HT STA are not required to be encrypted.

Pre-RSNA security methods shall not be used to protect unicast frames transmitted by an HT STA if the RA or address1 of the frame corresponds to an HT STA.

TGn Editor: On page 36 of TGn draft 1.01, before line 56, which reads “7.3.2.30 TSPEC element”, insert the following text and editor insruction:

7.3.2.25 RSN information element

7.3.2.25.1 Cipher suites

Insert the following text at the end of the clause:

The cipher suite selectors “use group cipher suite” and “TKIP” shall never be used as pairwise cipher suite selectors between any two HT STAs.

TGn Editor: On page 72 of TGn draft 1.01, after line 2, which reads “8 Securityt”, insert the following text and editor insruction:

8.3 RSNA data confidentiality protocols

8.3.1 Overview

Remove the phrase “NOTE—“ from the start of the last paragraph of this subcluase.

Insert the following text at the end of the subclause:

TKIP shall not be used as the pairwise cipher suite between any two HT STAs.

TGn Editor: On page 72 of TGn draft 1.01 near line 1, after the header “8. Security, insert the following text and editor insruction:

8.3.3.3.5 CCM originator processing

Change item c) of the second paragraph of this subclause as follows:

c) Frame body: the frame body of the MPDU (1–7919 octets; 7919 = 7935 – 8 MIC octets – 8 CCMP

header octets).

References: