IEEE P802.11
Wireless LANs

| TGp Comment Resolution Annex A Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-6 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wayne Fisher | ARINC, Inc | 2551 Riva Road, Annapolis, MD 21401 | 410-266-4958 | wfisher@arinc.com |

Abstract

In response to the many comments received on P802.11p D1.0 this document presents a draft of changes to Annex A.  This document presents a draft of these changes and is provided for discussion during the TGp teleconferences.

Note, this draft of changes to Annex A removes all references to Clause 20 which have been incorporated into Clause 17 and Annexes I and J.  The Transmit Specifications and Receiver Specifications and data rates for WAVE are addressed by the 10 MHz channel specifications already defined in Clause 17.  Only a few additional PICS are required to identify the 5.9 GHz band for WAVE and the additional temperature range to support devices operating in the automotive environment.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

|  |  |  |  | LB81 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| Multiple | Multiple | Annex A | Wayne Fisher |  | September 6, 2006 |

- COMMENT:  [From Spreadsheet]

In response to comments received for Clause 20.

- Commenter’s Suggested Remedy (If appropriate):  [From Spreadsheet]

Not included here. (Not directly applicable.) Note, many comments were incorporated into the draft for changes to Clause 20, Annex I, and Annex J.

- Background, Explanation, Discussion, etc.:

In response to the many comments received on Clause 20 of P802.11p D1.0 and the resulting changes to Clause 17, Annex I, and Annex J, section 4 presents a draft of changes to Annex A as a result of these changes.

- Recommended Resolution of the Comment:

A draft of changes/additions to Annex A is provided:

# Annex A

(normative)

# Protocol Implementation Conformance Statement (PICS) proforma

# A.4 PCIS proforma—IEEE Std 802.11™, 2006 Edition

# A.4.3  IUT Configuration

Insert the following into A.4.3, renumbering as necessary:

| Item | IUT Configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF6A | OFDM PHY for the 5.9 GHz band | ─ | O.2 | Yes □ No □ |

# A.4.8 OFDM PHY function

Insert the following into A.4.8, renumbering as necessary:

| Item | Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
| OF3: PMD Operating Specification General |  |  |  |  |
| *OF3.2.6 | 5.85─5.925 GHz band | Annex J | O.1 | Yes □ No □ |
| OF3.3.10A | 5.9 GHz band (10 MHz channel spacing) | Annex J | CF11 & OF3.2.6 & OF1.7:M | Yes □ No □ |
| OF3.10.4 | Type 4 (-40 °C to 85 °C) | 17.3.8.8 | O | Yes □ No □ |
| OF4: PMD Transmit Specification |  |  |  |  |
| OF4.1.4 | Power level (5.85-5.925 GHz) | 17.3.9.1 | OF3.3.10A:M | Yes □ No □ |

Some notes relating to recent updates to Annex A PICS for WAVE:

RE: OF4: PMD Transmit Specification

Editor Note:  WAVE power levels now defined within 17.3.9.1 and Annex J.

RE:  OF5: PMD Receiver Specifications

Editor Note:  WAVE Receiver Specifications track P802.11ma references but use a different table within that reference.

RE:  OF9: OFDM PMD Sublayer

Editor Note:  Table 152, Tx Power Levels expanded to 64 levels.  No change to references.

RE:  Regulatory Domain Extensions (A.4.10)

Editor Note:  A.4.10 applies to WAVE.  Including: MD1 (Country Information element) and MD12 (Regulatory and Coverage classes).  No changes for WAVE identified.

RE:  Regulatory Classes Extensions (A.4.13)

Editor Note:  A.4.13 applies to WAVE.  Including: RC1 (Regulatory and Coverage classes) and RC3 (Regulatory and Coverage classes (10 MHz channel spacing).  No changes for WAVE identified.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to accept the draft Annex A changes as defined in section 4 of this document (1357r0)

And instruct the editor to incorporate these changes into a new draft P802.11p amendment as appropriate.

Motion by: ____________________Date: _________________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |