IEEE P802.11
Wireless LANs

| TGn LB84 Submission Related to MAC comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 7350, 65, 1485, 4529, 7701, 7702, 7703, 7704, 10029, 414, 1803, 3623, 1808, 4047, 4048, 7465, 11994, 5130, 8110, 11945, 11947, 11948, 11949, 1977, 11082, 7229, 11952, 850, 1153, 2237, 11954, 2550, 988, 2551, 1298, 1324, 2553, 4079, 7472, 780, 7610, 745, 1315, 9984, 2657, 2658, 549, 3888, 1, 7874, 509, 2750, 437, 4009, 4010, 2751, 4011, 2752, 2753, 438, 4012, 4008, 2756, 507, 508, 10343, 1001, 2766, 4013, 9884, 1002, 2767, 10574, 1003, 3374, 5123, 1004, 1005, 7476, 7477, 837, 2770, 3600, 10577, 2772, 2773, 2774, 3375, 1010, 3606, 5125, 1016, 1015, 7497, 9895, 7499, 441, 840, 1020, 1021, 1022, 7042, 1023, 4016, 12007, 1025, 3613, 1053, 1054, 2855, 10519, 7489, 5120, 10713, 10712, 11992, 10300, 10314, 10316, 10317, 11909, 11910, 10321, 10322, 10328, 10329, 11871, 3152, 11787, 108, 1557, 804, 7028, 7341, 10364, 59, 60

These comments were selected as those that had not yet been decided in the MAC ad-hoc, and would probably be considered as uncontroversial.

The changes marked in this document are based on TGn Draft version D1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Proposed Resolutions

| CID | Clause(Ed) | Comment | Proposed Change | Resolution | APS Proposed Resolution |
| --- | --- | --- | --- | --- | --- |
| 7350 | 5.2.7.2 | This table contains a number of unclear shalls, making an informative section normative. What does "shall receive" mean? | Remove table | Transfer MAC AdHoc group | Proposed accept. This has already been actioned in D1.03 in approved submission 11-06/755r2. |
| 65 | General | Aggregated PPDU is A-MPDU. | Change "aggregated PPDU" to "A-MPDU". Also "non-aggregated PPDU" should be changed to other appropriate word, such as "single MPDU". | Transfer to MAC AdHoc Group | Proposed accept. |
| 1485 | General | there seem to be quite a few instances of the use of the term "frame" which should really usually be "MPDU" | find suspect uses of "frame" and replace with "MPDU" or "A-MPDU" or "PSDU" or "MSDU" or whatever, as appropriate | Transfer to MAC AdHoc Group | Proposed reject. The usage in this amendment follows the baseline. Frame and MPDU are synonyms. Frame tends to be used in clause 7. MPDU and frame are used elsewhere without distinction. There is no point adopting any alternative convention in 802.11n unless we also fix up the thousands of uses in the baseline. |
| 4529 | General | What is an aggregate PPDU? I think you mean MPDU | replace "PPDU" with "MPDU" | General AdHoc: Transfer to MAC Group See CID 65 | Proposed counter. See resolution to CID 1485. |
| 7701 | General | The MIMO power save doesn't say how to handle DLS links. | Either disable MIMO power-save mode while a DLS link is set up or require the DLS peer to notify all its DLS partners using a reliable exchange. | Gen AdHoc: Transfer to MAC | Proposed accept. Add the following to D1.03 11.2.3.1: "A STA that has one or more DLS links shall notify all STA with which it has a DLS link of any change in SM power save mode before operating in that mode." |
| 7702 | General | For DLS links it is not clear if a single DLS link can act bi-directionally - i.e. support RD. We need the keys to be established before data can be exchanged. | Indicate that RD should only be offered on a DLS link if a DLS link has also been created in the reverse direction between the two peers. | Gen AdHoc: Transfer to MAC | Proposed reject. It should be up to the transmitter of a frame not to transmit a frame until all appropriate security matters have been completed. This is how it works with non-RD data. A STA that is offered RD and has not yet established an appropriate security context can choose to transmit unencrypted data, or no data at all. We don't need to do anything additional to support this choice. |
| 7703 | General | A-MSDU or MSDU. Although the draft has updated some sections that use the term MSDU to "A-MSDU or MSDU", a thorough review is necessary to make sure we've caught them all. | Perform a review of the baseline documents looking for MSDU and determine if it refers to an MSDU (related to LLC interface) or an "A-MSDU or MSDU", which is more or less everything else. | General AdHoc: Transfer to MAC Group | Proposed accept. See submission 11-06-1316-00-000n-lb84-submission-related-to-frame-mpdu-and-msdu-terminology.doc. |
| 7704 | General | A-MPDU or MPDU. Although the draft has updated some sections that use the term MPDU to "A-MPDU or MPDU", a thorough review is necessary to make sure we've caught them all. | Perform a review of the baseline documents looking for MPDU or "frame" and determine if it refers to an MPDU or an "A-MPDU or MPDU". | SEE CID: 7703 | Proposed accept. See submission 11-06-1316-00-000n-lb84-submission-related-to-frame-mpdu-and-msdu-terminology.doc. |
| 10029 | General | Naming of blockAck flavors to N-… seems a little bit odd. Need better names for blockAck features. |  | Gen AdHoc: No specific names suggested. Current names were chosen by the majority. The Use of "N-" is a concern. N-Immediate Block Ack and N-Delayed Block Ack are the names in the current draft. Transfer to the MAC Group to remove the "N-" in a consistent and Technically correct way. | Proposed counter. Other editorial changes have changed the N- to HT- in D1.03. |
| 414 | 3 | It is a bit ambiguous to have a definition for just the term 'aggregate' particularly as there is both MPDU and MSDU aggregation. | Suggest changing this term to Aggregate PSDU to clarify. | Gen AdHoc: We believe that this should be changed, but want the MAC group to ensure that the use is consistent and the definition is correct. Transfer to the MAC Group. | Proposed counter. The D1.03 draft contains definitions for both "aggregate MSDU (A-MSDU)" and "aggregate MPDU (A-MPDU)". Instruct the editor to replace any uses of the word "aggregate" with one of these two terms. |
| 1803 | 3 | Term "Initiator" has many uses other than the one in this definition | Use a more specific term than the general one chosen | Gen AdHoc: Initiator vs initiator is being used in the draft. The Capital version should always match the definition, and the lower case should be the generic use, Transfer to the MAC group to ensure consistency. | Proposed assign to Yuichi, who has a related submission. |
| 3623 | 3 | The concept of "Long NAV" is alreay present in the base standard. The definition appears to be unnecessary. | Either remove it or rewrite the parts of the base standard which refer to this mechanism. | Ed: reclassified as technical Gen AdHoc:Transfer to MAC | Proposed reject. What's new about LongNAV is deliberate overestimation and truncation. This is not present in the baseline. |
| 1808 | 3 | Term "Responder" has many uses other than the one in this definition | Use a more specific term than the general one chosen | Gen AdHoc: Responder vs responder is being used in the draft. The Capital version should always match the definition, and the lower case should be the generic use, Transfer to the MAC group to ensure consistency. | Proposed assign to Yuichi, who has a related submission. |
| 4047 | 7.1.3.1.10 | "The HT Control Field may be included in any frame except a non-QoS Data frame." implies that this may be used in control or management frames. I believe that this is not the intention. | "The HT Control Field may be included only in QoS Data frames." If the intention is really to use it also in control or management frames, a bunch of further changes in the corresponding sections would be required. |  | Proposed reject. The intention (and the effect of the current draft) is to allow Control+HTC and Management+HTC frames. |
| 4048 | 7.1.3.1.10 | The obvious overloading of the Order bit in the QoS Data frames of type +HTC must be made more explicit | Verbage to the order of : The order bit is set to 0 in QoS Data frames, hence the setting of the order bit to 1 in the +HTC frames distinguishes a QoS Data frame that is not +HTC from one that is of the type +HTC. |  | Proposed counter. Accept the new text, but precede by "NOTE-" as it doesn't contain any normative specification. |
| 7465 | 7.1.3.1.10 | "The HT Control Field may be included in any frame except a non-QoS Data frame." implies that this may be used in control or management frames. This shoud not be the the intention. | "The HT Control Field may be included only in QoS Data frames." | A MAC question | Proposed reject. The intention (and the effect of the current draft) is to allow Control+HTC and Management+HTC frames. |
| 11994 | 7.1.3.2 | Third party shall respect the +HTC frames | Insert the following at the end of the sub clause: "All HT STA shall update their NAV settings using the duration value of the +HTC frames as appropriate under the coordination function rules " | MAC - to decide on the text to be included in clause 9 | Proposed counter. The D1.03 draft includes the following text: "An HT STA that does not support +HTC shall decode the Order field (see 7.1.3.1.10 (Order field)) of the Frame control field and perform the CRC on the extended length of the MPDU in order to properly respect any Duration/ID field setting." |
| 5130 | 7.1.3.5.3 | in table 6. The ack for frame appearing in ULT under MTBA/PSMP is to be received in THE subsequent DLT | replace "in a subsequent DLT" by "in the subsequent DLT" | A MAC question | Proposed reject. The requested change places unnecessary constraints on the scheduler within the AP. It is already aware of any QoS requirements and can schedule MTBA transmissions in a timely fashion without demanding they always come next. |
| 8110 | 7.1.3.5.3 | The 0-1 case has been overloaded to represent two different things, however there is no text here to indicate how you know "when" to use each. i.e in this clause there is no description of when the 0-1 Ack Policy is used for "No explicit acknowledgement" and when it is used for "scheduled acknowledgement under MTBA/PSMP agreement". Does this imply that there is some state information present that dictates when each of the two are used? Or does it depend on the whether it is in an A-MPDU or A-MSDU? | Add text to clarify what one should base the decision of which use of this Ack Policy mode is meant. Perhaps some forward references would be warranted. |  | Proposed accept. The two cases can be distinguished by looking at bit 6 of the frame control field (the "Null data" bit). Related to D1.03, change the "meaning" field for the 0-1 case as follows: 1. Replace "When used for "no explicit acknowledgement" with "When bit 6 of the frame control field is set to 1" 2. Replace "When used for scheduled acknowledgement ... agreement" with "When bit 6 of the frame control field is set to 0." |
| 11945 | 7.2.1.7 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. | Ed: reclassified as technical | Proposed accept |
| 11947 | 7.2.1.7.2 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. It is reserved under N-Immediate BlockAck. | Ed: reclassified as technical | Proposed accept |
| 11948 | 7.2.1.7.3 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. It is reserved under N-Immediate BlockAck. | Ed: reclassified as technical | Proposed accept |
| 11949 | 7.2.1.8 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. | Ed: reclassified as technical | Proposed accept |
| 1977 | 7.2.1.8 | "MTID" and "Compressed BA" would be better combined into a single 2-bit field, especially since "Compressed BA" by itself doesn't always mean Compressed BA | Make change indicated in comment | Ed: reclassified as technical | Proposed counter. The commenter is confusing the "Compressed BA" field with the Compressed BlockAck frame format. In order to reduce the possibility of confusion, rename the "Compressed BA" field as "Compressed Bitmap" |
| 11082 | 7.2.1.8 | "MTID" and "Compressed BA" would be better combined into a single 2-bit field, especially since "Compressed BA" by itself doesn't always mean Compressed BA | As in comment | Ed: reclassified as technical | Proposed duplicate of 1977 |
| 7229 | 7.2.1.8.1 | The statement "The Fragment Number subfield is always set to 0." should be eliminated. The Simply BlockAck case is the original 11.e BlockAck and Fragment numbers maybe used. | Fix it. | Ed: this is technical, transfer to MAC | Proposed accept |
| 11952 | 7.2.1.8.3 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. It is reserved under N-Immediate BlockAck. | Ed: reclassified as technical | Proposed accept |
| 850 | 7.2.2.1 | In clause 7.2.2.1 A-MSDU, page 31, line 6, it states "The A-MSDU lifetime is defined by the maximum lifetime of its constituent MSDUs. An A-MSDU may be transmitted until its lifetime expires or it is received correctly at the receiver...". This section is for A-MSDU definition, and "lifetime" is not shown as a field of an A-MSDU. Therefore the text is out of position. | Create a new section in clause 9 discussing the access rules for a station using A-MSDU including the lifetime calculation for A-MSDU. Additionally, specify the access rules when an A-MSDU aggregates MSDUs with different user priorities (or mapping to access category at MAC). | There is clearly a need for creating a sub-section in clause 9 to respond to numerous comments on the same issues | D1.03 9.7b has been introduced. It would be a suitable home for this subsection. The lifetime calculation could be moved there. Regarding the channel access rules, they are easy to specify as follows: "An A-MSDU is composed of MSDUs with the same TID value. The channel access rules for a QoS Data MPDU carrying an A-MSDU (or fragment thereof) are the same as a Data MPDU carrying an MSDU (or fragment thereof) of the same TID." |
| 1153 | 7.2.3.1 | Use of extension channel element in IBSS is problematic. As membership changes, what happens to extension channel element? If interference shows up, how is a migration to another channel effected? Do the basic aspects of the BSS need to be stable, such that if the original, oldest TSF STA leaves the IBSS, then is a new STA the new determinant of the characteristics of the IBSS? OR because of beacon information adoption, will all of the lesser-TSF STA already have the identical information, so that the loss of the oldest-TSF STA will not cause a change in the parameters of the IBSS? What if some STA other than the oldest TSF-STA decides that the IBSS needs to move the extension channel? I guess this is simply not done? | Not sure how to address these problems. |  | Proposed reject. In reply to the commenter: Nothing additional needs to be specified in TGn related to this. An IBSS either supports spectrum management or it doesn't. If it does, all STA in the IBSS are capable of operating as a DFS owner. A single STA - the DFS owner - makes decisions about any channel switching and advertises these using the Channel Switch action frame. If the IBSS doesn't support spectrum management, then its parameters (primary and secondary channels) are fixed, established when the first STA starts beaconing in the BSS. Note, a STA can tell its peers to transmit to it using 20MHz only, even in a 20/40MHz BSS. And this would be an appropriate response if the STA became aware that the secondary channel was no longer suitable. |
| 2237 | 7.4.7.2 | The frame formats would be much better shown as a Figure than a table | Change to a Figure showing sizes of each field, and a short paragraph below with the information currently in the "Value" column | Related to 2231 | Proposed counter. A tabular notation is used in the baseline for management frames and the QoS action frames. Propose that the "name" and "value" (D1.03) columns be removed. Any any normative specification in these columns be placed after the table. (This makes them look just like the QoS Action frame format). Note, other changes proposed in Frame move the definition of fixed fields or structures out of the action frame definition, this simplifies the task of making this change as most of the normative specification will have been removed. For consistency, perform the same operation in all HT action frame subclauses. |
| 11954 | 9.2.3.5 | Meaning is not clear. Needs some rewording. |  | Ed: reclassified as technical. | Proposed accept. D1.03 incorporates changes according to CID 1422 and CID 7653, which may provide the requested clarification. |
| 2550 | 9.13 | Suggest reorganizing this text | 9.13 is generally about protection schemes. Change title of 9.13 to be just that (Protection mechanisms). Add 9.13.1 "Overview of protection mechanisms" with a few sentences about what the problem is that they are trying to solve. Like the first few sentences currently in P802.11REV-ma-D6.0 9.13, except that its too specific to NAV. Then 9.13.2 "Protection mechanisms for non-ERP receivers" with current text. Then 9.13.3 "Protection mechanisms for different HT PHY options", 9.13.4 "L-SIG TXOP protection", 9.13.5 "Protection mechamisms for Aggregation Exchange sequences" | COEX Deferred then passed to MAC for later submission | Proposed counter. A similar structure was adopted in D1.03 (see submission 11-06-0962r0). |
| 988 | 9.15 | L-SIG TXOP protection is not defined | Define L-SIG TXOP protection | COEX Deferred then passed to MAC for later submission | Proposed accept. D1.03 includes the following definition: "3.n22 non-HT SIGNAL field transmit opportunity protection (L-SIG TXOP): A protection mechanism in which protection is established by the non-HT SIG Length and Rate fields indicating a duration that is longer than the duration of the packet itself." |
| 2551 | 9.14.1 | If 9.14 is not combined with 9.13, then the notion of "protection" needs some clarification | Add text defining the notion of "protection" at the start of 9.14 | COEX Deferred then passed to MAC for later submission | Proposed reject. 9.13 now includes 9.14, and the introductory text in 9.13 suffices: "9.13.1 Introduction These protection mechanisms ensure that a STA defers transmission for a period of time. These mechanisms are used to insure that non-ERP STAs do not interfere with ERP-frame exchanges between ERP STAs and that non-HT STA do not interfere with HT-frame exchanges between HT STAs. Thereby, allowing non-ERP and/or non-HT STAs to coexist with ERP and/or HT STAs." |
| 1298 | 9.14.2 | vague terminology | replace the sentence "All STAs in the BSS shall protect Green Field PPDUs when there is at least one non-HT or non-GF STA associated with this BSS." with "A STA which is associated with a BSS shall protect Green Field PPDUs when there is at least one non-HT or non-GF STA associated with that BSS." | COEX Deferred then passed to MAC: for later submission and discussion | Proposed counter: "A STA that is associated with a BSS shall protect Green Field PPDUs when its AP transmits an HT Information element with the Non-Greenfield STAs Present field set to 1." |
| 1324 | 9.14.2 | "when there is at least one non-HT or non-GF STA associated with this BSS": How would an individual STA know that? | Specify that the AP has to set a certain field in an HT IE to indicate the condition and that the STA has to obey what is indicated in this field. | MAC - this comment is NOT correct! It does not belong to this author! Probably actually from Lusheng Ji or Harry Worstell. COEX Deferred then passed to MAC: for later submission and discussion | Ed: NOTE. This comment is corrupted. Mat is recovering it. |
| 2553 | 9.14.2 | In addition to stating that Green Field PPDUs shall be protected, the text needs to state how this is done. | State how Green Field PPDUs are protected, or provide a cross reference to the clause where this is specified. | COEX Deferred then passed to MAC: for later submission and discussion | Proposed accept. After "shall protect Greenfield PPDUs" add "using any of the protection mechanisms described in 9.13.15.1 <D1.03 reference: Protection mechanisms for A-MPDU… / Overview>". |
| 4079 | 9.14.2 | "when there is at least one non-HT or non-GF STA 9 associated with this BSS": How can an individual STA know that? | Specify that the AP has to set a certain field in an HT IE to indicate the condition and that the STA has to obey what is indicated in this field. | COEX Deferred then passed to MAC: for later submission and discussion | Proposed accept. Replace: "... when there is at least one non-HT or non-GF STA associated with this BSS." with "... when the Non-Greenfield STAs Present subfield of the HT Information Element transmitted by its AP is set to 1". Proposal NOTE: the MAC ad-hoc may want to consider requiring that the "Non-Greenfield STAs Present" be set to 1 in beacons transmitted in an IBSS. Alternatively, we may have to specify rules here about how to interpret this field in an IBSS if we see some beacons with it and others without it. |
| 7472 | 9.14.2 | "when there is at least one non-HT or non-GF STA associated with this BSS": How would an individual STA know that? | Specify that the AP has to set a certain field in an HT IE to indicate the condition and that the STA has to obey what is indicated in this field. | COEX Deferred then passed to MAC: for later submission and discussion | Proposed duplicate of 4079 |
| 780 | 9.14.2 | self conflicting. | how can you have a green field if a non-HT STA is present? Need green field defintion. | COEX Deferred then passed to MAC: for later submission and discussion | Proposed counter. Greenfield defines a PPDU format, not a mode of operation of the BSS. The signalling added in CID 4079 addresses the condition under which greenfield PPDUs should be protected. |
| 7610 | 9.14.2 | "All STAs in the BSS shall protect Green Field PPDUs when there is at least one non-HT or non-GF STA associated with this BSS." This needs to be related to signalling in the beacon. | Replace with "All STAs in the BSS shall protect Green Field PPDUs when the Non-GF Devices Present subfield of the Additional HT Informtiaon Elements element is set to 1". | COEX Deferred then passed to MAC: for later submission and discussion | Proposed counter, see 4079. |
| 745 | 9.15 | The last sentance in the first paragraph on p106 is very confusing and seems to be contradictory. | Reword or remove sentance | COEX Deferred then passed to MAC with following suggested text for replacing the referenced sentence: An L-SIG duration is defined as the duration contained in an L-SIG field provided the duration in the L-SIG field does not represent the actual duration of the frame. "Note: need to clarify whether the L-SIG field is necessarily greater than the frame duration. | Proposed accept. Roughly half the paragraph has been reworded by prior resolutions in D1.03 thus: "An HT STA that asserted the L-SIG TXOP Protection Support field upon association that receives an L-SIG protected PPDU containing valid L-SIG Parity and HT-SIG CRC fields and that contains no valid MPDU from which a Duration/ID value can be determined shall, at the end of the PPDU, update its NAV to a value equal to L-SIG duration - HT-SIG duration.This NAV update operation takes place at the termination of the time/length value represented in the HT-SIG field." |
| 1315 | 9.15.2 | missing information | at the end of the sentence, add "through the end of the current L-SIG TXOP." | COEX Deferred then passed to MAC: for later submission and discussion by Matt Fischer | Proposed counter. There is no such thing as an L-SIG TXOP, only an L-SIG duration. The sentence has been clarified in D1.03 thus: "After transmission of an L-SIG TXOP Protection initial response frame, the responder’s HT Mixed Mode PPDU transmissions shall contain an L-SIG Duration that extends to the endpoint indicated by the MAC Duration/ID field." |
| 9984 | 9.16.3 | "Each sequence may include multiple PPDUs sent and received" This sentence is incomplete. There could be multiple receivers that the initiator sends/receives PPDUs to/from. If this is the case, then, very likely the AP is the owner of the TXOP. Hence, only AP should send the CF-End frame, because it will be hard to decide as to which STA should transmit the CF-End under this scenario. Again, the problem of unfairness and setting of NAV in OBSS is obvious in this case. | Change it to: "Each sequence may include multiple PPDUs of the same AC sent and received with the same or different STAs. Under this scenario, only the AP sends a CF-End frame to truncate the TXOP". | COEX Deferred then passed to MAC: later submission and discussion. Note: not clear whether the comment relates to the specific example or a general requirement. | Proposed counter. The sentence is intended to be informative, so may is inappropriate. It is unclear what the normative intent of "only the AP sends" is, and the statement is not true as a non-AP may validly want to truncate an EDCA TXOP. Editor replace: "Each sequence may include multiple PPDUs sent and received" with "Each sequence might include multiple PPDUs of the same AC sent and received" |
| 2657 | 9.2 | Beam forming should only be used for unicast frames. | Specify what frames may use beam forming. |  | Proposed reject. 9.20.1 states: "In order for a transmitting STA to calculate an appropriate steering matrix for transmit spatial processing when transmitting to a specific receiving STA, the transmitting STA needs to have an accurate estimate of the channel that it is transmitting over." However, if an implementation found some way of using the mechanisms described here to determine the channel accurately for all STA in its BSS, and could find a way to use those channels to determine a beamsteering matrix that improved reception at those STA, why should the standard seek to prohibit that? |
| 2658 | 9.2 | It is not clear how to invoke Mac protection for transmissions using beam forming. MAC protections such as RTS/CTS and beacon initiated CFP all designed for omni-directional data transmissions since the protection frames are broadcast. With beam forming, area covered by MAC protection mechanism is no longer the same as data transmission, hence no longer effective. | Either take beam forming out of this specification, or provide a reasonable solution such as directional MAC to solve this problem. |  | Proposed reject. The purpose of beamforming is not to extend range (which is limited by omidirectional RTS/CTS and signal field transmission), but to increase rate at range. The range of the BSS is limited by the omnidirectional transmissions, and this is not changed by the existence of beamforming. Current systems successfully cope with a disparity of protection range (RTS/CTS) and data range. |
| 549 | 9.2 | Throughout the section the description of transmit beamforming switches between AP-to-STA terminology and STA-to-STA terminology. To avoid confusion, consistent terminology should be used throughout the section. | Consistently describe beamforming examples and procedures between "STA A" and "STA B" in the text and figures of this section. Replace instances of "AP" and "Client". | Should be transferred to the technical adhoc. | Proposed accept. |
| 3888 | 9.23.6 | I did read 9.23.8 and still not sure what non-HT duplicated (a new term is). Please clarify | Please clarify | COEX Deferred then passed to MAC for later submission and discussion | Proposed accept. The terminology now used is non-HT duplicate, and a definition has been added to clause 3. |
| 1 | 10.3 | To use HT functions in direct link, information of HT Capabilities Information field in the DLS Request and Response frames should be indicated to the SME. | Add clauses 10.3.25.2 (MLME-DLS.confirm) and 10.3.25.3 (MLME-DLS.indication) and add HT Capabilities to those primitive parameters. |  | Proposed accept. |
| 7874 | 10.3 | There is no way to use HT mode in direct link. | Add specification for DLS. |  | Proposed accept. Covered by CID 1. |
| 509 | 10.3.2.2.2 | I believe that this description must remain accurate whether a device is HT or not. So mandating this here for non HT devices seems unnecessary. | Make presence contingent on some MIB variable or capability. |  | Proposed accept. See CID 2750 |
| 2750 | 10.3.2.2.2 | Addition to an existing MLME interface must be conditional on a MIB variable or equivalent | Add that these fields are only present if dot11HTCapabilityImplemented is set true |  | Proposed accept. Add the following to the Description for both HT elements: "The parameter shall be present only if the MIB attribute dot11HighThroughputOptionImplemented is true." |
| 437 | 10.3.6.3.2 | "The parameter may be present only if the MIB attribute dot11HTCapabilityImplemented is true." | "The parameter shall be present only if the MIB attribute dot11HTCapabilityImplemented is true." |  | Proposed counter. Note this relates to clause 10.3.6.1.2. "The parameter shall be present only if the MIB attribute dot11HighThroughputOptionImplemented is true." |
| 4009 | 10.3.6.1.2 | dot11HTCapabilityImplemented not defined in MIB | Replace with HighThroughputOptionImplemented/ or define new attribute |  | Proposed accept. |
| 4010 | 10.3.6.3.2 | dot11HTCapabilityImplemented not defined in MIB | Replace with HighThroughputOptionImplemented/ or define new attribute |  | Proposed accept. |
| 2751 | 10.3.7.1.2 | missing several parameters added by 11r | track 11r |  | Proposed accept. The D1.03 is up to date with respect to 802.11r D2.1. |
| 4011 | 10.3.7.1.2 | dot11HTCapabilityImplemented not defined in MIB | Replace with HighThroughputOptionImplemented/ or define new attribute |  | Proposed accept. |
| 2752 | 10.3.7.2.2 | missing several parameters added by 11r | track 11r |  | Proposed accept. The D1.03 is up to date with respect to 802.11r D2.1. |
| 2753 | 10.3.7.3.2 | missing several parameters added by 11r | track 11r |  | Proposed accept. The D1.03 is up to date with respect to 802.11r D2.1. |
| 438 | 10.3.7.3.2 | "The parameter may be present only if the MIB attribute dot11HTCapabilityImplemented is true." | "The parameter shall be present only if the MIB attribute dot11HTCapabilityImplemented is true." |  | Proposed reject. This parameter will be absent when associating with a non-HT AP. |
| 4012 | 10.3.7.3.2 | dot11HTCapabilityImplemented not defined in MIB | Replace with HighThroughputOptionImplemented/ or define new attribute |  | Proposed accept. |
| 4008 | 10.3.10.1.2 | dot11HTCapabilityImplemented not defined in MIB | Replace with HighThroughputOptionImplemented/ or define new attribute |  | Proposed accept. |
| 2756 | 10.3.15.1.2 | Table at line 10 of this page belongs in 10.3.15.1.2 | Move table to 10.3.15.1.2 |  | Proposed accept. |
| 507 | 10.3.15.3.1 | I believe that this description must remain accurate whether a device is HT or not. So mandating this here for non HT devices seems unnecessary. | Make presence contingent on some MIB variable or capability. |  | Proposed counter. Replace: "The 20/40 capable BSS/IBSS shall insert this parameter." with "Present only when the BSS is operating in 20/40 mode." Also, for consistency make the same change in 10.3.15.4.2. Also, for consistency replace "The 20/40 capable BSS/IBSS shall insert this parameter." in 10.3.15.1 with "Shall be present when the BSS is operating in 20/40 mode." |
| 508 | 10.3.15.4.2 | I believe that this description must remain accurate whether a device is HT or not. So mandating this here for non HT devices seems unnecessary. | Make presence contingent on some MIB variable or capability. |  | Proposed counter. See resolution to CID 507. |
| 10343 | 11 | After combination of the base standard (802.11ma), subsequent ammendments and more focused individual changes, there remain unresolved inconsistencies between the text in this clause and other clauses. This applies to this clause and all subclauses thereof. | Resolve the inconsistencies. |  | Proposed reject. The commenter does not provide sufficient information to determine either the problem or a remedy that would suffice. |
| 1001 | 11.1.2.1.1 | use correct term | substitute the correct term for "a Basic STBC MCS" which I think is "BSSSTBCBasicMCS" or something like that |  | Proposed counter. Replace "a Basic STBC MCS" with "the Basic STBC MCS, as signalled in the Basic STBC MCS field of its HT Information Element" |
| 2766 | 11.1.2.1.1 | "using a Basic STBC MCS." - This clause is MAC procedures, and this statement is very PHY-specific. Statement needs to be made in terms of MAC and PLME/PHY interfaces. | Suggest "using a second PHY or set of PHY characteristics." |  | Proposed counter. This value is signalled in the HT Information element. See response to CID 1001. |
| 4013 | 11.1.2.1.1 | How does the transmission of the secondary beacon reconcile to the PSMP Service Interval schedules that have already been established as stated in 11.4.4(page 147, lines 11-14); the transmission time of the secondary beacon, may co-incide with the scheduled service interval of an STA. | Once an STBC capable QSTA is admitted into the BSS,does it include a recycling of the TSPEC's that have already been established. This would be a very high overhead. |  | Proposed reject. The commenter proposes no change. In reply to the commenter - the service period is not necessarily tied to the beacon interval, or any multiple or submultiple of it - although it may be. An AP may position a service period so that it doesn't collide with either beacon or secondary beacon - i.e. by using the same period as the beacon and a start offset that is not half a beacon period. |
| 9884 | 11.1.2.1.1 | Secondary beacon is transmitted using "a Basic STBCS MCS". How many Basic STBC MCS rates are there? | Needs clarification |  | Proposed accept. See response to CID 1001. |
| 1002 | 11.1.2.1.1 | vague language | change "be the actual timestamp" to "be set so that it equals the value of the STA’s TSF timer at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STA’s delays through its local PHY from the MAC-PHY interface to its interface with the WM [e.g., antenna, light-emitting diode (LED) emission surface]." |  | Proposed counter. Remove the sentence: "The TSF timestamp of the secondary beacon shall be the actual timestamp" as the behaviour described in 11.1.2 (reproduced by the commenter in the proposed resolution) applies to any beacon transmission. It is therefore not necessary to repeat this for secondary beacons. |
| 2767 | 11.1.2.1.1 | The offset of half a beacon period, if that information is to be used in any way by the STA, needs to be normative on the AP. | Either make such a normative statement about sending the secondary beacon, or delete the statement about TBTT being half period. |  | Proposed accept. Replace ", so that STAs know that the TBTT for this beacon has an offset of half a beacon period." with ". The target transmission time for the secondary beacon is TBTT plus half a beacon period." |
| 10574 | 11.1.2.1.1 | The offset of half a beacon period, if that information is to be used in any way by the STA, needs to be normative on the AP. | Either make such a normative statement about sending the secondary beacon, or drop the statement about TBTT being half period. |  | Proposed duplicate of 2767 |
| 1003 | 11.1.2.1.1 | incorrect language | change "shall be" to "shall contain values which are" |  | Proposed counter. Replace: "Broadcast/multicast traffic transmitted after the secondary beacon shall be identical to the broadcast/multicast traffic transmitted after the primary beacon." with "After transmission of a secondary beacon, the AP shall repeat the transmission using the Basic STBC MCS of all broadcast or mulitcast MPDUs that were transmitted since the previous beacon transmission." This also contains the resolution to CID 1004. |
| 3374 | 11.1.2.1.1 | It is said that all the fields except Timestamp shall be the same as in the primary beacon. This may cause false behaviour if TIM element is forced to be the same. Consider the following case: Non-AP STA (in sleep mode) receives primary beacon indicating it has buffered data. Non-AP STA retrieves the buffered data using legacy or APSD mechanisms BEFORE the secondary beacon => this would cause resetting the TIM field. Now if the TIM element is repeated in the secondary beacon the STA may believe that it has again buffered data which may not be true => it awakes unnecessarily. | TIM element should reflect the actual situation in the secondary beacons. |  | Proposed accept. |
| 5123 | 11.1.2.1.1 | A PS-mode STBC STA in range to decode normal beacon may retrieve all of its buffered data between the primary and secondary beacon. Hence it would make sense to not copy the TIM from the primary beacon into the STBC beacon | allow for STBC beacon's TIM field to differ from primary beacon TIM |  | Proposed accept. |
| 1004 | 11.1.2.1.1 | missing adjective phrase | add to the end of the paragraph: "except that the frames shall be transmitted using the BSSSTBCBasicMCS." |  | Proposed accept. This is incorporated into the response to CID 1003. |
| 1005 | 11.1.2.1.1 | need another sentence to define primary beacons | add a paragraph: "Beacons which are not secondary beacons are primary beacons." |  | Proposed counter. It it preferable to avoid defining a new term if it is just used in one place. Replace: "... shall be identical to the primary beacon" with "... shall be identical to the previous beacon transmission in which the Secondary Beacon field is set to 0." |
| 7476 | 11.1.2.1.1 | "a Basic STBC MCS" - implies there's more than one of them, which is wrong | Replace with "the basic..." |  | Proposed accept. |
| 7477 | 11.1.2.1.1 | "Any other fields inside the beacon shall be identical to the primary beacon. MC/BC traffic transmitted after the secondary beacon shall be identical to the MC/BC traffic transmitted after the primary beacon." This means that a STA that capable of receiving STBC frames that is also close enough to receive the normal beacon and MC/BC traffic will see duplication of MSDUs. We should surely try and minimise this effect. | Add the following: "An STBC-capable STA shall discard either all received STBC encoded BC/MC Data frames, or all received non-STBC encoded BC/MC Data frames. How it makes this decision is a matter of local policy." |  | Proposed accept :0) |
| 837 | 11.1.3.2.2 | The text in this clause could be interpreted as applying to all STA and not just HT-STA | Change to read "The proobe requested transmitted by a HT-STA should contain the HT Capabilities element. This enables a STA transmitting a probe response to ensure that it selects an MCS supported by the intended receiver. In response to a probe request transmitted by a HT-STA, the AP transmits a probe response using a MCS from the supported MCS set received in the HT capabilities element of the corresponding Probe request." |  | Proposed counter - remove the subclause. See CID 3600. |
| 2770 | 11.1.3.2.2 | If a probe request is being transmitted on a channel upon which decodable traffic has been present during the latest 802.11k scan, the probe shall be sent at a rate corresponding to the lowest AP rate detected rather than the lowest possible rate. and probe transmission must respect CCA and CFP protocols, including the extension channel, if any. | Insert language such as "Probe requests must be viewed a supplementary requests for information. Scheduled transmissions and transmissions in progress must supercede probe requests in priority." |  | Proposed reject. The text added by TGn relates to selection of an appropriate MCS, not when transmission of the probe response occurs. See response to CID 3600. |
| 3600 | 11.1.3.2.2 | The added text does not any more information nor does it change the active scanning procedure and there is no reason for adding this (oh btw, if the AP should be able to first decode the Probe response frame - this implies the AP should be quite capable of sendng a response as well. :) | Delete the added text. |  | Proposed accept. Less is more. |
| 10577 | 11.1.3.2.2 | If a probe request is being transmitted on a channel upon which decodable traffic has been present during the latest 802.11k scan, the probe shall be sent at a rate corresponding to the lowest AP rate detected rather than the lowest possible rate. and probe transmission must respect CCA and CFP protocols, including the extension channel, if any. | Insert normative text such as "Probe requests are a supplementary requests for information. Scheduled transmissions and transmissions in progress shall supercede probe requests in priority." |  | Proposed duplicate of 2770 |
| 2772 | 11.2.1.3 | "using a Basic STBC MCS." - This clause is MAC procedures, and this statement is very PHY-specific. Statement needs to be made in terms of MAC and PLME/PHY interfaces. | Suggest "using a second PHY or set of PHY characteristics." |  | Proposed counter. Replace "a Basic STBC MCS" with "the Basic STBC MCS, as signalled in the Basic STBC MCS field of its HT Information Element" |
| 2773 | 11.2.1.5 | "using a Basic STBC MCS." - This clause is MAC procedures, and this statement is very PHY-specific. Statement needs to be made in terms of MAC and PLME/PHY interfaces. | Suggest "using a second PHY or set of PHY characteristics." |  | Proposed counter. Replace "a Basic STBC MCS" with "the Basic STBC MCS, as signalled in the Basic STBC MCS field of its HT Information Element" |
| 2774 | 11.2.1.6 | "using a Basic STBC MCS." - This clause is MAC procedures, and this statement is very PHY-specific. Statement needs to be made in terms of MAC and PLME/PHY interfaces. | Suggest "using a second PHY or set of PHY characteristics." |  | Proposed counter. Replace "a Basic STBC MCS" with "the Basic STBC MCS, as signalled in the Basic STBC MCS field of its HT Information Element" |
| 3375 | 11.2.1.6 | of that type' - what this means? Same in 11.2.1.8. | Clarify |  | Comment relates to 11.2.1.7 |
| 1010 | 11.2.3.1 | some wording problems | change "The MIMO Power Save feature allows a STA to spend most of its time with only one active Rx chain." to "The MIMO Power Save feature allows a STA to operate with only active RX chain for a significant portion of time." |  | Proposed accept. |
| 3606 | 11.2.3.1 | Define HT and non-HT RTS/CTS frames | As in the comment |  | Proposed counter. Replace: remove "non-HT" from "A non-HT RTS/CTS sequence may be used for this purpose" as rules defined elsewhere will ensure that RTS/CTS is a single spatial stream |
| 5125 | 11.2.3.1 | Single stream HT-RTS-CTS may be used as well to enable multiple Rx Chain | Change "non-HT RTS/CTS sequence may be used..." to "either a non-HT RTS/CTS or a HT-RTS/HT-CTS with single stream may be used to ..." |  | Proposed counter. This is unnecessarily verbose and introduces new terms HT-RTS and HT-CTS. Remove "non-HT" as shown in CID 3606. |
| 1016 | 11.2.3.1 | eliminate article confusion + insert a shall | Change "The receiver" to "A dyanmic MIMO power save STA" change "between RTS/CTS" to "between an RTS/CTS" change "true MIMO" to "a true MIMO" change "always enables" to "it shall always enable" change "multiple" to "additional" change the last "it" to "itself" |  | Proposed accept. |
| 1015 | 11.2.3.1 | the language in the clause explicitly mentions an RTS as triggering the mode change - the text needs to be explicit that any frame with RA=dynamic MIMO power save STA's address will suffice | change add a sentence: "Any frame with the value in the RA field matching that of the dyanmic MIMO power save STA shall cause the dynamic MIMO power save STA to enable all of its RX chains for the duration of the current TXOP." |  | Proposed accept. |
| 7497 | 11.2.3.1 | This section doesn't describe how a STA should respond to a peer's static MIMO power saving mode. | Add the statement: " In the case of static MIMO power save mode, the STA maintains only a single active Rx chain active will in this mode. An HT STA shall only transmit single-stream packets directed to a STA that is in static MIMO power save mode." |  | Proposed counter: same intent, but corrected for grammar: "In the case of static MIMO power save mode, the STA maintains only a single active Rx chain active while in this mode. An HT STA shall only transmit single-stream packets directed to a STA that is in static MIMO power save mode." |
| 9895 | 11.2.3.1 | Replies to the single-spatial RTS should also be sent as single-spatial CTS. It is not stated in this section, unless I am missing something. | Add a sentence to state that CTS is sent as single-spatial CTS. |  | Proposed counter. Make no change here, but check that the rules for RTS/CTS MCS selection in 11.6 inisist on single spatial stream. I would suggest that a statement that the basic MCS set should include only MCSs with a single spatial stream suffices. |
| 7499 | 11.2.3.2 | This section seems to be an earlier version of 11.2.3.1 certainly there is no management action frame called reduce MIMO capability. This has been replaced by the static MIMO power saving mode. | Remove the whole subclause. |  | I declare an interest. If TGn accepts this, it resolves the following 10 comments. |
| 441 | 11.2.3.2 | "capable when gets associated." Huh? | Correct the grammar. "when it associates" is suggested. |  | Proposed counter. See CID 7499. |
| 840 | 11.2.3.2 | text should be changed to read the following: | … "to signal whether it is MIMO capable when it associates with another STA" |  | Proposed counter. See CID 7499. |
| 1020 | 11.2.3.2 | article confusion | change "The STA" to "A STA" at the beginning of each of the first three paragraphs |  | Proposed counter. See CID 7499. |
| 1021 | 11.2.3.2 | clumsy wording | change "when gets associated" to "when it successfully associates" |  | Proposed counter. See CID 7499. |
| 1022 | 11.2.3.2 | wording changes | change "accordingly" to "according to" change "associates" to "successfully associates" |  | Proposed counter. See CID 7499. |
| 7042 | 11.2.3.2 | There is no HT capability element called "Max number of Rx spatial channels" | please clarify, or remove sentence |  | Proposed counter. See CID 7499. |
| 1023 | 11.2.3.2 | wording changes, including adding a shall | change "and inform its peers" to "and when it does so, shall inform other STA" |  | Proposed counter. See CID 7499. |
| 4016 | 11.2.3.2 | A STA may dynamically … | Reduced MIMO Capability Management Action Frame is not defined in Table n20; |  | Proposed counter. See CID 7499. |
| 12007 | 11.2.3.2 | Change the rule for the static MIMO power save to be not dependent on beaconing | Change the last sentence to: "An HT STA that transmits this frame should wait to receive the non MIMO frame for transitions into static MIMO power save mode before locally operating its MIMO power save mode." |  | Proposed counter. See CID 7499. |
| 1025 | 11.2.3.2 | indicate the precedence between the different signalling methods for communicating the MIMO power save mode | add text that says that the last received MIMO power save mode indication is the current one |  | Proposed counter. See CID 7499. |
| 3613 | 11.5.2 | Another example of writing reams on a very obvious mechanism. OK, AP and station should establish Block Ack sessions if they have data; what is so special about PSMP that it needs a special mention? | Delete the entire subclause |  | Proposed accept |
| 1053 | 11.5.2 | change the section from informative to normative | change the section from informative to normative |  | Proposed counter. Delete subclause. See CID 3613. |
| 1054 | 11.5.2 | the table herein contains information which is unqualifiedly assertive in construction and which does not appear elsewhere in the document, and therefore needs to be expressed in a normative manner | find the unqualifiedly assertive sentences and change them to normative statements (change the verb tense to use "shall") e.g. the statements about beacons are all candidates for this treatment -- if any of the statements are actually found elsewhere, then do NOT make this a duplicate occurrence of that pre-existing rule |  | Note: this is actually 11.15.2 |
| 2855 | 11.16 | 11.16 is a MAC layer function, not really MAC Management function. It would be better placed in clause 9 | Make change indicated in comment | Defer- Needs more discussions Ed AdHoc: Transfer to Gen AdHoc GenAdHoc: Agrees that this needs more discussion and transfers to MAC | Proposed accept. Move to after 9.15 (PSMP Access). |
| 10519 | 11.16 | 11.16 is a MAC layer function, not really MAC Management function. It would be better placed in clause 9 | as in comment | Defer - Refer to CID-2855 Ed AdHoc: Transfer to Gen AdHoc GenAdHoc: Transfer to MAC | proposed duplicate of 2855 |
| 7489 | 11.16 | It seems that the operation of PCO is a bit like the operation of TSF or beaconing, and should be in section 9. | Find it a new home in section 9. | Defer - Needs more discussions Ed AdHoc: Transfer to Gen AdHoc GenAdhoc: Transfer to MAC | proposed duplicate of 2855 |
| 5120 | 20 | A system containing 1-stream devices can adversely impact the operation of 2-stream devices and the effective bandwidth available to the latter. The result is that the applications running on the 2-stream devices may not work as intended. It is unclear how this undesirable effect is mitigated by the proposed draft. | Clarify how this issue is addressed by the current draft. Define a mechanism that resolves this problem. | Reason for transfer: This comment appears to relate to resource sharing, which is not a PHY issue. | Proposed reject. The same issue already exists with .11b vs .11g devices. The AP has a degree of control over this by defining an appropriate basic rate/mcs set, and refusing association from devices that don't support characteristics it desires. It is neither desirable not feasible to standardise this behaviour. |
| 10713 | 20.1.3 | Greenfield operation shall not be possible if an active scan determines that existing APs are operating. | Institute initialization process consistent with legacy equipment including active scan. | see 2924 | Proposed duplicate of 2924 |
| 10712 | 20.1.3 | Greenfield operation shall not be possible if a scan detected existing APs are operating. | Institute initialization process consistent with legacy equipment including 802.11k scan. | reason for transfer: don't think this is a PHY issue, it relates to MAC protection mechanisms | Proposed duplicate of 2924 |
| 11992 | 8 | The authentication of the HT Control field is not defined. The proposal is to change the related sentence in the 802.11ma-D5.2. Add the following after "The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission)." | Add the following after "The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). "The AAD does not include the header HT Control field, because the HTC field can be incorporated or expelled due to normal IEEE 802.11n operation (e.g., can be included or excluded during retransmission). Example: The initiator transmits aggregation with MPDUs containing the HTC with feedback request. This aggregation gets BA, so the feedback request has been delivered. But part of these MPDUs should be retransmitted. There is no need to attach HTC to those MPDUs. |  | Proposed accept, although the editor will need to edit for use of language (e.g. "expelled"). |
| 10300 | 8.3 | The authentication of the HT Control fileld is not defined. The proposal is to change the related sentence in the 802.11ma-D5.2. Add the following after "The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission)." | Add the following after "The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation (e.g., a rate change during retransmission). "The AAD does not include the header HT Control field, because the HTC field can be incorporated or expelled due to normal IEEE 802.11n operation (e.g., can be included or excluded during retransmission). Example: The initator transmits agrergation with MPDUs containing the HTC with feedback request. This aggregation gets BA, so the feedback request has been delivered. But part of these MPDUs should be retransmitted. There is no need to attach HTC to those MPDUs. |  | Proposed duplicate of 11992 |
| 10314 | Annex A | Capability should be mandatory for HT-APs, optional otherwise. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. | Proposed reject. The PICS correctly represents the requirements of the preceding clauses in which support for HT-delayed BA is implicit if you support delayed BA and you are an HT STA. |
| 10316 | Annex A | Capability should be mandatory for all STAs.. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. | Proposed duplicate of 3144 |
| 10317 | Annex A | Capability should be mandatory for HT-APs, optional otherwise. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. | Proposed reject. The PICS correctly represents the functionality of the preceding clauses in which a STA never has to offer an RD grant, and a STA that is offered an RD grant can completely ignore this. |
| 11909 | A4.15.1 | HTM11.1: QoS Null embedding can be used with HT Control for Link Adaptation in immediate response exchange | add reference to 7.2.2.2 to HTM11.1 | Gen AdHoc: Transfer to MAC | Proposed accept |
| 11910 | A4.15.1 | HTM11: No reference to link adaptation with implicit feedback | Add HTM11.3: Protocol capability: Link adaptation using implicit feedback; References 7.1.3.8, 7.2.2.2, 9.19.5 (proposed new section), 9.20.2; Status: CF15:O | Gen AdHoc: Transfer to MAC | Proposed reject. This PICS entry relates to MCS feedback. There is nothing we need to say about implicit MCS feedback, because there is no associated signaling or normative behaviour. Implicit TxBF feedback does have associated signalling and behaviour, and this is included in item HTM12. |
| 10321 | Annex A | Capability should be mandatory for HT-APs, optional only in STAs which implement HTM11.2. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC | Proposed reject. The PICS correctly represents the functionality of the preceding clauses in which a STA need never generate a +HTC frame, and a STA need never generate a response that is modified in any way by the existence +HTC frame. |
| 10322 | Annex A | Capability should be mandatory for HT-APs, optional only in STAs which implement HTM11.1. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC | Proposed reject. The PICS correctly represents the functionality of the preceding clauses in which a STA need never generate a +HTC frame, and a STA need never generate a response that is modified in any way by the existence +HTC frame. |
| 10328 | Annex A | Capability should be mandatory for HT-APs, optional otherwise. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. | Proposed reject. The PICS correctly represents the functionality of the preceding clauses in which support for STBC is optional for both non-AP STAs and APs. |
| 10329 | Annex A | Capability should be mandatory for HT-APs, optional otherwise. | Modify Status column accordingly. | Gen AdHoc: Transfer to MAC PICS should match feature in resultant text provide in Draft. | Proposed reject. The PICS correctly represents the functionality of the preceding clauses in which support for STBC is optional for both non-AP STAs and APs. |
| 11871 | Annex C | Nss=1 (incl. STA to STA) should benefit from other HT enhancements over 11a and 11g, such as ABF, Short-GI and LDPC | HTP2.3.2 optional, not mandatory |  | Note, the comment wrongly refers to Annex C. Proposed reject. This entry in the PICS states that Nss=2 is mandatory for an AP, but not a non-AP STA. There is nothing preventing support of only Nss=1 between STAs, which is what the commenter appears to want. |
| 3152 | Annex C | Annex C is informative, not normative. Was changed in P802.11REV-ma-D6.0. | Delete all changes to Annex C, as they are no longer needed |  | Proposed counter. Update subclause to track changes in the baseline. However, because there is already a statement of exclusion relating to QoS STA in the baseline, it does not harm to add the same statement relating to HT STA, as it helps the reader of Annex C to discover the scope of what it does actually support. |
| 11787 | Annex C | Annex C is informative, not normative. Was changed in 11ma D6.0 | Delete all changes to Annex C, as they are no longer needed |  | Proposed duplicate of 11787 |
| 108 | Annex C | This section looks a little vague and non-specific. | Eliminate it, or fill it in with the relevant changes. |  | Proposed reject. The changes help the reader of Annex C to determine its scope more accurately. See CID 3152. |
| 1557 | Annex C | No changes to this section at all? | The draft seems to specify quite a number of changes |  | Proposed reject. The commenter does not provide a recommended change. However, in response to the commenter, please see response to CID 3152. |
| 804 | annex c | missing section | formal description of MAC operation, empty. |  | Proposed reject. 802.11e established the precident of not maintaining Annex C. It is not feasible for TGn to maintain Annex C as it is dependent on 802.11e behaviour. |
| 7028 | Annex C | No description of the impact to the MAC of an HT environment | Complete Annex C | Ed: Commenter did not provide T/E classification. Assuming T is the response. | Proposed duplicate of 804 |
| 7341 | Annex C | The formal description of MAC operation is incomplete; actually it's non-existant truth be told. The only logical conclusion is that the drafters didn't believe that the HT mode would require any changes to the previous formal description of the MAC. I don't believe this is the case. I believe the MAC is significantly impacted by the proposed amendment and that a formal description of those changes, and the amended operation thereof is necessary. | Add the modifications to and a complete formal description of the MAC operation as required. |  | Proposed duplicate of 804 |
| 10364 | Annex C | After combination of the base standard (802.11ma), subsequent ammendments and more focused individual changes, there remain unresolved inconsistencies between the text in this clause and other clauses. This applies to this clause and all subclauses thereof. | Resolve the inconsistencies. |  | Proposed reject. It is hard to understand how the additional statement that something is not described is inconsistent. |
| 59 | Annex D | There are dot11RTSEPPSuccessCount and dott11RTSEPPFailureCount. As the word EPP is not used and it is L-SIG protection instead, the names should be changed. | Change the names of the counters to appropriate ones. | Ed AdHoc: Transfer to Gen AdHoc Gen AdHoc: Assign to MAC AdHoc | Proposed Accept. Replace EPP by LSIG. Do a global search and replace for these names. |
| 60 | Annex D | There are dot11RTSEPPSuccessCount and dott11RTSEPPFailureCount. As the word EPP is not used and it is L-SIG protection instead, the names should be changed. | Change the names of the counters to appropriate ones. | Ed AdHoc: Transfer to Gen AdHoc Gen AdHoc: Assign to MAC AdHoc | Proposed accept. Covered by response to CiD 59. |

# Spreadsheet

For convenience of use of this material,  a spreadsheet version of the same information is present in the attached object.