IEEE P802.11
Wireless LANs

| Proposal for the text change to avoid confusion on the matrix dimension in 7.4.8.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joonsuk Kim | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3455 | joonsuk@broadcom.com |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +97248655738 | solomon.trainin@intel.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvalce, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Tomoya Yamaura | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | yamauratomoya@yahoo.co.jp |
| Yuichi Morioka | Sony Corporation | 6-7-35 Kitashinagawa, Shinagawa-ku Tokyo, 1410001 Japan | +81-3-6409-3201 | Yuichi.Morioka@jp.sony.com |
| Assaf Kasher | Intel Corporation | POB 1659, Haifa 31015, Israel | +972-4-8651547 | assaf.kasher@intel.com |
| Gal Basson | Intel Corporation | POB 1659, Haifa 31015, Israel | +97248655405 | gal.basson@intel.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | +44 1223763457 | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 2256, 3808, 3809, 7821, and 12011 in relation to 7.4.8.6

The changes marked in this document are based on TGn Draft version D1.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

CID 2256 addresses the concern about the confusion of the dimension of the matrix. From the draft, subcluases 7.4.8.6 and 7.4.8.7 are about the implicit feedback cases where the channel measurement occurs at the transmitter sending the beamformed packet, while the draft does not clarify it. We propose to change some text to make clear that those subclauses describe such cases to avoid any confusion. This document includes some text change proposed in 06/886r3 which are already accepted such as Sounding Time Stamp and values of unused bits.

# Reference

11-06-886r3 lb84-comment-resolution-explicit-feedback-submission.doc

# CID 2256

TGn Editor: Modify the description of Calibration in the Table n19 (line 11, page 40) as follows:

Replace “MIMO Channel Measurement Report” with “MIMO Reciprocal Channel Measurement Report”.

TGn Editor: Change the Meaning in Action field value of “4” in the Table n22 (line 47, page 46) as follows:

Replace “MIMO Channel Measurement Report” with “MIMO Reciprocal Channel Measurement Report”.

TGn Editor: Replace the title of 7.4.8.6 as follows:

Replace “MIMO Channel Measurement” with “MIMO Reciprocal Channel Measurement”.

TGn Editor: Change the first sentence of 7.4.8.6 as follows:

The MIMO Reciprocal Channel Measurement frame is a management action frame is of category HT. The format of the MIMO Reciprocal Channel Measurement frame body is defined in Table n27.

TGn Editor: Insert the text ("x") in the first two columns of figure n25 in 7.4.8.6 (Ed: Transmit beamforming control field) as follows:

| B0 B1 | B2 B3 | B4 B5 | B6 | B7 | B8 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- | --- |
| Nrx | Ntx | Calibration Sequence | Calibration Complete | Explicit Channel Feedback | Explicit Feedback Format | Reserved |

Figure n25—Transmit Beamforming Control field

TGn Editor: Change line 60-62 in page 50 as follows:

The MCMR Segment Sequence (1 octet) contains the segment sequence number when the MIMO Reciprocal Channel Measurement Report has been segmented. The MCMR Segment Sequence of the first segment is 0. The complete MIMO Reciprocal Channel Measurement Report contains Ns × Ni × Nr Nst  Ntx  Nrx complex coefficients.

TGn Editor: Change line 29 in page 50 as follows:

Hence the total length of the complete MIMO Reciprocal Channel Measurement Report is 3 × Ns × Ni × Nr 3 × Nst  Ntx  Nrx octets.

TGn Editor: Change figure n27 as follows

Figure n27: Encoding of MIMO Reciprocal Channl Measurement Report (40MHz)

For each subcarrier in the following order: (-58, …-2, 2, …, +58), include { For each of Nitx antennas at the initiating STA in order: (1, …, Nitx) { Include Nrx complex coefficients one for each antenna at the receiving STA in order: (1, …, Nrx ) } }

TGn Editor: Change figure n28 as follows

Figure n28: Encoding of MIMO Reciprocal Channel Measurement Report (20MHz)

For each subcarrier in the following order: (-28, …-1, 1, …, +28), include { For each of Nitx antennas at the initiating STA in order: (1, …, Nitx) { Include Nrx complex coefficients one for each antenna at the receiving STA in order: (1, …, Nrx ) } }

TGn Efitor: Change the text from line 64 in Page 51 to line 21 in page 52 as follows:

The complete MIMO Reciprocal Channel Measurement Report may need to be transmitted using more than one MAC frame. For example, with Nst = 114, Nitx = 2, Nrx = 2, there are 1368 octets (456 coefficients) in a single MIMO Reciprocal Channel Measurement Report element. The maximum length of the MIMO Reciprocal Channel Measurement Report field is 5472 octets (1824 coefficients) for the case with Nst = 114, Nitx = 4, and Nrx = 4. If the MIMO Reciprocal Channel Measurement Report is longer than 1368 octets, the first 1368 octets are included in the first frame (MCMR Segment Sequence = 0) and the remaining octets are included in subsequent frames (MCMR Segment Sequence = 1, 2, etc.).

TGn Editor: Modify lines 59-65 in page 52 as follows:

The values of the three fields Nrx, Nitx, and Calibration Sequence are set to the same values as in the received MIMO Reciprocal Channel Measurement Report.

The Reciprocity Correction Vector contains Nst × Nrx complex coefficients. Each complex coefficient (3 octets) is represented by its 12-bit twos complement real and 12-bit twos complement imaginary parts (see Table n29). The order of correction vector coefficients is as shown in Figure n29 and Figure n30.

TGn Editor: Modify figure 29 (Ed: Encoding of the Reciprocity Correction Vector (40MHz)) as follows:

For each subcarrier in the following order: (-58, …-2, 2, …, +58), include { Include Nrx complex coefficients one for each antenna at the receiving STA in order: (1, …, Nrx ) }

TGn Editor: Modify Figure n30 (Ed: Encoding of the Reciprocity Correction Vector (20MHz)) as follows:

For each subcarrier in the following order: (-28, …-1, 1, …, +28), include { Include Nrx complex coefficients one for each antenna at the receiving STA in order: (1, …, Nrx ) }

TGn Editor: Modify the description of “A responder in a calibration exchange” in the Table n56 in 9.17.2.1 as follows:

Replace “MIMO Channel Measurement Report” with “MIMO Reciprocal Channel Measurement Report”.

TGn Editor: Change the description of the frame from STA B in Step 2 in the Figure n45 in 9.17.2.2.2 (page 112) as follows:

Replace “MIMO Channel Measurement Report” with “MIMO Reciprocal Channel Measurement Report”.

TGn Editor: Replace the word in the first sentence in line 31in 9.17.2.1 (page 112) as follows:

Replace “MIMO Channel Measurement Report” with “MIMO Reciprocal Channel Measurement Report”.

TGn Editor: Change the paragraph from line 31 to line 47 in 9.17.2.1 (page 112) as follows:

When the MIMO channel measurements become available at STA B, STA B sends one or more MIMO Reciprocal Channel Measurement frames that contain the MIMO Reciprocal Channel Measurement Report (Step 2 in Table n45). STA B sets the Calibration Complete field to 1 in this packet in case the STA B is not capable of transmit beamforming or does not desire to obtain its Reciprocity Correction matrices from STA A. In this case STA A does not send a Reciprocity Correction frame and the calibration procedure is complete.

In case that both STAs are capable of transmit beamforming, once STA A has the MIMO Reciprocal Channel Measurement Report from STA B, along with its local channel measurement measured from the Calibration Sounding Response transmitted by STA B, STA A can compute the correction matrices required for both itself and STA B (Step 3 in Table n45). If STA B did not set the Calibration Complete field to 1 in the MIMO Reciprocal Channel Measurement frame to 1, STA A transmits the Reciprocity Correction frame to complete the calibration procedure (Step 4 in Table n45). The message includes the Reciprocity Correction Vector for STA B that provides reciprocity correction on the receive channel estimates at STA B. STA B applies the supplied reciprocity correction to its transmit chain. The Calibration Complete field in the Reciprocity Correction frame is set to 1.

# CID 2256, 3809, 12011

TGn Editor: Modify the Table n27 in 7.4.8.6 and Table n29 in 7.4.8.7 (including title) as follows:

- Table n27 – MIMO Reciprocal Channel Measurement

| Order | Information | Name | Size (octets) | Value |
| --- | --- | --- | --- | --- |
| 1 | Category | HT | 1 | 7 |
| 2 | Action | MIMO Reciprocal Channel Measurement Report | 1 | 4 |
| 3 | Transmit Beamforming Control | Transmit Beamforming Control | 2 | See Figure n25 |
| 4 | MCMR Segment Sequence | MCMR Segment Sequence | 1 | See text |
| 5 | Explicit Feedback Sequence Sounding Time Stamp | Explicit Feedback Sequence Sounding Time Stamp | 1 4 | Reserved |
| 6 | MIMO Reciprocal Channel Measurement Report | MIMO Reciprocal Channel Measurement Report | 3 × Nst × Ntx × Nrx | See Figure n27and Figure n28 |
| Where Nst = Number of subcarriers in the Sounding HT-LTF Nrx = Number of antennas at the STA transmitting the MIMO Reciprocal Channel Measurement Ntx = Number of antennas at the STA to which the frame is directed |  |  |  |  |

- Table n29 – Reciprocity Correction

| Order | Information | Name | Size (octets) | Value |
| --- | --- | --- | --- | --- |
| 1 | Category | HT | 1 | 7 |
| 2 | Action | Reciprocity Correction | 1 | 5 |
| 3 | Transmit Beamforming Control | Transmit Beamforming Control | 2 | See Figure n25 |
| 4 | MCMR Segment Sequence | MCMR Segment Sequence | 1 | See text |
| 5 | Explicit Feedback Sequence Sounding Time Stamp | Explicit Feedback Sequence Sounding Time Stamp | 1 4 | Reserved |
| 6 | Reciprocity Correction Vector | Reciprocity Correction Vector | 3 × Nst × Nrx | See text |
| Where Nst = Number of subcarriers in the Calibration Training symbols Nrx = Number of antennas at the STA to which this frame is directed |  |  |  |  |

# CID 2256, 3808

TGn Editor: Modify the Table n28 in 7.4.8.6 as follows:

Table n28—Use of transmit beamforming control fields (MIMO Reciprocal Channel measurement)

| Field | Description |
| --- | --- |
| Nrx Index | Number of antennas at the STA transmitting the MIMO Reciprocal Channel Measurement Report, less one Set to 0 for 1 antenna Set to 1 for 2 antennas Set to 2 for 3 antennas Set to 3 for 4 antennas |
| Ntx Index | Number of antennas at the STA to which the frame is addressed, less one Set to 0 for 1 antenna Set to 1 for 2 antennas Set to 2 for 3 antennas Set to 3 for 4 antennas |
| Calibration sequence | The value of this field is unchanged for the frame exchanges during one calibration procedure |
| Calibration complete | The Calibration Complete field is set to 1 if the STA transmitting the MIMO Reciprocal Channel Measurement frame(s) declares the calibration procedure complete without requiring the receiving STA to transmit a Reciprocity Correction Vector frame. |
| Explicit Channel Feedback | Set to 0 |
| Explicit Feedback Format | Set to 0000 |

# CID 7821

TGn Editor: Modify the Table n30 in 7.4.8.7 as follows:

Table n30—Use of transmit beamforming control fields (MIMO Reciprocal Channel measurement)

| Field | Description |
| --- | --- |
| Nrx Index | Number of antennas at the STA transmitting the MIMO Reciprocal Channel Measurement Report, less one Set to 0 for 1 antenna Set to 1 for 2 antennas Set to 2 for 3 antennas Set to 3 for 4 antennas |
| Ntx Index | Number of antennas at the STA to which the frame is addressed, less one Set to 0 for 1 antenna Set to 1 for 2 antennas Set to 2 for 3 antennas Set to 3 for 4 antennas |
| Calibration sequence | The value of this field is unchanged for the frame exchanges during one calibration procedure |
| Calibration complete | The Calibration Complete field is set to 1 |
| Explicit Channel Feedback | Set to 0 |
| Explicit Feedback Format | Set to 0000 |