IEEE P802.11
Wireless LANs

| TGn LB84 Submission on PHY SAP Interface Beamforming Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel | POB 1659, Haifa 31015, Israel | 97248651547 | assaf.kasher@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 2645, 2674, 2682, 2696, 2709, 2714, 3087

The changes marked in this document are based on TGn Draft version D1.03 and on the acception of submission 1315.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

## Introduction to PLME interface comments

The PLME (PHY Layer Management Entitty) interface is the virtual management interface defined between the MAC and the PHY consisting of the RXVECTOR and the TXVECTOR.  It is virtual in the sense that it is not designed to enable independent implementation of MAC and PHY.  It does not specify field widths and field coding.  It defines the information being passed between the MAC and PHY but not how it is passed.

Several comments address "missing" parts in the PLME interface.  These parts are CSI matrices, calibration information, steering matrix (as calculated by the PHY) and the estimated MCS.  The current interface has the EXPANSION_MAT field in the TXVECTOR to pass steering matrices from the MAC to the PHY.  It also has the CHAN_MAT field in the RXVECTOR to pass channel measurements from the MAC to the PHY. This is based on the idea that all information needed for CSI, compressed and un-compressed steering matrices and calibration response is available at the MAC based on the CHAN_MAT information.  This idea simplifies the interface.  However, it means that all the information in sections 21.3.11.2.1 (CSI Matrices Feedback), 21.3.1.11.2.2, (Non-compressed Steering Matrices FB), 21.3.11.2.3 (Compressed Steering Matrices Feedback) will have to go into clause 7.4.8 which is already overloaded.  I believe most people will agree that the information provided in these subclauses is essentially "PHY" information and would be better present in a PHY clause.   Therefore I suggest accepting the comments, at least in principle.  The calibration matrices will be handed through the CSI matrices feedback.

## Discussion of each Comment:

| 2645 | Y | 118 | 20 | 9.19.2 | T |  | How is the MCS estimate transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

TGn Editor:  Insert the following line to the TXVECTOR/RXVECTOR table (see submission 06-1315)

|  | Condition | Value | Parameter is present in which? (See NOTE) |  |
| --- | --- | --- | --- | --- |
|  |  |  | TXVECTOR Y/N/O | RXVECTOR Y/N/O |
| REC_MCS | FORMAT is HT_MM or HT_GF | The MCS that the STA's receiver recommends. | N | O |

| 2674 | Y | 121 | 11 | 9.20.2 | T |  | How is the Steering Matrix transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See below

| 2682 | Y | 122 | 16 | 9.20.2.1.2 | T |  | How is the Calibration Sounding Response information transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See below as CSI_MATRICES

| 2696 | Y | 124 | 17 | 9.20.3.3 | T |  | How is the CSI information transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

See below

| 2709 | Y | 126 | 15 | 9.21.2 | T |  | How is the Channel State information transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Reason for reject:  As in CID 2696

| 2714 | Y | 127 | 15 | 9.21.2 | T |  | How is the Channel State information transferred from the PHY up to the MAC for the MAC to respond? | Specify this in the PLME in 10.4, or PHY service spec in 12.3. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Reject

Reason for reject:  Sounding will be passed to the MAC as CSI matrices.

| 3087 | Y | 217 | 23 | 20.3.5.2.3 | ST |  | This should be a ref to the PLME interface that the MAC uses to fatch this info from the PHY. There should be no mention in the PHY clause about things like Action Frames. | delete the sentence, or define and reference the PLME interface that is used. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Counter

There is no need to reference the PLME as the actions defined here are not specified as action of the PHY, but as a procedure performed by the STA.  The reference to action frame or assembly can be removed.

TGn Editor: replace the last sentence in the first paragraph of section 20.3.5.2.3:

These compressed matrices are assembled in an action frame asdescribed in 7.4.7.8.

with:

These compressed matrices are sent to the transmitter. (Ed: Compressed steering Matrices Feedback frame)

TGn Editor: replace the following lines from the RXVECTOR/TXVECTOR table (submission 1315) w

| EXPANSION_MAT | FORMAT is HT_MM or HT_GF | Contains the spatial expansion matrices for transforming space time streams into transmit streams. The use of spatial expansion matrices is defined in (insert ref to 21.3.10.10.1 – spatial mapping). This parameter contains complex matrices of size . This field is only present if transmit beamforming is applied. |
| --- | --- | --- |
|  | Otherwise | Not present |
| EXPANSION_MAT_ON | FORMAT is HT_MM or GF | Enumerated type: EXPANSION_MAT_ON indicates that EXPANSION_MAT is valid NO_EXPANSION_MAT_ON indicates that EXPANSION_MAT is not valid |
|  | Otherwise | Not present |
| CHAN_MAT |  | The CHAN_MAT parameter contains the channel response matrices that were measured during the reception of the current PPDU. Each matrix has rows and columns corresponding to the number of receive chains and the number of space time streams that were measured during the current PPDU. Each element in the matrix is a complex number. The number of matrices is - the sum of the number of data subcarriers and the number of pilot subcarriers in the current received PPDU. |

with

|  |  |  |  | Condition |  |  |  | Value |  |  |  | Parameter is present in which? (See NOTE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | TXVECTOR Y/N/O |  | RXVECTOR Y/N/O |  |  |  |
| EXPANSION_MAT_TYPE |  |  | FORMAT is HT_MM or GF |  |  |  | Enumerated type: NO_EXPANSION_MAT indicates that EXPANSION_MAT is not valid COMPRESSED_SV indicates that EXPANSION_MAT is a set of compressed steering matrices NON_COMPRESSED_SV indicates that EXPANSION_MAT is a set of non-compressed steering matrices. CSI_MATRICES indicates that EXAPNSION_MAT is a set of channel state matrices. |  |  |  | Y |  |  |  | N |  |  |
|  |  | Otherwise |  |  |  | Not present |  |  |  |  |  |  |  |  |  |  |  |
| EXPANSION_MAT |  | EXPANSION_MAT_TYPE is COMPRESSED_SV |  |  |  | Contains a set of compressed steering matrices as defined in 21.3.11.2.3 (Ed: Compressed steering matrix feedback). The number of elements depends on the number of spatial streams and the number of transmit chains. |  |  |  | Y |  |  |  |  |  | N |  |
|  |  | EXPANSION_MAT_TYPE is NON_COMPRESSED_SV |  |  |  | Contains a set of non-compressed steering matrices as defined in 21.3.11.2.2 (Ed: Non-compressed steering matrix feedback). The number of complex elements is NST×Nr×Ncc where NST is the total number of subcarriers, Nc is the number of columns and Nr is the number of rows in each matrix. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | EXPANSION _MAT_TYPE is CSI_MATRICES |  |  |  | Contains a set of CSI marices as defined in 21.3.11.2.1 (Ed: CSI matrices feedback). The number of complex elements is NST×Nr×Nc where NST is the total number of subcarriers, Nc is the number of columns and Nr is the number of rows in each matrix. |  |  |  |  |  |  |  |  |  |  |  |
|  | (FORMAT is not HT_MM or GF) or EXPANSION_MAT_TYPE is NO_EXPANSION_MAT |  |  |  | Not present |  |  |  |  |  |  |  |  |  |  |  |  |
| CHAN_MAT_TYPE |  | FORMAT is HT_MM or GF |  |  |  | Enumerated type: COMPRESSED_SV indicates that CHAN_MAT is a set of compressed steering vector matrices NON_COMPRESSED_SV indicates that CHAN_MAT is a set of non-compressed steering vector matrices. CSI_MATRICES indicates that CHAN_MAT is a set of channel state matrices. |  |  |  | N |  |  |  |  |  | Y |  |
|  |  | Otherwise |  |  |  | Not present |  |  |  |  |  |  |  |  |  |  |  |
| CHAN_MAT |  | CHAN_MAT_TYPE is COMPRESSED_SV |  |  |  | CHAN_MAT contains a set of compressed steering matrices as defined in 21.3.11.2.3 (Ed: Compressed steering matrix feedback) based on the channel measured during the training symbols of the received PPDU. The number of elements depends on the number of spatial streams and the number of transmit chains. |  |  |  | N |  |  |  |  |  | Y |  |
|  |  | CHAN_MAT_TYPE is NON_COMPRESSED_SV |  |  |  | CHAN_MAT contains a set of non-compressed steering matrices as defined in 21.3.11.2.2 (Ed: non-compressed steering matrix feedback) based on the channel measured during the training symbols of the received PPDU. The number of complex elements is NST×Nr×Nc where NST is the total number of subcarriers, Nc is the number of columns and Nr is the number of rows in each matrix. |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CHAN_MAT_TYPE is CSI_MATRICES |  |  |  | CHAN_MAT contains a set of CSI matrices as defined in 21.3.11.2.1 (Ed: CSI matrices feedback) based on the channel measured during the training symbols of the received PPDU. The number of complex elements is NST×Nr×Nc where NST is the total number of subcarriers, Nc is the number of columns and Nr is the number of rows in each matrix. |  |  |  |  |  |  |  |  |  |  |  |