IEEE P802.11
Wireless LANs

| Information Elements for RA-OLSR protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kyeongsoo (Joseph) Kim | STMicroelectronics, Inc. | 1060 East Brokaw Road, MS 212, San Jose, CA 95131 | +1-408-451-8137 | kyeongsoo.kim@st.com |
| Azman-Osman Lim | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6868 | aolim@nict.go.jp |
| Youiti Kado | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6900 | kado@nict.go.jp |
| Bing Zhang | National Institute of Information and Communications Technology (NICT) | 3-5 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0289, Japan | +81-774-98-6820 | zhang@nict.go.jp |
| Masanori Nozaki | Oki Electric Industry Co., Ltd. | 2-5-7 Honmachi, Chuo-ku, Osaka 541-0053, Japan | +81-6-6260-0700 | nozaki765@oki.com |
| Oyunchimeg Shagdar | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1532 | oyunaa@atr.jp |
| Mahdad N. Shirazi | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1506 | shirazi@atr.jp |
| Suhua Tang | ATR Adaptive Communication Research Laboratories | 2-2-2 Hikaridai, Seika-cho, Soraku-gun, Kyoto 619-0288, Japan | +81-774-95-1544 | shtang@atr.jp |

Abstract

This submission provides normative texts specifying information elements and an action frame format used by the RA-OLSR protocol, which are missing in the current draft D0.03.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Background

The current draft P802.11s/D0.03 [1] describes the RA-OLSR as an optional routing protocol in clause 11A.5 but does not specify an action frame format and information elements used by the protocol in relevant clauses. This contribution provides normative texts for the specification of action frame format and information elements for the RA-OLSR protocol.

- The following is normative text proposed as an amendment to P802.11s/D0.03.

Insert new clauses after 7.3.2.58 as follows:

      - RA-OLSR Information Elements

RA-OLSR IEs are carried in RA-OLSR frames (defined in clause 7.4.5.14) and have the following common format:

| Octets: 1 | 1 | 1 | 6 | 1 | 1 | 2 | Variable |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Vtime | Originator Address | Time To Live (TTL) | Hop Count | Message Sequence Number (MSN) | IE-specific fields |

Figure s1: Format of RA-OLSR information elements

Table s1: Fields common to all RA-OLSR information elements

| Field | Value/Description |
| --- | --- |
| ID | TBD |
| Length | Length of the IE |
| Vtime | Validity time during which a node must consider the information contained in the IE as valid after its reception, unless a more recent update to the information is received. The validity time is represented by its mantissa (four highest bits of Vtime field) and by its exponent (four lowest bits of Vtime field). In other words: Vtime = C * (1+a/16) * 2^b [in seconds] where ‘a’ is the integer represented by the four highest bits of Vtime field and ‘b’ the integer represented by the four lowest bits of Vtime field. The recommended value of the scaling factor ‘C’ is specified in clause 11A.5.14. |
| Originator Address | The MAC address of the node that has originally generated this IE. This may be different from the sender-address in case that multiple IEs are encapsulated in one frame. |
| Time To Live (TTL) | The maximum number of retransmissions of an IE. Before an IE is retransmitted, the TTL must be decremented by 1. An IE with TTL equal to 0 or 1 must not be retransmitted under any circumstances. |
| Hop Count | The number of hops an IE has attained. Before an IE is retransmitted, the Hop Count must be incremented by 1. The Hop Count is initialized to 0 by the originator of the IE. |
| Message Sequence Number (MSN) | A sequence number assigned by the originator node, which ensures that each message (i.e., IE) can be uniquely identified in the network. The MSN is increased by 1 for each IE originating from the node. “Wrap-around” is handled as described in clause 11A.5.15. |
| IE-Specific Fields | Fields specific to each IE |

Fields specific to each IE are described in the following subclauses.

        - RA-OLSR HELLO Element

| Octets: 1 | 1 | 1 | 2 | 6 | 4 | … | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Htime | Willingness | Link Code | Link Message Size | Neighbor Interface Address | Link Metric | … | Neighbor Interface Address | Link Metric |

| … | 1 | 2 | 6 | 4 | … | 6 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| … | Link Code | Link Message Size | Neighbor Interface Address | Link Metric | … | Neighbor Interface Address | Link Metric |

Figure s2: Fields specific to RA-OLSR HELLO element

Table s2: Fields specific to RA-OLSR HELLO element

| Field | Value/Description |
| --- | --- |
| Htime | HELLO emission interval used by the node on this particular interface. The HELLO emission interval is represented by its mantissa (four highest bits of Vtime field) and by its exponent (four lowest bits of Vtime field). In other words: Vtime = C * (1+a/16) * 2^b [in seconds] where ‘a’ is the integer represented by the four highest bits of Vtime field and ‘b’ the integer represented by the four lowest bits of Vtime field. The recommended value of the scaling factor ‘C’ is specified in clause 11A.5.14. |
| Willingness | The willingness of an MP to carry and forward traffic for other MPs (i.e., to be selected as MPR). An MP with willingness WILL_NEVER must never be selected as MPR by any MP. An MP with willingness WILL_ALWAYS must always be selected as MPR. By default, an MP should advertise a willingness of WILL_DEFAULT (see clause 11A.5.14.6 for willingness constants). |
| Link Code | Link state (see Clause 11A.3.2). One additional link state: MPR_NEIGH – indicating that the neighbors have at least one symmetrical link AND have been selected as MPR by the sender. |
| Link Message Size | The size of each link message counted in octets and measured from the beginning of the preceding “Link Code” field until the next “Link Code” field (or to the end of the IE if there are no more link types). |
| Neighbor Interface Address | The MAC address of an interface of a neighbor MP. |
| Link Metric | An example is the Airtime cost in clause 11A.4.2.1. |

        - RA-OLSR Topology Control (TC) Element

| Octets: 2 | 6 | 4 | … | 6 | 4 |
| --- | --- | --- | --- | --- | --- |
| Advertised Neighbor Sequence Number (ANSN) | Advertised Neighbor Main Addresses | Link Metric | … | Advertised Neighbor Main Addresses | Link Metric |

Figure s3: Fields specific to RA-OLSR TC element

Table s3: Fields specific to RA-OLSR TC element

| Field | Value/Description |
| --- | --- |
| ANSN | A sequence number associated with the advertised neighbor set. Every time an MP detects a change in its advertised neighbor set, it increments this sequence number (“Wrap-around” is handled as described in clause 11A.5. 15). This number is sent in this ANSN field of the TC IE to keep track of the most recent information. When an MP receives a TC IE, it can decide on the basis of this advertised ANSN, whether or not the received information about the advertised neighbors of the originator MP is more recent than what it already has. |
| Advertised Neighbor Main Address | The main address of a neighbor MP. All main addresses of the advertised neighbors of the originator MP are put in the TC IE. If the resulting IE cannot fit into one frame (due to maximum allowed frame size as imposed by the network), more TC IEs will be generated for any advertised neighbor addresses that have not been transmitted and carried in separate frames until the entire advertised neighbor set has been sent. Extra main addresses of neighbor MPs may be included, if redundancy is desired. Advertisement Neighbor Main Address pairs with its link metric. If an advertised neighbor is reachable through more than one link, the link with the best quality (smallest cost value) is selected and advertised. |
| Link Metric | An example is the Airtime cost in clause 11A.4.2.1. |

        - RA-OLSR Multiple Interface Declaration (MID) Element

| Octets: 6 | 6 | … |
| --- | --- | --- |
| RA-OLSR Interface Address | RA-OLSR Interface Address | … |

Figure s4: Fields specific to RA-OLSR MID element

Table s4: Fields specific to RA-OLSR MID element

| Field | Value/Description |
| --- | --- |
| RA-OLSR Interface Address | The address of an RA-OLSR interface of the MP, excluding the MPs main address (which is already indicated in the Originator Address field). All interface addresses other than the main address of the originator MP are put in the MID IE. If the resulting IE cannot fit into one frame (due to maximum allowed frame size as imposed by the network), more MID IEs will be generated for any interface addresses that have not been transmitted and carried in separate frames until the entire interface addresses have been sent. |

        - RA-OLSR Local Association Base Advertisement (LABA) Element

| Octets: 1 | 1 | 6 | 1 | … | 6 | 1 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Block Index (1) | Block Message Size | STA Address | STA Sequence Number | … | STA Address | STA Sequence Number | … |

| 1 | 1 | 6 | 1 | … | 6 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Block Index (N) | Block Message Size | STA Address | STA Sequence Number | … | STA Address | STA Sequence Number |

Figure s5: Fields specific to RA-OLSR LABA element

Table s22:  Fields specific to RA-OLSR LABA element

| Field | Value/Description |
| --- | --- |
| Block Index | The index of a block that stores a list of STA addresses with their sequence numbers |
| Block Message Size | The size of a block counted in octets and measured from the beginning of the preceding “Block Index” field until the next “Block Index” field (or to the end of the IE if there are no more blocks). |
| STA Address | The MAC address of the associated STA. Note that a station address does not include the “Group MAC address bit” in the 48-bit MAC address as described for Local Association Base (LAB) and Global Association Base (GAB) in clause 11A.5.4. |
| STA Sequence Number | The sequence number in the association management frame sent by the STA |

        - RA-OLSR Local Association Base Checksum Advertisement (LABCA) Element

| Octets: 1 | 16 | … | 1 | 16 |
| --- | --- | --- | --- | --- |
| Block Index (1) | Block Checksum | … | Block Index (N) | Block Checksum |

Figure s6: Fields specific to RA-OLSR LABCA element

Table s22:  Fields specific to RA-OLSR LABCA element

| Field | Value/Description |
| --- | --- |
| Block Index | The index of a block that stores a list of STA addresses with their sequence numbers |
| Block Checksum | The checksum of a block (see clause 11A.5.13.7.5 for details of the checksum calculation). |

        - RA-OLSR Association Base Block Request (ABBR) Element

| Octets: 1 | … | 1 |
| --- | --- | --- |
| Block Index | … | Block Index |

Figure s7: Fields specific to RA-OLSR ABBR element

Table s22:  Fields specific to RA-OLSR ABBR element

| Field | Value/Description |
| --- | --- |
| Block Index | The index of a block that has been detected to be inconsistent and is requested for readvertisement by the MP generating this IE. |

Update Table s15 in clause 7.4.5 as follows (changes are in red color):

Table s5: Mesh management Action field values

| Action field value | Description | Application |
| --- | --- | --- |
| 0 | Local Link state announcement | Neighbor discovery |
| 1 | Peer Link Disconnect | Neighbor discovery |
| 2 | Route Request | HWMP routing |
| 3 | Route Reply | HWMP routing |
| 4 | Route Error | HWMP routing |
| 5 | Route Reply Ack | HWMP routing |
| 6 | Congestion Control Request | Congestion Control |
| 7 | Congestion Control Response | Congestion Control |
| 8 | Neighborhood Congestion Announcement | Congestion Control |
| 9 | Mesh Deterministic Access (MDA) | MDA |
| 10 | Beacon Timing Request | Beaconing and Synchronization |
| 11 | Beacon Timing Response | Beaconing and Synchronization |
| 12 | Non-mesh Action Encapsulation | Action |
| 13 | Radio Aware Optimized Link State Routing (RA-OLSR) | RA-OLSR |
| 16-254 | Reserved |  |
| 255 | Vender Specific Mesh Management | Vender Specific |

Insert a new clause before 7.4.5.14 as follows:

- 7.4.5.14 RA-OLSR frame format

The RA-OLSR frame format uses the Action frame body format and is transmitted by an MP to one (in unicast manner) or more neighbor MPs (in broadcast manner) in a WLAN mesh where the RA-OLSR protocol is in use as its active path selection protocol. The frame format is shown in Figure s66.

| Octets: 1 | 1 | Variable |
| --- | --- | --- |
| Category | Action | One or more RA-OLSR elements |

Figure s66: RA-OLSR frame format

The Category field shall be set to 4 (representing mesh management).

The Action field shall be set to 13 (representing RA-OLSR).

The RA-OLSR elements are described in clause 7.3.2.59.

References:

- Draft in progress, IEEEP802.11s/D0.03 Draft Amendment to Standard for Information Technology - Telecommunications and Information Exchange Between Systems - LAN/MAN Specific Requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications: Amendment: ESS Mesh Networking, August 2006.

This is based on the assumption that MD5 is used in checksum calculation.